UnifiedPOS Retail Peripheral Architecture

Version 1.16  RCSD

International Standard

For Implementation of Point Of Service Peripherals

OMG Document Number: retail/2020-02-xx

Original submission date: February 18th, 2019

Standard document URL: https://www.omg.org/spec/UPOS/

This proposal adds to and extends the UPOS 1.15 standard.

Copyright © 2017, Object Management Group, Inc.

USE OF SPECIFICATION - TERMS, CONDITIONS & NOTICES

The material in this document details an Object Management Group specification in accordance with the terms, conditions and notices set forth below. This document does not represent a commitment to implement any portion of this specification in any company's products. The information contained in this document is subject to change without notice.

LICENSES

The companies listed above have granted to the Object Management Group, Inc. (OMG) a nonexclusive, royalty-free, paid up, worldwide license to copy and distribute this document and to modify this document and distribute copies of the modified version. Each of the copyright holders listed above has agreed that no person shall be deemed to have infringed the copyright in the included material of any such copyright holder by reason of having used the specification set forth herein or having conformed any computer software to the specification.

Subject to all of the terms and conditions below, the owners of the copyright in this specification hereby grant you a fully-paid up, non-exclusive, nontransferable, perpetual, worldwide license (without the right to sublicense), to use this specification to create and distribute software and special purpose specifications that are based upon this specification, and to use, copy, and distribute this specification as provided under the Copyright Act; provided that: (1) both the copyright notice identified above and this permission notice appear on any copies of this specification; (2) the use of the specifications is for informational purposes and will not be copied or posted on any network computer or broadcast in any media and will not be otherwise resold or transferred for commercial purposes; and (3) no modifications are made to this specification. This limited permission automatically terminates without notice if you breach any of these terms or conditions. Upon termination, you will destroy immediately any copies of the specifications in your possession or control.

PATENTS

The attention of adopters is directed to the possibility that compliance with or adoption of OMG specifications may require use of an invention covered by patent rights. OMG shall not be responsible for identifying patents for which a license may be required by any OMG specification, or for conducting legal inquiries into the legal validity or scope of those patents that are brought to its attention. OMG specifications are prospective and advisory only. Prospective users are responsible for protecting themselves against liability for infringement of patents.

IPR Mode

This specification is issued under the RAND Mode based on the OMG IPR Policy.

OMG IPR Policy   https://www.omg.org/cgi-bin/doc.cgi?ipr

GENERAL USE RESTRICTIONS

Any unauthorized use of this specification may violate copyright laws, trademark laws, and communications regulations and statutes. This document contains information which is protected by copyright. All Rights Reserved. No part of this work covered by copyright herein may be reproduced or used in any form or by any means--graphic, electronic, or mechanical, including photocopying, recording, taping, or information storage and retrieval systems--without permission of the copyright owner.

DISCLAIMER OF WARRANTY

WHILE THIS PUBLICATION IS BELIEVED TO BE ACCURATE, IT IS PROVIDED "AS IS" AND MAY CONTAIN ERRORS OR MISPRINTS. THE OBJECT MANAGEMENT GROUP AND THE COMPANIES LISTED ABOVE MAKE NO WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, WITH REGARD TO THIS PUBLICATION, INCLUDING BUT NOT LIMITED TO ANY WARRANTY OF TITLE OR OWNERSHIP, IMPLIED WARRANTY OF MERCHANTABILITY OR WARRANTY OF FITNESS FOR A PARTICULAR PURPOSE OR USE.  IN NO EVENT SHALL THE OBJECT MANAGEMENT GROUP OR ANY OF THE COMPANIES LISTED ABOVE BE LIABLE FOR ERRORS CONTAINED HEREIN OR FOR DIRECT, INDIRECT, INCIDENTAL, SPECIAL, CONSEQUENTIAL, RELIANCE OR COVER DAMAGES, INCLUDING LOSS OF PROFITS, REVENUE, DATA OR USE, INCURRED BY ANY USER OR ANY THIRD PARTY IN CONNECTION WITH THE FURNISHING, PERFORMANCE, OR USE OF THIS MATERIAL, EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

The entire risk as to the quality and performance of software developed using this specification is borne by you. This disclaimer of warranty constitutes an essential part of the license granted to you to use this specification.

RESTRICTED RIGHTS LEGEND

Use, duplication or disclosure by the U.S. Government  is subject to the restrictions set forth in subparagraph (c) (1) (ii) of The Rights in Technical Data and Computer Software Clause at DFARS 252.227-7013 or in subparagraph (c)(1) and (2) of the Commercial Computer Software - Restricted Rights clauses at 48 C.F.R. 52.227-19 or as specified in 48 C.F.R. 227-7202-2 of the DoD F.A.R. Supplement and its successors, or as specified in 48 C.F.R. 12.212 of the Federal Acquisition Regulations and its successors, as applicable. The specification copyright owners are as indicated above and may be contacted through the Object Management Group, 109 Highland Avenue, Needham, MA 02494, U.S.A.

TRADEMARKS

IMM®, MDA®, Model Driven Architecture®, UML®, UML Cube logo®, OMG Logo®, CORBA® and XMI® are registered trademarks of the Object Management Group, Inc., and Object Management Group™, OMG™ , Unified Modeling Language™, Model Driven Architecture Logo™, Model Driven Architecture Diagram™, CORBA logos™, XMI Logo™, CWM™, CWM Logo™, IIOP™ , MOF™ , OMG Interface Definition Language (IDL)™ , and OMG SysML™ are trademarks of the Object Management Group. All other products or company names mentioned are used for identification purposes only, and may be trademarks of their respective owners.

COMPLIANCE

The copyright holders listed above acknowledge that the Object Management Group (acting itself or through its designees) is and shall at all times be the sole entity that may authorize developers, suppliers and sellers of computer software to use certification marks, trademarks or other special designations to indicate compliance with these materials.

Software developed under the terms of this license may claim compliance or conformance with this specification if and only if the software compliance is of a nature fully matching the applicable compliance points as stated in the specification. Software developed only partially matching the applicable compliance points may claim only that the software was based on this specification, but may not claim compliance or conformance with this specification. In the event that testing suites are implemented or approved by Object Management Group, Inc., software developed using this specification may claim compliance or conformance with the specification only if the software satisfactorily completes the testing suites.

OMG’s Issue Reporting Procedure

All OMG specifications are subject to continuous review and improvement. As part of this process we encourage readers to report any ambiguities, inconsistencies, or inaccuracies they may find by completing the Issue Reporting Form listed on the main web page http://www.omg.org, under Documents, Report a Bug/Issue (http://www.omg.org/report_issue.)

Document Submitter

VINX Corp.

Document Publishing Supporters

OPOS-J

Sorimachi Giken Co. Ltd.

Microsoft Japan Ltd.

SEIKO EPSON Corp.

Toshiba TEC Corp.

Star Micronics Corp.

Fujitsu Frontec Corp.

NCR Corporation

Sharp Corporation

Omron Social Solutions Corp.

NEC Platforms Corp.

Transaction Media Networks Inc.

Preface	13

UPOS 1.16 RCSD Specification Overview	16

Updated Items in Release 1.16	16

Updated Items in CHAPTER 21 Lights	16

Updated Items in CHAPTER 29 POS Power	16

Added Chapters in Release 1.16	16

Chapter 21	17

Lights	17

Summary	17

General Information	20

Capabilities	20

Device Sharing	20

Lights Class Diagram	21

Lights Sequence Diagram	22

Properties (UML attributes)	24

CapAlarm Property	24

CapBlink Property	24

CapColor Property	24

CapPattern Property                         Added in Release 1.16	25

MaxLights Property	25

Methods (UML operations)	26

switchOff Method	26

switchOffPattern Method	26

switchOn Method                                  Updated in Release 1.16	27

switchOnMultiple Method          	  Added in Release 1.16	28

switchOnPattern Method           	  Added in Release 1.16	29

Events (UML interfaces)	30

DirectIOEvent	30

StatusUpdateEvent	30

Chapter 29	31

POS Power	31

Summary	31

General Information	35

Capabilities	35

Device Sharing	35

Model	36

POSPower Class Diagram	　　　　Updated in Release 1.16	37

POSPower Sequence Diagram	38

POSPower Standby Sequence Diagram	39

POSPower State Diagram	40

POSPower PowerState Diagram - Part 1	41

POSPower PowerState Diagram - Part 2	42

POSPower PowerState Diagram - Part 3	43

POSPower State Chart Diagram for Fan and Temperature	44

POSPower Battery State Diagram	45

POSPower Power Transitions State Diagram	46

Properties (UML attributes)	47

BatteryCapacityRemaining Property	47

BatteryCapacityRemainingInSeconds Property	Added in Release 1.16	47

BatteryCriticallyLowThreshold Property	47

BatteryCriticallyLowThresholdInSeconds Property 	         Added in Release 1.16	48

BatteryLowThreshold Property	48

BatteryLowThresholdInSeconds Property	 Added in Release 1.16	48

CapBatteryCapacityRemaining Property	49

CapBatteryCapacityRemainingInSeconds Property 	        Added in Release 1.16	49

CapChargeTime Property 	      Added in Release 1.16	49

CapFanAlarm Property	49

CapHeatAlarm Property	50

CapQuickCharge Property	50

CapRestartPOS Property	50

CapShutdownPOS Property	50

CapStandbyPOS Property	51

CapSuspendPOS Property	51

CapUPSChargeState Property	51

CapVariableBatteryCriticallyLowThreshold Property	52

CapVariableBatteryCriticallyLowThresholdInSeconds Property   	 　　　　　　　　　　　　　　　Added in Release 1.16	52

CapVariableBatteryLowThreshold Property	52

CapVariableBatteryLowThresholdInSeconds Property 	     　Added in Release 1.16	52

ChargeTime Property 	  　　Added in Release 1.16	53

EnforcedShutdownDelayTime Property	53

PowerFailDelayTime Property	53

PowerSource Property	54

QuickChargeMode Property	54

QuickChargeTime Property	54

UPSChargeState Property	55

Methods (UML operations)	56

restartPOS Method	56

shutdownPOS Method	56

standbyPOS Method	57

suspendPOS Method	57

Events (UML Interfaces)	58

DirectIOEvent	58

StatusUpdateEvent	58

Chapter 39	61

Video Capture	61

Summary	61

General Information	66

Capabilities	66

Video Capture Class Diagram	67

Model	68

Modes	68

Device behaviors	68

Photo Mode	68

Video Mode	69

Device Sharing	70

Properties (UML attributes)	71

AutoExposure Property	71

AutoFocus Property	71

AutoGain Property	71

AutoWhiteBalance Property	72

Brightness property	72

CapAssociatedHardTotalsDevice Property	72

CapAutoExposure Property	72

CapAutoFocus Property	73

CapAutoGain Property	73

CapAutoWhiteBalance Property	73

CapBrightness Property	73

CapContrast Property	73

CapExposure Property	74

CapGain Property	74

CapHorizontalFlip Property	74

CapHue Property	74

CapPhoto Property	74

CapPhotoColorSpace Property	75

CapPhotoFrameRate Property	75

CapPhotoResolution Property	75

CapPhotoType Property	75

CapSaturation Property	75

CapStorageTarget Property	76

CapVerticalFlip Property	76

CapVideo Property	76

CapVideoColorSpace Property	77

CapVideoFrameRate Property	77

CapVideoResolution Property	77

CapVideoType Property	77

Contrast Property	78

Exposure Property	78

Gain Property	78

HorizontalFlip Property	79

Hue Property	79

PhotoColorSpace Property	79

PhotoColorSpaceList Property	80

PhotoFrameRate Property	80

PhotoMaxFrameRate Property	80

PhotoResolution Property	81

PhotoResolutionList Property	81

PhotoType Property	81

PhotoTypeList Property	82

RemainingRecordingTimeInSec Property	82

Saturation Property	82

StorageTarget Property	83

VerticalFlip Property	83

VideoCaptureMode Property	84

VideoColorSpace Property	85

VideoColorSpaceList Property	85

VideoFrameRate Property	86

VideoMaxFrameRate Property	86

VideoResolution Property	86

VideoResolutionList Property	87

VideoType Property	87

VideoTypeList Property	87

Note: Video Capture Device Property Value Relationship	88

Methods (UML operations)	89

startVideo Method	89

stopVideo Method	90

takePhoto Method	90

Events (UML interfaces)	91

DirectIOEvent	91

ErrorEvent	92

StatusUpdateEvent	93

Chapter 40	94

Individual Recognition	94

Summary	94

General Information	97

Capabilities	97

Individual Recognition Class Diagram	97

Model	98

Input Model	98

Device Sharing	98

IndividualRecognitionFilter	99

IndividualRecognitionFilter Property Example Format	100

	Basic Items	100

	Face Recognition device example	101

IndividualRecognition Information Property Example Format	103

	Basic Items	103

	Face Recognition Device Example	104

Properties (UML attributes)	105

CapIndividualList Property	105

IndividualIDs Property	105

IndividualRecognitionFilter Property	105

IndividualRecognitionInformation Property	106

Events (UML interfaces)	107

DataEvent	107

DirectIOEvent	107

ErrorEvent	108

StatusUpdateEvent	109

Chapter 41	110

Sound Recorder	110

Summary	110

General Information	114

Capabilities	114

Sound Recorder Class Diagram	114

Model	115

Device Sharing	116

Properties(UML attributes)	117

CapAssociatedHardTotalsDevice Property	117

CapChannel Property	117

CapSamplingRate Property	117

CapSoundType Property	117

CapStorageTarget Property	118

CapRecordingLevel Property	118

Channel Property	118

ChannelList Property	119

RecordingLevel Property	119

RemainingRecordingTimeInSec Property	119

SamplingRate Property	119

SamplingRateList Property	120

SoundData Property	120

SoundType Property	120

SoundTypeList Property	120

StorageTarget Property	121

Methods(UML operations)	122

startRecording Method	122

stopRecording Method	123

Events(UML interfaces)	124

DataEvent	124

DirectIOEvent	124

ErrorEvent	125

StatusUpdateEvent	126

Chapter 42	127

Voice Recognition	127

Summary	127

General Information	131

Capabilities	131

Voice Recognition Class Diagram	131

Model	132

Device Sharing	133

Properties (UML attributes)	134

CapLanguage Property	134

HearingDataPattern Property	134

HearingDataWord Property	134

HearingDataWordList Property	135

HearingResult Property	136

HearingStatus Property	137

LanguageList Property	137

Methods (UML operations)	138

startHearingFree Method	138

startHearingSentence Method	139

startHearingWord Method	140

startHearingYesNo Method	141

stopHearing Method	141

Events (UML interfaces)	142

DataEvent	142

DirectIOEvent	142

ErrorEvent	143

StatusUpdateEvent	144

Chapter 43	145

Sound Player	145

Summary	145

General Information	148

Capabilities	148

Sound Player Class Diagram	148

Model	149

Device Sharing	150

Properties(UML attributes)	151

CapAssociatedHardTotalsDevice Property	151

CapMultiPlay Property	151

CapSoundTypeList Property	151

CapStorageTarget Property	151

CapVolume Property	152

DeviceSoundList Property	153

OutputIDList Property	153

StorageTarget Property	153

Volume Property	153

Methods (UML operations)	154

playSound Method	154

stopSound Method	154

Events (UML interfaces)	155

DirectIOEvent	155

ErrorEvent	156

OutputCompleteEvent	158

StatusUpdateEvent	158

Chapter 44	159

Speech Synthesis	159

Summary	159

General Information	163

Capabilities	163

Speech Synthesis Class Diagram	163

Model	164

Device Sharing	165

Properties (UML attributes)	166

CapLanguage Property	166

CapPitch Property	166

CapSpeed Property	166

CapVoice Property	166

CapVolume Property	167

Language Property	167

LanguageList Property	167

OutputIDList Property	167

Pitch Property	168

Speed Property	168

Voice Property	168

VoiceList Property	169

Volume Property	169

Methods (UML operations)	170

speak Method	170

speakImmediate Method	171

stopCurrentSpeaking Method	172

stopSpeaking Method	172

Events (UML interfaces)	173

DirectIOEvent	173

ErrorEvent	174

OutputCompleteEvent	176

StatusUpdateEvent	176

Chapter 45	177

Gesture Control	177

Summary	177

General Information	181

Capabilities	181

Gesture Control Class Diagram	181

Model	182

Automatic control	182

Pose / Motion	183

Device Sharing	183

Properties (UML attributes)	184

AutoMode Property	184

AutoModeList Property	184

CapAssociatedHardTotalsDevice Property	184

CapMotion Property	185

CapMotionCreation Property	185

CapPose Property	185

CapPoseCreation Property	185

CapStorageTarget Property	186

MotionList Property	186

PoseCreationMode Property	186

PoseList Property	187

StorageTarget Property	187

JointList Property	188

Table of Gesture Control Device Listed Items in Property	188

Methods (UML operations)	189

createMotion Method	189

createPose Method	189

getPosition Method	190

setPosition Method	191

setSpeed Method	192

startMotion Method	193

startPose Method	193

stopControl Method	194

Events (UML interfaces)	195

DirectIOEvent	195

ErrorEvent	196

OutputCompleteEvent	197

StatusUpdateEvent	198

Chapter 46	199

Device Monitor	199

Summary	199

General Information	202

Capabilities	202

Device Monitor Class Diagram	202

Model	203

Device Sharing	203

Properties (UML attributes)	204

DeviceData Property	204

DeviceList Property	205

MonitoringDeviceList Property	206

Methods (UML operations)	207

addMonitoringDevice Method	207

deleteMonitoringDevice Method	209

clearMonitoringDevices Method	209

getDeviceValue method	209

Events (UML interfaces)	210

DataEvent	210

DirectIOEvent	210

ErrorEvent	211

StatusUpdateEvent	213

Chapter 47	214

Graphic Display	214

Summary	214

General Information	218

Capabilities	218

Graphics Display Class Diagram	218

Model	219

Image Display Mode	219

Video Display Mode	219

Web Display Mode	220

Device Sharing	221

Properties (UML attributes)	222

Brightness Property	222

CapAssociatedHardTotalsDevice Property	222

CapBrightness Property	222

CapImageType Property	222

CapStorageTarget Property	223

CapURLBack Property	223

CapURLForward Property	223

CapVideoType Property	224

CapVolume Property	224

DisplayMode Property	225

ImageType Property	226

ImageTypeList Property	226

LoadStatus Property	226

StorageTarget Property	227

URL Property	227

VideoType Property	228

VideoTypeList Property	228

Volume Property	228

Methods (UML operations)	229

cancelURLLoading Method	229

goURLBack Method	229

goURLForward Method	229

loadImage Method	230

loadURL Method	230

playVideo Method	231

stopVideo Method	231

updateURLPage Method	232

Events (UML interfaces)	233

DirectIOEvent	233

ErrorEvent	233

OutputCompleteEvent	235

StatusUpdateEvent	235

Appendix K	236

Relationship to other OMG  specification and activities	236

Robotics Domain Task Force	236

Activities in Robotics Domain Task Force	236

RoIS Specification	236

Scope of RoIS specification	236

Robot Service Ontology [RoSO] RFP	237

Interoperability between UPOS RCSD and RoIs	238

Rleationsihp between UPOS RCSD and RoIS	238

Document History	240

Version History	240

Glossary	241

Preface

OMG

Founded in 1989, the Object Management Group, Inc. (OMG) is an open membership, not-for-profit computer industry standards consortium that produces and maintains computer industry specifications for interoperable, portable, and reusable enterprise applications in distributed, heterogeneous environments. Membership includes Information Technology vendors, end users, government agencies, and academia.

OMG member companies write, adopt, and maintain its specifications following a mature, open process. OMG’s specifications implement the Model Driven Architecture® (MDA®), maximizing ROI through a full-lifecycle approach to enterprise integration that covers multiple operating systems, programming languages, middleware and networking infrastructures, and software development environments. OMG’s specifications include: UML® (Unified Modeling Language™); CORBA® (Common Object Request Broker Architecture); CWM™ (Common Warehouse Metamodel); and industry-specific standards for dozens of vertical markets.　More information on the OMG is available at http://www.omg.org/.

OMG Specifications

As noted, OMG specifications address middleware, modeling and vertical domain frameworks. All OMG Specifications are available from the OMG website at:

http://www.omg.org/spec

Specifications are organized by the following categories:

Business Modeling Specifications

Middleware Specifications

- CORBA/IIOP

- Data Distribution Services

- Specialized CORBA

IDL/Language Mapping Specifications

Modeling and Metadata Specifications

- UML, MOF, CWM, XMI

- UML Profile

Modernization Specifications

Platform Independent Model (PIM), Platform Specific Model (PSM), Interface Specifications

- CORBAServices

- CORBAFacilities

OMG Domain Specifications

CORBA Embedded Intelligence Specifications

CORBA Security Specifications

All of OMG’s formal specifications may be downloaded without charge from our website. (Products implementing OMG specifications are available from individual suppliers.) Copies of specifications, available in PostScript and PDF format, may be obtained from the Specifications Catalog cited above or by contacting the Object Management Group, Inc. at:

OMG Headquarters
109 Highland Avenue
Needham, MA 02494
USA
Tel: +1-781-444-0404
Fax: +1-781-444-0320
Email: pubs@omg.org

Certain OMG specifications are also available as ISO standards. Please consult http://www.iso.org

Typographical Conventions

The type styles shown below are used in this document to distinguish programming statements from ordinary English. However, these conventions are not used in tables or section headings where no distinction is necessary.

Times/Times New Roman - 10 pt.:  Standard body text

NOTE:   Terms that appear in italics are defined in the glossary. Italic text also represents the name of a document, specification, or other publication.

Issues

The reader is encouraged to report any technical or editing issues/problems with this specification to http://www.omg.org/report_issue.htm.

UPOS 1.16 RCSD Specification Overview

Updated Items in Release 1.16

Chapter sections 23 and 38 from UPOS1.15 are included with annotations denoting the changes necessary for supporting the addition of the Retail Communications Service Devices.  Chapters 39-47 are new chapters for devices being added to UPOS v1.  The following is a list of the properties, methods and chapters.

Updated Items in CHAPTER 21 Lights

Properties

CapPattern Property

Methods

switchOn  Method

switchOnMultiple Method

switchOnPattern Method

switchOffPattern Method

Updated Items in CHAPTER 29 POS Power

Properties

CapBatteryCapacityRemainingInSeconds Property

CapVariableBatteryCriticallyLowThresholdInSeconds Property

CapVariableBatteryLowThresholdInSeconds Property

CapChargeTime Property

BatteryCapacityRemainingInSeconds Property

BatteryCriticallyLowThresholdInSeconds Property

BatteryLowThresholdInSeconds Property

ChargeTime Property

Added Chapters in Release 1.16

CHAPTER 39 Video Capture

CHAPTER 40 Individual Recognition

CHAPTER 41 Sound Recorder

CHAPTER 42 Voice Recognition

CHAPTER 43 Sound Player

CHAPTER 44 Speech Synthesis

CHAPTER 45 Gesture Control

CHAPTER 46 Device Monitor

CHAPTER 47 Graphic Display

Chapter 21

Lights

This Chapter defines the Lights device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.12 | Not Supported |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.12 | open |
| CapPowerReporting: | int32 | {read-only} | 1.12 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.12 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.12 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.12 | open |
| CheckHealthText: | string | {read-only} | 1.12 | open |
| Claimed: | boolean | {read-only} | 1.12 | open |
| DataCount: | int32 | {read-only} | 1.12 | Not Supported |
| DataEventEnabled: | boolean | {read-write} | 1.12 | Not Supported |
| DeviceEnabled: | boolean | {read-write} | 1.12 | open & claim |
| FreezeEvents: | boolean | {read-write} | 1.12 | open |
| OutputID: | int32 | {read-only} | 1.12 | Not Supported |
| PowerNotify: | int32 | {read-write} | 1.12 | open |
| PowerState: | int32 | {read-only} | 1.12 | open |
| State: | int32 | {read-only} | 1.12 | -- |
| DeviceControlDescription: | string | {read-only} | 1.12 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.12 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.12 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.12 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.12 | open |
| PhysicalDeviceName: | string | {read-only} | 1.12 | open |

| Properties (Continued) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specific | Type | Mutability | Version | May Use After |
| CapAlarm: | int32 | {read-only} | 1.12 | open |
| CapBlink: | boolean | {read-only} | 1.12 | open |
| CapColor: | int32 | {read-only} | 1.12 | open |
| CapPattern: | int32 | {read-only} | 1.16 | open |
| MaxLights: | int32 | {read-only} | 1.12 | open |

| Methods (UML operations) |  |
| --- | --- |
| Common |  |
| Name | Version |
| open (logicalDeviceName: string): void {raises-exception} | 1.12 |
| close ( ): void {raises-exception, use after open} | 1.12 |
| claim (timeout: int32): void {raises-exception, use after open} | 1.12 |
| release ( ): void {raises-exception, use after open, claim} | 1.12 |
| checkHealth (level: int32): void {raises-exception, use after open, enable} | 1.12 |
| clearInput ( ): void { } | Not supported |
| clearInputProperties ( ): void { } | Not supported |
| clearOutput ( ): void { } | Not supported |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} | 1.12 |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, enable} | 1.12 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.12 |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.12 |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, enable} | 1.12 |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.12 |
| Specific |  |
| Name |  |
| switchOff (lightNumber: int32): void {raises-exception, use after open, claim, enable} | 1.12 |
| switchOn (lightNumber: int32, blinkOnCycle: int32, blinkOffCycle: int32, color: int32, alarm: int32): void {raises-exception, use after open, claim, enable} | 1.12 |
| switchOnMultiple (lightNumbers: string, blinkOnCycle: int32, blinkOffCycle: int32, color: int32, alarm: int32): void {raises-exception, use after open, claim, enable} | 1.16 |
| switchOnPattern (pattern: int32, alarm: int32): void {raises-exception, use after open, claim, enable} | 1.16 |
| switchOffPattern ( ): void {raises-exception, use after open, claim, enable} | 1.16 |

| Events (UML interfaces) |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Mutability | Version |
| upos::events::DataEvent |  | Not Supported |  |
| upos::events::DirectIOEvent |  |  | 1.12 |
| EventNumber: | int32 | {read-only} |  |
| Data: | int32 | {read-write} |  |
| Obj: | object | {read-write} |  |
| upos::events::ErrorEvent |  | Not Supported |  |
| upos::events::OutputCompleteEvent |  | Not Supported |  |
| upos::events::StatusUpdateEvent |  |  | 1.12 |
| Status: | int32 | {read-only} |  |

General Information

The Lights programmatic name is “Lights”.

This device category was added to Version 1.12 of the specification.

Capabilities

- The Lights device control has the following capability:

- Supports commands to “switch on” and “switch off” a light.

- The Lights device control may have the following additional capabilities:

  - Supports device-level blinking at adjustable blink cycles.

  - Support multiple lights.

  - Supports different colors of a light.

  - Supports different alarms

Device Sharing

Lights is an exclusive-use device. Its device sharing rules are:

- The application must claim the device before enabling it.

- The application must claim and enable the device before accessing some of the properties and methods, or receiving events.

- See the “Summary” table for precise usage prerequisites.

Lights Class Diagram

Updated in Release 1.16

The following diagram shows the relationships between the Lights classes

Fig. Chap. 21-1 Lights Class Diagram

Lights Sequence Diagram

The following sequence diagram show the typical usage of the Lights device illustrating the handling of the media entry indicator lights.

Fig. Chap. 21-2 Lights Sequence Diagram (handling of the media entry indicator lights)

The following sequence diagram show the typical usage of the Lights device illustrating the handling of the pole lights.

Fig. Chap. 21-3 Lights Sequence Diagram (handling of the pole lights)

Properties (UML attributes)

CapAlarm Property

Syntax	CapAlarm: int32 {read-only, access after open}

Remarks	This capability indicates if the device supports different alarms.

CapAlarm is a logical OR combination of any of the following values:

Value	Meaning

LGT_ALARM_NOALARM	Alarms are not supported.

LGT_ALARM_SLOW		Supports a slow beep.

LGT_ALARM_MEDIUM	Supports a medium beep.

LGT_ALARM_FAST		Supports a fast beep.

LGT_ALARM_CUSTOM1	Supports 1st custom alarm.

LGT_ALARM_CUSTOM2	Supports 2nd custom alarm.

This property is initialized by the open method. If the device does not support alarms, it is initialized to LGT_ALARM_NOALARM.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

CapBlink Property

Syntax	CapBlink: boolean {read-only, access after open}

Remarks	If true, a blinking capability is supported. It may be either a physical capability of the device or emulated by the service.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

CapColor Property

Syntax	CapColor: int32 {read-only, access after open}

Remarks	This capability indicates if the device supports different colors.

CapColor is a logical OR combination of any of the following values:

Value	Meaning

LGT_COLOR_PRIMARY	Supports Primary Color (Usually Green).

LGT_COLOR_CUSTOM1	Supports 1st Custom Color (Usually Red).

LGT_COLOR_CUSTOM2	Supports 2nd Custom Color (Usually Yellow).

LGT_COLOR_CUSTOM3	Supports 3rd Custom Color.

LGT_COLOR_CUSTOM4	Supports 4th Custom Color.

LGT_COLOR_CUSTOM5	Supports 5th Custom Color.

This property is initialized by the open method. If the device supports only one color, it is initialized to LGT_COLOR_PRIMARY.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

CapPattern Property                         Added in Release 1.16

Syntax	CapPattern: int32 {read-only, access after open}

Remarks	This capability indicates if the device supports different lighting patterns.

CapPattern is a logical OR combination of any of the following values:

Value		Meaning

LGT_PATTERN_NOPATTERN								Lighting patterns are not supported.

LGT_PATTERN_CUSTOM			                 					1~32 Supports 1st to 32th Lighting Pattern.

This property is initialized by the open method. If the device does not support lighting pattern, it is initialized to LGT_PATTERN_NOPATTERN.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	switchOnPattern Method.

MaxLights Property

Syntax	MaxLights: int32 {read-only, access after open}

Remarks	MaxLights specifies the maximum number of lights that the device can support.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

Methods (UML operations)

switchOff Method

Syntax	switchOff (lightNumber: int32): 
          void {raises-exception, use after open-claim-enable}

Parameter	Description

lightNumber	Specifies the light number. Valid light numbers are 1 through MaxLights.

Remarks	Switches off the light specified by lightNumber.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

A possible value of the exception’s ErrorCode property is:

Value	Meaning

E_ILLEGAL	The lightNumber parameter exceeds MaxLights.

See Also	MaxLights Property.

switchOffPattern Method

Syntax	switchOff Pattern ( ): 
          void {raises-exception, use after open-claim-enable}

Remarks	Switches off the pattern lighting.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

A possible value of the exception’s ErrorCode property is:

Value	Meaning

E_ILLEGAL	Pattern lighting is not executed.

See Also	switchOnPattern Method.

switchOn Method                                  Updated in Release 1.16

Syntax	switchOn (lightNumber: int32, blinkOnCycle: int32,
      blinkOffCycle: int32, color: int32, alarm: int32): 
          void {raises-exception, use after open-claim-enable}

Parameter	Description

lightNumber	Specifies the light number. Valid light numbers are 1 through MaxLights.

blinkOnCycle	A zero (0) value indicates no blink cycle. A positive value indicates the blink on cycle time in milliseconds. Negative values are not allowed.

blinkOffCycle	A zero (0) value indicates no blink cycle. A positive value indicates the blink off cycle time in milliseconds. Negative values are not allowed.

color	Specifies the color of the light, must be one of the colors defined by CapColor.

alarm	Specifies the used alarm type, must be one of the alarms defined by CapAlarm.

Remarks	Switches on the light specified by lightNumber or let it blink.

If blinkOnCycle and blinkOffCycle are zero (0) or CapBlink is false, then the parameters blinkOnCycle and blinkOffCycle will be ignored and the light will only be switched on.

If CapBlink is true and blinkOnCycle and blinkOffCycle are positive, then the light will blink.

If CapColor is LGT_COLOR_PRIMARY the light does not support different colors and color is ignored, otherwise switchOn will use the color specified by color.

If CapAlarm is LGT_ALARM_NOALARM the light does not support different alarms and alarm is ignored, otherwise switchOn will use the alarm specified by alarm.

Subsequent calls to switchOn will change the blink cycles, the color or the alarm type of the light.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

A possible value of the exception’s ErrorCode property is:

Value	Meaning

E_ILLEGAL	The lightNumber parameter exceeds MaxLights, an invalid color or alarm was specified.

See Also	CapAlarm Property, CapBlink Property, CapColor Property, MaxLights Property.

switchOnMultiple Method          			Added in Release 1.16

Syntax	switchOnMultiple (lightNumbers: string, blinkOnCycle: int32,
                   blinkOffCycle: int32, color: int32, alarm: int32): 
          void {raises-exception, use after open-claim-enable}

Parameter	Description

lightNumbers	Specifies the comma-delimited list of light number. Valid light numbers are 1 through MaxLights.

blinkOnCycle	A zero (0) value indicates no blink cycle. A positive value indicates the blink on cycle time in milliseconds. Negative values are not allowed.

blinkOffCycle	A zero (0) value indicates no blink cycle. A positive value indicates the blink off cycle time in milliseconds. Negative values are not allowed.

color	Specifies the color of the light, must be one of the colors defined by CapColor.

alarm	Specifies the used alarm type, must be one of the alarms defined by CapAlarm.

Remarks	This method does the same as swicthOn but in a synchronized way such that all lights are switched on / blinking synchronously. Switches on the multiple lights specified by lightNumbers or let it blink.

If blinkOnCycle and blinkOffCycle are zero (0) or CapBlink is false, then the parameters blinkOnCycle and blinkOffCycle will be ignored and the light will only be switched on.

If CapBlink is true and blinkOnCycle and blinkOffCycle are positive, then the light will blink.

If CapColor is LGT_COLOR_PRIMARY the light does not support different colors and color is ignored, otherwise switchOnMultiple will use the color specified by color.

If CapAlarm is LGT_ALARM_NOALARM the light does not support different alarms and alarm is ignored, otherwise switchOnMultiple will use the alarm specified by alarm.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

A possible value of the exception’s ErrorCode property is:

Value	Meaning

E_ILLEGAL	The lightNumbers parameter exceeds MaxLights, an invalid value was specified.

See Also	CapAlarm Property, CapBlink Property, CapColor Property, MaxLights Property.

switchOnPattern Method           			Added in Release 1.16

Syntax	switchOnPattern (pattern: int32, alarm: int32): 
          void {raises-exception, use after open-claim-enable}

Parameter	Description

pattern	Specifies the lighting pattern, must be one of the patterns defined by CapPattern.

alarm	Specifies the used alarm type, must be one of the alarms defined by CapAlarm.

Remarks	Switches on the light specified by pattern.

If CapAlarm is LGT_ALARM_NOALARM the light does not support different alarms and alarm is ignored, otherwise switchOn and swithOnPattern will use the alarm specified by alarm.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

A possible value of the exception’s ErrorCode property is:

Value	Meaning

E_ILLEGAL	An invalid value was specified, or unsupported operation with the Device.

See Also	CapAlarm Property, CapPattern Property.

Events (UML interfaces)

DirectIOEvent

<< event >>	upos::events::DirectIOEvent
EventNumber	: int32 {read-only}
Data			: int32 {read-write}
Obj			: object{read-write}

Description 	Provides Service information directly to the application. This event provides a means for a vendor-specific Lights Service to provide events to the application that are not otherwise supported by the device control.

Attributes	This event contains the following attributes:

Attribute	Type	Description

EventNumber	int32	Event number whose specific values are assigned by the Service.

Data	int32	Additional numeric data. Specific values vary by the EventNumber and the Service. This property is settable.

Obj	Object	Additional data whose usage varies by the EventNumber and Service. This property is settable.

Remarks	This event is to be used only for those types of vendor specific functions that are not otherwise described. Use of this event may restrict the application program from being used with other vendor’s Lights devices which may not have any knowledge of the Service’s need for this event.

See Also	“Events" on page Intro-19, directIO Method.

StatusUpdateEvent

<< event >>	upos::events::StatusUpdateEvent
Status		: int32 {read-only}

Description 	Notifies the application that there is a change in the power status of a light.

Attributes	This event contains the following attribute:

Attribute	Type	Description

Status	int32	Reports a change in the power status of a light.

Note that Release 1.3 added Power State Reporting with additional Power reporting StatusUpdateEvent values.

The Update Firmware capability, added in Release 1.9, added additional Status values for communicating the status/progress of an asynchronous update firmware process.

See “StatusUpdateEvent” description on page 1-34.

Remarks	Enqueued when the light detects a power state change.

See Also	“Events" on page Intro-19.

Chapter 29

POS Power

This Chapter defines the POS Power device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.5 | Not Supported |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.9 | open |
| CapPowerReporting: | int32 | {read-only} | 1.3 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.8 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.9 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.8 | open |
| CheckHealthText: | string | {read-only} | 1.5 | open |
| Claimed: | boolean | {read-only | 1.5 | open |
| DataCount: | int32 | {read-only} | 1.5 | Not Supported |
| DataEventEnabled: | boolean | {read-write} | 1.5 | Not Supported |
| DeviceEnabled: | boolean | {read-write} | 1.5 | open & claim |
| FreezeEvents: | boolean | {read-write} | 1.5 | open |
| OutputID: | int32 | {read-only} | 1.5 | Not Supported |
| PowerNotify: | int32 | {read-write} | 1.5 | open |
| PowerState: | int32 | {read-only} | 1.5 | open |
| State: | int32 | {read-only} | 1.5 | -- |
| DeviceControlDescription: | string | {read-only} | 1.5 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.5 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.5 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.5 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.5 | open |
| PhysicalDeviceName: | string | {read-only} | 1.5 | open |

| Properties (Continued) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specific | Type | Mutability | Version | May Use After |
| BatteryCapacityRemaining: | int32 | {read-only} | 1.9 | open |
| BatteryCapacityRemainingInSeconds: | int32 | {read-only} | 1.16 | open |
| BatteryCriticallyLowThreshold: | int32 | {read-write} | 1.9 | open |
| BatteryCriticallyLowThresholdInSeconds: | int32 | {read-write} | 1.16 | open |
| BatteryLowThreshold: | int32 | {read-write} | 1.9 | open |
| BatteryLowThresholdInSeconds: | int32 | {read-write} | 1.16 | open |
| CapBatteryCapacityRemaining: | boolean | {read-only} | 1.9 | open |
| CapBatteryCapacityRemainingInSeconds: | boolean | {read-only} | 1.16 | open |
| CapChargeTime: | boolean | {read-only} | 1.16 | open |
| CapFanAlarm: | boolean | {read-only} | 1.5 | open |
| CapHeatAlarm: | boolean | {read-only} | 1.5 | open |
| CapQuickCharge: | boolean | {read-only} | 1.5 | open |
| CapRestartPOS: | boolean | {read-only} | 1.9 | open |
| CapShutdownPOS: | boolean | {read-only} | 1.5 | open |
| CapStandbyPOS: | boolean | {read-only} | 1.9 | open |
| CapSuspendPOS: | boolean | {read-only} | 1.9 | open |
| CapUPSChargeState: | int32 | {read-only} | 1.5 | open |
| CapVariableBatteryCriticallyLowThreshold: | boolean | {read-only} | 1.9 | open |
| CapVariableBatteryCriticallyLowThresholdInSeconds: | boolean | {read-only} | 1.16 | open |
| CapVariableBatteryLowThreshold: | boolean | {read-only} | 1.9 | open |
| CapVariableBatteryLowThresholdInSeconds: | boolean | {read-only} | 1.16 | open |
| ChargeTime: | int32 | {read-only} | 1.16 | open |
| EnforcedShutdownDelayTime: | int32 | {read-write} | 1.5 | open |
| PowerFailDelayTime: | int32 | {read-only} | 1.5 | open |
| PowerSource: | int32 | {read-only} | 1.9 | open |
| QuickChargeMode: | boolean | {read-only} | 1.5 | open |
| QuickChargeTime: | int32 | {read-only} | 1.5 | open |
| UPSChargeState: | int32 | {read-only} | 1.5 | open, claim & enable |

| Methods (UML operations) |  |
| --- | --- |
| Common |  |
| Name | Version |
| open (logicalDeviceName: string): void {raises-exception} | 1.5 |
| close ( ): void {raises-exception, use after open} | 1.5 |
| claim (timeout: int32 ): void {raises-exception, use after open} | 1.5 |
| release ( ): void {raises-exception, use after open, claim} | 1.5 |
| checkHealth (level: int32): void {raises-exception, use after open, enable} | 1.5 |
| clearInput ( ): void { } | Not supported |
| clearInputProperties ( ): void { } | Not supported |
| clearOutput ( ): void { } | Not supported |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} | 1.5 |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, claim, enable} | 1.9 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | 1.8 |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | 1.8 |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, claim, enable} | 1.9 |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | 1.8 |
| Specific |  |
| Name |  |
| restartPOS ( ): void {raises-exception, use after open, enable} | 1.9 |
| shutdownPOS ( ): void {raises-exception, use after open, enable} | 1.5 |
| standbyPOS (reason: int32 ): void {raises-exception, use after open, enable} | 1.9 |
| suspendPOS (reason: int32 ): void {raises-exception, use after open, enable} | 1.9 |

| Events (UML interfaces) |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Mutability | Version |
| upos::events::DataEvent |  | Not Supported |  |
| upos::events::DirectIOEvent |  |  | 1.5 |
| EventNumber: | int32 | {read-only} |  |
| Data: | int32 | {read-write} |  |
| Obj: | object | {read-write} |  |
| upos::events::ErrorEvent |  | Not Supported |  |
| upos::events::OutputCompleteEvent |  | Not Supported |  |
| upos::events::StatusUpdateEvent |  |  | 1.5 |
| Status: | int32 | {read-only} |  |

General Information

The POS Power programmatic name is “POSPower”.

Capabilities

The POSPower device class has the following capabilities:

- Supports a command to “shut down” the system.

- Supports a command to restart the system.

- Supports a command to “suspend” the system.

- Supports a command to have the system go to standby.

- Supports accessing a power handling mechanism of the underlying operating system and hardware.

- Informs the application if a power fail situation has occurred.

- Informs the application about battery level.

- Informs the application if the UPS charge state has changed.

- Informs the application about high CPU temperature.

- Informs the application about stopped CPU fan.

- Informs the application if an operating system dependent enforced shutdown mechanism is processed.

- Allows the application after saving application data locally or transferring application data to a server to shut down the POS terminal.

- Informs the application about an initiated shutdown.

Device Sharing

The POSPower is a sharable device. Its device sharing rules are:

- After opening and enabling the device, the application may access all properties and methods and will receive status update events.

- If more than one application has opened and enabled the device, all applications may access its properties and methods. Status update events are fired to all of the applications.

- If one application claims the POSPower, then only that application may call the shutdownPOS, standbyPOS, or suspendPOS methods. This feature provides a degree of security, such that these methods may effectively be restricted to the main POS application if that application claims the device at startup.

- See the “Summary” table for precise usage prerequisites.

Model

The general model of POSPower is based on the power model of each device in version 1.3 or later. The same common properties are used but all states relate to the POS terminal itself and not to a peripheral device.

There are three states of the POSPower:

- ONLINE. The POS terminal is powered on and ready for use. This is the “operational” state.

- OFF. The POS terminal is powered off or detached from the power supplying net. The POS terminal runs on battery power support. This is the powerfail situation.

- OFFLINE. The POS terminal is powered on but is running in a “lower-power-consumption” mode. It may need to be placed online by pressing a button or key or something else which may wake up the system.

Power reporting only occurs while the device is open, enabled and power notification is switched on.

In a powerfail situation - that means the POSPower is in the state OFF - the POS terminal will be shut down automatically after the last application has closed the POSPower device or the time specified by the EnforcedShutdownDelayTime property has been elapsed.

A call to the shutdownPOS method will always shut down the POS terminal independent of the system power state.

Version 1.9 or later

Support of battery powered devices is added. In addition to adding properties to report battery levels and power sources, properties are added to allow for the setting of low and critically low battery levels. The POSPower device also includes the ability to request or respond to request to enter the standby and suspend states. The model does not attempt to duplicate other power management models such as APM and ACPI, but leaves those implementation details to the provider. As a rule, the suspend state will consume less power than the standby state, which in turn will consume less power than the on state. A suggested mapping of these states to other power management models is:

| State | ACPI | APM | Description |
| --- | --- | --- | --- |
| On | S0 | ON | Active, Powered On |
| Standby | S1 | SUSPEND | Displays and drives off, CPU, RAM and fans powered on |
| Suspend | S3 | SUSPEND | Only RAM powered |
| Off | S5 | OFF | Completely powered off |

POSPower Class Diagram	　　　　Updated in Release 1.16

The following diagram shows the relationships between the POSPower classes.

Fig. Chap.29-1 POSPower Class Diagram

POSPower Sequence Diagram

The following sequence diagram shows the typical usage of the POSPower device for registering for StatusUpdateEvents and an atypical case of initiating a shutdownPOS call.

Fig. Chap. 29-2 POSPower Sequence Diagram

POSPower Standby Sequence Diagram

Fig. Chap. 29-3 POSPower Standby Sequence Diagram

POSPower State Diagram

The following state diagram depicts the POSPower Control device model.

Fig. Chap. 29-4 Power State Diagram (POSPower Control Device Model)

POSPower PowerState Diagram - Part 1

The following state diagram depicts the POSPower Power States.

Fig. Chap. 29-5 POSPower PowerState Diagram (Part 1)

POSPower PowerState Diagram - Part 2

The following state diagram depicts the POSPower PowerState ONLINE.

Fig. Chap. 29-6 POSPower PowerState Diagram (Part 2)

POSPower PowerState Diagram - Part 3

The following state diagram depicts the POSPower PowerState OFF.

Fig. Chap. 29-7 POSPower PowerState Diagram (Part 3)

POSPower State Chart Diagram for Fan and Temperature

The following state diagram depicts the handling of fan and temperature alarms.

Fig. Chap. 29-8 POSPower State Chart Diagram (Fan and Temperature)

POSPower Battery State Diagram

Fig. Chap. 29-9 POSPower Battery State Diagram

POSPower Power Transitions State Diagram

Fig. Chap. 29-10 POSPower Power Transitions State Diagram

Properties (UML attributes)

BatteryCapacityRemaining Property

Syntax	BatteryCapacityRemaining: int32 {read-only, access after open}

Remarks	A value of 0 to 100 represents percent of battery capacity remaining.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	CapBatteryCapacityRemaining Property

BatteryCapacityRemainingInSeconds Property	Added in Release 1.16

Syntax	BatteryCapacityRemainingInSeconds: int32 {read-only, access after open}

Remarks	A value of battery capacity remaining in seconds.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	CapBatteryCapacityRemainingInSeconds Property

BatteryCriticallyLowThreshold Property

Syntax	BatteryCriticallyLowThreshold: int32 {read-write, access after open}

Remarks	If not zero, this property holds the threshold at which a PWR_SUE_BAT_CRITICAL StatusUpdateEvent is generated. The values 1 through 99 represent the percentage of the capacity remaining. The value 0 indicates that Battery Critically Low reporting is not supported or is disabled.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	CapVariableBatteryCriticallyLowThreshold Property, StatusUpdateEvent

BatteryCriticallyLowThresholdInSeconds Property 										Added in Release 1.16

Syntax	BatteryCriticallyLowThresholdInSeconds: int32 {read-write, access after open}

Remarks	If not zero, this property holds the threshold at which a PWR_SUE_BAT_CRITICAL StatusUpdateEvent is generated. The values of seconds of the capacity remaining. The value 0 indicates that Battery Critically Low reporting is not supported or is disabled.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	CapVariableBatteryCriticallyLowThresholdInSeconds Property, StatusUpdateEvent

BatteryLowThreshold Property

Syntax	BatteryLowThreshold: int32 {read-write, access after open}

Remarks	If not zero, this property holds the threshold at which a PWR_SUE_BAT_LOW StatusUpdateEvent is generated. The value 1 to 99 represents the percent capacity remaining. The value 0 indicates that battery low reporting is not supported or is disabled. If variable battery low threshold is supported, setting a value between 1 and 99 sets the threshold to that value. Setting a value of zero disables battery low reporting.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	CapVariableBatteryLowThreshold Property, StatusUpdateEvent

BatteryLowThresholdInSeconds Property		Added in Release 1.16

Syntax	BatteryLowThresholdInSeconds: int32 {read-write, access after open}

Remarks	If not zero, this property holds the threshold at which a PWR_SUE_BAT_LOW StatusUpdateEvent is generated. The value of seconds of the capacity remaining. The value 0 indicates that battery low reporting is not supported or is disabled. If variable battery low threshold is supported, setting a value of seconds sets the threshold to that value. Setting a value of zero disables battery low reporting.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	CapVariableBatteryLowThresholdInSeconds Property, StatusUpdateEvent

CapBatteryCapacityRemaining Property

Syntax	CapBatteryCapacityRemaining: boolean {read-only, access after open}

Remarks	If true, the device is able to provide battery capacity information. Otherwise it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	BatteryCapacityRemaining Property

CapBatteryCapacityRemainingInSeconds Property 									Added in Release 1.16

Syntax	CapBatteryCapacityRemainingInSeconds								: boolean {read-only, access after open}

Remarks	If true, the device is able to provide battery capacity information seconds. Otherwise it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	BatteryCapacityRemainingInSeconds Property

CapChargeTime Property 			    Added in Release 1.16

Syntax	CapChargeTime: boolean {read-only, access after open}

Remarks	If true, the device is able to acquire the remaining time until full charging. Otherwise it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	ChargeTime Property.

CapFanAlarm Property

Syntax	CapFanAlarm: boolean {read-only, access after open}

Remarks	If true, the device is able to detect whether the CPU fan is stopped. Otherwise it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

CapHeatAlarm Property

Syntax	CapHeatAlarm: boolean {read-only, access after open}

Remarks	If true the device is able to detect whether the CPU is running at too high of a temperature. Otherwise it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

CapQuickCharge Property

Syntax	CapQuickCharge: boolean {read-only, access after open}

Remarks	If true, the power management allows the charging of the UPS battery in quick mode. The time for charging the battery is shorter than usual. Otherwise it is false.

This property is initialized by the open method.

Errors		A UposException may be thrown when this property is accessed. For further   information, see “Errors" on page Intro-20.

See Also	QuickChargeMode Property, QuickChargeTime Property.

CapRestartPOS Property

Syntax	CapRestartPOS: boolean {read-only, access after open}

Remarks	If true the device is able to explicitly restart the POS. Otherwise it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	restartPOS Method.

CapShutdownPOS Property

Syntax	CapShutdownPOS: boolean {read-only, access after open}

Remarks	If true the device is able to explicitly shut down the POS. Otherwise it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	shutdownPOS Method.

CapStandbyPOS Property

Syntax	CapStandbyPOS: boolean {read-only, access after open}

Remarks	If true, the device is able to request that the POS System enter the Standby state. Otherwise it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	standbyPOS Method.

CapSuspendPOS Property

Syntax	CapSuspendPOS: boolean {read-only, access after open}

Remarks	If true, the device is able to request that the POS System enter the Suspend state. Otherwise it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	suspendPOS Method.

CapUPSChargeState Property

Syntax	CapUPSChargeState: int32 {read-only, access after open}

Remarks	If not equal to zero, the UPS can deliver one or more charge states. It can contain any of the following values logically ORed together.

Value	Meaning

PWR_UPS_FULL	UPS battery is near full charge.

PWR_UPS_WARNING	UPS battery is near 50% charge.

PWR_UPS_LOW	UPS battery is near empty. Application shutdown should be started to ensure that is can be completed before the battery charge is depleted. A minimum of 2 minutes of normal system operation can be assumed when this state is entered unless this is the first state reported upon entering the “Off” power state.

PWR_UPS_CRITICAL	UPS battery is in a critical state and could be disconnected at any time without further warning.	This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	UPSChargeState Property.

CapVariableBatteryCriticallyLowThreshold Property

Syntax	CapVariableBatteryCriticallyLowThreshold:
			　　boolean {read-only, access after open}

Remarks	If true, the device supports a variable threshold for critically low battery. Otherwise it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	BatteryCriticallyLowThreshold Property, StatusUpdateEvent

CapVariableBatteryCriticallyLowThresholdInSeconds Property   		　　　　　　　　　　　　　　　Added in Release 1.16

Syntax	CapVariableBatteryCriticallyLowThresholdInSeconds:
			　　boolean {read-only, access after open}

Remarks	If true, the device supports a second’s variable threshold for critically low battery. Otherwise it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	BatteryCriticallyLowThresholdInSeconds Property, StatusUpdateEvent

CapVariableBatteryLowThreshold Property

Syntax	CapVariableBatteryLowThreshold: boolean {read-only, access after open}

Remarks	If true, the device supports a variable threshold for battery low. Otherwise it is false. This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	BatteryLowThreshold Property, StatusUpdateEvent

CapVariableBatteryLowThresholdInSeconds Property
						　Added in Release 1.16

Syntax	CapVariableBatteryLowThresholdInSeconds:
			　　boolean {read-only, access after open}

Remarks	If true, the device supports a second’s variable threshold for battery low. Otherwise it is false. This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	BatteryLowThresholdInSeconds Property, StatusUpdateEvent

ChargeTime Property 			　　Added in Release 1.16

Syntax	ChargeTime: int32 {read-only, access after open}

Remarks	Indicates the time remaining until the battery is fully charged in seconds.

If equal to zero the battery is not charging or not supported.

This property is only set if CapChargeTime is true.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also 	CapChargeTime Property.

EnforcedShutdownDelayTime Property

Syntax	EnforcedShutdownDelayTime: int32 {read-write, access after open}

Remarks	If not equal to zero the system has a built-in mechanism to shut down the POS terminal after a determined time in a power fail situation. This property contains the time in milliseconds when the system will shut down automatically after a power failure. A power failure is the situation when the POS terminal is powered off or detached from the power supplying net and runs on UPS. 
If zero no automatic shutdown is performed and the application has to call itself the shutdownPOS method.				　　　Applications will be informed about an initiated automatic shutdown.    This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	shutdownPOS Method.

PowerFailDelayTime Property

Syntax	PowerFailDelayTime: int32 {read-only, access after open}

Remarks	This property contains the time in milliseconds for power fail intervals which will not create a power fail situation. In some countries the power has sometimes short intervals where the power supply is interrupted. Those short intervals are in the range of milliseconds up to a few seconds and are handled by batteries or other electric equipment and should not cause a power fail situation. The power fail interval starts when the POS terminal is powered off or detached from the power supplying net and runs on UPS. The power fail interval ends when the POS terminal is again powered on or attached to the power supplying net. However, if the power fail interval is longer than the time specified in the PowerFailDelayTime property a power fail situation is created.

Usually this parameter is a configuration parameter of the underlying power management. So, the application can only read this property.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

PowerSource Property

Syntax	PowerSource: int32 {read-only, access after open}

Remarks	This property holds the current power source if power source reporting is available. A StatusUpdateEvent is generated each time this property is updated.

Value	Meaning

PWR_SOURCE_NA	Power source reporting is not available.

PWR_SOURCE_AC	The current power source is the AC line.

PWR_SOURCE_BATTERY	The current power source is a system battery. This value is only presented for systems that operate normally on battery.

PWR_SOURCE_BACKUP	The current power source is a backup source such as an UPS or backup battery.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	StatusUpdateEvent

QuickChargeMode Property

Syntax	QuickChargeMode: boolean {read-only, access after open}

Remarks	If true, the UPS battery is being recharged in a quick charge mode.
If false, it is being charged in a normal mode.

This property is only set if CapQuickCharge is true.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also 	CapQuickCharge Property, QuickChargeTime Property.

QuickChargeTime Property

Syntax	QuickChargeTime: int32 {read-only, access after open}

Remarks	This time specifies the remaining time for charging the UPS battery in quick charge mode. After the time has elapsed, the UPS battery charging mechanism of power management usually switches into normal mode.

This time is specified in milliseconds.

This property is only set if CapQuickCharge is true.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also 	CapQuickCharge Property, QuickChargeTime Property.

UPSChargeState Property

Syntax	UPSChargeState: int32 {read-only, access after open, enable}

Remarks	This property holds the actual UPS charge state.

It has one of the following values:

Value	Meaning

PWR_UPS_FULL	UPS battery is near full charge.

PWR_UPS_WARNING	UPS battery is near 50% charge.

PWR_UPS_LOW	UPS battery is near empty. Application shutdown should be started to ensure that is can be completed before the battery charge is depleted. A minimum of 2 minutes of normal system operation can be assumed when this state is entered unless this is the first state reported upon entering the “Off” power state.

PWR_UPS_CRITICAL	UPS battery is in a critical state and could be disconnected at any time without further warning.

This property is initialized and kept current while the device is enabled.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20

See Also	CapUPSChargeState Property.

Methods (UML operations)

restartPOS Method

Syntax	restartPOS ( ):void {raises-exception, use after open, enable}

Remarks	Call to restart the POS terminal. This method will always restart the system independent of the system power state.

If the POSPower is claimed, only the application which claimed the device is able to restart the POS terminal.

Applications will be informed about an initiated restart.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	This method is not supported (see the CapRestartPOS property)

See Also	CapRestartPOS Property

shutdownPOS Method

Syntax	shutdownPOS ( ):void {raises-exception, use after open, enable}

Remarks	Call to shut down the POS terminal. This method will always shut down the system independent of the system power state.

If the POSPower is claimed, only the application which claimed the device is able to shut down the POS terminal.

Applications will be informed about an initiated shutdown.

It is recommended that in a power fail situation an application has to call this method after saving all data and setting the application to a defined state.
If the EnforcedShutdownDelayTime property specifies a time greater than zero and the application did not call the shutdownPOS method within the time specified in EnforcedShutdownDelayTime, the system will be shut down automatically. This mechanism may be provided by an underlying operating system to prevent the battery from being emptied before the system is shut down.
This method is only supported if CapShutdownPOS is true.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	This method is not supported.		 	　　(See the CapShutdownPOS property)

See Also	CapShutdownPOS Property, EnforcedShutdownDelayTime Property.

standbyPOS Method

Syntax	standbyPOS (reason: int32): 
　　　　void {raises-exception, use after open, enable}

Remarks	Call to request that the system be placed into the Standby state or to respond to a request from the system, OS or other application that the system be put into Standby state.

The reason parameter indicates the reason the POS terminal should enter a standby state:

Value	Description

PWR_REASON_REQUEST	Call is to request that the system enter the standby state.

PWR_REASON_ALLOW	Call is a response to a standby Status Update Event and specifies that the request should be allowed.

PWR_REASON_DENY	Call is a response to a standby Status Update Event and specifies that the request should be denied.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	This method is not supported (see the CapStandbyPOS property)

See Also	CapStandbyPOS Property.

suspendPOS Method

Syntax	suspendPOS (reason: int32): 
void {raises-exception, use after open, enable}

Remarks	Call to request that the system be placed into the Suspend state or to respond to a request from the system, OS or other application that the system be put into Suspend state.

The reason parameter indicates the reason the POS terminal should enter a standby state:

Value	Description

PWR_REASON_REQUEST	Call is to request that the system enter the suspend state.

PWR_REASON_ALLOW	Call is a response to a suspend Status Update Event and specifies that the request should be allowed.

PWR_REASON_DENY	Call is a response to a suspend Status Update Event and specifies that the request should be denied.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	This method is not supported (see the CapSuspendPOS property)

See Also	CapSuspendPOS Property.

Events (UML Interfaces)

DirectIOEvent

<< event >>	upos::events::DirectIOEvent
EventNumber	: int32 {read-only}
Data			: int32 {read-write}
Obj			: object{read-write}

Description	Provides Service information directly to the application. This event provides a means for a vendor-specific POSPower Service to provide events to the application that are not otherwise supported by the device control.

Attributes	This event contains the following attributes:

Attributes	Type	Description

EventNumber	int32	Event number whose specific values are assigned by the Service.

Data	int32	Additional numeric data. Specific values vary by the EventNumber and the Service. This property is settable.

Obj	object	Additional data whose usage varies by the EventNumber and Service. This property is settable.

Remarks	This event is to be used only for those types of vendor specific functions that are not otherwise described. Use of this event may restrict the application program from being used with other vendor’s POSPower devices which may not have any knowledge of the Service’s need for this event.

See Also	“Errors" on page Intro-20, directIO Method.

StatusUpdateEvent

<< event >>	upos::events::StatusUpdateEvent
Status		: int32 {read-only}

Description	Delivered when UPSChargeState changes or an alarm situation occurs.

Attributes	This event contains the following attribute:

Attributes		 Type		Description

Status		int32		See below.

The Status property contains the updated power status or alarm status.

Value            		Meaning

PWR_SUE_UPS_FULL	UPS battery is near full charge. Can be returned 				if CapUPSChargeState contains 						PWR_UPS_FULL.

PWR_SUE_UPS_WARNING

UPS battery is near 50% charge. Can be returned if CapUPSChargeState contains PWR_UPS_WARNING.

PWR_SUE_UPS_LOW	UPS battery is near empty. Application shutdown 				should be started to ensure that it can be  					completed before the battery charge is depleted. 				A minimum of 2 minutes of normal system 				operation can be assumed when this state is 				entered unless this is the first charge state 					reported upon entering the “Off” state. Can be  				returned if CapUPSChargeState 	contains  				PWR_UPS_LOW.

PWR_SUE_UPS_CRITICAL

UPS is in critical state, and will in short time be 				disconnected. Can be returned if 						CapUPSChargeState contains 						PWR_UPS_CRITICAL.

PWR_SUE_FAN_STOPPED

The CPU fan is stopped. Can be returned if 					CapFanAlarm is true.

PWR_SUE_FAN_RUNNING

The CPU fan is running. Can be returned if 				CapFanAlarm is true.

PWR_SUE_TEMPERATURE_HIGH

The CPU is running on high temperature. Can be 					returned if CapHeatAlarm is true.

PWR_SUE_TEMPERATURE_OK

The CPU is running on normal temperature. Can				be returned if CapHeatAlarm is true.

PWR_SUE_SHUTDOWN											The system will shut down immediately.

PWR_SUE_BAT_LOW	The system remaining battery capacity is at or 					below the low battery threshold and the system is 					operating from the battery.

PWR_SUE_BAT_CRITICAL

The system remaining battery capacity is at or 					below the critically low battery threshold and the 					system is operating from the battery.

PWR_SUE_BAT_CAPACITY_REMAINING.

The BatteryCapacityRemaining property has been updated

PWR_SUE_BAT_CAPACITY_REMAINING_IN_SECONDS

The BatteryCapacityRemainingInSeconds property has been updated

PWR_SUE_RESTART	The system will restart immediately.

PWR_SUE_STANDBY	The system is requesting a transition to the     					Standby state

PWR_SUE_USER_STANDBY

The system is requesting a transition to the       Standby state as a result of user input.

PWR_SUE_SUSPEND	The system is requesting a transition to the 			Suspend state.

PWR_SUE_USER_SUSPEND

The system is requesting a transition to the Suspend state as a result of user input.

PWR_SUE_PWR_SOURCE

The PowerSource property has been updated.

Note that Release 1.3 added Power State Reporting with additional Power 	reporting StatusUpdateEvent values.

The Update Firmware capability, added in Release 1.9, added additional 	Status values for communicating the status/progress of an asynchronous update firmware process.  See “StatusUpdateEvent” description on page 1-34.

See Also		CapFanAlarm Property, CapHeatAlarm Property, 	CapUPSChargeState 	Property, UPSChargeState Property.

Chapter 39

Video Capture

This Chapter defines the Video Capture device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.16 | open |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.16 | open |
| CapPowerReporting: | int32 | {read-only} | 1.16 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.16 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.16 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.16 | open |
| CheckHealthText: | string | {read-only} | 1.16 | open |
| Claimed: | boolean | {read-only} | 1.16 | open |
| DataCount: | int32 | {read-only} | 1.16 | open |
| DataEventEnabled: | boolean | {read-write} | 1.16 | open |
| DeviceEnabled: | boolean | {read-write} | 1.16 | open & claim |
| FreezeEvents: | boolean | {read-write} | 1.16 | open |
| OutputID: | int32 | {read-only} | 1.16 | Not Supported |
| PowerNotify: | int32 | {read-write} | 1.16 | open |
| PowerState: | int32 | {read-only} | 1.16 | open |
| State: | int32 | {read-only} | 1.16 | -- |
| DeviceControlDescription: | string | {read-only} | 1.16 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.16 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.16 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.16 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.16 | open |
| PhysicalDeviceName: | string | {read-only} | 1.16 | open |

| Properties (Continued) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specific | Type | Mutability | Version | May Use After |
| CapAssociatedHardTotalsDevice: | string | {read-only} | 1.16 | open |
| CapAutoExposure: | boolean | {read-only} | 1.16 | open |
| CapAutoFocus: | boolean | {read-only} | 1.16 | open |
| CapAutoGain: | boolean | {read-only} | 1.16 | open |
| CapAutoWhiteBalance: | boolean | {read-only} | 1.16 | open |
| CapBrightness: | boolean | {read-only} | 1.16 | open |
| CapContrast: | boolean | {read-only} | 1.16 | open |
| CapExposure: | boolean | {read-only} | 1.16 | open |
| CapGain: | boolean | {read-only} | 1.16 | open |
| CapHorizontalFlip: | boolean | {read-only} | 1.16 | open |
| CapHue: | boolean | {read-only} | 1.16 | open |
| CapPhoto: | boolean | {read-only} | 1.16 | open |
| CapPhotoColorSpace: | boolean | {read-only} | 1.16 | open |
| CapPhotoFrameRate: | boolean | {read-only} | 1.16 | open |
| CapPhotoResolution: | boolean | {read-only} | 1.16 | open |
| CapPhotoType: | boolean | {read-only} | 1.16 | open |
| CapSaturation: | boolean | {read-only} | 1.16 | open |
| CapStorageTarget: | int32 | {read-only} | 1.16 | open |
| CapVerticalFlip: | boolean | {read-only} | 1.16 | open |
| CapVideo: | boolean | {read-only} | 1.16 | open |
| CapVideoColorSpace: | boolean | {read-only} | 1.16 | open |
| CapVideoFrameRate: | boolean | {read-only} | 1.16 | open |
| CapVideoResolution: | boolean | {read-only} | 1.16 | open |
| CapVideoType: | boolean | {read-only} | 1.16 | open |
| AutoExposure: | boolean | {read-write} | 1.16 | open, claim & enable |
| AutoFocus: | boolean | {read-write} | 1.16 | open, claim & enable |
| AutoGain: | boolean | {read-write} | 1.16 | open, claim & enable |
| AutoWhiteBalance: | boolean | {read-write} | 1.16 | open, claim & enable |
| Brightness: | int32 | {read-write} | 1.16 | open, claim & enable |
| Contrast: | int32 | {read-write} | 1.16 | open, claim & enable |
| Exposure: | int32 | {read-write} | 1.16 | open, claim & enable |
| Gain: | int32 | {read-write} | 1.16 | open, claim & enable |
| HorizontalFlip: | boolean | {read-write} | 1.16 | open, claim & enable |
| Hue: | int32 | {read-write} | 1.16 | open, claim & enable |
| PhotoColorSpace: | string | {read-write} | 1.16 | open, claim & enable |
| PhotoColorSpaceList: | string | {read-only} | 1.16 | open |
| PhotoFrameRate: | int32 | {read-write} | 1.16 | open, claim & enable |
| PhotoMaxFrameRate: | int32 | {read-only} | 1.16 | open |
| PhotoResolution: | string | {read-write} | 1.16 | open, claim & enable |
| PhotoResolutionList: | string | {read-only} | 1.16 | open |
| PhotoType: | string | {read-write} | 1.16 | open, claim & enable |
| PhotoTypeList: | string | {read-only} | 1.16 | open |
| RemainingRecordingTimeInSec: | int32 | {read-only} | 1.16 | open, claim & enable |
| Saturation: | int32 | {read-write} | 1.16 | open, claim & enable |
| StorageTarget: | int32 | {read-write} | 1.16 | open, claim & enable |
| VerticalFlip: | boolean | {read-write} | 1.16 | open, claim & enable |
| VideoCaptureMode: | int32 | {read-only} | 1.16 | open, claim & enable |
| VideoColorSpace: | string | {read-write} | 1.16 | open, claim & enable |
| VideoColorSpaceList: | string | {read-only} | 1.16 | open |
| VideoFrameRate: | int32 | {read-write} | 1.16 | open, claim & enable |
| VideoMaxFrameRate: | int32 | {read-only} | 1.16 | open |
| VideoResolution: | string | {read-write} | 1.16 | open, claim & enable |
| VideoResolutionList: | string | {read-only} | 1.16 | open |
| VideoType: | string | {read-write} | 1.16 | open, claim & enable |
| VideoTypeList: | string | {read-only} | 1.16 | open |

| Methods (UML operations) |  |
| --- | --- |
| Common |  |
| Name | Version |
| open (logicalDeviceName: string): void {raises-exception} | 1.16 |
| close ( ): void {raises-exception, use after open} | 1.16 |
| claim (timeout: int32): void {raises-exception, use after open} | 1.16 |
| release ( ): void {raises-exception, use after open, claim} | 1.16 |
| checkHealth (level: int32): void {raises-exception, use after open, enable} | 1.16 |
| clearInput ( ): void { } | 1.16 |
| clearInputProperties ( ): void { } | 1.16 |
| clearOutput ( ): void { } | Not supported |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} | 1.16 |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, enable} | 1.16 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.16 |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.16 |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, enable} | 1.16 |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.16 |
| Specific |  |
| Name |  |
| startVideo (fileName: string, overwrite: boolean, recordingTime: int32): void {raises-exception, use after open, claim, enable} | 1.16 |
| stopVideo (): void {raises-exception, use after open, claim, enable} | 1.16 |
| takePhoto (fileName: string, overwrite: int32): void {raises-exception, use after open, claim, enable} | 1.16 |

| Events (UML interfaces) |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Mutability | Version |
| upos::events::DataEvent |  | Not supported |  |
| Status: |  |  |  |
| upos::events::DirectIOEvent |  |  | 1.16 |
| EventNumber: | int32 | {read-only} |  |
| Data: | int32 | {read-write} |  |
| Obj: | object | {read-write} |  |
| upos::events::ErrorEvent |  |  | 1.16 |
| ErrorCode: | int32 | {read-only} |  |
| ErrorCodeExtended: | int32 | {read-only} |  |
| ErrorLocus: | int32 | {read-only} |  |
| *pErrorResponse | int32 | {read-write} |  |
| upos::events::OutputCompleteEvent |  | Not Supported |  |
| upos::events::StatusUpdateEvent |  |  | 1.16 |
| Status: | int32 | {read-only} |  |

General Information

The Video Capture Device name is “Video Capture”.

Capabilities

Video capture device class has the following capabilities:

- Take a photo and record it as a file in a host and may store it in the targeted storage device.

- Take a video and record it as a file in a host and may store it in the targeted storage device.

- May read the encoded data from the bar code label with the hydra connected  scanner device.

- May detect the individuals faces and/or  objects with the hydra connected individual recognition device.

Video Capture Class Diagram

The following diagram shows the relationships between the Video Capture classes.

Fig. Chap. 39-1 Video Capture Class Diagram

Model

Modes

The Video Capture Device has two operation modes.

・Photo Mode

・Video Mode

The operation of each mode is as follows.

・Photo Mode

Photo Mode may capture a photo image and may save it in a host as the image data file format, if CapPhoto property is true. Its’ capable data file format is indicated in the PhotoType property and all of the capable values are listed in the PhotoTypeList property. And the device may save the file in the targeted storage device that is specified by the StorageTarget property, if CapStorageTarget value is CST_HARDTOTAL_ONLY or CST_ALL.

・Video Mode

Video Mode may capture a video image data and may save it in a host as the video image data file format, if CapVideo property is true. Its’ capable data file format is indicated in the VideoType property and all of the capable values are listed in the VideoTypeList property. And the device may save the file in the targeted storage device that is specified by the StorageTarget property, if CapStorageTarget value is CST_HARDTOTAL_ONLY or CST_ALL.

Device behaviors

“Video capture device” device control follows the device behavior as follows.	　 They are different in each mode as described below.

Photo Mode

If CapPhoto property is true, this mode can be executed. 				            Prior to start this mode, “Video Capture Device” device control needs to set the VideoCaptureMode property as to be VCAP_VCMODE_PHOTO.  And each of CapPhotoColorSpace, CapPhotoFrameRate, CapPhotoResolution, CapPhotoType property is true and these PhotoColorSpaceList, PhotoMaxFrameRate, PhotoResolutionList and PhotoTypeList should have the appropriate values to be used as the photo file data in this targeted device.  And then it needs to set the appropriate values in the each of PhotoColorSpace property, PhotoFrameRate property, PhotoResolution property and PhotoType property.

It starts photo capturing by executing the takePhoto method. Then, “Video Capture Device” device control may capture a photo image and may save it in a host as an image data file format specified by the value of PhotoType property that is listed in the PhotoTypeList property. And may store it in the storage device specified by the StorageTarget property, if CapStorageTarget value is CST_HARTTOTAL_ONLY or CST_ALL. Then the file name is set by the takePhoto method parameter and can deliver the photo data file to the application. 　If device needs to be able to write the image data file to an associated HardTotals device, the CapAssociatedHardTotalsDevice property holds the open name of the associated HardTotals device.

This method is executed synchronously. Only one call to takePhoto method can be in progress at a time. An attempt to nest video capture device operations will result in an UPOSException being thrown.

When it exceeded the specified parameter time out or when photo file generation is finished or when clearInput method is executed, the taking photo process will be ended.

StatusUpdateEvent with status VCAP_SUE_START_PHOTO is evoked when takePhoto method is executed to notify the application that recording state has started.

When the taking photo is finished, or the specified time out has been exceeded, a StatusUpdateEvent with status VCAP_SUE_END_PHOTO is evoked to notify the application that photo taking has been ended.

An ErrorEvent event (or events) is enqueued if an error occurs while gathering or processing input, and is delivered to the application when DataEventEnabled property is true and other event delivery requirements are met.  The DataCount property may be read to obtain the total number of enqueued events.

If ErrorEvent response is ER_CONTINUEINPUT, the process of input processing continues. However, as long as the cause of the error is not resolved, the ErrorEvent will occur again immediately.

If ErrorEvent is ER_CLEAR, the input processing process is terminated and the taking photo is discarded.

All enqueued input may be deleted by calling clearInput method. See the clearInput method description for more details.

All data properties that are populated as a result of firing an ErrorEvent event can be set back to their default values by calling the clearInputProperties method.

Video Mode

Prior to start this mode, “Video Capture Device” device control needs to set the VideoCaptureMode property as to be VCAP_VCMODE_VIDEO.  And each of CapVideoColorSpace, CapVideoFrameRate, CapVideoResolution and CapVideoType property is true and these VideoColorSpaceList, VideoMaxFrameRate, VideoResolutionList and VideoTypeList should have the appropriate values to be used as the video image data file in this targeted device. And then it needs to set the appropriate values in the each of VideoColorSpace property, VideoFrameRate property, VideoResolution property and VideoType property.

It starts video image capturing by executing the startVideo method. Then “Video Capture Device” device control captures a video image and save it in a host with the filename specified value of VideoType property that is listed in the VideoTypeList property. And may store it in the storage device specified by the StorageTarget property, if CapStorageTarget value is CST_HARTTOTAL_ONLY or CST_ALL. And the file name is set by the startVideo method parameter and can deliver the video image data file to the application. This method is executed synchronously.

The video capturing ends after the specified time has elapsed or when stopVideo method is called or when clearInput method is called.

The remaining video capture recording time in seconds can be obtained from the property RemainingRecordingTimeInSec.

StatusUpdateEvent with status VCAP_SUE_START_VIDEO is evoked when startVideo method is executed to notify the application that taking video has been started.

When the taking video is finished, or the specified time out has been exceeded, a StatusUpdateEvent with status VCAP_SUE_END_VIDEO is evoked to notify the application that taking video has been ended.

If the time specified by the startVideo method is FOREVER(-1), execution will continue until the stopVideo method is called. When stopVideo is called, the previous taking video data may be recorded in a host and deliver to the targeted storage device specified by the StorageTarget property, if CapStorageTarget property value is CST_HARTTOTAL_ONLY or CST_ALL. And it can be delivered to the application with the specified file name that is set by the startVideo method.

Only one call to startVideo method can be in progress at a time. An attempt to nest taking video operations will result in an UPOSException being thrown.

If Error occurs during the execution of the startVideo method, application may call the stopVideo method to terminate the taking video process or cancel the taking video process by calling the clearInput method before ending the ErrorEvent processing. After this when the stopVideo method is called, the video file data until just before the ErrorEvent occur is stored  to the host and targeted storage device that is specified by the StorageTarget property, if CapStorageTarget property value is CST_HARTTOTAL_ONLY or CST_ALL, and can be delivered to the application.

If ErrorEvent response is ER_CONTINUEINPUT, the process of input processing continues. However, as long as the cause of the error is not resolved, the ErrorEvent will occur again immediately.

If ErrorEvent is ER_CLEAR, the input processing process is terminated and the taking video is discarded.

An ErrorEvent event (or events) is enqueued if an error occurs while gathering or processing input, and is delivered to the application when DataEventEnabled property is true and other event delivery requirements are met.  The DataCount property may be read to obtain the total number of enqueued events.

If there is no error during the execution of startVideo method, it is possible to terminate the taking video process and can stop the taking video anytime. When the stopVideo method is called, the video data until just before the method is called,  may be recorded in the host and targeted storage device that is specified by the StorageTarget property if CapStorageTarget property is CST_HARTTOTAL_ONLY or CST_ALL, and can deliver it to the application.

All enqueued input may be deleted by calling clearInput method. See the clearInput method description for more details.

All data properties that are populated as a result of firing an ErrorEvent event can be set back to their default values by calling the clearInputProperties method.

Device Sharing

Video capture is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before accessing many video         capture-specific properties.

- The application must claim and enable the device before calling methods that      manipulate the device.

- See the “Summary” table for precise usage prerequisites.

Properties (UML attributes)

AutoExposure Property

Syntax	AutoExposure: boolean {read-write, access after open}

Remarks	If true, auto exposure of camera is enabled.					Otherwise, it is false. This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20. 				Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified. 						Or it does not support this function.

See also	CapAutoExposure Property

AutoFocus Property

Syntax	AutoFocus: boolean {read-write, access after open}

Remarks	If true, auto focus of camera is enabled. Otherwise, it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20. 				Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified. 							Or it does not support this function.

See also	CapAutoFocus Property

AutoGain Property

Syntax	AutoGain: boolean {read-write, access after open}

Remarks       If true, auto gain of camera is enabled. Otherwise it is false.

When this property is true, it is possible to read the value of  Gain property. However, it is not possible to write and change the value of Gain property. 	  If AutoGain property is false, then, it is possible to read, write and change the value of Gain property.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed.			For further information, see “Errors" on page Intro-20. 				Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified. 							Or it does not support this function.

See also	CapAutoGain Property	Gain Property

AutoWhiteBalance Property

Syntax	AutoWhiteBalance: boolean {read-write, access after open}

Remarks	If true, auto white balance of camera is enabled. Otherwise, it is false.		This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20. 				Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified. 							Or it does not support this function.

See also	CapAutoWhiteBalance Property

Brightness property

Syntax		Brightness: int32 {read-write, access after open}

Remarks	Indicate the brightness of camera. 							Valid values range from 0 to 100.								This property is initialized by the open method.

Errors		A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20. 					Some possible values of the exception’s ErrorCode property are:

Value			Meaning                                                         			E_ILLEGAL		An invalid value was specified. 								Or it does not support this function.

See Also		CapBrightness Property

CapAssociatedHardTotalsDevice Property

Syntax	CapAssociatedHardTotalsDevice : string {read-write, access after open}

Remarks	Holds the open name of the associated HardTotals device if the device is able to write to such devices which is the case if CapStorageTarget is either CST_ALL or CST_HARDTOTALS_ONLY. If CapStorageTarget is CST_HOST_ONLY this property value must be the empty string.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	CapStorageTarget Property

CapAutoExposure Property

Syntax	       	CapAutoExposure: boolean {read-only, access after open}

Remarks	       	If true, can change the auto exposure of camera. Otherwise, it is false.		This property is initialized by the open method.

Errors           	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	        	AutoExposure Property

CapAutoFocus Property

Syntax	CapAutoFocus: boolean {read-only, access after open}

Remarks	If true, can change the auto focus of camera.  Otherwise, it is false.			This property is initialized by the open method.

Errors    	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	AutoFocus Property

CapAutoGain Property

Syntax	CapAutoGain: boolean {read-only, access after open}

Remarks	If true, automatic gain change of the camera is possible.  Otherwise, it is 		false. 	This property is initialized by the open method.

Errors   	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	AutoGain Property

CapAutoWhiteBalance Property

Syntax	CapAutoWhiteBalance: boolean {read-only, access after open}

Remarks	If true, auto white balance of camera is possible.  Otherwise, it is false.  		This property is initialized by the open method.

Errors   	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	AutoWhiteBalance Property

CapBrightness Property

Syntax	CapBrightness: boolean {read-only, access after open}

Remarks	If true, the brightness of camera can be changed.  Otherwise, it is false. 		This property is initialized by the open method.

Errors    	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	Brightness Property

CapContrast Property

Syntax	CapContrast: boolean {read-only, access after open}

Remarks	If true, can change the contrast of camera. Otherwise, it if false. 		      	This property is initialized by the open method.

Errors     	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	Contrast Property

CapExposure Property

Syntax	CapExposure: boolean {read-only, access after open}

Remarks	If true, can change the exposure of camera. Otherwise, it is false. 			This property is initialized by the open method.

Errors    	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	Exposure Property

CapGain Property

Syntax	CapGain: boolean {read-only, access after open}

Remarks	If true, can change the gain of camera.  Otherwise, it is false. 			This property is initialized by the open method.

Errors    	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	Gain Property

CapHorizontalFlip Property

Syntax	 CapHorizontalFlip: boolean {read-only, access after open}

Remarks	If true, can change the horizontal flip of camera. Otherwise, it is false. 		This property is initialized by the open method.

Errors    	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	HorizontalFlip Property

CapHue Property

Syntax	CapHue: boolean {read-only, access after open}

Remarks	If true, the hue of the camera can be changed. Otherwise, it is false. 			This property is initialized by the open method.

Errors 	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	Hue Property

CapPhoto Property

Syntax	CapPhoto: boolean {read-only, access after open}

Remarks	If true, it supports the photo function and can take a photo. And to activate the photo mode, the VideoCaptureMode property value needs to set VCAP_VCMODE_PHOTO.  If false, it’s not supporting the photo function.  This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	VideoCaptureMode Property

CapPhotoColorSpace Property

Syntax	CapPhotoColorSpace: boolean {read-only, access after open}

Remarks	If true, can handle and change the photo color space. Otherwise, it is false. 		This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20

See also	PhotoColorSpace Property

CapPhotoFrameRate Property

Syntax	CapPhotoFrameRate: boolean {read-only, access after open}

Remarks	If true, can handle and change the capture frame rate. Otherwise, it is false. 
		This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	PhotoFrameRate Property

CapPhotoResolution Property

Syntax	CapPhotoResolution: boolean {read-only, access after open}

Remarks	If true, taking photo resolution is handled and can be changed. Otherwise, 		it is false. This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	PhotoResolution Property

CapPhotoType Property

Syntax	CapPhotoType: boolean {read-only, access after open}

Remarks	If true, photo image format type can be changed. Otherwise, it is false.
			This property is initialized by the open method.

Errors		A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

CapSaturation Property

Syntax	CapSaturation: boolean {read-only, access after open}

Remarks	If true, can change the saturation of camera. Otherwise, it is false. 			This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	Saturation Property

CapStorageTarget Property

Syntax	CapStorageTarget: int32 {read-only, access after open}

Remarks	This is an enumeration and announces where the device is able to write the recorded sound data file to.
It holds one of the following values.

Value　　　　　　　　　　　	Meaning

CST_HARDTOTAL_ONLY　　	Only an associate HardTotals device is　supported.

CST_HOST_ONLY	Only the host’s file system is supported.

CST_ALL	Both, the associated HardTotals device and the host’s file system is supported.

This property is initialized by the open method.

If a HardTotals device is supported the storage target, the property value should be CST_HARDTOTAL_ONLY or CST_ALL, and the property CapAssociatedHardTotalsDevice holds the open name of the associated HardTotals device.

Errors		UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	StorageTarget Property, CapAssociatedHardTotalsDevice Property

CapVerticalFlip Property

Syntax	      CapVerticalFlip: boolean {read-only, access after open}

Remarks       If true, can change the vertical flip of camera.  Otherwise, it is false.		       This property is initialized by the open method.

Errors         A UposException may be thrown when this property is accessed. 		       For further information, see “Errors" on page Intro-20.

See also      VerticalFlip Property

CapVideo Property

Syntax	       CapVideo: boolean {read-only, access after open}

Remarks         If true, video function is supported. Otherwise, it is false. If this property        is true, taking video and recording can be done by calling the startVideo method. And to activate the video mode, the VideoCaptureMode property value needs to set VCAP_VCMODE_VIDEO. If false, taking video and recording cannot be performed. This property is initialized by the open method.

Errors		    A UposException may be thrown when this property is accessed. 		   For further information, see “Errors" on page Intro-20.

See also	         StartVideo Method, VideoCaptureMode Property

CapVideoColorSpace Property

Syntax	CapVideoColorSpace: boolean {read-only, access after open}

Remarks	If true, can change the color space when taking the video. Otherwise, it is 		false. This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 		For further information, see “Errors" on page Intro-20

See also	VideoColorSpace Property

CapVideoFrameRate Property

Syntax		CapVideoFrameRate: boolean {read-only, access after open}

Remarks	If true, can change the video frame rate from 1 to up to 						VideoMaxFrameRate property value. Otherwise, it is false.
			This property is initialized by the open method.

Errors		A UposException may be thrown when this property is accessed. 					For further information, see “Errors" on page Intro-20.

See also		VideoMaxFrameRate Property, VideoFrameRate Property

CapVideoResolution Property

Syntax		CapVideoResolution: boolean {read-only, access after open}

Remarks	If true, taking video resolution can be changed and all of possible values are listed in the VideoResolutionList property values. If false, taking video resolution cannot be changed. This property is initialized by the open method.

Errors	    A UposException may be thrown when this property is accessed. 		For further information, see “Errors" on page Intro-20.

See also		VideoResolutionList Property, VideoResolution Property

CapVideoType Property

Syntax		CapVideoType: boolean {read-only, access after open}

Remarks	If true, taking video type can be changed and all of possible values are 			listed in the VideoTypeList values. Otherwise, it is false.
			This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed.
For further information, see “Errors" on page Intro-20.

See also		VideoTypeList Property, VideoType Property

Contrast Property

Syntax		Contrast: int32 {read-write, access after open}

Remarks	Indicate the contrast of the camera. Valid values range from 0 to 100.			This property is initialized by the open method.

Errors		A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20. 					Some possible values of the exception’s ErrorCode property are:

Value			Meaning                                                         			E_ILLEGAL		An invalid value was specified. 								Or it does not support this function.

See Also	CapContrast Property

Exposure Property

Syntax	Exposure: int32 {read-write, access after open}           		     	    Remarks	Indicate the exposure of camera. Valid values range from 0 to 100. 				This property is initialized by the open method.                    		    Errors	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20. 					Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified.                          					Or it does not support this function.

See also	CapExposure Property

Gain Property

Syntax	Gain: int32 {read-write, access after open}

Remarks	Indicate the gain of camera. Valid values range from 0 to 100.                      If AutoGain property is true, it is possible to read the value of Gain property. However, it is not possible to write and change the value of Gain property.  If AutoGain property is false, then, it is possible to read, write and change the value of Gain property.                                                                      This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20. 				Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified. 							Or it does not support this function.

See also 	CapGain Property, AutoGain Property

HorizontalFlip Property

Syntax	HorizontalFlip: boolean {read-write, access after open}

Remarks	If true, horizontal flip of camera is enabled and it is possible to reverse the camera captured image horizontally.  Otherwise, it is false. There is a similar property called VerticalFlip property. However, each VerticalFlip property and HorizontalFlip property value can be set independently.  This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20. 					Some possible values of the exception’s ErrorCode property are:				Value		Meaning

E_ILLEGAL	An invalid value was specified. 			     			Or it does not support this function.

See Also	CapHorizontalFlip property, VerticalFlip property,
			CapVerticalFlip property

Hue Property

Syntax	 Hue: int32 {read-write, access after open}

Remarks	Indicate the hue of camera. Valid values range from 0 to 100.				This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20. 					Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified.                                 				Or it does not support this function.

See also	CapHue Property

PhotoColorSpace Property

Syntax		PhotoColorSpace: string {read-write, access after open}

Remarks	Indicates the photo color space ID of the frame data to be acquired by the			Video Capture Device, if CapPhotoColorSpace property is true. 				Valid values are one of the values listed in the PhotoColorSpaceList 			property.  This property is initialized by the open method.

Errors		A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20. 					Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified.

See also		PhotoColorSpaceList Property, CapPhotoColorSpace Property

PhotoColorSpaceList Property

Syntax	PhotoColorSpaceList: string {read-only, access after open}

Remarks	Photo Color space information supported by the device is indicated in a		comma-separated list. Each color space information is composed 			of the following information and is shown in the following order 			separated by a colon (":").							This property is initialized by the open method.

Parameter		Description

Color space ID		ID for identifying the color space of RGB, YUV 			422, etc. And they are indicating like 				“RGB;YUV422;…..”

Depth			Number of bits per 1 pixel

Errors		A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	PhotoColorSpace Property

PhotoFrameRate Property

Syntax	PhotoFrameRate: int32 {read-write, access after open}

Remarks 	Indicates the frame rate of frame data to be acquired by the Video Capture  		Device.  Valid values range from 1 to PhotoMaxFrameRate 			property. This property is initialized by the open method.

Errors		A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

Value			Meaning

E_ILLEGAL		An invalid value was specified.

See also		CapPhotoFrameRate Property, PhotoMaxFrameRate Property,

PhotoMaxFrameRate Property

Syntax	PhotoMaxFrameRate: int32 {read-only, access after open}

Remarks	Indicates the maximum frame rate that can be set for the 				PhotoFrameRate property.							This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 			For further Information, see “Errors" on page Intro-20.

See also	PhotoFrameRate Property

PhotoResolution Property

Syntax		PhotoResolution: string {read-write, access after open}

Remarks	It shows the resolution of the frame data acquired by the Video Capture Device  and the photo taken and recorded with the takePhoto method. Valid values are one of those listed in PhotoResolutionList property.   This property is initialized by the open method.

Errors		A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20. 					Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified.

See also	PhotoResolutionList Property, VideoCaptureMode Property,   takePhoto Method

PhotoResolutionList Property

Syntax	PhotoResolutionList: string {read-only, access after open}

Remarks	Indicating the comma-separated list of possible resolutions for the PhotoResolution property. Resolution is indicated in "horizontal x height" format. For example, when you support 320x240, 640x480, 640x360, it is the following. "320x240,640x480,640x360".
This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See also	PhotoResolution Property

PhotoType Property

Syntax		PhotoType:  string {read-write, access after open}

Remarks	Indicates the data format of photo taken with the takePhoto method. Valid values are one of the values listed in the PhotoTypeList property.
This property is initialized by the open method.

Errors		A UposException may be thrown when this property is accessed.
			For further information, see “Errors" on page Intro-20.
			Some possible values of the exception’s ErrorCode property are:

Value			Meaning						E_ILLEGAL		An invalid value was specified.

See also		PhotoTypeList Property, takePhoto Method

PhotoTypeList Property

Syntax		PhotoTypeList: string {read-only, access after open}

Remarks	A comma-separated list of photo image format values that can be set for 			the PhotoType property.									For example, when supporting BMP and JPEG, it is the following. 				"BMP,JPEG"

Note: The notation contents may be different depending on the device.		This property is initialized by the open method.

Errors			A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	PhotoType Property

RemainingRecordingTimeInSec Property

Syntax	RemainingRceordingTimeInSec: 				                                      int32 {read-only, access after open-claim-enable}

Remarks	This property holds the remaining recording time in seconds if a video recording is ongoing. If no video recording is ongoing its value is 0. When a call to method startVideo returns, this property initially holds the time passed as argument recordingTime to that call. If this argument value is FOREVER   (-1), this property also holds this value unchanged until stopVideo method has been called. This property is initialized during device set DeviceEnabled method to 0.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	startVideo Method, stopVideo Method

Saturation Property

Syntax	Saturation: int32 {read-write, access after open}

Remarks	Indicate the saturation of camera. 	Valid values range from 0 to 100. 			This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20. 					Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified. 						Or it does not support this function.

See also	CapCameraSaturation Property

StorageTarget Property

Syntax	StorageTarget: string {read-write, access after open-claim-enable}

Remarks	Holds the open name of HardTotals device, if CapStorageTarget property value is CST_HARTTOTAL_ONLY or CST_ALL. If CapStorageTarget property value is CST_HOST_ONLY, this property value must be the empty string. This property is initialized by the open method according to the value hold by CapStorageTarget.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	CapStorageTarget Property

VerticalFlip Property

Syntax		VerticalFlip: boolean {read-write, access after open}

Remarks　	If true, vertical flipping of the video is enabled and it is possible to reverse			the video or photo image capturing vertically. Otherwise, it is false.  			There is a similar property called HorizontalFlip property and each    	    		    	VerticalFlip property and HorizontalFlip property value can be 				set independently.  This property is initialized by the open method.

Errors	    	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20. 				Some possible values of the exception’s ErrorCode property are:

Value		Meaning

E_ILLEGAL	An invalid value was specified. 							Or it does not support this function.

See also	     	CapVerticalFlip Property, HorizontalFlip Property,
              	CapHorizontalFlip Property

VideoCaptureMode Property

Syntax		VideoCaptureMode: int32 {read-write, access after open}

Remarks	Indicate the operation mode of video capture device.
			Valid values are as follows

Parameter		Description

VCAP_VCMODE_PHOTO

This mode is for taking photo and their data recording. Can be set when CapPhoto property is true.
The values of the PhotoType property, PhotoColorSpace property, PhotoResolution property PhotoFrameRate property are applied to the taking photo image formats list in the PhotoTypeList property, the color space values list in the PhotoColorSpaceList property, the resolution values list in the PhotoResolutionList property, and the frame rate values within the values of PhotoMaxFrameRate property.  And taking photo is executed by the takePhoto method.

VCAP_VCMODE_VIDEO

This mode is for taking the videos and their data recording. Can be set when CapVideo property is true. The value of the VideoType property, VideoColorSpace property, VideoResolution property and VideoFrameRate property are applied to the taking video image format list in the VideoTypeList property, the color space values list in the VideoColorSpaceList property, the resolution values list in the VideoResolutionList property and frame rate values within the values of VideoMaxFrameRate property.  Taking the videos and  their data recording will be executed by the startVideo method and ends taking the video by using the stopVideo method.

This property is initialized by the by the open method. The default value of 				this property is VCAP_VCMODE_PHOTO.

Errors 	           	A UposException may be thrown when this property is accessed.
			            	For further information, see “Errors" on page Intro-20.

See also		PhotoColorSpace Property,  VideoColorSpace Property, 						PhotoResolution Property, VideoResolution Property, VideoFrameRate 				Property, PhotoFrameRate Property, CapPhotoColorSpace Property, 				CapVideoColorSpace Property, CapPhotoResolution Property, 					CapVidoeResolution Property, VideoMaxFrameRate Property,
				PhotoMaxFrameRate Property, VideoCaptureMode Property, 					CapPhoto Property, CapVideo Property, VideoType Property, 					VideoTypeList Property PhotoType Property, PhotoTypeList Property, 				takePhoto Method, startVideo Method, stopVideo Method.

VideoColorSpace Property

Syntax	VideoColorSpace: string {read-write, access after open}

Remarks	Indicates the video color space ID of the frame data to be used by 			startVideo method. Valid values are one of the values listed in the 			VideoColorSpaceList property. 							This property is referred to when VideoCaptureMode property value is 		VCAP_VCMODE_VIDEO and CapVideo is true. 					This property is initialized by the open method.

Errors		A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20. 				Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified.

See also	VideoColorSpaceList Property, VideoCaptureMode Property,			stardVideo Method

VideoColorSpaceList Property

Syntax	VideoColorSpaceList: string {read-only, access after open}

Remarks	Video Color space information supported by the device is indicated in a 		comma-separated list. Each color space information is composed 			of the following information and is shown in the following order 			separated by a colon (":").							This property is initialized by the open method.

Parameter		Description

Color space ID		ID for identifying the color space of RGB, YUV 			422, etc.  And they are indicating 					like“RGB;YUV422;…..”

Depth			Number of bits per 1 pixel

Errors		A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	CapVideoColorSpace Property, VideoColorSpace Property

VideoFrameRate Property

Syntax	           VideoFrameRate; int32 {read-write, access after open}

Remarks	Indicates the frame rate of the frame data recorded by the Video Capture                Device and the video image capturing and recorded with the startVideo method. Valid values range from 1 to VideoMaxFrameRate property and CapVideo property is true.  This property is only applied when VCAP_VCMODE_VIDEO is set in VideoCaptureMode property.   This property is initialized by the open method.

Errors			A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20. 				Some possible values of the exception’s ErrorCode property are:

Value			Meaning  		     				E_ILLEGAL		An invalid value was specified.

See also		VideoMaxFrameRate Property, CapVideo Property
			VideoCaptureMode Property, startVideo Method

VideoMaxFrameRate Property

Syntax		VideoMaxFrameRate: int32 {read-only, access after open}

Remarks	Indicates the maximum video recording frame rate that can be set in 			VideoFrameRate property. 								This property is initialized by the open method.

Errors     	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See also		VideoFrameRate Property

VideoResolution Property

Syntax		VideoResolution: string {read-write, access after open}

Remarks	Indicates the resolution of video image data acquired by the Video Capture              Device and recorded with the execution of startVideo method. Valid values are one of the values listed in the VideoResolutionList property. This property is only applied when VCAP_VCMODE_VIDEO is set in VideoCaptureMode property and if CapVideo property is true.     This property is initialized by the open method.

Errors			A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20. 				Some possible values of the exception’s ErrorCode property are:

Value			Meaning			           	E_ILLEGAL		An invalid value was specified.

See also	VideoResolutionList Property, CapVideo Property
		VideoCaptureMode Property, startVideo Method

VideoResolutionList Property

Syntax		VideoResolutionList: string {read-only, access after open}

Remarks	A comma-separated list of possible resolutions for the VideoResolution property. Resolution is indicated by "Horizontal resolution number x Vertical resolution number" format. For example, when it supports 320x240, 640x480, 640x360, it is the following. "320x240,640x480,640x360"					      This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed.
For further information, see “Errors" on page Intro-20.

See also		CapVideoResolution Property, VideoResolution Property

VideoType Property

Syntax		VideoType; string {read-write, access after open}

Remarks	Indicate the shape of the taking video and recorded with the						startVideo method. Valid values are one of those listed in VideoTypeList 				property. This property is applied when VCAP_VCMODE_VIDEO is set 				in VideoCaptureMode property and if CapVideo property is true.					This property is initialized by the open method.

Errors			A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20. 				Some possible values of the exception’s ErrorCode property are:

Value			Meaning             		    		E_ILLEGAL		An invalid value was specified.

See also		VideoCaptureMode Property, CapVideo Property, VideoTypeList 	Property, startVideo Method

VideoTypeList Property

Syntax		VideoTypeList: string {read-only, access after open}

Remarks	A comma-separated list of image format values that can be set for the 			VideoType property.									For example, when AVI_IYUV, AVI_MJPG is supported, it is the 				following "AVI_IYUV,AVI_MJPG".
			Note: The notation contents may be different depending on the device.
			This property is initialized by the open method.

Errors		A UposException may be thrown when this property is accessed.
			For further information, see “Errors" on page Intro-20.

See also		CapVideoType Property, VideoType Property

Note: Video Capture Device Property Value Relationship

Properties listed below are related within each Photo / Video Mode group, and if any value change occurs, other values may change accordingly.

Photo Mode Group Properties

PhotoType, PhotoColorSpace, PhotoColorSpaceList, PhotoFrameRate, 	PhotoMaxFrameRate, PhotoResolution, PohtoResolutionList

Video Mode Group Properties

VideoType, VideoColorSpace, VideoColorSpaceList, VideoFrameRate, 	VideoMaxFrameRate, VideoResolution, VideoResolutionList

Methods (UML operations)

startVideo Method

Syntax	startVideo (fileName : string, overwrite: boolean, recordingTime: int32):        		            void{raises-exception, use after open-claim-enable}
Parameter	Description

filename	Specify the name of the video file to be recorded. 		Overwrite	Specify the behavior when the same name file exists. 			If true, it is overwritten. 							If false, it will raise the UposException. 			recordingTime 	Specify the time for video recording in seconds.				If FOREVER (-1) is specified, recording will continue                        			until the stopVideo method is called.

Remarks	Before calling this method, it needs to set the VideoCaptureMode property to VCAP_VCMODE_VIDEO and CapVideo property is true. Video capturing and recording starts with the setting contents of the VideoColorSpace property, VideoResolution property, VideoFrameRate property and VideoType property. This method is executed asynchronously. StatusUpdateEvent will notify the application that there is a change in the power status or a state change during video capturing and recording.
When the time specified in recordingTime has elapsed, or by calling the stopVideo method, recording is completed and the video file specified by fileName is recorded and can deliver to the application
Also, S_BUSY is set in the Status property during video capturing and recording. The place where video files are recorded is the area as the value of StorageTarget Property.

Errors		A UposException may be thrown when this method is invoked.
For further information, see “Errors" on page Intro-20.
Some possible values of the exception's ErrorCode property are:

Value	              Meaning     	       			          E_ILLEGAL	fileName is too long or contains characters that  				cannot be used, or 0 is specified for recordingTime. 			VideoCaptureMode property is not  					VCAP_VCMODE_VIDEO			                   E_EXISTS	fileName already exists. (If overwrite is false)	     E_BUSY	Cannot execute because it is recording.

See also		VideoColorSpace Property, VideoResolution Property, 					VideoFrameRate Property, VideoType Property, stopVideo Method, 			StatusUpdateEvent Event, VideoCaptureMode Property

stopVideo Method

Syntax		stopVideo ( ):void {raises-exception, use after open-claim-enable}

Remarks	The video capturing and recording process started by the startVideo method has been ended and the taking video is completed. This method processed synchronously. StatusUpdateEvent will notify the application that there is a change in the power status or a state change during taking video and recording.

Errors		A UposException may be thrown when this method is invoked. 				For further information, see “Errors" on page Intro-20.					Some possible values of the exception's ErrorCode property are:

Value		Meaning

E_ILLEGAL	It is not recorded.

See also		startVideo Method, StatusUpdateEvent Event

takePhoto Method

Syntax		takePhoto (fileName: string, overwrite: boolean, timeout: int32):				                                 void{raises-exception, use after open-claim-enable}

Parameter	Description

fileName	Specify the image file name to be recorded.				overwrite	Specify the behavior when the same name file exists. 					If true it overwrites. If false, UposException is thrown.

Remarks	Take photo and record with setting contents of PhotoColorSpace property, PhotoResolution property, PhotoFrameRate Property and PhotoType property. Before calling this method, it needs to set the VideoCaptureMode property to VCAP_VCMODE_PHOTO and this method can be executed if CapPhoto property is true. This method is executed synchronously. The location where photo files are recorded is the area managed by the associated “Hard Totals" service device.

Errors		A UposException may be thrown when this method is invoked.				For further information, see “Errors" on page Intro-20.					Some possible values of the exception's ErrorCode property are:

Value           	Meaning

E_ILLEGAL	One of the following occurred. 			  FileName is too long or contains unusable characters. VideoCaptureMode property is not                                                                                                                                                                                                                                                         VCAP_VCMODE_PHOTO and CapPhoto property is not true.

E_EXISTS	fileName already exist. (When overwrite=false)

See also	VideoCaptureMode Property, PhotoColorSpace Property,
PhotoResolution Property, CapPhoto Property, PhotoType Property, PhotoFrameRate Property, StatusUpdateEvent Event

Events (UML interfaces)

DirectIOEvent

<<event>>	upos::events::DirectIOEvent

EventNumber	: int32 {read-only}
Data			: int32 {read-write}
Obj			: object {read-write}

Description 	Provides Service information directly to the application. This event provides a means for a vendor-specific Video Capture Service to provide events to the application that are not otherwise supported by the device control.

Attributes	This event contains the following attributes:

Attribute	Type	Description

EventNumber	int32	Event number whose specific values are assigned by the Service.

Data	int32	Additional numeric data. Specific values vary by the EventNumber and the Service. This attribute is settable.

Obj	object	Additional data whose usage varies by the EventNumber and the Service. This attribute is settable.

Remarks	This event is to be used only for those types of vendor specific functions that are not otherwise described.

Use of this event may restrict the application program programform being used with other vendor’s devices which may not have any knowledge of the Service’s need for this event.

See Also	“Events" on page Intro-19, directIO method

ErrorEvent

<<event>>     upos::events::ErrorEvent

ErrorCode		 : int32 {read-only}
		      ErrorCodeExtended	 : int32 {read-only}
		      ErrorLocus		 : int32 {read-only}
		      ErrorResponse	 : int32 {read-write}

Description	Notifies the application that a Video Capture Device error has been detected and suitable response by the application is necessary to process the error condition.

Attributes	This event contains the following attributes:

Attributes	Type	Description

ErrorCode	int32	Error code causing the error event. 				See a list of Error Codes on page 20.

ErrorCodeExtended	int32	Extended Error code causing the error event.

If ErrorCode is E_EXTENDED, then see values below. Otherwise, it may contain a Service-specific value.

ErrorLocus	int32	Location of the error. If EL_OUTPUT is specified. An error occurred during asynchronous action.

ErrorResponse	int32	Pointer to the error event response. See ErrorResponse below for values.

The ErrorLocus attribute has one of the following values:

Value	Meaning

EL_OUTPUT	Error occurred while processing asynchronous output.

EL_INPUT	Error occurred while gathering or processing event-driven input. No previously buffered input data is available.

EL_INPUT_DATA	Error occurred while gathering or processing event-driven input, and some previously buffered data is available.

The application’s error event handler can set the ErrorResponse attribute to one of the following values:

Value	Meaning

ER_RETRY	Retry sending the data. The error state is exited.
May be valid for some input devices when the locus is EL_INPUT, in which case the input is re-tried, and the error state is exited. Typically, valid for asynchronous output devices when the locus is EL_OUTPUT, in which case the asynchronous output is re-tried, and the error state is exited. This is the default response when the locus is EL_OUTPUT.

ER_CLEAR	Valid for all loci: EL_INPUT, EL_INPUT_DATA, and EL_OUTPUT.  Clear all buffered input or output data (including all asynchronous output).  The error state is exited.  This is the default response when the locus is EL_INPUT.

ER_CONTINUEINPUT 
Only valid when the locus is EL_INPUT_DATA.
Acknowledges that a data error has occurred and directs the Device to continue input processing. The Device remains in the error state and will deliver additional DataEvents as directed by the DataEventEnabled property. When all input has been delivered and DataEventEnabled is again set to true, then another ErrorEvent is delivered with locus EL_INPUT.
This is the default response when the locus is EL_INPUT_DATA.

Remarks	This event is enqueued when an error is detected and the Device’s State transitions into the error state. Input error events are not delivered until DataEventEnabled is true, so that proper application sequencing occurs.

Unlike a DataEvent, the Device does not disable further DataEvents or input ErrorEvents; it leaves the DataEventEnabled property value at true.    Note that the application may set DataEventEnabled to false within its event handler if subsequent input events need to be disabled for a period of time.

See Also 	“Device Input Model" on page Intro-22, “Error Handling" on page Intro-23, “Device Output Models" on page Intro-25.

StatusUpdateEvent

<< event >>	upos::events::StatusUpdateEvent

Status		: int32 {read-only}

Description	Notifies the application that there is a change in the power status or a state change of the Video Capture device.

Attributes		This event contains the following attribute:

Attributes	Type	Description

Status                 int32	Indicates a change in the power status or a sate                                                     change of the unit.

Note that Release 1.3 added Power State Reporting with additional Power 	reporting StatusUpdateEvent values.

The Update Firmware capability added additional Status values for communicating the status/progress of an asynchronous update firmware process. See “StatusUpdateEvent” description on page 1-34.

Value            		Meaning

VCAP_SUE_START_VIDEO_RECORDING
						It will be notified when video recording starts.

VCAP_SUE_STOP_VIDEO_RECORDING
						It will be notified when video recording stops.

VCAP_SUE_START_PHOTO 											It will be notified when photo capturing starts.

VCAP_SUE_END_PHOTO 											It will be notified when photo capturing ends.

Remarks	      Enqueued when the Video Capture Device detects a power state change or a status change.

See Also 	“Events" on page Intro-19.

Chapter 40

Individual Recognition

This Chapter defines the Individual Recognition device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.16 | Open |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.16 | Open |
| CapPowerReporting: | int32 | {read-only} | 1.16 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.16 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.16 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.16 | open |
| CheckHealthText: | string | {read-only} | 1.16 | open |
| Claimed: | boolean | {read-only} | 1.16 | open |
| DataCount: | int32 | {read-only} | 1.16 | open |
| DataEventEnabled: | boolean | {read-write} | 1.16 | open |
| DeviceEnabled: | boolean | {read-write} | 1.16 | open & claim |
| FreezeEvents: | boolean | {read-write} | 1.16 | open |
| OutputID: | int32 | {read-only} | 1.16 | Not Supported |
| PowerNotify: | int32 | {read-write} | 1.16 | open |
| PowerState: | int32 | {read-only} | 1.16 | open |
| State: | int32 | {read-only} | 1.16 | -- |
| DeviceControlDescription: | string | {read-only} | 1.16 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.16 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.16 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.16 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.16 | open |
| PhysicalDeviceName: | string | {read-only} | 1.16 | open |

| Properties (Continued) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specific | Type | Mutability | Version | May Use After |
| CapIndividualList: | string | {read-only} | 1.16 | open |
| IndividualIDs: | string | {read-only} | 1.16 | open, claim & enable |
| IndividualRecognitionFilter: | string | {read-write} | 1.16 | open |
| IndividualRecognitionInformation | string | {read-only} | 1.16 | open |

| Methods (UML operations) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Common |  |  |  |  |  |
| Name |  |  | Version |  |  |
| open (logicalDeviceName: string): void {raises-exception} |  |  | 1.16 |  |  |
| close ( ): void {raises-exception, use after open} |  |  | 1.16 |  |  |
| claim (timeout: int32): void {raises-exception, use after open} |  |  | 1.16 |  |  |
| release ( ): void {raises-exception, use after open, claim} |  |  | 1.16 |  |  |
| checkHealth (level: int32): void {raises-exception, use after open, enable} |  |  | 1.16 |  |  |
| clearInput ( ): void { } |  |  | 1.16 |  |  |
| clearInputProperties ( ): void { } |  |  | 1.16 |  |  |
| clearOutput ( ): void { } |  |  | Not supported |  |  |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, enable} |  |  | 1.16 |  |  |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} |  |  | 1.16 |  |  |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} |  |  | 1.16 |  |  |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, enable} |  |  | 1.16 |  |  |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, enable} |  |  | 1.16 |  |  |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} |  |  | 1.16 |  |  |
| Events (UML interfaces) |  |  |  |  |
| Name | Type | Mutability |  | Version |
| upos::events::DataEvent |  |  |  | 1.16 |
| Status: | int32 | {read-only} |  |  |
| upos::events::DirectIOEvent |  |  |  | 1.16 |
| EventNumber: | int32 | {read-only} |  |  |
| Data: | int32 | {read-write} |  |  |
| Obj: | object | {read-write} |  |  |
| upos::events::ErrorEvent |  |  |  | 1.16 |
| ErrorCode: | int32 | {read-only} |  |  |
| ErrorCodeExtended: | int32 | {read-only} |  |  |
| ErrorLocus: | int32 | {read-only} |  |  |
| *pErrorResponse: | int32 | {read-write} |  |  |
| upos::events::OutputCompleteEvent |  | Not Supported |  | 1.16 |
| upos::events::StatusUpdateEvent |  |  |  | 1.16 |
| Status: | int32 | {read-only} |  |  |

General Information

The Individual Recognition programmatic name is “Individual Recognition”.

Capabilities

The Individual Recognition has the following set of capabilities:

Analyzes the image of the camera and recognizes individuals such as people and listed goods.

Individual Recognition Class Diagram

The following diagram shows the relationships between the Individual Recognition classes.

Fig. Chap.40-1 Individual Recognition Class Diagram

Model

The Individual Recognition follows the general “Device Input Model” for event-driven input:

Input Model

- When an individual is recognized by this device, a DataEvent is delivered to the application after the IndividualIDs property was set to indicate the recognized individuals.

- Identifiable individuals are indicated by the CapIndividualList property.

- Check the functions supported by the device, set validity / invalidity, etc. with the IndividualRecognitionInformation property.

- Recognized data is stored in the IndividualRecognitionInformation property, IndividualIDs property.

- How to recognize the individuals depends on the IndividualRecognitionFilter function therefore, please refer to the IndividualRecognitionFiler section.

- Other device behavior about this device supports the general device input model as listed below.

- If the AutoDisable property is true, then the device automatically disables itself when a DataEvent is enqueued.

- An enqueued DataEvent can be delivered to the application when the DataEventEnabled property is true and other event delivery requirements are met. Just before delivering this event, data is copied into corresponding properties, and further data events are disabled by setting DataEventEnabled to false. This causes subsequent input data to be enqueued while the application processes the current input and associated properties. When the application has finished processing the current input and is ready for more data, it reenables events by setting DataEventEnabled to true.

- An ErrorEvent (or events) is enqueued if an error occurs while gathering or processing input, and is delivered to the application when DataEventEnabled is true and other event delivery requirements are met.

- The DataCount property may be read to obtain the total number of enqueued DataEvents.

- All enqueued input may be deleted by calling clearInput method. See the clearInput method description for more details.

- All data properties that are populated as a result of firing a DataEvent or ErrorEvent can be set back to their default values by calling the clearInputProperties method.

- The application will be informed about any status change with a StatusUpdateEvent, also all corresponding status properties will be updated before event delivery.

Device Sharing

The Individual Recognition is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before the device begins reading input.

- See the “Summary” table for precise usage prerequisites.

IndividualRecognitionFilter

The IndividualRecognitionFilter property defines the following data as information for the individual recognition function of Individual Recognition Device.

･	Various support function existence or not.	
	(Supported functions are defined by the device)

･	Enable, disable status of various functions.

･	Types handled by various functions (examples: “male”, “female” of gender recognition)

･	Filter setting of various functions.

The following data is defined in the IndividualRecognitionInformation property

･	Individual Recognition input data

The device defines the individual recognition function information and the individual recognition input data.

The application refers to these contents to determine the support range and so on.

In addition, the application changes the enabled / disabled state of various functions, the filter setting, and controls each function.

The enabled / disabled state of the various functions set by the application, and the filter settings are applied by setting the DeviceEnabled property to true and enabling the individual recognition function.

When the application set various functions, it is possible to specify and set only the target ones.

The device fires a DataEvent based on the content set by the application and stores the input data in IndividualRecognitionInformation property.

IndividualRecognitionFilter Property Example Format

The IndividualRecognitionFilter property of the individual recognition device may define various information. Here is the example described by using the JSON format.

- Basic Items

| Key |  |  |  | Value | Value change capability | Explanation |
| --- | --- | --- | --- | --- | --- | --- |
| IndividualRecognitionFilter |  |  |  | object | N | Information for the various individual recognition. Target device define the supporting individual recognition object. |
|  | [IndividualRecognitionID] |  |  | object | N | Recognizable individual recognition information. Key name is the ID of recognized individual |
|  |  | Enabled |  | boolean | Y | Enable or disable state of target individual recognition. Application can control the target individual recognition by referring or changing. |
|  |  | Properties |  | object | N | Property information of the target individual recognition. Application control the target individual recognition by referring or changing the defined property value. |
|  |  |  | [Property01] | - | - |  |
|  |  |  | [Property02] | - | - |  |
|  |  | Filters |  | object | N | Input data filter setting information. Application filter the target individual recognition input data by changing the defined value. |
|  |  |  | [Filter01] | - | - |  |
|  |  |  | [Filter02] | - | - |  |

- Face Recognition device example

| Key |  |  |  |  |  | Value | Value change capability | Explanation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IndividualRecognitionFilter |  |  |  |  |  | object | N |  |
|  | Face |  |  |  |  | object | N |  |
|  |  | Enabled |  |  |  | boolean | Y |  |
|  |  | Properties |  |  |  | object | N |  |
|  |  |  | FaceImageNamePrefix |  |  | string | Y | Output image file prefix for face recognition |
|  |  |  | Gender |  |  | object | N | Information on gender recognition |
|  |  |  |  | Enabled |  | boolean | Y | Gender recognition enable, disable state |
|  |  |  |  | CapTypeList |  | array | N | Type list (“female”, “male”) |
|  |  |  | Age |  |  | object | N | Information on age recognition. |
|  |  |  |  | Enabled |  | boolean | Y | Age recognition enable, disable state |
|  |  |  | Facial Expression |  |  | object | N | Information on facial expression recognition |
|  |  |  |  | Enabled |  | boolean | Y | Facial expression recognition enable, disable state. |
|  |  |  |  | CapTypeList |  | array | N | Type list (“smile”, “angry”,…) |
|  |  |  | Gaze |  |  | object | N | Information on gaze recognition |
|  |  |  |  | Enabled |  | boolean | Y | Gaze recognition enable, disable state. |
|  |  |  |  | CapTypeList |  | array | N | Type list (“gaze”, “nogaze”) |
|  |  |  | Distance |  |  | object | N | Information on distance recognition |
|  |  |  |  | Enabled |  | boolean | Y | Distance recognition enable, disable state |
|  |  |  |  | CapTypeList |  | array | N | Type list (“near”, “far”, “very far”,…) |
|  |  |  |  | NearLength |  | number | Y | Distance to recognize as “near”. A recognition event is fired when a person is recognized in the range from 0 to Near Length. |
|  |  |  |  | FarLength |  | number | Y | Distance to recognize as “far”, “very far”. A recognition event is fired when a person is recognized in the range from Near Length to Far Length. A recognition event is fired when a person is recognized in the range more than Far Length. |
|  |  |  | Authentication |  |  | object | N | Information on face authentication |
|  |  |  |  | Enabled |  | boolean | Y | Face authentication enable, disable state. |
|  |  |  |  | ImageList |  | array | Y | Image file name list for comparison. Event is fired when it matches the image specified here. (Wild card can be specified) |
|  |  | Filters |  |  |  | object | N |  |
|  |  |  | Gender |  |  | object | N | Information on gender recognition filter. |
|  |  |  |  | TypeList |  | array | Y | Target Filter TypeList. Valid values are defined by CapTypeList. Recognition target is specified. To disable the filter, null should be assigned in its value. |
|  |  |  |  | Score |  | number | Y | Recognition score. Valid values are from 0 to 100. The range of the score specified here is the recognition target. To disable the filter, -1 should be assigned in its value. |
|  |  |  | Age |  |  | object | N | Information on age recognition. |
|  |  |  |  |  | Min | number | Y | Minimum age. The age below the specified is not a recognition target. To disable the filter -1 should be specified in its value. |
|  |  |  |  |  | Max | number | Y | Maximum age. The age above the specified is not a recognition target. To disable the filter -1 should be specified in its value. |
|  |  |  | Expression |  |  | object | N | Information on facial expression recognition filter. |
|  |  |  |  |  | TypeList | array | Y | Filter target type list. Valid values are defined in CapTypeList. Recognition target type is specified. To disable the filter null should be assigned in its value. |
|  |  |  |  |  | Score | number | Y | Recognition score. Valid values are from 0 to 100. The range of the score specified here is to be recognized. To disable the filter -1 should be assigned in its value. |
|  |  |  | Gaze |  |  | object | N | Information on gaze recognition filter |
|  |  |  |  |  | TypeList | array | Y | Filter target type list. Valid values are defined by CapTypeList. Recognition target is specified. To disable the filter, null should be assigned in its value. |
|  |  |  | Distance |  |  | object | N | Information on distance recognition filter |
|  |  |  |  |  | TypeList | array | Y | Filter target type list. Valid values are defined by CapTypeList. Recognition target is specified. To disable the filter, null should be assigned in its value. |

IndividualRecognition Information Property Example Format

IndividualRecognitionInformation property of individual recognition device may define various information and here is the example format described by JSON.

- Basic Items

| Key |  |  |  | Value | Value change capability | Explanation |
| --- | --- | --- | --- | --- | --- | --- |
| IndividualRecognitionInformation |  |  |  | object | N | Various Individual recognition input data. |
|  | [IndividualRecognitionID] |  |  | object | N | Store the input data of individual recognition. Key name is ID of individual recognition. |
|  |  | Properties |  | Array <object> | N | Input data list of target individual recognition. The content of the data is different for each device or function. |
|  |  |  | [Data01] | - | - |  |
|  |  |  | [Data02] | - | - |  |

- Face Recognition Device Example

| Key |  |  |  |  | Value | Value change capability | Explanation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IndividualRecognitionInformation |  |  |  |  | object | N |  |
|  | Face |  |  |  | object | N |  |
|  |  | DataLists |  |  | array <object> | N |  |
|  |  |  | FaceID |  | string | N | ID assigned to the recognized face |
|  |  |  | FaceImageName |  | string | N | Recognized face image file name |
|  |  |  | Gender |  | object | N | Recognized gender information |
|  |  |  |  | Type | string | N | Recognized type |
|  |  |  |  | Score | number | N | Confidence score of recognized type. |
|  |  |  | Age |  | object | N | Recognized age information |
|  |  |  |  | Age | number | N | Recognized age |
|  |  |  | Expression |  | object | N | Recognized facial expression information |
|  |  |  |  | Type | string | N | Recognized type. One of CapTypeList items is set. |
|  |  |  |  | Score | number | N | Confidence score of recognized type. |
|  |  |  | Gaze |  | object | N | A gaze list for each recognized face ID. |
|  |  |  |  | Type | string | N | Recognized type |
|  |  |  | Distance |  | object | N | Recognized distance information |
|  |  |  |  | Type | string | N | Recognized type. One of CapTypeList items is set. |
|  |  |  | Authentication |  | object | N | Authentication result information |
|  |  |  |  | ImageName | string | N | Matched image file name |

Properties (UML attributes)

CapIndividualList Property

Syntax	CapIndividualList: string {read-only, access after open}

Remarks	Recognizable Individual information is indicated by the list separated by a separator ",".

Each Individual information consists of the following information and is shown in the following order, separated with a colon (":").

Parameter	Meaning

IndividualID	An ID indicated an identifiable Individual

IndividualName	A Name of an Individual.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	IndividualIDs Property.

IndividualIDs Property

Syntax	IndividualIDs: string {read-only, access after open}

Remarks	Set the IndividualIDs recognizable Individual recognition device.

IndividualIDs values are indicated by separated with a colon (":").

Its value is set prior to a DataEvent being delivered to the application.

Errors			A UposException may be thrown when this property is accessed.
							For further information, see “Errors" on page Intro-20.

See Also	CapIndividualList Property

IndividualRecognitionFilter Property

Syntax	IndividualRecognitionFilter: string {read-write, access after open}

Remarks	Individual Recognition Function Information:

- Supporting the various functions (Refer to the Individual Recognition Filter Example Format written by JSON and supported function examples.).

- Various Valid / Invalid State functions.

- Various handled function types (e.g., "male" "female" in gender recognition, etc.).

- Various filter function settings.

All Individual Recognition function data information is defined by the device. By referring to these contents, the application can determine the supporting scope. Thereby, the application can control each function by changing the valid / invalid state and / or the various filter function settings.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

IndividualRecognitionInformation Property

Syntax	IndividualRecognitionInformation: string {read-only, access after open}

Remarks	Holds data indicating the following. Individual recognition input data. All Individual recognition input data is defined by the device.

Errors			A UposException may be thrown when this property is accessed.
							For further information, see “Errors" on page Intro-20.

Events (UML interfaces)

DataEvent

<<event>>	upos::events::DataEvent

Status			: int32{read-only}

Description 	Notifies the application when data from the Individual Recognition device is available to be read.

Attributes	This event contains the following attributes:

Attribute	Type		Description

Status	int32		Set to 0.

Remarks	Before this event is delivered, the individual recognition information is enqueued into the area that is indicated by the readValue method.  Since the stored individual recognition information might be managed by the associated “Hard Totals” device service, therefore, the application might also support the “Hard Totals” service.

See Also	VideoCaptureMode Property, “Events" on page Intro-19, readFrame Method , directIO method.

DirectIOEvent

<<event>>	upos::events::DirectIOEvent

EventNumber	: int32 {read-only}
Data			: int32 {read-write}
Obj			: object {read-write}

Description 	Provides Service information directly to the application. This event provides a means for a vendor-specific Individual Recognition Service to provide events to the application that are not otherwise supported by the device control.

Attributes	This event contains the following attributes:

Attribute	Type	Description

EventNumber	int32	Event number whose specific values are assigned by the Service.

Data	int32	Additional numeric data. Specific values vary by the EventNumber and the Service. This attribute is settable.

Obj	object	Additional data whose usage varies by the EventNumber and the Service. This attribute is settable.

Remarks	This event is to be used only for those types of vendor specific functions that are not otherwise described.

Use of this event may restrict the application program programform being used with other vendor’s devices which may not have any knowledge of the Service’s need for this event.

See Also	"Events" on page Intro-19, directIO method.

ErrorEvent

<<event>>	upos::events:: ErrorEvent

ErrorCode			: int32 {read-only}
ErrorCodeExtended	: int32 {read-only}
ErrorLocus			: int32 {read-only}
*pErrorResponse		: int32{read-write}

Description	Notifies the application that an Individual Recognition Device error has been detected and suitable response by the application is necessary to process the error condition.

Attributes	This event contains the following attributes:

Attributes	Type	Description

ErrorCode	int32	Error code causing the error event. 				See a list of Error Codes on page 20.

ErrorCodeExtended	int32	Extended Error code causing the error event.

If ErrorCode is E_EXTENDED, then see values below. Otherwise, it may contain a Service-specific value.

ErrorLocus	int32	Location of the error. If EL_OUTPUT is specified. An error occurred during asynchronous action.

ErrorResponse	int32	Pointer to the error event response. See ErrorResponse below for values.

The ErrorLocus attribute has one of the following values:

Value	Meaning

EL_OUTPUT	Error occurred while processing asynchronous output.

EL_INPUT	Error occurred while gathering or processing event-driven input. No previously buffered input data is available.

EL_INPUT_DATA	Error occurred while gathering or processing event-driven input, and some previously buffered data is available.

The application’s error event handler can set the ErrorResponse attribute to one of the following values:

Value	Meaning

ER_RETRY	Retry sending the data. The error state is exited.
May be valid for some input devices when the locus is EL_INPUT, in which case the input is re-tried, and the error state is exited. Typically, valid for asynchronous output devices when the locus is EL_OUTPUT, in which case the asynchronous output is re-tried, and the error state is exited. This is the default response when the locus is EL_OUTPUT.

ER_CLEAR	Valid for all loci: EL_INPUT, EL_INPUT_DATA, and EL_OUTPUT.  Clear all buffered input or output data (including all asynchronous output).  The error state is exited.  This is the default response when the locus is EL_INPUT.

ER_CONTINUEINPUT 
Only valid when the locus is EL_INPUT_DATA.
Acknowledges that a data error has occurred and directs the Device to continue input processing. The Device remains in the error state and will deliver additional DataEvents as directed by the DataEventEnabled property. When all input has been delivered and DataEventEnabled is again set to true, then another ErrorEvent is delivered with locus EL_INPUT.
This is the default response when the locus is EL_INPUT_DATA.

Remarks	This event is enqueued when an error is detected and the Device’s State transitions into the error state. Input error events are not delivered until DataEventEnabled is true, so that proper application sequencing occurs.

Unlike a DataEvent, the Device does not disable further DataEvents or input ErrorEvents; it leaves the DataEventEnabled property value at true.    Note that the application may set DataEventEnabled to false within its event handler if subsequent input events need to be disabled for a period of time.

See Also 	“Device Input Model" on page Intro-22, “Error Handling" on page Intro-23, “Device Output Models" on page Intro-25.

StatusUpdateEvent

<<event>>	upos::events:: StatusUpdateEvent
Status			: int32 {read-only}

Description	Notifies the application that there is a change in the power status or a  status

of the Individual Recognition device.

Attributes	This event contains the following attribute:

Attribute	Type	Description

Status	int32	Indicates a change in the power status of the unit.

Note that Release 1.3 added Power State Reporting with additional Power reporting StatusUpdateEvent values.

The Update Firmware capability added additional Status values

for	communicating the status/progress of an asynchronous update firmware

process. See “StatusUpdateEvent” description on page 1-34.

Remarks	Enqueued when the Individual Recognition Device detects a power state

change or a status change.

See Also 	“Events" on page Intro-19

Chapter 41

Sound Recorder

This Chapter defines the Sound Recorder device category.

Summary

| Properties(UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.16 | open |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.16 | open |
| CapPowerReporting: | int32 | {read-only} | 1.16 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.16 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.16 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.16 | open |
| CheckHealthText: | string | {read-only} | 1.16 | open |
| Claimed: | boolean | {read-only} | 1.16 | open |
| DataCount: | int32 | {read-only} | 1.16 | open |
| DataEventEnabled: | boolean | {read-write} | 1.16 | open |
| DeviceEnabled: | boolean | {read-write} | 1.16 | open & claim |
| FreezeEvents: | boolean | {read-write} | 1.16 | open |
| OutputID: | int32 | {read-only} | 1.16 | Not Supported |
| PowerNotify: | int32 | {read-write} | 1.16 | open |
| PowerState: | int32 | {read-only} | 1.16 | open |
| State: | int32 | {read-only} | 1.16 | open |
| DeviceControlDescription: | string | {read-only} | 1.16 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.16 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.16 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.16 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.16 | open |
| PhysicalDeviceName: | string | {read-only} | 1.16 | open |

| Properties (Continued) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specific | Type | Mutability | Version | May Use After |
| CapAssociatedHardTotalsDevice: | string | {read-only} | 1.16 | open |
| CapChannel: | boolean | {read-only} | 1.16 | open |
| CapSamplingRate: | boolean | {read-only} | 1.16 | open |
| CapSoundType: | boolean | {read-only} | 1.16 | open |
| CapStorageTarget | boolean | {read-only} | 1.16 | open |
| CapRecordingLevel: | boolean | {read-only} | 1.16 | open |
| Channel: | string | {read-write} | 1.16 | open, claim & enable |
| ChannelList: | string | {read-only} | 1.16 | open |
| RecordingLevel: | int32 | {read-write} | 1.16 | open, claim & enable |
| RemainingRecordingTimeInSec: | int32 | {read-only} | 1.16 | open, claim & enable |
| SamplingRate: | string | {read-write} | 1.16 | open, claim & enable |
| SamplingRateList: | string | {read-only} | 1.16 | open |
| SoundData: | binary | {read-only} | 1.16 | open |
| SoundType: | string | {read-write} | 1.16 | open, claim & enable |
| SoundTypeList: | string | {read-only} | 1.16 | open |
| StorageTarget | int32 | {read-write} | 1.16 | open, claim & enable |

| Methods(UML operations) |  |
| --- | --- |
| Common |  |
| Name | Version |
| open (logicalDeviceName: string): void {raises-exception} | 1.16 |
| close ( ): void {raises-exception, use after open} | 1.16 |
| claim (timeout: int32): void {raises-exception, use after open} | 1.16 |
| release ( ): void {raises-exception, use after open, claim} | 1.16 |
| checkHealth (level: int32): void {raises-exception, use after open, enable} | 1.16 |
| clearInput ( ): void { } | 1.16 |
| Me Methods (UML operations)(continued) |  |
| Common |  |
| Name | Version |
| clearInputProperties ( ): void { } | 1.16 |
| clearOutput ( ): void { } | Not supported |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, enable} | 1.16 |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} | 1.16 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.16 |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.16 |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, enable} | 1.16 |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.16 |
| Specific |  |
| Name | Version |
| startRecording (FileName: string, OverWrite: boolean, RecordingTime:int32): void {raises-exception, use after open, claim, enable} | 1.16 |
| stopRecording ( ): Void {raises-exception, use after open, claim, enable} | 1.16 |

| Events (UML interfaces) |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Mutability | Version |
| upos::events::DataEvent |  |  | 1.16 |
| Status: | int32 | {read-only} |  |
| upos::events::DirectIOEvent |  |  | 1.16 |
| EventNumber: | int32 | {read-only} |  |
| Data: | int32 | {read-write} |  |
| Obj: | object | {read-write} |  |
| upos::events::ErrorEvent |  |  | 1.16 |
| ErrorCode: | int32 | {read-only} |  |
| ErrorCodeExtended: | int32 | {read-only} |  |
| ErrorLocus: | int32 | {read-only} |  |
| *pErrorResponse: | int32 | {read-write} |  |
| upos::events::OutputCompleteEvent |  | Not Supported | 1.16 |
| upos::events::StatusUpdateEvent |  |  | 1.16 |
| Status: | int32 | {read-only} |  |
| upos::events::TransitionEvent |  | Not Supported | 1.16 |

General Information

The Sound Recorder programmatic name is "Sound Recorder".

Capabilities

The Sound Recorder has the following capability:

Record the real-time audio to a file, deliver the recorded sound data to the property that application may read and / or retrieve, and save the recorded sound data file to device memory and / or other storage devices.

Sound Recorder Class Diagram

The following diagram shows the relationships between the Sound Recorder classes.

Fig. Chap. 41-1 Sound Recorder Class Diagram

Model

Sound Recorder Control follows a general “Device Input Model” in a broad sense. One point of difference is that the Sound Recorder device required the execution of methods to start and stop the sound recording process and creates a sound data file in real time, deliver the data to the property and save the file in device and / or associated storage device.

The Sound Recorder Model defines the following behavior: Sound Recorder device controls the Sound Recorder device to set the input (recording) conditions, specifies the start / end of input data acquisition by the method. And makes the sound data file in real time from the acquired audio and delivers the data to the appropriate property. At the same time, saves the recorded data file in device and /or associated storage devices.

“Sound Recorder” device control starts recording with the startRecording method.   Prior to execute the startRecording method each value setting of Channel property, SamplingRate property, and RecordingLevel property are required, if each of CapChannel property CapSamplingRate property is true. And also need to set the DataEventEnabled property to true. At the same time, the recording format setting starts with the SoundType property value, if CapSoundType property is true.

The recording ends after the specified time has elapsed or when stopRecording method is called or when clearInput method is called. The generated sound data file will be recorded for either the host file or the HardTotals device or both, after the end of recording. And generated sound data will be delivered to the SoundData property. Just after the delivery of sound data to the property, when the DataEventEnabled property is true, the DataEvent is enqueued and delivered to the application.

If the AutoDisable property is true, the device will automatically disable itself after the DataEvent is enqueued.

The remaining recording time in seconds can be obtained from the property RemainingRecordingTimeInSec

StatusUpdateEvent with status SERC_SUE_START_SOUND_RECORDING is evoked when startRecording method is executed to notify the application that recording state with has started.

When the sound recording is finished, if the specified time of startRecording method has elapsed or stopRecording method has been called, a StatusUpdateEvent with status SERC_SUE_STOP_SOUND_RECORDING is evoked to notify the application that recording has been stopped.

An enqueued DataEvent can be delivered to the application when the DataEventEnabled property is true and other event delivery requirements are met. Just before enqueuing this event, the device provides the recorded data to the SoundData property, and disables further data events by setting the DataEventEnabled property to false. This causes subsequent input data to be buffered by the device while the application processes the current input and associated properties. When the application has finished processing the current input and is ready for more data, it re-enables events by setting DataEventEnabled to true.

If ErrorEvent response is ER_CONTINUEINPUT, the process of input processing continues. However, as long as the cause of the error is not resolved, the ErrorEvent will occur again immediately.

If ErrorEvent is ER_CLEAR, the input processing process is terminated, and the record is discarded.

If the time specified by the startRecording method is FOREVER (-1), execution will       continue until the stopRecording method is called in the application. When stopRecording is called, the previous recording data is recorded to the host file, the HardTotals device, or both, with the specified file name, and the sound data will be delivered to the SoundData property. When DataEventEnabled property is true, the DataEvent is enqueued and delivered to the application.

Only one call to startRecording method can be in progress at a time. An attempt to nest sound recorder operations will result in an UposException being thrown.

If Error occurs during the execution of the startRecording method application should call the stopRecording method to terminate the recording process or cancel the recording process by calling the clearInput method before ending the ErrorEvent processing. After this when the stopRecording method is called, the recording data until just before the ErrorEvent occurs is recorded to the host file, the HardTotals device, or both. When DataEventEnabled property is true, the DataEvent is enqueued and delivered to the application.

If there is no Error during the execution of startRecording method can terminate the recording process and can stop the recording at any time. When the stopRecording method is called, the recording data until just before the method call is recorded to the host file, the HardTotals or both. When DataEventEnabled property is true, the DataEvent is enqueued and delivered to the application.

All input data enqueued by the device may be deleted by calling the clearInput method．All data properties that are populated as a result of a DataEvent or Error Event can be set back to their default values by calling the clearInputProperties method.

The device may have the ability to write encoded sound data files to either the HardTotals devices or the host file system, or both, and the CapStorageTarget property will show the device’s data storage location capability.

If device supports either or both HardTotals devices and the host file system, the application should set the StorageTarget property accordingly to tell where to write the encoded sound data file.

If device needs to be able to write the encoded sound data to an associated HardTotals device, the CapAssociatedHardTotalsDevice property holds the open name of the associated HardTotals device.

Device Sharing

The Sound Recorder is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before accessing some properties or calling methods that update the device.

- See the “Summary” table for precise usage prerequisites.

- The image display mode of the graphics device control is as follows.

Properties(UML attributes)

CapAssociatedHardTotalsDevice Property

Syntax	CapAssociatedHardTotalsDevice : string {read-write, access after open}

Remarks	Holds the open name of the associated HardTotals device if the device is able to write to such devices which is the case if CapStorageTarget is either CST_ALL or CST_HARDTOTALS_ONLY. If CapStorageTarget is CST_HOST_ONLY this property value must be the empty string.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	CapStorageTarget Property

CapChannel Property

Syntax	CapChannel: boolean{read-only, access after open}

Remarks	If true, the application can change the channel.
	If false, the application cannot change the channel.
This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	Channel Property

CapSamplingRate Property

Syntax	CapSamplingRate: boolean {read-only, access after open}

Remarks	If true, the application can change the sampling rate.
If false, the application cannot change the sampling rate.
This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed.
For further information, see “Errors" on page Intro-20.

See Also	SamplingRate Property.

CapSoundType Property

Syntax	CapSoundType: boolean {read-only, access after open}

Remarks	If true, the application can change the sound file type.
If false, the application cannot change the sound file type.
This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	SoundType Property.

CapStorageTarget Property

Syntax	CapStorageTarget: int32 {read-only, access after open}

Remarks	This is an enumeration and announces where the device is able to write the recorded sound data file to.
It holds one of the following values.

Value	Meaning

CST_HARDTOTAL_ONLY	Only an associate HardTotals device is supported.

CST_HOST_ONLY	Only the host’s file system is supported.

CST_ALL	Both, the associated HardTotals device and the host’s file system is supported.

This property is initialized by the open method.

If a HardTotals device is supported the StorageTarget the property value should be CST_HARDTOTAL_ONLY or CST_ALL, and the property CapAssociatedHardTotalsDevice holds the open name of the associated HardTotals device.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	StorageTarget Property, CapAssociatedHardTotalsDevice Property

CapRecordingLevel Property

Syntax	CapRecordingLevel: boolean {read-only, access after open}

Remarks	If true, the application can change the recording level.
If false, the application cannot change the recording level.
This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	CapRecordingLevel Property.

Channel Property

Syntax	Channel : string {read-write, access after open-claim-enable}

Remarks	Holds the channel during recording.
Valid values are one of the values listed in the ChannelList property.
This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	CapChannel Property, ChannelList Property

ChannelList Property

Syntax	ChannelList : string {read only, access after open}

Remarks	Contains the comma-delimited list of channels that is supported by the device.

For example, if the device only supports channel1and channel2 and channel4, then this property should be set to "1,2,4".
This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	Channel Property.

RecordingLevel Property

Syntax	RecordingLevel : int32 {read-write, access after open- claim-enable}

Remarks	Holds the recording level during recording.
Legal values range from zero through 100.
This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	CapRecordingLevel Property

RemainingRecordingTimeInSec Property

Syntax	RemainingRceordingTimeInSec: 				                                      int32 {read-only, access after open-claim-enable}

Remarks	This property holds the remaining recording time in seconds if a recording is ongoing. If no recording is ongoing its value is 0. When a call to method startRecording returns, this property initially holds the time passed as argument recordingTime to that call. If this argument value is FOREVER, this property also holds this value unchanged until stopRecording has been called.

This property is initialized during device setDeviceEnbaled method to 0.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	startRecording Method, stopRecording Method

SamplingRate Property

Syntax	SamplingRate : string{read-write, access after open-claim-enable}

Remarks	Holds the sampling rate during recording.
Valid values are one of the values listed in the SamplingRateList property.
This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	CapSamplingRate Property, SamplingRateList Property

SamplingRateList Property

Syntax	SamplingRateList : string {read only, access after open}

Remarks		Contains the comma-delimited list of sampling rate that are supported by the device.
For example, if the device only supports 44.1kHz and 48kHz and 96kHz, then this property should be set to "44100,48000,96000".
This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	SamplingRate Property.

SoundData Property

Syntax	　SoundData: binary { read-only, access after open }

Remarks　	This property is used to store the sound data after the recording time elapse of startRecording method or stopRecording method is called. If no recorded sound data was available, the SoundData property will be set to zero length (or empty). Its value is set prior to a DataEvent to be enqueued. This property is initialized to zero length by the open method.

Errors	A UposException may be thrown when this property is accessed. 	       For further information, see “Errors" on page Intro-21.

See Also     startRecording Method, stopRecording Method, DataEvent.

SoundType Property

Syntax	SoundType : string {read-write, access after open, claim}

Remarks	Holds the audio file format to be recorded.
Valid values are one of the values listed in the CapSoundTypeList property.
This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	CapSoundType Property, CapSoundTypeList Property.

SoundTypeList Property

Syntax	SoundTypeList : string{read only, access after open}

Remarks		Contains the comma-delimited list of sound file type that is supported by the device.
For example, if the device only supports WAV and OGG, then this property should be set to "WAV,OGG".
This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed.
For further information, see “Errors" on page Intro-20.

See Also	SoundType Property.

StorageTarget Property

Syntax	StorageTarget: int32 {read-write, access after open-claim-enable}

Remarks	This is an enumeration and defines where the device writes the recorded sound data file to. Should be set before a call to startRecording.
It holds one of the following values.

Value                                 Meaning

ST_HARDTOTAL	The encoded data file is written to the associated  HardTotals device. The property CapAssociatedHardTotalsDevice holds the open name of the associated HardTotals device.

ST_HOST              	The encoded data file is written to the host’s file system.

ST_HOST&HARDTOTAL

The encoded data file is written to the associated HardTotals device and host’s file system. The property CapAssociatedHardTotalsDevice holds the open name of the associated HardTotals device.

This property is initialized by the open method according to the value hold by CapStorageTarget. If CapStorageTarget has the value CST_ALL it is initialized to ST_HOST.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

Value	Meaning

E_ILLEGAL	An invalid value was specified, or recording is ongoing.

See Also	CapStorageTarget Property, CapAssociatedHardTotalsDevice Property

Methods(UML operations)

startRecording Method

Syntax	startRecording (fileName: string, overWrite: boolean, 	recordingTime : int32):
		       void{raises-exception, use after open-claim-enable}

Parameter	Description

fileName	Specify the file name of the sound to be recorded.

overWrite	Specify the behavior when the same name file exists. If it is true it will be overwritten and if false it will raise the UPOSException.

recordingTime	Specify the time for recording in seconds. If OPOS_FOREVER (-1) is specified, recording will continue until the stopRecording method is called.

Remarks	Sound recording starts with the settings of the Channel property, SamplingRate property, and RecordingLevel property and need to set DataEventEnabled property to true. At the same time, recording format setting starts with the SoundType property. When this method is called, if specified recording time is elapsed, recording process will be ended and recorded sound data is provided at the SoundData property that the application may read it and / or process the stored sound data file given as filename argument. When the DataEventEnabled property is true, the DataEvent is enqueued and delivered to the application. StatusUpdateEvent with state SREC_SUE_START_SOUND_RECORDING is evoked when startRecording method is executed to notify the application, the recording has started. When the sound recording is finished, if the specified time of startRecording method has elapsed or stopRecording method has been called, the value of StatusUpDateEvent with state SREC_SUE_STOP_SOUND_RECORDING is evoked to notify the application, the recording has stopped

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	FileName is too long or contains characters that cannot be used, or 0 is specified for RecordingTime.

E_EXISTS	FileName already exists. (When OverWrite is FALSE)

E_BUSY	It cannot be executed as it is recording.

See Also	Channel Property, SamplingRate Property, SoundData Property, SoundType Property, RecordingLevel Property, stopRecording Method, StatusUpdateEvent Event

stopRecording Method

Syntax	stopRecording ():
			void {raises-exception, use after open-claim-enable}

Remarks	When this method is called the sound recording process that started by startRecording method is ended and the recording is finished. This method is processed synchronously. After recording and decoding process has been finished, the recorded sound data will be provided at the SoundData property prior to the Data Event is enqueued, when DataEventEnabled property is true. When stopRecording method is called, a StatusUpdateEvent with status SREC_SUE_STOP_SOUND_RECORDING is evoked to notify the application, the recording has stopped.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20 Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	It is not recorded.

See Also	StartRecording Property, SoundData Property, StatusUpdateEvent. Event

Events(UML interfaces)

DataEvent

<<event>>	upos::events::DataEvent

Status			:int32{read-only}

Description 	Notifies the application when data from the Sound Recorder  device is available to be read.

Attributes	This event contains the following attributes:

Attribute	Type		Description

Status	int32		Set to 0.

Remarks	Before this event is delivered, the Sound Recorder information is enqueued into the area that is indicated by the startRecording method.  Since the stored sound recorder device information might be managed by the associated “Hard Totals” device service, therefore, the application might also support the “Hard Totals” service.

See Also        Channel Property, SamplingRate Property, SoundType property, RecordingLevel Property, stopRecording Method, startRecording Method

DirectIOEvent

<<event>>	upos::events::DirectIOEvent

EventNumber	: int32 {read-only}
Data			: int32 {read-write}
Obj			: object {read-write}

Description 	Provides Service information directly to the application. This event provides a means for a vendor-specific Individual Recognition Service to provide events to the application that are not otherwise supported by the device control.

Attributes	This event contains the following attributes:

Attribute	Type	Description

EventNumber	int32	Event number whose specific values are assigned by the Service.

Data	int32	Additional numeric data. Specific values vary by the EventNumber and the Service. This attribute is settable.

Obj	object	Additional data whose usage varies by the EventNumber and the Service. This attribute is settable.

Remarks	This event is to be used only for those types of vendor specific functions that are not otherwise described.

Use of this event may restrict the application program programform being used with other vendor’s devices which may not have any knowledge of the Service’s need for this event.

See Also	"Events" on page Intro-19, directIO method.

ErrorEvent

<<event>> 	upos::events:: ErrorEvent
ErrorCode			: int32{read-write}
ErrorCodeExtended	: int32{read-write}
ErrorLocus 		: int32{read-write}
*pErrorResponse		: int32{read-write}

Description	Notifies the application that a Sound Recorder Device error has been detected and suitable response by the application is necessary to process the error condition.

Attributes  This event contains following attributes.

Attributes	Type	 Description

Error Code	int32	Error Code causing the error event. 			See the list of Error Code.

ErrortCodeExtended	int32	Error Code causing the error event. 			These values are device category    			specific.

ErrorLocus	int32	Location of the error. See values below.

pErrorResponse	int32	Error response, whose default value may 		             	be overridden by the application     			(i.e., this attribute is settable). 				See values below.

The ErrorLocus attribute has one of the following values:

Value	Meaning

EL_INPUT 	Error occurred while gathering or Processing event-driven input. No previously buffered input data is available.

EL_INPUT_DATA	Error occurred while gathering or processing

event-driven input, and some previously buffered

data is available.

If ResultCode is E_EXTENDED, ResultCodeExtended is set to one of the following values.

Value	Meaning

ETOT_NOROOM	There is not enough space to create the file.

The application’s error event handler can set the ErrorResponse attribute to one of the following values:

Value	 	Meaning

ER_CLEAR	I will try its asynchronous output again.

The error condition is exited.

ER_CONTINUEINPUT

Only valid when the locus is EL_INPUT_DATA.

Acknowledges that a data error has occurred and directs the Device to continue input processing. The Device remains in the error state and will deliver additional DataEvents as directed by the DataEventEnabled property. When all input has been delivered and DataEventEnabled is again set to true, then another ErrorEvent is delivered with locus EL_INPUT. This is the default response when the locus is	 EL_INPUT_DATA.

Remarks  	   It notifies you when an error is detected during recording. Input error events are    	not  delivered until DataEventEnabled is true, so that proper application  	sequencing occurs.

See Also	Status, Error code, State model

StatusUpdateEvent

<<event>>	upos::events:: StatusUpdateEvent
Status			: int32 {read-only}

Description	Notifies the application that there is a change in the power status or a status  of the Sound Recorder device.

Attributes		This event contains the following attribute:

Attributes	Type	Description

Status                   int32	Indicates a change in the power status or a status of the unit.

Note that Release 1.3 added Power State Reporting with additional Power 	reporting StatusUpdateEvent values.

The Update Firmware capability added additional Status values for communicating the status/progress of an asynchronous update firmware process. See “StatusUpdateEvent” description on page 1-34.

Value            		Meaning

SREC_SUE_START_SOUND_RECORDING									It will be notified when sound recording starts.

SREC_SUE_STOP_SOUND_RECORDING									It will be notified when sound recording stops.

Remarks	      Enqueued when the Sound Recorder Device detects a power state change or a status change.

See Also 	“Events" on page Intro-19.

Chapter 42

Voice Recognition

This Chapter defines the Voice Recognition device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.16 | open |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.16 | open |
| CapPowerReporting: | int32 | {read-only} | 1.16 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.16 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.16 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.16 | open |
| CheckHealthText: | string | {read-only} | 1.16 | open |
| Claimed: | boolean | {read-only} | 1.16 | open |
| DataCount: | int32 | {read-only} | 1.16 | open |
| DataEventEnabled: | boolean | {read-write} | 1.16 | open |
| DeviceEnabled: | boolean | {read-write} | 1.16 | open & claim |
| FreezeEvents: | boolean | {read-write} | 1.16 | open |
| OutputID: | int32 | {read-only} | 1.16 | Not Supported |
| PowerNotify: | int32 | {read-write} | 1.16 | open |
| PowerState: | int32 | {read-only} | 1.16 | open |
| State: | int32 | {read-only} | 1.16 | -- |
| DeviceControlDescription: | string | {read-only} | 1.16 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.16 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.16 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.16 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.16 | open |
| PhysicalDeviceName: | string | {read-only} | 1.16 | open |

| Properties (Continued) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Specific | Type | Mutability | Version | May Use After |  |
| CapLanguage: | boolean | {read-only} | 1.16 | open |  |
| HearingDataPattern: | string | {read-only} | 1.16 | open, claim & enable |  |
| HearingDataWord: | string | {read-only} | 1.16 | open, claim & enable |  |
| HearingDataWordList: | string | {read-only} | 1.16 | open, claim & enable |  |
| HearingResult: | int32 | {read-only} | 1.16 | open, claim & enable |  |
| HearingStatus: | int32 | {read-only} | 1.16 | open, claim & enable |  |
| LanguageList: | string | {read-only} | 1.16 | open |  |
| Me Methods (UML operations) |  |  |  |  |  |
| Common |  |  |  |  |  |
| Name |  |  |  |  | Version |
| open (logicalDeviceName: string): void {raises-exception} |  |  |  |  | 1.16 |
| close ( ): void {raises-exception, use after open} |  |  |  |  | 1.16 |
| claim ( timeout: int32 ): void {raises-exception, use after open} |  |  |  |  | 1.16 |
| release ( ): void {raises-exception, use after open, claim} |  |  |  |  | 1.16 |
| checkHealth ( level: int32 ): void {raises-exception, use after open, enable} |  |  |  |  | 1.16 |
| clearInput ( ): void { } |  |  |  |  | 1.16 |
| clearInputProperties ( ): void { } |  |  |  |  | 1.16 |
| clearOutput ( ): void { } |  |  |  |  | Not supported |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} |  |  |  |  | 1.16 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| retrieveStatistics ( inout statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| Me Methods (UML operations)(continued) |  |  |  |  |  |
| Common |  |  |  |  |  |
| Name |  |  |  |  | Version |
| updateFirmware ( firmwareFileName: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| updateStatistics ( statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| Specific |  |  |  |  |  |
| Name |  |  |  |  |  |
| startHearingFree (language: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| startHearingSentence (language: string, wordList: string, patternList: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| startHearingWord (language: string, wordList: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| startHearingYesNo (language: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| stopHearing ( ): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |

| Events (UML interfaces) |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Mutability | Version |
| upos::events::DataEvent |  |  | 1.16 |
| Status: | int32 | {read-only} |  |
| upos::events::DirectIOEvent |  |  | 1.16 |
| EventNumber: | int32 | {read-only} |  |
| Data: | int32 | {read-write} |  |
| Obj: | object | {read-write} |  |
| upos::events::ErrorEvent |  |  | 1.16 |
| ErrorCode: | int32 | {read-only} |  |
| ErrorCodeExtended: | int32 | {read-only} |  |
| ErrorLocus: | int32 | {read-only} |  |
| *pErrorResponse: | int32 | {read-write} |  |
| upos::events::OutputCompleteEvent |  | Not Supported |  |
| upos::events::StatusUpdateEvent |  |  | 1.16 |
| Status: | int32 | {read-only} |  |

General Information

The Voice Recognition programmatic name is "Voice Recognition".

Capabilities

The Voice Recognition has the following capability:

- Convert spoken words to strings.

Voice Recognition Class Diagram

The following diagram shows the relationships between the Voice Recognition classes.

Fig. Chap. 42-1 Voice Recognition Class Diagram

Model

The Voice Recognition follows the general “Device Input Model” for event-driven input:

Device control starts voice recognition with the startHearingYesNo method, startHearingSentence method, etc., and generates DataEvent when recognizing voice.

If the AutoDisable property is true, then the device automatically disables itself when a DataEvent is enqueued.

An enqueued DataEvent can be delivered to the application when the DataEventEnabled property is true and other event delivery requirements are met. Just before delivering this event, data is copied into corresponding properties, and further data events are disabled by setting DataEventEnabled to false. This causes subsequent input data to be enqueued while the application processes the current input and associated properties. When the application has finished processing the current input and is ready for more data, it reenables events by setting DataEventEnabled to true.

An ErrorEvent (or events) is enqueued if an error occurs while gathering or processing input, and is delivered to the application when DataEventEnabled is true and other event delivery requirements are met.

The DataCount property may be read to obtain the total number of enqueued DataEvents.

All enqueued input may be deleted by calling clearInput method. See the clearInput method description for more details.

All data properties that are populated as a result of firing a DataEvent or ErrorEvent can be set back to their default values by calling the clearInputProperties method.

The application will be informed about any status change with a StatusUpdateEvent, also all corresponding status properties will be updated before event delivery.

Types of voice recognition

Voice recognition is mainly a method of specifying word candidates to be recognized and waiting for those words.

There are the following four types of voice recognition.

Yes/No/Cancel recognition

It listens to the sound of words classified as Yes / No / Cancel defined by the device.

For example, the voice ""OK."" is classified as Yes.

The recognized content is set in the HearingDataWord property.

For details, refer to the startHearingYesNo method.

Word recognition

The application specifies a list of words and listens for the voice of that word.

The recognized content is set in the HearingDataWord property.

For details, refer to the startHearingWord method.

Sentence recognition

The application specifies a word and a list of patterns of the sentences using it and awaits the sound of the sentence.

The recognized content is set in the HearingDataWordList property, HearingDataPattern property.

For details, see the startHearingSentence method.

Free recognition

Voice recognition leave to the device is performed without specifying the word to wait.

It does not specify waiting words and performs voice recognition entrusted to the device.

The recognized content is set in the HearingDataWord property.

For details, see the startHearingFree method.

When recognizing voice, the kind of recognition was stored in the HearingResult property.

Device Sharing

The Voice Recognition is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before accessing some properties or calling methods that update the device.

- See the “Summary” table for precise usage prerequisites.

Properties (UML attributes)

CapLanguage Property

Syntax	CapLanguage: boolean {read-only, access after open}

Remarks	If true, the application can change the language. If false, the application cannot change the language.
This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

HearingDataPattern Property

Syntax	HearingDataPattern: string {read-only, access after open-claim-enable}

Remarks	The pattern ID recognized by the startHearingSentence method is set.
This property is set by the device control just before the DataEvent is enqueued.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	startHearingSentence Method

HearingDataWord Property

Syntax	HearingDataWord: string {read-only, access after open-claim-enable}

Remarks	The content of voice recognition is set.
This property is set as input data of the following method. To know which method it is for, check the HearingResult property.

Methods	Meaning

startHearingYesNo Method

The recognized word is set.

startHearingWord Method

Recognized words are set among the word candidates

specified by the startHearingWord method.

startHearingFree Method

Recognized words and sentences are set.

The alphabet 's uppercase letters, Japanese kanji,

hiragana, katakana, etc., the contents to be set varies

depending on the device.

This property is set by the device control just before the DataEvent is enqueued..

Errors	A UposException may be thrown when this property is accessed.          	      For further information, see “Errors" on page Intro-20.

See Also	HearingResult Property, startHearingYesNo Method,

startHearingWord Method, startHearingFree Method

HearingDataWordList Property

Syntax	HearingDataWordList: string {read-only, access after open-claim-enable}

Remarks	Comma-separated list of word information recognized by the startHearingSentence method.
Each word information consists of the following information and is shown in the following order separated by a colon (":").

Parameter	Description

WordGoupID	Recognized word group ID

Word	Recognized words. The content defined in the word

group is set.

For example, in the startHearingSentence method, set candidates as follows, Word list: "item:coffee:tea, count:a:two:three"
Pattern list: "P1:[count] cup of [item], P2:[item]"
startHearingSentence ("en-US", "item:coffee:tea, count:a:two", "P1:[count] cup of [item],P2:[item]")

If you speak "Give me two cups of coffee", device recognize “Pattern” as "P1" and “WordList” as "item:coffee, count:two".

The properties are set as follows,
HearingDataPattern="P1";
HearingDataWordList="item:coffee, count:two";

This property is set by the device control just before the DataEvent is notified.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	startHearingSentence Method

HearingResult Property

Syntax	HearingResult: int32 {read-only, access after open-claim-enable}

Remarks	A value indicating the voice recognition result is set.
The parameters to be set are as follows.

Value	Meaning

VRCG_HRESULT_YESNO_YES

Voice recognition result of StartHearingYesNo

method. Also, Device got an answer that is classified as YES. The recognized content is set in the HearingDataWord property.

VRCG_HRESULT_YESNO_NO

Voice recognition result of startHearingYesNo

method. Also, Device got an answer that

is classified as NO. The recognition content is set in

the HearingDataWord property.

VRCG_HRESULT_YESNO_CANCEL

Voice recognition result of startHearingYesNo

method. Also, Device got responses that are classified

as CANCEL. The recognition content is set in the

HearingDataWord property.

VRCG_HRESULT_WORD

Recognition result of startHearingWord method.  The recognition content is set in the HearingDataWord property.

VRCG_HRESULT_SENTENCE

Recognition result of startHearingSentence method.

The recognition content is set in the

HearingDataWordList property and

HearingDataPattern property.

VRCG_HRESULT_FREE

Recognition result of startHearingFree method. The

recognition content is set in the HearingDataWord

property.

This property is set by the device control just before the DataEvent is enqueued.

Errors	A UposException may be thrown when this property is accessed.
For further information, see “Errors" on page Intro-20.

See Also	HearingDataWord Property, HearingDataWordList Property, HearingDataPattern Property, startHearingYesNo Method, startHearingWord Method, startHearingSentence Method, startHearingFree Method.

HearingStatus Property

Syntax	HearingStatus: int32 {read-only, access after open-claim-enable}

Remarks	A value indicating the voice recognition status is set.

Value		Meaning

VRCG_HSTATUS_NONE	Voice recognition is not running.

VRCG_HSTATUS_YESNO	Voice recognition by the startHearingYesNo  	method is in progress.

VRCG_HSTATUS_WORD	Voice recognition by the 	startHearingWord 	method is in progress.

VRCG_HSTATUS_SENTENCE

Voice recognition by the 	startHearingSentence 	method is in progress.

VRCG_HSTATUS_FREE	Voice recognition by the startHearingFree

method is in progress.

This property is initialized by the open method. Also, it is set by the device control just before the voice recognition state changes.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	startHearingYesNo Method, startHearingWord Method, startHearingSentence Method, startHearingFree Method

LanguageList Property

Syntax	LanguageList: string {read-only, access after open}

Remarks	Contains the comma-delimited list of language that are supported by the device.
The value representing the language is a value consisting of the language and country code defined in RFC 4664.
For example, when the device supports US / English, Japan / Japanese, it will be as follows.
"en-US, ja-JP"
This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	startHearingYesNo Method, startHearingWord Method, startHearingSentence Method, startHearingFree Method.

Methods (UML operations)

startHearingFree Method

Syntax	startHearingFree (language: string):

void {raises-exception, use after open-claim-enable}

Parameter	Description

Language	Specify the language to recognize. Specify one of the

values listed in the LanguageList property.

Remarks	This method can make a voice recognition from the listed language in the LanguageList property. In addition, this method can be called without specifying the word candidate to be recognized from the application, however recognized word depends on the word recognizing device capability.  When this method is called, proper values are set in the HearingDataWord property, HearingResult property and HearingStatus property just before the DataEvent issuing.  This method is executed asynchronously . Voice recognition ends when stopHearing method is called.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value		Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported language was specified.

E_BUSY	Voice recognition in progress so it cannot be executed.

See Also	HearingDataWord Property, HearingResult Property, HearingStatus Property, LanguageList Property, stopHearing Method.

startHearingSentence Method

Syntax	startHearingSentence (language: string, wordList: string,           patternList: string):

void {raises-exception, use after open-claim-enable}
Parameter	Description

language	Specify the language to recognize. Specify one of the values listed in the LanguageList property.

wordList	Specify word candidates to be waited on in a comma-separated list.

patternList	Specify the sentence pattern information to be waited for in a comma-separated list.

Each word information specified in wordList consists of the following information and is shown in the following order, separated by a colon (":").

Parameter	Description

wordGroupID	ID to identify word list

wordList	A word candidate to be waited for being separated by a colon (":")

For example, to specify word candidates "one" and "two" for word candidate’s "coffee" "tea" and word group "number" in the single item group "product", specify as follows. "item:coffee:tea, number:one:two"

Each word information specified in patternList consists of the following information, and it is shown in the following order separated by a colon (":").

Parameter	Description

patternID	ID to identify the pattern

pattern	A sentence pattern to wait. To add the word list specified in wordList to the candidate, enclose the word group ID with "[" and "]". Example: "[word group ID1]" [word group ID2] "

Example: You can order coffee or tea. You can also specify how many cups you need. If you want to recognize it by voice, do as follows.

Set the startHearingSentence method parameter as follows:
WordList:"item:coffee:tea, count:a:two:three" 			              Coffee, Tea	         -> item:coffee:tea
How many cups   -> count:a:two:three

Invoke the method.
startHearingSentence ("en-US", "item:coffee:tea,count:a:two", "P1:[count] cup of [item],P2:[item]")
HearingStatus=VRCG_HSTATUS_SENTENCE;

People talk to "Give me two cups of coffee"

Speech recognition is performed, properties are set, and an event is notified.
HearingResult=VRCG_HRESULT_SENTENCE;
HearingDataPattern="P1";
HearingDataWordList="item:coffee,count:two";
raise DataEvent(0);

Remarks	This method can make a voice recognition from the listed language in the LanguageList property. In addition, this method can recognize the words and sentences that are defined in wordList and patternList as parameter. When this method is called, proper values are set in the HearingDataWord property. HearingResult property and HearingStatus property, just before DataEvent issuing. This method is executed asynchronously. Voice recognition ends when stopHearing method is called.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported

language was specified.

E_BUSY	Voice recognition in progress so it cannot be 		executed.

See Also	HearingDataWord Property, HearingResult Property, HearingStatus Property, LanguageList Property, stopHearing Method

startHearingWord Method

Syntax	startHearingWord (language: string, wordList: string):

void {raises-exception, use after open-claim-enable}
Parameter		     Description

language	Specify the language to recognize. Specify one of the values listed in the LanguageList property.

wordList	Specify word candidates to be waited on in a comma-separated list.
Example: "word1, word2, word3"

Remarks	This method can make a voice recognition from the listed language in the LanguageList property. In addition, this method can recognize the words that are defined in wordList as parameter. When this method is called, proper values are set in the HearingDataWord property, HearingResult property and HearingStatus property just before DataEvent issuing.   This method is executed asynchronously. Voice recognition ends when stopHearing method is called.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported

language was specified.

E_BUSY	Voice recognition in progress so it cannot be

executed.

See Also	HearingDataWord Property, HearingResult Property, HearingStatus Property, LanguageList Property, stopHearing Method.

startHearingYesNo Method

Syntax	startHearingYesNo (language: string):

void {raises-exception, use after open-claim-enable}
Parameter		Description

language                    Specify the language to recognize. Specify one of the

values listed in the LanguageList property.

Remarks	This method can make a voice recognition from the listed language in the LanguageList property. In addition, this method can recognize the words that are defined in the device as the recognition candidate corresponding to "Yes" "No" "Cancel".  When this method is called, proper values are set in the HearingDataWord property, HearingResult property and HearingStatus property, just before DataEvent issuing.   This method is executed asynchronously. Voice recognition ends when stopHearing method is called.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported

language was specified.

E_BUSY	Voice recognition in progress so it cannot be executed.

See Also	LanguageList Property, HearingDataWord Property, Hearing Result Property, LanguageList Property, stopHearing Method.

stopHearing Method

Syntax	stopHearing ( ):

void {raises-exception, use after open-claim-enable}
Remarks	Voice Recognition ends when this property called. This method is executed synchronously.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported

language was specified.

Events (UML interfaces)

DataEvent

<<event>>	upos::events::DataEvent

Status			: int32{read-only}

Description 	Notifies the application when data from the Voice Recognition device is available to be read.

Attributes	This event contains the following attributes:

Attribute	Type		Description

Status	int32		Set to 0.

Remarks	Before this event is delivered, the voice recognition information is enqueued into the area that is indicated by the startHearingXXX kinds of methods.

See Also	HearingResult Property, “Events" on page Intro-19, StartHearingYesNo,StartHearingWord,StartHearingSentence,StartHearingFree Method , directIO method.

DirectIOEvent

<<event>>	upos::events::DirectIOEvent

EventNumber	: int32 {read-only}
Data			: int32 {read-write}
Obj			: object {read-write}

Description 	Provides Service information directly to the application. This event provides a means for a vendor-specific Voice Recognition Service to provide events to the application that are not otherwise supported by the device control.

Attributes	This event contains the following attributes:

Attribute	Type	Description

EventNumber	int32	Event number whose specific values are assigned by the Service.

Data	int32	Additional numeric data. Specific values vary by the EventNumber and the Service. This attribute is settable.

Obj	object	Additional data whose usage varies by the EventNumber and the Service. This attribute is settable.

Remarks	This event is to be used only for those types of vendor specific functions that are not otherwise described.

Use of this event may restrict the application program programform being used with other vendor’s devices which may not have any knowledge of the Service’s need for this event.

See Also	“Events" on page Intro-19, directIO method.

ErrorEvent

<<event>> 	upos::events:: ErrorEvent
ErrorCode			: int32{read-write}
ErrorCodeExtended	: int32{read-write}
ErrorLocus 		: int32{read-write}
*pErrorResponse		: int32{read-write}

Description	Notifies the application that a Voice Recognition Device error has been detected and suitable response by the application is necessary to process the error condition.

Attributes	This event contains the following attributes:

Attributes	Type	Description

ErrorCode	int32	Error code causing the error event. 				See a list of Error Codes on page 20.

ErrorCodeExtended	int32	Extended Error code causing the error event.

If ErrorCode is E_EXTENDED, then see values below. Otherwise, it may contain a Service-specific value.

ErrorLocus	int32	Location of the error. If EL_OUTPUT is specified. An error occurred during asynchronous action.

ErrorResponse	int32	Pointer to the error event response. See ErrorResponse below for values.

The ErrorLocus attribute has one of the following values:

Value	Meaning

EL_OUTPUT	Error occurred while processing asynchronous output.

EL_INPUT	Error occurred while gathering or processing event-driven input. No previously buffered input data is available.

EL_INPUT_DATA	Error occurred while gathering or processing event-driven input, and some previously buffered data is available.

The application’s error event handler can set the ErrorResponse attribute to one of the following values:

Value	Meaning

ER_RETRY	Retry sending the data. The error state is exited.
May be valid for some input devices when the locus is EL_INPUT, in which case the input is re-tried, and the error state is exited. Typically, valid for asynchronous output devices when the locus is EL_OUTPUT, in which case the asynchronous output is re-tried, and the error state is exited. This is the default response when the locus is EL_OUTPUT.

ER_CLEAR	Valid for all loci: EL_INPUT, EL_INPUT_DATA, and EL_OUTPUT.  Clear all buffered input or output data (including all asynchronous output).  The error state is exited.  This is the default response when the locus is EL_INPUT.

ER_CONTINUEINPUT 
Only valid when the locus is EL_INPUT_DATA.
Acknowledges that a data error has occurred and directs the Device to continue input processing. The Device remains in the error state and will deliver additional DataEvents as directed by the DataEventEnabled property. When all input has been delivered and DataEventEnabled is again set to true, then another ErrorEvent is delivered with locus EL_INPUT.
This is the default response when the locus is EL_INPUT_DATA.

Remarks	This event is enqueued when an error is detected and the Device’s State transitions into the error state. Input error events are not delivered until DataEventEnabled is true, so that proper application sequencing occurs.

Unlike a DataEvent, the Device does not disable further DataEvents or input ErrorEvents; it leaves the DataEventEnabled property value at true.    Note that the application may set DataEventEnabled to false within its event handler if subsequent input events need to be disabled for a period of time.

See Also 	“Device Input Model" on page Intro-22, “Error Handling" on page Intro-23, “Device Output Models" on page Intro-25.

StatusUpdateEvent

<<event>>	upos::events:: StatusUpdateEvent
Status			: int32 {read-only}

Description	Notifies the application that there is a change in the power status or a status of the Voice  Recognition device.

Attributes		This event contains the following attribute:

Attributes	Type	Description

Status	int32	Indicates a change in the power status of the unit.

Note that Release 1.3 added Power State Reporting with additional Power reporting StatusUpdateEvent values.

The Update Firmware capability added additional Status values for communicating the status/progress of an asynchronous update firmware process. See “StatusUpdateEvent” description on page 1-34.

Value			Meaning

VRCG_SUE_START_HEARING _FREE
					It will be notified when hearing free starts.

VRCG_SUE_START_HEARING _SENTENCE
					It will be notified when hearing sentence starts.

VRCG_SUE_START_HEARING _WORD
				 	It will be notified when hearing word starts.

VRCG_SUE_START_HEARING _YESNO
					It will be notified when hearing yesno starts.

VRCG_SUE_STOP_HEARING
					It will be notified when hearing stops.

Remarks	Enqueued when the Voice Recognition Device detects a power state change or a status change.

See Also 	“Events" on page Intro-19.

Chapter 43

Sound Player

This Chapter defines the Sound Player device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.16 | Not Supported |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.16 | open |
| CapPowerReporting: | int32 | {read-only} | 1.16 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.16 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.16 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.16 | open |
| CheckHealthText: | string | {read-only} | 1.16 | open |
| Claimed: | boolean | {read-only} | 1.16 | open |
| DataCount: | int32 | {read-only} | 1.16 | Not supported |
| DataEventEnabled: | boolean | {read-write} | 1.16 | Not supported |
| DeviceEnabled: | boolean | {read-write} | 1.16 | open & claim |
| FreezeEvents: | boolean | {read-write} | 1.16 | open |
| OutputID: | int32 | {read-only} | 1.16 | open |
| PowerNotify: | int32 | {read-write} | 1.16 | open |
| PowerState: | int32 | {read-only} | 1.16 | open |
| State: | int32 | {read-only} | 1.16 | -- |
| DeviceControlDescription: | string | {read-only} | 1.16 | - |
| DeviceControlVersion: | int32 | {read-only} | 1.16 | - |
| DeviceServiceDescription: | string | {read-only} | 1.16 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.16 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.16 | open |
| PhysicalDeviceName: | string | {read-only} | 1.16 | open |

| Properties (Continued) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specific | Type | Mutability | Version | May Use After |
| CapAssociatedHardTotalsDevice | string | {read-write} | 1.16 | open |
| CapMultiPlay: | boolean | {read-only} | 1.16 | open |
| CapSoundTypeList: | string | {read-only} | 1.16 | open |
| CapStorageTarget | int32 | {read-only} | 1.16 | open |
| CapVolume: | boolean | {read-only} | 1.16 | open |
| DeviceSoundList: | string | {read-only} | 1.16 | open |
| OutputIDList: | string | {read-only} | 1.16 | open, claim & enable |
| StorageTarget | string | {read-write} | 1.16 | open, claim & enable |
| Volume: | int32 | {read-write} | 1.16 | open, claim & enable |

| Me Methods (UML operations) |  |
| --- | --- |
| Common |  |
| Name | Version |
| open (logicalDeviceName: string): void {raises-exception} | 1.16 |
| close ( ): void {raises-exception, use after open} | 1.16 |
| claim (timeout: int32): void {raises-exception, use after open} | 1.16 |
| release ( ): void {raises-exception, use after open, claim} | 1.16 |
| checkHealth (level: int32): void {raises-exception, use after open, enable} | 1.16 |
| clearInput ( ): void { } | Not supported |
| clearInputProperties ( ): void { } | Not supported |
| clearOutput ( ): void { } | 1.16 |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} | 1.16 |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, claim, enable} | 1.16 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | 1.16 |
| retrieveStatistics ( inout statisticsBuffer: string ): void {raises-exception, use after open, claim, enable} | 1.16 |
| Me Methods (UML operations)(continued) |  |
| Common |  |
| Name | Version |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, claim, enable} | 1.16 |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | 1.16 |
| Specific |  |
| Name | Version |
| playSound(fileName: string, loop: boolean): void { raises-exception, use after open, claim, enable} | 1.16 |
| stopSound(outputID:int32): void {raises-exception, use after open, claim, enable} | 1.16 |

| Events (UML interfaces) |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Mutability | Version |
| upos::events::DataEvent |  | Not Supported | 1.16 |
| upos::events::DirectIOEvent |  |  | 1.16 |
| EventNumber: | int32 | {read-only} |  |
| Data: | int32 | {read-write} |  |
| Obj: | object | {read-write} |  |
| upos::events::ErrorEvent |  |  | 1.16 |
| ErrorCode: | int32 | {read-only} |  |
| ErrorCodeExtended: | int32 | {read-only} |  |
| ErrorLocus: | int32 | {read-only} |  |
| *pErrorResponse: | int32 | {read-write} |  |
| upos::events::OutputCompleteEvent |  |  | 1.16 |
| OutputID: | int32 | {read-only} |  |
| upos::events::StatusUpdateEvent |  |  | 1.16 |
| Status: | int32 | {read-only} |  |

General Information

The Sound Player programmatic name is "Sound Player".

Capabilities

The Sound Player has the following capability:

- Play audio file.

Sound Player Class Diagram

The following diagram shows the relationships between the Sound player classes.

Fig. Chap.43-1 Sound Player Class Diagram

Model

The Sound Player follows the general device behavior model for asynchronous output devices:

- The Device validates the method parameters and produces an error condition immediately if necessary. If the validation is successful, the Device does the following:

- Audio files will be played sequentially. When playSound method is called, device starts the playing sound that is specified by the method parameters and the requested sound file data placed in a queue and corresponding OutputID is stored at OutputID property and added to the OutputIDList property as a listed value. And sets the OutputID property to a unique integer identifier for this request.

- When the sound playing starts StatusUpdateEvent is evoked as the value of  SPLY_SUE_START_PLAY_SOUND. 					   When the sound playing is finished an OutputCompleteEvent is enqueued for the delivery to the application and corresponding OutputID is stored in OutputID property. At the same time, StatusUpdateEvent is evoked as the value of SPLY_SUE_STOP_PLAY_SOUND. The application should compare the returned OutputCompleteEvent property OutputID value with the OutputID value set by the asynchronous process method call used to send the data in order to track what data has been successfully sent to the device.

- When stopSound method is called, device stop the playing sound according to the OutputID property value and the current playing sound is terminated and enqueued sound file data is cleared. After this method is executed, corresponding OutputID property and OutputIDList values are not changed. No OutputCompleteEvent is fired and only StatusUpdateEvent will be evoked the value of SPLY_SUE_STOP_PLAY_SOUND.

- If an error occurs while processing a request, an ErrorEvent is enqueued which will be delivered to the application after the events already enqueued, including OutputCompleteEvent. No further asynchronous output will occur until the event has been delivered to the application. If the response is ER_CLEAR, then outstanding asynchronous output is cleared. If the response is ER_RETRY, then output is retried; note that if several outputs were simultaneously in progress at the time that the error was detected, then the Service may need to retry all of these outputs.

- Asynchronous output is always performed on a first-in first-out basis. If the device supports concurrent playback, the request will be executed simultaneously. To check if the device supports simultaneous playback, check the CapMultiPlay property.

- If the request is terminated before completion, due to reasons such as the application calling the clearOutput method, then no OutputCompleteEvent is delivered.

- Application can also delete the output individually by calling the stopSound method. Also, in this case OutputCompleteEvent will not be notified."

- The CapSoundTypeList property lists audio file types that the device can play.

- The application will be informed about any status change with a StatusUpdateEvent, also all corresponding status properties will be updated before event delivery.

- If device supports either or both of HardTotals devices and the host file system, the                                                                                   application should set the StorageTarget property accordingly to tell where to access the data file.

- If device needs to be able to access the audio files played with playSound method from a HardTotals device, the CapAssociatedHardTotalsDevice property holds the open name of the associated HardTotals device.

Device Sharing

The Sound Player is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before accessing some properties or calling methods that update the device.

- See the “Summary” table for precise usage prerequisites.

Properties(UML attributes)

## CapAssociatedHardTotalsDevice Property

Syntax	CapAssociatedHardTotalsDevice : string {read-write, access after open}

Remarks	Holds the open name of the associated HardTotals device if the device is able to write to such devices which is the case if CapStorageTarget is either CST_ALL or CST_HARDTOTALS_ONLY. If CapStorageTarget is CST_HOST_ONLY this property value must be the empty string.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	CapStorageTarget Property

CapMultiPlay Property

Syntax	CapMultiPlay : boolean {read-only, access after open}

Remarks	If true, the application can play sound simultaneously.

If false, the application cannot play sound simultaneously.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	playSound Method.

CapSoundTypeList Property

Syntax	CapSoundTypeList : string {read-only, access after open}

Remarks	Contains the comma-delimited list of file type that is supported by the device.

For example, if the device only supports WAV and OGG, then this property should be set to “WAV, OGG". This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	playSound Method

## CapStorageTarget Property

Syntax	CapStorageTarget: int32 {read-only, access after open}

Remarks	This is an enumeration and announces where the device is able to write the recorded sound data file to.
It holds one of the following values.

Value	Meaning

CST_HARDTOTAL_ONLY	Only an associate HardTotals device is supported.

CST_HOST_ONLY	Only the host’s file system is supported.

CST_ALL	Both, the associated HardTotals device and the host’s file system is supported.

This property is initialized by the open method.

If a HardTotals device is supported the storage target, the property value should be CST_HARDTOTAL_ONLY or CST_ALL and the property CapAssociatedHardTotalsDevice holds the open name of the associated HardTotals device.

CapVolume Property

Syntax	CapVolume: boolean {read-only, access after open}

Remarks	If true, the application can change the volume during playback.

If false, the application cannot change the volume during playback.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	Volume Property.

DeviceSoundList Property

Syntax	DeviceSoundList : string {read-only, access after open}

Remarks	Contains the comma-delimited list of device sound ID that is supported by the device. This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	playSound Method

OutputIDList Property

Syntax	OutputIDList : string {read-only, access after open, claim-enable}

Remarks	Contains the comma-delimited list of OutputID that is output by the playSound method. This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	playSound Method

## StorageTarget Property

Syntax	StorageTarget: string {read-write, access after open-claim-enable}

Remarks	Holds the open name of HardTotals device, if CapStorageTarget property value is CST_HARTTOTAL_ONLY or CST_ALL. If CapStorageTarget property value is CST_HOST_ONLY, this property value must be the empty string. This property is initialized by the open method according to the value hold by CapStorageTarget.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	CapStorageTarget Property

Volume Property

Syntax	Volume : int32 {read-write, access after open-claim-enable}

Remarks	Holds the volume at playing sound.

Legal values range from zero through 100.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	playSound Method

Methods (UML operations)

playSound Method

Syntax	playSound (fileName : string, loop : boolean):
		void{raises-exception, use after open-claim-enable}

Parameter	Description

fileName	Specifies the file name of audio file. Or, specifies one of the sound ID defined by DeviceSoundList.

loop	When true is specified, loop playback is performed, and if false is specified, loop playback will not be performed.

Remarks	Play audio file specified by fileName or device definition sound.

Audio files might be located in the area managed by "Hard Totals" service.

This method will be performed asynchronously. To stop playback, call the stopSound method.

Errors	A UposException may be thrown when this method is invoked. For further information , see “Errors” on page Intro-20.  Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported sound file was specified.

E_NOEXIST	File does not exist.

See Also	CapSoundType Property, DeviceSoundList Property, stopSound Method

stopSound Method

Syntax	StopSound(outputID: int32):
		void{raises-exception, use after open-claim-enable}

Parameter	Description

outputID	Specify the outputID of the sound to stop.

Remarks	Terminates specified audio playback according to the OutputID property value.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors” on page Intro-20.  Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	The specified sound is not being played.

See Also	OutputID Property, startSound Method

Events (UML interfaces)

DirectIOEvent

<<event>>	upos::events::DirectIOEvent

EventNumber	: int32 {read-only}
Data			: int32 {read-write}
Obj			: object {read-write}

Description 	Provides Service information directly to the application. This event provides a means for a vendor-specific Sound Player Service to provide events to the application that are not otherwise supported by the device control.

Attributes	This event contains the following attributes:

Attribute	Type	Description

EventNumber	int32	Event number whose specific values are assigned by the Service.

Data	int32	Additional numeric data. Specific values vary by the EventNumber and the Service. This attribute is settable.

Obj	object	Additional data whose usage varies by the EventNumber and the Service. This attribute is settable.

Remarks	This event is to be used only for those types of vendor specific functions that are not otherwise described.

Use of this event may restrict the application program programform being used with other vendor’s devices which may not have any knowledge of the Service’s need for this event.

See Also	“Events" on page Intro-19, directIO method

ErrorEvent

<<event>> 	upos::events:: ErrorEvent
ErrorCode			: int32{read-write}
ErrorCodeExtended	: int32{read-write}
ErrorLocus 		: int32{read-write}
*pErrorResponse		: int32{read-write}

Description	Notifies the application that a Sound Player Device error has been detected and suitable response by the application is necessary to process the error condition.

Attributes	This event contains the following attributes:

Attributes	Type	Description

ErrorCode	int32	Error code causing the error event. 				See a list of Error Codes on page 20.

ErrorCodeExtended	int32	Extended Error code causing the error event.

If ErrorCode is E_EXTENDED, then see values below. Otherwise, it may contain a Service-specific value.

ErrorLocus	int32	Location of the error. If EL_OUTPUT is specified. An error occurred during asynchronous action.

ErrorResponse	int32	Pointer to the error event response. See ErrorResponse below for values.

The ErrorLocus attribute has one of the following values:

Value	Meaning

EL_OUTPUT	Error occurred while processing asynchronous output.

EL_INPUT	Error occurred while gathering or processing event-driven input. No previously buffered input data is available.

EL_INPUT_DATA	Error occurred while gathering or processing event-driven input, and some previously buffered data is available.

The application’s error event handler can set the ErrorResponse attribute to one of the following values:

Value	Meaning

ER_RETRY	Retry sending the data. The error state is exited.
May be valid for some input devices when the locus is EL_INPUT, in which case the input is re-tried, and the error state is exited. Typically, valid for asynchronous output devices when the locus is EL_OUTPUT, in which case the asynchronous output is re-tried, and the error state is exited. This is the default response when the locus is EL_OUTPUT.

ER_CLEAR	Valid for all loci: EL_INPUT, EL_INPUT_DATA, and EL_OUTPUT.  Clear all buffered input or output data (including all asynchronous output).  The error state is exited.  This is the default response when the locus is EL_INPUT.

ER_CONTINUEINPUT 
Only valid when the locus is EL_INPUT_DATA.
Acknowledges that a data error has occurred and directs the Device to continue input processing. The Device remains in the error state and will deliver additional DataEvents as directed by the DataEventEnabled property. When all input has been delivered and DataEventEnabled is again set to true, then another ErrorEvent is delivered with locus EL_INPUT.
This is the default response when the locus is EL_INPUT_DATA.

Remarks	This event is enqueued when an error is detected and the Device’s State transitions into the error state. Input error events are not delivered until DataEventEnabled is true, so that proper application sequencing occurs.

Unlike a DataEvent, the Device does not disable further DataEvents or input ErrorEvents; it leaves the DataEventEnabled property value at true.    Note that the application may set DataEventEnabled to false within its event handler if subsequent input events need to be disabled for a period of time.

See Also 	“Device Input Model" on page Intro-22, “Error Handling" on page Intro-23, “Device Output Models" on page Intro-25.

OutputCompleteEvent

<<event>>	upos::events::OutputCompleteEvent 
OutputID		: int32{read-only}

Description 	Notify the application that the queued output request associated with the outputID property has completed successfully.

Attributes	This event contains the following attributes:

Attribute	Type	Description

OutputID 	int32	The ID number of the asynchronous output request that is complete.

Remarks	This event is enqueued after the request’s data has been both sent, and the Service has confirmation that it was processed by the device successfully.

See Also	“Device Output Models" on page Intro-25

StatusUpdateEvent

<<event>>	upos::events:: StatusUpdateEvent
Status			: int32 {read-only}

Description    Notifies the application that there is an operation status change or a status of the sound player device.

Attributes	This event contains the following attribute:

Attributes	    Type	      Description

Status	    int32        Indicates a change of operation status of sound player 					　  device.

Note that Release 1.3 added Power State Reporting with additional Power reporting StatusUpdateEvent values.

The Update Firmware capability added additional Status values for communicating the status/progress of an asynchronous update firmware process.

See “StatusUpdateEvent” description on page 1-34.

Value            		Meaning

SPLY_SUE_START_PLAY_SOUND										It will be notified when sound playing start.

SPLY_SUE_STOP_PLAY_SOUND										It will be notified when sound playing stop.

Remarks	Enqueued when the Sound Player Device detects a power state change or a status change.

See Also 	“Events" on page Intro-19.

Chapter 44

Speech Synthesis

This Chapter defines the Speech Synthesis device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.16 | Not Supported |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.16 | open |
| CapPowerReporting: | int32 | {read-only} | 1.16 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.16 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.16 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.16 | open |
| CheckHealthText: | string | {read-only} | 1.16 | open |
| Claimed: | boolean | {read-only} | 1.16 | open |
| DataCount: | int32 | {read-only} | 1.16 | Not Supported |
| DataEventEnabled: | boolean | {read-write} | 1.16 | Not Supported |
| DeviceEnabled: | boolean | {read-write} | 1.16 | open & claim |
| FreezeEvents: | boolean | {read-write} | 1.16 | open |
| OutputID: | int32 | {read-only} | 1.16 | open |
| PowerNotify: | int32 | {read-write} | 1.16 | open |
| PowerState: | int32 | {read-only} | 1.16 | open |
| State: | int32 | {read-only} | 1.16 | -- |
| DeviceControlDescription: | string | {read-only} | 1.16 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.16 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.16 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.16 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.16 | open |
| PhysicalDeviceName: | string | {read-only} | 1.16 | open |

| Properties (Continued) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Specific |  | Type | Mutability | Version | May Use After |  |
| CapLanguage: |  | boolean | {read-only} | 1.16 | open |  |
| CapPitch: |  | boolean | {read-only} | 1.16 | open |  |
| CapSpeed: |  | boolean | {read-only} | 1.16 | open |  |
| CapVoice: |  | boolean | {read-only} | 1.16 | open |  |
| CapVolume: |  | boolean | {read-only} | 1.16 | open |  |
| Language: |  | string | {read-write} | 1.16 | open, claim & enable |  |
| LanguageList: |  | string | {read-only} | 1.16 | open |  |
| OutputIDList: |  | string | {read-only} | 1.16 | open, claim & enable |  |
| Pitch: |  | int32 | {read-write} | 1.16 | open, claim & enable |  |
| Speed: |  | int32 | {read-write} | 1.16 | open, claim & enable |  |
| Voice: |  | string | {read-write} | 1.16 | open, claim & enable |  |
| VoiceList: |  | string | {read-only} | 1.16 | open |  |
| Volume: |  | int32 | {read-write} | 1.16 | open, claim & enable |  |
| Me Methods (UML operations) |  |  |  |  |  |  |
| Common |  |  |  |  |  |  |
| Name |  |  |  |  | Version |  |
| open (logicalDeviceName: string): void {raises-exception} |  |  |  |  | 1.16 |  |
| close ( ): void {raises-exception, use after open} |  |  |  |  | 1.16 |  |
| claim ( timeout: int32): void {raises-exception, use after open} |  |  |  |  | 1.16 |  |
| release ( ): void {raises-exception, use after open, claim} |  |  |  |  | 1.16 |  |
| checkHealth (level: int32): void {raises-exception, use after open, enable} |  |  |  |  | 1.16 |  |
| clearInput ( ): void { } |  |  |  |  | Not supported |  |
| clearInputProperties ( ): void { } |  |  |  |  | Not supported |  |
| Methods (UML operations)(continued) |  |  |  |  |  |  |
| clearOutput ( ): void { } |  |  |  |  | 1.16 |  |
| Common |  |  |  |  |  |  |
| Name |  |  |  |  | Version |  |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} |  |  |  |  | 1.16 |  |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |
| Specific |  |  |  |  |  |  |
| Name |  |  |  |  |  |  |
| speak (text: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |
| speakImmediate (text: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |
| stopCurrentSpeaking ( ): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |
| stopSpeaking (outputID: int32 ): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |

| Events (UML interfaces) |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Mutability | Version |
| upos::events::DataEvent |  | Not Supported |  |
| upos::events::DirectIOEvent |  |  | 1.16 |
| EventNumber: | int32 | {read-only} |  |
| Data: | int32 | {read-write} |  |
| Obj: | object | {read-write} |  |
| upos::events::ErrorEvent |  |  | 1.16 |
| ErrorCode: | int32 | {read-only} |  |
| ErrorCodeExtended: | int32 | {read-only} |  |
| ErrorLocus: | int32 | {read-only} |  |
| *pErrorResponse: | int32 | {read-write} |  |
| upos::events::OutputCompleteEvent |  |  | 1.16 |
| OutputID: | int32 | {read-only} |  |
| upos::events::StatusUpdateEvent |  |  | 1.16 |
| Status: | int32 | {read-only} |  |

General Information

The Speech Synthesis programmatic name is "Speech Synthesis".

Capabilities

The Speech Synthesis has the following capability:

- Convert text to speech and read it aloud.

Speech Synthesis Class Diagram

The following diagram shows the relationships between the Speech Synthesis classes.

Fig. Chap. 44-1 Speech Synthesis Class Diagram

Model

The Speech Synthesis follows the general device behavior model for output devices with some enhancements.

The application calls a speak method or speakImmediate method to speech.

The speak method acts to start speaking from the words specified by text, while the speakImmediate method ends immediately previous speak method, and starts speaking the word specified by text asynchronously and immediately.

When speak or speakImmediate method is called device start the speaking based on the setting value of Language, Volume, Pitch and Speed properties. And requested utterance written by text data placed in a queue and corresponding OutputID is stored at OutputID property and added to the OutputIDList property as listed value. And sets the OutputID property to a unique integer identifier for this request.

When an utterance of speak method or speakImmediate method starts, StatusUpdateEvent is evoked as the value of SPSY_SUE_START_SPEAK. When the utterance is finished an OutputCompleteEvent is enqueued for the delivery to the application and corresponding OutputID is stored in OutputID property. At the same time StatusUpDateEvent is evoked as the value of SPSY_SUE_STOP_SPEAK. The application should compare the returned OutputCompleteEvent property OutputID value with OutputID value set by the asynchronous process method call used to send the data in order to track what data has been successfully sent to the device

When speakImmediate method is called during the utterance of speak method or speakImmediate method call, utterance will be stopped immediately. And StatusUpdateEvent is evoked as the value of SPSY_SUE_STOP_SPEAK.  However, OutputCompleteEvent is not fired. And current speak method or speakImmediate method corresponding OutputID property and OutputIDList property values are not changed.

When stopCurrentSpeaking method is called, current utterance generated by speak method or speakImmediate method will be stopped and StatusUpdateEvent is evoked as the value of  SPSY_SUE_STOP_SPEAK. And no OutputCompleteEvent is fired. And current speak method or speakImmediate method corresponding OutputID property and OutputIDList property values are not changed.

When stopSpeaking method is called, specified OutputID valued utterance is stopped and deleted. And OutputID property value in the OutputIDList property is eliminated.

When utterance is stopped StatusUpdateEvent is evoked as the value of SPSY_SUE_STOP_SPEAK. And no OutputCompleteEvent is fired.

If an error occurs while processing a request, an ErrorEvent is enqueued which will be delivered to the application after the events already enqueued, including OutputCompleteEvent. No further asynchronous output will occur until the event has been delivered to the application. If the response is ER_CLEAR, then outstanding asynchronous output is cleared. If the response is ER_RETRY, then output is retried; note that if several outputs were simultaneously in progress at the time that the error was detected, then the service may need to retry all of these outputs.

Asynchronous output is always performed on a first-in first-out basis.

If the request is terminated before completion, due to reasons such as the application calling the clearOutput method, then no OutputCompleteEvent is delivered.

The application will be informed about any status change with a StatusUpdateEvent, also all corresponding status properties will be updated before event delivery.

Device Sharing

The Speech Synthesis is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before accessing some properties or calling methods that update the device.

- See the “Summary” table for precise usage prerequisites.

Properties (UML attributes)

CapLanguage Property

Syntax	CapLanguage: boolean {read-only, access after open}

Remarks	If true, the application can change the language. If false, the application cannot change the language.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	Language Property

CapPitch Property

Syntax	CapPitch: boolean {read-only, access after open}

Remarks	If true, the application can change the pitch. If false, the application cannot change the pitch.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	Pitch Property

CapSpeed Property

Syntax	CapSpeed: boolean {read-only, access after open}

Remarks	If true, the application can change the speed. If false, the application cannot change the speed.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	Speed Property

CapVoice Property

Syntax	CapVoice: boolean {read-only, access after open}

Remarks	If true, the application can change the voice. If false, the application cannot change the voice.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	Voice Property

CapVolume Property

Syntax	CapVolume: boolean {read-only, access after open}

Remarks	If true, the application can change the volume. If false, the application cannot change the volume.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	Volume Property

Language Property

Syntax	Language: string {read-write, access after open-claim-enable}

Remarks	Indicates the language to speak. Valid values are one of the values listed in the LanguageList property.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported

language was specified.

See Also	speak Method, speakImmediate Method

LanguageList Property

Syntax	LanguageList: string {read-only, access after open}

Remarks	Contains the comma-delimited list of language that are supported by the device. The value representing the language is a value consisting of the language and country code defined in RFC 4664. For example, when the device supports US / English, Japan / Japanese, it will be as follows.
"en-US, ja-JP"

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	Language Property

OutputIDList Property

Syntax	OutputIDList : string {read-write, access after open-claim-enable}

Remarks	Comma-separated list of OutputID property values of audio being played by Speak method or SpeakImmediate method. This list indicates the capability how many and what kinds of utterance can be done by the targeted Speech Synthesis device

This property is initialized by the open method. It will also be updated as the speech request increases or decreases.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	speak Method, speakImmediate Method

Pitch Property

Syntax	Pitch: int32 {read-write, access after open-claim-enable}

Remarks	Holds the pitch at speech. Legal values range from 50% through 200%.

This property is initialized to 100% by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	speak Method, speakImmediate Method

Speed Property

Syntax	Speed: int32 {read-write, access after open-claim-enable}

Remarks	Holds the speed at speech. Legal values range from 50% through 200%.

This property is initialized to 100% by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	speak Method, speakImmediate Method

Voice Property

Syntax	Voice : string {read-write, access after open-claim-enable }

Remarks	Indicates the voice tone to speak. Valid values are one of the values listed in the VoiceList property.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported

voice was specified.

See Also	speak Method, speakImmediate Method

VoiceList Property

Syntax	VoiceList: string { read-only, access after open }

Remarks	A list of speech able voices are shown in a comma-separated list. For example, when the device supports male and female voice tones, it looks like the following.
"MALE_VOICE, FEMALE_VOICE"
(The content of the value depends on the device)

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	Voice Property

Volume Property

Syntax	Volume : int32 {read-write, access after open-claim-enable}

Remarks	Holds the volume at speech. Legal values range from zero through 100.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	speak Method, speakImmediate Method

Methods (UML operations)

speak Method

Syntax	speak (text: string):

void {raises-exception, use after open-claim-enable}

Parameter		Description

Text	Specify the text to speak.

Remarks	Device utters after converting the specified string into speech.

The utterance is executed according to the setting contents of Language property, Volume property, Pitch property, Speed property, but by inserting the following tag in the text, it is possible to change the utterance after the tag.

Content written in text is uttered with the following parameter settings.

| Tag | Description | Value (decimal integer) | Default Value (decimal integer) |
| --- | --- | --- | --- |
| volume | Specify the volume of the uttered voice. | 1 to 100 | 50 |
| pitch | Specify the high or low of the uttered voice. | 50 to 200 | 100 |
| speed | Specify the speed of the uttered voice. | 50 to 200 | 100 |
| pause | Specify the time to pause in milliseconds. | 1 to 50000 | 1 |
| reset | Rest the effect of volume, pitch, speed to the default value. | - | - |

If dialogue is “ Hello. Today, it’s nice weather.”

Then if you would like to use the default setting of speed, volume, pitch for the “Hello”. And would like to put a pose between “Hello” and “Today” 1000 milliseconds and would like to change the speaking pith of “Today” to 150 and increase the volume to 80. Then for the “It’s nice weather” would like return to the default value by using the reset. It is described as follows

Hello.{pause=1000,pitch=150,volue=80}Today,{reset}It's nice weather.

Those utterance defined as follows.

| Name | Data | Remarks |  |
| --- | --- | --- | --- |
| Utterance written by text with the speak method parameter. Text will be spoken under the assigned parameter condition. | {#=f}XXXX{#=f}YYYY | #:Tag names It is volume, pitch, speed, pause and reset. | f:Tag values It is described in the Tag Value Table. |

When this method is called by the application, device validate the method parameters, and if validation is successful buffer the request in program memory and deliver it to the device and process it. And device sets the unique integer identifier into the OutputID property. When device successfully complete a request an OutputCompleteEvent is enqueued for delivery to the application.  If the device does not support volume change etc., that tag will be ignored.  This method is executed asynchronously. To end an utterance halfway, call the stopCurrentSpeaking method or the stopSpeaking method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.  Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL		An invalid value was specified. The language set in  the Language property and the language specified by Text do not match.

See Also	Language Property, Volume Property, Pitch Property, OutputID Property,    Speed Property, stopCurrentSpeaking Method, stopSpeaking Method

speakImmediate Method

Syntax	speakImmediate (text: string):

void {raises-exception, use after open-claim-enable}
Parameter	Description

text	Specify the text to speak.

Remarks	The speak method acts to start speaking the words specified by text, while the speakImmediate method ends immediately previous speak method, and starts speaking the word specified by text asynchronously and immediately.

After executing the same processing as the clearOutput method, speak the wording specified by text.

Like this speak method, this method can also change a specific wording by inserting a tag. For details, refer to the description of speak method.

This method is executed asynchronously. To end an utterance halfway, call the stopCurrentSpeaking method or the stopSpeaking method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.  Some possible values of the exception’s ErrorCode property are:

Value          	Meaning

E_ILLEGAL 	An invalid value was specified. The language set in    the Language property and the language specified by Text do not match.

See Also	Language Property, Volume Property, Pitch Property, Speed Property,             speak Method, stopCurrentSpeaking Method, stopSpeaking Method

stopCurrentSpeaking Method

Syntax	stopCurrentSpeaking ( ):

void {raises-exception, use after open-claim-enable}

Remarks	The speak method and speakImmediate method start the speaking words specified by text and ends when stopCurrentSpeaking method is called. This method handles asynchronously.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	Speech is not running.

See Also	speak Method, speakImmediate Method

stopSpeaking Method

Syntax	stopSpeaking (outputID : int32):

void {raises-exception, use after open, claim, enable}

Parameter	Description

outputID	Specify the value of the OutputID property you wish to terminate.

Remarks	Stop and delete the utterance specified in OutputID.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	OutputID Property, speak Method, speakImmediate Method

Events (UML interfaces)

DirectIOEvent

<<event>>	upos::events::DirectIOEvent

EventNumber	: int32 {read-only}
Data			: int32 {read-write}
Obj			: object {read-write}

Description 	Provides Service information directly to the application. This event provides a means for a vendor-specific Sound Player Service to provide events to the application that are not otherwise supported by the device control.

Attributes	This event contains the following attributes:

Attribute	Type	Description

EventNumber	int32	Event number whose specific values are assigned by the Service.

Data	int32	Additional numeric data. Specific values vary by the EventNumber and the Service. This attribute is settable.

Obj	object	Additional data whose usage varies by the EventNumber and the Service. This attribute is settable.

Remarks	This event is to be used only for those types of vendor specific functions that are not otherwise described.

Use of this event may restrict the application program programform being used with other vendor’s devices which may not have any knowledge of the Service’s need for this event.

See Also	“Events" on page Intro-19, directIO method

ErrorEvent

<<event>> 	upos::events:: ErrorEvent
ErrorCode			: int32{read-write}
ErrorCodeExtended	: int32{read-write}
ErrorLocus 		: int32{read-write}
*pErrorResponse		: int32{read-write}

Description	Notifies the application that a Speech Synthesis Device error has been detected and suitable response by the application is necessary to process the error condition.

Attributes	This event contains the following attributes:

Attributes	Type	Description

ErrorCode	int32	Error code causing the error event. 				See a list of Error Codes on page 20.

ErrorCodeExtended	int32	Extended Error code causing the error event.

If ErrorCode is E_EXTENDED, then see values below. Otherwise, it may contain a Service-specific value.

ErrorLocus	int32	Location of the error. If EL_OUTPUT is specified. An error occurred during asynchronous action.

ErrorResponse	int32	Pointer to the error event response. See ErrorResponse below for values.

The ErrorLocus attribute has one of the following values:

Value	Meaning

EL_OUTPUT	Error occurred while processing asynchronous output.

EL_INPUT	Error occurred while gathering or processing event-driven input. No previously buffered input data is available.

EL_INPUT_DATA	Error occurred while gathering or processing event-driven input, and some previously buffered data is available.

The application’s error event handler can set the ErrorResponse attribute to one of the following values:

Value	Meaning

ER_RETRY	Retry sending the data. The error state is exited.
May be valid for some input devices when the locus is EL_INPUT, in which case the input is re-tried, and the error state is exited. Typically, valid for asynchronous output devices when the locus is EL_OUTPUT, in which case the asynchronous output is re-tried, and the error state is exited. This is the default response when the locus is EL_OUTPUT.

ER_CLEAR	Valid for all loci: EL_INPUT, EL_INPUT_DATA, and EL_OUTPUT.  Clear all buffered input or output data (including all asynchronous output).  The error state is exited.  This is the default response when the locus is EL_INPUT.

ER_CONTINUEINPUT 
Only valid when the locus is EL_INPUT_DATA.
Acknowledges that a data error has occurred and directs the Device to continue input processing. The Device remains in the error state and will deliver additional DataEvents as directed by the DataEventEnabled property. When all input has been delivered and DataEventEnabled is again set to true, then another ErrorEvent is delivered with locus EL_INPUT.
This is the default response when the locus is EL_INPUT_DATA.

Remarks	This event is enqueued when an error is detected and the Device’s State transitions into the error state. Input error events are not delivered until DataEventEnabled is true, so that proper application sequencing occurs.

Unlike a DataEvent, the Device does not disable further DataEvents or input ErrorEvents; it leaves the DataEventEnabled property value at true.    Note that the application may set DataEventEnabled to false within its event handler if subsequent input events need to be disabled for a period of time.

See Also 	“Device Input Model" on page Intro-22, “Error Handling" on page Intro-23, “Device Output Models" on page Intro-25.

OutputCompleteEvent

<<event>>	upos::events::OutputCompleteEvent
OutputID		: int32{read-only}

Description 	Notify the application that the queued output request associated with the outputID property has completed successfully.

Attributes	This event contains the following attributes:

Attribute	Type	Description

OutputID 	int32	The ID number of the asynchronous output request that is complete.

Remarks	This event is enqueued after the request’s data has been both sent, and the Service has confirmation that it was processed by the device successfully.

See Also	“Device Output Models" on page Intro-25

StatusUpdateEvent

<<event>>	upos::events:: StatusUpdateEvent
Status			: int32 {read-only}

Description	Notifies the application that there is an operation status change or a status of the Speech Synthesis device.

Attributes		This event contains the following attribute:

Attribute	Type	Description

Status            int32	Indicates a change of operation status of sound player 			device

.

Note that Release 1.3 added Power State Reporting with additional Power 	reporting StatusUpdateEvent values.

The Update Firmware capability added additional Status values for communicating the status/progress of an asynchronous update firmware process. See “StatusUpdateEvent” description on page 1-34.

Value            		Meaning

SPSY_SUE_START_SPEAK										It will be notified when speech synthesis starts.

SPSY_SUE_STOP_SPEAK										It will be notified when speech synthesis stops.

Remarks	Enqueued when the Speech Synthesis Device detects a power state change or a status change.

See Also 	“Events" on page Intro-19.

Chapter 45

Gesture Control

This Chapter defines the Gesture Control device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | --- | Not supported |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.16 | open |
| CapPowerReporting: | int32 | {read-only} | 1.16 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.16 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.16 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.16 | open |
| CheckHealthText: | string | {read-only} | 1.16 | open |
| Claimed: | boolean | {read-only} | 1.16 | open |
| DataCount: | int32 | {read-only} | --- | Not supported |
| DataEventEnabled: | boolean | {read-write} | --- | Not supported |
| DeviceEnabled: | boolean | {read-write} | 1.16 | open & claim |
| FreezeEvents: | boolean | {read-write} | 1.16 | open |
| OutputID: | int32 | {read-only} | 1.16 | open |
| PowerNotify: | int32 | {read-write} | 1.16 | open |
| PowerState: | int32 | {read-only} | 1.16 | open |
| State: | int32 | {read-only} | 1.16 | -- |
| DeviceControlDescription: | string | {read-only} | 1.16 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.16 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.16 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.16 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.16 | open |
| PhysicalDeviceName: | string | {read-only} | 1.16 | open |

| Properties (Continued) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specific | Type | Mutability | Version | May Use After |
| CapAssociatedHardTotalsDevice: | string | {read-only} | 1.16 | open |
| CapMotion: | boolean | {read-only} | 1.16 | open |
| CapMotionCreation: | boolean | {read-only} | 1.16 | open |
| CapPose: | boolean | {read-only} | 1.16 | open |
| CapPoseCreation: | boolean | {read-only} | 1.16 | open |
| CapStorageTarget: | int32 | {read-only} | 1.16 | open |
| AutoMode: | string | {read-write} | 1.16 | open, claim & enable |
| AutoModeList: | string | {read-only} | 1.16 | open |
| MotionList: | string | {read-only} | 1.16 | open |
| PoseCreationMode: | boolean | {read-write} | 1.16 | open, claim & enable |
| PoseList: | string | {read-only} | 1.16 | open |
| StorageTarget: | int32 | {read-write} | 1.16 | open, claim & enable |
| JointList: | string | {read-only} | 1.16 | open |

| Methods (UML operations) |  |
| --- | --- |
| Common |  |
| Name | Version |
| open (logicalDeviceName: string): void {raises-exception} | 1.16 |
| close ( ): void {raises-exception, use after open} | 1.16 |
| claim (timeout: int32): void {raises-exception, use after open} | 1.16 |
| release ( ): void {raises-exception, use after open, claim} | 1.16 |
| checkHealth (level: int32): void {raises-exception, use after open, enable} | 1.16 |
| clearInput ( ): void { } | Not supported |
| clearInputProperties ( ): void { } | Not supported |
| clearOutput ( ): void { } | 1.16 |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, enable} | 1.16 |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} | 1.16 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.16 |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.16 |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, enable} | 1.16 |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.16 |

| Me Methods (UML operations)(continued) |  |
| --- | --- |
| Specific |  |
| Name | Version |
| createMotion (fileName: string, poseList: string): void { raises-exception, use after open, claim, enable } | 1.16 |
| createPose (fileName: string, time: int32): void { raises-exception, use after open, claim, enable } | 1.16 |
| getPosition (jointID: string, position: out int32 by reference): void { raises-exception, use after open, claim, enable } | 1.16 |
| setPosition (positionList: string, time: int32 , absolute: boolean): void { raises-exception, use after open, claim, enable } | 1.16 |
| setSpeed (speedList: string, time: int32): void { raises-exception, use after open, claim, enable } | 1.16 |
| startMotion (fileName: string): void { raises-exception, use after open, claim, enable } | 1.16 |
| startPose (fileName: string): void { raises-exception, use after open, claim, enable } | 1.16 |
| stopControl (outputID: int32): void { raises-exception, use after open, claim, enable } | 1.16 |

| Events (UML interfaces) |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Mutability | Version |
| upos::events::DataEvent |  | Not Supported |  |
| upos::events::DirectIOEvent |  |  | 1.16 |
| EventNumber: | int32 | {read-only} |  |
| Data: | int32 | {read-write} |  |
| Obj: | object | {read-write} |  |
| upos::events::ErrorEvent |  |  | 1.16 |
| ErrorCode: | int32 | {read-only} |  |
| ErrorCodeExtended: | int32 | {read-only} |  |
| ErrorLocus: | int32 | {read-only} |  |
| *pErrorResponse: | int32 | {read-write} |  |
| upos::events::OutputCompleteEvent |  |  | 1.16 |
| OutputID: | int32 | {read-only} |  |
| upos::events::StatusUpdateEvent |  |  | 1.16 |
| Status: | int32 | {read-only} |  |

General Information

The Gesture Control device programmatic name is "Gesture Control".

Capabilities

The Gesture Control device has the following capability:

- It controls the behavior of various joint components and parts.

- The operation is automatically controlled by interlocking various joints and other devices.

- Register and play the defined pose and motion.

Gesture Control Class Diagram

The following diagram shows the relationships between the Gesture Control classes.

Fig. Chap. 45-1 Gesture Control Class Diagram

Model

The Gesture Control follows the general device behavior model for asynchronous output devices:

- The application calls a setPosition, setSpeed, startPose, startMotion method to start output. The Device validates the method parameters and produces an error condition immediately if necessary. If the validation is successful, the Device does the following:

･Buffers the request in program memory, for delivery to the Physical Device as soon as the Physical Device can receive and process it.

･Sets the OutputID property to a unique integer identifier for this request.

･Returns as soon as possible.

- When the Device successfully completes a request, an OutputCompleteEvent is enqueued for delivery to the application. A property of this event contains the outputID of the completed request. The application should compare the returned OutputCompleteEvent property OutputID value with the OutputID value set by the asynchronous process method call used to send the data, in order to track what data has been successfully sent to the device.

- If an error occurs while processing a request, an ErrorEvent is enqueued which will be delivered to the application after the events already enqueued, including OutputCompleteEvent. No further asynchronous output will occur until the event has been delivered to the application. If the response is ER_CLEAR, then outstanding asynchronous output is cleared. If the response is ER_RETRY, then output is retried; note that if several outputs were simultaneously in progress at the time that the error was detected, then the Service may need to retry all of these outputs.

- Asynchronous output is always performed on a first-in first-out basis.

- If the request is terminated before completion, due to reasons such as the application calling the clearOutput method, then no OutputCompleteEvent is delivered.

- Application can also delete the output individually by calling the stopControl method. Also, in this case OutputCompleteEvent will not be notified.

- The application will be informed about any status change with a StatusUpdateEvent, also all corresponding status properties will be updated before event delivery.

Automatic control

Automatic control of a joint means to automatically control a joint on the device side, such as tracking according to the movement of a person's face, in cooperation with a camera or the like connected to the device.

The automatic control function is device dependent. For possible automatic control, it is enabled by confirming with the AutoModeList property and setting a value in the AutoMode property.

Pose / Motion

Pose refers to setting the position of one or more defined joints.

For example, it is an action that lifts a hand.

To execute a pose, specify the pose file name by the startPose method or the pose name defined in the device.

Create the pose file with the createPose method described later. Pose defined in the device will be checked in the value of PoseList property.

To execute motion, specify the motion file name or the motion name defined in the device with the startMotion method.

Motion files are created by the createMotion method to be described later. Motion defined in the device can be checked with the value of MotionList property.

To create a pose file, first set the PoseCreationMode property to TRUE and enable the pose registration function. When pose registration function is enabled, each joint is set to the default position. At this time, if the automatic control mode is enabled, the automatic control mode is temporarily invalidated.

Then, an application can create a pose file by setting the value defined as a pose with the setPosition method and calling the createPose method.

A motion file can be created and recorded by specifying the pose defined in the 	       created pose file or the pose defined in the device and creating it as a series of continuously changing actions and calling the createMotion method.

Since the created pose and motion files are recorded in the area may store in either the HardTotals devices or the host file system, or both, and the CapStorageTarget property will show the device’s data file storage location capability.

If device supports either of both HardTotals devices and the host file system, the application should set the StorageTarget property accordingly to tell where to write the data file.

If device needs to be able to write the pose and motion files to a HardTotals device, the CapAssociatedHardTotalsDevice property holds the open name of the associated HardTotals device.

Device Sharing

The Gesture Control Device is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before accessing some properties or calling methods that update the device.

- See the “Summary” table for precise usage prerequisites.

Properties (UML attributes)

AutoMode Property

Syntax	AutoMode: string {read-write, access after open-claim-enable}

Remarks	Indicates automatic control mode ID. Valid values are the empty string "" or one of the AutoModeList properties listed.

If you set one of the properties described in the AutoModeList property for this property, the automatic control mode will be enabled in the set mode.

Setting the empty character "" disables the automatic control mode.

This property is initialized to the empty string "" by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	AutoModeList Property

AutoModeList Property

Syntax	AutoModeList: string {read-only, access after open}

Remarks	Comma-separated list of joint automatic control IDs supported by the device.

For example, in conjunction with the camera, if the mode of tracking the face of a person by moving only the joint of Joint01, this is “FaceTrack_Joint01”.

Another example, in conjunction with the camera, if the mode of tracking the face of a person by moving all joints are supported, this is “FaceTrack_ALL".

(Content and order are dependent on the device.)

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	AutoMode Property.

CapAssociatedHardTotalsDevice Property

Syntax	CapAssociatedHardTotalsDevice : string {read-only, access after open}

Remarks	Holds the open name of the associated HardTotals device if the device is able to write to such devices which is the case if CapStorageTarget is either CST_ALL or CST_HARDTOTALS_ONLY. If CapStorageTarget is CST_HOST_ONLY this property value must be the empty string.

Errors		UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	CapStorageTarget Property

CapMotion Property

Syntax	CapMotion: boolean {read-only, access after open}

Remarks	If true, the device supports making the motion function. Otherwise, it is false.

When this property is false, startMotion method, createMotion method is not available.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	startMotion Method, createMotion Method.

CapMotionCreation Property

Syntax	CapMotionCreation: boolean {read-only, access after open}

Remarks	If true, the device supports motion registration function.

If false, the device does not support motion registration function.

If this property is FALSE, the createMotion method is not available.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	createMotion Method.

CapPose Property

Syntax	CapPose: boolean {read-only, access after open}

Remarks	If true, the device supports pose function. Otherwise, it is false.

When this property is FALSE, value PoseCreationMode property cannot be changed, and startPose method, createPose method are not available.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	PoseCreationMode Property, startPose Method, createPose Method.

CapPoseCreation Property

Syntax	CapPoseCreation: boolean {read-only, access after open}

Remarks	If true, the device supports pose registration function.

If false, the device does not support pose registration function.

If this property is FALSE, you cannot use the createPose method to change the PoseCreationMode property.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	PoseCreationMode Property, createPose Method.

CapStorageTarget Property

Syntax	CapStorageTarget: int32 {read-only, access after open}

Remarks	This is an enumeration and announces where the device is able to write the recorded motion and/or pose data file to.
It holds one of the following values.

Value	Meaning

CST_HARDTOTAL_ONLY	Only an associate HardTotals device is supported.

CST_HOST_ONLY	Only the host’s file system is supported.

CST_ALL	Both, the associated HardTotals device and the host’s file system is supported.

This property is initialized by the open method.

If a HardTotals device is supported the StorageTarget the property value should be CST_HARDTOTAL_ONLY or CST_ALL, and the property CapAssociatedHardTotalsDevice holds the open name of the associated HardTotals device.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	StorageTarget Property, CapAssociatedHardTotalsDevice Property

MotionList Property

Syntax	MotionList: string {read-only, access after open}

Remarks	Comma-separated list of motion IDs defined on the device.

For example, “bowing, welcoming, clapping,…”

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

PoseCreationMode Property

Syntax	PoseCreationMode: boolean {read-write, access after open-claim-enable}

Remarks	If true, pose registration function is enabled.

If false, pose registration function is invalid.

When this property is set to true, pose registration function is enabled. When false is set, the pose registration function is disabled.

This property is initialized to false when you first enable the device after calling the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	CapPose Property, CapPoseCreation Property.

PoseList Property

Syntax	PoseList: string {read-only, access after open}

Remarks	A comma-separated list of pose IDs defined on the device.

For example, “surprise, bow, think,….”

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

StorageTarget Property

Syntax	StorageTarget: int32 {read-write, access after open-claim-enable}

Remarks	This is an enumeration and defines where the device writes the recorded motion and/or pose data file to. Should be set before an appropriate method call.
It holds one of the following values.

Value                                 Meaning

ST_HARDTOTAL	The motion and/or pose data file is written to the associated  HardTotals device. The property CapAssociatedHardTotalsDevice holds the open name of the associated HardTotals device.

ST_HOST              	The motion and/or pose data file is written to the host’s file system.

ST_HOST&HARDTOTAL

The motion and/or pose data file is written to the associated HardTotals device and host’s file system. The property CapAssociatedHardTotalsDevice holds the open name of the associated HardTotals device.

This property is initialized by the open method according to the value hold by CapStorageTarget. If CapStorageTarget has the value CST_ALL it is initialized to ST_HOST.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

Value	Meaning

E_ILLEGAL	An invalid value was specified, or recording is ongoing.

See Also	CapStorageTarget Property, CapAssociatedHardTotalsDevice Property

JointList Property

Syntax	JointList: string {read-only, access after open}

Remarks	Comma-separated list of joint information supported by the device.

Each piece of joint information consists of the following information and is 	shown in the following order, separated by a colon (":").

Parameter   	Description

JointID	Indicates a unique ID in the service that identifies the

joint.

Position range availability. If position range is 0, the

Joint does not have the position range. If position range

is 1, the joint holds the position range.

For example, arm joint has a range of rotation width

but wheel for movement does not have the range of

movement amount.

If there is a device with joints that supports pitch, roll,

yaw and wheels that supports rotating and moving back

and forth. In this case they are indicated as follows.

"Joint01_Pitch:1, Joint01_Roll:1, Joint01_Yaw:1,

Wheel_Turn:0, Wheel_Move:0"

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Table of Gesture Control Device Listed Items in Property

| Property Name | Item ID, File Name, Name | Parameter | Remarks |
| --- | --- | --- | --- |
| AutoModeList | Face Track | Joint01 Joint_ALL |  |
|  | Chase | Joint01, Wheel01, Wheel02 Joint_All, Wheel_ALL, |  |
| MotionList | Bowing, Welcoming, Clapping, Farewelling01, Farewelling02, Greeting01, Greeting02 , |  |  |
| PoseList | Surprise, Bow01, Bow02, Think01, Think02 Doubt01, Doubt02 |  |  |
| JointList | Joint | Pitch Roll Yaw |  |
|  | Wheel | Turn Move Back Move Forth |  |

Methods (UML operations)

createMotion Method

Syntax	createMotion (fileName: string, poseList: string): 
		void {raises-exception, use after open-claim-enable}

Parameter	Description

fileName	Specify the motion file name recorded as motion.

poseList	Specify the comma-separated list of pose information to be registered.

Remarks      A motion file can be created and recorded by specifying the pose defined in the 	       created pose file or the pose defined in the device and creating it as a series of 	       continuously changing actions.  			       			       The place where the motion file is recorded is the area value in the 		       StorageTarget property.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	fileName is too long or contains unusable characters.

E_EXISTS	fileName already exists.

createPose Method

Syntax	     createPose (fileName: string, time: int32): 
          				void {raises-exception, use after open-claim-enable}

Parameter	Description

fileName	Specify the pose file name to record the pose.

time	Specify the time to reach the pose position.

Remarks	Record the position of each joint in the pose file.

Before calling this method, you need to set the PoseCreationMode property to TRUE and enable pose registration mode.

The place where the pose file is recorded is the area value in the StorageTarget property.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	FileName is too long or contains unusable characters. Or PoseCreationMode is FALSE.

E_EXISTS	FileName already exists.

See Also	PoseCreationMode Property.

getPosition Method

Syntax	getPosition (jointID: string, position:out int32 by reference): 
void {raises-exception, use after open-claim-enable}

Parameter	Description

jointID		Specify the one of the joint ID values that are listed in the JointList property.  And specified JointList property should be the position range present one.

position		Store the specified value as the position associated with jointID.

Remarks	Acquires the value of position specified by jointID and stores it as position value.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	JointList Property.

setPosition Method

Syntax	setPosition (positionList: string, time: int32, absolute: boolean): 
		void {raises-exception, use after open-claim-enable}

Parameter	Description

positionList	Specify the position information in a comma-separated list.

time	Specify the time to device control completion in seconds. If this value is too small, it will be changed to an appropriate value depending on the service.

absolute	If true, the specified position indicates the absolute value. If false, the specified position indicates relative value.

Each position information specified in the positionList consists of the following information and is shown in the following order separated by a colon (":").

Parameter	Description

jointID	Specify the joint ID. Specify one of the values listed in the JointList property. However, it must be an ID whose position range is present.

position	Specify the position to be set. Valid values range from -100 to 100.

100 represents the limit value in the positive direction of the target joint, and -100 represents the limit value in the negative direction.

If absolute is a relative value (false) and the value specified here exceeds the limit value, it will be changed to an appropriate value by the service

For example, to move Yow of Joint01 up to the limit of the positive direction and move Pitch of Joint01to the middle, specify as follows.

"Joint01_Yaw:100,Joint01:Pitch:0"

Remarks	The joint position is set with the contents specified in PositionList and device control is started so that device control is completed at the time specified by Time.

Joints that can be specified with this method are only those that have a position range.

Check the JointList property for the presence or absence of the position range.

This method is executed asynchronously. To terminate the operation prematurely, call the stopControl method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	JointList Property, stopControl Method.

setSpeed Method

Syntax	setSpeed (speedList: string, time: int32): 
		void {raises-exception, use after open-claim-enable}

Parameter	Description

speedList		Specify speed information in a comma-separated list.

time		Specify the time to device control in seconds. If you specify FOREVER(-1), it will continue to operate until you call the stopControl method.

Each speed information specified in the SpeedList consists of the following

information, and it is shown in the following order separated by a colon (":").

Parameter	Description

jointID	Specify the joint ID. Specify one of the values listed in the JointList property.

speed	Specify the speed to set. Valid values range from -100 to 100.100 represents the maximum speed in the positive direction of the target joint, and -100 represents the maximum speed in the negative direction.

For example, to move Wheel's X at the maximum speed in the positive direction and Y at the Wheel at half the speed in the negative direction, specify as follows.

"Wheel_X:100, Wheel_Y:-50"

Remarks	It sets the speed of the joint with the contents specified by speedList and performs device control for the time specified by time.

This method is executed asynchronously. To terminate the operation prematurely, call the stopControl method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	JointList Property, stopControl Method.

startMotion Method

Syntax	startMotion (fileName: string): 
		void {raises-exception, use after open-claim-enable}

Parameter	Description

fileName	Prior to start this method, need to specify the name of the motion file or the motion ID value that is listed in the MotionList property.

Remarks	Start the motion defined by fileName or motion defined by the device. This method is executed asynchronously and when the device successfully completes a request, an OutputCompleteEvent is enqueued and a property of corresponding event’s OutputID is placed into the OutputID property.         The application should compare the returned OutputCompleteEvent property outputID value set by this method to track what data has been sent to device.

Motion files are placed in the area as the value of StorageTarget property.

To terminate motion control prematurely, call the stopControl method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

E_NOEXIST	File does not exist.

See Also	MotionList Property.

startPose Method

Syntax	startPose (fileName: string): 
		void {raises-exception, use after open-claim-enable}

Parameter	Description

fileName	Specify the name of the pose file to start. Or one of the pose ID lists listed in the PoseList property.

Remarks	Start the pose defined by the pose file or device specified by fileName. This method is executed asynchronously and when the device successfully completes a request, an OutputCompleteEvent is enqueued and a property of corresponding event’s OutputID is placed into the OutputID property. 	     The application should compare the returned OutputCompleteEvent property OutputID value set by this method to track what data has been sent to device.

Pose files are placed in the area as the values of StorageTarget property.

To terminate pause control prematurely, call the stopControl method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

E_NOEXISTS	File does not exist.

See Also	PoseList Property, stopControl Method.

stopControl Method

Syntax	stopControl (outputID: int32): 
		void {raises-exception, use after open-claim-enable}

Parameter	Description

outputID	Specify the value of the OutputID property you wish to terminate.

Remarks	Stop the control specified for outputID. When device successfully complete the request, and OutputCompleteEvent is enqueued. A property of this event contains the outputID of the completed request. The application should compare the returned OutputCompleteEvent property OutputID value with OutputID value set by this method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	setPosition Method, setSpeed Method, startPose Method, startMotion Method.

Events (UML interfaces)

DirectIOEvent

<<event>>	upos::events::DirectIOEvent

EventNumber	: int32 {read-only}
Data			: int32 {read-write}
Obj			: object {read-write}

Description 	Provides Service information directly to the application. This event provides a means for a vendor-specific Sound Player Service to provide events to the application that are not otherwise supported by the device control.

Attributes	This event contains the following attributes:

Attribute	Type	Description

EventNumber	int32	Event number whose specific values are assigned by the Service.

Data	int32	Additional numeric data. Specific values vary by the EventNumber and the Service. This attribute is settable.

Obj	object	Additional data whose usage varies by the EventNumber and the Service. This attribute is settable.

Remarks	This event is to be used only for those types of vendor specific functions that are not otherwise described.

Use of this event may restrict the application program programform being used with other vendor’s devices which may not have any knowledge of the Service’s need for this event.

See Also	“Events" on page Intro-19, directIO method

ErrorEvent

<<event>> 	upos::events:: ErrorEvent
ErrorCode			: int32{read-write}
ErrorCodeExtended	: int32{read-write}
ErrorLocus 		: int32{read-write}
*pErrorResponse		: int32{read-write}

Description	Notifies the application that a Gesture Control Device error has been detected and suitable response by the application is necessary to process the error condition.

Attributes	This event contains the following attributes:

Attributes	Type	Description

ErrorCode	int32	Error code causing the error event. 				See a list of Error Codes on page 20.

ErrorCodeExtended	int32	Extended Error code causing the error event.

If ErrorCode is E_EXTENDED, then see values below. Otherwise, it may contain a Service-specific value.

ErrorLocus	int32	Location of the error. If EL_OUTPUT is specified. An error occurred during asynchronous action.

ErrorResponse	int32	Pointer to the error event response. See ErrorResponse below for values.

The ErrorLocus attribute has one of the following values:

Value	Meaning

EL_OUTPUT	Error occurred while processing asynchronous output.

EL_INPUT	Error occurred while gathering or processing event-driven input. No previously buffered input data is available.

EL_INPUT_DATA	Error occurred while gathering or processing event-driven input, and some previously buffered data is available.

The application’s error event handler can set the ErrorResponse attribute to one of the following values:

Value	Meaning

ER_RETRY	Retry sending the data. The error state is exited.
May be valid for some input devices when the locus is EL_INPUT, in which case the input is re-tried, and the error state is exited. Typically, valid for asynchronous output devices when the locus is EL_OUTPUT, in which case the asynchronous output is re-tried, and the error state is exited. This is the default response when the locus is EL_OUTPUT.

ER_CLEAR	Valid for all loci: EL_INPUT, EL_INPUT_DATA, and EL_OUTPUT.  Clear all buffered input or output data (including all asynchronous output).  The error state is exited.  This is the default response when the locus is EL_INPUT.

ER_CONTINUEINPUT 
Only valid when the locus is EL_INPUT_DATA.
Acknowledges that a data error has occurred and directs the Device to continue input processing. The Device remains in the error state and will deliver additional DataEvents as directed by the DataEventEnabled property. When all input has been delivered and DataEventEnabled is again set to true, then another ErrorEvent is delivered with locus EL_INPUT.
This is the default response when the locus is EL_INPUT_DATA.

Remarks	This event is enqueued when an error is detected and the Device’s State transitions into the error state. Input error events are not delivered until DataEventEnabled is true, so that proper application sequencing occurs.

Unlike a DataEvent, the Device does not disable further DataEvents or input ErrorEvents; it leaves the DataEventEnabled property value at true.     Note that the application may set DataEventEnabled to false within its event handler if subsequent input events need to be disabled for a period time.

See Also 	“Device Input Model" on page Intro-22, “Error Handling" on page Intro-23, “Device Output Models" on page Intro-25.

OutputCompleteEvent

<<event>>	upos::events::OutputCompleteEvent
		OutputID: int32{read-only}

Description 	Notify the application that the queued output request associated with the outputID property has completed successfully.

Attributes	This event contains the following attributes:

Attribute	Type	Description

OutputID 	int32	The ID number of the asynchronous output request that is complete.

Remarks	This event is enqueued after the request’s data has been both sent, and the Service has confirmation that it was processed by the device successfully.

See Also	“Device Output Models" on page Intro-25

StatusUpdateEvent

<<event>>	upos::events:: StatusUpdateEvent
		Status : int32 {read-only}

Description	Notifies the application that there is an operation status change or a status of the Gesture Control device.

Attributes		This event contains the following attribute:

Attributes	Type	   Description

Status                int32        Indicates a change of operation status of sound player device

.	Note that Release 1.3 added Power State Reporting with additional Power  reporting StatusUpdateEvent values.

The Update Firmware capability added additional Status values for communicating the status/progress of an asynchronous update firmware process. See “StatusUpdateEvent” description on page 1-34.

Value            		Meaning

GCTL_SUE_START_MOTION									It will be notified when Gesture Motion  start.

GCTL_SUE_STOP_MOTION										It will be notified when Gesture Motion stop.

Remarks	Enqueued when the Gesture Control Device detects a power state change or a status change.

See Also 	“Events" on page Intro-19.

Chapter 46

Device Monitor

This Chapter defines the Device Monitor device category.

Summary

| Pro Properties (UML attributes) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Common |  | Type |  | Mutability |  | Version |  | May Use After |  |
| AutoDisable: |  | boolean |  | {read-write} |  | 1.16 |  | open |  |
| CapCompareFirmwareVersion: |  | boolean |  | {read-only} |  | 1.16 |  | open |  |
| CapPowerReporting: |  | int32 |  | {read-only} |  | 1.16 |  | open |  |
| CapStatisticsReporting: |  | boolean |  | {read-only} |  | 1.16 |  | open |  |
| CapUpdateFirmware: |  | boolean |  | {read-only} |  | 1.16 |  | open |  |
| CapUpdateStatistics: |  | boolean |  | {read-only} |  | 1.16 |  | open |  |
| CheckHealthText: |  | string |  | {read-only} |  | 1.16 |  | open |  |
| Claimed: |  | boolean |  | {read-only} |  | 1.16 |  | open |  |
| DataCount: |  | int32 |  | {read-only} |  | 1.16 |  | open |  |
| DataEventEnabled: |  | boolean |  | {read-write} |  | 1.16 |  | open |  |
| DeviceEnabled: |  | boolean |  | {read-write} |  | 1.16 |  | open & claim |  |
| FreezeEvents: |  | boolean |  | {read-write} |  | 1.16 |  | open |  |
| OutputID: |  | int32 |  | {read-only} |  | 1.16 |  | Not Supported |  |
| PowerNotify: |  | int32 |  | {read-write} |  | 1.16 |  | open |  |
| PowerState: |  | int32 |  | {read-only} |  | 1.16 |  | open |  |
| State: |  | int32 |  | {read-only} |  | 1.16 |  | -- |  |
| DeviceControlDescription: |  | string |  | {read-only} |  | 1.16 |  | -- |  |
| DeviceControlVersion: |  | int32 |  | {read-only} |  | 1.16 |  | -- |  |
| DeviceServiceDescription: |  | string |  | {read-only} |  | 1.16 |  | open |  |
| DeviceServiceVersion: |  | int32 |  | {read-only} |  | 1.16 |  | open |  |
| PhysicalDeviceDescription: |  | string |  | {read-only} |  | 1.16 |  | open |  |
| PhysicalDeviceName: |  | string |  | {read-only} |  | 1.16 |  | open |  |

| Properties (Continued) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Specific | Type | Mutability |  | Version |  | May Use After |  |
| DeviceData: | string | {read-only} |  | 1.16 |  | open, claim & enable |  |
| DeviceList: | string | {read-only} |  | 1.16 |  | open |  |
| MonitoringDeviceList: | string | {read-only |  | 1.16 |  | open, claim & enable |  |
| Me Methods (UML operations) |  |  |  |  |  |  |  |
| Common |  |  |  |  |  |  |  |
| Name |  |  |  |  |  |  | Version |
| open (logicalDeviceName: string): void {raises-exception} |  |  |  |  |  |  | 1.16 |
| close ( ): void {raises-exception, use after open} |  |  |  |  |  |  | 1.16 |
| claim (timeout: int32): void {raises-exception, use after open} |  |  |  |  |  |  | 1.16 |
| release ( ): void {raises-exception, use after open, claim} |  |  |  |  |  |  | 1.16 |
| checkHealth (level: int32): void {raises-exception, use after open, enable} |  |  |  |  |  |  | 1.16 |
| clearInput ( ): void { } |  |  |  |  |  |  | 1.16 |
| clearInputProperties ( ): void { } |  |  |  |  |  |  | 1.16 |
| clearOutput ( ): void { } |  |  |  |  |  |  | Not supported |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, enable} |  |  |  |  |  |  | 1.16 |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} |  |  |  |  |  |  | 1.16 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} |  |  |  |  |  |  | 1.16 |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, enable} |  |  |  |  |  |  | 1.16 |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, enable} |  |  |  |  |  |  | 1.16 |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} |  |  |  |  |  |  | 1.16 |
| Specific |  |  |  |  |  |  |  |
| addMonitoringDevice (deviceID: string, monitoringMode: int32, boundary: int32, subBoundary: int32, intervalTime: int32 ): void {raises-exception, use after open, claim, enable} |  |  |  |  |  |  | 1.16 |
| deleteMonitoringDevice (deviceID: string ): void {raises-exception, use after open, claim, enable} |  |  |  |  |  |  | 1.16 |
| clearMonitoringDevices ( ): void {raises-exception, use after open, claim, enable} |  |  |  |  |  |  | 1.16 |
| getDeviceValue (deviceID: string, inout value: int32) void {raises-exception, use after open} |  |  |  |  |  |  | 1.16 |
| Events (UML interfaces) |  |  |  |  |  |  |  |
| Name | Type |  | Mutability |  | Version |  |  |
| upos::events::DataEvent |  |  |  |  | 1.16 |  |  |
| Status: | int32 |  | {read-only} |  |  |  |  |
| upos::events::DirectIOEvent |  |  |  |  | 1.16 |  |  |
| EventNumber: | int32 |  | {read-only} |  |  |  |  |
| Data: | int32 |  | {read-write} |  |  |  |  |
| Obj: | object |  | {read-write} |  |  |  |  |
| upos::events::ErrorEvent |  |  |  |  | 1.16 |  |  |
| ErrorCode: | int32 |  | {read-only} |  |  |  |  |
| ErrorCodeExtended: | int32 |  | {read-only} |  |  |  |  |
| ErrorLocus: | int32 |  | {read-only} |  |  |  |  |
| ErrorResponse: | int32 |  | {read-write} |  |  |  |  |
| upos::events::OutputCompleteEvent |  |  | Not Supported |  |  |  |  |
| upos::events::StatusUpdateEvent |  |  |  |  | 1.16 |  |  |
| Status: | int32 |  | {read-only} |  |  |  |  |

General Information

The Device Monitor programmatic name is "Device Monitor".

Capabilities

The Device Monitor Device has the following capability:

- Get values measured by various devices.

- Notify the application of changes in values measured by various devices.

Device Monitor Class Diagram

The following diagram shows the relationships between the Device Monitor classes.

Fig. Chap. 46-1 Device Monitor Class Diagram

Model

The Device Monitor follows the general “Device Input Model” for event-driven input:

- The Device Monitor supports monitoring of values measured by multiple devices connected to the device. A device that can be monitored and its type / value unit is listed in the DeviceList property.

- Device Monitor receives a change in the value measured by the device set as the monitoring target, and generates a DataEvent when it matches the specified condition.

- To add a device to be monitored, specify the monitoring mode with the addMonitoringDevice method and add it. For details on monitoring mode, see the description of addMonitoringDevice method.

- If the AutoDisable property is true, the device will automatically disable itself when a DataEvent is enqueued.

- An enqueued DataEvent can be delivered to the application when the DataEventEnabled property is true and other event delivery requirements are met. Just before delivering this event, data is copied into corresponding properties, and further data events are disabled by setting DataEventEnabled to false. This causes subsequent input data to be enqueued while the application processes the current input and associated properties. When the application has finished processing the current input and is ready for more data, it reenables events by setting DataEventEnabled to true.

- An ErrorEvent (or events) is enqueued if an error occurs while gathering or processing input, and is delivered to the application when DataEventEnabled is true and other event delivery requirements are met.

- The DataCount property can be read to obtain the total number of enqueued DataEvents.

- All enqueued input may be deleted by calling ClearInput method. See the ClearInput method description for more details.

- All data properties that are populated as a result of firing a DataEvent or ErrorEvent can be set back to their default values by calling the clearInputProperties method.

- The notified data is stored in the DeviceData property.

- In the Device Monitor device control, the measured values of the devices are managed most of cases with the int32 type integers, but some are decimals. In that case, the decimals are implicit, and the actual value can be calculated by dividing the measured value by the coefficient of each device that can be obtained in the DeviceList property.

- The application will be informed about any status change with a StatusUpdateEvent,

also, all corresponding status properties will be updated before event delivery.

Device Sharing

The Device Monitor is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before the device begins reading input, or before calling methods that manipulate the device.

See the “Summary” table for precise usage prerequisites.

Properties (UML attributes)

DeviceData Property

Syntax	DeviceData: string {read-only, access after open-claim-enable}

Remarks	Measurement information of the device that matches the condition registered by addMonitoringDevice method is set.

Each measurement information consists of the following information and is shown in the following order, separated by a colon (":").

Parameter	Description

DeviceID	The target device ID.

Measured value		Measurement value of the device. The measured value is represented by an integer type. To convert it to an actual value, divide the measured value by the 	coefficient acquired by the DeviceList property.

For example,"Device01:365"

Its value is set prior to a DataEvent being delivered  to the application.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

DeviceList Property

Syntax	DeviceList: string {read-only, access after open}

Remarks	Contains the comma-delimited list of device information that are supported by the device.

Each object information consists of the following information and is shown in the following order, separated by a colon (":").

Parameter	Description

DeviceID	Indicates a unique ID in the service that identifies the device.

Type	Indicates the device type. For example, if it is a touch sensor it is expressed as "Touch Sensor" and so on. However, this value depends on the service.

Unit	Indicates the unit of value held by various devices. For example, it is expressed as "on / off" for a touch sensor, "rad / s" for a gyroscope. However, this value depends on the service.

Coefficient	Indicates the coefficient for calculating the actual measured value held by various devices. The DeviceData property and the measured value of the device that can be obtained with the GetDeviceValue method are expressed as integers, but by dividing this value by the coefficient it is the actual value.  Example: Device value = 365, coefficient = 10, actual value = 36.5			     For example, if one device supports one touch sensor and one gyroscope, it will be as follows. "Touch 01: Touch Sensor: ON/OFF: 1, GyroX: Gyroscope: rad/s: 100000, GyroY: Gyroscope: rad/s: 100000, GyroZ: Gyroscope: rad/s: 100000"

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	DeviceData Property, addMonitoringDevice Method, getDeviceValue Method.

MonitoringDeviceList Property

Syntax	MonitoringDeviceList: string {read-only, access after open-claim-enable}

Remarks	Contains the comma-delimited list of monitoring information on registered devices that are supported by the device.

Each monitoring information consists of the following information and is shown in the following order, separated by a colon (":").

Parameter	Description

DeviceID	Registered devices ID.

Monitoring mode	Registered monitoring mode.

Boundary	Registered boundary value. This value is set to 0 when the monitoring mode does not require a boundary value.

Sub boundary		Registered sub boundary value. This value is set to 0 	when the monitoring mode does not require a sub 	boundary value.

Interval		Registered interval. (millisecond)

For example, if you set monitoring targets as follows,

[Monitor target 1]

Device ID = Device 01, monitoring mode = DMON_MM_UPDATE,

boundary line = 0, sub boundary line = 0, interval time = 0

[Monitor target 2]

Device ID = Device 02, monitoring mode = DMON_MM_STRADDLED,

boundary line = 365, sub boundary line = 0, interval time = 500

The values shown are as follows.

"Device01:0:0:0:0, Device02:1:365:0:500"

This property is initialized by the open method. It is also updated by calling

addMonitoringDevice method, deleteMonitoringDevice method,

clearMonitoringDevice method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	addMonitoringDevice Method, deleteMonitoringDevice Method, clearMonitoringDevice Method.

Methods (UML operations)

addMonitoringDevice Method

Syntax	addMonitoringDevice (deviceID:string, monitoringMode:string, boundary:int32, subBoundary:int32, intervalTime:int32):
		void{raises-exception, use after open-claim-enable}

Parameter	Description

deviceID	The deviceID of the monitored device. Valid values 		are one of the device ID lists listed in the DeviceList 		property.

monitoringMode	Specify the monitoring mode for monitoring.

boundary	Specify the boundary value to be monitored.

subBoundary	Specify the sub boundary value to be monitored. 			This value must be less than Boundary. 	intervalTime	Specify the interval in milliseconds between the 			occurrence of the event and the start of the next 			monitoring.
		The monitoring modes specified for MonitoringMode 			are as follows.

Value	Description

DMON_MMODE_UPDATE
		Every time the measured value of the target device is 		updated, an event is notified. When set to this mode, 		the values of the argument boundary and 			subBoundary are ignored.

DMON_MMODE_STRADDLED
		When the measured value of the target device 			crosses the value of the argument boundary, 			it notifies the event. In addition, when the measured		value matches the value of boundary, it notifies the		event even when it changes from the matched state. 		When set to this mode, the value of the argument		subBoundary is ignored.

DMON_MMODE_HIGH
		When the measured value of the target device			becomes equal to or larger than the value of the		argument Boundary, it notifies the event. 			Even if the measured value is updated and it was 		again equal to or greater than the value of 			boundary, we will notify the event each time. 			When set to this mode, the value of the 			argument subBoundary is ignored.

DMON_MMODE_LOW
			          Notifies the event when the measured value of the 			          target device becomes less than or equal to the   	                                     value of the argument boundary. Even when the 			          measured value is updated and it was again less 			          than the value of boundary, we will notify the event 			          each time.  When set to this mode, the value of the 			          argument subBoundary is ignored.

DMON_MMODE_WITHIN
			           It notifies the event while the measured value of 			           the target device is within the range specified by 			           the argument boundary and subBoundary.
			           Even if the measured value is updated and its value 			           is	 within the range again, the event is notified each 			           time.

DMON_MMODE_OUTSIDE
			           It notifies the event while the measured value of 			           the target device is outside the range specified 			           by the argument boundary and subBoundary.
			           Even if the measured value is updated and its value 			           was out of range again, we will notify the event                      			           each time.

DMON_MMODE_POLLING
			           It notifies the measured value of the target device 			           at the interval specified by intervalTime.
			           When set to this mode, the values of the argument			            boundary and subBoundary are ignored.

Remarks	Add the device specified by deviceID to the monitoring target.
The monitoring mode is specified for monitoringMode, but there are monitoring modes not supported by some devices. In that case, E_ILLEGAL is raised as the UPOS exception.
Devices added by this method will be added to the list of MonitoringDeviceList properties. If a device to be monitored is specified, it will be changed to a new condition. To exclude the added device from the monitoring target, call deleteMonitoringDevice method or clearMonitoringDevice method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.  Some possible values of the exception’s ErrorCode property are:

Value                	Description

E_ILLEGAL	An invalid value was specified, or unsupported 						operation with the Device

See Also	DeviceList Property, MonitoringDeviceList Property, deleteMonitoringDevice Method, clearMonitoringDevice Method, DataEvent.

deleteMonitoringDevice Method

Syntax	deleteMonitoringDevice (deviceID: string):
		void{raises-exception, use after open-claim-enable}

Parameter		Description

deviceID		Specify the device ID of the device to be excluded from 			monitoring targets.

Remarks	Exclude the device specified by deviceID from monitoring targets.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.
Some possible values of the exception’s ErrorCode property are:

Value		Description

E_ILLEGAL	An invalid value was specified, or unsupported operation

with the Device.

See Also	AddMonitoringDevice Method.

clearMonitoringDevices Method

Syntax	clearMonitoringDevices ():
		void{raises-exception, use after open-claim-enable}

Remarks	Exclude all devices to be monitored.

Errors	A UposException may be thrown when this method is invoked.
For further information, see “Errors" on page Intro-20.

See Also	addMonitoringDevice Method.

getDeviceValue method

Syntax		getDeviceValue (deviceID: string, pValue: *int32):
		void{raises-exception, use after open}

Parameter	Description

deviceID		Specify the device ID of the device from which the 			measurement value is to be acquired. Specify one of the 			device ID lists listed in the DeviceList property.

pValue 		Pointer that stores measurement values obtained from the 			device.

Remarks		Get the measured value of the device specified by deviceID. The retrieved value is stored in pValue.

Errors		A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.
Some possible values of the exception’s ErrorCode property are:

Value		Description

E_ILLEGAL	An invalid value was specified, or unsupported operation with the Device.

See Also		DeviceList Property.

Events (UML interfaces)

DataEvent

<<event>>	upos::events::DataEvent

Status			: int32{read-only}

Description 	Notifies the application when data from the Device Monitor device is available to be read.

Attributes	This event contains the following attributes:

Attribute	Type		Description

Status	int32		Set to 0.

Remarks	Before this event is delivered, the individual recognition information is enqueued into the area that is indicated by the addMonitoringDevice method.

See Also	addMonitoringDevice method.

DirectIOEvent

<<event>>	upos::events::DirectIOEvent

EventNumber	: int32 {read-only}				           Data			: int32 {read-write}
Obj			: object {read-write}

Description 	Provides Service information directly to the application. This event provides a means for a vendor-specific Device Monitor Device Service to provide events to the application that are not otherwise supported by the device control.

Attributes	This event contains the following attributes:

Attribute	Type	Description

EventNumber	int32	Event number whose specific values are assigned by the Service.

Data	int32	Additional numeric data. Specific values vary by the EventNumber and the Service. This attribute is settable.

Obj	object	Additional data whose usage varies by the EventNumber and the Service. This attribute is settable.

Remarks	This event is to be used only for those types of vendor specific functions that are not otherwise described.

Use of this event may restrict the application program programform being used with other vendor’s devices which may not have any knowledge of the Service’s need for this event.

See Also	“Events" on page Intro-19, directIO method

ErrorEvent

<<event>> 	upos::events:: ErrorEvent
ErrorCode			: int32{read-write}
ErrorCodeExtended	: int32{read-write}
ErrorLocus 		: int32{read-write}
*pErrorResponse		: int32{read-write}

Description	Notifies the application that a Device Monitor Device error has been detected and suitable response by the application is necessary to process the error condition.

Attributes	This event contains the following attributes:

Attributes	Type	Description

ErrorCode	int32	Error code causing the error event. 				See a list of Error Codes on page 20.

ErrorCodeExtended	int32	Extended Error code causing the error event.

If ErrorCode is E_EXTENDED, then see values below. Otherwise, it may contain a Service-specific value.

ErrorLocus	int32	Location of the error. If EL_OUTPUT is specified. An error occurred during asynchronous action.

ErrorResponse	int32	Pointer to the error event response. See ErrorResponse below for values.

The ErrorLocus attribute has one of the following values:

Value	Meaning

EL_OUTPUT	Error occurred while processing asynchronous output.

EL_INPUT	Error occurred while gathering or processing event-driven input. No previously buffered input data is available.

EL_INPUT_DATA	Error occurred while gathering or processing event-driven input, and some previously buffered data is available.

The application’s error event handler can set the ErrorResponse attribute to one of the following values:

Value	Meaning

ER_RETRY	Retry sending the data. The error state is exited.
May be valid for some input devices when the locus is EL_INPUT, in which case the input is re-tried, and the error state is exited. Typically, valid for asynchronous output devices when the locus is EL_OUTPUT, in which case the asynchronous output is re-tried, and the error state is exited. This is the default response when the locus is EL_OUTPUT.

ER_CLEAR	Valid for all loci: EL_INPUT, EL_INPUT_DATA, and EL_OUTPUT.  Clear all buffered input or output data (including all asynchronous output).  The error state is exited.  This is the default response when the locus is EL_INPUT.

ER_CONTINUEINPUT 
Only valid when the locus is EL_INPUT_DATA.
Acknowledges that a data error has occurred and directs the Device to continue input processing. The Device remains in the error state and will deliver additional DataEvents as directed by the DataEventEnabled property. When all input has been delivered and DataEventEnabled is again set to true, then another ErrorEvent is delivered with locus EL_INPUT.
This is the default response when the locus is EL_INPUT_DATA.

Remarks	This event is enqueued when an error is detected and the Device’s State transitions into the error state. Input error events are not delivered until DataEventEnabled is true, so that proper application sequencing occurs.

Unlike a DataEvent, the Device does not disable further DataEvents or input ErrorEvents; it leaves the DataEventEnabled property value at true.    Note that the application may set DataEventEnabled to false within its event handler if subsequent input events need to be disabled for a period of time.

See Also 	“Device Input Model" on page Intro-22, “Error Handling" on page Intro-23, “Device Output Models" on page Intro-25.

StatusUpdateEvent

<<event>>	       upos::events:: StatusUpdateEvent
                                    Status			: int32 {read-only}

Description	Notifies the application that there is an operation status change or a status of the Device Monitor device.

Attributes		This event contains the following attribute:

Attributes        Type	Description

Status               int32	Indicates a change in the Device Monitor status 	of  the unit.

Note that Release 1.3 added Power State Reporting with additional Power 	reporting StatusUpdateEvent values.

The Update Firmware capability added additional Status values for communicating the status/progress of an asynchronous update firmware process. See “StatusUpdateEvent” description on page 1-34.

Value            		Meaning

DMON_SUE_START_MONITERING
						It will be notified when Device Monitoring start.

DMON_SUE_STOP_MONITORING
						It will be notified when Device Monitoring stop.

Remarks	      Enqueued when the Device Monitor Device detects a power state change or a status change.

See Also 	“Events" on page Intro-19.

Chapter 47

Graphic Display

This Chapter defines the Graphic Display device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.16 | open |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.16 | open |
| CapPowerReporting: | int32 | {read-only} | 1.16 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.16 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.16 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.16 | open |
| CheckHealthText: | string | {read-only} | 1.16 | open |
| Claimed: | boolean | {read-only} | 1.16 | open |
| DataCount: | int32 | {read-only} | 1.16 | open |
| DataEventEnabled: | boolean | {read-write} | 1.16 | open |
| DeviceEnabled: | boolean | {read-write} | 1.16 | open, & claim |
| FreezeEvents: | boolean | {read-write} | 1.16 | open |
| OutputID: | int32 | {read-only} | 1.16 | open |
| PowerNotify: | int32 | {read-write} | 1.16 | open |
| PowerState: | int32 | {read-only} | 1.16 | open |
| State: | int32 | {read-only} | 1.16 | -- |
| DeviceControlDescription: | string | {read-only} | 1.16 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.16 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.16 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.16 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.16 | open |
| PhysicalDeviceName: | string | {read-only} | 1.16 | open |

| Properties (Continued) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Specific | Type | Mutability | Version | May Use After |  |  |
| CapAssociatedHardTotalsDevice: | string | {read-only} | 1.16 | open |  |  |
| CapBrightness: | boolean | {read-only} | 1.16 | open |  |  |
| CapImageType: | boolean | {read-only} | 1.16 | open |  |  |
| CapStorageTarget: | int32 | {read-only} | 1.16 | open |  |  |
| CapURLBack: | boolean | {read-only} | 1.16 | open |  |  |
| CapURLForward: | boolean | {read-only} | 1.16 | open |  |  |
| CapVideoType: | boolean | {read-only} | 1.16 | open |  |  |
| CapVolume: | boolean | {read-only} | 1.16 | open |  |  |
| Brightness: | int32 | {read-write} | 1.16 | open, claim & enable |  |  |
| DisplayMode: | int32 | {read-write} | 1.16 | open, claim & enable |  |  |
| ImageType: | int32 | {read-write} | 1.16 | open, claim & enable |  |  |
| ImageTypeList: | string | {read-only} | 1.16 | open |  |  |
| LoadStatus: | int32 | {read-only} | 1.16 | open |  |  |
| StorageTarget: | int32 | {read-write} | 1.16 | open, claim & enable |  |  |
| URL: | string | {read-only} | 1.16 | open |  |  |
| VideoType: | int32 | {read-write} | 1.16 | open, claim & enable |  |  |
| VideoTypeList: | string | {read-only} | 1.16 | open |  |  |
| Volume: | int32 | {read-write} | 1.16 | open, claim & enable |  |  |
| Methods (UML operations) |  |  |  |  |  |
| Common |  |  |  |  |  |
| Name |  |  |  |  | Version |
| open (logicalDeviceName: string): void {raises-exception} |  |  |  |  | 1.16 |
| close ( ): void {raises-exception, use after open} |  |  |  |  | 1.16 |
| claim (timeout: int32): void {raises-exception, use after open} |  |  |  |  | 1.16 |
| release ( ): void {raises-exception, use after open, claim} |  |  |  |  | 1.16 |
| checkHealth (level: int32): void {raises-exception, use after open, enable} |  |  |  |  | 1.16 |
| clearInput ( ): void { } |  |  |  |  | 1.16 |
| clearInputProperties ( ): void { } |  |  |  |  | 1.16 |
| Methods (UML operations)(Continued) |  |  |  |  |  |
| clearOutput ( ): void { } |  |  |  |  | 1.16 |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, enable} |  |  |  |  | 1.16 |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} |  |  |  |  | 1.16 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} |  |  |  |  | 1.16 |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, enable} |  |  |  |  | 1.16 |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, enable} |  |  |  |  | 1.16 |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} |  |  |  |  | 1.16 |
| Specific |  |  |  |  |  |
| Name |  |  |  |  | Version |
| cancelLoading ( ): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| goURLBack ( ): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| goURLForward ( ): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| loadImage (fileName: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| loadURL (uRL: string ): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| playVideo (fileName: string, loop: boolean): void { raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| stopVideo ( ): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| updateURLPage ( ): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |

| Events (UML interfaces) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name |  | Type | Mutability | Version |
| upos::events::DataEvent |  |  |  |  |
| Status: |  |  | Not Supported |  |
| upos::events::DirectIOEvent |  |  |  | 1.16 |
| EventNumber: |  | int32 | {read-only} |  |
| Data: |  | int32 | {read-write} |  |
| Obj: |  | object | {read-write} |  |
| upos::events::ErrorEvent |  |  |  | 1.16 |
| ErrorCode: |  | int32 | {read-only} |  |
| ErrorCodeExtended: |  | int32 | {read-only} |  |
| ErrorLocus: |  | int32 | {read-only} |  |
| ErrorResponse |  | int32 | {read-write} |  |
| upos::events::OutputCompleteEvent |  |  |  | 1.16 |
| OutputID: | int32 | {read-only} |  |  |
| upos::events::StatusUpdateEvent |  |  |  | 1.16 |
| Status: |  | int32 | {read-only} |  |

General Information

The Graphic Display programmatic name is “Graphic Display”.

Capabilities

The Graphic Display has the following capability:

Displays the specified image files.

Play the specified video.

Display the specified web page.

Notify the application of changes in the load status of the web page.

Graphics Display Class Diagram

The following diagram shows the relationships between the Graphic Display classes.

Fig. Chap. 47-1 Graphic Display Class Diagram

Model

The following display modes exist in the graphics control, and the model differs depending on the display mode:

- Image display mode

- Video display mode.

- Web display mode.

The application can change the display mode by changing the value of the DisplayMode property.

Image Display Mode

The image display mode of the graphics control is as follows. 				        Prior to start this mode, need to set the appropriate image type file value in the 	      	        ImageType property from the listed values in the ImageTypeList property, if 		        CapImageType property is true.  Then the application can call the loadImage method  	        to display the image.

Raises StatusUpdateEvent at the status change timing of image load start with status  GDSP_SUE_START_IMAGE_LOAD,  and image load end with status GDSP_SUE_END_IMAGE_LOAD. The ImageTypeList property lists image files that the device can display.

Applications may need to support “Hard Totals” services as image files displaying with loadImage method might be placed in the area managed by the associated “Hard Totals" service device. If the CapStorageTarget is either CST_ALL or CST_HARDTOTALS_ONLY, it is possible to store it in the Associated Hard Totals device and storage device’s open name is held in the CapAssociatedHardTotalsDevice property.

If device supports both Hard Totals device and the host file system, the application should set the StorageTarget property accordingly to tell where to write the image data file.

Video Display Mode

The video display mode of Graphic Display follows the general device behavior model for asynchronous output devices. The graphics control of video display modes are as follows.

Prior to start this mode, need to set the appropriate video type file value in the VideoType property from the listed values in the VideoTypeList property, if CapVideoType property is true. 						          Then the application can call the playVideo method to display the video. Also, the video being displayed is stopped by calling the stopVideo method.

Raises StatusUpdateEvent at the status change timing of start play video with status  GDSP_SUE_START  PLAY_VIDEO, and stop play video with status GDSP_SUE_STOP_PLAY_VIDEO.

The Device validates the method parameters an error condition immediately if necessary. If the validation is successful, the Device does the following:

• 	Buffers the request in program memory, for delivery to the Physical Device as soon as the Physical Device can receive and process it.

• 	Sets the OutputID property to a unique integer identifier for this request.

• 	Returns as soon as possible.

When the Device successfully completes a request, an OutputCompleteEvent is enqueued for delivery to the application.

A property of this event contains the output ID of the completed request.

The application should compare the returned OutputCompleteEvent property OutputID value with the OutputID value set by the asynchronous process method call used to send the data in order to track what data has been successfully sent to the device.

If an error occurs while processing a request, an ErrorEvent is enqueued which will be delivered to the application after the events already enqueued, including OutputCompleteEvents. No further asynchronous output will occur until the event has been delivered to the application. If the response is ER_CLEAR, then outstanding asynchronous output is cleared.

If the response is ER_RETRY, then output is retried; note that if several outputs were simultaneously in progress at the time that the error was detected, then	the Service may need to retry all of these outputs.

Asynchronous output is always performed on a first-in first-out basis. If the device supports concurrent playback, the request will be executed simultaneously. To check if the device supports simultaneous playback, check the CapMultiPlay property.

If the request is terminated before completion, due to reasons such as the application calling the clearOutput method, then no OutputCompleteEvent is delivered. You can also delete the output individually by calling the stopVideo method. Also, in this case OutputCompleteEvent will not be notified.

The video files that the device can display are listed in the VideoTypeList property.

Since video files to be displayed using the playVideo method must be placed in an area managed by the associated “Hard Totals" service device. If the CapStorageTarget is either CST_ALL or CST_HARDTOTALS_ONLY, it is possible to store it in the Associated Hard Totals device and storage device’s open name is held in the CapAssociatedHardTotalsDevice property.

If device supports either or both Hard Totals device and the host file system, the application should set the StorageTarget property accordingly to tell where to write the image data file.

The video display mode of graphics control follows an asynchronous output model.

Raises StatusUpdateEvent if Graphic Display device power status or a device status changes are occurred during the video displaying.

Web Display Mode

The web display mode of graphics control is as follows.

The application calls the loadURL method to display the web page.

Raises StatusUpdateEvent at the timing of Web page load start with status GDSP_SUE_START_LOAD_WEBPAGE, load finish with status GDSP_SUE_FINISH_LOAD_WEBPAGE, and load cancel with status GDSP_SUE_CANCEL_LOAD_WEBPAGE. And application can detect the web page loading status.

The latest loading status of the web page is stored in the LoadStatus property when loadURL method is called, and its URL information is stored in the URL property.

In case when cancelLoading method is called during the loading process, current accessed URL information will be stored in the URL property.

The graphics control web display mode follows an asynchronous output model.

Device Sharing

The Graphic Display Device is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before accessing some properties or calling methods that update the device.

See the “Summary” table for precise usage prerequisites.

Properties (UML attributes)

Brightness Property

Syntax	Brightness: int32 {read-write, access after open-claim-enable}

Remarks	Holds the brightness of screen. Legal values range from zero through 100.
This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	CapBrightness Property.

CapAssociatedHardTotalsDevice Property

Syntax	CapAssociatedHardTotalsDevice : string {read-write, access after open}

Remarks	Holds the open name of the associated HardTotals device if the device is able to write to such devices which is the case if CapStorageTarget is either CST_ALL or CST_HARDTOTALS_ONLY. If CapStorageTarget is CST_HOST_ONLY this property value must be the empty string.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	CapStorageTarget Property

CapBrightness Property

Syntax	CapBrightness: boolean {read-only, access after open}

Remarks	If true, the application can change the screen brightness.
If false, the application cannot change the screen brightness.
This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	Brightness Property.

CapImageType Property

Syntax	CapImageType: boolean {read-only, access after open}

Remarks	If true, indicate the image type file to be used in this target device as the value of the ImageType property. Otherwise it is false.  This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	ImageType Property, ImageTypeList Property

CapStorageTarget Property

Syntax	CapStorageTarget: int32 {read-only, access after open}

Remarks	This is an enumeration and announces where the device is able to write the image data file to.
It holds one of the following values.

Value	Meaning

CST_HARDTOTAL_ONLY	Only an associate HardTotals device is supported.

CST_HOST_ONLY	Only the host’s file system is supported.

CST_ALL	Both, the associated HardTotals device and the host’s file system is supported.

This property is initialized by the open method.

If a HardTotals device is supported the storage target the property value should be CST_HARDTOTAL_ONLY or CST_ALL, and the property CapAssociatedHardTotalsDevice holds the open name of the associated HardTotals device.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	StorageTarget Property, CapAssociatedHardTotalsDevice Property

CapURLBack Property

Syntax	CapURLBack: boolean {read-only, access after open}

Remarks	If true, the previous page exists in the browsing history. Application can return to the previous page with goURLBack method.

If false, there is no previous page in the browsing history.

This property is initialized to false by the open method. Also, as the web page loading state changes, it is set by the device control.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	goURLBack Method.

CapURLForward Property

Syntax	CapURLForward: boolean {read-only, access after open}

Remarks	If true, the next page exists in the browsing history. Application can go to the next page with the goURLForward method.
If false, there is no next page in the browsing history.
This property is initialized to false by the open method. Also, as the web page loading state changes, it is set by the device control.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	goURLForward Method.

CapVideoType Property

Syntax	CapVideoType: boolean {read-only, access after open}

Remarks	If true, indicate the vide type value that can be used in this targeted graphics display device as the value of VideoType Property. Otherwise, it is false.            This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	VideoType Property, VideoTypeList Property .

CapVolume Property

Syntax	CapVolume: boolean {read-only, access after open}

Remarks	If true, the application can change the volume of video.
If false, the application cannot change the volume of video.
This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	Volume Property.

DisplayMode Property

Syntax	DisplayMode: int32 {read-write, access after open-claim-enable}

Remarks	Holds the image and/or video displaying mode.

Value	Meaning

GDSP_DMODE_HIDDEN

It is a mode to hide images and/or video

GDSP_DMODE_IMAGE_FIT

It is a mode to display images. The displayed image is enlarged / reduced to the size that maintains the aspect and just enters the screen.

GDSP_DMODE_IMAGE_FILL

It is a mode to display images.

The displayed image is scaled to the size that

maintains the aspect and covers the entire screen.

GDSP_DMODE_IMAGE_CENTER

It is a mode to display images.

The displayed image is displayed in the center of the screen without changing the size.

GDSP_DMODE_VIDEO_NORMAL

It is a mode to display video. The displayed video

will be displayed in the center of the screen without resizing.

GDSP_DMODE_VIDEO_FULL

It is a mode to display video.

The displayed video will be displayed in full screen.

GDSP_DMODE_WEB

Display the web screen.

If application hide other modes and screens while displaying images, movies, or web, all displayed contents will be cleared. The movie will be stopped while the movie is playing.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value		       Meaning

E_ILLEGAL	An invalid value was specified.

See Also	CapCaptureColorSpaceList Property, VideoCaptureMode Property

ImageType Property

Syntax	ImageType: string {read-write, access after open-claim-enable}

Remarks	Contains the image file type that are support by the device, if CapImageType property is true. For example, if the device supports BMP, then this property should be set to “BMP”. This property value should be set prior to execute the loadImage method.  All of the capable image file types are listed in the ImageTypeList property.  *Notation contents may be different depending on the device. This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	CapImageType Property, ImageTypeList Property, loadImage Method.

ImageTypeList Property

Syntax	ImageTypeList: string {read-only, access after open}

Remarks	Contains the comma-delimited list of image file type that are support by the device. For example, if the device only supports BMP and JPEG, then this property should be set to “BMP,JPEG” One of value in the property should be set in the ImageType property, if CapImageType property is true,  prior to execute the loadImage method.

*Notation contents may be different depending on the device.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	CapImageType Property, ImageType Property, loadImage Method.

LoadStatus Property

Syntax	LoadStatus: int32 {read-only, access after open-claim-enable}

Remarks	Holds loading state of web page.

The parameters to be set are as follows.

Value			Meaning

GDSP_LSTATUS_START		Start loading the web page.

GDSP_LSTATUS_FINISH		It have finished loading the web page.

GDSP_LSTATUS_CANCEL 	It have canceled loading the web page

Its value is set prior to a StatusUpdateEvent being delivered to the application.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

StorageTarget Property

Syntax	StorageTarget: int32 {read-write, access after open-claim-enable}

Remarks	This is an enumeration and defines where the device writes the recorded image data file to. Should be set before an appropriate method call.
It holds one of the following values.

Value                                 Meaning

ST_HARDTOTAL	The image data file is written to the associated  HardTotals device. The property CapAssociatedHardTotalsDevice holds the open name of the associated HardTotals device.

ST_HOST              	The image data file is written to the host’s file system.

ST_HOST&HARDTOTAL

The encoded data file is written to the associated HardTotals device and host’s file system. The property CapAssociatedHardTotalsDevice holds the open name of the associated HardTotals device.

This property is initialized by the open method according to the value hold by CapStorageTarget. If CapStorageTarget has the value CST_ALL it is initialized to ST_HOST.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

Value	Meaning

E_ILLEGAL	An invalid value was specified, or recording is ongoing.

See Also	CapStorageTarget Property, CapAssociatedHardTotalsDevice Property

URL Property

Syntax	URL: string {read-only, access after open-claim-enable}

Remarks	When the LoadStatus property is GDSP_LSTATUS_START, the URL of the Web page that starts loading is set.

When the LoadStatus property is GDSP_LSTATUS_FINISH, the URL of the loaded Web page is set.

When the LoadStatus property is GDSP_STATUS_CALCEL, the URL of the canceled Web page is set.

Its value is set prior to a StatusUpdateEvent being delivered to the application.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	loadStatus Property.

VideoType Property

Syntax	VideoType: int32 {read-write, access after open-claim-enable}

Remarks	Contains the video file type that are support by the device, if CapVideoType property is true. For example, if the device supports AVI MJPG, then this property should be set to “AVI MJPG”. This property value should be set prior to execute the playVideo method.  All of the capable video file types are listed in the VideoTypeList property.         					　*Notation contents may be different depending on the device.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	CapVideoType Property, VideoType Property, playVideo Method.

VideoTypeList Property

Syntax	VideoTypeList: string {read-only, access after open}

Remarks	Contains the comma-delimited list of video file type that are support by the device. if the device only supports AVI_IYUV and AVI_MJPG, then this property should be set to “AVI_IYUV,AVI_MJPG”.　One of value in the property should be set in the VideoType property, if CapImageType property is true,  prior to execute the playVideo method.

*Notation contents may be different depending on the device.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	CapVideoType Property, VideoType Property, playVideo Method.

Volume Property

Syntax	Volume: int32 {read-write, access after open-claim-enable}

Remarks	Holds the volume at playing video. Legal values range from zero through 100.
This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	CapVolume Property, playVideo Method.

Methods (UML operations)

cancelURLLoading Method

Syntax	cancelURLLoading ( ): 
		void {raises-exception, use after open-claim-enable}

Remarks	Cancel loading web page.

This method is executed asynchronously. The load status is reported by StatusUpdateEvent and OutputCompleteEvent or ErrorEvent.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	It is not loading.

goURLBack Method

Syntax	goURLBack ( ): 
		void {raises-exception, use after open-claim-enable}

Remarks	It returns to the previous page of browsing history.

This method is executed asynchronously. The load status is reported by StatusUpdateEvent and OutputCompleteEvent or ErrorEvent.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	There is no previous page in the browsing history.

See Also	CapURLBack Property.

goURLForward Method

Syntax	goURLForward ( ):
		void {raises-exception, use after open-claim-enable}

Remarks	Go to the next page of browsing history.

This method is executed asynchronously. The load status is reported by StatusUpdateEvent and OutputCompleteEvent or ErrorEvent.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	There is no next page in the browsing history.

See Also	CapURLForward Property.

loadImage Method

Syntax	loadImage (fileName: string): 
		void {raises-exception, use after open-claim-enable}

Parameter	Description

fileName	Specify the file name of the image to be loaded.

Remarks	Load the specified image.

This method fails if the value of the DisplayMode Property is not set to GDSP_DMODE_IMAGE_FIT, GDSP_DMODE_IMAGE_FILL, or GDSP_DMODE_IMAGE_CENTER.

Image files are located in the area as the stored values of the StorageTarget property.

This method is executed asynchronously. Image file loading status is reported by StatusUpDateEvent and OutputCompleteEvent or ErrorEvent.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	 Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported image file was specified.

E_NOEXIST	File does not exist.

See Also	DisplayMode Property.

loadURL Method

Syntax	loadURL (uRL: string): 
		void {raises-exception, use after open-claim-enable}

Parameter	Description

uRL	Specify the uRL of the web page to load.

Remarks	Load the web page with the specified URL.

This method is executed asynchronously. The load status is reported by StatusUpdateEvent and OutputCompleteEvent or ErrorEvent.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

playVideo Method

Syntax	playVideo (fileName: string, loop: boolean): 
		void {raises-exception, use after open-claim-enable}

Parameter	Description

fileName		Specify the file name of the video to be played.

loop		If true, loop playback is performed, and if false, loop playback is not performed.

Remarks	Play the specified video that are loaded in the storage area by the loadImage method. All of loaded file images are listed in the ImageTypeList property.

If the value of the DisplayMode property is not set to GDSP_DMODE_VIDEO_NORMAL, GDSP_DMODE_VIDEO_FULL, this method will fail.

This method is executed asynchronously. To stop video displaying in the middle, call the stopVideo method.

Video files are located in the area as the stored values of the StorageTarget property

The video file playing status will be informed by the StatusUpdateEvent.

This method is executed asynchronously. Image file loading status and video file playing status are reported by StatusUpdateEvent and OutputCompleteEvent or ErrorEvent.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported video file was specified.

E_NOEXIST	File does not exist.

See Also	DisplayMode Property.

stopVideo Method

Syntax	stopVideo ( ): 
void {raises-exception, use after open-claim-enable}

Remarks	Stop the video being displayed.

This method is executed asynchronously. Inage file loading status is reported by StatusUpdateEvent and OutputCompleteEvent or ErrorEvent.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	The Video is not playing.

See Also	startVideo Method.

updateURLPage Method

Syntax	updateURLPage ( ): 
		void {raises-exception, use after open-claim-enable}

Remarks	Reload the current web page.

This method is executed asynchronously. The load status is reported by StatusUpdateEvent and OutputCompleteEvent or ErrorEvent.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	Web page loading.

Events (UML interfaces)

DirectIOEvent

<<event>>	upos::events::DirectIOEvent

EventNumber	: int32 {read-only}
Data			: int32 {read-write}
Obj			: object {read-write}

Description 	Provides Service information directly to the application. This event provides a means for a vendor-specific Sound Player Service to provide events to the application that are not otherwise supported by the device control.

Attributes	This event contains the following attributes:

Attribute	Type	Description

EventNumber	int32	Event number whose specific values are assigned by the Service.

Data	int32	Additional numeric data. Specific values vary by the EventNumber and the Service. This attribute is settable.

Obj	object	Additional data whose usage varies by the EventNumber and the Service. This attribute is settable.

Remarks	This event is to be used only for those types of vendor specific functions that are not otherwise described.

Use of this event may restrict the application program programform being used with other vendor’s devices which may not have any knowledge of the Service’s need for this event.

See Also	“Events" on page Intro-19, directIO method

ErrorEvent

<<event>> 	upos::events:: ErrorEvent
ErrorCode			: int32{read-write}
ErrorCodeExtended	: int32{read-write}

ErrorLocus 		: int32{read-write}
*pErrorResponse		: int32{read-write}

Description	Notifies the application that a Graphic Display Device error has been detected and suitable response by the application is necessary to process the error condition.

Attributes	This event contains the following attributes:

Attributes	Type	Description

ErrorCode	int32	Error code causing the error event. 				See a list of Error Codes on page 20.

ErrorCodeExtended	int32	Extended Error code causing the error event.

If ErrorCode is E_EXTENDED, then see values below. Otherwise, it may contain a Service-specific value.

ErrorLocus	int32	Location of the error. If EL_OUTPUT is specified. An error occurred during asynchronous action.

ErrorResponse	int32	Pointer to the error event response. See ErrorResponse below for values.

The ErrorLocus attribute has one of the following values:

Value	Meaning

EL_OUTPUT	Error occurred while processing asynchronous output.

EL_INPUT	Error occurred while gathering or processing event-driven input. No previously buffered input data is available.

EL_INPUT_DATA	Error occurred while gathering or processing event-driven input, and some previously buffered data is available.

The application’s error event handler can set the ErrorResponse attribute to one of the following values:

Value	Meaning

ER_RETRY	Retry sending the data. The error state is exited.
May be valid for some input devices when the locus is EL_INPUT, in which case the input is retried and the error state is exited. Typically, valid for asynchronous output devices when the locus is EL_OUTPUT, in which case the asynchronous output is retried and the error state is exited. This is the default response when the locus is EL_OUTPUT.

ER_CLEAR	Valid for all loci: EL_INPUT, EL_INPUT_DATA, and EL_OUTPUT.  Clear all buffered input or output data (including all asynchronous output).  The error state is exited.  This is the default response when the locus is EL_INPUT.

ER_CONTINUEINPUT 
Only valid when the locus is EL_INPUT_DATA.
Acknowledges that a data error has occurred and directs the Device to continue input processing. The Device remains in the error state and will deliver additional DataEvents as directed by the DataEventEnabled property. When all input has been delivered and DataEventEnabled is again set to true, then another ErrorEvent is delivered with locus EL_INPUT.
This is the default response when the locus is EL_INPUT_DATA.

Remarks	This event is enqueued when an error is detected and the Device’s State transitions into the error state. Input error events are not delivered until DataEventEnabled is true, so that proper application sequencing occurs.

Unlike a DataEvent, the Device does not disable further DataEvents or input ErrorEvents; it leaves the DataEventEnabled property value at true.    Note that the application may set DataEventEnabled to false within its event handler if subsequent input events need to be disabled for a period of time.

See Also 	“Device Input Model" on page Intro-22, “Error Handling" on page Intro-23, “Device Output Models" on page Intro-25.

OutputCompleteEvent

<<event>>	upos::events::OutputCompleteEvent 
		OutputID : int32{read-only}

Description 	Notify the application that the queued output request associated with the outputID property has completed successfully.

Attributes	This event contains the following attributes:

Attribute	Type	Description

OutputID 	int32	The ID number of the asynchronous output request that is complete.

Remarks	This event is enqueued after the request’s data has been both sent and the Service has confirmation that it was processed by the device successfully.

See Also	“Device Output Models" on page Intro-25

StatusUpdateEvent

<<event>>		upos::events:: StatusUpdateEvent
	Status			 : int32 {read-only}

Description	Notifies the application that there is an operation status change or a status of the Graphic Display device.

Attributes		This event contains the following attribute:

Attributes	Type	Description

Status           int32     Indicates a change of operation status of graphic display 			device

Note that Release 1.3 added Power State Reporting with additional Power 	reporting StatusUpdateEvent values.

The Update Firmware capability added additional Status values for communicating the status/progress of an asynchronous update firmware process. See “StatusUpdateEvent” description on page 1-34.

Value            		Meaning

GDSP_SUE_START_IMAGE_LOAD
				It will be notified when image loading start.

GDSP_SUE_END_IMAGE_LOAD
										It will be notified when image loading end.

GDSP_SUE_START_LOAD_WEBPAGE
		Start loading the web page.

GDSP_SUE_FINISH_LOAD_WEBPAGE
		It has finished loading the web page.

GDSP_SUE_CANCEL_LOAD_WEBPAGE
		It has canceled loading the web page.

GDSP_SUE_START_PLAY_VIDEO

Start playing video.

GDSP_SUE_STOP_PLAY_VIDEO

Stop playing video.

Remarks	Enqueued when the Graphic Display Device detects a power state change or a status change.

See Also 	“Events" on page Intro-19.

Appendix K

Relationship to other OMG  specification and activities

Robotics Domain Task Force

Activities in Robotics Domain Task Force

The OMG Robotics Domain Task Force (Robotics DTF) fosters the integration of robotics systems from modular components through the adoption of OMG standards. It recommends the adoption and extends OMG technologies that apply to the specific domain of robotics systems where no current baseline specifications exist, such as MDA for Robotics. The object technology is not solely limited to software but is extended to real objects. It also collaborates with other organizations for standardization, such as the one for home information appliances, and makes an open effort to increase interoperability in the field of robotics.

(https://www.omg.org/robotics/)

RoIS Specification

Robotic Interaction Service Framework [RoIS] defines several functional components for robotic interaction services.

Definitions related to locations of entities in robotic services will be described with Robotic Localization Service[RLS]. Definitions of status of components in services will be described in conjunction with Robotic Technology Component [RTC], Finite State Machine Component for RTC [FSM4RTC] and Unified Component Model for Distributed Real-Time and Embedded Systems [UCM].

RoIS specification seeks that specify a RoIS framework, on top of which   various service robot applications are developed.

Scope of RoIS specification

They are summarized in the following items.

- Interface between service application and Human Robot Interaction (HRI) engine

- Interface to obtain information from HRI Engine according to the timing of the service application’s needs (Query)

- Interface to receive information from HRI Engine triggered by real time events (Event notification / subscription / cancellation)

- Interface for instructions to device control HRI Engine functions (Command)

- Definition of common messages for all HRI Engines

Robot Service Ontology [RoSO] RFP

A new RFP of Robot Service Ontology[RoSO] currently being discussed in Robotics DTF are based on the concept of RoIS.

RoSO is aiming to define the specification (ontology) that clarifies the concept of a common vocabulary and / or a robot service in order to describe a service provided by a robot or exchange a description of a service provided by a service robot

Below is an example of HRI main component examples from this point of view.

Table K-1 – (From RoIS 1.2) Basic HRI Components

| HRI Component Name | Description |
| --- | --- |
| system information | Provides the information of the system such as status of the system and position of the physical unit. |
| person detection | Detects number of people |
| person localization | Detects position of people |
| person identification | Identifies ID (name) of people |
| face detection | Detects number of human faces |
| face localization | Detects position of human faces |
| sound detection | Detects number of sound sources |
| sound localization | Detects position of sound sources |
| speech recognition | Recognizes person’s speech |
| gesture recognition | Recognizes person’s gesture |
| speech synthesis | Generates robot speech |
| reaction | Performs specified reaction |
| navigation | Moves to specified target location |
| follow | Follows a specified target object |
| move | Moves to specified distance or curve |

Interoperability between UPOS RCSD and RoIs

Rleationsihp between UPOS RCSD and RoIS

OMG's Robotics standard provides a lower level control layer to manage Robot Device with finer granularity and higher accuracy to accommodate a wide range of industry applications.

On the other hand, the UPOS RCSD specification focuses on the functioning of robotic equipment within the retail store environment. In the UPOS RCSD specification robots are treated as peripheral equipment of the latest POS system. Therefore, the UPOS RCSD specification focuses on the definition of the interface between the POS and the robotic device.

RoIS is already existing as OMG standard and it defined a component frame service that was intended for robotic communication services with people.

Therefore, ROIS developed a general robot service framework, which is different from UPOS RCSD, but it is possible to describe the function of UPOS RCSD.

To confirm the compatibility and interoperability of the RCSD functions of RoIS and UPOS, both DTFs created and confirmed the function mapping table.

For this purpose, we use the general RoIS HRI component defined in the RoIS 1.2 specification.

The two teams continue to collaborate between the part of their separate RFP’s and standards that will be established.

For that purpose, it is very necessary to understand the common vocabulary of the robot service and the needs of the ontology.

If each team’s specification satisfies the above mapping table, it is confirmed that the standard can be maintained independently.

In addition, the figure below shows a typical scenario where RCSD and RoIS work independently or in conjunction.

Document History

Version History

| Ver | Date | Sections | Description of Change |
| --- | --- | --- | --- |
| 1.0 | 2019-2-18 |  | Initial Version – additions and updates to UPOS v1.15 |
| 1.1 | 2019-7-09 |  | Revised for the issus and additions from the Review |
| 1.2 | 2020-2-15 |  | Issues, Upates are added version from the Review |

Glossary

| Term | Definition |
| --- | --- |
| EVRW | Electronic Value Reader Writer |
| CAT | Credit Authorization Terminal |