UnifiedPOS Retail Peripheral Architecture

Version 1.16  RCSD

International Standard

For Implementation of Point Of Service Peripherals

OMG Document Number: retail/2019-04-01

Original submission date: February 18th, 2019

Standard document URL: https://www.omg.org/spec/UPOS/

This proposal adds to and extends the UPOS 1.15 standard.

Copyright © 2017, Object Management Group, Inc.

USE OF SPECIFICATION - TERMS, CONDITIONS & NOTICES

The material in this document details an Object Management Group specification in accordance with the terms, conditions and notices set forth below. This document does not represent a commitment to implement any portion of this specification in any company's products. The information contained in this document is subject to change without notice.

LICENSES

The companies listed above have granted to the Object Management Group, Inc. (OMG) a nonexclusive, royalty-free, paid up, worldwide license to copy and distribute this document and to modify this document and distribute copies of the modified version. Each of the copyright holders listed above has agreed that no person shall be deemed to have infringed the copyright in the included material of any such copyright holder by reason of having used the specification set forth herein or having conformed any computer software to the specification.

Subject to all of the terms and conditions below, the owners of the copyright in this specification hereby grant you a fully-paid up, non-exclusive, nontransferable, perpetual, worldwide license (without the right to sublicense), to use this specification to create and distribute software and special purpose specifications that are based upon this specification, and to use, copy, and distribute this specification as provided under the Copyright Act; provided that: (1) both the copyright notice identified above and this permission notice appear on any copies of this specification; (2) the use of the specifications is for informational purposes and will not be copied or posted on any network computer or broadcast in any media and will not be otherwise resold or transferred for commercial purposes; and (3) no modifications are made to this specification. This limited permission automatically terminates without notice if you breach any of these terms or conditions. Upon termination, you will destroy immediately any copies of the specifications in your possession or control.

PATENTS

The attention of adopters is directed to the possibility that compliance with or adoption of OMG specifications may require use of an invention covered by patent rights. OMG shall not be responsible for identifying patents for which a license may be required by any OMG specification, or for conducting legal inquiries into the legal validity or scope of those patents that are brought to its attention. OMG specifications are prospective and advisory only. Prospective users are responsible for protecting themselves against liability for infringement of patents.

IPR Mode

This specification is issued under the RAND Mode base on the OMG IPR Policy.

OMG IPR Policy   https://www.omg.org/cgi-bin/doc.cgi?ipr

GENERAL USE RESTRICTIONS

Any unauthorized use of this specification may violate copyright laws, trademark laws, and communications regulations and statutes. This document contains information which is protected by copyright. All Rights Reserved. No part of this work covered by copyright herein may be reproduced or used in any form or by any means--graphic, electronic, or mechanical, including photocopying, recording, taping, or information storage and retrieval systems--without permission of the copyright owner.

DISCLAIMER OF WARRANTY

WHILE THIS PUBLICATION IS BELIEVED TO BE ACCURATE, IT IS PROVIDED "AS IS" AND MAY CONTAIN ERRORS OR MISPRINTS. THE OBJECT MANAGEMENT GROUP AND THE COMPANIES LISTED ABOVE MAKE NO WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, WITH REGARD TO THIS PUBLICATION, INCLUDING BUT NOT LIMITED TO ANY WARRANTY OF TITLE OR OWNERSHIP, IMPLIED WARRANTY OF MERCHANTABILITY OR WARRANTY OF FITNESS FOR A PARTICULAR PURPOSE OR USE.  IN NO EVENT SHALL THE OBJECT MANAGEMENT GROUP OR ANY OF THE COMPANIES LISTED ABOVE BE LIABLE FOR ERRORS CONTAINED HEREIN OR FOR DIRECT, INDIRECT, INCIDENTAL, SPECIAL, CONSEQUENTIAL, RELIANCE OR COVER DAMAGES, INCLUDING LOSS OF PROFITS, REVENUE, DATA OR USE, INCURRED BY ANY USER OR ANY THIRD PARTY IN CONNECTION WITH THE FURNISHING, PERFORMANCE, OR USE OF THIS MATERIAL, EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

The entire risk as to the quality and performance of software developed using this specification is borne by you. This disclaimer of warranty constitutes an essential part of the license granted to you to use this specification.

RESTRICTED RIGHTS LEGEND

Use, duplication or disclosure by the U.S. Government  is subject to the restrictions set forth in subparagraph (c) (1) (ii) of The Rights in Technical Data and Computer Software Clause at DFARS 252.227-7013 or in subparagraph (c)(1) and (2) of the Commercial Computer Software - Restricted Rights clauses at 48 C.F.R. 52.227-19 or as specified in 48 C.F.R. 227-7202-2 of the DoD F.A.R. Supplement and its successors, or as specified in 48 C.F.R. 12.212 of the Federal Acquisition Regulations and its successors, as applicable. The specification copyright owners are as indicated above and may be contacted through the Object Management Group, 109 Highland Avenue, Needham, MA 02494, U.S.A.

TRADEMARKS

IMM®, MDA®, Model Driven Architecture®, UML®, UML Cube logo®, OMG Logo®, CORBA® and XMI® are registered trademarks of the Object Management Group, Inc., and Object Management Group™, OMG™ , Unified Modeling Language™, Model Driven Architecture Logo™, Model Driven Architecture Diagram™, CORBA logos™, XMI Logo™, CWM™, CWM Logo™, IIOP™ , MOF™ , OMG Interface Definition Language (IDL)™ , and OMG SysML™ are trademarks of the Object Management Group. All other products or company names mentioned are used for identification purposes only, and may be trademarks of their respective owners.

COMPLIANCE

The copyright holders listed above acknowledge that the Object Management Group (acting itself or through its designees) is and shall at all times be the sole entity that may authorize developers, suppliers and sellers of computer software to use certification marks, trademarks or other special designations to indicate compliance with these materials.

Software developed under the terms of this license may claim compliance or conformance with this specification if and only if the software compliance is of a nature fully matching the applicable compliance points as stated in the specification. Software developed only partially matching the applicable compliance points may claim only that the software was based on this specification, but may not claim compliance or conformance with this specification. In the event that testing suites are implemented or approved by Object Management Group, Inc., software developed using this specification may claim compliance or conformance with the specification only if the software satisfactorily completes the testing suites.

OMG’s Issue Reporting Procedure

All OMG specifications are subject to continuous review and improvement. As part of this process we encourage readers to report any ambiguities, inconsistencies, or inaccuracies they may find by completing the Issue Reporting Form listed on the main web page http://www.omg.org, under Documents, Report a Bug/Issue (http://www.omg.org/report_issue.)

Document Sumbitter

Vinx Corp.

Document Publishing Supportes

OPOS-J

Sorimachi Giken Co. Ltd.

Microsoft Japan Ltd.

SEIKO EPSON Corp.

Toshiba TEC Corp.

Star Micronics Corp.

Fujitsu Frontec Corp.

NCR Corporation

Sharp Corporation

Omron Social Solutions Corp.

NEC Platforms Corp.

Transaction Media Networks Inc.

- Table of Content

Preface	7

UPOS 1.16 RCSD Specification Overivew	10

Updated Items in Release 1.16	10

Updated Items in ＣＨＡＰＴＥＲ 21 Lights	10

Properties	10

Methods	10

Updated Items in ＣＨＡＰＴＥＲ 29 POS Power	10

Properties	10

Added Chapters in Release 1.16	10

ＣＨＡＰＴＥＲ 21	11

Lights	11

Summary	11

General Information	15

Capabilities	15

Device Sharing	15

Lights Class Diagram	16

Lights Sequence Diagram	17

Properties (UML attributes)	19

Methods (UML operations)	22

Events (UML interfaces)	26

ＣＨＡＰＴＥＲ 29	28

POS Power	28

Summary	28

General Information	32

Capabilities	32

Device Sharing	32

Model	33

POSPower Class Diagram	34

POSPower Sequence Diagram	35

POSPower Standby Sequence Diagram	36

POSPower PowerState Diagram - Part 2	39

POSPower PowerState Diagram - Part 3	40

POSPower State Chart Diagram for Fan and Temperature	41

POSPower Battery State Diagram	42

POSPower Power Transitions State Diagram	43

Properties (UML attributes)	44

Methods (UML operations)	52

Events (UML Interfaces)	55

ＣＨＡＰＴＥＲ 39	59

Video Capture	59

Summary	59

General Information	64

Capabilities	64

Video Capture Class Diagram	65

Model	66

Capture only mode	66

Photo shooting mode	66

Movie shooting mode	66

Input Model	67

Bar Code Scan	67

Individual Recognition	68

Device Sharing	68

Properties (UML attributes)	69

Methods (UML operations)	88

Events (UML interfaces)	91

ＣＨＡＰＴＥＲ 40	95

Individual Recognition	95

Summary	95

General Information	98

Capabilities	98

Individual Recognition Class Diagram	98

Model	99

Device Sharing	99

Properties (UML attributes)	100

ＣＨＡＰＴＥＲ 41	102

Sound Recorder	102

Summary	102

General Information	106

Capabilities	106

Sound Recorder Class Diagram	106

Model	107

Device Sharing	107

Properties(UML attributes)	108

Methods(UML operations)	112

Events(UML interfaces)	113

ＣＨＡＰＴＥＲ 42	115

Voice Recognition	115

Summary	115

General Information	119

Capabilities	119

Voice Recognition Class Diagram	119

Model	120

Device Sharing	121

Properties (UML attributes)	122

Methods (UML operations)	126

ＣＨＡＰＴＥＲ 43	130

Sound Player	130

Summary	130

General Information	134

Capabilities	134

Sound Player Class Diagram	134

Model	135

Device Sharing	135

Properties(UML attributes)	136

Methods (UML operations)	138

ＣＨＡＰＴＥＲ 44	139

Speech Synthesis	139

Summary	139

General Information	143

Capabilities	143

Speech Synthesis Class Diagram	143

Model	144

Device Sharing	144

Properties (UML attributes)	145

Methods (UML operations)	150

ＣＨＡＰＴＥＲ 45	153

Gesture Control	153

Summary	153

General Information	157

Capabilities	157

Gesture Control Class Diagram	158

Model	159

Automatic control	159

Pose / Motion	160

Device Sharing	160

Properties (UML attributes)	161

Methods (UML operations)	165

ＣＨＡＰＴＥＲ 46	170

Device Monitor	170

Summary	170

General Information	173

Capabilities	173

Device Monitor Class Diagram	173

Model	174

Device Sharing	174

Properties (UML attributes)	175

Methods (UML operations)	178

ＣＨＡＰＴＥＲ 47	182

Graphic Display	182

Summary	182

General Information	186

Capabilities	186

Graphics Display Class Diagram	186

Model	187

Image Display Mode	187

Movie Display Mode	188

Web Display Mode	189

Device Sharing	189

Properties (UML attributes)	190

Methods (UML operations)	195

Ａｐｐｅｎｄｉｘ K	199

Relationship to other OMG  specification and activities	199

Robotics Domain Task Force	199

Activities in Robotics Domain Task Force	199

RoIS Specification	199

Scope of RoIS specification	199

Robot Servcie Ontology [RoSO] RFP	200

Interoperability between UPOS RCSD and RoIs	201

Rleationsihp between UPOS RCSD and RoIS	201

Document History	203

Version History	203

Glossary	204

- Preface

OMG

Founded in 1989, the Object Management Group, Inc. (OMG) is an open membership, not-for-profit computer industry standards consortium that produces and maintains computer industry specifications for interoperable, portable, and reusable enterprise applications in distributed, heterogeneous environments. Membership includes Information Technology vendors, end users, government agencies, and academia.

OMG member companies write, adopt, and maintain its specifications following a mature, open process. OMG’s specifications implement the Model Driven Architecture® (MDA®), maximizing ROI through a full-lifecycle approach to enterprise integration that covers multiple operating systems, programming languages, middleware and networking infrastructures, and software development environments. OMG’s specifications include: UML® (Unified Modeling Language™); CORBA® (Common Object Request Broker Architecture); CWM™ (Common Warehouse Metamodel); and industry-specific standards for dozens of vertical markets.

More information on the OMG is available at http://www.omg.org/.

OMG Specifications

As noted, OMG specifications address middleware, modeling and vertical domain frameworks. All OMG Specifications are available from the OMG website at:

http://www.omg.org/spec

Specifications are organized by the following categories:

Business Modeling Specifications

Middleware Specifications

- CORBA/IIOP

- Data Distribution Services

- Specialized CORBA

IDL/Language Mapping Specifications

Modeling and Metadata Specifications

- UML, MOF, CWM, XMI

- UML Profile

Modernization Specifications

Platform Independent Model (PIM), Platform Specific Model (PSM), Interface Specifications

- CORBAServices

- CORBAFacilities

OMG Domain Specifications

CORBA Embedded Intelligence Specifications

CORBA Security Specifications

All of OMG’s formal specifications may be downloaded without charge from our website. (Products implementing OMG specifications are available from individual suppliers.) Copies of specifications, available in PostScript and PDF format, may be obtained from the Specifications Catalog cited above or by contacting the Object Management Group, Inc. at:

OMG Headquarters
109 Highland Avenue
Needham, MA 02494
USA
Tel: +1-781-444-0404
Fax: +1-781-444-0320
Email: pubs@omg.org

Certain OMG specifications are also available as ISO standards. Please consult http://www.iso.org

Typographical Conventions

The type styles shown below are used in this document to distinguish programming statements from ordinary English. However, these conventions are not used in tables or section headings where no distinction is necessary.

Times/Times New Roman - 10 pt.:  Standard body text

NOTE:   Terms that appear in italics are defined in the glossary. Italic text also represents the name of a document, specification, or other publication.

Issues

The reader is encouraged to report any technical or editing issues/problems with this specification to http://www.omg.org/report_issue.htm.

- UPOS 1.16 RCSD Specification Overivew

Updated Items in Release 1.16

Chapter sections 23 and 38 from UPOS1.15 are included with annotations denoting the changes necessary for supporting the addition of the Retail Communications Service Devices.  Chapters 39-47 are new chapters for devices being added to UPOS v1.  The following is a list of the proterties, methods and chapters.

Updated Items in ＣＨＡＰＴＥＲ 21 Lights

Properties

CapFullColor Property

CapPattern Property

FullColor Property

Methods

switchOn  Method

switchONMultiple Method

switchOnPattern Method

switchOffPattern Method

Updated Items in ＣＨＡＰＴＥＲ 29 POS Power

Properties

CapChargeTime Property

CapTimeMode Property

ChargeTime Property

TimeMode Property

Added Chapters in Release 1.16

ＣＨＡＰＴＥＲ 39 Video Capture

ＣＨＡＰＴＥＲ 40 Individual Recognition

ＣＨＡＰＴＥＲ 41 Sound Recorder

ＣＨＡＰＴＥＲ 42 Voice Recognition

ＣＨＡＰＴＥＲ 43 Sound Player

ＣＨＡＰＴＥＲ 44 Speech Synthesis

ＣＨＡＰＴＥＲ 45 Gesture Control

ＣＨＡＰＴＥＲ 46 Device Monitor

ＣＨＡＰＴＥＲ 47 Graphic Display

- ＣＨＡＰＴＥＲ 21

Lights

This Chapter defines the Lights device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.12 | Not Supported |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.12 | open |
| CapPowerReporting: | int32 | {read-only} | 1.12 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.12 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.12 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.12 | open |
| CheckHealthText: | string | {read-only} | 1.12 | open |
| Claimed: | boolean | {read-only} | 1.12 | open |
| DataCount: | int32 | {read-only} | 1.12 | Not Supported |
| DataEventEnabled: | boolean | {read-write} | 1.12 | Not Supported |
| DeviceEnabled: | boolean | {read-write} | 1.12 | open, claim |
| FreezeEvents: | boolean | {read-write} | 1.12 | open |
| OutputID: | int32 | {read-only} | 1.12 | Not Supported |
| PowerNotify: | int32 | {read-write} | 1.12 | open |
| PowerState: | int32 | {read-only} | 1.12 | open |
| State: | int32 | {read-only} | 1.12 | -- |
| DeviceControlDescription: | string | {read-only} | 1.12 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.12 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.12 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.12 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.12 | open |
| PhysicalDeviceName: | string | {read-only} | 1.12 | open |

| Properties (Continued) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specific | Type | Mutability | Version | May Use After |
| CapAlarm: | int32 | {read-only} | 1.12 | open |
| CapBlink: | boolean | {read-only} | 1.12 | open |
| CapColor: | int32 | {read-only} | 1.12 | open |
| CapFullColor: | boolean | {read-only} | 1.16 | open |
| CapPattern: | int32 | {read-only} | 1.16 | open |
| FullColor: | boolean | {read-write} | 1.16 | open |
| MaxLights: | int32 | {read-only} | 1.12 | open |

| Methods (UML operations) |  |
| --- | --- |
| Common |  |
| Name | Version |
| open (logicalDeviceName: string): void {raises-exception} | 1.12 |
| close ( ): void {raises-exception, use after open} | 1.12 |
| claim (timeout: int32): void {raises-exception, use after open} | 1.12 |
| release ( ): void {raises-exception, use after open, claim} | 1.12 |
| checkHealth (level: int32): void {raises-exception, use after open, enable} | 1.12 |
| clearInput ( ): void { } | Not supported |
| clearInputProperties ( ): void { } | Not supported |
| clearOutput ( ): void { } | Not supported |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} | 1.12 |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, enable} | 1.12 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.12 |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.12 |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, enable} | 1.12 |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.12 |
| Specific |  |
| Name |  |
| switchOff (lightNumber: int32): void {raises-exception, use after open, claim, enable} | 1.12 |
| switchOn (lightNumber: int32, blinkOnCycle: int32, blinkOffCycle: int32, color: int32, alarm: int32): void {raises-exception, use after open, claim, enable} | 1.16 |
| switchOnMultiple (lightNumbers: string, blinkOnCycle: int32, blinkOffCycle: int32, color: int32, alarm: int32): void {raises-exception, use after open, claim, enable} | 1.16 |
| switchOnPattern (pattern: int32, alarm: int32): void {raises-exception, use after open, claim, enable} | 1.16 |
| switchOffPattern ( ): void {raises-exception, use after open, claim, enable} | 1.16 |

| Events (UML interfaces) |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Mutability | Version |
| upos::events::DataEvent |  | Not Supported |  |
| upos::events::DirectIOEvent |  |  | 1.12 |
| EventNumber: | int32 | {read-only} |  |
| Data: | int32 | {read-write} |  |
| Obj: | object | {read-write} |  |
| upos::events::ErrorEvent |  | Not Supported |  |
| upos::events::OutputCompleteEvent |  | Not Supported |  |
| upos::events::StatusUpdateEvent |  |  | 1.12 |
| Status: | int32 | {read-only} |  |

## General Information

The Lights programmatic name is “Lights”.

This device category was added to Version 1.12 of the specification.

- Capabilities

- The Lights Control has the following capability:

- Supports commands to “switch on” and “switch off” a light.

- The Lights Control may have the following additional capabilities:

  - Supports device-level blinking at adjustable blink cycles.

  - Support multiple lights.

  - Supports different colors of a light.

  - Supports different alarms

- Device Sharing

Lights is an exclusive-use device. Its device sharing rules are:

- The application must claim the device before enabling it.

- The application must claim and enable the device before accessing some of the properties and methods, or receiving events.

- See the “Summary” table for precise usage prerequisites.

- Lights Class Diagram

Updated in Release 1.16

The following diagram shows the relationships between the Lights classes

Fig. Chap. 21-1 Lights Class Diagram

- Lights Sequence Diagram

The following sequence diagram show the typical usage of the Lights device illustrating the handling of the media entry indicator lights.

Fig. Chap. 21-2 Lights Sequence Diagram (handling of the media entry indicator lights)

The following sequence diagram show the typical usage of the Lights device illustrating the handling of the pole lights.

Fig. Chap. 21-3 Lights Sequence Diagram (handling of the pole lights)

Properties (UML attributes)

CapAlarm Property

Syntax	CapAlarm: int32 {read-only, access after open}

Remarks	This capability indicates if the device supports different alarms.

CapAlarm is a logical OR combination of any of the following values:

Value	Meaning

LGT_ALARM_NOALARM	Alarms are not supported.

LGT_ALARM_SLOW		Supports a slow beep.

LGT_ALARM_MEDIUM	Supports a medium beep.

LGT_ALARM_FAST		Supports a fast beep.

LGT_ALARM_CUSTOM1	Supports 1st custom alarm.

LGT_ALARM_CUSTOM2	Supports 2nd custom alarm.

This property is initialized by the open method. If the device does not support alarms, it is initialized to LGT_ALARM_NOALARM.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

CapBlink Property

Syntax	CapBlink: boolean {read-only, access after open}

Remarks	If true, a blinking capability is supported. It may be either a physical capability of the device or emulated by the service.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

CapColor Property

Syntax	CapColor: int32 {read-only, access after open}

Remarks	This capability indicates if the device supports different colors.

CapColor is a logical OR combination of any of the following values:

Value	Meaning

LGT_COLOR_PRIMARY	Supports Primary Color (Usually Green).

LGT_COLOR_CUSTOM1	Supports 1st Custom Color (Usually Red).

LGT_COLOR_CUSTOM2	Supports 2nd Custom Color (Usually Yellow).

LGT_COLOR_CUSTOM3	Supports 3rd Custom Color.

LGT_COLOR_CUSTOM4	Supports 4th Custom Color.

LGT_COLOR_CUSTOM5	Supports 5th Custom Color.

This property is initialized by the open method. If the device supports only one color, it is initialized to LGT_COLOR_PRIMARY.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

CapFullColor Property　　                    Added in Release 1.16

Syntax	CapColor: boolean {read-only, access after open}

Remarks	If true, the application can set FullColor property to true and specify full color.

If false, the application cannot specify full color.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	FullColor Property, switchOn Method, switchOnMultiple Method.

CapPattern Property                         Added in Release 1.16

Syntax	CapColor: int32 {read-only, access after open}

Remarks	This capability indicates if the device supports different lighting patterns.

CapPattern is a logical OR combination of any of the following values:

Value		Meaning

LGT_PATTERN_NOPATTERN								Lighting patterns are not supported.

LGT_ PATTERN _CUSTOM			                 				1~32 Supports 1st to 32th Lighting Pattern.

This property is initialized by the open method. If the device does not support lighting pattern, it is initialized to LGT_PATTERN_NOPATTERN.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	switchOnPattern Method.

FullColor Property                           Added in Release 1.16

Syntax	FullColor: boolean {read-write, access after open}

Remarks	Holds the format of the value to specify for the Color parameter of SwitchOn method and SwitchOnMultiple method.

If true, the Color parameter format is full color of 0xRRGGBB format.

If false, the Color parameter format is one of the colors defined by CapColor.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	CapFullColor Property, switchOn Method, switchOnMultiple Method.

MaxLights Property

Syntax	MaxLights: int32 {read-only, access after open}

Remarks	MaxLights specifies the maximum number of lights that the device can support.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

## Methods (UML operations)

switchOff Method

Syntax	switchOff (lightNumber: int32): 
          void {raises-exception, use after open, claim, enable}

Parameter	Description

lightNumber	Specifies the light number. Valid light numbers are 1 through MaxLights.

Remarks	Switches off the light specified by lightNumber.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

A possible value of the exception’s ErrorCode property is:

Value	Meaning

E_ILLEGAL	The lightNumber parameter exceeds MaxLights.

See Also	MaxLights Property.

switchOffPattern Method

Syntax	switchOff Pattern ( ): 
          void {raises-exception, use after open, claim, enable}

Remarks	Switches off the pattern lighting.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

A possible value of the exception’s ErrorCode property is:

Value	Meaning

E_ILLEGAL	Pattern lighting is not executed.

See Also	switchOnPattern Method.

switchOn Method                                  Updated in Release 1.16

Syntax	switchOn (lightNumber: int32, blinkOnCycle: int32,
      blinkOffCycle: int32, color: int32, alarm: int32): 
          void {raises-exception, use after open, claim, enable}

Parameter	Description

lightNumber	Specifies the light number. Valid light numbers are 1 through MaxLights.

blinkOnCycle	A zero (0) value indicates no blink cycle. A positive value indicates the blink on cycle time in milliseconds. Negative values are not allowed.

blinkOffCycle	A zero (0) value indicates no blink cycle. A positive value indicates the blink off cycle time in milliseconds. Negative values are not allowed.

color	If FullColor is true, specifies the color of the light, must be full color of 0xRRGGBB format.

If FullColor is false, specifies the color of the light, must be one of the colors defined by CapColor.

alarm	Specifies the used alarm type, must be one of the alarms defined by CapAlarm.

Remarks	Switches on the light specified by lightNumber or let it blink.

If blinkOnCycle and blinkOffCycle are zero (0) or CapBlink is false, then the parameters blinkOnCycle and blinkOffCycle will be ignored and the light will only be switched on.

If CapBlink is true and blinkOnCycle and blinkOffCycle are positive then the light will blink.

If CapColor is LGT_COLOR_PRIMARY the light does not support different colors and color is ignored, otherwise switchOn will use the color specified by color.

If CapAlarm is LGT_ALARM_NOALARM the light does not support different alarms and alarm is ignored, otherwise switchOn will use the alarm specified by alarm.

Subsequent calls to switchOn will change the blink cycles, the color or the alarm type of the light.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

A possible value of the exception’s ErrorCode property is:

Value	Meaning

E_ILLEGAL	The lightNumber parameter exceeds MaxLights, an invalid color or alarm was specified.

See Also	CapAlarm Property, CapBlink Property, CapColor Property, 	  FullColor Property, MaxLights Property.

switchOnMultiple Method          				Added in Release 1.16

Syntax	switchOnMultiple (lightNumbers: string, blinkOnCycle: int32,
                   blinkOffCycle: int32, color: int32, alarm: int32): 
          void {raises-exception, use after open, claim, enable}

Parameter	Description

lightNumbers	Specifies the comma-delimited list of light number. Valid light numbers are 1 through MaxLights.

blinkOnCycle	A zero (0) value indicates no blink cycle. A positive value indicates the blink on cycle time in milliseconds. Negative values are not allowed.

blinkOffCycle	A zero (0) value indicates no blink cycle. A positive value indicates the blink off cycle time in milliseconds. Negative values are not allowed.

color	If FullColor is true, specifies the color of the light, must be full color of 0xRRGGBB format.

If FullColor is false, specifies the color of the light, must be one of the colors defined by CapColor.

alarm	Specifies the used alarm type, must be one of the alarms defined by CapAlarm.

Remarks	Switches on the multiple lights specified by lightNumbers or let it blink.

If blinkOnCycle and blinkOffCycle are zero (0) or CapBlink is false, then the parameters blinkOnCycle and blinkOffCycle will be ignored and the light will only be switched on.

If CapBlink is true and blinkOnCycle and blinkOffCycle are positive then the light will blink.

If CapColor is LGT_COLOR_PRIMARY the light does not support different colors and color is ignored, otherwise switchOnMultiple will use the color specified by color.

If CapAlarm is LGT_ALARM_NOALARM the light does not support different alarms and alarm is ignored, otherwise switchOnMultiple will use the alarm specified by alarm.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

A possible value of the exception’s ErrorCode property is:

Value	Meaning

E_ILLEGAL	The lightNumbers parameter exceeds MaxLights, an invalid value was specified.

See Also	CapAlarm Property, CapBlink Property, CapColor Property,        FullColor Property, MaxLights Property.

switchOnPattern Method                      			Added in Release 1.16

Syntax	switchOnPattern (pattern: int32, alarm: int32): 
          void {raises-exception, use after open, claim, enable}

Parameter	Description

pattern	Specifies the lighting pattern, must be one of the pattern defined by CapPattern.

alarm	Specifies the used alarm type, must be one of the alarms defined by CapAlarm.

Remarks	Switches on the light specified by pattern.

If CapAlarm is LGT_ALARM_NOALARM the light does not support different alarms and alarm is ignored, otherwise switchOn and swithOnPattern will use the alarm specified by alarm.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

A possible value of the exception’s ErrorCode property is:

Value	Meaning

E_ILLEGAL	An invalid value was specified, or unsupported operation with the Device.

See Also	CapAlarm Property, CapPattern Property.

## Events (UML interfaces)

DirectIOEvent

<< event >>   upos::events::DirectIOEvent
EventNumber: 	int32 {read-only}
Data:			int32 {read-write}
Obj:			object{read-write}

Description 	Provides Service information directly to the application. This event provides a means for a vendor-specific Lights Service to provide events to the application that are not otherwise supported by the Control.

Attributes	This event contains the following attributes:

Attribute	Type	Description

EventNumber	int32	Event number whose specific values are assigned by the Service.

Data	int32	Additional numeric data. Specific values vary by the EventNumber and the Service. This property is settable.

Obj	Object	Additional data whose usage varies by the EventNumber and Service. This property is settable.

Remarks	This event is to be used only for those types of vendor specific functions that are not otherwise described. Use of this event may restrict the application program from being used with other vendor’s Lights devices which may not have any knowledge of the Service’s need for this event.

See Also	“Events" on page Intro-19, directIO Method.

StatusUpdateEvent

<< event >>   upos::events::StatusUpdateEvent
		Status: int32 {read-only}

Description 	Notifies the application that there is a change in the power status of a light.

Attributes	This event contains the following attribute:

Attribute	Type	       Description

Status	int32	Reports a change in the power status of a light.

Note that Release 1.3 added Power State Reporting with additional Power reporting StatusUpdateEvent values.

The Update Firmware capability, added in Release 1.9, added additional Status values for communicating the status/progress of an asynchronous update firmware process.

See “StatusUpdateEvent” description on page 1-34.

Remarks	Enqueued when the light detects a power state change.

See Also	“Events" on page Intro-19.

- ＣＨＡＰＴＥＲ 29

POS Power

This Chapter defines the POS Power device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.5 | Not Supported |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.9 | open |
| CapPowerReporting: | int32 | {read-only} | 1.3 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.8 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.9 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.8 | open |
| CheckHealthText: | string | {read-only} | 1.5 | open |
| Claimed: | boolean | {read-only | 1.5 | open |
| DataCount: | int32 | {read-only} | 1.5 | Not Supported |
| DataEventEnabled: | boolean | {read-write} | 1.5 | Not Supported |
| DeviceEnabled: | boolean | {read-write} | 1.5 | open, claim |
| FreezeEvents: | boolean | {read-write} | 1.5 | open |
| OutputID: | int32 | {read-only} | 1.5 | Not Supported |
| PowerNotify: | int32 | {read-write} | 1.5 | open |
| PowerState: | int32 | {read-only} | 1.5 | open |
| State: | int32 | {read-only} | 1.5 | -- |
| DeviceControlDescription: | string | {read-only} | 1.5 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.5 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.5 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.5 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.5 | open |
| PhysicalDeviceName: | string | {read-only} | 1.5 | open |

| Properties (Continued) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specific | Type | Mutability | Version | May Use After |
| BatteryCapacityRemaining: | int32 | {read-only} | 1.9 | open |
| BatteryCriticallyLowThreshold: | int32 | {read-write} | 1.9 | open |
| BatteryLowThreshold: | int32 | {read-write} | 1.9 | open |
| CapBatteryCapacityRemaining: | boolean | {read-only} | 1.9 | open |
| CapChargeTime: | boolean | {read-only} | 1.16 | open |
| CapFanAlarm: | boolean | {read-only} | 1.5 | open |
| CapHeatAlarm: | boolean | {read-only} | 1.5 | open |
| CapQuickCharge: | boolean | {read-only} | 1.5 | open |
| CapRestartPOS: | boolean | {read-only} | 1.9 | open |
| CapShutdownPOS: | boolean | {read-only} | 1.5 | open |
| CapStandbyPOS: | boolean | {read-only} | 1.9 | open |
| CapSuspendPOS: | boolean | {read-only} | 1.9 | open |
| CapTimeMode: | boolean | {read-only} | 1.16 | open |
| CapUPSChargeState: | int32 | {read-only} | 1.5 | open |
| CapVariableBatteryCriticallyLowThreshold: | boolean | {read-only} | 1.9 | open |
| CapVariableBatteryLowThreshold: | boolean | {read-only} | 1.9 | open |
| ChargeTime: | int32 | {read-only} | 1.16 | open |
| EnforcedShutdownDelayTime: | int32 | {read-write} | 1.5 | open |
| PowerFailDelayTime: | int32 | {read-only} | 1.5 | open |
| PowerSource: | int32 | {read-only} | 1.9 | open |
| QuickChargeMode: | boolean | {read-only} | 1.5 | open |
| QuickChargeTime: | int32 | {read-only} | 1.5 | open |
| TimeMode: | boolean | {read-write} | 1.16 | open |
| UPSChargeState: | int32 | {read-only} | 1.5 | open, claim & enable |

| Methods (UML operations) |  |
| --- | --- |
| Common |  |
| Name | Version |
| open (logicalDeviceName: string): void {raises-exception} | 1.5 |
| close ( ): void {raises-exception, use after open} | 1.5 |
| claim (timeout: int32 ): void {raises-exception, use after open} | 1.5 |
| release ( ): void {raises-exception, use after open, claim} | 1.5 |
| checkHealth (level: int32): void {raises-exception, use after open, enable} | 1.5 |
| clearInput ( ): void { } | Not supported |
| clearInputProperties ( ): void { } | Not supported |
| clearOutput ( ): void { } | Not supported |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} | 1.5 |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, claim, enable} | 1.9 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | 1.8 |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | 1.8 |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, claim, enable} | 1.9 |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | 1.8 |
| Specific |  |
| Name |  |
| restartPOS ( ): void {raises-exception, use after open, enable} | 1.9 |
| shutdownPOS ( ): void {raises-exception, use after open, enable} | 1.5 |
| standbyPOS (reason: int32 ): void {raises-exception, use after open, enable} | 1.9 |
| suspendPOS (reason: int32 ): void {raises-exception, use after open, enable} | 1.9 |

| Events (UML interfaces) |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Mutability | Version |
| upos::events::DataEvent |  | Not Supported |  |
| upos::events::DirectIOEvent |  |  | 1.5 |
| EventNumber: | int32 | {read-only} |  |
| Data: | int32 | {read-write} |  |
| Obj: | object | {read-write} |  |
| upos::events::ErrorEvent |  | Not Supported |  |
| upos::events::OutputCompleteEvent |  | Not Supported |  |
| upos::events::StatusUpdateEvent |  |  | 1.5 |
| Status: | int32 | {read-only} |  |

## General Information

The POS Power programmatic name is “POSPower”.

- Capabilities

The POSPower device class has the following capabilities:

- Supports a command to “shut down” the system.

- Supports a command to restart the system.

- Supports a command to “suspend” the system.

- Supports a command to have the system go to standby.

- Supports accessing a power handling mechanism of the underlying operating system and hardware.

- Informs the application if a power fail situation has occurred.

- Informs the application about battery level.

- Informs the application if the UPS charge state has changed.

- Informs the application about high CPU temperature.

- Informs the application about stopped CPU fan.

- Informs the application if an operating system dependent enforced shutdown mechanism is processed.

- Allows the application after saving application data locally or transferring application data to a server to shut down the POS terminal.

- Informs the application about an initiated shutdown.

### Device Sharing

The POSPower is a sharable device. Its device sharing rules are:

- After opening and enabling the device, the application may access all properties and methods and will receive status update events.

- If more than one application has opened and enabled the device, all applications may access its properties and methods. Status update events are fired to all of the applications.

- If one application claims the POSPower, then only that application may call the shutdownPOS, standbyPOS, or suspendPOS methods. This feature provides a degree of security, such that these methods may effectively be restricted to the main POS application if that application claims the device at startup.

- See the “Summary” table for precise usage prerequisites.

### Model

The general model of POSPower is based on the power model of each device in version 1.3 or later. The same common properties are used but all states relate to the POS terminal itself and not to a peripheral device.

There are three states of the POSPower:

- ONLINE. The POS terminal is powered on and ready for use. This is the “operational” state.

- OFF. The POS terminal is powered off or detached from the power supplying net. The POS terminal runs on battery power support. This is the powerfail situation.

- OFFLINE. The POS terminal is powered on but is running in a “lower-power-consumption” mode. It may need to be placed online by pressing a button or key or something else which may wake up the system.

Power reporting only occurs while the device is open, enabled and power notification is switched on.

In a powerfail situation - that means the POSPower is in the state OFF - the POS terminal will be shut down automatically after the last application has closed the POSPower device or the time specified by the EnforcedShutdownDelayTime property has been elapsed.

A call to the shutdownPOS method will always shut down the POS terminal independent of the system power state.

Version 1.9 or later

Support of battery powered devices is added. In addition to adding properties to report battery levels and power sources, properties are added to allow for the setting of low and critically low battery levels. The POSPower device also includes the ability to request or respond to request to enter the standby and suspend states. The model does not attempt to duplicate other power management models such as APM and ACPI, but leaves those implementation details to the provider. As a rule, the suspend state will consume less power than the standby state, which in turn will consume less power than the on state. A suggested mapping of these states to other power management models is:

| State | ACPI | APM | Description |
| --- | --- | --- | --- |
| On | S0 | ON | Active, Powered On |
| Standby | S1 | SUSPEND | Displays and drives off, CPU, RAM and fans powered on |
| Suspend | S3 | SUSPEND | Only RAM powered |
| Off | S5 | OFF | Completely powered off |

- POSPower Class Diagram

Updated in Release 1.16

The following diagram shows the relationships between the POSPower classes.

Fig. Chap.29-1 POSPower Class Diagram

- POSPower Sequence Diagram

The following sequence diagram shows the typical usage of the POSPower device for registering for StatusUpdateEvents and an atypical case of initiating a shutdownPOS call.

Fig. Chap. 29-2 POSPower Sequence Diagram

- POSPower Standby Sequence Diagram

Fig. Chap. 29-3 POSPower Standby Sequence Diagram

POSPower State Diagram

The following state diagram depicts the POSPower Control device model.

Fig. Chap. 29-4 Power State Diagram (POSPoer Control Device Model)

POSPower PowerState Diagram - Part 1

The following state diagram depicts the POSPower Power States.

Fig. Chap. 29-5 POSPower PowerState Diagram (Part 1)

- POSPower PowerState Diagram - Part 2

The following state diagram depicts the POSPower PowerState ONLINE.

Fig. Chap. 29-6 POSPower PowerState Diagram (Part 2)

- POSPower PowerState Diagram - Part 3

The following state diagram depicts the POSPower PowerState OFF.

Fig. Chap. 29-7 POSPower PowerState Diagram (Part 3)

- POSPower State Chart Diagram for Fan and Temperature

The following state diagram depicts the handling of fan and temperature alarms.

Fig. Chap. 29-8 POSPower State Chart Diagram (Fan and Temperature)

- POSPower Battery State Diagram

Fig. Chap. 29-9 POSPower Battery State Diagram

- POSPower Power Transitions State Diagram

Fig. Chap. 29-10 POSPower Power Transitions State Diagram

Properties (UML attributes)

BatteryCapacityRemaining Property

Syntax	BatteryCapacityRemaining: int32 {read-only, access after open}

Remarks	A value of 0 to 100 represents percent of battery capacity remaining.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	CapBatteryCapacityRemaining Property

BatteryCriticallyLowThreshold Property

Syntax	BatteryCriticallyLowThreshold: int32 {read-write, access after open}

Remarks	If not zero, this property holds the threshold at which a PWR_SUE_BAT_CRITICAL Status Update Event is generated. The values 1 through 99 represent the percentage of the capacity remaining. The value 0 indicates that Battery Critically Low reporting is not supported or is disabled.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	CapVariableBatteryCriticallyLowThreshold Property, StatusUpdateEvent

BatteryLowThreshold Property

Syntax	BatteryLowThreshold: int32 {read-write, access after open}

Remarks	If not zero, this property holds the threshold at which a PWR_SUE_BAT_LOW Status Update Event is generated. The value 1 to 99 represents the percent capacity remaining. The value 0 indicates that battery low reporting is not supported or is disabled. If variable battery low threshold is supported, setting a value between 1 and 99 sets the threshold to that value. Setting a value of zero disables battery low reporting.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	CapVariableBatteryLowThreshold Property, StatusUpdateEvent

CapBatteryCapacityRemaining Property

Syntax	CapBatteryCapacityRemaining: boolean {read-only, access after open}

Remarks	If true, the device is able to provide battery capacity information. Otherwise it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	BatteryCapacityRemaining Property

CapChargeTime Property 			       Added in Release 1.16

Syntax	CapChargeTime: boolean {read-only, access after open}

Remarks	If true, the device is able to acquire the remaining time until full charging. Otherwise it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	ChargeTime Property.

CapFanAlarm Property

Syntax	CapFanAlarm: boolean {read-only, access after open}

Remarks	If true, the device is able to detect whether the CPU fan is stopped. Otherwise it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

CapHeatAlarm Property

Syntax	CapHeatAlarm: boolean {read-only, access after open}

Remarks	If true the device is able to detect whether the CPU is running at too high of a temperature. Otherwise it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

CapQuickCharge Property

Syntax	CapQuickCharge: boolean {read-only, access after open}

Remarks	If true, the power management allows the charging of the UPS battery in quick mode. The time for charging the battery is shorter than usual. Otherwise it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	QuickChargeMode Property, QuickChargeTime Property.

CapRestartPOS Property

Syntax	CapRestartPOS: boolean {read-only, access after open}

Remarks	If true the device is able to explicitly restart the POS. Otherwise it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	restartPOS Method.

CapShutdownPOS Property

Syntax	CapShutdownPOS: boolean {read-only, access after open}

Remarks	If true the device is able to explicitly shut down the POS. Otherwise it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	shutdownPOS Method.

CapStandbyPOS Property

Syntax	CapStandbyPOS: boolean {read-only, access after open}

Remarks	If true, the device is able to request that the POS System enter the Standby state. Otherwise it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	standbyPOS Method.

CapSuspendPOS Property

Syntax	CapSuspendPOS: boolean {read-only, access after open}

Remarks	If true, the device is able to request that the POS System enter the Suspend state. Otherwise it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	suspendPOS Method.

CapTimeMode Property 	        			       Added in Release 1.16

Syntax	CapTimeMode: boolean {read-only, access after open}

Remarks	If true the device is able to switch the unit of battery remaining / threshold related property value to seconds. Otherwise it is false.　　　　　　　　　　This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	TimeMode Property

CapUPSChargeState Property

Syntax	CapUPSChargeState: int32 {read-only, access after open}

Remarks	If not equal to zero, the UPS can deliver one or more charge states. It can contain any of the following values logically ORed together.

Value	Meaning

PWR_UPS_FULL	UPS battery is near full charge.

PWR_UPS_WARNING	UPS battery is near 50% charge.

PWR_UPS_LOW	UPS battery is near empty. Application shutdown should be started to ensure that is can be completed before the battery charge is depleted. A minimum of 2 minutes of normal system operation can be assumed when this state is entered unless this is the　first state reported upon entering the “Off” power state.

PWR_UPS_CRITICAL	UPS battery is in a critical state and could be disconnected at any time without further warning.	This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	UPSChargeState Property.

CapVariableBatteryCriticallyLowThreshold Property

Syntax	CapVariableBatteryCriticallyLowThreshold: 										boolean {read-only, access after open}

Remarks	If true, the device supports a variable threshold for critically low battery. Otherwise it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	BatteryCriticallyLowThreshold Property, StatusUpdateEvent

CapVariableBatteryLowThreshold Property

Syntax	CapVariableBatteryLowThreshold: boolean {read-only, access after open}

Remarks	If true, the device supports a variable threshold for battery low. Otherwise it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	BatteryLowThreshold Property, StatusUpdateEvent

ChargeTime Property 					Added in Release 1.16

Syntax	ChargeTime: int32 {read-only, access after open}

Remarks	Indicates the time remaining until the battery is fully charged in seconds.

If equal to zero the battery is not charging or not supported.

This property is only set if CapChargeTime is true.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also 	CapChargeTime Property.

EnforcedShutdownDelayTime Property

Syntax	EnforcedShutdownDelayTime: int32 {read-write, access after open}

Remarks	If not equal to zero the system has a built-in mechanism to shut down the POS　terminal after a determined time in a power fail situation. This property contains the time in milliseconds when the system will shut down automatically after a power failure. A power failure is the situation when the POS terminal is powered off or detached from the power supplying net and runs on UPS. 
If zero no automatic shutdown is performed and the application has to call itself the shutdownPOS method.								　　　Applications will be informed about an initiated automatic shutdown.　　　　This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	shutdownPOS Method.

PowerFailDelayTime Property

Syntax	PowerFailDelayTime: int32 {read-only, access after open}

Remarks	This property contains the time in milliseconds for power fail intervals which will not create a power fail situation. In some countries the power has sometimes short intervals where the power supply is interrupted. Those short intervals are in the range of milliseconds up to a few seconds and are handled by batteries or other electric equipment and should not cause a power fail situation. The power fail interval starts when the POS terminal is powered off or detached from the power supplying net and runs on UPS. The power fail interval ends when the POS terminal is again powered on or attached to the power supplying net. However, if the power fail interval is longer than the time specified in the PowerFailDelayTime property a power fail situation is created.

Usually this parameter is a configuration parameter of the underlying power management. So, the application can only read this property.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

PowerSource Property

Syntax	PowerSource: int32 {read-only, access after open}

Remarks	This property holds the current power source if power source reporting is available. A StatusUpdateEvent is generated each time this property is updated.

Value	Meaning

PWR_SOURCE_NA	Power source reporting is not available.

PWR_SOURCE_AC	The current power source is the AC line.

PWR_SOURCE_BATTERY	The current power source is a system battery. This value is only presented for systems that operate normally on battery.

PWR_SOURCE_BACKUP	The current power source is a backup source such as an UPS or backup battery.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	StatusUpdateEvent

QuickChargeMode Property

Syntax	QuickChargeMode: boolean {read-only, access after open}

Remarks	If true, the UPS battery is being recharged in a quick charge mode.
If false, it is being charged in a normal mode.

This property is only set if CapQuickCharge is true.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also 	CapQuickCharge Property, QuickChargeTime Property.

QuickChargeTime Property

Syntax	QuickChargeTime: int32 {read-only, access after open}

Remarks	This time specifies the remaining time for charging the UPS battery in quick charge mode. After the time has elapsed, the UPS battery charging mechanism of power management usually switches into normal mode.

This time is specified in milliseconds.

This property is only set if CapQuickCharge is true.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also 	CapQuickCharge Property, QuickChargeTime Property.

TimeMode Property                                                   Added in Release 1.16

Syntax	UPSChargeState: boolean {read-write, access after open}

Remarks	If true, the value of the battery remaining / threshold related property is in seconds.

If false, the value of the battery remaining / threshold related property is in percent.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20

See Also	CapTimeMode Property, BatteryCapacityRemaining Property, BatteryCriticallyLowThreshold Property, BatteryLowThreshold Property.

UPSChargeState Property

Syntax	UPSChargeState: int32 {read-only, access after open, enable}

Remarks	This property holds the actual UPS charge state.

It has one of the following values:

Value	Meaning

PWR_UPS_FULL	UPS battery is near full charge.

PWR_UPS_WARNING	UPS battery is near 50% charge.

PWR_UPS_LOW	UPS battery is near empty. Application shutdown should be started to ensure that is can be completed before the battery charge is depleted. A minimum of 2 minutes of normal system operation can be assumed when this state is entered unless this is the first state reported upon entering the “Off” power state.

PWR_UPS_CRITICAL	UPS battery is in a critical state and could be disconnected at any time without further warning.

This property is initialized and kept current while the device is enabled.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20

See Also	CapUPSChargeState Property.

Methods (UML operations)

restartPOS Method

Syntax	restartPOS ( ): 	
void {raises-exception, use after open, enable}

Remarks	Call to restart the POS terminal. This method will always restart the system independent of the system power state.

If the POSPower is claimed, only the application which claimed the device is able to restart the POS terminal.

Applications will be informed about an initiated restart.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	This method is not supported (see the CapRestartPOS property)

See Also	CapRestartPOS Property

shutdownPOS Method

Syntax	shutdownPOS ( ): 	
void {raises-exception, use after open, enable}

Remarks	Call to shut down the POS terminal. This method will always shut down the system independent of the system power state.

If the POSPower is claimed, only the application which claimed the device is able to shut down the POS terminal.

Applications will be informed about an initiated shutdown.

It is recommended that in a power fail situation an application has to call this method after saving all data and setting the application to a defined state.
If the EnforcedShutdownDelayTime property specifies a time greater than zero and the application did not call the shutdownPOS method within the time specified in EnforcedShutdownDelayTime, the system will be shut down automatically. This mechanism may be provided by an underlying operating system to prevent the battery from being emptied before the system is shut down.
This method is only supported if CapShutdownPOS is true.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20

Some possible values of the exception’s ErrorCode property are:

Value		Meaning

E_ILLEGAL	This method is not supported.		 			(See the CapshutdownPOS property)

See Also	CapShutdownPOS Property, EnforcedShutdownDelayTime Property.

standbyPOS Method

Syntax	standbyPOS (reason: int32): 	
void {raises-exception, use after open, enable}

Remarks	Call to request that the system be placed into the Standby state or to respond to a request from the system, OS or other application that the system be put into Standby state.

The reason parameter indicates the reason the POS terminal should enter a standby state:

Value	Description

PWR_REASON_REQUEST	Call is to request that the system enter the standby state.

PWR_REASON_ALLOW	Call is a response to a standby Status Update Event and specifies that the request should be allowed.

PWR_REASON_DENY	Call is a response to a standby Status Update Event and specifies that the request should be denied.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	This method is not supported (see the CapStandbyPOS property)

See Also	CapStandbyPOS Property.

suspendPOS Method

Syntax	suspendPOS (reason: int32): 	
void {raises-exception, use after open, enable}

Remarks	Call to request that the system be placed into the Suspend state or to respond to a request from the system, OS or other application that the system be put into Suspend state.

The reason parameter indicates the reason the POS terminal should enter a standby state:

Value	Description

PWR_REASON_REQUEST	Call is to request that the system enter the suspend state.

PWR_REASON_ALLOW	Call is a response to a suspend Status Update Event and specifies that the request should be allowed.

PWR_REASON_DENY	Call is a response to a suspend Status Update Event and specifies that the request should be denied.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	This method is not supported (see the CapSuspendPOS property)

See Also	CapSuspendPOS Property.

Events (UML Interfaces)

DirectIOEvent

<< event >>   upos::events::DirectIOEvent
		EventNumber	: int32 {read-only}
		Data		: int32 {read-write}
 		Obj		: object{read-write}

Description	Provides Service information directly to the application. This event provides a means for a vendor-specific POSPower Service to provide events to the application that are not otherwise supported by the Control.

Attributes	This event contains the following attributes:

Attributes	Type		Description

EventNumber	int32	Event number whose specific values are assigned by the Service.

Data	int32	Additional numeric data. Specific values vary by the EventNumber and the Service. This property is settable.

Obj	object	Additional data whose usage varies by the EventNumber and Service. This property is settable.

Remarks	This event is to be used only for those types of vendor specific functions that are not otherwise described. Use of this event may restrict the application program from being used with other vendor’s POSPower devices which may not have any knowledge of the Service’s need for this event.

See Also	“Errors" on page Intro-20, directIO Method.

StatusUpdateEvent

<< event >>   upos::events::StatusUpdateEvent
     Status: int32 {read-only}

Description	Delivered when UPSChargeState changes or an alarm situation occurs.

Attributes	This event contains the following attribute:

Attributes		 Type		Description

Status		int32		See below.

The Status property contains the updated power status or alarm status.

Value            		Meaning

PWR_SUE_UPS_FULL

UPS battery is near full charge. Can be returned if CapUPSChargeState contains PWR_UPS_FULL.

PWR_SUE_UPS_WARNING

UPS battery is near 50% charge. Can be returned if CapUPSChargeState contains PWR_UPS_WARNING.

PWR_SUE_UPS_LOW											UPS battery is near empty. Application shutdown 					should be started to ensure that it can be  						completed before the battery charge is depleted. 					A minimum of 2 minutes of normal system 					operation can be assumed when this state is 					entered unless this is the first charge state 						reported upon entering the “Off” state. Can be  					returned if CapUPSChargeState 	contains  					PWR_UPS_LOW.

PWR_SUE_UPS_CRITICAL

UPS is in critical state, and will in short time be 					disconnected. Can be returned if 							CapUPSChargeState contains 							PWR_UPS_CRITICAL.

PWR_SUE_FAN_STOPPED

The CPU fan is stopped. Can be returned if 						CapFanAlarm is true.

PWR_SUE_FAN_RUNNING

The CPU fan is running. Can be returned if 					CapFanAlarm is true.

PWR_SUE_TEMPERATURE_HIGH

The CPU is running on high temperature. Can be 						returned if CapHeatAlarm is true.

PWR_SUE_TEMPERATURE_OK

The CPU is running on normal temperature. Can					be returned if CapHeatAlarm is true.

PWR_SUE_SHUTDOWN												The system will shut down immediately.

PWR_SUE_BAT_LOW

The system remaining battery capacity is at or 						below the low battery threshold and the system is 						operating from the battery.

PWR_SUE_BAT_CRITICAL

The system remaining battery capacity is at or 						below the critically low battery threshold and the 						system is operating from the battery.

PWR_SUE_BAT_CAPACITY_REMAINING.

The BatteryCapacityRemaining property has been updated

PWR_SUE_RESTART

The system will restart immediately.

PWR_SUE_STANDBY

The system is requesting a transition to the 　　　　Standby state

PWR_SUE_USER_STANDBY

The system is requesting a transition to the 　　　　　　Standby state as a result of user input.

PWR_SUE_SUSPEND										The system is requesting a transition to the 				Suspend state.

PWR_SUE_USER_SUSPEND

The system is requesting a transition to the Suspend state as a result of user input.

PWR_SUE_PWR_SOURCE

The PowerSource property has been updated.

Note that Release 1.3 added Power State Reporting with additional Power 	reporting StatusUpdateEvent values.

The Update Firmware capability, added in Release 1.9, added additional 	Status values for communicating the status/progress of an asynchronous update firmware process.  See “StatusUpdateEvent” description on page 1-34.

See Also		CapFanAlarm Property, CapHeatAlarm Property, 	CapUPSChargeState 	Property, UPSChargeState Property.

- ＣＨＡＰＴＥＲ 39

Video Capture

This Chapter defines the Video Capture device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.16 | open |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.16 | open |
| CapPowerReporting: | int32 | {read-only} | 1.16 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.16 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.16 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.16 | open |
| CheckHealthText: | string | {read-only} | 1.16 | open |
| Claimed: | boolean | {read-only} | 1.16 | open |
| DataCount: | int32 | {read-only} | 1.16 | open |
| DataEventEnabled: | boolean | {read-write} | 1.16 | open |
| DeviceEnabled: | boolean | {read-write} | 1.16 | open, claim |
| FreezeEvents: | boolean | {read-write} | 1.16 | open |
| OutputID: | int32 | {read-only} | 1.16 | Not Supported |
| PowerNotify: | int32 | {read-write} | 1.16 | open |
| PowerState: | int32 | {read-only} | 1.16 | open |
| State: | int32 | {read-only} | 1.16 | -- |
| DeviceControlDescription: | string | {read-only} | 1.16 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.16 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.16 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.16 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.16 | open |
| PhysicalDeviceName: | string | {read-only} | 1.16 | open |

| Properties (Continued) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specific | Type | Mutability | Version | May Use After |
| CapCameraAutoExposition: | boolean | {read-only} | 1.16 | open |
| CapCameraAutoFocus: | boolean | {read-only} | 1.16 | open |
| CapCameraAutoGain: | boolean | {read-only} | 1.16 | open |
| CapCameraAutoWhiteBalance: | boolean | {read-only} | 1.16 | open |
| CapCameraBrightness: | boolean | {read-only} | 1.16 | open |
| CapCameraContrast: | boolean | {read-only} | 1.16 | open |
| CapCameraExposure: | boolean | {read-only} | 1.16 | open |
| CapCameraGain: | boolean | {read-only} | 1.16 | open |
| CapCameraHorizontalFlip: | boolean | {read-only} | 1.16 | open |
| CapCameraHue: | boolean | {read-only} | 1.16 | open |
| CapCameraSaturation: | boolean | {read-only} | 1.16 | open |
| CapCameraVerticalFlip: | boolean | {read-only} | 1.16 | open |
| CapCapture: | boolean | {read-only} | 1.16 | open |
| CapCaptureColorSpace: | boolean | {read-only} | 1.16 | open |
| CapCaptureColorSpaceList: | string | {read-only} | 1.16 | open |
| CapCaptureFrameRate: | boolean | {read-only} | 1.16 | open |
| CapCaptureMaxFrameRate: | int32 | {read-only} | 1.16 | open |
| CapCaptureResolution: | boolean | {read-only} | 1.16 | open |
| CapCaptureResolutionList: | string | {read-only} | 1.16 | open |
| CapDecodeData: | boolean | {read-only} | 1.16 | open |
| CapIndividualRecognition: | boolean | {read-only} | 1.16 | open |
| CapPhotograph: | boolean | {read-only} | 1.16 | open |
| CapPhotographResolution: | boolean | {read-only} | 1.16 | open |
| CapPhotographResolutionList | string | {read-only} | 1.16 | open |
| CapPhotographType: | boolean | {read-only} | 1.16 | open |
| CapPhotographTypeList: | string | {read-only} | 1.16 | open |
| CapVideoRecording: | boolean | {read-only} | 1.16 | open |
| CapVideoRecordingFrameRate: | boolean | {read-only} | 1.16 | open |
| CapVideoRecordingMaxFrameRate: | int32 | {read-only} | 1.16 | open |
| CapVideoRecordingResolution: | boolean | {read-only} | 1.16 | open |
| CapVideoRecordingResolutionList: | string | {read-only} | 1.16 | open |
| CapVideoRecordingType: | boolean | {read-only} | 1.16 | open |
| CapVideoRecordingTypeList: | string | {read-only} | 1.16 | open |
| BarCodeEnabled: | boolean | {read-write} | 1.16 | open, claim & enable |
| CameraAutoExposure: | boolean | {read-write} | 1.16 | open, claim & enable |
| CameraAutoFocus: | boolean | {read-write} | 1.16 | open, claim & enable |
| CameraAutoGain: | boolean | {read-write} | 1.16 | open, claim & enable |
| CameraAutoWhiteBalance: | boolean | {read-write} | 1.16 | open, claim & enable |
| CameraBrightness: | int32 | {read-write} | 1.16 | open, claim & enable |
| CameraContrast: | int32 | {read-write} | 1.16 | open, claim & enable |
| CameraExposure | int32 | {read-write} | 1.16 | open, claim & enable |
| CameraGain: | int32 | {read-write} | 1.16 | open, claim & enable |
| CameraHorizontalFlip: | boolean | {read-write} | 1.16 | open, claim & enable |
| CameraHue: | int32 | {read-write} | 1.16 | open, claim & enable |
| CameraSaturation: | int32 | {read-write} | 1.16 | open, claim & enable |
| CameraVerticalFlip: | boolean | {read-write} | 1.16 | open, claim & enable |
| CaptureColorSpace: | string | {read-write} | 1.16 | open, claim & enable |
| CaptureFrameRate: | int32 | {read-write} | 1.16 | open, claim & enable |
| CaptureResolution: | string | {read-write} | 1.16 | open, claim & enable |
| IndividualRecognitionEnabled: | boolean | {read-write} | 1.16 | open, claim & enable |
| PhotographResolution: | string | {read-write} | 1.16 | open, claim & enable |
| PhotographType: | int32 | {read-write} | 1.16 | open, claim & enable |
| VideoCaptureMode: | int32 | {read-write} | 1.16 | open, claim & enable |
| VideoRecordingFrameRate: | int32 | {read-write} | 1.16 | open, claim & enable |
| VideoRecordingResolution: | int32 | {read-write} | 1.16 | open, claim & enable |
| VideoRecordingType: | string | {read-write} | 1.16 | open, claim & enable |

| Methods (UML operations) |  |
| --- | --- |
| Common |  |
| Name | Version |
| open (logicalDeviceName: string): void {raises-exception} | 1.16 |
| close ( ): void {raises-exception, use after open} | 1.16 |
| claim (timeout: int32): void {raises-exception, use after open} | 1.16 |
| release ( ): void {raises-exception, use after open, claim} | 1.16 |
| checkHealth (level: int32): void {raises-exception, use after open, enable} | 1.16 |
| clearInput ( ): void { } | Not supported |
| clearInputProperties ( ): void { } | Not supported |
| clearOutput ( ): void { } | Not supported |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} | 1.16 |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, enable} | 1.16 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.16 |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.16 |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, enable} | 1.16 |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.16 |
| Specific |  |
| Name |  |
| readFrame (frameData: string): void {raises-exception, use after open, claim, enable} | 1.16 |
| startVideoRecording (fileName: string, overwrite: boolean, recordingTime: int32): void {raises-exception, use after open, claim, enable} | 1.16 |
| stopVideoRecording ( ): void {raises-exception, use after open, claim, enable} | 1.16 |
| takePhotograph (fileName: string, overwrite: int32): void {raises-exception, use after open, claim, enable} | 1.16 |

| Events (UML interfaces) |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Mutability | Version |
| upos::events::DataEvent |  |  | 1.16 |
| Status: | int32 | {read-only} |  |
| upos::events::DirectIOEvent |  |  | 1.16 |
| EventNumber: | int32 | {read-only} |  |
| Data: | int32 | {read-write} |  |
| Obj: | object | {read-write} |  |
| upos::events::ErrorEvent |  |  | 1.16 |
| ErrorCode: | int32 | {read-only} |  |
| ErrorCodeExtended: | int32 | {read-only} |  |
| ErrorLocus: | int32 | {read-only} |  |
| ErrorResponse | int32 | {read-write} |  |
| upos::events::OutputCompleteEvent |  | Not Supported |  |
| upos::events::StatusUpdateEvent |  |  | 1.16 |
| Status: | int32 | {read-only} |  |

General Information

The Video Capture Device name is “VideoCapture”.

- Capabilities

Video capture device class has the following capabilities:

- Get the captured frame data.

- Take a photograph and record it in a file.

- Take a movie and record it in a file.

- Read the encoded data from the bar code label.

- Detect the objects such as faces.

- Video Capture Class Diagram

The following diagram shows the relationships between the Video Capture classes.

Fig. Chap. 39-1 Video Capture Class Diagram

- Model

When video capture is enabled, the capture begins and the frame data can be retrieved by calling   the readFrame method.

The resolution and frame rate of the frame data to be acquired depend on the operation mode set in the VideoCaptureMode property.

The following shows the setting to refer to each operation mode and the property for confirming valid values.

- Capture only mode

Color space: CaptureColorSpace property										 => Valid value confirmation with CapCaptureColorSpaceList property

Resolution: CaptureResolution property 						         => Valid value confirmation with CapCaptureResolutionList property

Frame rate: CaptureFrameRate property

=> Valid value confirmation with CapCaptureMaxFrameRate property

- Photo shooting mode

Color space: CaptureColorSpace property

=> Valid value confirmation with CapCaptureColorSpaceList property

Resolution: PhotographResolution property

=> Valid value confirmation with CapPhotographResolutionList property

Frame rate: CaptureFrameRate property

=> Valid value confirmation with CapCaptureMaxFrameRate property

Remarks: You can take pictures with takePhotograph method only in this mode.

- Movie shooting mode

Color space: CaptureColorSpace property

=> Valid value confirmation with CapCaptureColorSpaceList property

Resolution: VideoRecordingResolution property

=> Valid value confirmation with the CapVideoRecordingResolutionList property

Frame rate: VideoRecordingFrameRate property

=> Valid value confirmation with CapVideoRecordingMaxFrameRate property

Remarks: It is possible to shoot movies with the startVideoRecording method only in this mode. Since the captured image / movie file is recorded in the area managed by the "hard total" service, the application must also support "hard total" service.

- Input Model

Video capture control follows a common input model of event driven input, although there are some differences.

"Control” raises a DataEvent event when the recording started by the startVideoRecording method. And it ends when the specified time elapses and the recording to the specified file is completed.

When an application calls the stopVideoRecording method to end recording, DataEvent event will not occur. "

Also, by activating the FaceCatchEnabled property, face recognition is started, and even when a face is recognized, a DataEvent event is generated.

To distinguish between Recording Completed to File by Recording and DataEvent event of Face Recognition, refer to the DataEventType property.

The control sets VCP_ET_VIDEO when recording to the file by recording is completed, and sets VCP_ET_FACECATCH to the DataEventType property when recognizing the face. "

If the AutoDisable property is true, control will be disabled automatically when queuing DataEvent event.

If the DataEventEnabled property is true, the queued DataEvent is notified to the application. Just before triggering this event, the control copies the data to the property and sets the DataEventEnabled property to false to prevent further data events firing. This allows the control to queue subsequent input data while the application is processing the current input and processing the related properties. When the application finishes processing the current input data and is ready for the next data processing, setting the DataEventEnabled property to true will notify the Data Event again.

If an error occurs in the control while reading or processing the input data, an ErrorEvent is issued, and if the DataEventEnabled property is true, the application is notified.

By reading the DataCount property you get the number of Data Events queued by the control.

All input data queued in the control can be deleted by calling the clearInput method.

All data properties entered by DataEvent or ErrorEvent occurrence can be restored to the default value by calling the clearInputProperties method.

- Bar Code Scan

By setting the BarcodeEnabled property to true for video capture, it is possible to scan the bar code by the camera.

When reading data from the bar code, the DataEvent event is queued in the scanner service object.

Scanned data is stored in the ScanData property. If the application sets the DecodeData property to true, the data is decoded to ScanDataLabel and ScanDataType.

- Individual Recognition

By setting the IndividualRecognitionEnabled property to true for video capture, it is possible for objects to be recognized by the camera.

When an object is detected, a DataEvent is queued in the object recognition service object.

The detected data is stored in the IndividualRecognitionInformation and IdividualIDs of Individual Recognition Device properties.

### Device Sharing

Video capture is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before accessing many video         capture-specific properties.

- The application must claim and enable the device before calling methods that      manipulate the device.

- See the “Summary” table for precise usage prerequisites.

Properties (UML attributes)

BarCodeEnabled Property

Syntax	BarCodeEnabled: boolean {read-write, access after open}

Remarks  	If true, bar code scan is enabled. If false, bar code scan is disabled.				This property is initialized to false by the open method.

Errors	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.					Some possible values of the exception’s ErrorCode property are:

Value 			Meaning        	                        					E_ILLEGAL		Bar code scanning function is not supported						(If it is set true)

See also	CapDecodeData Property

#### CameraAutoExposure Property

Syntax	CameraAutoExposure: boolean {read-write, access after open}

Remarks	If true, auto exposure of camera is enabled.						If false, auto exposure of camera is disabled.						This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20. 					Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified. 							Or it does not support this function.

See also	CapCameraAutoExposition Property

#### CameraAutoFocus Property

Syntax	CameraAutoFocus: boolean {read-write, access after open}

Remarks	If true, auto focus of camera is enabled. 								If false, auto focus of camera is disabled.								This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20. 					Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified. 								Or it does not support this function.

See also	CapCameraAutoFocus Property

CameraAutoGain Property

Syntax	CameraAutoGain: boolean {read-write, access after open}

Remarks	If true, auto gain of camera is enabled.							If false, auto gain of camera is disabled.							This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed.				For further information, see “Errors" on page Intro-20. 					Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified. 								Or it does not support this function.

See also	CapCameraAutoGain Property

CameraAutoWhiteBalance Property

Syntax	CameraAutoWhiteBalance: boolean {read-write, access after open}

Remarks	If true, auto white balance of camera is enabled.
		If false, auto white balance of camera is disabled.						This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20. 					Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified. 								Or it does not support this function.

See also	CapCameraAutoWhiteBalance Property

#### CameraBrightness property

#### Syntax		CameraBrightness: int32 {read-write, access after open}

#### Remarks	Indicate the brightness of camera. 									Valid values range from 0 to 100.										This property is initialized by the open method.

#### Errors		A UposException may be thrown when this property is accessed. 						For further information, see “Errors" on page Intro-20. 							Some possible values of the exception’s ErrorCode property are:

#### Value			Meaning                                                         			E_ILLEGAL		An invalid value was specified. 									Or it does not support this function.

#### See Also		CapCameraBrightness Property

#### CameraContrast Property

Syntax		CameraContrast: int32 {read-write, access after open}

Remarks	Indicate the contrast of the camera. 								Valid values range from 0 to 100.									This property is initialized by the open method.

Errors		A UposException may be thrown when this property is accessed. 					For further information, see “Errors" on page Intro-20. 						Some possible values of the exception’s ErrorCode property are:

Value			Meaning                                                         				E_ILLEGAL		An invalid value was specified. 									Or it does not support this function.

See Also		CapCameraContrast Property

CameraExposure Property

Syntax	CameraExposure: int32 {read-write, access after open}

Remarks	Indicate the exposure of camera. Valid values range from 0 to 100.　				This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 					For further information, see “Errors" on page Intro-20. 						Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified.                          						Or it does not support this function.

See also	CapCameraExposure Property

#### CameraGain Property

Syntax	CameraGain: int32 {read-write, access after open}

Remarks	Indicate the gain of camera. Valid values range from 0 to 100.			This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20. 					Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified. 							Or it does not support this function.

See also 	CapCameraGain Property

#### CameraHorizontalFlip Property

Syntax	CameraHorizontalFlip: boolean {read-write, access after open}

Remarks	If true, horizontal flip of camera is enabled.							If false, horizontal flip of camera is disabled.							This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 					For further information, see “Errors" on page Intro-20. 						Some possible values of the exception’s ErrorCode property are:

Value		Meaning

E_ILLEGAL	An invalid value was specified. 			     			Or it does not support this function.

See Also	CapCameraHorizontalFlip property

#### CameraHue Property

Syntax	 CameraHue: int32 {read-write, access after open}

Remarks	Indicate the hue of camera. Valid values range from 0 to 100.					This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 					For further information, see “Errors" on page Intro-20. 						Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified.                                 				Or it does not support this function.

See also	CapCameraHue Property

#### CameraSaturation Property

Syntax	CameraSaturation: int32 {read-write, access after open}

Remarks	Indicate the saturation of camera. 									Valid values range from 0 to 100. 								This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 					For further information, see “Errors" on page Intro-20. 						Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified. 							Or it does not support this function.

See also	CapCameraSaturation Property

- CameraVerticalFlip Property

Syntax	CameraVerticalFlip: boolean {read-write, access after open}

Remarks	If true, vertical flipping of the camera is enabled. 						If false, vertical flipping of camera is disabled. 						This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20. 					Some possible values of the exception’s ErrorCode property are:

Value 			Meaning

E_ILLEGAL		An invalid value was specified. 								Or it does not support this function.

See also	CapCameraVerticalFlip Property

CapCameraAutoExposition Property

Syntax	CapCameraAutoExposition: boolean {read-only, access after open}

Remarks	If true, can change the auto exposition of camera. 						If false, cannot change the auto exposition of camera. 					This property is initialized by the open method.

Errors    	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See also	CameraAutoExposition Property

CapCameraAutoFocus Property

Syntax	CapCameraAutoFocus: boolean {read-only, access after open}

Remarks	If true, can change the auto focus of camera. 						If false, cannot change the auto focus of camera. 						This property is initialized by the open method.

Errors    	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See also	CameraAutoFocus Property

CapCameraAutoGain Property

Syntax	CapCameraAutoGain: boolean {read-only, access after open}

Remarks	If true, automatic gain change of the camera is possible. 					If false, automatic gain change of camera is not possible. 					This property is initialized by the open method.

Errors   	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See also	CameraAutoGain Property

CapCameraAutoWhiteBalance Property

Syntax	CapCameraAutoWhiteBalance: boolean {read-only, access after open}

Remarks	If true, auto white balance of camera is possible. 						If false, auto white balance of camera is not possible. 					This property is initialized by the open method.

Errors   	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See also	CameraAutoWhiteBalance Property

CapCameraBrightness Property

Syntax	CapCameraBrightness: boolean {read-only, access after open}

Remarks	If true, the brightness of camera can be changed. 						If false, the brightness of the camera cannot be changed. 					This property is initialized by the open method.

Errors    	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See also	CameraBrightness Property

CapCameraContrast Property

Syntax	CapCameraContrast: boolean {read-only, access after open}

Remarks	If true, can change the contrast of camera. 							If false, cannot change the contrast of camera.				      		This property is initialized by the open method.

Errors     	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See also	CameraContrast Property

CapCameraExposure Property

Syntax	CapCameraExposure: boolean {read-only, access after open}

Remarks	If true, can change the exposure of camera. 						If false, cannot change the exposure of camera. 						This property is initialized by the open method.

Errors    	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See also	CameraExposure Property

CapCameraGain Property

Syntax	CapCameraGain: boolean {read-only, access after open}

Remarks	If true, can change the gain of camera. 							If false, cannot change the gain of camera. 						This property is initialized by the open method.

Errors    	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See also	CameraGain Property

CapCameraHorizontalFlip Property

Syntax	 CapCameraHorizontalFlip: boolean {read-only, access after open}

Remarks	If true, can change the horizontal flip of camera. 						If false, cannot change the horizontal flip of camera. 					This property is initialized by the open method.

Errors    	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See also	CameraHorizontalFlip Property

CapCameraHue Property

Syntax	CapCameraHue: boolean {read-only, access after open}

Remarks	If true, the hue of the camera can be changed. 						If false, hue of the camera cannot be changed. 						This property is initialized by the open method.

Errors 	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See also	CameraHue Property

CapCameraSaturation Property

Syntax	CapCameraSaturation: boolean {read-only, access after open}

Remarks	If true, can change the saturation of camera. 						If false, cannot change the saturation of camera. 						This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See also	CameraSaturation Property

CapCameraVerticalFlip Property

Syntax	 CapCameraVerticalFlip: boolean {read-only, access after open}

Remarks	If true, can change the vertical flip of camera. 						If false, cannot change the vertical flip of camera. 	 					This property is initialized by the open method.

Errors    	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See also	CameraVerticalFlip Property

CapCapture Property

Syntax	CapCapture: boolean {read-only, access after open}

Remarks	If true, it supports the capture function and can call the readFrame method 			and retrieve the frame data.
		If false, it does not support the capture function and cannot retrieve the 			frame data. This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See also	readFrame Method

CapCaptureColorSpace Property

Syntax	CapCaptureColorSpace: boolean {read-only, access after open}

Remarks	If true, can change the capture color space.
		If false, cannot change the capture color space. 						This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

#### CapCaptureColorSpaceList Property

Syntax	CapCaptureColorSpaceList: string {read-only, access after open}

Remarks	Color space information supported by the device is indicated in a 				comma-separated list. Each color space information is composed 				of the following information and is shown in the following order 				separated by a colon (":").								This property is initialized by the open method.

Parameter		Description

Color space ID		ID for identifying the color space of RGB, YUV 					422, etc.

Depth			Number of bits per 1 pixel

Errors		A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See also	CaptureColorSpace Property

CapCaptureFrameRate Property

Syntax	CapCaptureFrameRate: boolean {read-only, access after open}

Remarks	If true, can change the capture frame rate.
		If false, cannot change the capture frame rate.						This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

CapCaptureMaxFrameRate Property

Syntax	CapCaptureMaxFrameRate: int32 {read-only, access after open}

Remarks	Indicates the maximum frame rate that can be set for the 					CaptureFrameRate property.								This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 				For further Information, see “Errors" on page Intro-20.

See also	CaptureFrameRate Property

CapCaptureResolution Property

Syntax	CapCaptureResolution: boolean {read-only, access after open}

Remarks	If true, capture resolution is enabled.					   If false, capture resolution is disabled.				             This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See also	CaptureResolution Property

CapCaptureResolutionList Property

Syntax	CapCaptureResolutionList: string {read-only, access after open}

Remarks	Indicating the comma-separated list of possible resolutions for the CaptureResolution property. Resolution is indicated in "horizontal x height" format. For example, when you support 320x240, 640x480, 640x360, it is the following. "320 x 240, 640 x 480, 640 x 360".
This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See also	CaptureResolution Property

CapDecodeData Property

Syntax	CapDecodeData: boolean {read-only, access after open}

Remarks	If true, the image scanner can read the bar code data. 					The scanned bar code data is sent to the scanner service.					This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

#### CapIndividualRecognition Property

Syntax	Cap Individual Recognition: boolean {read-only, access after open}

Remarks	If true, individual recognition function is supported.
			If false, individual recognition function is not supported.						If this property is true, individual recognition can be done by setting 				IndividualRecognitionEnabled property to true. 							If false, individual recognition cannot be performed.						This property is initialized by the open method.

Errors		A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See also	IndividualRecognitionEnabled Property

#### CapPhotograph Property

Syntax		CapPhotograp: boolean {read-only, access after open}

Remarks	If true, photograph function is supported.								If false, photograph function is not supported. 							If true, it is possible taking a photograph by calling the takePhotograph 				method. If false, it is not possible taking a photograph.						This property is initialized by the open method.

Errors			A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See also	   	takePhotograph Method

#### CapPhotographResolution Property

Syntax		CapPhotographResolution: boolean {read-only, access after open}

Remarks	If true, it is possible changing the photograph resolution.
			If false, it is not possible changing the photograph resolution.					This property is initialized by the open method.

Errors			A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

#### CapPhotographResolutionList Property

Syntax		CapPhotographResolutionList: 								                  	   string {read-only, access after open}

Remarks	A comma-separated list of possible resolutions for							PhotographResolution property.									Resolution is indicated by Syntax "Horizontal x Vertical".						For example, when you support 320x240, 640x480, 640x360, it is the 				following. "320x240,640x480,640x360" 								This property is initialized by the open method.

Errors			A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See also	PhotographResolution Property

#### CapPhotographType Property

Syntax		CapPhotographType: boolean {read-only, access after open}

Remarks	If true, photograph type can be changed.
			If false, photograph type cannot be changed.							This property is initialized by the open method.

Errors			A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

#### CapPhotographTypeList Property

Syntax		CapPhotographTypeList: string {read-only, access after open}

Remarks	A comma-separated list of image format values that can be set for 					the PhotographType property.									For example, when supporting BMP and JPEG, it is the following. 					"BMP, JPEG"

Note: The notation contents may be different depending on the device.			This property is initialized by the open method.

Errors			A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See also	PhotographType Property

#### CaptureColorSpace Property

Syntax	CaptureColorSpace: string {read-write, access after open}

Remarks	Indicates the color space ID of the frame data to be acquired by the 				readFrame method. Valid values are one of the values listed in the 				CapCaptureColorSpaceList property. 							This property is referred to regardless of which operation mode is set by 			VideoCaptureMode property.								This property is initialized by the open method.

Errors		A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20. 					Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified.

See also	CapCaptureColorSpaceList Property、VideoCaptureMode Property、			readFrame Method

#### CaptureFrameRate Property

Syntax	CaptureFrameRate: int32 {read-write, access after open}

Remarks	Indicates the frame rate of frame data to be acquired by the readFrame 			method. Valid values range from 1 to CapCaptureMaxFrameRate 			property. This property is only referenced when VCP_VCM_CAPTURE is 			set in VideoCaptureMode property.							This property is initialized by the open method.

Errors			A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

Value			Meaning

E_ILLEGAL		An invalid value was specified.

See also	CapCaptureMaxFrameRate Property, VideoCaptureMode Property、	 		readFrame Method

CaptureResolution Property

Syntax	 	CaptureResolution: string {read-write, access after open}

Remarks	Indicates the resolution of the frame data acquired by the readFrame method. Valid values are one of those listed in CapCaptureResolutionList property.	　　This property is only referenced when VCP_VCM_CAPTURE is set in VideoCaptureMode property.					　　     This property is initialized by the open method.

Errors			A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.					Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified.

See also		CapCaptureResolutionList Property、VideoCaptureMode Property、				readFrame Method

#### CapVideoRecording Property

Syntax		CapVideoRecording: boolean {read-only, access after open}

Remarks	If true, video recording function is supported.
			If false video recording function is not supported.							If this property is true, movie recording can be done by calling the 					startVideoRecording method. 									If false, movie recording cannot be performed.							This property is initialized by the open method.

Errors			A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See also		StartVideoRecording Property

#### CapVideoRecordingFrameRate Property

Syntax		CapVideoRecordingFrameRate	: boolean {read-only, access after open}

Remarks	If true, video recording frame rate can be changed.
			If false, video recording frame rate cannot be changed.						This property is initialized by the open method.

Errors			A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

#### CapVideoRecordingMaxFrameRate Property

Syntax		CapVideoRecordingMaxFrameRate: int32 {read-only, access after open}

Remarks	 Indicates the maximum frame rate that can be set in 						VideoRecordingFrameRate property. 								This property is initialized by the open method.

Errors     	A UposException may be thrown when this property is accessed. 					For further information, see “Errors" on page Intro-20.

See also		VideoRecordingFrameRate Property

#### CapVideoRecordingResolution Property

Syntax		CapVideoRecordingResolution: boolean {read-only, access after open}

Remarks		If true, video recording resolution can be changed. 
			If false, video recording resolution cannot be changed.						This property is initialized by the open method.				Errors		A UposException may be thrown when this property is accessed. 					For further information, see “Errors" on page Intro-20.

#### CapVideoRecordingResolutionList Property

Syntax		CapVideoRecordingResolutionList: string {read-only, access after open}

Remarks	A comma-separated list of possible resolutions for the 						VideoRecordingResolution property.								Resolution is indicated by "Horizontal x Vertical" format.						For example, when it supports 320x240, 640x480, 640x360, it is the 				following. "320x240,640x480,640x360"								This property is initialized by the open method.				Errors		A UposException may be thrown when this property is accessed.   					For further information, see “Errors" on page Intro-20.

See also		VideoRecordingResolution Property

#### CapVideoRecordingType Property

Syntax		CapVideoRecordingType: boolean {read-only, access after open}

Remarks	If true, video recording type can be changed.
			If false, video recording type cannot be changed.							This property is initialized by the open method.

Errors		A UposException may be thrown when this property is accessed. 					For further information, see “Errors" on page Intro-20.

#### CapVideoRecordingTypeList Property

Syntax		CapVideoRecordingTypeList: string {read-only, access after open}

Remarks	A comma-separated list of image format values that can be set for the 				VideoRecordingType property.									For example, when AVI_IYUV, AVI_MJPG is supported, it is the 					following. "AVI_IYUV, AVI_MJPG"								Note:    The notation contents may be different depending on the device.			　　     This property is initialized by the open method.					Errors		A UposException may be thrown when this property is accessed. 					For further information, see “Errors" on page Intro-20.

See also		VideoRecordingType Property

IndividualRecognitionEnabled Property

Syntax 		IndividualRecognitionEnabled:										boolean{read-write, access after open}

Remarks	If true individual recognition is enabled. 								If false, individual recognition is disabled.                	   					This property is initialized to false by the open method.

Errors		A UposException may be thrown when this property is accessed. 						For further information, see “Errors" on page Intro-20.							Some possible values of the exception’s ErrorCode property are:		 			Value			Meaning                                        						E_ILLEGAL		Individual recognition function is not supported 								(If it is set true)

See also		CapIndividual Recognition Property

PhotographResolution Property

Syntax		PhotographResolution: string {read-write, access after open}

Remarks	It shows the resolution of the frame data acquired by the readFrame					method and the photograph taken with the takePhotograph method. 					Valid values are one of those listed in CapPhotographResolutionList 					property. 												This property is referenced only when VCP_VCM_PHOTO is set in 					VideoCaptureMode property.										This property is initialized by the open method.

Errors		A UposException may be thrown when this property is accessed. 					For further information, see “Errors" on page Intro-20. 						Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified.

See also	CapPhotographResolutionList Property, VideoCaptureMode Property, readFrame Method,  takePhotograph Method

PhotographType Property

Syntax		PhotographType: int32 {read-write, access after open}

Remarks	Indicates the image format of photos taken with the takePhotograph method. Valid values are one of the values listed in the CapPhotographTypeList property.	 			     			            This property is referenced only when VCP_VCM_PHOTO is set in   VideoCaptureMode property.					            This property is initialized by the open method.

Remarks	Indicates the image format of photos taken with the takePhotograph method. Valid values are one of the values listed in the            CapPhotographTypeList property.				            This property is referenced only when VCP_VCM_PHOTO is set in VideoCaptureMode property.					            This property is initialized by the open method

Errors		A UposException may be thrown when this property is accessed. 						For further information, see “Errors" on page Intro-20. 							Some possible values of the exception’s ErrorCode property are:

Value			Meaning							E_ILLEGAL		An invalid value was specified.

See also		CapPhotographTypeList Property、takePhotograph Method

#### VideoCaptureMode Property

Syntax		VideoCaptureMode: int32 {read-write, access after open}

Remarks	Indicate the operation mode of video capture.             			　　						Valid values are as follows

Parameter		Description

VCP_VCMODE_CAPTURE									This mode is for capture only. 
	       		The values of the CaptureColorSpace,                                             				CaptureResolution, and aptureFrameRate 						properties are applied to the color space, 							resolution, and frame rate of frame data that can 						be acquired with the readFrame method.

VCP_VCMODE_PHOTO

This mode is for capture and taking photograph. 			The values of the CaptureColorSpace and 			CaptureFrameRate properties are applied to the 			color space and frame rate of the frame data that 			can be acquired by the readFrame method, and 			the resolution is applied to the resolution of the 			CapPhotographResolution property.

VCP_VCMODE_VIDEO

This mode is for capture and movie shooting. The value of the CaptureColorSpace property is applied to the color space of the frame data that can be acquired by the readFrame method, the values of the CapVideoRecordingResolution property and the CapVideoRecordingFrameRate property are applied to the resolution and the frame rate.

This property is initialized to VCP_VCMODE_CAPTURE by the open method. Indicate the operation mode of video capture.

Errors 		A UposException may be thrown when this property is accessed. 	    					For further information, see “Errors" on page Intro-20.

See also		CaptureColorSpace Property、CaptureResolution Property, 		CaptureFrameRate Property、CapPhotographResolution Property、	CapVideoRecordingResolution Property、                             CapVideoRecordingFrameRate Property、readFrame Method

VideoRecordingFrameRate Property

Syntax		VideoRecordingFrameRate; int32 {read-write, access after open}

Remarks	Indicates the frame rate of the frame data acquired by the readFrame 				method and the movie taken with the startVideoRecording method. 				Valid values range from 1 to CapVideoRecordingMaxFrameRate　				property.　　											This property is only referred when VCP_VCM_VIDEO is set in 					VideoCaptureMode property. 									This property is initialized by the open method.

Errors			A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20. 					Some possible values of the exception’s ErrorCode property are:

Value			Meaning  		     					E_ILLEGAL		An invalid value was specified.

See also		CapVideoRecordingMaxFrameRate Property、						VideoCaptureMode Property、readFrame Method, startVideoRecording Method

VideoRecordingResolution Property

Syntax		VideoRecordingResolution: int32 {read-write, access after open}

Remarks	Indicates the resolution of the frame data acquired by the readFrame 					method and the photograph taken with the startVideoRecording 						method.	 Valid values are one of the values listed in the							CapVideoRecordingResolutionList property.								This property is only referred when VCP_VCM_VIDEO is set in 						VideoCaptureMode property.										This property is initialized by the open method.

Errors			A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20. 					Some possible values of the exception’s ErrorCode property are:

Value			Meaning			           	E_ILLEGAL		An invalid value was specified.

See also	CapVideoRecordingResolutionList Property, 					VideoCaptureMode Property、readFrame Method 				startVideoRecording Method

VideoRecordingType Property

Syntax		VideoRecordingType; string {read-write, access after open}

Remarks	Indicate the shape of the movie taken with the startVideoRecording 					method. 												Valid values are one of those listed in CapVideoRecordingTypeList 					property. 												This property is only referred when VCP_VCM_VIDEO is set in 						VideoCaptureMode property.										This property is initialized by the open method.

Errors			A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20. 					Some possible values of the exception’s ErrorCode property are:

Value			Meaning             		    		E_ILLEGAL		An invalid value was specified.

See also		CapVideoRecordingTypeList Property, 					startVideoRecording Method

## Methods (UML operations)

readFrame Method

Syntax		readFrame (frameData: string): 									void {raises-exception, use after open, claim, enable}			Parameter   	Description		                            				frameData	Indicates the area where frame data is stored.

Remarks	Acquires the captured frame data and stores it in frameData.					The color space and resolution of frame data differs depending on the 				operation mode set in the VideoCaptureMode property.						For details, refer to the VideoCaptureMode property.						This method is executed synchronously.

Errors		A UposException may be thrown when this method is invoked. 					For further information, see “Errors" on page Intro-20.						Some possible values of the exception's ErrorCode property are:

Value		Meaning

E_ILLEGAL	This function is not supported

See also		VideoCaptureMode Property

startVideoRecording method

Syntax	startVideoRecording (fileName : string, overwrite: boolean, recordingTime: int32):			            				void{raises-exception, use after open, claim, enable}              Parameter	Description

filename	Specify the name of the movie file to be recorded. 		Overwrite	Specify the behavior when the same name file exists. 				If true, it is overwritten. 								If false, it will raise the UposException. 			recordingTime 	Specify the time for recording in seconds.						If FOREVER (-1) is specified, recording will continue                        			until the stopVideoRecording method is called.

Remarks	Recording starts with the setting contents of the CaptureColorSpace and 				VideoRecordingResolution properties, and recording starts in the format         			set by the VideoRecordingType property. 							This method is executed asynchronously.								When the time specified in RecordingTime has elapsed, or by calling the 				stopVideoRecording method, recording is completed and the movie file 				specified by fileName is recorded.								Also, S_BUSY is set in the Status property during movie execution. 				The place where video files are recorded is the area managed by 					"hard  total" service.							Errors		A UposException may be thrown when this method is invoked. 					For further information, see “Errors" on page Intro-20.						Some possible values of the exception's ErrorCode property are:

Value		Meaning     				                 	E_ILLEGAL	fileName is too long or contains characters that  					cannot be used, or 0 is specified for    						recordingTime.					E_EXISTS	fileName already exists. (If overwrite is false)		E_BUSY	Cannot execute because it is recording.

See also		CaptureColorSpace Property、VideoRecordingResolution Property、				VideoRecordingType Property、stopVideoRecording Method

stopVideoRecording method

Syntax		stopVideoRecording ( ):

void {raises-exception, use after open, claim, enable}

Remarks	The recording process started by the startVideoRecording method has 				ended and the recording of the movie image file is completed.

Errors		A UposException may be thrown when this method is invoked. 					For further information, see “Errors" on page Intro-20.						Some possible values of the exception's ErrorCode property are:

Value		Meaning

E_ILLEGAL	It is not recorded.

See also		startVideoRecording Method

takePhotograph Method

Syntax		takePhotograph (fileName: string, overwrite: int32, timeout: int32):			                               void{raises-exception, use after open, claim, enable}

Parameter	Description

fileName	Specify the image file name to be recorded.					overwrite	Specify the behavior when the same name file exists. 						If true it overwrites. If false, UposException is thrown.

Remarks	Take photos with setting contents of CaptureColorSpace property, 		PhotographResolution property, PhotographType property and record 	images.	Before calling this method, it needs to set the  			VideoCaptureMode property to VCP_VCM_PHOTO and change to the 	photo shooting mode. This method is executed synchronously.		The location where image files are recorded is the area managed by 		"hard total" service.

Errors		A UposException may be thrown when this method is invoked.					For further information, see “Errors" on page Intro-20.						Some possible values of the exception's ErrorCode property are:

Value           	Meaning

E_ILLEGAL	One of the following occurred. 									FileName is too long or contains unusable characters.						VideoCaptureMode property is not VCM_PHOTO				E_EXISTS	fileName already exist. (When overwrite=false)

See also		VideoCaptureMode Property、CaptureColorSpace Property、				PhotographResolution Property、PhotographType Property

## Events (UML interfaces)

DataEvent

<<event>>	upos::events::DataEvent

Status:int32{read-only}

Description 	Notifies the application when data from the Video Capture device is available to be read.

Attributes	This event contains the following attributes:

Attribute	Type		Description

Status	int32		Set to 0.

Remarks	Before this event is delivered, the Video Capture movie image is placed into readFrame.

This event is to be used only for those types of vendor specific functions that are not otherwise described.

Use of this event may restrict the application program programform being used with other vendor’s devices which may not have any knowledge of the Service’s need for this event.

See Also	“Events" on page Intro-19, directIO method

DirectIOEvent

<<event>>	upos::events::DirectIOEvent

EventNumber	: int32 {read-only}					 Data			: int32 {read-write}
Obj			: object {read-write}

Description 	Provides Service information directly to the application. This event provides a means for a vendor-specific Video Capture Service to provide events to the application that are not otherwise supported by the Control.

Attributes	This event contains the following attributes:

Attribute	Type	Description

EventNumber	int32	Event number whose specific values are assigned by the Service.

Data	int32	Additional numeric data. Specific values vary by the EventNumber and the Service. This attribute is settable.

Obj	object	Additional data whose usage varies by the EventNumber and the Service. This attribute is settable.

Remarks	This event is to be used only for those types of vendor specific functions that are not otherwise described.

Use of this event may restrict the application program programform being used with other vendor’s devices which may not have any knowledge of the Service’s need for this event.

See Also	“Events" on page Intro-19, directIO method

ErrorEvent

<<event>>	  upos::events::ErrorEvent

ErrorCode		: int32 {read-only}

ErrorCodeExtended	: int32 {read-only}

ErrorLocus		: int32 {read-only}

ErrorResponse		: int32 {read-write}

Description	Notifies the application that a Video Capture Device error has been detected and suitable response by the application is necessary to process the error condition.

Attributes	This event contains the following attributes:

Attributes	Type	Description

ErrorCode	int32	Error code causing the error event. 				See a list of Error Codes on page 20.

ErrorCodeExtended	int32	Extended Error code causing the error event.

If ErrorCode is E_EXTENDED, then see values below. Otherwise, it may contain a Service-specific value.

ErrorLocus	int32	Location of the error. If EL_OUTPUT is specified. An error occurred during asynchronous action.

ErrorResponse	int32	Pointer to the error event response. See ErrorResponse below for values.

The ErrorLocus attribute has one of the following values:

Value	Meaning

EL_OUTPUT	Error occurred while processing asynchronous output.

EL_INPUT	Error occurred while gathering or processing event-driven input. No previously buffered input data is available.

EL_INPUT_DATA	Error occurred while gathering or processing event-driven input, and some previously buffered data is available.

The application’s error event handler can set the ErrorResponse attribute to one of the following values:

Value	Meaning

ER_RETRY	Retry sending the data. The error state is exited.
May be valid for some input devices when the locus is EL_INPUT, in which case the input is retried and the error state is exited. Typically, valid for asynchronous output devices when the locus is EL_OUTPUT, in which case the asynchronous output is retried and the error state is exited. This is the default response when the locus is EL_OUTPUT.

ER_CLEAR	Valid for all loci: EL_INPUT, EL_INPUT_DATA, and EL_OUTPUT.  Clear all buffered input or output data (including all asynchronous output).  The error state is exited.  This is the default response when the locus is EL_INPUT.

ER_CONTINUEINPUT 
Only valid when the locus is EL_INPUT_DATA.
Acknowledges that a data error has occurred and directs the Device to continue input processing. The Device remains in the error state and will deliver additional DataEvents as directed by the DataEventEnabled property. When all input has been delivered and DataEventEnabled is again set to true, then another ErrorEvent is delivered with locus EL_INPUT.
This is the default response when the locus is EL_INPUT_DATA.

Remarks	This event is enqueued when an error is detected and the Device’s State transitions into the error state. Input error events are not delivered until DataEventEnabled is true, so that proper application sequencing occurs.

Unlike a DataEvent, the Device does not disable further DataEvents or input ErrorEvents; it leaves the DataEventEnabled property value at true. 　　　Note that the application may set DataEventEnabled to false within its event handler if subsequent input events need to be disabled for a period of time.

See Also 	“Device Input Model" on page Intro-22, “Error Handling" on page Intro-23, “Device Output Models" on page Intro-25.

StatusUpdateEvent

<< event >>            upos::events::StatusUpdateEvent

Status: int32 {read-only}

Description	Notifies the application that there is a change in the power status of the Video Capture device.

Attributes		This event contains the following attribute:

Attributes	Type	Description

Status		int32	Indicates a change in the power status of the unit.

Note that Release 1.3 added Power State Reporting with additional Power 	reporting StatusUpdateEvent values.

The Update Firmware capability added additional Status values for communicating the status/progress of an asynchronous update firmware process. See “StatusUpdateEvent” description on page 1-34.

- ＣＨＡＰＴＥＲ 40

Individual Recognition

This Chapter defines the Individual Recognition device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.16 | open |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.16 | open |
| CapPowerReporting: | int32 | {read-only} | 1.16 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.16 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.16 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.16 | open |
| CheckHealthText: | string | {read-only} | 1.16 | open |
| Claimed: | boolean | {read-only} | 1.16 | open |
| DataCount: | int32 | {read-only} | 1.16 | open |
| DataEventEnabled: | boolean | {read-write} | 1.16 | open |
| DeviceEnabled: | boolean | {read-write} | 1.16 | open, claim |
| FreezeEvents: | boolean | {read-write} | 1.16 | open |
| OutputID: | int32 | {read-only} | 1.16 | Not Supported |
| PowerNotify: | int32 | {read-write} | 1.16 | open |
| PowerState: | int32 | {read-only} | 1.16 | open |
| State: | int32 | {read-only} | 1.16 | -- |
| DeviceControlDescription: | string | {read-only} | 1.16 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.16 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.16 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.16 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.16 | open |
| PhysicalDeviceName: | string | {read-only} | 1.16 | Open |

| Specific | Type | Mutability | Version | May Use After |
| --- | --- | --- | --- | --- |
| CapIndividualList: | string | {read-only} | 1.16 | open |
| IndividualRecognitionFilter | string | {read-writer} | 1.16 | open |
| IndividualRecognitionInformation | string | {read-only} | 1.16 | open |
| IndividualIDs: | string | {read-write} | 1.16 | open, claim & enable |

| Methods (UML operations) |  |
| --- | --- |
| Common |  |
| Name | Version |
| open (logicalDeviceName: string): void {raises-exception} | 1.16 |
| close ( ): void {raises-exception, use after open} | 1.16 |
| claim (timeout: int32): void {raises-exception, use after open} | 1.16 |
| release ( ): void {raises-exception, use after open, claim} | 1.16 |
| checkHealth (level: int32): void {raises-exception, use after open, enable} | 1.16 |
| clearInput ( ): void { } | Not supported |
| clearInputProperties ( ): void { } | Not supported |
| clearOutput ( ): void { } | Not supported |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, enable} | 1.16 |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} | 1.16 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.16 |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.16 |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, enable} | 1.16 |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.16 |

| Events (UML interfaces) |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Mutability | Version |
| upos::events::DataEvent |  |  | 1.16 |
| Status: | int32 | {read-only} |  |
| upos::events::DirectIOEvent |  |  | 1.16 |
| EventNumber: | int32 | {read-only} |  |
| Data: | int32 | {read-write} |  |
| Obj: | object | {read-write} |  |
| upos::events::ErrorEvent |  |  | 1.16 |
| ErrorCode: | int32 | {read-only} |  |
| ErrorCodeExtended: | int32 | {read-only} |  |
| ErrorLocus: | int32 | {read-only} |  |
| ErrorResponse: | int32 | {read-write} |  |
| upos::events::OutputCompleteEvent |  | Not Supported | 1.16 |
| upos::events::StatusUpdateEvent |  |  | 1.16 |
| Status: | int32 | {read-only} |  |

## General Information

The Individual Recognition programmatic name is “Individual Recognition”.

- Capabilities

The Individual Recognition has the following set of capabilities:

Analyzes the image of the camera and recognizes Individuals such as people and balls.

- Individual Recognition Class Diagram

The following diagram shows the relationships between the Individual Recognition classes.

Fig. Chap.40-1 Individual Recognition Class Diagram

- Model

The Individual Recognition follows the general “Device Input Model” for event-driven input:

#### Input Model

The readValue method follows the UnifiedPOS Input model.

- If the AutoDisable property is true, then the device automatically disables itself when a DataEvent is enqueued.

- An enqueued DataEvent can be delivered to the application when the DataEventEnabled property is true and other event delivery requirements are met. Just before delivering this event, data is copied into corresponding properties, and further data events are disabled by setting DataEventEnabled to false. This causes subsequent input data to be enqueued while the application processes the current input and associated properties. When the application has finished processing the current input and is ready for more data, it reenables events by setting DataEventEnabled to true.

- An ErrorEvent (or events) is enqueued if an error occurs while gathering or processing input, and is delivered to the application when DataEventEnabled is true and other event delivery requirements are met.

- The DataCount property may be read to obtain the total number of enqueued DataEvents.

- All enqueued input may be deleted by calling clearInput. See the clearInput method description for more details.

- All data properties that are populated as a result of firing a DataEvent or ErrorEvent can be set back to their default values by calling the clearInputProperties method.

- Identifiable individuals are indicated by the CapIndividualList property.

- Check the functions supported by the device, set validity / invalidity, etc. with the IndividualRecognitionInformation property.

- Recognized data is stored in the IndividualRecognitionInformation property, IndividualIDs.

### Device Sharing

The Individual Recognition is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before the device begins reading input.

- See the “Summary” table for precise usage prerequisites.

## Properties (UML attributes)

CapIndividualList Property

Syntax	CapIndividualList: string {read-only, access after open}

Remarks	Recognizable Individual information is indicated by the list separated by a separator ",".

Each Individual information consists of the following information and is shown in the following order, separated with a colon (":").

Parameter	Meaning

IndividualID	An ID indicated an identifiable Individual

IndividualName	A Name of an Individual.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	“IndividualIDs" Property on page XX-11

IndividualRecognitionFilter Property

Syntax	IndividualRecognitionFilter: string {read-write, access after open}

Remarks	Holds data indicating the following.

Individual Recognition Function Information:

- Support for various functions (supported functions are defined by the device).

- Valid / invalid state of various functions.

- Types handled by various functions (e.g., "male" "female" in gender recognition, etc.).

- Filter setting of various functions.

All Individual recognition function information data is defined by the device. By referring to these contents, the application can determine the support scope etc. Thereby, the application can control each function by changing the valid / invalid state and / or the filter setting of various functions.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

IndividualRecognitionInformation Property

Syntax	IndividualRecognitionInformation: string {read-only, access after open}

Remarks	Holds data indicating the following.  						 Individual recognition input data.                                     		              All Individual recognition input data is defined by the device.

Errors			A UposException may be thrown when this property is accessed. 									For further information, see “Errors" on page Intro-20.

IndividualIDs Property

Syntax	IndividualIDs: string {read-write, access after open}

Remarks	Holds an IndividualID recognized by Individual recognition and indicated by separated with a colon (":").

Its value is set prior to a DataEvent being delivered to the application.

Errors			A UposException may be thrown when this property is accessed. 									For further information, see “Errors" on page Intro-20.

See Also	CapIndividualList Property

- ＣＨＡＰＴＥＲ 41

Sound Recorder

This Chapter defines the Sound Recorder device category.

Summary

| Properties(UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.16 | open |
| CapCompareFirmwareVersion: | int32 | {read-only} | 1.16 | open |
| CapPowerReporting: | boolean | {read-only} | 1.16 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.16 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.16 | open |
| CapUpdateStatistics: | string | {read-only} | 1.16 | open |
| CheckHealthText: | boolean | {read-only} | 1.16 | open |
| Claimed: | int32 | {read-only} | 1.16 | open |
| DataCount: | boolean | {read-only} | 1.16 | open |
| DataEventEnabled: | boolean | {read-write} | 1.16 | open |
| DeviceEnabled: | boolean | {read-write} | 1.16 | open, claim |
| FreezeEvents: | int32 | {read-write} | 1.16 | open |
| OutputID: | int32 | {read-only} | 1.16 | Not Supported |
| PowerNotify: | int32 | {read-write} | 1.16 | open |
| PowerState: | boolean | {read-only} | 1.16 | open |
| State: | boolean | {read-only} | 1.16 | open |
| DeviceControlDescription: | string | {read-only} | 1.16 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.16 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.16 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.16 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.16 | Open |
| Properties (Continued) |  |  |  |  |
| Common(continued) | Type | Mutability | Version | May Use After |
| PhysicalDeviceName: | string | {read-only} | 1.16 | open |
| Specific | Type | Mutability | Version | May Use After |
| CapChannel: | boolean | {read-only} | 1.16 | open |
| CapSamplingRate: | boolean | {read-only} | 1.16 | open |
| CapSoundType: | boolean | {read-only} | 1.16 | open |
| CapRecordingLevel: | boolean | {read-only} | 1.16 | open |
| CapChannelList: | string | {read-only} | 1.16 | open |
| CapSamplingRateList: | string | {read-only} | 1.16 | open |
| CapSoundTypeList: | string | {read-only} | 1.16 | open |
| Channel: | string | {read-write} | 1.16 | open, claim & enable |
| SamplingRate: | string | {read-write} | 1.16 | open, claim & enable |
| SoundType: | string | {read-write} | 1.16 | open, claim & enable |
| RecordingLevel: | int32 | {read-write} | 1.16 | open, claim & enable |

| Methods(UML operations) |  |
| --- | --- |
| Common |  |
| Name | Version |
| open (logicalDeviceName: string): void {raises-exception} | 1.16 |
| close ( ): void {raises-exception, use after open} | 1.16 |
| claim (timeout: int32): void {raises-exception, use after open} | 1.16 |
| release ( ): void {raises-exception, use after open, claim} | 1.16 |
| checkHealth (level: int32): void {raises-exception, use after open, enable} | 1.16 |
| clearInput ( ): void { } | Not supported |
| clearInputProperties ( ): void { } | Not supported |
| Me Methods (UML operations)(continued) |  |
| Common |  |
| Name | Version |
| clearOutput ( ): void { } | Not supported |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, enable} | 1.16 |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} | 1.16 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.16 |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.16 |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, enable} | 1.16 |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.16 |
| Specific |  |
| Name | Version |
| startRecording (FileName: string, OverWrite: boolean, RecordingTime:int32): void {raises-exception, use after open, claim, enable} | 1.16 |
| stopRecording ( ): Void {raises-exception, use after open, claim, enable} | 1.16 |

| Events (UML interfaces) |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Mutability | Version |
| upos::events::DataEvent |  |  | 1.16 |
| Status: | int32 | {read-only} |  |
| upos::events::DirectIOEvent |  |  | 1.16 |
| EventNumber: | int32 | {read-only} |  |
| Data: | int32 | {read-write} |  |
| Obj: | object | {read-write} |  |
| upos::events::ErrorEvent |  |  | 1.16 |
| ErrorCode: | int32 | {read-only} |  |
| ErrorCodeExtended: | int32 | {read-only} |  |
| ErrorLocus: | int32 | {read-only} |  |
| ErrorResponse: | int32 | {read-write} |  |
| upos::events::OutputCompleteEvent |  | Not Supported | 1.16 |
| upos::events::StatusUpdateEvent |  |  | 1.16 |
| Status: | int32 | {read-only} |  |

## General Information

The Sound Recorder programmatic name is "SoundRecorder".

### Capabilities

The Sound Recorder has the following capability:

- Save the recorded sound to a file.

### Sound Recorder Class Diagram

The following diagram shows the relationships between the Sound Recorder classes.

Fig. Chap. 41-1 Sound Recorder Class Diagram

### Model

The Sound Recorder follows the general “Device Input Model” for event-driven input:

- "The control will generate a DataEvent when the recording started by the startRecording method ends when the specified time elapses and the recording to the specified file is completed.

- When an application calls the stopRecording method to end recording, DataEvent will not occur."

- If the AutoDisable property is true, then the device automatically disables itself when a DataEvent is enqueued.

- An enqueued DataEvent can be delivered to the application when the DataEventEnabled property is true and other event delivery requirements are met. Just before delivering this event, data is copied into corresponding properties, and further data events are disabled by setting DataEventEnabled to false. This causes subsequent input data to be enqueued while the application processes the current input and associated properties. When the application has finished processing the current input and is ready for more data, it reenables events by setting DataEventEnabled to true.

- An ErrorEvent (or events) is enqueued if an error occurs while gathering or processing input, and is delivered to the application when DataEventEnabled is true and other event delivery requirements are met.

- The DataCount property may be read to obtain the total number of enqueued DataEvents.

- All enqueued input may be deleted by calling clearInput. See the clearInput method description for more details.

- All data properties that are populated as a result of firing a DataEvent or ErrorEvent can be set back to their default values by calling the clearInputProperties method.

- Since audio files are recorded in the area managed by the "hard total" service, the application must also support "hard total" services.

- Device Sharing

The Sound Recorder is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before accessing some properties or calling methods that update the device.

- See the “Summary” table for precise usage prerequisites.

- The image display mode of the graphics control is as follows.

## Properties(UML attributes)

CapChannel Property

Syntax	CapChannel: boolean{read-only, access after open}

Remarks	If true, the application can change the channel.

If false, the application cannot change the channel.

This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See Also	Channel Property

CapSamplingRate Property

Syntax	CapSamplingRate: boolean {read-only, access after open}

Remarks	If true, the application can change the sampling rate.

If false, the application cannot change the sampling rate.

This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See Also	SamplingRate Property.

CapSoundType Property

Syntax	CapSoundType: boolean {read-only, access after open}

Remarks	If true, the application can change the sound file type.

If false, the application cannot change the sound file type.

This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See Also	SoundType Property.

CapRecordingLevel Property

Syntax	CapRecordingLevel: boolean {read-only, access after open}

Remarks	If true, the application can change the recording level.

If false, the application cannot change the recording level.

This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See Also	CapRecordingLevel Property.

CapChannelList Property

Syntax	CapChannelList : string {read only, access after open}

Remarks	Contains the comma-delimited list of channel that is supported by the device.

For example, if the device only supports 1ch and 2ch and 4ch, then this property should be set to "1,2,4".

This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See Also	Channel Property.

CapSamplingRateList Property

Syntax	CapSamplingRateList : string {read only, access after open}

Remarks		Contains the comma-delimited list of sampling rate that are supported by the device.

For example, if the device only supports 44.1KHz and 48KHz and 96KHz, then this property should be set to "44100,48000,96000".

This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See Also	SamplingRate Property.

CapSoundTypeList Property

Syntax	CapSoundTypeList : string{read only, access after open}

Remarks		Contains the comma-delimited list of sound file type that is supported by the device.

For example, if the device only supports WAV and OGG, then this property should be set to "WAV,OGG".

This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See Also	SoundType Property.

Channel Property

Syntax	Channel : string {read-write, access after open, claim}

Remarks	Holds the channel during recording.

Valid values ​​are one of the values ​​listed in the CapChannelList property.

This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

Value	Meaning

E_ILLEGAL	An invalid value was specified.

E_BUSY	Property could not be set because it is recording.

See Also	CapChannel Property、CapChannelList Property

SamplingRate Property

Syntax	SamplingRate : string{read-write, access after open, claim}

Remarks	Holds the sampling rate during recording.

Valid values ​​are one of the values ​​listed in the CapSamplingRateList property.

This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

Value	Meaning

E_ILLEGAL	An invalid value was specified.

E_BUSY	Property could not be set because it is recording.

See Also	CapSamplingRate Property、CapSamplingRateList Property

SoundType Property

Syntax	SoundType : string {read-write, access after open, claim}

Remarks	Holds the audio file format to be recorded.

Valid values ​​are one of the values ​​listed in the CapSoundTypeList property.

This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed. 					For further information, see “Errors" on page Intro-20.

Value	Meaning

E_ILLEGAL	An invalid value was specified.

E_BUSY	Property could not be set because it is recording.

See Also	CapSoundType Property、CapSoundTypeList Property

RecordingLevel Property

Syntax	RecordingLevel : int32 {read-write, access after open, claim}

Remarks	Holds the recording level during recording.

Legal values range from zero through 100.

This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	CapRecordingLevel Property

## Methods(UML operations)

startRecording Method

Syntax	startRecording (fileName : string, overWrite : boolean,		recordingTime : int32):  										     void{raises-exception, use after open, claim, enable}

Parameter	Description

fileName	Specify the file name of the image to be loaded.

overWrite	Specify the behavior when the same name file exists. If it is true it will be overwritten and if false it will return an error.

recordingTime	Specify the time for recording in seconds. If OPOS_FOREVER (-1) is specified, recording will continue until you call the stopRecording method.

Remarks	Recording starts with the settings of the Channel property, SamplingRate property, and RecordingLevel property, and recording starts in the format set by SoundType.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	FileName is too long or contains characters that cannot be used, or 0 is specified for RecordingTime.

E_EXISTS	FileName already exists. (When OverWrite is FALSE)

E_BUSY	It cannot be executed as it is recording.

See Also	Channel Property、SamplingRate Property、SoundType Property、RecordingLevel Property、stopRecording Method

stopRecording Method

Syntax	stopRecording ( ):															      void {raises-exception, use after open, claim, enable}

Remarks	Finish the recording and complete the recording of the audio file.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20 Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	It is not recorded.

See Also	StartRecording Property

## Events(UML interfaces)

ErrorEvent

<<event>> 	upos::events:: ErrorEvent										　ErrorCode 		: int32{read-write}						　ErrorCodeExtended	: int32{read-write} 						　ErrorLocus 		: int32{read-write} 						* pErrorResponse	: int32{read-write}

Attributes  This event contains following attributes.

Attributes	Type	 Description

Error Code	int32	Error Code causing the error event. 			See the list of Error Code.

ErrortCodeExtended	int32	Error Code causing the error event. 			These values are device category    			specific.

ErrorLocus	int32	Location of the error. See values below.

pErrorResponse	int32	Error response, whose default value may 		              be overridden by the application     			(i.e., this attribute is settable). 				See values below.

The ErrorLocus attribute has one of the following values:

Value	Meaning

EL_INPUT 	Error occurred while gathering or Processing event-driven input. No previously buffered input data is available.

EL_INPUT_DATA	Error occurred while gathering or processing

event-driven input, and some previously buffered

data is available.

If ResultCode is E_EXTENDED, ResultCodeExtended is set to one of the following values.

Value	Meaning

ETOT_NOROOM	There is not enough space to create the file.

The application’s error event handler can set the ErrorResponse attribute to one of the following values:

Value	 Meaning

ER_CLEAR	I will try its asynchronous output again.

The error condition is exited.

ER_CONTINUEINPUT

Only valid when the locus is EL_INPUT_DATA.

Acknowledges that a data error has occurred and

directs the Device to continue input processing.

The Device remains in the error state and will deliver

additional DataEvents as directed by the

DataEventEnabled property.

When all input has been delivered and

DataEventEnabled is again set to true, then another

ErrorEvent is delivered with locus EL_INPUT.

This is the default response when the locus is

EL_INPUT_DATA.

Remarks		It notifies you when an error is detected during recording.

Input error events are not delivered until DataEventEnabled is true, so that proper application sequencing occurs.

See Also		Status, Error code, State model

- ＣＨＡＰＴＥＲ 42

Voice Recognition

This Chapter defines the Voice Recognition device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.16 | open |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.16 | open |
| CapPowerReporting: | int32 | {read-only} | 1.16 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.16 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.16 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.16 | open |
| CheckHealthText: | string | {read-only} | 1.16 | open |
| Claimed: | boolean | {read-only} | 1.16 | open |
| DataCount: | int32 | {read-only} | 1.16 | open |
| DataEventEnabled: | boolean | {read-write} | 1.16 | open |
| DeviceEnabled: | boolean | {read-write} | 1.16 | open, claim |
| FreezeEvents: | boolean | {read-write} | 1.16 | open |
| OutputID: | int32 | {read-only} | 1.16 | Not Supported |
| PowerNotify: | int32 | {read-write} | 1.16 | open |
| PowerState: | int32 | {read-only} | 1.16 | open |
| State: | int32 | {read-only} | 1.16 | -- |
| DeviceControlDescription: | string | {read-only} | 1.16 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.16 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.16 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.16 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.16 | open |
| PhysicalDeviceName: | string | {read-only} | 1.16 | open |

| Properties (Continued) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Specific | Type | Mutability | Version | May Use After |  |
| CapLanguage: | boolean | {read-only} | 1.16 | open |  |
| HearingDataPattern: | string | {read-only} | 1.16 | open |  |
| HearingDataWord: | string | {read-only} | 1.16 | open |  |
| HearingDataWordList: | string | {read-only} | 1.16 | open |  |
| HearingResult: | int32 | {read-only} | 1.16 | open |  |
| HearingStatus: | int32 | {read-only} | 1.16 | open |  |
| LanguageList: | string | {read-only} | 1.16 | open |  |
| Me Methods (UML operations) |  |  |  |  |  |
| Common |  |  |  |  |  |
| Name |  |  |  |  | Version |
| open (logicalDeviceName: string): void {raises-exception} |  |  |  |  | 1.16 |
| close ( ): void {raises-exception, use after open} |  |  |  |  | 1.16 |
| claim ( timeout: int32 ): void {raises-exception, use after open} |  |  |  |  | 1.16 |
| release ( ): void {raises-exception, use after open, claim} |  |  |  |  | 1.16 |
| checkHealth ( level: int32 ): void {raises-exception, use after open, enable} |  |  |  |  | 1.16 |
| clearInput ( ): void { } |  |  |  |  | Not supported |
| clearInputProperties ( ): void { } |  |  |  |  | Not supported |
| clearOutput ( ): void { } |  |  |  |  | Not supported |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} |  |  |  |  | 1.16 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| retrieveStatistics ( inout statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| Me Methods (UML operations)(continued) |  |  |  |  |  |
| Common |  |  |  |  |  |
| Name |  |  |  |  | Version |
| updateFirmware ( firmwareFileName: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| updateStatistics ( statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| Specific |  |  |  |  |  |
| Name |  |  |  |  |  |
| startHearingFree (language: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| startHearingSentence (language: string, wordList: string, patternList: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| startHearingWord (language: string, wordList: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| startHearingYesNo (language: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| stopHearing ( ): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |

| Events (UML interfaces) |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Mutability | Version |
| upos::events::DataEvent |  |  | 1.16 |
| Status: | int32 | {read-only} |  |
| upos::events::DirectIOEvent |  |  | 1.16 |
| EventNumber: | int32 | {read-only} |  |
| Data: | int32 | {read-write} |  |
| Obj: | object | {read-write} |  |
| upos::events::ErrorEvent |  |  | 1.16 |
| ErrorCode: | int32 | {read-only} |  |
| ErrorCodeExtended: | int32 | {read-only} |  |
| ErrorLocus: | int32 | {read-only} |  |
| ErrorResponse: | int32 | {read-write} |  |
| upos::events::OutputCompleteEvent |  | Not Supported |  |
| upos::events::StatusUpdateEvent |  |  | 1.16 |
| Status: | int32 | {read-only} |  |

## General Information

The Voice Recognition programmatic name is "VoiceRecognition".

### Capabilities

The Voice Recognition has the following capability:

- Convert spoken words to strings.

### Voice Recognition Class Diagram

The following diagram shows the relationships between the Voice Recognition classes.

Fig. Chap. 42-1 Voice Recognition Class Diagram

### Model

The Voice Recognition follows the general “Device Input Model” for event-driven input:

Control starts voice recognition with the startHearingYesNo method, startHearingSentence method, etc., and generates DataEvent when recognizing voice.

If the AutoDisable property is true, then the device automatically disables itself when a DataEvent is enqueued.

An enqueued DataEvent can be delivered to the application when the DataEventEnabled property is true and other event delivery requirements are met. Just before delivering this event, data is copied into corresponding properties, and further data events are disabled by setting DataEventEnabled to false. This causes subsequent input data to be enqueued while the application processes the current input and associated properties. When the application has finished processing the current input and is ready for more data, it reenables events by setting DataEventEnabled to true.

An ErrorEvent (or events) is enqueued if an error occurs while gathering or processing input, and is delivered to the application when DataEventEnabled is true and other event delivery requirements are met.

The DataCount property may be read to obtain the total number of enqueued DataEvents.

All enqueued input may be deleted by calling clearInput. See the clearInput method description for more details.

All data properties that are populated as a result of firing a DataEvent or ErrorEvent can be set back to their default values by calling the clearInputProperties method.

Types of voice recognition

Voice recognition is mainly a method of specifying word candidates to be recognized and waiting for those words.

There are the following four types of voice recognition.

Yes/No/Cancel recognition

It listens to the sound of words classified as Yes / No / Cancel defined by the device.

For example, the voice ""OK."" is classified as Yes.

The recognized content is set in the HearingDataWord property.

For details, refer to the startHearingYesNo method.

Word recognition

The application specifies a list of words and listens for the voice of that word.

The recognized content is set in the HearingDataWord property.

For details, refer to the startHearingWord method.

Sentence recognition

The application specifies a word and a list of patterns of the sentences using it and awaits the sound of the sentence.

The recognized content is set in the HearingDataWordList property, HearingDataPattern property.

For details, see the startHearingSentence method.

Free recognition

Voice recognition leave to the device is performed without specifying the word to wait.

The recognized content is set in the HearingDataWord property.

For details, see the startHearingFree method.

When recognizing voice, the kind of recognition was stored in the HearingResult property.

- Device Sharing

The Voice Recognition is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before accessing some properties or calling methods that update the device.

- See the “Summary” table for precise usage prerequisites.

## Properties (UML attributes)

CapLanguage Property

Syntax	CapLanguage: boolean {read-only, access after open}

Remarks	If true, the application can change the language. If false, the application cannot change the language.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

HearingDataPattern Property

Syntax	HearingDataPattern: string {read-only, access after open}

Remarks	The pattern ID recognized by the startHearingSentence method is set.

This property is set by the control just before the DataEvent is notified.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	startHearingSentence Method

HearingDataWord Property

Syntax	HearingDataWord: string {read-only, access after open}

Remarks	The content of voice recognition is set.

This property is set as input data of the following method. To know which method it is for, check the HearingResult property.

Methods	Meaning

startHearingYesNo Method

The recognized word is set.

startHearingWord Method

Recognized words are set among the word candidates

specified by the startHearingWord method.

startHearingFree Method

Recognized words and sentences are set.

The alphabet 's uppercase letters, Japanese kanji,

hiragana, katakana, etc., the contents to be set varies

depending on the device.

This property is set by the control just before the DataEvent is notified.

Errors	A UposException may be thrown when this property is accessed.          	      For further information, see “Errors" on page Intro-20.

See Also	HearingResult Property, startHearingYesNo Method,

startHearingWord Method, startHearingFree Method

HearingDataWordList Property

Syntax	HearingDataWordList: string {read-only, access after open}

Remarks	Comma-separated list of word information recognized by the startHearingSentence method.

Each word information consists of the following information and is shown in the following order separated by a colon (":").

Parameter	Description

WordGoupiID	Recognized word group ID

Word	Recognized words. The content defined in the word

group is set.

For example, in the startHearingSentence method, set candidates as follows, Word list: "Item: coffee: tea, number: one: two"

Sentence pattern: "Pattern 01: [product] as [number], Pattern 02: as [goods] please"

When you recognize the word "one coffee."

In the pattern "Pattern 01", "coffee" of the word group "product" and "one" of "number" are recognized.

At that time, it looks like the following.

"Item: coffee, number: one"

This property is set by the control just before the DataEvent is notified.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	startHearingSentence Method

HearingResult Property

Syntax	HearingStatus: int32 {read-only, access after open}

Remarks	A value indicating the voice recognition result is set.

The parameters to be set are as follows.

Value	Meaning

TTS_HRESULT_YESNO_YES

Voice recognition result of Finish running voice

recognition. method. Also, Device got an answer that

is classified as YES. The recognition content is set in

the Finish running voice recognition property.

TTS_HRESULT_YESNO_NO

Voice recognition result of Finish running voice

recognition. method. Also, Device got an answer that

is classified as NO. The recognition content is set in

the HearingDataWord property.

TTS_HRESULT_YESNO_CANCEL

Voice recognition result of startHearingYesNo

method. Also, Device got responses that are classified

as CANCEL. The recognition content is set in the

HearingDataWord property.

TTS_HRESULT_WORD

Recognition result of startHearingWord method.  The recognition content is set in the HearingDataWord property.

TTS_HRESULT_SENTENCE

Recognition result of startHearingSentence method.

The recognition content is set in the

HearingDataWordList property,

HearingDataPattern property.

TTS_HRESULT_FREE

Recognition result of startHearingFree method. The

recognition content is set in the HearingDataWord

property.

This property is set by the control just before the DataEvent is notified.

Errors	A UposException may be thrown when this property is accessed.

For further information, see “Errors" on page Intro-20.

See Also	HearingDataWord Property, HearingDataWordList Property, HearingDataPattern Property, startHearingYesNo Method, startHearingWord Method, startHearingSentence Method, startHearingFree Method

HearingStatus Property

Syntax	HearingStatus: int32 {read-only, access after open}

Remarks	A value indicating the voice recognition status is set.

Value			Meaning

TTS_HSTATUS_NONE		Voice recognition is not running.

TTS_HSTATUS_YESNO	Voice recognition by the

startHearingYesNo method is in progress.

TTS_HSTATUS_WORD	Voice recognition by the startHearingWord

method is in progress.

TTS_HSTATUS_SENTENCE

Voice recognition by the startHearingSentence

method is in progress.

TTS_HSTATUS_FREE		Voice recognition by the startHearingFree

method is in progress.

This property is initialized by the open method. Also, it is set by the control just before the voice recognition state changes.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	startHearingYesNo Method, startHearingWord Method, startHearingSentence Method, startHearingFree Method

LanguageList Property

Syntax	LanguageList: string {read-only, access after open}

Remarks	Contains the comma-delimited list of language that are supported by the device.
The value representing the language is a value consisting of the language and country code defined in RFC 4664.
For example, when the device supports US / English, Japan / Japanese, it will be as follows.
"en-US, ja-JP"

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	startHearingYesNo Method, startHearingWord Method, startHearingSentence Method, startHearingFree Method

## Methods (UML operations)

startHearingFree Method

Syntax	startHearingFree (language: string):

void {raises-exception, use after open, claim, enable}

Parameter	Description

Language	Specify the language to recognize. Specify one of the

values ​​listed in the LanguageList property.

Remarks	Device will start waiting without specifying waiting candidates.

This method is executed asynchronously. You can end voice recognition by calling the stopHearing method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value		Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported language was specified.

E_BUSY	Voice recognition in progress so it cannot be executed.

See Also	LanguageList Property, stopHearing Method

startHearingSentence Method

Syntax	startHearingSentence (language: string, wordList: string, 
patternList: string):

void {raises-exception, use after open, claim, enable}

Parameter	Description

language	Specify the language to recognize. Specify one of the values ​​listed in the LanguageList property.

wordList	Specify word candidates to be waited on in a comma-separated list.

patternList	Specify the sentence pattern information to be waited for in a comma-separated list.

Each word information specified in wordList consists of the following information and is shown in the following order, separated by a colon (":").

Parameter	Description

wordGroupID	ID to identify word list

wordList	A word candidate to be awaited for being separated by a colon (":")

For example, to specify word candidates "one" and "two" for word candidates "coffee" "tea" and word group "number" in the single item group "product", specify as follows.

"Item: coffee: tea, number: one: two"

Each word information specified in patternList consists of the following information, and it is shown in the following order separated by a colon (":").

Parameter	Description

patternID	ID to identify the pattern

pattern	A sentence pattern to wait. To add the word list specified in wordList to the candidate, enclose the word group ID with "[" and "]".
Example: "[word group ID 1]" [word group ID 2] "

For example, in wordList, "Item: coffee: tea, number: one: two" is specified, and a pattern requesting goods and number such as "Two coffee please" and a pattern requesting goods such as "Coffee please" When defining, specify as follows.

"Pattern 01: [Number] [Product] Please, Pattern 02: [Product] please"

Remarks	Start waiting for sentences defined in wordList and patternList.

This method is executed asynchronously. You can end voice recognition by calling the stopHearing method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported

language was specified.

E_BUSY	Voice recognition in progress so it cannot be 		executed.

See Also	LanguageList Property, stopHearing Method

startHearingWord Method

Syntax	startHearingWord (language: string, wordList: string):

void {raises-exception, use after open, claim, enable}

Parameter	Description

language	Specify the language to recognize. Specify one of the values ​​listed in the LanguageList property.

wordList	Specify word candidates to be waited on in a comma-separated list.
Example: "word 1, word 2, word 3"

Remarks	Start waiting for word candidates specified in wordList.

This method is executed asynchronously. Application can end voice recognition by calling the stopHearing method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported

language was specified.

E_BUSY	Voice recognition in progress so it cannot be executed.

See Also	LanguageList Property, stopHearing Method

startHearingYesNo Method

Syntax	startHearingYesNo (language: string):

void {raises-exception, use after open, claim, enable}

Parameter	Description

language	Specify the language to recognize. Specify one of the values ​​listed in the LanguageList property.

Remarks	Waiting for word candidates corresponding to "Yes" "No" "Cancel" defined by the device is started.

This method is executed asynchronously. Application can end voice recognition by calling the stopHearing method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported

language was specified.

E_BUSY	Voice recognition in progress so it cannot be executed.

See Also	LanguageList Property, stopHearing Method

stopHearing Method

Syntax	stopHearing ( ):

void {raises-exception, use after open, claim, enable}

Remarks	Finish running voice recognition.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported

language was specified.

- ＣＨＡＰＴＥＲ 43

Sound Player

This Chapter defines the Sound Player device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.16 | Not Supported |
| CapCompareFirmwareVersion: | boolean | {read-write} | 1.16 | open |
| CapPowerReporting: | int32 | {read-only} | 1.16 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.16 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.16 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.16 | open |
| CheckHealthText: | string | {read-only} | 1.16 | open |
| Claimed: | boolean | {read-only} | 1.16 | open |
| DataCount: | int32 | {read-only} | 1.16 | open |
| DataEventEnabled: | boolean | {read-write} | 1.16 | open |
| DeviceEnabled: | boolean | {read-write} | 1.16 | open, claim |
| FreezeEvents: | boolean | {read-write} | 1.16 | open |
| OutputID: | nt32 | {read-only} | 1.16 | open |
| PowerNotify: | nt32 | {read-write} | 1.16 | open |
| PowerState: | nt32 | {read-only} | 1.16 | open |
| State: | nt32 | {read-only} | 1.16 | -- |
| DeviceControlDescription: | string | {read-only} | 1.16 | - |
| DeviceControlVersion: | int32 | {read-only} | 1.16 | - |
| DeviceServiceDescription: | string | {read-only} | 1.16 | open |
| DeviceServiceVersion: | Int32 | {read-only} | 1.16 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.16 | open |
| PhysicalDeviceName: | string | {read-only} | 1.16 | open |
| Properties (Continued) |  |  |  |  |
| Specific | Type | Mutability | Version | May Use After |
| CapVolume: | boolean | {read-only} | 1.16 | open |
| CapMultiPlay: | boolean | {read-only} | 1.16 | open |
| CapSoundTypeList: | string | {read-only} | 1.16 | open |
| DeviceSoundList: | string | {read-only} | 1.16 | open |
| Volume: | int32 | {read-write} | 1.16 | open, claim & enable |
| OutputIDList: | string | {read-only} | 1.16 | open, claim & enable |

| Me Methods (UML operations) |  |
| --- | --- |
| Common |  |
| Name | Version |
| open (logicalDeviceName: string): void {raises-exception} | 1.16 |
| close ( ): void {raises-exception, use after open} | 1.16 |
| claim (timeout: int32): void {raises-exception, use after open} | 1.16 |
| release ( ): void {raises-exception, use after open, claim} | 1.16 |
| checkHealth (level: int32): void {raises-exception, use after open, enable} | 1.16 |
| clearInput ( ): void { } | Not supported |
| clearInputProperties ( ): void { } | Not supported |
| clearOutput ( ): void { } | Not supported |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} | 1.16 |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, claim, enable} | 1.16 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | 1.16 |
| retrieveStatistics ( inout statisticsBuffer: string ): void {raises-exception, use after open, claim, enable} | 1.16 |
| Me Methods (UML operations)(continued) |  |
| Common |  |
| Name | Version |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, claim, enable} | 1.16 |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | 1.16 |
| Specific |  |
| Name | Version |
| playSound(fileName: string, loop: boolean): void { raises-exception, use after open, claim, enable} | 1.16 |
| stopSound(outputID:int32): void {raises-exception, use after open, claim, enable} | 1.16 |

| Events (UML interfaces) |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Mutability | Version |
| upos::events::DataEvent |  | Not Supported | 1.16 |
| upos::events::DirectIOEvent |  |  | 1.16 |
| EventNumber: | int32 | {read-only} |  |
| Data: | int32 | {read-write} |  |
| Obj: | object | {read-write} |  |
| upos::events::ErrorEvent |  |  | 1.16 |
| ErrorCode: | int32 | {read-only} |  |
| ErrorCodeExtended: | int32 | {read-only} |  |
| ErrorLocus: | int32 | {read-only} |  |
| ErrorResponse: | int32 | {read-write} |  |
| upos::events::OutputCompleteEvent |  |  | 1.16 |
| OutputID: | int32 | {read-only} |  |
| upos::events::StatusUpdateEvent |  |  | 1.16 |
| Status: | int32 | {read-only} |  |

## General Information

The Sound Player programmatic name is "SoundPlayer".

### Capabilities

The Sound Player has the following capability:

- Play audio file.

### Sound Player Class Diagram

The following diagram shows the relationships between the Sound player classes.

Fig. Chap.43-1 Sound Playter Class Diagram

### Model

The Sound Player follows the general device behavior model for asynchronous output devices:

- The application calls a startSound method to start playing sound. The Device validates the method parameters and produces an error condition immediately if necessary. If the validation is successful, the Device does the following:

- "1. Buffers the request in program memory, for delivery to the Physical Device as soon as the Physical Device can receive and process it.

- 2. Sets the OutputID property to a unique integer identifier for this request.

- 3. Returns as soon as possible."

- When the Device successfully completes a request, an OutputCompleteEvent is enqueued for delivery to the application. A property of this event contains the output ID of the completed request. The application should compare the returned OutputCompleteEvent property OutputID value with the OutputID value set by the asynchronous process method call used to send the data in order to track what data has been successfully sent to the device.

- If an error occurs while processing a request, an ErrorEvent is enqueued which will be delivered to the application after the events already enqueued, including OutputCompleteEvents .I No further asynchronous output will occur until the event has been delivered to the application. If the response is ER_CLEAR, then outstanding asynchronous output is cleared. If the response is ER_RETRY, then output is retried; note that if several outputs were simultaneously in progress at the time that the error was detected, then the Service may need to retry all of these outputs.

- Asynchronous output is always performed on a first-in first-out basis. If the device supports concurrent playback, the request will be executed simultaneously. To check if the device supports simultaneous playback, check the CapMultiPlay property.

- "If the request is terminated before completion, due to reasons such as the application calling the clearOutput method, then no OutputCompleteEvent is delivered.

- Application can also delete the output individually by calling the stopSound method. Also in this case OutputCompleteEvent will not be notified."

- The CapSoundTypeList property lists audio files that the device can play.

- Applications need to support "hard total" services as audio files played with the startSound method must be placed in the area managed by the "hard total" service.

- Device Sharing

The Sound Player is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before accessing some properties or calling methods that update the device.

- See the “Summary” table for precise usage prerequisites.

## Properties(UML attributes)

CapVolume Property

Syntax	CapVolume: boolean {read-only, access after open}

Remarks	If true, the application can change the volume during playback.

If false, the application cannot change the volume during playback.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	Volume Property.

CapMultiPlay Property

Syntax	CapMultiPlay : boolean {read-only, access after open}

Remarks	If true, the application can play sound simultaneously.

If false, the application cannot play sound simultaneously.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	playSound Method.

CapSoundTypeList Property

Syntax	CapSoundTypeList : string {read-only, access after open}

Remarks	Contains the comma-delimited list of file type that is supported by the device.

For example, if the device only supports WAV and OGG, then this property should be set to “WAV,OGG". This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	playSound Method

DeviceSoundList Property

Syntax	DeviceSoundList : string {read-only, access after open}

Remarks	Contains the comma-delimited list of device sound ID that is supported by the device. This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	playSound Method

OutputIDList Property

Syntax	OutputIDList : string {read-only, access after open, claim}

Remarks	Contains the comma-delimited list of OutputID that is output by the playSound method. This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	playSound Method

Volume Property

Syntax	Volume : int32 {read-write, access after open, claim}

Remarks	Holds the volume at playing sound.

Legal values range from zero through 100.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	playSound Method

## Methods (UML operations)

playSound Method

Syntax	playSound (fileName : string, loop : boolean):				
          void{raises-exception, use after open, claim, enable}

Parameter	Description

fileName	Specifies the file name of audio file. Or, Specifies one of the sound ID defined by DeviceSoundList.

loop	When true is specified, loop playback is performed, and if false is specified, loop playback will not be performed.

Remarks	Play audio file specified by fileName or device definition sound.

Audio files must be located in the area managed by "Hard Total" service.

This method will be performed asynchronously. To stop playback, call the stopSound method.

Errors	A UposException may be thrown when this method is invoked. For further information , see “Errors” on page Intro-20.  Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported sound file was specified.

E_NOEXIST	File does not exist.

See Also	CapSoundType Property, DeviceSoundList Property, stopSound Method

stopSound Method

Syntax	StopSound(outputID: int32):							
          void{raises-exception, use after open, claim, enable}

Parameter	Description

outputID	Specify the outputID of the sound to stop.

Remarks	Terminates specified audio playback.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors” on page Intro-20.  Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	The specified sound is not being played.

See Also	OutputID Property, startSound Method

- ＣＨＡＰＴＥＲ 44

Speech Synthesis

This Chapter defines the Speech Synthesis device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.16 | Not Supported |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.16 | open |
| CapPowerReporting: | int32 | {read-only} | 1.16 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.16 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.16 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.16 | open |
| CheckHealthText: | string | {read-only} | 1.16 | open |
| Claimed: | boolean | {read-only} | 1.16 | open |
| DataCount: | int32 | {read-only} | 1.16 | Not Supported |
| DataEventEnabled: | boolean | {read-write} | 1.16 | Not Supported |
| DeviceEnabled: | boolean | {read-write} | 1.16 | open, claim |
| FreezeEvents: | boolean | {read-write} | 1.16 | open |
| OutputID: | int32 | {read-only} | 1.16 | open |
| PowerNotify: | int32 | {read-write} | 1.16 | open |
| PowerState: | int32 | {read-only} | 1.16 | open |
| State: | int32 | {read-only} | 1.16 | -- |
| DeviceControlDescription: | string | {read-only} | 1.16 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.16 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.16 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.16 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.16 | open |
| PhysicalDeviceName: | string | {read-only} | 1.16 | open |

| Properties (Continued) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Specific |  | Type | Mutability | Version | May Use After |  |
| CapLanguage: |  | boolean | {read-only} | 1.16 | open |  |
| CapPitch: |  | boolean | {read-only} | 1.16 | open |  |
| CapSpeed: |  | boolean | {read-only} | 1.16 | open |  |
| CapVoice: |  | boolean | {read-only} | 1.16 | open |  |
| CapVolume: |  | boolean | {read-only} | 1.16 | open |  |
| Language: |  | string | {read-write} | 1.16 | open, claim & enable |  |
| LanguageList: |  | string | {read-only} | 1.16 | open |  |
| OutputIDList: |  | string | {read-only} | 1.16 | open, claim & enable |  |
| Pitch: |  | int32 | {read-write} | 1.16 | open, claim & enable |  |
| Speed: |  | int32 | {read-write} | 1.16 | open, claim & enable |  |
| Voice: |  | string | {read-write} | 1.16 | open, claim & enable |  |
| VoiceList: |  | string | {read-only} | 1.16 | open |  |
| Volume: |  | int32 | {read-write} | 1.16 | open, claim & enable |  |
| Me Methods (UML operations) |  |  |  |  |  |  |
| Common |  |  |  |  |  |  |
| Name |  |  |  |  | Version |  |
| open (logicalDeviceName: string): void {raises-exception} |  |  |  |  | 1.16 |  |
| close ( ): void {raises-exception, use after open} |  |  |  |  | 1.16 |  |
| claim ( timeout: int32): void {raises-exception, use after open} |  |  |  |  | 1.16 |  |
| release ( ): void {raises-exception, use after open, claim} |  |  |  |  | 1.16 |  |
| checkHealth (level: int32): void {raises-exception, use after open, enable} |  |  |  |  | 1.16 |  |
| clearInput ( ): void { } |  |  |  |  | Not supported |  |
| clearInputProperties ( ): void { } |  |  |  |  | Not supported |  |
| Methods (UML operations)(continued) clearOutput ( ): void { } |  |  |  |  | Not supported |  |
| Me |  |  |  |  |  |  |
| Common |  |  |  |  |  |  |
| Name |  |  |  |  | Version |  |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} |  |  |  |  | 1.16 |  |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |
| Specific |  |  |  |  |  |  |
| Name |  |  |  |  |  |  |
| speak (text: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |
| speakImmediate (text: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |
| stopCurrentSpeaking ( ): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |
| stopSpeaking (outputID: int32 ): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |

| Events (UML interfaces) |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Mutability | Version |
| upos::events::DataEvent |  | Not Supported |  |
| upos::events::DirectIOEvent |  |  | 1.16 |
| EventNumber: | int32 | {read-only} |  |
| Data: | int32 | {read-write} |  |
| Obj: | object | {read-write} |  |
| upos::events::ErrorEvent |  |  | 1.16 |
| ErrorCode: | int32 | {read-only} |  |
| ErrorCodeExtended: | int32 | {read-only} |  |
| ErrorLocus: | int32 | {read-only} |  |
| ErrorResponse: | int32 | {read-write} |  |
| upos::events::OutputCompleteEvent |  |  | 1.16 |
| OutputID: | int32 | {read-only} |  |
| upos::events::StatusUpdateEvent |  |  | 1.16 |
| Status: | int32 | {read-only} |  |

## General Information

The Speech Synthesis programmatic name is "SpeechSynthesis".

### Capabilities

The Speech Synthesis has the following capability:

- Convert text to speech and speak.

### Speech Synthesis Class Diagram

The following diagram shows the relationships between the Speech Synthesis classes.

Fig. Chap. 44-1 Speech Synthesis Class Diagram

### Model

The Speech Synthesis follows the general device behavior model for asynchronous output devices:

The application calls a speak method or speakImmediate method to speech.

The speak method acts to start speaking the words specified by text, while the speakImmediate method ended immediately previous speak method, and start speaking the word specified by text asynchronously and immediately.

The device validates the method parameters and produces an error condition immediately if necessary. If the validation is successful, the device does the following:

- Buffers the request in program memory, for delivery to the physical device as soon as the physical device can receive and process it.

- Sets the OutputID property to a unique integer identifier for this request.

- Returns as soon as possible.

When the device successfully completes a request, an OutputCompleteEvent is enqueued for delivery to the application. A property of this event contains the output ID of the completed request. The application should compare the returned OutputCompleteEvent property's OutputID value with the OutputID value set by the asynchronous process method call used to send the data in order to track what data has been successfully sent to the device.

If an error occurs while processing a request, an ErrorEvent is enqueued which will be delivered to the application after the events already enqueued, including OutputCompleteEvent. No further asynchronous output will occur until the event has been delivered to the application. If the response is ER_CLEAR, then outstanding asynchronous output is cleared. If the response is ER_RETRY, then output is retried; note that if several outputs were simultaneously in progress at the time that the error was detected, then the service may need to retry all of these outputs.

Asynchronous output is always performed on a first-in first-out basis.

If the request is terminated before completion, due to reasons such as the application calling the clearOutput method, then no OutputCompleteEvent is delivered.

Application can also delete the output individually by calling the stopCurrentSpeaking, stopSpeaking method. Also in this case OutputCompleteEvent will not be notified.

- Device Sharing

The Speech Synthesis is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before accessing some properties or calling methods that update the device.

- See the “Summary” table for precise usage prerequisites.

## Properties (UML attributes)

CapLanguage Property

Syntax	CapLanguage: boolean {read-only, access after open}

Remarks	If true, the application can change the language. If false, the application cannot change the language.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	Language Property

CapPitch Property

Syntax	CapPitch: boolean {read-only, access after open}

Remarks	If true, the application can change the pitch. If false, the application cannot change the pitch.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	Pitch Property

CapSpeed Property

Syntax	CapSpeed: boolean {read-only, access after open}

Remarks	If true, the application can change the speed. If false, the application cannot change the speed.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	Speed Property

CapVoice Property

Syntax	CapVoice: boolean {read-only, access after open}

Remarks	If true, the application can change the voice. If false, the application cannot change the voice.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	Voice Property

CapVolume Property

Syntax	CapVolume: boolean {read-only, access after open}

Remarks	If true, the application can change the volume. If false, the application cannot change the volume.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	Volume Property

Language Property

Syntax	Language: string {read-write, access after open, claim, enable}

Remarks	Indicates the language to speak. Valid values ​​are one of the values ​​listed in the LanguageList property.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported

language was specified.

See Also	speak Method, speakImmediate Method

LanguageList Property

Syntax	LanguageList: string {read-only, access after open}

Remarks	Contains the comma-delimited list of language that are supported by the device. The value representing the language is a value consisting of the language and country code defined in RFC 4664. For example, when the device supports US / English, Japan / Japanese, it will be as follows.
"en-US, ja-JP"

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	Language Property

OutputIDList Property

Syntax	OutputIDList : string {read-write, access after open, claim, enable}

Remarks	Comma-separated list of OutputID property values ​​of audio being played by Speak method or SpeakImmediate method.

This property is initialized by the open method. It will also be updated as the speech request increases or decreases.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	speak Method, speakImmediate Method

Pitch Property

Syntax	Pitch: int32 {read-write, access after open, claim, enable}

Remarks	Holds the pitch at speech. Legal values range from 50% through 200%.

This property is initialized to 100% by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	speak Method, speakImmediate Method

Speed Property

Syntax	Speed: int32 {read-write, access after open, claim, enable}

Remarks	Holds the speed at speech. Legal values range from 50% through 200%.

This property is initialized to 100% by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	speak Method, speakImmediate Method

Voice Property

Syntax	Voice : string {read-write, access after open, claim, enable }

Remarks	Indicates the voice tone to speak. Valid values ​​are one of the values ​​listed in the VoiceList property.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported

voice was specified.

See Also	speak Method, speakImmediate Method

VoiceList Property

Syntax	VoiceList: string { read-only, access after open }

Remarks	A list of speech able voices are shown in a comma-separated list. For example, when the device supports male and female voice tones, it looks like the following.
"MALE_VOICE, FEMALE_VOICE"
(The content of the value depends on the device)

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	Voice Property

Volume Property

Syntax	Volume : int32 {read-write, access after open, claim, enable}

Remarks	Holds the volume at speech. Legal values range from zero through 100.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	speak Method, speakImmediate Method

## Methods (UML operations)

speak Method

Syntax	speak (text: string):

void {raises-exception, use after open, claim, enable}

Parameter	Description

text	Specify the text to speak.

Remarks	Device will utter the words specified by Text.

The utterance is executed according to the setting contents of Language property, Volume property, Pitch property, Speed ​​property, but by inserting the following tag in the text, it is possible to change the utterance after the tag.

Tag	Description

volume	Specify the volume of the uttered voice. Valid values ​​range from 1 to 100.

pitch	Specify the high or low of the uttered voice. Valid values ​​range from 50 to 200.

speed	Specify the speed of the uttered voice. Valid values ​​range from 50 to 200.

pause	Specify the time to pause in milliseconds.

reset	Delete the effect of volume, pitch, speed.

Tags without reset are specified in the form of "\\ tag = value \\". For example, when specifying Text as follows, "Hello \\ pause = 1000 \ \ \ pitch = 150 \ \ It's nice weather today \ \ reset \\". "Hello" speaks according to the

original setting. Then wait for 1000 milliseconds. "Today" speaks Pitch at 150%. "Nice weather," I will speak according to the original settings.

If the device does not support Volume change etc, that tag will be ignored.

This method is executed asynchronously. To end an utterance halfway, call the stopCurrentSpeaking method or the stopSpeaking method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified. The language set in  the Language property and the language specified by Text do not match.

See Also	Language Property, Volume Property, Pitch Property, Speed Property, stopCurrentSpeaking Method, stopSpeaking Method

speakImmediate Method

Syntax	speakImmediate (text: string):

void {raises-exception, use after open, claim, enable}

Parameter	Description

text	Specify the text to speak.

Remarks	The speak method acts to start speaking the words specified by text, while the speakImmediate method ended immediately previous speak method, and start speaking the word specified by text asynchronously and immediately.

After executing the same processing as the clearOutput method, speak the wording specified by text.

Like this speak method, this method can also change a specific wording by inserting a tag. For details, refer to the description of speak method.

This method is executed asynchronously. To end an utterance halfway, call the stopCurrentSpeaking method or the stopSpeaking method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value          	Meaning

E_ILLEGAL 	An invalid value was specified. The language set in    the Language property and the language specified by Text do not match.

See Also	Language Property, Volume Property, Pitch Property, Speed Property, stopCurrentSpeaking Method, stopSpeaking Method

stopCurrentSpeaking Method

Syntax	stopCurrentSpeaking ( ):

void {raises-exception, use after open, claim, enable}

Remarks	Stops the currently executed utterance.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	Speech is not running.

See Also	speak Method, speakImmediate Method

stopSpeaking Method

Syntax	stopSpeaking (outputID : int32):

void {raises-exception, use after open, claim, enable}

Parameter	Description

outputID	Specify the value of the OutputID property you wish to terminate.

Remarks	Stop and delete the utterance specified in OutputID.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	OutputID Property, speak Method, speakImmediate Method

- ＣＨＡＰＴＥＲ 45

Gesture Control

This Chapter defines the Gesture Control device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.16 | open |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.16 | open |
| CapPowerReporting: | int32 | {read-only} | 1.16 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.16 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.16 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.16 | open |
| CheckHealthText: | string | {read-only} | 1.16 | open |
| Claimed: | boolean | {read-only} | 1.16 | open |
| DataCount: | int32 | {read-only} | 1.16 | open |
| DataEventEnabled: | boolean | {read-write} | 1.16 | open |
| DeviceEnabled: | boolean | {read-write} | 1.16 | open, claim |
| FreezeEvents: | boolean | {read-write} | 1.16 | open |
| OutputID: | int32 | {read-only} | 1.16 | open |
| PowerNotify: | int32 | {read-write} | 1.16 | open |
| PowerState: | int32 | {read-only} | 1.16 | open |
| State: | int32 | {read-only} | 1.16 | -- |
| DeviceControlDescription: | string | {read-only} | 1.16 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.16 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.16 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.16 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.16 | open |
| PhysicalDeviceName: | string | {read-only} | 1.16 | open |

| Properties (Continued) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specific | Type | Mutability | Version | May Use After |
| JointList: | string | {read-only} | 1.16 | open |
| AutoModeList: | string | {read-only} | 1.16 | open |
| AutoMode: | string | {read-write} | 1.16 | open, claim & enable |
| CapMotion: | boolean | {read-only} | 1.16 | open |
| CapPose: | boolean | {read-only} | 1.16 | open |
| CapMotionCreation: | boolean | {read-only} | 1.16 | open |
| CapPoseCreation: | boolean | {read-only} | 1.16 | open |
| MotionList: | string | {read-only} | 1.16 | open |
| PoseList: | string | {read-only} | 1.16 | open |
| PoseCreationMode: | boolean | {read-write} | 1.16 | open, claim & enable |

| Methods (UML operations) |  |
| --- | --- |
| Common |  |
| Name | Version |
| open (logicalDeviceName: string): void {raises-exception} | 1.16 |
| close ( ): void {raises-exception, use after open} | 1.16 |
| claim (timeout: int32): void {raises-exception, use after open} | 1.16 |
| release ( ): void {raises-exception, use after open, claim} | 1.16 |
| checkHealth (level: int32): void {raises-exception, use after open, enable} | 1.16 |
| clearInput ( ): void { } | Not supported |
| clearInputProperties ( ): void { } | Not supported |
| clearOutput ( ): void { } | Not supported |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, enable} | 1.16 |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} | 1.16 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.16 |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.16 |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, enable} | 1.16 |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.16 |
| Me Methods (UML operations)(continued) |  |
| Specific |  |
| Name | Version |
| setPotision (positionList: string, time: int32 , absolute: boolean): void { raises-exception, use after open, claim, enable } | 1.16 |
| setSpeed (speedList: string, time: int32): void { raises-exception, use after open, claim, enable } | 1.16 |
| getPosition (jointID: string, position: int32 by reference): void { raises-exception, use after open, claim, enable } | 1.16 |
| startMotion (fileName: string): void { raises-exception, use after open, claim, enable } | 1.16 |
| createMotion (fileName: string, poseList: string): void { raises-exception, use after open, claim, enable } | 1.16 |
| startPose (fileName: string): void { raises-exception, use after open, claim, enable } | 1.16 |
| createPose (fileName: string, time: int32): void { raises-exception, use after open, claim, enable } | 1.16 |
| stopControl (outputID: int32): void { raises-exception, use after open, claim, enable } | 1.16 |

| Events (UML interfaces) |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Mutability | Version |
| upos::events::DataEvent |  | Not Supported |  |
| upos::events::DirectIOEvent |  |  | 1.16 |
| EventNumber: | int32 | {read-only} |  |
| Data: | int32 | {read-write} |  |
| Obj: | object | {read-write} |  |
| upos::events::ErrorEvent |  |  | 1.16 |
| ErrorCode: | int32 | {read-only} |  |
| ErrorCodeExtended: | int32 | {read-only} |  |
| ErrorLocus: | int32 | {read-only} |  |
| ErrorResponse: | int32 | {read-write} |  |
| upos::events::OutputCompleteEvent |  |  | 1.16 |
| OutputID: | int32 | {read-only} |  |
| upos::events::StatusUpdateEvent |  |  | 1.16 |
| Status: | int32 | {read-only} |  |

General Information

The Gesture Control programmatic name is "GestureControl".

- Capabilities

The Gesture Control has the following capability:

- It controls the operation of various joints.

- The operation is automatically controlled by interlocking various joints and other devices.

- Register and play the defined pose and motion.

### Gesture Control Class Diagram

The following diagram shows the relationships between the Gesture Control classes.

Fig. Chap. 45-1 Gesture Control Class Diagram

- Model

The Gesture Control follows the general device behavior model for asynchronous output devices:

- The application calls a setPosition, setSpeed, startPose, startMotion method to start output. The Device validates the method parameters and produces an error condition immediately if necessary. If the validation is successful, the Device does the following:

- Buffers the request in program memory, for delivery to the Physical Device as soon as the Physical Device can receive and process it.

- Sets the OutputID property to a unique integer identifier for this request.

- Returns as soon as possible.

- When the Device successfully completes a request, an OutputCompleteEvent is enqueued for delivery to the application. A property of this event contains the output ID of the completed request. The application should compare the returned OutputCompleteEvent property OutputID value with the OutputID value set by the asynchronous process method call used to send the data in order to track what data has been successfully sent to the device.

- If an error occurs while processing a request, an ErrorEvent is enqueued which will be delivered to the application after the events already enqueued, including OutputCompleteEvent. No further asynchronous output will occur until the event has been delivered to the application. If the response is ER_CLEAR, then outstanding asynchronous output is cleared. If the response is ER_RETRY, then output is retried; note that if several outputs were simultaneously in progress at the time that the error was detected, then the Service may need to retry all of these outputs.

- Asynchronous output is always performed on a first-in first-out basis.

- If the request is terminated before completion, due to reasons such as the application calling the clearOutput method, then no OutputCompleteEvent is delivered.

- Application can also delete the output individually by calling the stopControl method. Also in this case OutputCompleteEvent will not be notified.

- Automatic control

Automatic control of a joint means to automatically control a joint on the device side, such as tracking according to the movement of a person's face, in cooperation with a camera or the like connected to the device.

The automatic control function is device dependent. For possible automatic control, it is enabled by confirming with the AutoModeList property and setting a value in the AutoMode property.

- Pose / Motion

Pose refers to setting the position of one or more defined joints.

For example, it is an action that lifts a hand.

To execute a pose, specify the pose file name in the startPose method or the pose name defined in the device.

Create the pose file with the createPose method described later. Pose defined on the device will check the PoseList property.

To execute motion, specify the motion file name or the motion name defined in the device in the startMotion method.

Motion files are created by the createMotion method to be described later. Motion defined on the device will check the MotionList property.

To create a pose file, first set the PoseCreationMode property to TRUE and enable the pose registration function. When pose registration function is enabled, each joint is set to the default position. At this time, if the automatic control mode is enabled, the automatic control mode is temporarily invalidated.

Application can then create a pose file by setting the value you want to define as a pose with the setPosition method and calling the createPose method.

A motion file can be created by specifying the pose defined by the created pause file or device and calling the createMotion method.

Since the created pause and motion files are recorded in the area managed by the "hard total" service, the application must also support "hard total" service.

- Device Sharing

The Gesture Control is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before accessing some properties or calling methods that update the device.

- See the “Summary” table for precise usage prerequisites.

Properties (UML attributes)

JointList Property

Syntax	JointList: string {read-only, access after open}

Remarks	Comma-separated list of joint information supported by the device.

Each piece of joint information consists of the following information and is 	shown in the following order, separated by a colon (":").

Parameter   	Description

JointID	Indicates a unique ID in the service that identifies the

joint. Position range availability

If 0, the joint does not have the position range,

1 holds the position range. For example, the arm

joint has a range of rotation width, but the wheel

for movement does not have the range of movement

amount.

For example, for a device that supports pitch, roll, and yaw joints and a device	that supports rotation by wheel and joint that can move forward and backward, it is as follows.

"Joint 01 _Pitch: 1, Joint 01 _ Roll: 1, Joint 01 _ Yaw: 1, Wheel_Turn: 0, Wheel_Move: 0"

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

AutoModeList Property

Syntax	AutoModeList: string {read-only, access after open}

Remarks	Comma-separated list of joint automatic control IDs supported by the device.

For example, in conjunction with the camera, if the mode of tracking the face of a person by moving only the joint of Joint 01 and the mode of tracking by moving all joints are supported as follows.

"FaceTrack_Joint 01, FaceTrack_ALL"

(Content and order are dependent on the device.)

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	AutoMode Property.

AutoMode Property

Syntax	AutoMode: string {read-write, access after open, claim, enable}

Remarks	Indicates automatic control mode ID. Valid values ​​are the empty string "" or one of the AutoModeList properties listed.

If you set one of the properties described in the AutoModeList property for this property, the automatic control mode will be enabled in the set mode.

Setting the empty character "" disables the automatic control mode.

This property is initialized to the empty string "" by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	AutoModeList Property

CapMotion Property

Syntax	CapMotion: boolean {read-only, access after open}

Remarks	If true, the device supports pose function.

If false, the device does not support pose function.

If this property is false, change of PoseCreationMode property, startPose method, createPose method is not available.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	startMotion Method, createMotion Method.

CapPose Property

Syntax	CapPose: boolean {read-only, access after open}

Remarks	If true, the device supports pose function.

If false, the device does not support pose function.

If this property is FALSE, change of PoseCreationMode property, startPose method, createPose method is not available.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	PoseCreationMode Property, startPose Method, createPose Method.

CapMotionCreation Property

Syntax	CapMotionCreation: boolean {read-only, access after open}

Remarks	If true, the device supports motion registration function.

If false, the device does not support motion registration function.

If this property is FALSE, the createMotion method is not available.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	createMotion Method.

CapPoseCreation Property

Syntax	CapPoseCreation: boolean {read-only, access after open}

Remarks	If true, the device supports pose registration function.

If false, the device does not support pose registration function.

If this property is FALSE, you cannot use the createPose method to change the PoseCreationMode property.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	PoseCreationMode Property, createPose Method.

MotionList Property

Syntax	MotionList: string {read-only, access after open}

Remarks	Comma-separated list of motion IDs defined on the device.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

PoseList Property

Syntax	PoseList: string {read-only, access after open}

Remarks	A comma-separated list of pause IDs defined on the device.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

PoseCreationMode Property

Syntax	PoseCreationMode: boolean {read-write, access after open, claim, enable}

Remarks	If true, pose registration function is enabled.

If false, pose registration function is invalid.

When this property is set to true, pause registration function is enabled. When false is set, the pause registration function is disabled.

This property is initialized to false when you first enable the device after calling the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	CapPose Property, CapPoseCreation Property.

Methods (UML operations)

setPosition Method

Syntax	setPosition (positionList: string, time: int32, absolute: boolean): 
          void {raises-exception, use after open, claim, enable}

Parameter	Description

positionList	Specify the position information in a comma-separated list.

time	Specify the time to control completion in seconds. If this value is too small, it will be changed to an appropriate value depending on the service.

absolute	If true, the specified position indicates the absolute value. If false, the specified position indicates relative value.

Each position information specified in the positionList consists of the following information and is shown in the following order separated by a colon (":").

Parameter	Description

jointID	Specify the joint ID. Specify one of the values ​​listed in the JointList property. However, it must be an ID whose position range exists or not.

position	Specify the position to be set. Valid values ​​range from -100 to 100.

100 represents the limit value in the positive direction of the target joint, and -100 represents the limit value in the negative direction.

If Absolute is a relative value (false) and the value specified here exceeds the limit value, it will be changed to an appropriate value by the service

For example, to move Yow of Joint 01 up to the limit of the positive direction and move Pitch of Joint 02 to the middle, specify as follows.

"Joint01_Yaw:100,Joint01:Pitch:0"

Remarks	The joint position is set with the contents specified in PositionList and control is started so that control is completed at the time specified by Time.

Joints that can be specified with this method are only those that have a position range.

Check the JointList property for the presence or absence of the position range.

This method is executed asynchronously. To terminate the operation prematurely, call the stopControl method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	JointList Property, stopControl Method.

setSpeed Method

Syntax	setSpeed (speedList: string, time: int32): 
          void {raises-exception, use after open, claim, enable}

Parameter	Description

speedList		Specify speed information in a comma-separated list.

time		Specify the time to control in seconds. If you specify FOREVER(-1), it will continue to operate until you call the stopControl method.

Each speed information specified in the SpeedList consists of the following

information, and it is shown in the following order separated by a colon (":").

Parameter	Description

jointID	Specify the joint ID. Specify one of the values ​​listed in the JointList property.

speed	Specify the speed to set. Valid values ​​range from -100 to 100.100 represents the maximum speed in the positive direction of the target joint, and -100 represents the maximum speed in the negative direction.

For example, to move Wheel's X at the maximum speed in the positive direction and Y at the Wheel at half the speed in the negative direction, specify as follows.

"Wheel_X: 100, Wheel_Y: -50"

Remarks	It sets the speed of the joint with the contents specified by speedList and performs control for the time specified by time.

This method is executed asynchronously. To terminate the operation prematurely, call the stopControl method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	JointList Property, stopControl Method.

getPosition Method

Syntax	getPosition (jointID: string, position: int32 by reference): 
void {raises-exception, use after open, claim, enable}

Parameter	Description

jointID		Specify the joint ID. Specify one of the values ​​listed in the JointList property. However, it must be an ID whose position range exists or not.

position		The position of the joint specified by JointID is stored.

Remarks	It acquires the position specified by jointID and stores it in position.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	JointList Property.

startMotion Method

Syntax	startMotion (fileName: string): 
          void {raises-exception, use after open, claim, enable}

Parameter	Description

fileName	Specify the name of the motion file to start. Or one of the motion ID lists listed in the MotionList property.

Remarks	Start motion defined by fileName or motion defined by the device.

Motion files need to be placed in the area managed by "hard total" service.

This method is executed asynchronously. To terminate motion control prematurely, call the stopControl method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

E_NOEXIST	File does not exist.

See Also	MotionList Property.

createMotion Method

Syntax	createMotion (fileName: string, poseList: string): 
          void {raises-exception, use after open, claim, enable}

Parameter	Description

fileName	Specify the motion file name to record motion.

poseList	Specify the comma-separated list of pause information to be registered.

Remarks	Specify the registered pose and record it in the motion file.

The place where the motion file is recorded is the area managed by the "hard total" service.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	fileName is too long or contains unusable characters.

E_EXISTS	fileName already exists.

startPose Method

Syntax	startPose (fileName: string): 
          void {raises-exception, use after open, claim, enable}

Parameter	Description

fileName	Specify the name of the pause file to start. Or one of the pose ID lists listed in the PoseList property.

Remarks	Begin pause defined by the pause file or device specified by fileName.

Pose files must be placed in the area managed by "hard total" service.

This method is executed asynchronously. To terminate pause control prematurely, call the stopControl method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

E_NOEXISTS	File does not exist.

See Also	PoseList Property, stopControl Method.

createPose Method

Syntax	createPose (fileName: string, time: int32): 
          void {raises-exception, use after open, claim, enable}

Parameter	Description

fileName	Specify the pose file name to record the pose.

time	Specify the time to reach the pose position.

Remarks	Record the position of each joint in the pause file.

Before calling this method, you need to set the PoseCreationMode property to TRUE and enable pause registration mode.

The place where the pause file is recorded is the area managed by the "hard total" service.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	FileName is too long or contains unusable characters. Or PoseCreationMode is FALSE.

E_EXISTS	FileName already exists.

See Also	PoseCreationMode Property.

stopControl Method

Syntax	stopControl (outputID: int32): 
          void {raises-exception, use after open, claim, enable}

Parameter	Description

outputID	Specify the value of the OutputID property you wish to terminate.

Remarks	Stop the control specified for outputID.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	setPosition Method, setSpeed Method, startPose Method, startMotion Method.

- ＣＨＡＰＴＥＲ 46

Device Monitor

This Chapter defines the Device Monitor device category.

Summary

| Pro Properties (UML attributes) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Common |  | Type |  | Mutability |  | Version |  | May Use After |  |
| AutoDisable: |  | boolean |  | {read-write} |  | 1.16 |  | open |  |
| CapCompareFirmwareVersion: |  | boolean |  | {read-only} |  | 1.16 |  | open |  |
| CapPowerReporting: |  | int32 |  | {read-only} |  | 1.16 |  | open |  |
| CapStatisticsReporting: |  | boolean |  | {read-only} |  | 1.16 |  | open |  |
| CapUpdateFirmware: |  | boolean |  | {read-only} |  | 1.16 |  | open |  |
| CapUpdateStatistics: |  | boolean |  | {read-only} |  | 1.16 |  | open |  |
| CheckHealthText: |  | string |  | {read-only} |  | 1.16 |  | open |  |
| Claimed: |  | boolean |  | {read-only} |  | 1.16 |  | open |  |
| DataCount: |  | int32 |  | {read-only} |  | 1.16 |  | open |  |
| DataEventEnabled: |  | boolean |  | {read-write} |  | 1.16 |  | open |  |
| DeviceEnabled: |  | boolean |  | {read-write} |  | 1.16 |  | open, claim |  |
| FreezeEvents: |  | boolean |  | {read-write} |  | 1.16 |  | open |  |
| OutputID: |  | int32 |  | {read-only} |  | 1.16 |  | Not Supported |  |
| PowerNotify: |  | int32 |  | {read-write} |  | 1.16 |  | open |  |
| PowerState: |  | int32 |  | {read-only} |  | 1.16 |  | open |  |
| State: |  | int32 |  | {read-only} |  | 1.16 |  | -- |  |
| DeviceControlDescription: |  | string |  | {read-only} |  | 1.16 |  | -- |  |
| DeviceControlVersion: |  | int32 |  | {read-only} |  | 1.16 |  | -- |  |
| DeviceServiceDescription: |  | string |  | {read-only} |  | 1.16 |  | open |  |
| DeviceServiceVersion: |  | int32 |  | {read-only} |  | 1.16 |  | open |  |
| PhysicalDeviceDescription: |  | string |  | {read-only} |  | 1.16 |  | open |  |
| PhysicalDeviceName: |  | string |  | {read-only} |  | 1.16 |  | open |  |

| Properties (Continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Specific |  |  | Type |  | Mutability |  | Version |  | May Use After |  |  |  |  |  |
| DeviceList: |  | string |  | {read-only} |  |  | 1.16 |  | open |  |  |  |  |  |
| MonitoringDeviceList: |  | string |  | {read-only |  |  | 1.16 |  | open, claim & enable |  |  |  |  |  |
| DeviceData: |  | string |  | {read-only} |  |  | 1.16 |  | open, claim & enable |  |  |  |  |  |
| Me Methods (UML operations) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Common |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name |  |  |  |  |  |  |  |  |  |  |  | Version |  |  |
| open (logicalDeviceName: string): void {raises-exception} |  |  |  |  |  |  |  |  |  |  |  | 1.16 |  |  |
| close ( ): void {raises-exception, use after open} |  |  |  |  |  |  |  |  |  |  |  | 1.16 |  |  |
| claim (timeout: int32): void {raises-exception, use after open} |  |  |  |  |  |  |  |  |  |  |  | 1.16 |  |  |
| release ( ): void {raises-exception, use after open, claim} |  |  |  |  |  |  |  |  |  |  |  | 1.16 |  |  |
| checkHealth (level: int32): void {raises-exception, use after open, enable} |  |  |  |  |  |  |  |  |  |  |  | 1.16 |  |  |
| clearInput ( ): void { } |  |  |  |  |  |  |  |  |  |  |  | Not supported |  |  |
| clearInputProperties ( ): void { } |  |  |  |  |  |  |  |  |  |  |  | Not supported |  |  |
| clearOutput ( ): void { } |  |  |  |  |  |  |  |  |  |  |  | Not supported |  |  |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, enable} |  |  |  |  |  |  |  |  |  |  |  | 1.16 |  |  |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} |  |  |  |  |  |  |  |  |  |  |  | 1.16 |  |  |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} |  |  |  |  |  |  |  |  |  |  |  | 1.16 |  |  |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, enable} |  |  |  |  |  |  |  |  |  |  |  | 1.16 |  |  |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, enable} |  |  |  |  |  |  |  |  |  |  |  | 1.16 |  |  |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} |  |  |  |  |  |  |  |  |  |  |  | 1.16 |  |  |
| Specific |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| addMonitoringDevice (deviceID: string, monitoringMode: int32, boundary: int32, subBoundary: int32, intervalTime: int32 ): void {raises-exception, use after open, claim, enable} |  |  |  |  |  |  |  |  |  |  |  | 1.16 |  |  |
| deleteMonitoringDevice (deviceID: string ): void {raises-exception, use after open, claim, enable} |  |  |  |  |  |  |  |  |  |  |  | 1.16 |  |  |
| clearMonitoringDevice ( ): void {raises-exception, use after open, claim, enable} |  |  |  |  |  |  |  |  |  |  |  | 1.16 |  |  |
| getDeviceValue (deviceID: string, inout value: int32) void {raises-exception, use after open} |  |  |  |  |  |  |  |  |  |  |  | 1.16 |  |  |
| Events (UML interfaces) |  |  |  |  |  |  |  |  |  |  |  |
| Name |  | Type |  | Mutability |  |  |  |  |  | Version |  |
| upos::events::DataEvent |  |  |  |  |  |  |  |  |  |  | 1.16 |
| Status: |  | int32 |  | {read-only} |  |  |  |  |  |  |  |
| upos::events::DirectIOEvent |  |  |  |  |  |  |  |  |  |  | 1.16 |
| EventNumber: |  | int32 |  | {read-only} |  |  |  |  |  |  |  |
| Data: |  | int32 |  | {read-write} |  |  |  |  |  |  |  |
| Obj: |  | object |  | {read-write} |  |  |  |  |  |  |  |
| upos::events::ErrorEvent |  |  |  |  |  |  |  |  |  |  | 1.16 |
| ErrorCode: |  | int32 |  | {read-only} |  |  |  |  |  |  |  |
| ErrorCodeExtended: |  | int32 |  | {read-only} |  |  |  |  |  |  |  |
| ErrorLocus: |  | int32 |  | {read-only} |  |  |  |  |  |  |  |
| ErrorResponse: |  | int32 |  | {read-write} |  |  |  |  |  |  |  |
| upos::events::OutputCompleteEvent |  |  |  | Not Supported |  |  |  |  |  |  |  |
| upos::events::StatusUpdateEvent |  |  |  |  |  |  |  |  |  |  | 1.16 |
| Status: |  | int32 |  | {read-only} |  |  |  |  |  |  |  |

General Information

The Device Monitor programmatic name is "DeviceMonitor".

- Capabilities

The Device Monitor Device has the following capability:

- Get values ​​measured by various devices.

- Notify the application of changes in values ​​measured by various devices.

- Device Monitor Class Diagram

The following diagram shows the relationships between the Device Monitor classes.

Fig. Chap. 46-1 Device Monitor Class Diagram

### Model

The Device Monitor follows the general “Device Input Model” for event-driven input:

- The Device Monitor supports monitoring of values ​​measured by multiple devices connected to the device. A device that can be monitored and its type / value unit is listed in the DeviceList property.

- Device Monitor receives a change in the value measured by the device set as the monitoring target, and generates a DataEvent when it matches the specified condition.

- To add a device to be monitored, specify the monitoring mode with the addMonitoringDevice method and add it. For details on monitoring mode, see the description of addMonitoringDevice method.

- If the AutoDisable property is true, the device will automatically disable itself when a DataEvent is enqueued.

- An enqueued DataEvent can be delivered to the application when the DataEventEnabled property is true and other event delivery requirements are met. Just before delivering this event, data is copied into corresponding properties, and further data events are disabled by setting DataEventEnabled to false. This causes subsequent input data to be enqueued while the application processes the current input and associated properties. When the application has finished processing the current input and is ready for more data, it reenables events by setting DataEventEnabled to true.

- An ErrorEvent (or events) is enqueued if an error occurs while gathering or processing input, and is delivered to the application when DataEventEnabled is true and other event delivery requirements are met.

- The DataCount property can be read to obtain the total number of enqueued DataEvents.

- All enqueued input may be deleted by calling ClearInput. See the ClearInput method description for more details.

- All data properties that are populated as a result of firing a DataEvent or ErrorEvent can be set back to their default values by calling the clearInputProperties method.

- The notified data is stored in the DeviceData property.

- In the device control, the measured value of the device is managed with an integer value of int32 type, but some devices handle decimal values. In that case, you can calculate the actual value by dividing the measured value by the factor for each device that can be acquired with the DeviceList property.

- Device Sharing

The Device Monitor is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before the device begins reading input, or before calling methods that manipulate the device.

See the “Summary” table for precise usage prerequisites.

- Properties (UML attributes)

DeviceList Property

Syntax	DeviceList: string {read-only, access after open}

Remarks	Contains the comma-delimited list of device information that are supported by the device.

Each object information consists of the following information and is shown in the following order, separated by a colon (":").

Parameter	Description

DeviceID	Indicates a unique ID in the service that identifies the　device.

Type	Indicates the device type. For example, if it is a touch sensor it is expressed as "TouchSensor" and so on. However, this value depends on the service.

Unit	Indicates the unit of value held by various devices. For example, it is expressed as "on / off" for a touch sensor, "rad / s" for a gyroscope. However, this value depends on the service.

Coefficient	Indicates the coefficient for calculating the actual measured value held by various devices. The DeviceData property and the measured value of the device that can be obtained with the GetDeviceValue method are expressed as integers, but by dividing this value by the coefficient it is the actual value.  Example: Device value = 365, coefficient = 10, actual value = 36.5					     For example, if one device supports one touch sensor and one gyroscope, it will be as follows.       "Touch 01: Touch Sensor: ON/OFF: 1, GyroX: Gyroscope: rad/s: 100000, GyroY: Gyroscope: rad/s: 100000, GyroZ: Gyroscope: rad/s: 100000"

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	DeviceData Property, addMonitoringDevice Method,

getDeviceValue Method

MonitoringDeviceList Property

Syntax	MonitoringDeviceList: string {read-only, access after open, claim}

Remarks	Contains the comma-delimited list of monitoring information on registered devices that are supported by the device.

Each monitoring information consists of the following information and is shown in the following order, separated by a colon (":").

Parameter	Description

DeviceID	Registered devices ID.

Monitoring mode	Registered monitoring mode.

Boundary	Registered boundary value. This value is set to 0 when the monitoring mode does not require a boundary value.

Sub boundary		Registered sub boundary value. This value is set to 0 	when the monitoring mode does not require a sub 	boundary value.

Interval		Registered interval. (millisecond)

For example, if you set monitoring targets as follows,

[Monitor target 1]

Device ID = Device 01, monitoring mode = SNS_MM_UPDATE,

boundary line = 0, sub boundary line = 0, interval time = 0

[Monitor target 2]

Device ID = Device 02, monitoring mode = SNS_MM_STRADDLED, boundary

line = 365, sub boundary line = 0, interval time = 500

The values ​​shown are as follows.

"Device 01: 0: 0: 0: 0, Device 02: 1: 365: 0: 500"

This property is initialized by the open method. It is also updated by calling

addMonitoringDevice method, deleteMonitoringDevice method,

clearMonitoringDevice method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	addMonitoringDevice Method, deleteMonitoringDevice Method, clearMonitoringDevice Method

DeviceData Property

Syntax	DeviceData: string {read-only, access after open, claim}

Remarks	Measurement information of the device that matches the condition registered by addMonitoringDevice method is set.

Each measurement information consists of the following information and is shown in the following order, separated by a colon (":").

Parameter	Description

DeviceID	The target device ID.

Measured value		Measurement value of the device. The measured value is represented by an integer type. To convert it to an actual value, divide the measured value by the 	coefficient acquired by the DeviceList property.

For example,"Device01:365"

Its value is set prior to a DataEvent being delivered  to the application.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

Methods (UML operations)

addMonitoringDevice Method

Syntax	addMonitoringDevice (deviceID: string, monitoringMode:int32, boundary:int32, subBoundary:int32, intervalTime:int32)		         : void{raises-exception, use after open, claim, enable}

Parameter	Description

deviceID	The deviceID of the monitored device. Valid values ​​		are one of the device ID lists listed in the DeviceList 		property.

monitoringMode	Specify the monitoring mode for monitoring.

boundary	Specify the boundary value to be monitored.

subBoundary	Specify the sub boundary value to be monitored. 			This value must be less than Boundary. 	intervalTime	Specify the interval in milliseconds between the 			occurrence of the event and the start of the next 			monitoring. 								The monitoring modes specified for MonitoringMode 			are as follows.

Value	Description

DM_MMODE_UPDATE									Every time the measured value of the target device is 			updated, an event is notified. When set to this mode, 			the values of the argument boundary and 				subBoundary are ignored.

DM_MMODE_STRADDLED

When the measured value of the target device 				crosses the value of the argument boundary, 				it notifies the event. In addition, when the measured			value matches the value of boundary, it notifies the			event even when it changes from the matched state. 			When set to this mode, the value of the argument              		SubBoundary is ignored.		DM_MMODE_HIGH     										When the measured value of the target device				becomes equal to or larger than the value of the			argument Boundary, it notifies the event. 				Even if the measured value is updated and it was 			again equal to or greater than the value of 				boundary, we will notify the event each time. 				When set to this mode, the value of the 				argument subBoundary is ignored.

DM_MMODE_LOW										         	Notifies the event when the measured value of the 				         		target device becomes less than or equal to the value 				         	of the argument boundary. Even when the measured 				         	value is updated and it was again less than the value 				         	of boundary, we will notify the event each time. 				         		When set to this mode, the value of the argument 				         		subBoundary is ignored.

DM_MMODE_WITHIN																				         		It notifies the event while the measured value of 				         		the target device is within the range specified by 				         		the argument boundary and subBoundary.					         		Even if the measured value 							        	 is updated and its value is 				     		         		within the range again, the event is notified each 				         		time.

DM_MMODE_OUTSIDE																			         		It notifies the event while the measured value of 				         		the target device is outside the range specified 				         		by the argument boundary and subBoundary. 					         	Even if the measured value is updated and its value 				         	was out of range again, we will notify the event        		  		each time.

DM_MMODE_POLLING																			         		It notifies the measured value of the target device at 				         	the interval specified by intervalTime. 					         		When set to this mode, the values ​​of the argument 				         		boundary and subBoundary are ignored.

Remarks	Add the device specified by deviceID to the monitoring target.
The monitoring mode is specified for monitoringMode, but there are monitoring modes not supported by some devices. In that case, E_ILLEGAL is raised as the UPOS exception.

Devices added by this method will be added to the list of MonitoringDeviceList properties. If a device to be monitored is specified, it will be changed to a new condition. To exclude the added device from the monitoring target, call deleteMonitoringDevice method or clearMonitoringDevice method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.  Some possible values of the exception’s ErrorCode property are:

Value                	Description

E_ILLEGAL	An invalid value was specified, or unsupported 						operation with the Device

See Also	DeviceList Property, MonitoringDeviceList Property, deleteMonitoringDevice Method, clearMonitoringDevice Method

deleteMonitoringDevice Method

Syntax	deleteMonitoringDevice (deviceID: string):						void{raises-exception, use after open, claim, enable}  Parameter		Description

deviceID		Specify the device ID of the device to be excluded from 				monitoring targets.

Remarks	Exclude the device specified by deviceID from monitoring targets.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.
Some possible values of the exception’s ErrorCode property are:

Value		Description

E_ILLEGAL	An invalid value was specified, or unsupported operation

with the Device.

See Also	AddMonitoringDevice Method

clearMonitoringDevice Method

Syntax	cleareMonitoringDevice ( ):						void{raises-exception, use after open, claim, enable}

Remarks	Exclude all devices to be monitored.

Errors	A UposException may be thrown when this method is invoked.

For further information, see “Errors" on page Intro-20.

See Also	addMonitoringDevice method

getDeviceValue method

Syntax		getDeviceValue (deviceID: string, inout value: int32) 				                : void{raises-exception, use after open}

Parameter	Description

deviceID		Specify the device ID of the device from which the 				measurement value is to be acquired. Specify one of the 				device ID lists listed in the DeviceList property.

value　		Measured value obtained from the device.

Remarks		Get the measured value of the device specified by deviceID. The retrieved value is stored in pValue.

Errors		A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.
Some possible values of the exception’s ErrorCode property are:

Valu　　　　　　Description

E_ILLEGAL	An invalid value was specified, or unsupported operation with the Device.

See Also		DeviceList Property

- ＣＨＡＰＴＥＲ 47

Graphic Display

This Chapter defines the Graphic Display device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.16 | open |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.16 | open |
| CapPowerReporting: | int32 | {read-only} | 1.16 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.16 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.16 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.16 | open |
| CheckHealthText: | string | {read-only} | 1.16 | open |
| Claimed: | boolean | {read-only} | 1.16 | open |
| DataCount: | int32 | {read-only} | 1.16 | open |
| DataEventEnabled: | boolean | {read-write} | 1.16 | open |
| DeviceEnabled: | boolean | {read-write} | 1.16 | open, claim |
| FreezeEvents: | boolean | {read-write} | 1.16 | open |
| OutputID: | int32 | {read-only} | 1.16 | open |
| PowerNotify: | int32 | {read-write} | 1.16 | open |
| PowerState: | int32 | {read-only} | 1.16 | open |
| State: | int32 | {read-only} | 1.16 | -- |
| DeviceControlDescription: | string | {read-only} | 1.16 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.16 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.16 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.16 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.16 | open |
| PhysicalDeviceName: | string | {read-only} | 1.16 | open |

| Properties (Continued) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specific | Type | Mutability | Version | May Use After |
| CapVolume: | boolean | {read-only} | 1.16 | open |
| CapBrightness: | boolean | {read-only} | 1.16 | open |
| Volume: | int32 | {read-write} | 1.16 | open, claim & enable |
| Brightness: | int32 | {read-write} | 1.16 | open, claim & enable |
| DisplayMode: | int32 | {read-write} | 1.16 | open, claim & enable |
| CapImageTypeList: | string | {read-only} | 1.16 | open |
| CapVideoTypeList: | string | {read-only} | 1.16 | open |
| CapBack: | boolean | {read-only} | 1.16 | open |
| CapForward: | boolean | {read-only} | 1.16 | open |
| LoadStatus: | int32 | {read-only} | 1.16 | open |
| URL: | string | {read-only} | 1.16 | open |

| Methods (UML operations) |  |
| --- | --- |
| Common |  |
| Name | Version |
| open (logicalDeviceName: string): void {raises-exception} | 1.16 |
| close ( ): void {raises-exception, use after open} | 1.16 |
| claim (timeout: int32): void {raises-exception, use after open} | 1.16 |
| release ( ): void {raises-exception, use after open, claim} | 1.16 |
| checkHealth (level: int32): void {raises-exception, use after open, enable} | 1.16 |
| clearInput ( ): void { } | Not supported |
| clearInputProperties ( ): void { } | Not supported |
| clearOutput ( ): void { } | Not supported |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, enable} | 1.16 |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} | 1.16 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.16 |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.16 |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, enable} | 1.16 |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, enable} | 1.16 |
| Specific |  |
| Name | Version |
| loadImage (fileName: string): void {raises-exception, use after open, claim, enable} | 1.16 |
| playVideo (fileName: string, loop: boolean): void { raises-exception, use after open, claim, enable} | 1.16 |
| stopVideo ( ): void {raises-exception, use after open, claim, enable} | 1.16 |
| loadURL (uRL: string ): void {raises-exception, use after open, claim, enable} | 1.16 |
| goBack ( ): void {raises-exception, use after open, claim, enable} | 1.16 |
| goForward ( ): void {raises-exception, use after open, claim, enable} | 1.16 |
| updatePage ( ): void {raises-exception, use after open, claim, enable} | 1.16 |
| cancelLoading ( ): void {raises-exception, use after open, claim, enable} | 1.16 |

| Events (UML interfaces) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | Type |  | Mutability |  |  | Version |
| upos::events::DataEvent |  |  |  |  |  |  | 1.16 |
| Status: |  | int32 |  | {read-only} |  |  |  |
| upos::events::DirectIOEvent |  |  |  |  |  |  | 1.16 |
| EventNumber: |  | int32 |  | {read-only} |  |  |  |
| Data: |  | int32 |  | {read-write} |  |  |  |
| Obj: |  | object |  | {read-write} |  |  |  |
| upos::events::ErrorEvent |  |  |  |  |  |  | 1.16 |
| ErrorCode: |  | int32 |  | {read-only} |  |  |  |
| ErrorCodeExtended: |  | int32 |  | {read-only} |  |  |  |
| ErrorLocus: |  | int32 |  | {read-only} |  |  |  |
| ErrorResponse |  | int32 |  | {read-write} |  |  |  |
| upos::events::OutputCompleteEvent |  |  |  |  |  |  | 1.16 |
| OutputID: |  | int32 |  | {read-only} |  |  |  |
| upos::events::StatusUpdateEvent |  |  |  |  |  |  | 1.16 |
| Status: |  | int32 |  | {read-only} |  |  |  |

General Information

The Graphic Display programmatic name is “GraphicDisplay”.

### Capabilities

The Graphic Display has the following capability:

Displays the specified image.

Play the specified movie.

Display the specified web page.

Notify the application of changes in the load status of the web page.

- Graphics Display Class Diagram

The following diagram shows the relationships between the Graphic Display classes.

Fig. Chap. 47-1 Graphic Display Class Diagram

### Model

The following display modes exist in the graphics control, and the model differs depending on the display mode:

- Image display mode

- Movie display mode.

- Web display mode.

The application can change the display mode by changing the value of the DisplayMode property.

- Image Display Mode

The image display mode of the graphics control is as follows.

The application calls the loadImage method to display the image.

The CapImageTypeList property lists image files that the device can display.

Applications need to support “hard total” services as image files displaying with loadImage method must be placed in the area managed by the “hard total” service.

### Movie Display Mode

The movie display mode of Graphic Display follows the general device behavior model for asynchronous output devices:

The application calls a playVideo method to start playing video. The Device 	validates the method parameters an error condition immediately if necessary. 	If the validation is successful, the Device does the following:

1. Buffers the request in program memory, for delivery to the Physical
  	Device as soon as the Physical Device can receive and process it.

2. Sets the OutputID property to a unique integer identifier for this request.

3. Returns as soon as possible.

When the Device successfully completes a request, an OutputCompleteEvent is enqueued for delivery to the application.

A property of this event contains the output ID of the completed request.

The application should compare the returned OutputCompleteEvent property OutputID value with the OutputID value set by the asynchronous process method call used to send the data in order to track what data has been successfully sent to the device.

If an error occurs while processing a request, an ErrorEvent is enqueued which will be delivered to the application after the events already enqueued, including OutputCompleteEvents. No further asynchronous output will occur until the event has been delivered to the application. If the response is ER_CLEAR, then outstanding asynchronous output is cleared.

If the response is ER_RETRY, then output is retried; note that if several outputs were simultaneously in progress at the time that the error was detected, then	the Service may need to retry all of these outputs.

Asynchronous output is always performed on a first-in first-out basis. If the device supports concurrent playback, the request will be executed simultaneously. To check if the device supports simultaneous playback, check the CapMultiPlay property.

If the request is terminated before completion, due to reasons such as the application calling the clearOutput method, then no OutputCompleteEvent is delivered. You can also delete the output individually by calling the stopVideo method. Also in this case OutputCompleteEvent will not be notified.

The CapVideoTypeList property lists video files that the device can play.

Applications need to support "hard total" services as video files played with the playVideo method must be placed in the area managed by the "hard total" service.

### Web Display Mode

The web display mode of the Graphics Display follows the general “Device Input Model” for event-driven input:

When input is received from the Graphics Display, a DataEvent is enqueued.

If the AutoDisable property is true, then the device automatically disables 	itself when a DataEvent is enqueued.

An enqueued DataEvent can be delivered to the application when the DataEventEnabled property is true and other event delivery requirements are met. Just before delivering this event, data is copied into corresponding properties, and further data events are disabled by setting DataEventEnabled to false. 　　　　　This causes subsequent input data to be enqueued while the application processes the current input and associated properties. When the application has finished processing the current input and is ready for more data, it reenables events by setting DataEventEnabled to true.

An ErrorEvent (or events) is enqueued if an error occurs while gathering or processing input, and is delivered to the application when DataEventEnabled is true and other event delivery requirements are met.

The DataCount property may be read to obtain the total number of enqueued DataEvents.

All enqueued input may be deleted by calling clearInput. See the clearInput method description for more details.

All data properties that are populated as a result of firing a DataEvent or ErrorEvent can be set back to their default values by calling the clearInputProperties method.

The load state of the web page is stored in the LoadStatus property, and the URL is stored in the URL property.

- Device Sharing

The web browser is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before accessing some properties or calling methods that update the device.

See the “Summary” table for precise usage prerequisites.

## Properties (UML attributes)

CapVolume Property

Syntax	CapVolume: boolean {read-only, access after open}

Remarks	If true, the application can change the volume of video.

If false, the application cannot change the volume of video.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	Volume Property.

CapBrightness Property

Syntax	CapBrightness: boolean {read-only, access after open}

Remarks	If true, the application can change the screen brightness.

If false, the application cannot change the screen brightness.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	Brightness Property.

Volume Property

Syntax	Volume: int32 {read-write, access after open, claim, enable}

Remarks	Holds the volume at playing video.

Legal values range from zero through 100.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	CapVolume Property, playVideo Method.

Brightness Property

Syntax	Brightness: int32 {read-write, access after open, claim, enable}

Remarks	Holds the brightness of screen.

Legal values range from zero through 100.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	CapBrightness Property.

DisplayMode Property

Syntax	DisplayMode: int32 {read-write, access after open, claim, enable}

Remarks	Holds the display mode.

Value	Meaning

GDISP_DMODE_HIDDEN

Hide the screen.

GDISP_DMODE_IMAGE_FIT

It is a mode to display images.　The displayed image is enlarged / reduced to the size that maintains the aspect and just enters the screen.

GDISP_DMODE_IMAGE_FILL

It is a mode to display images.

The displayed image is scaled to the size that

maintains the aspect and covers the entire screen.

GDISP_DMODE_IMAGE_CENTER

It is a mode to display images.

The displayed image is displayed in the center of the screen without changing the size.

GDISP_DMODE_VIDEO_NORMAL

It is a mode to display movies. The displayed movie

will be displayed in the center of the screen without resizing it.

GDISP_DMODE_VIDEO_FULL

It is a mode to display movies.

The displayed video will be displayed in full screen.

GDISP_DMODE_WEB

Display the web screen.

If application hide other modes and screens while displaying images, movies, or web, all displayed contents will be cleared. The movie will be stopped while the movie is playing.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value		       Meaning

E_ILLEGAL	An invalid value was specified.

See Also	CapCaptureColorSpaceList Property, VideoCaptureMode Property, readFrame Method.

CapImageTypeList Property

Syntax	CapImageTypeList: string {read-only, access after open}

Remarks	Contains the comma-delimited list of image file type that are support by the device. For example, if the device only supports BMP and JPEG, then this property should be set to “BMP,JPEG”

*Notation contents may be different depending on the device.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	loadImage Method.

CapVideoTypeList Property

Syntax	CapVideoTypeList: string {read-only, access after open}

Remarks	Contains the comma-delimited list of video file type that are supported by the device. For example, if the device only supports AVI_IYUV and AVI_MJPG, then this property should be set to “AVI_IYUV,AVI_MJPG”.

*Notation contents may be different depending on the device.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	playVideo Method.

CapBack Property

Syntax	CapBack: boolean {read-only, access after open}

Remarks	If true, the previous page exists in the browsing history. Application can return to the previous page with goBack method.

If false, there is no previous page in the browsing history.

This property is initialized to false by the open method. Also, as the web page loading state changes, it is set by the control.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	goBack Method.

CapForward Property

Syntax	CapForward: boolean {read-only, access after open}

Remarks	If true, the next page exists in the browsing history. Application can go to the next page with the goForward method.

If false, there is no next page in the browsing history.

This property is initialized to false by the open method. Also, as the web page loading state changes, it is set by the control.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	goForward Method.

LoadStatus Property

Syntax	LoadStatus: int32 {read-only, access after open}

Remarks	Holds loading state of web page.

The parameters to be set are as follows.

Value			Meaning

GDISP_LSTATUS_START		Start loading the web page.

GDISP_LSTATUS_FINISH		It have finished loading the web page.

GDISP_LSTATUS_CANCEL 	It have canceled loading the web page.

Its value is set prior to a DataEvent being delivered to the application.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

URL Property

Syntax	URL: string {read-only, access after open}

Remarks	When the LoadStatus property is GDISP_LSTATUS_START, the URL of the Web page that starts loading is set.

When the LoadStatus property is GDISP_LSTATUS_FINISH, the URL of the loaded Web page is set.

When the LoadStatus property is GDISP_STATUS_CALCEL, the URL of the canceled Web page is set.

Its value is set prior to a DataEvent being delivered to the application.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	loadStatus Method.

## Methods (UML operations)

loadImage Method

Syntax	loadImage (fileName: string): 
          void {raises-exception, use after open, claim, enable}

Parameter	Description

fileName	Specify the file name of the image to be loaded.

Remarks	Load the specified image.

This method fails if the value of the DisplayMode Property is not set to GDISP_DMODE_IMAGE_FIT, GDISP_DMODE_IMAGE_FILL, or GDISP_DMODE_IMAGE_CENTER.

Image files must be located in the area managed by "Hard Total" service.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	 Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported image file was specified.

E_NOEXIST	File does not exist.

See Also	DisplayMode Property.

playVideo Method

Syntax	playVideo (fileName: string, loop: boolean): 
          void {raises-exception, use after open, claim, enable}

Parameter	Description

fileName		Specify the file name of the video to be played.

loop		If true, loop playback is performed, and if false, loop playback is not performed.

Remarks	Play the specified video.

If the value of the DisplayMode property is not set to GDISP_DMODE_VIDEO_NORMAL, GDISP_DMODE_VIDEO_FULL, this method will fail.

This method is executed asynchronously. To stop video playback in the middle, call the stopVideo method.

Video files must be located in the area managed by "Hard Total" service.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported video file was specified.

E_NOEXIST	File does not exist.

See Also	DisplayMode Property.

stopVideo Method

Syntax	stopVideo ( ): 
void {raises-exception, use after open, claim, enable}

Remarks	Stop the video being played.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	The movie is not playing.

See Also	startVideo Method.

loadURL Method

Syntax	loadURL (uRL: string): 
          void {raises-exception, use after open, claim, enable}

Parameter	Description

uRL	Specify the uRL of the web page to load.

Remarks	Load the web page with the specified uRL.

This method is executed asynchronously. The load status is reported by DataEvent or ErrorEvent.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

goBack Method

Syntax	goBack ( ): 
       void {raises-exception, use after open, claim, enable}

Remarks	It returns to the previous page of browsing history.

This method is executed asynchronously. The load status is reported by DataEvent or ErrorEvent.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	There is no previous page in the browsing history.

See Also	CapBack Property.

goForward Method

Syntax	goForward ( ):
          void {raises-exception, use after open, claim, enable}

Remarks	Go to the next page of browsing history.

This method is executed asynchronously. The load status is reported by DataEvent or ErrorEvent.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	There is no next page in the browsing history.

See Also	CapForward Property.

updatePage Method

Syntax	updatePage ( ): 
          void {raises-exception, use after open, claim, enable}

Remarks	Reload the current web page.

This method is executed asynchronously. The load status is reported by DataEvent or ErrorEvent.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	Web page loading.

cancelLoading Method

Syntax	cancelLoading ( ): 
          void {raises-exception, use after open, claim, enable}

Remarks	Cancel loading web page.

This method is executed asynchronously. The load status is reported by DataEvent or ErrorEvent.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	It is not loading.

- Ａｐｐｅｎｄｉｘ K

Relationship to other OMG  specification and activities

## Robotics Domain Task Force

- Activities in Robotics Domain Task Force

The OMG Robotics Domain Task Force (Robotics DTF) fosters the integration of robotics systems from modular components through the adoption of OMG standards. It recommends the adoption and extends OMG technologies that apply to the specific domain of robotics systems where no current baseline specifications exist, such as MDA for Robotics. The object technology is not solely limited to software but is extended to real objects. It also collaborates with other organizations for standardization, such as the one for home information appliances, and makes an open effort to increase interoperability in the field of robotics.

(https://www.omg.org/robotics/)

### RoIS Specification

Robotic Interaction Service Framework [RoIS] defines several functional components for robotic interaction services.

Definitions related to locations of entities in robotic services will be described with Robotic Localization Service[RLS]. Definitions of status of components in services will be described in conjunction with Robotic Technology Component [RTC], Finite State Machine Component for RTC [FSM4RTC] and Unified Component Model for Distributed Real-Time and Embedded Systems [UCM].

RoIS specification seeks that specify a RoIS framework, on top of which   various service robot applications are developed.

- Scope of RoIS specification

They are summarized in the following items.

- Interface between service application and Human Robot Interaction (HRI) engine

- Interface to obtain information from HRI Engine according to the timing of the service application’s needs (Query)

- Interface to receive information from HRI Engine triggered by real time events (Event notification / subscription / cancellation)

- Interface for instructions to control HRI Engine functions (Command)

- Definition of common messages for all HRI Engines

- Robot Servcie Ontology [RoSO] RFP

A new RFP of Robot Service Ontology[RoSO] currently being discussed in Robotics DTF are based on the concept of RoIS.

RoSO is aiming to define the specification (ontology) that clarifies the concept of a common vocabulary and / or a robot service in order to describe a service provided by a robot or exchange a description of a service provided by a service robot

Below is an example of HRI main component examples from this point of view.

Table K-1 – (From RoIS 1.2) Basic HRI Components

| HRI Component Name | Description |
| --- | --- |
| system information | Provides the information of the system such as status of the system and position of the physical unit. |
| person detection | Detects number of people |
| person localization | Detects position of people |
| person identification | Identifies ID (name) of people |
| face detection | Detects number of human faces |
| face localization | Detects position of human faces |
| sound detection | Detects number of sound sources |
| sound localization | Detects position of sound sources |
| speech recognition | Recognizes person’s speech |
| gesture recognition | Recognizes person’s gesture |
| speech synthesis | Generates robot speech |
| reaction | Performs specified reaction |
| navigation | Moves to specified target location |
| follow | Follows a specified target object |
| move | Moves to specified distance or curve |

## Interoperability between UPOS RCSD and RoIs

### Rleationsihp between UPOS RCSD and RoIS

OMG's Robotics standard provides a lower level control layer to manage Robot Device with finer granularity and higher accuracy to accommodate a wide range of industry applications.

On the other hand, the UPOS RCSD specification focuses on the functioning of robotic equipment within the retail store environment. In the UPOS RCSD specification robots are treated as peripheral equipment of the latest POS system. Therefore, the UPOS RCSD specification focuses on the definition of the interface between the POS and the robotic device.

RoIS is already existing as OMG standard and it defined a component frame service that was intended for robotic communication services with people.

Therefore, ROIS developed a general robot service framework, which is different from UPOS RCSD, but it is possible to describe the function of UPOS RCSD.

To confirm the compatibility and interoperability of the RCSD functions of RoIS and UPOS, both DTFs created and confirmed the function mapping table.

For this purpose, we use the general RoIS HRI component defined in the RoIS 1.2 specification.

The two teams continue to collaborate between the part of their separate RFP’s and standards that will be established.

For that purpose, it is very necessary to understand the common vocabulary of the robot service and the needs of the ontology.

If each team’s specification satisfies the above mapping table, it is confirmed that the standard can be maintained independently.

In addition, the figure below shows a typical scenario where RCSD and RoIS work independently or in conjunction.

- Document History

Version History

| Ver | Date | Sections | Description of Change |
| --- | --- | --- | --- |
| 1.0 | 2019-2-18 |  | Initial Version – additions and updates to UPOS v1.15 |
| 1.1 | 2019-7-04 |  | Issues and Upates were included & corrected verion description |
| 1.2 | 2020-2-15 |  | Issues,Suggestion,Updates ware included & corrected version |

- Glossary

| Term | Definition |
| --- | --- |
| EVRW | Electronic Value Reader Writer |
| CAT | Credit Authorization Terminal |