UnifiedPOS Retail Peripheral Architecture

Version 1.16.1 RCSD

International Standard

For Implementation of Point Of Service Peripherals

OMG Document Number: dtc/20-12-02

Original submission date: February 18th, 2019

Standard document URL: https://www.omg.org/spec/UPOS/

This proposal adds to and extends the UPOS 1.16 standard.

Copyright © 2017, Object Management Group, Inc.

USE OF SPECIFICATION - TERMS, CONDITIONS & NOTICES

The material in this document details an Object Management Group specification in accordance with the terms, conditions and notices set forth below. This document does not represent a commitment to implement any portion of this specification in any company's products. The information contained in this document is subject to change without notice.

LICENSES

The companies listed above have granted to the Object Management Group, Inc. (OMG) a nonexclusive, royalty-free, paid up, worldwide license to copy and distribute this document and to modify this document and distribute copies of the modified version. Each of the copyright holders listed above has agreed that no person shall be deemed to have infringed the copyright in the included material of any such copyright holder by reason of having used the specification set forth herein or having conformed any computer software to the specification.

Subject to all of the terms and conditions below, the owners of the copyright in this specification hereby grant you a fully-paid up, non-exclusive, nontransferable, perpetual, worldwide license (without the right to sublicense), to use this specification to create and distribute software and special purpose specifications that are based upon this specification, and to use, copy, and distribute this specification as provided under the Copyright Act; provided that: (1) both the copyright notice identified above and this permission notice appear on any copies of this specification; (2) the use of the specifications is for informational purposes and will not be copied or posted on any network computer or broadcast in any media and will not be otherwise resold or transferred for commercial purposes; and (3) no modifications are made to this specification. This limited permission automatically terminates without notice if you breach any of these terms or conditions. Upon termination, you will destroy immediately any copies of the specifications in your possession or control.

PATENTS

The attention of adopters is directed to the possibility that compliance with or adoption of OMG specifications may require use of an invention covered by patent rights. OMG shall not be responsible for identifying patents for which a license may be required by any OMG specification, or for conducting legal inquiries into the legal validity or scope of those patents that are brought to its attention. OMG specifications are prospective and advisory only. Prospective users are responsible for protecting themselves against liability for infringement of patents.

IPR Mode

This specification is issued under the RAND Mode based on the OMG IPR Policy.

OMG IPR Policy   https://www.omg.org/cgi-bin/doc.cgi?ipr

GENERAL USE RESTRICTIONS

Any unauthorized use of this specification may violate copyright laws, trademark laws, and communications regulations and statutes. This document contains information which is protected by copyright. All Rights Reserved. No part of this work covered by copyright herein may be reproduced or used in any form or by any means--graphic, electronic, or mechanical, including photocopying, recording, taping, or information storage and retrieval systems--without permission of the copyright owner.

DISCLAIMER OF WARRANTY

WHILE THIS PUBLICATION IS BELIEVED TO BE ACCURATE, IT IS PROVIDED "AS IS" AND MAY CONTAIN ERRORS OR MISPRINTS. THE OBJECT MANAGEMENT GROUP AND THE COMPANIES LISTED ABOVE MAKE NO WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, WITH REGARD TO THIS PUBLICATION, INCLUDING BUT NOT LIMITED TO ANY WARRANTY OF TITLE OR OWNERSHIP, IMPLIED WARRANTY OF MERCHANTABILITY OR WARRANTY OF FITNESS FOR A PARTICULAR PURPOSE OR USE.  IN NO EVENT SHALL THE OBJECT MANAGEMENT GROUP OR ANY OF THE COMPANIES LISTED ABOVE BE LIABLE FOR ERRORS CONTAINED HEREIN OR FOR DIRECT, INDIRECT, INCIDENTAL, SPECIAL, CONSEQUENTIAL, RELIANCE OR COVER DAMAGES, INCLUDING LOSS OF PROFITS, REVENUE, DATA OR USE, INCURRED BY ANY USER OR ANY THIRD PARTY IN CONNECTION WITH THE FURNISHING, PERFORMANCE, OR USE OF THIS MATERIAL, EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

The entire risk as to the quality and performance of software developed using this specification is borne by you. This disclaimer of warranty constitutes an essential part of the license granted to you to use this specification.

RESTRICTED RIGHTS LEGEND

Use, duplication or disclosure by the U.S. Government  is subject to the restrictions set forth in subparagraph (c) (1) (ii) of The Rights in Technical Data and Computer Software Clause at DFARS 252.227-7013 or in subparagraph (c)(1) and (2) of the Commercial Computer Software - Restricted Rights clauses at 48 C.F.R. 52.227-19 or as specified in 48 C.F.R. 227-7202-2 of the DoD F.A.R. Supplement and its successors, or as specified in 48 C.F.R. 12.212 of the Federal Acquisition Regulations and its successors, as applicable. The specification copyright owners are as indicated above and may be contacted through the Object Management Group, 109 Highland Avenue, Needham, MA 02494, U.S.A.

TRADEMARKS

IMM®, MDA®, Model Driven Architecture®, UML®, UML Cube logo®, OMG Logo®, CORBA® and XMI® are registered trademarks of the Object Management Group, Inc., and Object Management Group™, OMG™ , Unified Modeling Language™, Model Driven Architecture Logo™, Model Driven Architecture Diagram™, CORBA logos™, XMI Logo™, CWM™, CWM Logo™, IIOP™ , MOF™ , OMG Interface Definition Language (IDL)™ , and OMG SysML™ are trademarks of the Object Management Group. All other products or company names mentioned are used for identification purposes only, and may be trademarks of their respective owners.

COMPLIANCE

The copyright holders listed above acknowledge that the Object Management Group (acting itself or through its designees) is and shall at all times be the sole entity that may authorize developers, suppliers and sellers of computer software to use certification marks, trademarks or other special designations to indicate compliance with these materials.

Software developed under the terms of this license may claim compliance or conformance with this specification if and only if the software compliance is of a nature fully matching the applicable compliance points as stated in the specification. Software developed only partially matching the applicable compliance points may claim only that the software was based on this specification, but may not claim compliance or conformance with this specification. In the event that testing suites are implemented or approved by Object Management Group, Inc., software developed using this specification may claim compliance or conformance with the specification only if the software satisfactorily completes the testing suites.

OMG’s Issue Reporting Procedure

All OMG specifications are subject to continuous review and improvement. As part of this process we encourage readers to report any ambiguities, inconsistencies, or inaccuracies they may find by completing the Issue Reporting Form listed on the main web page http://www.omg.org, under Documents, Report a Bug/Issue (http://www.omg.org/report_issue.)

Document Submitter

VINX Corp.

Document Publishing Supporters

OPOS-J

SorimachiGiken Co. Ltd.

Microsoft Japan Ltd.

SEIKO EPSON Corp.

Toshiba TEC Corp.

Star Micronics Corp.

Fujitsu Frontec Corp.

NCR Corporation

Sharp Corporation

Omron Social Solutions Corp.

NEC Platforms Corp.

Transaction Media Networks Inc.

TABLE OF CONTENTS

TABLE OF CONTENTS	5

Preface	14

Updated Items in Release 1.16.1	17

Chapter 21	37

Lights	37

Summary	37

General Information	40

Capabilities	40

Device Sharing	40

Lights Class Diagram	41

Lights Sequence Diagram	42

Properties(UML attributes)	44

CapAlarm Property	44

CapBlink Property	44

CapColor Property	44

CapPattern Property                         Added in Release 1.16	45

MaxLights Property	45

Methods (UML operations)	46

switchOff Method	46

switchOffPattern Method	46

switchOn Method                               Updated in Release1.12	47

switchOnMultiple Method          	  Added in Release 1.16	48

switchOnPattern Method           	  Added in Release 1.16	49

Events (UML interfaces)	50

DirectIOEvent	50

StatusUpdateEvent	50

Chapter 29	51

POS Power	51

Summary	51

General Information	55

Capabilities	55

Device Sharing	55

Model	56

POSPower Sequence Diagram	58

POSPower Standby Sequence Diagram	59

POSPower State Diagram	60

POSPower PowerState Diagram - Part 1	61

POSPower PowerState Diagram - Part 2	62

POSPower PowerState Diagram - Part 3	63

POSPower State Chart Diagram for Fan and Temperature	64

POSPower Battery State Diagram	65

POSPower Power Transitions State Diagram	66

Properties (UML attributes)	67

BatteryCapacityRemaining Property	67

BatteryCapacityRemainingInSeconds Property	Added in Release 1.16	67

BatteryCriticallyLowThreshold Property	67

BatteryCriticallyLowThresholdInSeconds Property	   Added in Release 1.16	67

BatteryLowThreshold Property	68

BatteryLowThresholdInSeconds Property	 Added in Release 1.16	68

CapBatteryCapacityRemaining Property	70

CapBatteryCapacityRemainingInSeconds Property 	        Added in Release 1.16	70

CapChargeTime Property 	      Added in Release 1.16	70

CapFanAlarm Property	70

CapHeatAlarm Property	71

CapQuickCharge Property	71

CapRestartPOS Property	71

CapShutdownPOS Property	71

CapStandbyPOS Property	72

CapSuspendPOS Property	72

CapUPSChargeState Property	72

CapVariableBatteryCriticallyLowThreshold Property	73

CapVariableBatteryCriticallyLowThresholdInSeconds Property   	 　　　　　　　　　　　　　　　Added in Release 1.16	73

CapVariableBatteryLowThreshold Property	73

CapVariableBatteryLowThresholdInSeconds Property 	     　       Added in Release 1.16	73

ChargeTime Property 	  　　Added in Release 1.16	74

EnforcedShutdownDelayTime Property	74

PowerFailDelayTime Property	74

PowerSource Property	75

QuickChargeMode Property	75

QuickChargeTime Property	75

UPSChargeState Property	76

Methods (UML operations)	77

restartPOS Method	77

shutdownPOS Method	77

standbyPOS Method	78

suspendPOS Method	78

Events (UML Interfaces)	79

DirectIOEvent	79

StatusUpdateEvent	79

Chapter 39	82

Video Capture	82

Summary	82

General Information	87

Capabilities	87

Video Capture Class Diagram	88

Model	89

Modes	89

Device behaviors	89

Photo Mode	89

Video Mode	90

Device Sharing	91

Properties (UML attributes)	92

AutoExposure Property	92

AutoFocus Property	92

AutoGain Property	92

AutoWhiteBalance Property	93

Brightness property	93

CapAssociatedHardTotalsDevice Property	93

CapAutoExposure Property	93

CapAutoFocus Property	94

CapAutoGain Property	94

CapAutoWhiteBalance Property	94

CapBrightness Property	94

CapContrast Property	94

CapExposure Property	95

CapGain Property	95

CapHorizontalFlip Property	95

CapHue Property	95

CapPhoto Property	95

CapPhotoColorSpace Property	96

CapPhotoFrameRate Property	96

CapPhotoResolution Property	96

CapPhotoType Property	96

CapSaturation Property	96

CapStorage Property	97

CapVerticalFlip Property	97

CapVideo Property	97

CapVideoColorSpace Property	98

CapVideoFrameRate Property	98

CapVideoResolution Property	98

CapVideoType Property	98

Contrast Property	99

Exposure Property	99

Gain Property	99

HorizontalFlip Property	100

Hue Property	100

PhotoColorSpace Property	100

PhotoColorSpaceList Property	101

PhotoFrameRate Property	101

PhotoMaxFrameRate Property	101

PhotoResolution Property	102

PhotoResolutionList Property	102

PhotoType Property	102

PhotoTypeList Property	103

RemainingRecordingTimeInSec Property	103

Saturation Property	103

Storage Property	104

VerticalFlip Property	104

VideoCaptureMode Property	105

VideoColorSpace Property	106

VideoColorSpaceList Property	106

VideoFrameRate Property	106

VideoMaxFrameRate Property	107

VideoResolution Property	107

VideoResolutionList Property	107

VideoType Property	107

VideoTypeList Property	108

Note: Video Capture Device Property Value Relationship	108

Methods (UML operations)	109

startVideo Method	109

stopVideo Method	110

takePhoto Method	111

Events (UML interfaces)	112

DirectIOEvent	112

ErrorEvent	113

StatusUpdateEvent	114

Chapter 40	115

Individual Recognition	115

Summary	115

General Information	118

Capabilities	118

Individual Recognition Class Diagram	118

Model	119

Input Model	119

Device Sharing	119

IndividualRecognitionFilter	120

IndividualRecognitionFilter Property Example Format	121

	Basic Items	121

	Face Recognition device example	122

IndividualRecognition Information Property Example Format	124

	Basic Items	124

	Face Recognition Device Example	125

Properties (UML attributes)	126

CapIndividualList Property	126

IndividualIDs Property	126

IndividualRecognitionFilter Property	126

IndividualRecognitionInformation Property	127

Events (UML interfaces)	128

DataEvent	128

DirectIOEvent	128

ErrorEvent	129

StatusUpdateEvent	130

Chapter 41	131

Sound Recorder	131

Summary	131

General Information	135

Capabilities	135

Sound Recorder Class Diagram	135

Model	136

Device Sharing	137

Properties(UML attributes)	138

CapAssociatedHardTotalsDevice Property	138

CapChannel Property	138

CapSamplingRate Property	138

CapSoundType Property	138

CapStorage Property	139

CapRecordingLevel Property	139

Channel Property	139

ChannelList Property	140

RecordingLevel Property	140

RemainingRecordingTimeInSec Property	140

SamplingRate Property	140

SamplingRateList Property	141

SoundData Property	141

SoundType Property	141

SoundTypeList Property	141

Storage Property	142

Methods(UML operations)	143

startRecording Method	143

stopRecording Method	144

Events(UML interfaces)	145

DataEvent	145

DirectIOEvent	145

ErrorEvent	146

StatusUpdateEvent	147

Chapter 42	148

Voice Recognition	148

Summary	148

General Information	152

Capabilities	152

Voice Recognition Class Diagram	152

Model	153

Device Sharing	154

Properties (UML attributes)	155

CapLanguage Property	155

HearingDataPattern Property	155

HearingDataWord Property	155

HearingDataWordList Property	156

HearingResult Property	157

HearingStatus Property	158

LanguageList Property	158

Methods (UML operations)	159

startHearingFree Method	159

startHearingSentence Method	160

startHearingWord Method	161

startHearingYesNo Method	162

stopHearing Method	162

Events (UML interfaces)	163

DataEvent	163

DirectIOEvent	163

ErrorEvent	164

StatusUpdateEvent	166

Chapter 43	167

Sound Player	167

Summary	167

General Information	170

Capabilities	170

Sound Player Class Diagram	170

Model	171

Device Sharing	172

Properties(UML attributes)	173

CapAssociatedHardTotalsDevice Property	173

CapMultiPlay Property	173

CapSoundTypeList Property	173

CapStorage Property	174

CapVolume Property	174

DeviceSoundList Property	174

OutputIDList Property	174

Storage Property	175

Volume Property	175

Methods (UML operations)	175

playSound Method	176

stopSound Method	176

Events (UML interfaces)	177

DirectIOEvent	177

ErrorEvent	178

OutputCompleteEvent	179

StatusUpdateEvent	179

Chapter 44	180

Speech Synthesis	180

Summary	180

General Information	184

Capabilities	184

Speech Synthesis Class Diagram	184

Model	185

Device Sharing	186

Properties (UML attributes)	187

CapLanguage Property	187

CapPitch Property	187

CapSpeed Property	187

CapVoice Property	187

CapVolume Property	188

Language Property	188

LanguageList Property	188

OutputIDList Property	188

Pitch Property	189

Speed Property	189

Voice Property	189

VoiceList Property	190

Volume Property	190

Methods (UML operations)	191

speak Method	191

speakImmediate Method	193

stopCurrentSpeaking Method	193

stopSpeaking Method	194

Events (UML interfaces)	195

DirectIOEvent	195

ErrorEvent	196

OutputCompleteEvent	197

StatusUpdateEvent	197

Chapter 45	198

Gesture Control	198

Summary	198

General Information	202

Capabilities	202

Gesture Control Class Diagram	202

Model	203

Automatic control	203

Pose / Motion	204

Device Sharing	204

Properties (UML attributes)	205

AutoMode Property	205

AutoModeList Property	205

CapAssociatedHardTotalsDevice Property	205

CapMotion Property	206

CapMotionCreation Property	206

CapPose Property	206

CapPoseCreation Property	206

CapStorage Property	207

JointList Property	207

MotionList Property	208

PoseCreationMode Property	208

PoseList Property	208

Storage Property	209

Table of Gesture Control Device Listed Items in Property	210

Methods (UML operations)	211

createMotion Method	211

createPose Method	211

getPosition Method	212

setPosition Method	213

setSpeed Method	214

startMotion Method	215

startPose Method	216

stopControl Method	216

Events (UML interfaces)	217

DirectIOEvent	217

ErrorEvent	218

OutputCompleteEvent	219

StatusUpdateEvent	219

Chapter 46	220

Device Monitor	220

Summary	220

General Information	223

Capabilities	223

Device Monitor Class Diagram	223

Model	224

Device Sharing	224

Properties (UML attributes)	225

DeviceData Property	225

DeviceList Property	226

MonitoringDeviceList Property	227

Methods (UML operations)	228

addMonitoringDevice Method	228

clearMonitoringDevices Method	230

deleteMonitoringDevice Method	230

getDeviceValue method	230

Events (UML interfaces)	231

DataEvent	231

DirectIOEvent	231

ErrorEvent	232

StatusUpdateEvent	233

Chapter 47	234

Graphic Display	234

Summary	234

General Information	238

Capabilities	238

Graphics Display Class Diagram	239

Model	240

Image Display Mode	240

Video Display Mode	240

Web Display Mode	241

Device Sharing	242

Properties (UML attributes)	243

Brightness Property	243

CapAssociatedHardTotalsDevice Property	243

CapBrightness Property	243

CapImageType Property	243

CapStorage Property	244

CapURLBack Property	244

CapURLForward Property	244

CapVideoType Property	245

CapVolume Property	245

DisplayMode Property	246

ImageType Property	247

ImageTypeList Property	247

LoadStatus Property	247

Storage Property	248

URL Property	248

VideoType Property	249

VideoTypeList Property	249

Volume Property	249

Methods (UML operations)	250

cancelURLLoading Method	250

goURLBack Method	250

goURLForward Method	250

loadImage Method	251

loadURL Method	251

playVideo Method	252

stopVideo Method	252

updateURLPage Method	253

Events (UML interfaces)	254

DirectIOEvent	254

ErrorEvent	254

OutputCompleteEvent	256

StatusUpdateEvent	256

Relationship to other OMG  specification and activities	257

Robotics Domain Task Force	257

Activities in Robotics Domain Task Force	257

RoIS Specification	257

Scope of RoIS specification	257

Robot Service Ontology [RoSO] RFP	258

Interoperability between UPOS RCSD and RoIs	259

Rleationsihp between UPOS RCSD and RoIS	259

Document History	261

Version History	261

Glossary	262

Date: Nov.April 20210

Goto Table145

Unified POS RCSD, v1.16.1

FTF Beta 1

This specification adds to and extends the UPOS 1.156 specification.

_______________________________________________

OMG Document Number: dtc/20-04-02

Normative reference: https://www.omg.org/spec/UPOS/

https://www.omg.org/spec/UPOS/20200301/DeviceMonitorClassDiagram.xmi https://www.omg.org/spec/UPOS/20200301/GestureControlClassDiagram.xmi https://www.omg.org/spec/UPOS/20200301/GraphicDisplayClassDiagram.xmi https://www.omg.org/spec/UPOS/20200301/IndividualRecognitionClassDiagram.xmi https://www.omg.org/spec/UPOS/20200301/LightsClassDiagram.xmi https://www.omg.org/spec/UPOS/20200301/POSPowerClassDiagram.xmi https://www.omg.org/spec/UPOS/20200301/SoundPlayerClassDiagram.xmi https://www.omg.org/spec/UPOS/20200301/SoundRecorderClassDiagram.xmi https://www.omg.org/spec/UPOS/20200301/SpeechSynthesisClassDiagram.xmi https://www.omg.org/spec/UPOS/20200301/VideoCaptureClassDiagram.xmi https://www.omg.org/spec/UPOS/20200301/VoiceRecognitionClassDiagram.xmi

_______________________________________________

This OMG document replaces the submission document (retail/2019-06-01, Alpha). It is an OMG Adopted Beta Specification and is currently in the finalization phase. Comments on the content of this document are welcome, and should be directed to issues@omg.org by October 25, 2019.

You may view the pending issues for this specification from the OMG revision issues web page https://issues.omg.org/issues/lists.

The FTF Recommendation and Report for this specification will be published in July 2020. If you are reading this after that date, please download the available specification from the OMG Specifications Catalog.

Preface

OMG

Founded in 1989, the Object Management Group, Inc. (OMG) is an open membership, not-for-profit computer industry standards consortium that produces and maintains computer industry specifications for interoperable, portable, and reusable enterprise applications in distributed, heterogeneous environments. Membership includes Information Technology vendors, end users, government agencies, and academia.

OMG member companies write, adopt, and maintain its specifications following a mature, open process. OMG’s specifications implement the Model Driven Architecture® (MDA®), maximizing ROI through a full-lifecycle approach to enterprise integration that covers multiple operating systems, programming languages, middleware and networking infrastructures, and software development environments. OMG’s specifications include: UML® (Unified Modeling Language™); CORBA® (Common Object Request Broker Architecture); CWM™ (Common Warehouse Metamodel); and industry-specific standards for dozens of vertical markets.　More information on the OMG is available at http://www.omg.org/.

OMG Specifications

As noted, OMG specifications address middleware, modeling and vertical domain frameworks. All OMG Specifications are available from the OMG website at:

http://www.omg.org/spec

Specifications are organized by the following categories:

Business Modeling Specifications

Middleware Specifications

- CORBA/IIOP

- Data Distribution Services

- Specialized CORBA

IDL/Language Mapping Specifications

Modeling and Metadata Specifications

- UML, MOF, CWM, XMI

- UML Profile

Modernization Specifications

Platform Independent Model (PIM), Platform Specific Model (PSM), Interface Specifications

- CORBAServices

- CORBAFacilities

OMG Domain Specifications

CORBA Embedded Intelligence Specifications

CORBA Security Specifications

All of OMG’s formal specifications may be downloaded without charge from our website. (Products implementing OMG specifications are available from individual suppliers.) Copies of specifications, available in PostScript and PDF format, may be obtained from the Specifications Catalog cited above or by contacting the Object Management Group, Inc. at:

OMG Headquarters
109 Highland Avenue
Needham, MA 02494
USA
Tel: +1-781-444-0404
Fax: +1-781-444-0320
Email: pubs@omg.org

Certain OMG specifications are also available as ISO standards. Please consult http://www.iso.org

Typographical Conventions

The type styles shown below are used in this document to distinguish programming statements from ordinary English. However, these conventions are not used in tables or section headings where no distinction is necessary.

Times/Times New Roman - 10 pt.:  Standard body text

NOTE:   Terms that appear in italics are defined in the glossary. Italic text also represents the name of a document, specification, or other publication.

Issues

The reader is encouraged to report any technical or editing issues/problems with this specification to http://www.omg.org/report_issue.htm.

UPOS 1.16.1 RCSD Specification Overview

Updated Items in Release 1.16.1

In the current UPOS 1.16, heavy technical review was done and found some area to be changed. Those changes are pointed out and all of changes are listed in the attached table. Please refer to the changes details in the table.

Table 1. Edited Items List for the dtc/20-12-01

Note: Characters in Blue with double erase lines were eliminated, Characters in Red were added.

| No | Chapter/ Device Name | Items to be corrected | Applied Resolution | Page No. |
| --- | --- | --- | --- | --- |
| 1 | 21 Lights | checkHealth Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 38 |
| 2 | 21 Lights | compareFirmwareVersion Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 38 |
| 3 | 21 Lights | resetStatistics Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 38 |
| 4 | 21 Lights | retrieveStatistics Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 38 |
| 5 | 21 Lights | updateFirmware Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 38 |
| 6 | 21 Lights | updateStatistics Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 38 |
| 7 | 29 POS Power | UPSChargeState Property in Summary section. | This property may use after open & enabled. However, mistakenly claim was added. Therefore, claim was eliminated | 52 |
| 8 | 29 POS Power | BatteryCriticallyLowThreshold Property in Property section. | This property is able to read and write. In that case there should be the E_ILLEGAL error Code. However, that description was missing. Therefore, E_ILLEGAL related description was added as listed below. Some possible values of the exception’s ErrorCode property are: Value Meaning E_ILLEGAL An invalid value was specified. Or it does not support this function. | 67 |
| 9 | 29 POS Power | BatteryCriticallyLowThresholdInSeconds Property in Property section. | This property is able to read and write. In that case there should be the E_ILLEGAL error Code. However, that description was missing. Therefore, E_ILLEGAL related description was added as listed below. Some possible values of the exception’s ErrorCode property are: Value Meaning E_ILLEGAL An invalid value was specified. Or it does not support this function. | 68 |
| 10 | 29 POS Power | BatteryLowThreshold Property in Property section. | This property is able to read and write. In that case there should be the E_ILLEGAL error Code. However, that description was missing. Therefore, E_ILLEGAL related description was added as listed below. Some possible values of the exception’s ErrorCode property are: Value Meaning E_ILLEGAL An invalid value was specified. Or it does not support this function. | 68 |
| 11 | 29 POS Power | BatteryLowThresholdInSeconds Property in Property section. | This property is able to read and write. In that case there should be the E_ILLEGAL error Code. However, that description was missing. Therefore, E_ILLEGAL related description was added as listed below. Some possible values of the exception’s ErrorCode property are: Value Meaning E_ILLEGAL An invalid value was specified. Or it does not support this function. | 69 |
| 12 | 29 POS Power | EnforcedShutdownDelayTime Property In Property section. | This property is able to read and write. In that case there should be the E_ILLEGAL error Code. However, that description was missing. Therefore, E_ILLEGAL related description was added as listed below. Some possible values of the exception’s ErrorCode property are: Value Meaning E_ILLEGAL An invalid value was specified. Or it does not support this function. | 74 |
| 13 | 39 Video Capturer | VideoCaptureMode Property in Summary section. | This property’s mutability should be read-write. Therefore, this sections read-only description was corrected as read-write. | 84 |
| 14 | 39 Video Capturer | checkHealth Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 85 |
| 15 | 39 Video Capturer | clearInput Method in Summary section | This method behavior was missing. Therefore, those description should be added. void:{raises-exception, use after open, claim} | 85 |
| 16 | 39 Video Capturer | compareFirmwareVersion Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 85 |
| 17 | 39 Video Capturer | resetStatistics Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 85 |
| 18 | 39 Video Capturer | retrieveStatistics Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 85 |
| 19 | 39 Video Capturer | updateFirmware Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 85 |
| 20 | 39 Video Capturer | updateStatistics Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 85 |
| 21 | 39 Video Capturer | VCAP_CST_HARTTOTALS_ONLY had a TYPO (1st one) in Device behaviors Photo Mode | This should be corrected as follows. VCAP_CST_HARDTTOTALS_ONLY | 89 |
| 22 | 39 Video Capturer | Model Section Device behaviors of Photo mode | This method is executed synchronously. This is not enough explanation. Therefore, it was changed as “This method is executedperformed synchronously as the process of taking photo.” And added those since StatusUpdateEvent use are newly added. “The process of recorded data storing is performed asynchronously. StatusUpdateEvents are invoked for the application when the start and the end of device states are changed.” And the description “An attempt to nest the video capture device operations will result in an UPOSException being thrown.” is changed as follows. “If you try An attempt to nest the video capture device operations of the device, before the storing is finished, will result in an UPOSException will being thrown.” | 89 |
| 23 | 39 Video Capturer | Model Section Device behaviors of Photo mode ER_CONTINUEINPUT was incorrect and it was changed to ER_RETRY | Error Event behavior was incorrect and they were changed as follows. “If ErrorEvent response is ER_CONTINUEINPUT, the process of input processing continues. was changed as follows.” was changed below. “If ErrorEvent response is ER_RETRYCONTINUEINPUT, the process of recorded data storing was retried input processing continues. However, as long as the cause of the error is not resolved, the ErrorEvent will occur again immediately. | 90 |
| 24 | 39 Video Capture | Model Section Device behaviors of Photo mode ER_CLEAR description was changed. | ER_CLEAR description was not good enough. “If ErrorEvent is ER_CLEAR, the input processing process is terminated and the taking photo is discarded. Is as changed as follows.” was changed as follows. “If ErrorEvent is ER_CLEAR, all of the device buffered data is cleared the input processing process is terminated and the takePhoto method taking photo is discarded.” | 90 |
| 25 | 39 Video Capture | Model Section Device behaviors of Video mode | Added the startVideo method behavior as follows, since current description was not good enough. “This method is executed synchronously. During video image capturing, recorded data storing is processed asynchronously and when the start and the end device state is changed, StatusUpdateEvents are invoked for the application. In addition, remaining device recording time is updated in the RemainingRecordingTimeInSec property.” | 90 |
| 26 | 39 Video Capture | VCAP_CST_HARTTOTALS_ONLY had a TYPO (2nd one) in Device behaviors Vide Mode | This should be corrected as follows. VCAP_CST_HARDTTOTALS_ONLY | 90 |
| 27 | 39 Video Capture | Model Section Device behaviors of Video mode | “The video capturing ends after the specified time has elapsed or when stopVideo method is called or when clearInput method is called” was changed as follows since it needs additional description. “The video capturing ends after the specified time has elapsed or when stopVideo method is called or when clearInput method is called even startVideo method is called.” | 90 |
| 28 | 39 Video Capture | VCAP_CST_HARTTOTALS_ONLY had a TYPO (3rd and 4th ones) in Device behavior Video Mode. | This should be corrected as follows. VCAP_CST_HARDTTOTALS_ONLY | 91 |
| 29 | 39 Video Capture | Model Section Device behaviors of Video mode ER_CONTINUEINPUT was incorrect and it was changed to ER_RETRY | Error Event behavior was incorrect. “If ErrorEvent response is ER_CONTINUEINPUT, the process of input processing continues. was changed as follows.” was changed as follows. “If ErrorEvent response is ER_RETRYCONTINUEINPUT, the process of recorded data storing was retried input processing continues. However, as long as the cause of the error is not resolved, the ErrorEvent will occur again immediately.” | 91 |
| 30 | 39 Video Capture | Model Section Device behaviors of Video mode ER_CLEAR description was changed. | ER_CLEAR description needs to change.. “If ErrorEvent is ER_CLEAR, the input processing process is terminated and the taking photo is discarded. is changed as follows.” was changed as follows.” was changed as follows. “If ErrorEvent is ER_CLEAR, all of the device buffered data is cleared and the error state is exited input processing process is terminated and the taking video capturing process is discarded.” | 91 |
| 31 | 39 Video Capture | Model Section Device behaviors of Video mode Error Event description | ErrorEvent description was changed, since description was good enough. “An ErrorEvent event (or events) is enqueued if an error occurs while gathering or processing input.” was changed as follows. “An ErrorEvent event (or events) is enqueued if an error occurs while gathering or processing the data input.” | 91 |
| 32 | 39 Video Capture | VCAP_CST_HARTTOTALS_ONLY had a TYPO (5th one) | This should be corrected as follows. VCAP_CST_HARDTTOTALS_ONLY | 91 |
| 33 | 39 Video Capture | Model Section Device behaviors of Video mode clearInput description | Regarding clearInput method description was changed as follows. All enqueued data input may be deleted by calling clearInput method. See the clearInput method description for more details. | 91 |
| 34 | 39 Video Capture | AutoExposure Property | This property needs to be used under the open, claim enable, however in the access after section claim and enable were missing. In the access after section changed and claim enable were added. | 92 |
| 35 | 39 Video Capture | AutoFocus Property | This property needs to be used under the open, claim enable, however in the access after section claim and enable were missing. In the access after section changed and claim enable were added. | 92 |
| 36 | 39 Video Capture | AutoGain Property | This property needs to be used under the open, claim enable, however in the access after section claim and enable were missing. In the access after section changed and claim enable were added. | 92 |
| 37 | 39 Video Capture | AutoWhiteBalance Property | This property needs to be used under the open, claim enable, however in the access after section claim and enable were missing. In the access after section changed and claim enable were added. | 93 |
| 38 | 39 Video Capture | Brightness property | This property needs to be used under the open, claim enable, however in the access after section claim and enable were missing. In the access after section changed and claim enable were added. | 93 |
| 39 | 39 Video Capture | CapAssociatedHardTotalsDevice Property | Capability needs to be used under the read-only, however description was read-write. Therefore, it was changed from read-write to read-writeonly. In addition, regarding the remarks section, description was not good enough to explain this property’s behavior. Therefore, “Holds the open name of the associated Hard Totals device, if the device is able to write to such devices which is the case, if CapStorage is either VCAP_CST_ALL or VCAP_CST_HARDTOTALS_ONLY. If CapStorage is VCAP_CST_HOST_ONLY this property value must be the empty string. “was corrected as follows. “Indicate that the device is able to store the recorded data into the Associated Hard Totals device and Hholds the its open name of the associated Hard Totals Device, if the device is able to write to such devices which is the case, if CapStorage is either VCAP_CST_ALL or VCAP_CST_HARDTOTALS_ONLY. If CapStorage is VCAP_CST_HOST_ONLY, the device is not able to store the data into the Associated Hard Totals device and this property value must be the empty string. This property is initialized by the open method. | 93 |
| 40 | 39 Video Capture | CapPhotoType Property | This property description’s See Also section missing the reference property name. Added the See Also and described below. See also PhotoType Property | 96 |
| 41 | 39 Video Capture | CapStorage Property | This property description’s Remarks did have the incorrect description. Current This is an enumeration and announces where the device is able to write the recorded sound data file to. corrected as follows. This is an enumeration and announces where the device is able to write the recorded video or photo sound data file to. | 97 |
| 42 | 39 Video Capture | Contrast Property | This property needs to be used under the open, claim enable, however is the spec claim and enable were missing and newly added. Access after open, claim enable. | 99 |
| 43 | 39 Video Capture | Exposure Property | This property needs to be used under the open, claim enable, however is the spec claim and enable were missing and newly added. Access after open, claim enable. | 99 |
| 44 | 39 Video Capture | Gain Property | This property needs to be used under the open, claim enable, however is the spec claim and enable were missing and newly added. Access after open, claim enable. | 99 |
| 45 | 39 Video Capture | HorizontalFlip Property | This property needs to be used under the open, claim enable, however is the spec claim and enable were missing and newly added. Access after open, claim enable. | 100 |
| 46 | 39 Video Capture | Hue Property | This property needs to be used under the open, claim enable, however is the spec claim and enable were missing and newly added. Access after open, claim enable. | 100 |
| 47 | 39 Video Capture | PhotoColorSpace Property | This property needs to be used under the open, claim enable, however is the spec claim and enable were missing and newly added. Access after open, claim enable. And this property’s Remarks description was incorrect therefore, it was corrected. Currently “Indicates the photo color space ID of the frame data to be acquired by the Video Capture Device, if CapPhotoColorSpace property is true. Valid values are one of the values listed in the CapPhotoColorSpaceList property. This property is initialized by the open method.” Revised “Indicates the photo color space ID of the frame data to be acquired by the Video Capture Device, if CapPhotoColorSpace property is true and it is used takePhoto method. Valid values are one of the values listed in the CapPhotoColorSpaceList property. This property is referred to when VideoCaptureMode property value is VCAP_VCMODE_PHOTO and CapPhoto is true. This property is initialized by the open method.” In this property, in the See also section several property and method are missing. Therefore, those property and method are added. See also PhotoColorSpaceList Property, VideoCaptureMode property, CapPhoto Property, CapPhotoColorSpace Property, takePhoto Method | 100 |
| 48 | 39 Video Capture | PhotoColorSpaceList Property | In Remarks, there was a word of Photo Color space, this should be Photo color space. In the Color space ID parameter, Description was not good enough. RGB depth description was changed. Currently: ID for identifying the color space of RGB, YUV422, etc. And they are indicating like “RGB;YUV422;…..” Revised: ID for identifying the color space of RGB, YUV422, etc. Then if RGB Depth was 16 bits, YUV422 Depth was 32 bits, And they are indicating like “RGB:16, YUV422:32, …..” In the See Also section, CapPhotoColorSpace Property was missing, therefore it was added. | 101 |
| 49 | 39 Video Capture | PhotoFrameRate Property | This property needs to be used under the open, claim enable, however is the spec claim and enable were missing and newly added. Access after open, claim enable. In Remarks, the device behavior description was not good enough, therefore, description was changed as follows. Currently: “Indicates the frame rate of frame data to be acquired by the Video Capture Device. Valid values range from 1 to PhotoMaxFrameRate property.” Revised: “Indicates the frame rate of frame data recorded to be acquired by the Video Capture Device and the photo image capturing and recorded with the takePhoto method. This property is only applied when VCAP_VCMODE_PHOTO is set in VideoCaptureMode property. Valid values range from 1 to PhotoMaxFrameRate property and CapPhoto property is true.” In the See Also section, CapPhoto property, takePhoto method were missing, therefore they were added. | 101 |
| 50 | 39 Video Capture | PhotoResolution Property | This property needs to be used under the open, claim enable, however is the spec claim and enable were missing and newly added. Access after open, claim, enable. In Remarks, VideoCaptureMode property related information was missing. Therefore, this description was added. This property is only applied when VCAP_VCMODE_PHOTO is set in VideoCaptureMode property and if CapPhoto is true. In the See Also section, CapPhoto Property was missing, therefore it was added. | 102 |
| 51 | 39 Video Capture | PhotoResolutionList Property | In the See Also section, CapPhotoResolution Property was missing, therefore it was added. | 102 |
| 52 | 39 Video Capture | PhotoType Property | This property needs to be used under the open, claim enable, however is the spec claim and enable were missing and newly added. Access after open, claim enable. In Remarks, there is CapPhotoTypeList property, but this is not existing property and it is corrected as PhotoTypeList property. In the See Also section, CapPhoto Property was missing therefore, it was added. And CapPhotoTypeList property was corrected as CapPhotoTypeList property. | 102 |
| 53 | 39 Video Capture | Saturation Property | This property needs to be used under the open, claim enable, however is the spec claim and enable were missing and newly added. Access after open, claim enable. | 103 |
| 54 | 39 Video Capture | Storage Property | This property is able to read and write. In that case there should be the E_ILLEGAL error Code. However, that description was missing. Therefore, E_ILLEGAL related description was added as listed below. Some possible values of the exception’s ErrorCode property are: Value Meaning E_ILLEGAL An invalid value was specified or recording is ongoing. | 104 |
| 55 | 39 Video Capture | VerticalFlip Property | This property needs to be used under the open, claim enable, however is the spec claim and enable were missing and newly added. Access after open, claim enable. | 104 |
| 56 | 39 Video Capture | VideoCaptureMode Property | This property is able to read and write. In that case there should be the E_ILLEGAL error Code. However, that description was missing. Therefore, E_ILLEGAL related description was added as listed below. Some possible values of the exception’s ErrorCode property are: Value Meaning E_ILLEGAL An invalid value was specified. In the See also section, there was a description of VideoCaptureMode property, since this is the VideoCaptureMode property section, therefore it is eliminated from the See also section like this. VideoCaptureMode property. | 105 |
| 57 | 39 Video Capture | VideoColorSpace Property | This property needs to be used under the open, claim enable, however is the spec claim and enable were missing and newly added. Access after open, claim enable. In Remarks, description of “Indicates the video color space ID of the frame data to be used by startVideo method.” was not good enough. Therefore, corrected as follows. “Indicates the video color space ID of the frame data to be acquired by the Video Capture Device, if CapVideoColorSpace property is true and it is used by startVideo method.” In the See also section, CapVideoColorSpace property was missing therefore, it was added. | 106 |
| 58 | 39 Video Capture | VideoColorSpaceList Property | In Remarks, regarding the Color space ID parameter, Description was not good enough. RGB depth description was changed. Currently: ID for identifying the color space of RGB, YUV422, etc. And they are indicating like “RGB;YUV422;…..” Revised: ID for identifying the color space of RGB, YUV422, etc. Then if RGB Depth was 16 bits, YUV422 Depth was 32 bits, And they are indicating like “RGB:16, YUV422:32, …..” In the See Also section, VideoCaptureMode Property was missing, therefore it was added. | 106 |
| 59 | 39 Video Capture | VideoFrameRate Property | This property needs to be used under the open, claim enable, however is the spec claim and enable were missing and newly added. Access after open, claim enable. In the See also section, CapVideoFrameRate Property was missing, therefore it was added. | 106 |
| 60 | 39 Video Capture | VideoMaxFrameRate Property | In the See also section, VideoCaptureMode Property was missing, therefore it was added. | 107 |
| 61 | 39 Video Capture | VideoResolution Property | This property needs to be used under the open, claim enable, however is the spec claim and enable were missing and newly added. Access after open, claim enable. | 107 |
| 62 | 39 Video Capture | VideoType Property | This property needs to be used under the open, claim enable, however is the spec claim and enable were missing and newly added. Access after open, claim enable. | 107 |
| 63 | 39 Video Capture | VideoTypeList Property | In the Remarks, list of image description missing the word of video and it was corrected from “list of image format” to “list of video image format". In the See also section, VideoCaptureMode Property was missing and it was added. CapVideoType property is not needed in this section therefore it is eliminated. | 108 |
| 64 | 39 Video Capture | startVideo Method | In the Remarks, StatusUpdateEvent related description was not precise therefore, this description was added. “During the video image capturing, the recorded data storing is processed asynchronously and when the start and stop state are changed, StatusUpdateEvents are invoked for the application.” And this description was eliminated. “StatusUpdateEvent will notify the application that there is a change in the power status or a state change during video capturing and recording.” In the Errors section, in the E_ILLEGAL, need to add the description after VCAP_VCMODE_VIDEO. It is “and CapVideo is not true.” | 109 |
| 65 | 39 Video Capture | stopVideo Method | In the Remarks, StatusUpdateEvent related description was changed for the better description. Currently “StatusUpdateEvent will notify the application that there is a change in the power status or a state change during taking video and recording.” Revised “The device invokes StatusUpdateEvent will to notify the application that there is a change in the power status or a state change during taking video capturing and recording were stopped.” | 110 |
| 66 | 39 Video Capture | takePhoto Method | In the Remarks, StatusUpdateEvent related description was not good enough. Therefore, those description was added. “The process of recorded data storing is performed asynchronously. StatusUpdateEvents are invoked for the application when the start and the end state were changed.” | 111 |
| 67 | 39 Video Capture | ErrorEvent | In the ErrorLocus section, EL_OUTPUT should be corrected since OUTPUT cancelling method is clearInput method. Therefore, it is changed from EL_OUTPUT to EL_INPUT. Then description will be as follows. Attributes Type Description ErrorLocus int32 Location of the error. If EL_INOUTPUT is specified. An error occurred during asynchronous process action. In the ErrorLocus attribute, EL_OUTPUT related descriptions are changed to EL_INPUT. The ErrorLocus attribute has one of the following values: Value Meaning EL_INOUTPUT Error occurred while processing asynchronous output. Also, ErrorResponse will be changed as follows. Value Meaning ER_RETRY Retry recorded sending the data storing. The error state is exited. Typically, valid for asynchronous recorded data storing output devices when the locus is EL_INOUTPUT, in which case the asynchronous recorded data storing output is retried, and the error state is exited. This is the default response when the locus is EL_OUTPUT. ER_CLEAR Valid for loci: EL_OUTPUT. Clear all buffered captured input or stored output data (including all asynchronous output). The error state is exited In the See also section, “Device Output Models" on page Intro-25. was eliminated and “Device information Reporting Model “ on Page Intro-30” was added. | 113 |
| 68 | 40 Individual Recognition | clearInput Method in Summary section | This method behavior was missing. Therefore, those description should be added. void:{raises-exception, use after open, claim} | 116 |
| 69 | 40 Individual Recognition | clearInputProperties Method in Summary section | This method behavior was missing. Therefore, those description should be added. void:{raises-exception, use after open, claim} | 116 |
| 70 | 40 Individual Recognition | compareFirmwareVersion Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 116 |
| 71 | 40 Individual Recognition | resetStatistics Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 116 |
| 72 | 40 Individual Recognition | retrieveStatistics Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 116 |
| 73 | 40 Individual Recognition | updateFirmware Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 116 |
| 74 | 40 Individual Recognition | updateStatistics Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 116 |
| 75 | 40 Individual Recognition | IndividualRecognitionFilter Property | This property can be accessed after open claim enable. And current description missing the claim, enable and they are added. This property is able to read and write. In that case there should be the E_ILLEGAL error Code. However, that description was missing. Therefore, E_ILLEGAL related description was added as listed below. Some possible values of the exception’s ErrorCode property are: Value Meaning E_ILLEGAL An invalid value was specified. | 126 |
| 76 | 40 Individual Recognition | ErrorEvent | In the ErrorLocus description, EL_OUTPUT related description was existing. However, there is no EL_OUTPUT error since this is input device. Therefore, Added See values below. And added those since there is no need the EL_OUTPUT related description and it is eliminated. It is If EL_OUTPUT is specified. An error occurred during asynchronous action. IN the ErrorLocus attribute, EL_OUTPUT related descriptions are eliminated. Value Meaning EL_OUTPUT Error occurred while processing asynchronous output. Also, since this device is Input device there is no need the ER_RETRY. Then ER_RETRY description was eliminated as follows. Value Meaning ER_RETRY Retry sending the data. The error state is exited. May be valid for some input devices when the locus is EL_INPUT, in which case the input is re-tried, and the error state is exited. Typically, valid for asynchronous output devices when the locus is EL_OUTPUT, in which case the asynchronous output is re-tried, and the error state is exited. This is the default response when the locus is EL_OUTPUT. And ER_CLEAR description was corrected as follows. Loci: was a typo of locus and it was corrected as “Valid for all loci: to all locus:” And there is no EL_OUTPUT since this device is input device. Therefore, and EL_OUTPUT description was eliminated. Then they are described as follows.ted. Then they are described as follows. Value Meaning ER_CLEAR Valid for all locusloci: EL_INPUT, EL_INPUT_DATA, and EL_OUTPUT. Clear all buffered input or output data (including all asynchronous output). The error state is exited. This is the default response when the locus is EL_INPUT. In the See also section, “Device Output Models" on page Intro-25. was eliminated since this is input device. | 129 |
| 77 | 41 Sound Recorder | clearInput Method in Summary section | This method behavior was missing. Therefore, those description should be added. void:{raises-exception, use after open, claim} | 132 |
| 78 | 41 Sound Recorder | clearInputProperties Method in Summary section | This method behavior was missing. Therefore, those description should be added. void:{raises-exception, use after open, claim} | 133 |
| 79 | 41 Sound Recorder | compareFirmwareVersion Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 133 |
| 80 | 41 Sound Recorder | resetStatistics Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 133 |
| 81 | 41 Sound Recorder | retrieveStatistics Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 133 |
| 82 | 41 Sound Recorder | updateFirmware Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 133 |
| 83 | 41 Sound Recorder | updateStatistics Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 133 |
| 84 | 41 Sound Recorder | CapAssociatedHardTotalsDevice Property | In the Remarks initialization description was missing. Therefore, this description was added. This property is initialized by the open method. | 138 |
| 85 | 41 Sound Recorder | Channel Property | This property is able to read and write. In that case there should be the E_ILLEGAL error code. However, that description was missing. Therefore, E_ILLEGAL related description was added as listed below. Some possible values of the exception’s ErrorCode property are: Value Meaning E_ILLEGAL An invalid value was specified. | 139 |
| 86 | 41 Sound Recorder | SamplingRate Property | This property is able to read and write. In that case there should be the E_ILLEGAL error Code. However, that description was missing. Therefore, E_ILLEGAL related description was added as listed below. Some possible values of the exception’s ErrorCode property are: Value Meaning E_ILLEGAL An invalid value was specified. | 140 |
| 87 | 41 Sound Recorder | SoundType Property | This property can be accessed after open claim enable. And current description missing the enable description and they are added. This property is able to read and write. In that case there should be the E_ILLEGAL error Code. However, that description was missing. Therefore, E_ILLEGAL related description was added as listed below. Some possible values of the exception’s ErrorCode property are: Value Meaning E_ILLEGAL An invalid value was specified. | 141 |
| 88 | 41 Sound Recorder | ErrorEvent | In the See also section, Status, Error code, State model description was not needed therefore it is eliminated. Then “Device Input Model” on page Intro-22” and “Error Handling” on page Intro-23 was added instead. | 146 |
| 89 | 42 Voice Recognition | checkHealth Method in Summary section | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 149 |
| 90 | 42 Voice Recognition | clearInput Method in Summary section | This method behavior description missing the several conditions. Therefore, those are newly added. void:{raises-exception, use after open, claim} | 149 |
| 91 | 42 Voice Recognition | clearInputProperties Method in Summary section | This method behavior description missing the several conditions. Therefore, those are newly added. void:{raises-exception, use after open, claim} | 149 |
| 92 | 42 Voice Recognition | ErrorEvent | In the Attribute section, regarding the ErrorLocus, there is no EL_OUTPUT since this device is input device. Therefore, description of See values below. was added and If EL_OUTPUT is specified. An error occurred during asynchronous action was eliminated. Then corrected as follows Attributes Type Description ErrorLocus nt32 Location of the error. See values below. If EL_OUTPUT is specified. An error occurred during asynchronous action. In the ErrorLocus attribute vale, EL_OUTPUT related descriptions are eliminated then corrected as follows. Value Meaning EL_OUTPUT Error occurred while processing asynchronous output. In the ErrorResponse attribute value, ER_RETRY and its meaning is eliminated since this is input device. They are corrected as follows. Value Meaning ER_RETRY Retry sending the data. The error state is exited. May be valid for some input devices when the locus is EL_INPUT, in which case the input is re-tried, and the error state is exited. Typically, valid for asynchronous output devices when the locus is EL_OUTPUT, in which case the asynchronous output is re-tried, and the error state is exited. This is the default response when the locus is EL_OUTPUT And ER_CLEAR description was corrected, since loci: was a typo of locus and it was corrected as “Valid for all loci: to all locus:” And EL_OUTPUT was eliminated, since this is a input device. Therefore, they are corrected as follows. Value Meaning ER_CLEAR Valid for all locusloci: EL_INPUT, EL_INPUT_DATA and EL_OUTPUT. Clear all buffered input or output data (including all asynchronous output). In the See also section, “Device Output Models" on page Intro-25. was eliminated since this is input device. | 164 |
| 93 | 43 Sound Player | checkHealth Method in Summary section | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 168 |
| 94 | 43 Sound Player | clearInput Method in Summary section | This method behavior description missing the several conditions. Therefore, those are newly added. void:{raises-exception, use after open, claim} | 168 |
| 95 | 43 Sound Player n | clearInputProperties Method in Summary section | This method behavior description missing the several conditions. Therefore, those are newly added. void:{raises-exception, use after open, claim} | 168 |
| 96 | 43 Sound Player | CapAssociatedHardTotalsDevice Property | This the capability property, therefore, can be used as read only, therefore, read-write was corrected as read-only. In the Remarks initialization description was missing. Therefore, this description was added. This property is initialized by the open method. | 173 |
| 97 | 43 Sound Player | CapStorage Property | In the See also section, referenced properties were missing, therefore, Storage Property, CapAssociatedHardTotalsDevice Property were added. | 174 |
| 98 | 43 Sound Player | Storage Property | This property is able to read and write. In that case there should be the E_ILLEGAL error code. However, that description was missing. Therefore, E_ILLEGAL related description was added as listed below. Some possible values of the exception’s ErrorCode property are: Value Meaning E_ILLEGAL An invalid value was specified or recording is ongoing. | 175 |
| 99 | 43 Sound Player | ErrorEvent | In the Attribute section, regarding the ErrorLocus description “An error occurred during asynchronous action.” was eliminated and added “It is indicating that the error occurred while processing asynchronous recorded data storing.” instead. Therefore it is corrected as follows. Attributes Type Description ErrorLocus int32 Location of the error. If EL_OUTPUT is specified. It is indicating that the error occurred while processing asynchronous recorded data storing. An error occurred during asynchronous action. In the ErrorLocus description of EL_OUTPUT, the description, “The ErrorLocus attribute has the one of following values:” was changed since only EL_OUTPUT was existing. That is to say, “The ErrorLocus attribute has the one of following values:” In the ErrorResponse attribute values, ER_RETRY and ER_CLEAR were changed since this is the output device and there is no EL_OUTPUT. They are corrected as follows. Value Meaning ER_RETRY Retry sending the asynchronous output. data. The error state is exited. May be valid for some input devices when the locus is EL_INPUT, in which case the input is re-tried, and the error state is exited. Typically, valid for synchronous output devices when the locus is EL_OUTPUT, in which case the asynchronous output is re-tried, and the error state is exited. This is the default response when the locus is EL_OUTPUT. ER_CLEAR Valid for all loci: EL_INPUT, EL_INPUT_DATA, and EL_OUTPUT. Clear all buffered input or output data (including all asynchronous output). (The effect is the same as when clearOutput method is called.） The error state is exited. | 178 |
| 100 | 44 Speech Synthesis | checkHealth Method in Summary section | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 181 |
| 101 | 44 Speech Synthesis | clearInput Method in Summary section | This method behavior description missing the several conditions. Therefore, those are newly added. void:{raises-exception, use after open, claim} | 181 |
| 102 | 44 Speech Synthesis | clearInputProperties Method in Summary section | This method behavior description missing the several conditions. Therefore, those are newly added. void:{raises-exception, use after open, claim} | 181 |
| 103 | 44 Speech Synthesis | clearOutput Method in Summary section | This method behavior description missing the several conditions. Therefore, those are newly added. void:{raises-exception, use after open, claim} | 182 |
| 104 | 44 Speech Synthesis | ErrorEvent | In the Attribute section, regarding the ErrorLocus description “An error occurred during asynchronous action.” was eliminated and added “It is indicating that the error occurred while processing asynchronous recorded data storing.” instead. In the ErrorLocus description of EL_OUTPUT, the description, “The ErrorLocus attribute has the one of following values:” was changed since only EL_OUTPUT was existing. That is to say, “The ErrorLocus attribute has the one of following values:” In the ErrorResponse, ER_RETRY and ER_CLEAR was changed since this is the output device and there is no EL_OUTPUT. Value Meaning ER_RETRY Retry sending the asynchronous output. data. The error state is exited. May be valid for some input devices when the locus is EL_INPUT, in which case the input is re-tried, and the error state is exited. Typically, valid for synchronous output devices when the locus is EL_OUTPUT, in which case the asynchronous output is re-tried, and the error state is exited. This is the default response when the locus is EL_OUTPUT. ER_CLEAR Valid for all loci: EL_INPUT, EL_INPUT_DATA, and EL_OUTPUT. Clear all buffered input or output data (including all asynchronous output). The error state is exited. (The effect is the same as when clearOutput method is called.）The error state is existed. | 196 |
| 105 | 45 Gesture Control | checkHealth Method in Summary section | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 199 |
| 106 | 45 Gesture Control | clearInput Method in Summary section | This method behavior description missing the several conditions. Therefore, those are newly added. void:{raises-exception, use after open, claim} | 199 |
| 107 | 45 Gesture Control | clearInputProperties Method in Summary section | This method behavior description missing the several conditions. Therefore, those are newly added. void:{raises-exception, use after open, claim} | 199 |
| 108 | 45 Gesture Control | clearOutput Method in Summary section | This method behavior description missing the several conditions. Therefore, those are newly added. void:{raises-exception, use after open, claim} | 199 |
| 109 | 45 Gesture Control | compareFirmwareVersion Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 199 |
| 110 | 45 Gesture Control | resetStatistics Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 199 |
| 111 | 45 Gesture Control | retrieveStatistics Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 199 |
| 112 | 45 Gesture Control | updateFirmware Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 199 |
| 113 | 45 Gesture Control | updateStatistics Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 199 |
| 114 | 45 Gesture Control | CapAssociatedHardTotalsDevice Property | In the Remarks initialization description was missing. Therefore, this description was added. This property is initialized by the open method. | 205 |
| 115 | 45 Gesture Control | ErrorEvent | In the Attribute section, regarding the ErrorLocus description “An error occurred during asynchronous action.” was eliminated and added “It is indicating that the error occurred while processing asynchronous recorded data storing.” instead. In the ErrorLocus description of EL_OUTPUT, the description, “The ErrorLocus attribute has the one of following values:” was changed since only EL_OUTPUT was existing. That is to say, “The ErrorLocus attribute has the one of following values:” In the ErrorResponse, ER_RETRY and ER_CLEAR was changed since this is the output device and there is no EL_OUTPUT. Value Meaning ER_RETRY Retry sending the asynchronous output. data. The error state is exited. May be valid for some input devices when the locus is EL_INPUT, in which case the input is re-tried, and the error state is exited. Typically, valid for synchronous output devices when the locus is EL_OUTPUT, in which case the asynchronous output is re-tried, and the error state is exited. This is the default response when the locus is EL_OUTPUT. ER_CLEAR Valid for all loci: EL_INPUT, EL_INPUT_DATA, and EL_OUTPUT. Clear all buffered input or output data (including all asynchronous output). The error state is exited. (The effect is the same as when clearOutput method is called.）The error state is existed. | 218 |
| 116 | 46 Device Monitor | checkHealth Method in Summary section | This method behavior description missing the several conditions. Therefore, in use after claim was added. | 221 |
| 117 | 46 Device Monitor | clearInput Method in Summary section | This method behavior description missing the several conditions. Therefore, those are newly added. void:{raises-exception, use after open, claim} | 221 |
| 118 | 46 Device Monitor | clearInputProperties Method in Summary section | This method behavior description missing the several conditions. Therefore, those are newly added. void:{raises-exception, use after open, claim} | 221 |
| 119 | 46 Device Monitor | compareFirmwareVersion Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 221 |
| 120 | 46 Device Monitor | resetStatistics Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 221 |
| 121 | 46 Device Monitor | retrieveStatistics Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 221 |
| 122 | 46 Device Monitor | updateFirmware Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 221 |
| 123 | 46 Device Monitor | updateStatistics Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 221 |
| 124 | 46 Device Monitor | ErrorEvent | In Attribute, regarding the ErrorResponse value, ER_RETRY should be eliminated, since this is the input device. Also, in the ER_CLEAR output device related description was corrected since there are typo of loci and there is not output data because of input device. Therefore, these descriptions were changed as follows. Value Meaning ER_RETRY Retry sending the data. The error state is exited. May be valid for some input devices when the locus is EL_INPUT, in which case the input is re-tried, and the error state is exited. ER_CLEAR Valid for all locius: EL_INPUT and EL_INPUT_DATA. Clear all buffered input or output data (including all asynchronous output). The error state is exited. This is the default response when the locus is EL_INPUT. | 232 |
| 125 | 47 Graphic Display | checkHealth Method in Summary section | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 235 |
| 126 | 47 Graphic Display | clearInput Method in Summary section | This method behavior description missing the several conditions. Therefore, those are newly added. void:{raises-exception, use after open, claim} | 235 |
| 127 | 47 Graphic Display | clearInputProperties Method in Summary section | This method behavior description missing the several conditions. Therefore, those are newly added. void:{raises-exception, use after open, claim} | 235 |
| 128 | 47 Graphic Display | clearOutput Method in Summary section | This method behavior description missing the several conditions. Therefore, those are newly added. void:{raises-exception, use after open, claim} | 236 |
| 129 | 47 Graphic Display | compareFirmwareVersion Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 236 |
| 130 | 47 Graphic Display | resetStatistics Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 236 |
| 131 | 47 Graphic Display | retrieveStatistics Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 236 |
| 132 | 47 Graphic Display | updateFirmware Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 236 |
| 133 | 47 Graphic Display | updateStatistics Method in Summary section. | This method can be used after open claim enable, however summary section missing the claim and it was added in the use after section. | 236 |
| 134 | 47 Graphic Display | Model section, Image Display Mode | The description “The ImageTypeList property lists image files that the device can display.“ is duplicated the previous description, therefore, it is eliminated. | 240 |
| 135 | 47 Graphic Display | Model section, Video Display Mode | There was a description regarding the CapMultiDisplay property. However, it was not an existing property and already eliminated one. Therefore, the description, “ To check if the device supports simultaneous playback, check the CapMultiPlay property” was eliminated. | 241 |
| 136 | 47 Graphic Display | Model section, Web Display Mode | There was a description of cancelLoading method. This was incorrect and corrected as cancelURLLoading method. | 241 |
| 137 | 47 Graphic Display | CapAssociatedHardTotalsDevice Property | This the capability property, therefore, can be used as read only, therefore, read-write was corrected as read-only. In the Remarks initialization description was missing. Therefore, this description was added. This property is initialized by the open method. | 243 |
| 138 | 47 Graphic Display | DisplayMode Property | In the See also section, referencing method and property was incorrect. Therefore, loadImage Method and playVideo Method were newly added and Video Capture Device Property of CapCaptureColorSpaceList Property and VideoCaptureMode Property were eliminated. | 246 |
| 139 | 47 Graphic Display | ImageType Property | This property is able to read and write. In that case there should be the E_ILLEGAL error Code. However, that description was missing. Therefore, E_ILLEGAL related description was added as listed below. Some possible values of the exception’s ErrorCode property are: Value Meaning E_ILLEGAL An invalid value was specified | 247 |
| 140 | 47 Graphic Display | LoadStatus Property | This property can be accessed after open, therefore, current description of claim, enable were eliminated. | 247 |
| 141 | 47 Graphic Display | VideoType Property | This property is able to read and write. In that case there should be the E_ILLEGAL error Code. However, that description was missing. Therefore, E_ILLEGAL related description was added as listed below. Some possible values of the exception’s ErrorCode property are: Value Meaning E_ILLEGAL An invalid value was specified | 249 |
| 142 | 47 Graphic Display | playVideo Method | There was a description in the Remarks regarding not Video but Image. Since this is explain how to display the video therefore, image related information was corrected as Video related information. Then description was corrected as follows. Play the specified video type file that are set in the VideoType property loaded in the storage area by the loadImage method. All of the video loaded file values are listed in the ImageVideoTypeList property, if CapVideoType property is true. Regarding the DisplayMode property values, they were written as GDISP_DMODE_VIDEO_NORMAL, GDISP_DMODE_VIDEO_FULL. Since GDISP was incorrect should be GDSP. Therefore, they were corrected as follows. GDSP_DMODE_VIDEO_NORMAL, GDSP_DMODE_VIDEO_FULL | 252 |
| 143 | 47 Graphic Display | stopVideo Method | In the Remarks section, there was a word of Image file, since this method handling the Video therefore, this was corrected from Image file to Video file. In the See also section, referencing method startVideo method was incorrect and corrected as playVideo method. | 252 |
| 144 | 47 Graphic Display | ErrorEvent | In the Attribute section, regarding the ErrorLocus description “An error occurred during asynchronous action.” was eliminated and added as “It is indicating that the error occurred while processing asynchronous recorded data storing.” instead. Therefore they were corrected as follows. Attributes Type Description ErrorLocus int32 Location of the error. If EL_OUTPUT is specified. It is indicating that the error occurred while processing asynchronous recorded data storing. An error occurred during asynchronous action. In the ErrorLocus description of EL_OUTPUT, the description, “The ErrorLocus attribute has the one of following values:” was changed since only EL_OUTPUT was existing. That is to say, “The ErrorLocus attribute has the one of following values:” In the ErrorResponse attribute values, ER_RETRY and ER_CLEAR were changed since this is the output device and there is no EL_OUTPUT. They are corrected as follows. Value Meaning ER_RETRY Retry sending the asynchronous output. data. The error state is exited. May be valid for some input devices when the locus is EL_INPUT, in which case the input is re-tried, and the error state is exited. Typically, valid for synchronous output devices when the locus is EL_OUTPUT, in which case the asynchronous output is re-tried, and the error state is exited. This is the default response when the locus is EL_OUTPUT. ER_CLEAR Valid for all loci: EL_INPUT, EL_INPUT_DATA, and EL_OUTPUT. Clear all buffered input or output data (including all asynchronous output). (The effect is the same as when clearOutput method is called.） The error state is exited. | 254 |
| 145 | Cover Page | Specification Version No. and Submission Date Correction | In the cover page there was an old description. This specification proposing the UPOS 1.16.1 and it editing the UPOS1.16. Also, documentation submission date is Sept. 2021. Such information was added and corrected as follows. Date: Nov.April 20210 Table 145 Unified POS RCSD, v1.16.1 FTF Beta 1 This specification adds to and extends the UPOS 1.156 specification. | 14 |
| 146 | 39/ Video Capture | StatusUpdateEvent value in Video Mode | The StatusUpdateEvent value was not VCAP SUE END VIDEO but VCAP_SUE_STOP_VIDEO therefore, the following descriptions were corrected. When the taking video is finished, or the specified time out has been exceeded, a StatusUpdateEvent with status VCAP_SUE_STOPEND_VIDEO is evoked to notify the application that taking video has been ended. | 90 |
| 147 | 39 Video Capture | StatusUpdateEvent values in StatusUpdateEvent Section | The StatusUpdateEvent values were incorrect and they were corrected as follows. Value Meaning Goto Table147 VCAP_SUE_START_VIDEO_RECORDING It will be notified when video recording starts. VCAP_SUE_STOP_VIDEO_RECORDING It will be notified when video recording stops. | 114 |
| 148 | 40 Individual Recognition | DataEvent description in DataEvent section | The DataEvent descriptions were incorrect and they were corrected as follows. Goto Table148 Remarks Before this event is delivered, the data is copied into corresponding properties individual recognition information is enqueued into the area that is indicated by the readValue method. Since the stored individual recognition information might be managed by the associated “Hard Totals” device service, therefore, the application might also support the “Hard Totals” service. See Also VideoCaptureMode Property, “Events" on page Intro-19, directIO method. | 128 |
| 149 | 41 Sound Recorder | startRecordingMethod recordingTime parameter value. | This method’s recordingTime value description OPOS_FOREVER(-1) was incorrect. Ando it was changed as FOREVER(-1). | 143 |

Chapter 21

Lights

This Chapter defines the Lights device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.12 | Not supported |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.12 | open |
| CapPowerReporting: | int32 | {read-only} | 1.12 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.12 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.12 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.12 | open |
| CheckHealthText: | string | {read-only} | 1.12 | open |
| Claimed: | boolean | {read-only} | 1.12 | open |
| DataCount: | int32 | {read-only} | 1.12 | Not supported |
| DataEventEnabled: | boolean | {read-write} | 1.12 | Not supported |
| DeviceEnabled: | boolean | {read-write} | 1.12 | open & claim |
| FreezeEvents: | boolean | {read-write} | 1.12 | open |
| OutputID: | int32 | {read-only} | 1.12 | Not supported |
| PowerNotify: | int32 | {read-write} | 1.12 | open |
| PowerState: | int32 | {read-only} | 1.12 | open |
| State: | int32 | {read-only} | 1.12 | -- |
| DeviceControlDescription: | string | {read-only} | 1.12 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.12 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.12 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.12 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.12 | open |
| PhysicalDeviceName: | string | {read-only} | 1.12 | open |

| Properties (Continued) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specific | Type | Mutability | Version | May Use After |
| CapAlarm: | int32 | {read-only} | 1.12 | open |
| CapBlink: | boolean | {read-only} | 1.12 | open |
| CapColor: | int32 | {read-only} | 1.12 | open |
| CapPattern: | int32 | {read-only} | 1.16 | open |
| MaxLights: | int32 | {read-only} | 1.12 | open |

Goto Table6

Goto Table5

Goto Table4

Goto Table2

Goto Table3

Goto Table1

| Methods (UML operations) |  |
| --- | --- |
| Common |  |
| Name | Version |
| open (logicalDeviceName: string): void {raises-exception} | 1.12 |
| close ( ): void {raises-exception, use after open} | 1.12 |
| claim (timeout: int32): void {raises-exception, use after open} | 1.12 |
| release ( ): void {raises-exception, use after open, claim} | 1.12 |
| checkHealth (level: int32): void {raises-exception, use after open, claim, enable} | 1.12 |
| clearInput ( ): void { } | Not supported |
| clearInputProperties ( ): void { } | Not supported |
| clearOutput ( ): void { } | Not supported |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} | 1.12 |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, claim, enable} | 1.12 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | 1.12 |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | 1.12 |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, claim, enable} | 1.12 |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | 1.12 |
| Specific |  |
| Name |  |
| switchOff (lightNumber: int32): void {raises-exception, use after open, claim, enable} | 1.12 |
| switchOn (lightNumber: int32, blinkOnCycle: int32, blinkOffCycle: int32, color: int32, alarm: int32): void {raises-exception, use after open, claim, enable} | 1.12 |
| switchOnMultiple (lightNumbers: string, blinkOnCycle: int32, blinkOffCycle: int32, color: int32, alarm: int32): void {raises-exception, use after open, claim, enable} | 1.16 |
| switchOnPattern (pattern: int32, alarm: int32): void {raises-exception, use after open, claim, enable} | 1.16 |
| switchOffPattern ( ): void {raises-exception, use after open, claim, enable} | 1.16 |

| Events (UML interfaces) |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Mutability | Version |
| upos::events::DataEvent |  | Not supported |  |
| upos::events::DirectIOEvent |  |  | 1.12 |
| EventNumber: | int32 | {read-only} |  |
| Data: | int32 | {read-write} |  |
| Obj: | object | {read-write} |  |
| upos::events::ErrorEvent |  | Not supported |  |
| upos::events::OutputCompleteEvent |  | Not supported |  |
| upos::events::StatusUpdateEvent |  |  | 1.12 |
| Status: | int32 | {read-only} |  |
| upos::events::TransitionEvent |  | Not supported | 1.16 |

General Information

The Lights programmatic name is “Lights”.

This device category was added to Version 1.12 of the specification.

Capabilities

- The Lights device control has the following capability:

- Supports commands to “switch on” and “switch off” a light.

- The Lights device control may have the following additional capabilities:

  - Supports device-level blinking at adjustable blink cycles.

  - Support multiple lights.

  - Supports different colors of a light.

  - Supports different alarms

Device Sharing

Lights is an exclusive-use device. Its device sharing rules are:

- The application must claim the device before enabling it.

- The application must claim and enable the device before accessing some of the properties and methods, or receiving events.

- See the “Summary” table for precise usage prerequisites.

Lights Class Diagram

Updated in Release 1.16

The following diagram shows the relationships between the Lights classes

Fig. Chap. 21-1 Lights Class Diagram

Lights Sequence Diagram

The following sequence diagram show the typical usage of the Lights device illustrating the handling of the media entry indicator lights.

Fig. Chap. 21-2 Lights Sequence Diagram (handling of the media entry indicator lights)

The following sequence diagram show the typical usage of the Lights device illustrating the handling of the pole lights.

Fig. Chap. 21-3 Lights Sequence Diagram (handling of the pole lights)

Properties(UML attributes)

CapAlarm Property

Syntax	CapAlarm: int32 {read-only, access after open}

Remarks	This capability indicates if the device supports different alarms.

CapAlarm is a logical OR combination of any of the following values:

Value	Meaning

LGT_ALARM_NOALARM	Alarms are not supported.

LGT_ALARM_SLOW		Supports a slow beep.

LGT_ALARM_MEDIUM	Supports a medium beep.

LGT_ALARM_FAST		Supports a fast beep.

LGT_ALARM_CUSTOM1	Supports 1st custom alarm.

LGT_ALARM_CUSTOM2	Supports 2nd custom alarm.

This property is initialized by the open method. If the device does not support alarms, it is initialized to LGT_ALARM_NOALARM.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

CapBlink Property

Syntax	CapBlink: boolean {read-only, access after open}

Remarks	If true, a blinking capability is supported. It may be either a physical capability of the device or emulated by the service.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

CapColor Property

Syntax	CapColor: int32 {read-only, access after open}

Remarks	This capability indicates if the device supports different colors.

CapColor is a logical OR combination of any of the following values:

Value	Meaning

LGT_COLOR_PRIMARY	Supports Primary Color (Usually Green).

LGT_COLOR_CUSTOM1	Supports 1st Custom Color (Usually Red).

LGT_COLOR_CUSTOM2	Supports 2nd Custom Color (Usually Yellow).

LGT_COLOR_CUSTOM3	Supports 3rd Custom Color.

LGT_COLOR_CUSTOM4	Supports 4th Custom Color.

LGT_COLOR_CUSTOM5	Supports 5th Custom Color.

This property is initialized by the open method. If the device supports only one color, it is initialized to LGT_COLOR_PRIMARY.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

CapPattern Property                         Added in Release 1.16

Syntax	CapPattern: int32 {read-only, access after open}

Remarks	This capability indicates if the device supports different lighting patterns.

CapPattern is a logical OR combination of any of the following values:

Value		Meaning

LGT_PATTERN_NOPATTERN								Lighting patterns are not supported.

LGT_PATTERN_CUSTOM			                 					1~32 Supports 1st to 32th Lighting Pattern.

This property is initialized by the open method. If the device does not support lighting pattern, it is initialized to LGT_PATTERN_NOPATTERN.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	switchOnPattern Method.

MaxLights Property

Syntax	MaxLights: int32 {read-only, access after open}

Remarks	MaxLights specifies the maximum number of lights that the device can support.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

Methods (UML operations)

switchOff Method

Syntax	switchOff (lightNumber: int32): 
          void {raises-exception, use after open-claim-enable}

Parameter	Description

lightNumber	Specifies the light number. Valid light numbers are 1 through MaxLights.

Remarks	Switches off the light specified by lightNumber.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

A possible value of the exception’s ErrorCode property is:

Value	Meaning

E_ILLEGAL	The lightNumber parameter exceeds MaxLights.

See Also	MaxLights Property.

switchOffPattern Method

Syntax	switchOff Pattern ( ): 
          void {raises-exception, use after open-claim-enable}

Remarks	Switches off the pattern lighting.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

A possible value of the exception’s ErrorCode property is:

Value	Meaning

E_ILLEGAL	Pattern lighting is not executed.

See Also	switchOnPattern Method.

switchOn Method                               Updated in Release1.12

Syntax	switchOn (lightNumber: int32, blinkOnCycle: int32,
      blinkOffCycle: int32, color: int32, alarm: int32): 
          void {raises-exception, use after open-claim-enable}

Parameter	Description

lightNumber	Specifies the light number. Valid light numbers are 1 through MaxLights.

blinkOnCycle	A zero (0) value indicates no blink cycle. A positive value indicates the blink on cycle time in milliseconds. Negative values are not allowed.

blinkOffCycle	A zero (0) value indicates no blink cycle. A positive value indicates the blink off cycle time in milliseconds. Negative values are not allowed.

color	Specifies the color of the light, must be one of the colors defined by CapColor.

alarm	Specifies the used alarm type, must be one of the alarms defined by CapAlarm.

Remarks	Switches on the light specified by lightNumber or let it blink.

If blinkOnCycle and blinkOffCycle are zero (0) or CapBlink is false, then the parameters blinkOnCycle and blinkOffCycle will be ignored and the light will only be switched on.

If CapBlink is true and blinkOnCycle and blinkOffCycle are positive, then the light will blink.

If CapColor is LGT_COLOR_PRIMARY the light does not support different colors and color is ignored, otherwise switchOn will use the color specified by color.

If CapAlarm is LGT_ALARM_NOALARM the light does not support different alarms and alarm is ignored, otherwise switchOn will use the alarm specified by alarm.

Subsequent calls to switchOn will change the blink cycles, the color or the alarm type of the light.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

A possible value of the exception’s ErrorCode property is:

Value	Meaning

E_ILLEGAL	The lightNumber parameter exceeds MaxLights, an invalid color or alarm was specified.

See Also	CapAlarm Property, CapBlink Property, CapColor Property, MaxLights Property.

switchOnMultiple Method          			Added in Release 1.16

Syntax	switchOnMultiple (lightNumbers: string, blinkOnCycle: int32,
                   blinkOffCycle: int32, color: int32, alarm: int32): 
          void {raises-exception, use after open-claim-enable}

Parameter	Description

lightNumbers	Specifies the comma-delimited list of light number. Valid light numbers are 1 through MaxLights.

blinkOnCycle	A zero (0) value indicates no blink cycle. A positive value indicates the blink on cycle time in milliseconds. Negative values are not allowed.

blinkOffCycle	A zero (0) value indicates no blink cycle. A positive value indicates the blink off cycle time in milliseconds. Negative values are not allowed.

color	Specifies the color of the light, must be one of the colors defined by CapColor.

alarm	Specifies the used alarm type, must be one of the alarms defined by CapAlarm.

Remarks	This method does the same as switchOn but in a synchronized way such that all lights are switched on / blinking synchronously. Switches on the multiple lights specified by lightNumbers or let it blink.

If blinkOnCycle and blinkOffCycle are zero (0) or CapBlink is false, then the parameters blinkOnCycle and blinkOffCycle will be ignored and the light will only be switched on.

If CapBlink is true and blinkOnCycle and blinkOffCycle are positive, then the light will blink.

If CapColor is LGT_COLOR_PRIMARY the light does not support different colors and color is ignored, otherwise switchOnMultiple will use the color specified by color.

If CapAlarm is LGT_ALARM_NOALARM the light does not support different alarms and alarm is ignored, otherwise switchOnMultiple will use the alarm specified by alarm.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

A possible value of the exception’s ErrorCode property is:

Value	Meaning

E_ILLEGAL	The lightNumbers parameter exceeds MaxLights, an invalid value was specified.

See Also	CapAlarm Property, CapBlink Property, CapColor Property, MaxLights Property.

switchOnPattern Method           			Added in Release 1.16

Syntax	switchOnPattern (pattern: int32, alarm: int32): 
          void {raises-exception, use after open-claim-enable}

Parameter	Description

pattern	Specifies the lighting pattern, must be one of the patterns defined by CapPattern.

alarm	Specifies the used alarm type, must be one of the alarms defined by CapAlarm.

Remarks	Switches on the light specified by pattern.

If CapAlarm is LGT_ALARM_NOALARM the light does not support different alarms and alarm is ignored, otherwise switchOn and switchOnPattern will use the alarm specified by alarm.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

A possible value of the exception’s ErrorCode property is:

Value	Meaning

E_ILLEGAL	An invalid value was specified, or unsupported operation with the Device.

See Also	CapAlarm Property, CapPattern Property.

Events (UML interfaces)

DirectIOEvent

<< event >>	upos::events::DirectIOEvent
EventNumber	: int32 {read-only}
Data			: int32 {read-write}
Obj			: object{read-write}

Description 	Provides Service information directly to the application. This event provides a means for a vendor-specific Lights Service to provide events to the application that are not otherwise supported by the device control.

Attributes	This event contains the following attributes:

Attribute	Type	Description

EventNumber	int32	Event number whose specific values are assigned by the Service.

Data	int32	Additional numeric data. Specific values vary by the EventNumber and the Service. This property is settable.

Obj	Object	Additional data whose usage varies by the EventNumber and Service. This property is settable.

Remarks	This event is to be used only for those types of vendor specific functions that are not otherwise described. Use of this event may restrict the application program from being used with other vendor’s Lights devices which may not have any knowledge of the Service’s need for this event.

See Also	“Events" on page Intro-19, directIO Method.

StatusUpdateEvent

<< event >>	upos::events::StatusUpdateEvent
Status		: int32 {read-only}

Description 	Notifies the application that there is a change in the power status of a light.

Attributes	This event contains the following attribute:

Attribute	Type	Description

Status	int32	Reports a change in the power status of a light.

Note that Release 1.3 added Power State Reporting with additional Power reporting StatusUpdateEvent values.

The Update Firmware capability, added in Release 1.9, added additional Status values for communicating the status/progress of an asynchronous update firmware process.

See “StatusUpdateEvent” description on page 1-34.

Remarks	Enqueued when the light detects a power state change.

See Also	“Events" on page Intro-19.

Chapter 29

POS Power

This Chapter defines the POS Power device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.5 | Not supported |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.9 | open |
| CapPowerReporting: | int32 | {read-only} | 1.3 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.8 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.9 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.8 | open |
| CheckHealthText: | string | {read-only} | 1.5 | open |
| Claimed: | boolean | {read-only | 1.5 | open |
| DataCount: | int32 | {read-only} | 1.5 | Not supported |
| DataEventEnabled: | boolean | {read-write} | 1.5 | Not supported |
| DeviceEnabled: | boolean | {read-write} | 1.5 | open & claim |
| FreezeEvents: | boolean | {read-write} | 1.5 | open |
| OutputID: | int32 | {read-only} | 1.5 | Not supported |
| PowerNotify: | int32 | {read-write} | 1.5 | open |
| PowerState: | int32 | {read-only} | 1.5 | open |
| State: | int32 | {read-only} | 1.5 | -- |
| DeviceControlDescription: | string | {read-only} | 1.5 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.5 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.5 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.5 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.5 | open |
| PhysicalDeviceName: | string | {read-only} | 1.5 | open |

| Properties (Continued) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specific | Type | Mutability | Version | May Use After |
| CapBatteryCapacityRemaining: | boolean | {read-only} | 1.9 | open |
| CapBatteryCapacityRemainingInSeconds: | boolean | {read-only} | 1.16 | open |
| CapChargeTime: | boolean | {read-only} | 1.16 | open |
| CapFanAlarm: | boolean | {read-only} | 1.5 | open |
| CapHeatAlarm: | boolean | {read-only} | 1.5 | open |
| CapQuickCharge: | boolean | {read-only} | 1.5 | open |
| CapRestartPOS: | boolean | {read-only} | 1.9 | open |
| CapShutdownPOS: | boolean | {read-only} | 1.5 | open |
| CapStandbyPOS: | boolean | {read-only} | 1.9 | open |
| CapSuspendPOS: | boolean | {read-only} | 1.9 | open |
| CapUPSChargeState: | int32 | {read-only} | 1.5 | open |
| CapVariableBatteryCriticallyLowThreshold: | boolean | {read-only} | 1.9 | open |
| CapVariableBatteryCriticallyLowThresholdInSeconds: | boolean | {read-only} | 1.16 | open |
| CapVariableBatteryLowThreshold: | boolean | {read-only} | 1.9 | open |
| CapVariableBatteryLowThresholdInSeconds: | boolean | {read-only} | 1.16 | open |
| BatteryCapacityRemaining: | int32 | {read-only} | 1.9 | open |
| BatteryCapacityRemainingInSeconds: | int32 | {read-only} | 1.16 | open |
| BatteryCriticallyLowThreshold: | int32 | {read-write} | 1.9 | open |
| BatteryCriticallyLowThresholdInSeconds: | int32 | {read-write} | 1.16 | open |
| BatteryLowThreshold: | int32 | {read-write} | 1.9 | open |
| BatteryLowThresholdInSeconds: | int32 | {read-write} | 1.16 | open |
| ChargeTime: | int32 | {read-only} | 1.16 | open |
| EnforcedShutdownDelayTime: | int32 | {read-write} | 1.5 | open |
| PowerFailDelayTime: | int32 | {read-only} | 1.5 | open |
| PowerSource: | int32 | {read-only} | 1.9 | open |
| QuickChargeMode: | boolean | {read-only} | 1.5 | open |
| QuickChargeTime: | int32 | {read-only} | 1.5 | open |
| UPSChargeState: | int32 | {read-only} | 1.5 | open, claim & enable |

Goto Table7

| Methods (UML operations) |  |
| --- | --- |
| Common |  |
| Name | Version |
| open (logicalDeviceName: string): void {raises-exception} | 1.5 |
| close ( ): void {raises-exception, use after open} | 1.5 |
| claim (timeout: int32 ): void {raises-exception, use after open} | 1.5 |
| release ( ): void {raises-exception, use after open, claim} | 1.5 |
| checkHealth (level: int32): void {raises-exception, use after open, enable} | 1.5 |
| clearInput ( ): void { } | Not supported |
| clearInputProperties ( ): void { } | Not supported |
| clearOutput ( ): void { } | Not supported |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} | 1.5 |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, claim, enable} | 1.9 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | 1.8 |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | 1.8 |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, claim, enable} | 1.9 |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | 1.8 |
| Specific |  |
| Name |  |
| restartPOS ( ): void {raises-exception, use after open, enable} | 1.9 |
| shutdownPOS ( ): void {raises-exception, use after open, enable} | 1.5 |
| standbyPOS (reason: int32 ): void {raises-exception, use after open, enable} | 1.9 |
| suspendPOS (reason: int32 ): void {raises-exception, use after open, enable} | 1.9 |

| Events (UML interfaces) |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Mutability | Version |
| upos::events::DataEvent |  | Not supported |  |
| upos::events::DirectIOEvent |  |  | 1.5 |
| EventNumber: | int32 | {read-only} |  |
| Data: | int32 | {read-write} |  |
| Obj: | object | {read-write} |  |
| upos::events::ErrorEvent |  | Not supported |  |
| upos::events::OutputCompleteEvent |  | Not supported |  |
| upos::events::StatusUpdateEvent |  |  | 1.5 |
| Status: | int32 | {read-only} |  |
| upos::events::TransitionEvent |  | Not supported | 1.16 |

General Information

The POS Power programmatic name is “POSPower”.

Capabilities

The POSPower device class has the following capabilities:

- Supports a command to “shut down” the system.

- Supports a command to restart the system.

- Supports a command to “suspend” the system.

- Supports a command to have the system go to standby.

- Supports accessing a power handling mechanism of the underlying operating system and hardware.

- Informs the application if a power fail situation has occurred.

- Informs the application about battery level.

- Informs the application if the UPS charge state has changed.

- Informs the application about high CPU temperature.

- Informs the application about stopped CPU fan.

- Informs the application if an operating system dependent enforced shutdown mechanism is processed.

- Allows the application after saving application data locally or transferring application data to a server to shut down the POS terminal.

- Informs the application about an initiated shutdown.

Device Sharing

The POSPower is a sharable device. Its device sharing rules are:

- After opening and enabling the device, the application may access all properties and methods and will receive status update events.

- If more than one application has opened and enabled the device, all applications may access its properties and methods. Status update events are fired to all of the applications.

- If one application claims the POSPower, then only that application may call the shutdownPOS, standbyPOS, or suspendPOS methods. This feature provides a degree of security, such that these methods may effectively be restricted to the main POS application if that application claims the device at startup.

- See the “Summary” table for precise usage prerequisites.

Model

The general model of POSPower is based on the power model of each device in version 1.3 or later. The same common properties are used but all states relate to the POS terminal itself and not to a peripheral device.

There are three states of the POSPower:

- ONLINE. The POS terminal is powered on and ready for use. This is the “operational” state.

- OFF. The POS terminal is powered off or detached from the power supplying net. The POS terminal runs on battery power support. This is the powerfail situation.

- OFFLINE. The POS terminal is powered on but is running in a “lower-power-consumption” mode. It may need to be placed online by pressing a button or key or something else which may wake up the system.

Power reporting only occurs while the device is open, enabled and power notification is switched on.

In a powerfail situation - that means the POSPower is in the state OFF - the POS terminal will be shut down automatically after the last application has closed the POSPower device or the time specified by the EnforcedShutdownDelayTime property has been elapsed.

A call to the shutdownPOS method will always shut down the POS terminal independent of the system power state.

Version 1.9 or later

Support of battery powered devices is added. In addition to adding properties to report battery levels and power sources, properties are added to allow for the setting of low and critically low battery levels. The POSPower device also includes the ability to request or respond to request to enter the standby and suspend states. The model does not attempt to duplicate other power management models such as APM and ACPI, but leaves those implementation details to the provider. As a rule, the suspend state will consume less power than the standby state, which in turn will consume less power than the on state. A suggested mapping of these states to other power management models is:

| State | ACPI | APM | Description |
| --- | --- | --- | --- |
| On | S0 | ON | Active, Powered On |
| Standby | S1 | SUSPEND | Displays and drives off, CPU, RAM and fans powered on |
| Suspend | S3 | SUSPEND | Only RAM powered |
| Off | S5 | OFF | Completely powered off |

POSPower Class Diagram	　　　　Updated in Release 1.16

The following diagram shows the relationships between the POSPower classes.

Fig. Chap.29-1 POSPower Class Diagram

POSPower Sequence Diagram

The following sequence diagram shows the typical usage of the POSPower device for registering for StatusUpdateEvents and an atypical case of initiating a shutdownPOS call.

Fig. Chap. 29-2 POSPower Sequence Diagram

POSPower Standby Sequence Diagram

Fig. Chap. 29-3 POSPower Standby Sequence Diagram

POSPower State Diagram

The following state diagram depicts the POSPower Control device model.

Fig. Chap. 29-4 Power State Diagram (POSPower Control Device Model)

POSPower PowerState Diagram - Part 1

The following state diagram depicts the POSPower Power States.

Fig. Chap. 29-5 POSPower PowerState Diagram (Part 1)

POSPower PowerState Diagram - Part 2

The following state diagram depicts the POSPower PowerState ONLINE.

Fig. Chap. 29-6 POSPower PowerState Diagram (Part 2)

POSPower PowerState Diagram - Part 3

The following state diagram depicts the POSPower PowerState OFF.

Fig. Chap. 29-7 POSPower PowerState Diagram (Part 3)

POSPower State Chart Diagram for Fan and Temperature

The following state diagram depicts the handling of fan and temperature alarms.

Fig. Chap. 29-8 POSPower State Chart Diagram (Fan and Temperature)

POSPower Battery State Diagram

Fig. Chap. 29-9 POSPower Battery State Diagram

POSPower Power Transitions State Diagram

Fig. Chap. 29-10 POSPower Power Transitions State Diagram

Properties (UML attributes)

BatteryCapacityRemaining Property

Syntax	BatteryCapacityRemaining: int32 {read-only, access after open}

Remarks	A value of 0 to 100 represents percent of battery capacity remaining.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	CapBatteryCapacityRemaining Property

BatteryCapacityRemainingInSeconds Property	Added in Release 1.16

Syntax	BatteryCapacityRemainingInSeconds: int32 {read-only, access after open}

Remarks	A value of battery capacity remaining in seconds.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	CapBatteryCapacityRemainingInSeconds Property

Goto Table8

BatteryCriticallyLowThreshold Property

Syntax	BatteryCriticallyLowThreshold: int32 {read-write, access after open}

Remarks	If not zero, this property holds the threshold at which a PWR_SUE_BAT_CRITICAL StatusUpdateEvent is generated. The values 1 through 99 represent the percentage of the capacity remaining. The value 0 indicates that Battery Critically Low reporting is not supported or is disabled.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20. Some possible values of the exception’s ErrorCode property are:

Value		Meaning

E_ILLEGAL	An invalid value was specified. 						Or it does not support this function.

See Also	CapVariableBatteryCriticallyLowThreshold Property, StatusUpdateEvent

BatteryCriticallyLowThresholdInSeconds Property				Added in Release 1.16

Syntax	BatteryCriticallyLowThresholdInSeconds: int32 {read-write, access after open}

Remarks	If not zero, this property holds the threshold at which a PWR_SUE_BAT_CRITICAL StatusUpdateEvent is generated. The values of seconds of the capacity remaining. The value 0 indicates that Battery Critically Low reporting is not supported or is disabled.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further  information, see “Errors" on page Intro-20. Some possible values of the exception’s ErrorCode property are:

Value		Meaning

Goto Table9

E_ILLEGAL	An invalid value was specified. 						Or it does not support this function.

See Also	CapVariableBatteryCriticallyLowThresholdInSeconds Property, StatusUpdateEvent

BatteryLowThreshold Property

Syntax	BatteryLowThreshold: int32 {read-write, access after open}

Remarks	If not zero, this property holds the threshold at which a PWR_SUE_BAT_LOW StatusUpdateEvent is generated. The value 1 to 99 represents the percent capacity remaining. The value 0 indicates that battery low reporting is not supported or is disabled. If variable battery low threshold is supported, setting a value between 1 and 99 sets the threshold to that value. Setting a value of zero disables battery low reporting.

This property is initialized by the open method.

Goto Table10

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20. Some possible values of the exception’s ErrorCode property are:

Value		Meaning

E_ILLEGAL	An invalid value was specified. 						Or it does not support this function.

See Also	CapVariableBatteryLowThreshold Property, StatusUpdateEvent

BatteryLowThresholdInSeconds Property		Added in Release 1.16

Syntax	BatteryLowThresholdInSeconds: int32 {read-write, access after open}

Goto Table11

Remarks	If not zero, this property holds the threshold at which a PWR_SUE_BAT_LOW StatusUpdateEvent is generated. The value of seconds of the capacity remaining. The value 0 indicates that battery low reporting is not supported or is disabled. If variable battery low threshold is supported, setting a value of seconds sets the threshold to that value. Setting a value of zero disables battery low reporting. This property is initialized by the open method. Some possible values of the exception’s ErrorCode property are:

Value		Meaning

E_ILLEGAL	An invalid value was specified. 						Or it does not support this function.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	CapVariableBatteryLowThresholdInSeconds Property, StatusUpdateEvent

CapBatteryCapacityRemaining Property

Syntax	CapBatteryCapacityRemaining: boolean {read-only, access after open}

Remarks	If true, the device is able to provide battery capacity information. Otherwise, it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	BatteryCapacityRemaining Property

CapBatteryCapacityRemainingInSeconds Property 									Added in Release 1.16

Syntax	CapBatteryCapacityRemainingInSeconds								: boolean {read-only, access after open}

Remarks	If true, the device is able to provide battery capacity information seconds. Otherwise, it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	BatteryCapacityRemainingInSeconds Property

CapChargeTime Property 			    Added in Release 1.16

Syntax	CapChargeTime: boolean {read-only, access after open}

Remarks	If true, the device is able to acquire the remaining time until full charging. Otherwise, it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	ChargeTime Property.

CapFanAlarm Property

Syntax	CapFanAlarm: boolean {read-only, access after open}

Remarks	If true, the device is able to detect whether the CPU fan is stopped. Otherwise, it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

CapHeatAlarm Property

Syntax	CapHeatAlarm: boolean {read-only, access after open}

Remarks	If true the device is able to detect whether the CPU is running at too high of a temperature. Otherwise, it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

CapQuickCharge Property

Syntax	CapQuickCharge: boolean {read-only, access after open}

Remarks	If true, the power management allows the charging of the UPS battery in quick mode. The time for charging the battery is shorter than usual. Otherwise, it is false.

This property is initialized by the open method.

Errors		A UposException may be thrown when this property is accessed. For further   information, see “Errors" on page Intro-20.

See Also	QuickChargeMode Property, QuickChargeTime Property.

CapRestartPOS Property

Syntax	CapRestartPOS: boolean {read-only, access after open}

Remarks	If true the device is able to explicitly restart the POS. Otherwise, it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	restartPOS Method.

CapShutdownPOS Property

Syntax	CapShutdownPOS: boolean {read-only, access after open}

Remarks	If true the device is able to explicitly shut down the POS. Otherwise, it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	shutdownPOS Method.

CapStandbyPOS Property

Syntax	CapStandbyPOS: boolean {read-only, access after open}

Remarks	If true, the device is able to request that the POS System enter the Standby state. Otherwise, it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	standbyPOS Method.

CapSuspendPOS Property

Syntax	CapSuspendPOS: boolean {read-only, access after open}

Remarks	If true, the device is able to request that the POS System enter the Suspend state. Otherwise, it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	suspendPOS Method.

CapUPSChargeState Property

Syntax	CapUPSChargeState: int32 {read-only, access after open}

Remarks	If not equal to zero, the UPS can deliver one or more charge states. It can contain any of the following values logically ORed together.

Value	Meaning

PWR_UPS_FULL	UPS battery is near full charge.

PWR_UPS_WARNING	UPS battery is near 50% charge.

PWR_UPS_LOW	UPS battery is near empty. Application shutdown should be started to ensure that can be completed before the battery charge is depleted. A minimum of 2 minutes of normal system operation can be assumed when this state is entered unless this is the first state reported upon entering the “Off” power state.

PWR_UPS_CRITICAL	UPS battery is in a critical state and could be disconnected at any time without further warning.	This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	UPSChargeState Property.

CapVariableBatteryCriticallyLowThreshold Property

Syntax	CapVariableBatteryCriticallyLowThreshold:
			　　boolean {read-only, access after open}

Remarks	If true, the device supports a variable threshold for critically low battery. Otherwise, it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	BatteryCriticallyLowThreshold Property, StatusUpdateEvent

CapVariableBatteryCriticallyLowThresholdInSeconds Property   		　　　　　　　　　　　　　　　Added in Release 1.16

Syntax	CapVariableBatteryCriticallyLowThresholdInSeconds:
			　　boolean {read-only, access after open}

Remarks	If true, the device supports a second’s variable threshold for critically low battery. Otherwise, it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	BatteryCriticallyLowThresholdInSeconds Property, StatusUpdateEvent

CapVariableBatteryLowThreshold Property

Syntax	CapVariableBatteryLowThreshold: boolean {read-only, access after open}

Remarks	If true, the device supports a variable threshold for battery low. Otherwise, it is false. This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	BatteryLowThreshold Property, StatusUpdateEvent

CapVariableBatteryLowThresholdInSeconds Property
						　       Added in Release 1.16

Syntax	CapVariableBatteryLowThresholdInSeconds:
			　　boolean {read-only, access after open}

Remarks	If true, the device supports a second’s variable threshold for battery low. Otherwise, it is false. This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	BatteryLowThresholdInSeconds Property, StatusUpdateEvent

ChargeTime Property 			　　Added in Release 1.16

Syntax	ChargeTime: int32 {read-only, access after open}

Remarks	Indicates the time remaining until the battery is fully charged in seconds.

If equal to zero the battery is not charging or not supported.

This property is only set if CapChargeTime is true.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also 	CapChargeTime Property.

EnforcedShutdownDelayTime Property

Syntax	EnforcedShutdownDelayTime: int32 {read-write, access after open}

Goto Table12

Remarks	If not equal to zero the system has a built-in mechanism to shut down the POS terminal after a determined time in a power fail situation. This property contains the time in milliseconds when the system will shut down automatically after a power failure. A power failure is the situation when the POS terminal is powered off or detached from the power supplying net and runs on UPS. 
If zero no automatic shutdown is performed and the application has to call itself the shutdownPOS method.				　　　Applications will be informed about an initiated automatic shutdown.    This property is initialized by the open method. Some possible values of the exception’s ErrorCode property are:

Value		Meaning

E_ILLEGAL	An invalid value was specified. 						Or it does not support this function.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	shutdownPOS Method.

PowerFailDelayTime Property

Syntax	PowerFailDelayTime: int32 {read-only, access after open}

Remarks	This property contains the time in milliseconds for power fail intervals which will not create a power fail situation. In some countries the power has sometimes short intervals where the power supply is interrupted. Those short intervals are in the range of milliseconds up to a few seconds and are handled by batteries or other electric equipment and should not cause a power fail situation. The power fail interval starts when the POS terminal is powered off or detached from the power supplying net and runs on UPS. The power fail interval ends when the POS terminal is again powered on or attached to the power supplying net. However, if the power fail interval is longer than the time specified in the PowerFailDelayTime property a power fail situation is created.

Usually, this parameter is a configuration parameter of the underlying power management. So, the application can only read this property.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

PowerSource Property

Syntax	PowerSource: int32 {read-only, access after open}

Remarks	This property holds the current power source if power source reporting is available. A StatusUpdateEvent is generated each time this property is updated.

Value	Meaning

PWR_SOURCE_NA	Power source reporting is not available.

PWR_SOURCE_AC	The current power source is the AC line.

PWR_SOURCE_BATTERY	The current power source is a system battery. This value is only presented for systems that operate normally on battery.

PWR_SOURCE_BACKUP	The current power source is a backup source such as an UPS or backup battery.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	StatusUpdateEvent

QuickChargeMode Property

Syntax	QuickChargeMode: boolean {read-only, access after open}

Remarks	If true, the UPS battery is being recharged in a quick charge mode.
If false, it is being charged in a normal mode.

This property is only set if CapQuickCharge is true.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also 	CapQuickCharge Property, QuickChargeTime Property.

QuickChargeTime Property

Syntax	QuickChargeTime: int32 {read-only, access after open}

Remarks	This time specifies the remaining time for charging the UPS battery in quick charge mode. After the time has elapsed, the UPS battery charging mechanism of power management usually switches into normal mode.

This time is specified in milliseconds.

This property is only set if CapQuickCharge is true.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also 	CapQuickCharge Property, QuickChargeTime Property.

UPSChargeState Property

Syntax	UPSChargeState: int32 {read-only, access after open, enable}

Remarks	This property holds the actual UPS charge state.

It has one of the following values:

Value	Meaning

PWR_UPS_FULL	UPS battery is near full charge.

PWR_UPS_WARNING	UPS battery is near 50% charge.

PWR_UPS_LOW	UPS battery is near empty. Application shutdown should be started to ensure that is can be completed before the battery charge is depleted. A minimum of 2 minutes of normal system operation can be assumed when this state is entered unless this is the first state reported upon entering the “Off” power state.

PWR_UPS_CRITICAL	UPS battery is in a critical state and could be disconnected at any time without further warning.

This property is initialized and kept current while the device is enabled.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20

See Also	CapUPSChargeState Property.

Methods (UML operations)

restartPOS Method

Syntax	restartPOS ( ):void {raises-exception, use after open, enable}

Remarks	Call to restart the POS terminal. This method will always restart the system independent of the system power state.

If the POSPower is claimed, only the application which claimed the device is able to restart the POS terminal.

Applications will be informed about an initiated restart.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	This method is not supported (see the CapRestartPOS property)

See Also	CapRestartPOS Property

shutdownPOS Method

Syntax	shutdownPOS ( ):void {raises-exception, use after open, enable}

Remarks	Call to shut down the POS terminal. This method will always shut down the system independent of the system power state.

If the POSPower is claimed, only the application which claimed the device is able to shut down the POS terminal.

Applications will be informed about an initiated shutdown.

It is recommended that in a power fail situation an application has to call this method after saving all data and setting the application to a defined state.
If the EnforcedShutdownDelayTime property specifies a time greater than zero and the application did not call the shutdownPOS method within the time specified in EnforcedShutdownDelayTime, the system will be shut down automatically. This mechanism may be provided by an underlying operating system to prevent the battery from being emptied before the system is shut down.
This method is only supported if CapShutdownPOS is true.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	This method is not supported.		 	　　(See the CapShutdownPOS property)

See Also	CapShutdownPOS Property, EnforcedShutdownDelayTime Property.

standbyPOS Method

Syntax	standbyPOS (reason: int32): 
　　　　void {raises-exception, use after open, enable}

Remarks	Call to request that the system be placed into the Standby state or to respond to a request from the system, OS or other application that the system be put into Standby state.

The reason parameter indicates the reason the POS terminal should enter a standby state:

Value	Description

PWR_REASON_REQUEST	Call is to request that the system enter the standby state.

PWR_REASON_ALLOW	Call is a response to a standby Status Update Event and specifies that the request should be allowed.

PWR_REASON_DENY	Call is a response to a standby Status Update Event and specifies that the request should be denied.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	This method is not supported (see the CapStandbyPOS property)

See Also	CapStandbyPOS Property.

suspendPOS Method

Syntax	suspendPOS (reason: int32): 
void {raises-exception, use after open, enable}

Remarks	Call to request that the system be placed into the Suspend state or to respond to a request from the system, OS or other application that the system be put into Suspend state.

The reason parameter indicates the reason the POS terminal should enter a standby state:

Value	Description

PWR_REASON_REQUEST	Call is to request that the system enter the suspend state.

PWR_REASON_ALLOW	Call is a response to a suspend Status Update Event and specifies that the request should be allowed.

PWR_REASON_DENY	Call is a response to a suspend Status Update Event and specifies that the request should be denied.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	This method is not supported (see the CapSuspendPOS property)

See Also	CapSuspendPOS Property.

Events (UML Interfaces)

DirectIOEvent

<< event >>	upos::events::DirectIOEvent
EventNumber	: int32 {read-only}
Data			: int32 {read-write}
Obj			: object{read-write}

Description	Provides Service information directly to the application. This event provides a means for a vendor specific POSPower Service to provide events to the application that are not otherwise supported by the device control.

Attributes	This event contains the following attributes:

Attributes	Type	Description

EventNumber	int32	Event number whose specific values are assigned by the Service.

Data	int32	Additional numeric data. Specific values vary by the EventNumber and the Service. This property is settable.

Obj	object	Additional data whose usage varies by the EventNumber and Service. This property is settable.

Remarks	This event is to be used only for those types of vendor specific functions that are not otherwise described. Use of this event may restrict the application program from being used with other vendor’s POSPower devices which may not have any knowledge of the Service’s need for this event.

See Also	“Errors" on page Intro-20, directIO Method.

StatusUpdateEvent

<< event >>	upos::events::StatusUpdateEvent
Status		: int32 {read-only}

Description	Delivered when UPSChargeState changes or an alarm situation occurs.

Attributes	This event contains the following attribute:

Attributes		 Type		Description

Status		int32		See below.

The Status property contains the updated power status or alarm status.

Value            		Meaning

PWR_SUE_UPS_FULL	UPS battery is near full charge. Can be returned 				if CapUPSChargeState contains 						PWR_UPS_FULL.

PWR_SUE_UPS_WARNING

UPS battery is near 50% charge. Can be returned if CapUPSChargeState contains PWR_UPS_WARNING.

PWR_SUE_UPS_LOW	UPS battery is near empty. Application shutdown 				should be started to ensure that it can be  					completed before the battery charge is depleted. 				A minimum of 2 minutes of normal system 				operation can be assumed when this state is 				entered unless this is the first charge state 					reported upon entering the “Off” state. Can be  				returned if CapUPSChargeState 	contains  				PWR_UPS_LOW.

PWR_SUE_UPS_CRITICAL

UPS is in critical state and will in short time be 				disconnected. Can be returned if 						CapUPSChargeState contains 						PWR_UPS_CRITICAL.

PWR_SUE_FAN_STOPPED

The CPU fan is stopped. Can be returned if 					CapFanAlarm is true.

PWR_SUE_FAN_RUNNING

The CPU fan is running. Can be returned if 				CapFanAlarm is true.

PWR_SUE_TEMPERATURE_HIGH

The CPU is running on high temperature. Can be 					returned if CapHeatAlarm is true.

PWR_SUE_TEMPERATURE_OK

The CPU is running on normal temperature. Can				be returned if CapHeatAlarm is true.

PWR_SUE_SHUTDOWN											The system will shut down immediately.

PWR_SUE_BAT_LOW	The system remaining battery capacity is at or 					below the low battery threshold and the system is 					operating from the battery.

PWR_SUE_BAT_CRITICAL

The system remaining battery capacity is at or 					below the critically low battery threshold and the 					system is operating from the battery.

PWR_SUE_BAT_CAPACITY_REMAINING.

The BatteryCapacityRemaining property has been updated

PWR_SUE_BAT_CAPACITY_REMAINING_IN_SECONDS

The BatteryCapacityRemainingInSeconds property has been updated

PWR_SUE_RESTART	The system will restart immediately.

PWR_SUE_STANDBY	The system is requesting a transition to the     					Standby state

PWR_SUE_USER_STANDBY

The system is requesting a transition to the       Standby state, as a result of user input.

PWR_SUE_SUSPEND	The system is requesting a transition to the 			Suspend state.

PWR_SUE_USER_SUSPEND

The system is requesting a transition to the Suspend state, as a result of user input.

PWR_SUE_PWR_SOURCE

The PowerSource property has been updated.

Note that Release 1.3 added Power State Reporting with additional Power 	reporting StatusUpdateEvent values.

The Update Firmware capability, added in Release 1.9, added additional 	Status values for communicating the status/progress of an asynchronous update firmware process.  See “StatusUpdateEvent” description on page 1-34.

See Also		CapFanAlarm Property, CapHeatAlarm Property, 	CapUPSChargeState 	Property, UPSChargeState Property.

Chapter 39

Video Capture

This Chapter defines the Video Capture device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.16 | Not supported |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.16 | open |
| CapPowerReporting: | int32 | {read-only} | 1.16 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.16 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.16 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.16 | open |
| CheckHealthText: | string | {read-only} | 1.16 | open |
| Claimed: | boolean | {read-only} | 1.16 | open |
| DataCount: | int32 | {read-only} | 1.16 | Not supported |
| DataEventEnabled: | boolean | {read-write} | 1.16 | Not supported |
| DeviceEnabled: | boolean | {read-write} | 1.16 | open & claim |
| FreezeEvents: | boolean | {read-write} | 1.16 | open |
| OutputID: | int32 | {read-only} | 1.16 | Not supported |
| PowerNotify: | int32 | {read-write} | 1.16 | open |
| PowerState: | int32 | {read-only} | 1.16 | open |
| State: | int32 | {read-only} | 1.16 | -- |
| DeviceControlDescription: | string | {read-only} | 1.16 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.16 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.16 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.16 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.16 | open |
| PhysicalDeviceName: | string | {read-only} | 1.16 | open |

| Properties (Continued) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specific | Type | Mutability | Version | May Use After |
| CapAssociatedHardTotalsDevice: | string | {read-only} | 1.16 | open |
| CapAutoExposure: | boolean | {read-only} | 1.16 | open |
| CapAutoFocus: | boolean | {read-only} | 1.16 | open |
| CapAutoGain: | boolean | {read-only} | 1.16 | open |
| CapAutoWhiteBalance: | boolean | {read-only} | 1.16 | open |
| CapBrightness: | boolean | {read-only} | 1.16 | open |
| CapContrast: | boolean | {read-only} | 1.16 | open |
| CapExposure: | boolean | {read-only} | 1.16 | open |
| CapGain: | boolean | {read-only} | 1.16 | open |
| CapHorizontalFlip: | boolean | {read-only} | 1.16 | open |
| CapHue: | boolean | {read-only} | 1.16 | open |
| CapPhoto: | boolean | {read-only} | 1.16 | open |
| CapPhotoColorSpace: | boolean | {read-only} | 1.16 | open |
| CapPhotoFrameRate: | boolean | {read-only} | 1.16 | open |
| CapPhotoResolution: | boolean | {read-only} | 1.16 | open |
| CapPhotoType: | boolean | {read-only} | 1.16 | open |
| CapSaturation: | boolean | {read-only} | 1.16 | open |
| CapStorage: | int32 | {read-only} | 1.16 | open |
| CapVerticalFlip: | boolean | {read-only} | 1.16 | open |
| CapVideo: | boolean | {read-only} | 1.16 | open |
| CapVideoColorSpace: | boolean | {read-only} | 1.16 | open |
| CapVideoFrameRate: | boolean | {read-only} | 1.16 | open |
| CapVideoResolution: | boolean | {read-only} | 1.16 | open |
| CapVideoType: | boolean | {read-only} | 1.16 | open |
| AutoExposure: | boolean | {read-write} | 1.16 | open, claim & enable |
| AutoFocus: | boolean | {read-write} | 1.16 | open, claim & enable |
| AutoGain: | boolean | {read-write} | 1.16 | open, claim & enable |
| AutoWhiteBalance: | boolean | {read-write} | 1.16 | open, claim & enable |
| Brightness: | int32 | {read-write} | 1.16 | open, claim & enable |
| Contrast: | int32 | {read-write} | 1.16 | open, claim & enable |
| Exposure: | int32 | {read-write} | 1.16 | open, claim & enable |
| Gain: | int32 | {read-write} | 1.16 | open, claim & enable |
| HorizontalFlip: | boolean | {read-write} | 1.16 | open, claim & enable |
| Hue: | int32 | {read-write} | 1.16 | open, claim & enable |
| PhotoColorSpace: | string | {read-write} | 1.16 | open, claim & enable |
| PhotoColorSpaceList: | string | {read-only} | 1.16 | open |
| PhotoFrameRate: | int32 | {read-write} | 1.16 | open, claim & enable |
| PhotoMaxFrameRate: | int32 | {read-only} | 1.16 | open |
| PhotoResolution: | string | {read-write} | 1.16 | open, claim & enable |
| PhotoResolutionList: | string | {read-only} | 1.16 | open |
| PhotoType: | string | {read-write} | 1.16 | open, claim & enable |
| PhotoTypeList: | string | {read-only} | 1.16 | open |
| RemainingRecordingTimeInSec: | int32 | {read-only} | 1.16 | open, claim & enable |
| Saturation: | int32 | {read-write} | 1.16 | open, claim & enable |
| Storage: | int32 | {read-write} | 1.16 | open, claim & enable |
| VerticalFlip: | boolean | {read-write} | 1.16 | open, claim & enable |
| VideoCaptureMode: | int32 | {read-writeonly} | 1.16 | open, claim & enable |
| VideoColorSpace: | string | {read-write} | 1.16 | open, claim & enable |
| VideoColorSpaceList: | string | {read-only} | 1.16 | open |
| VideoFrameRate: | int32 | {read-write} | 1.16 | open, claim & enable |
| VideoMaxFrameRate: | int32 | {read-only} | 1.16 | open |
| VideoResolution: | string | {read-write} | 1.16 | open, claim & enable |
| VideoResolutionList: | string | {read-only} | 1.16 | open |
| VideoType: | string | {read-write} | 1.16 | open, claim & enable |
| VideoTypeList: | string | {read-only} | 1.16 | Goto Table13 open |

| Methods (UML operations) |  |
| --- | --- |
| Common |  |
| Name | Version |
| open (logicalDeviceName: string): void {raises-exception} | 1.16 |
| close ( ): void {raises-exception, use after open} | 1.16 |
| claim (timeout: int32): void {raises-exception, use after open} | 1.16 |
| release ( ): void {raises-exception, use after open, claim} | 1.16 |
| checkHealth (level: int32): void {raises-exception, use after open, claim, enable} | Goto Table14 1.16 |
| clearInput ( ): void {raises-exception, use after open, claim} | Goto Table15 1.16 |
| clearInputProperties ( ): void { } | Not supported |
| clearOutput ( ): void { } | Not supported |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} | 1.16 |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, claim, enable} | Goto Table16 1.16 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | Goto Table17 1.16 |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | Goto Table18 1.16 |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, claim, enable} | Goto Table19 1.16 |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | Goto Table20 1.16 |
| Specific |  |
| Name |  |
| startVideo (fileName: string, overwrite: boolean, recordingTime: int32): void {raises-exception, use after open, claim, enable} | 1.16 |
| stopVideo (): void {raises-exception, use after open, claim, enable} | 1.16 |
| takePhoto (fileName: string, overwrite: boolean, timeout:int32): void {raises-exception, use after open, claim, enable} | 1.16 |

| Events (UML interfaces) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | Type |  | Mutability |  | Version |  |
| upos::events::DataEvent |  |  |  | Not supported |  |  |  |
| Status: |  |  |  |  |  |  |  |
| upos::events::DirectIOEvent |  |  |  |  |  | 1.16 |  |
| EventNumber: |  | int32 |  | {read-only} |  |  |  |
| Data: |  | int32 |  | {read-write} |  |  |  |
| Obj: |  | object |  | {read-write} |  |  |  |
| upos::events::ErrorEvent |  |  |  |  |  | 1.16 |  |
| ErrorCode: |  | int32 |  | {read-only} |  |  |  |
| ErrorCodeExtended: |  | int32 |  | {read-only} |  |  |  |
| ErrorLocus: |  | int32 |  | {read-only} |  |  |  |
| ErrorResponse |  | int32 |  | {read-write} |  |  |  |
| upos::events::OutputCompleteEvent |  |  |  | Not supported |  |  |  |
| upos::events::StatusUpdateEvent |  |  |  |  |  | 1.16 |  |
| Status: |  | int32 |  | {read-only} |  |  |  |
| upos::events::TransitionEvent |  |  |  | Not supported |  | 1.16 |  |

General Information

The Video Capture Device name is “Video Capture”.

Capabilities

Video capture device class has the following capabilities:

- Take a photo and record it as a file in a host and may store it in the targeted storage device.

- Take a video and record it as a file in a host and may store it in the targeted storage device.

- May read the encoded data from the bar code label with the hydra connected scanner device.

- May detect the individuals faces and/or objects with the hydra connected individual recognition device.

Video Capture Class Diagram

The following diagram shows the relationships between the Video Capture classes.

Fig. Chap. 39-1 Video Capture Class Diagram

Model

Modes

The Video Capture Device has two operation modes.

・Photo Mode

・Video Mode

The operation of each mode is as follows.

・Photo Mode

Photo Mode may capture a photo image and may save it in a host as the image data file format, if CapPhoto property is true. Its’ capable data file format is indicated in the PhotoType property and all of the capable values are listed in the PhotoTypeList property. And the device may save the file in the targeted storage device that is specified by the Storage property, if CapStorage value is VCAP_CST_HARDTOTALS_ONLY or VCAP_CST_ALL.

・Video Mode

Video Mode may capture a video image data and may save it in a host as the video image data file format, if CapVideo property is true. Its’ capable data file format is indicated in the VideoType property and all of the capable values are listed in the VideoTypeList property. And the device may save the file in the targeted storage device that is specified by the Storage property, if CapStorage value is VCAP_CST_HARDTOTALS_ONLY or VCAP_CST_ALL.

Device behaviors

“Video capture device” device control follows the device behavior as follows.	　 They are different in each mode as described below.

Photo Mode

If CapPhoto property is true, this mode can be executed. 				            Prior to start this mode, “Video Capture Device” device control needs to set the VideoCaptureMode property as to be VCAP_VCMODE_PHOTO.  And each of CapPhotoColorSpace, CapPhotoFrameRate, CapPhotoResolution, CapPhotoType property is true and these PhotoColorSpaceList, PhotoMaxFrameRate, PhotoResolutionList and PhotoTypeList should have the appropriate values to be used as the photo file data in this targeted device.  And then it needs to set the appropriate values in the each of PhotoColorSpace property, PhotoFrameRate property, PhotoResolution property and PhotoType property.

Goto Table21

It starts photo capturing by executing the takePhoto method. Then, “Video Capture Device” device control may capture a photo image and may save it in a host as an image data file format specified by the value of PhotoType property that is listed in the PhotoTypeList property. And may store it in the storage device specified by the Storage property, if CapStorage value is VCAP_CST_HARDTTOTALS_ONLY or VCAP_CST_ALL. Then the file name is set by the takePhoto method parameter and can deliver the photo data file to the application. 　If device needs to be able to write the image data file to an associated Hard Totals device, the CapAssociatedHardTotalsDevice property holds the open name of the associated Hard Totals device.

Goto Table22

This method is perfomedexecuted synchronously as the process of taking photo. The process of recorded data storing is performed asynchronously.  StatusUpdateEvents are invoked for the application when the start and the end of device states are changed. Only one call to takePhoto method can be in progress at a time. If you try An attempt to nest the video capture device operations of the device, before the storing is finished, will result in an UPOSException will being thrown.

When it exceeded the specified parameter time out or when photo file generation is finished or when clearInput method is executed, the taking photo process will be ended.

StatusUpdateEvent with status VCAP_SUE_START_PHOTO is evoked when takePhoto method is executed to notify the application that recording state has started.

When the taking photo is finished, or the specified time out has been exceeded, a StatusUpdateEvent with status VCAP_SUE_END_PHOTO is evoked to notify the application that photo taking has been ended.

An ErrorEvent event (or events) is enqueued if an error occurs while gathering or processing input.

Goto Table23

If ErrorEvent response is ER_RETRYCONTINUEINPUT, the process of recorded data storing was retried input processing continues. However, as long as the cause of the error is not resolved, the ErrorEvent will occur again immediately.

Goto Table24

If ErrorEvent is ER_CLEAR, all of the device data is cleared the input processing process is terminated and the takePhoto method taking photo is discarded.

All enqueued input may be deleted by calling clearInput method. See the clearInput method description for more details.

Video Mode

Prior to start this mode, “Video Capture Device” device control needs to set the VideoCaptureMode property as to be VCAP_VCMODE_VIDEO.  And each of CapVideoColorSpace, CapVideoFrameRate, CapVideoResolution and CapVideoType property is true and these VideoColorSpaceList, VideoMaxFrameRate, VideoResolutionList and VideoTypeList should have the appropriate values to be used as the video image data file in this targeted device. And then it needs to set the appropriate values in the each of VideoColorSpace property, VideoFrameRate property, VideoResolution property and VideoType property.

Goto Table25

It starts video image capturing by executing the startVideo method. This method is executed synchronously. During video image capturing, recorded data storing is processed asynchronously and when the start and end the device state is changed, StatusUpdateEvents are invoked for the application. In addition, remaining device recording time is updated in the RemainingRecordingTimeInSec property.

Goto Table26

Then “Video Capture Device” device control captures a video image and save it in a host with the filename specified value of VideoType property that is listed in the VideoTypeList property. And may store it in the storage device specified by the Storage property, if CapStorage value is VCAP_CST_HARDTTOTALS_ONLY or VCAP_CST_ALL. And the file name is set by the startVideo method parameter and can deliver the video image data file to the application. This method is executed synchronously.

Goto Table27

The video capturing ends after the specified time has elapsed or when stopVideo method is called or when clearInput method is called even startVideo method is called.

The remaining video capture recording time in seconds can be obtained from the property RemainingRecordingTimeInSec.

Goto Table146

StatusUpdateEvent with status VCAP_SUE_START_VIDEO is evoked when startVideo method is executed to notify the application that taking video has been started.

When the taking video is finished, or the specified time out has been exceeded, a StatusUpdateEvent with status VCAP_SUE_STOPEND_VIDEO is evoked to notify the application that taking video has been ended.

If the time specified by the startVideo method is FOREVER(-1), execution will continue until the stopVideo method is called. When stopVideo is called, the previous taking video data may be recorded in a host and deliver to the targeted storage device specified by the Storage property, if CapStorage property value is VCAP_CST_HARDTTOTALS_ONLY or VCAP_CST_ALL. And it can be delivered to the application with the specified file name that is set by the startVideo method.

Goto Table28

Only one call to startVideo method can be in progress at a time. An attempt to nest taking video operations will result in an UPOSException being thrown.

If Error occurs during the execution of the startVideo method, application may call the stopVideo method to terminate the taking video process or cancel the taking video process by calling the clearInput method before ending the ErrorEvent processing. After this when the stopVideo method is called, the video file data until just before the ErrorEvent occur is stored to the host and targeted storage device that is specified by the Storage property, if CapStorage property value is VCAP_CST_HARDTTOTALS_ONLY or VCAP_CST_ALL, and can be delivered to the application.

Goto Table29

Goto Table30

If ErrorEvent response is ER_RETRYCONTINUEINPUT, the process of recorded data storing was retried input processing continues. However, as long as the cause of the error is not resolved, the ErrorEvent will occur again immediately.

Goto Table31

If ErrorEvent is ER_CLEAR, all of the device buffered data is cleared and the error state is exited input processing is terminated and the taking video capturing process is discarded.

An ErrorEvent event (or events) is enqueued if an error occurs while gathering or processing the data input.

Goto Table32

Goto Table33

If there is no error during the execution of startVideo method, it is possible to terminate the taking video process and can stop the taking video anytime. When the stopVideo method is called, the video data until just before the method is called, may be recorded in the host and targeted storage device that is specified by the Storage property if CapStorage property is VCAP_CST_HARDTTOTAL_ONLY or VCAP_CST_ALL, and can deliver it to the application.

All enqueued data input may be deleted by calling clearInput method. See the clearInput method description for more details.

Device Sharing

Video capture is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before accessing many video         capture-specific properties.

- The application must claim and enable the device before calling methods that      manipulate the device.

- See the “Summary” table for precise usage prerequisites.

Properties (UML attributes)

AutoExposure Property

Syntax	AutoExposure: boolean {read-write, access after open, claim, enable}

Goto Table34

Remarks	If true, auto exposure of camera is enabled. Otherwise, it is false. 			This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20. 				Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified. 						Or it does not support this function.

See also	CapAutoExposure Property

AutoFocus Property

Syntax	AutoFocus: boolean {read-write, access after open, claim enable}

Goto Table35

Remarks	If true, auto focus of camera is enabled. Otherwise, it is false.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20. 				Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified. 							Or it does not support this function.

See also	CapAutoFocus Property

Goto Table36

AutoGain Property

Syntax	AutoGain: boolean {read-write, access after open, claim, enable}

Remarks	If true, auto gain of camera is enabled. Otherwise, it is false.

When this property is true, it is possible to read the value of Gain property. However, it is not possible to write and change the value of Gain property. If AutoGain property is false, then, it is possible to read, write and change the value of Gain property.  					     This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed.			For further information, see “Errors" on page Intro-20. 				Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified. 							Or it does not support this function.

See also	CapAutoGain Property	Gain Property

AutoWhiteBalance Property

Goto Table37

Syntax          AutoWhiteBalance: boolean {read-write, access after open, claim,                                     enable}

Remarks	If true, auto white balance of camera is enabled. Otherwise, it is false.	  	This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20. 				Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified. 							Or it does not support this function.

See also	CapAutoWhiteBalance Property

Brightness property

Syntax		Brightness: int32 {read-write, access after open, claim, enable }

Goto Table38

Remarks	Indicate the brightness of camera. 							Valid values range from 0 to 100.								This property is initialized by the open method.

Errors		A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20. 					Some possible values of the exception’s ErrorCode property are:

Value			Meaning                                                         			E_ILLEGAL		An invalid value was specified. 								Or it does not support this function.

See Also	CapBrightness Property

Goto Table39

CapAssociatedHardTotalsDevice Property

Syntax	CapAssociatedHardTotalsDevice: string {read-onlywrite, access after open}

Remarks	Indicate that the device is able to store the recorded data into the Associated Hard Totals device and Hholds the its open name of the associated Hard Totals Device, if the device is able to write to such devices which is the case, if CapStorage is either VCAP_CST_ALL or VCAP_CST_HARDTOTALS_ONLY. If CapStorage is VCAP_CST_HOST_ONLY the device is not able to store the data into the Associated Hard Totals device and this property value must be the empty string.  This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	CapStorage Property

CapAutoExposure Property

Syntax	       	CapAutoExposure: boolean {read-only, access after open}

Remarks	       	If true, the auto exposure of camera can be changed.				Otherwise, it is false. This property is initialized by the open method.

Errors           	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	        	AutoExposure Property

CapAutoFocus Property

Syntax	CapAutoFocus: boolean {read-only, access after open}

Remarks	If true, can change the auto focus of camera.  Otherwise, it is false.			This property is initialized by the open method.

Errors    	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	AutoFocus Property

CapAutoGain Property

Syntax	CapAutoGain: boolean {read-only, access after open}

Remarks	If true, automatic gain change of the camera is possible.  Otherwise, it is 		false. This property is initialized by the open method.

Errors   	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	AutoGain Property

CapAutoWhiteBalance Property

Syntax	CapAutoWhiteBalance: boolean {read-only, access after open}

Remarks	If true, auto white balance of camera is possible.  Otherwise, it is false.  		This property is initialized by the open method.

Errors   	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	AutoWhiteBalance Property

CapBrightness Property

Syntax	CapBrightness: boolean {read-only, access after open}

Remarks	If true, the brightness of camera can be changed.  Otherwise, it is false. 		This property is initialized by the open method.

Errors    	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	Brightness Property

CapContrast Property

Syntax	CapContrast: boolean {read-only, access after open}

Remarks	If true, can change the contrast of camera. Otherwise, it if false. 		      	This property is initialized by the open method.

Errors     	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	Contrast Property

CapExposure Property

Syntax	CapExposure: boolean {read-only, access after open}

Remarks	If true, can change the exposure of camera. Otherwise, it is false. 			This property is initialized by the open method.

Errors    	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	Exposure Property

CapGain Property

Syntax	CapGain: boolean {read-only, access after open}

Remarks	If true, can change the gain of camera.  Otherwise, it is false. 			This property is initialized by the open method.

Errors    	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	Gain Property

CapHorizontalFlip Property

Syntax	 CapHorizontalFlip: boolean {read-only, access after open}

Remarks	If true, can change the horizontal flip of camera. Otherwise, it is false. 		This property is initialized by the open method.

Errors    	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	HorizontalFlip Property

CapHue Property

Syntax	CapHue: boolean {read-only, access after open}

Remarks	If true, the hue of the camera can be changed. Otherwise, it is false. 			This property is initialized by the open method.

Errors 	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	Hue Property

CapPhoto Property

Syntax	CapPhoto: boolean {read-only, access after open}

Remarks	If true, it supports the photo function and can take a photo. And to activate the photo mode, the VideoCaptureMode property value needs to set VCAP_VCMODE_PHOTO.  If false, it is not supporting the photo function. This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	takePhoto Method, VideoCaptureMode Property

CapPhotoColorSpace Property

Syntax	CapPhotoColorSpace: boolean {read-only, access after open}

Remarks	If true, can handle and change the photo color space. Otherwise, it is false. 		This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20

See also	PhotoColorSpace Property

CapPhotoFrameRate Property

Syntax	CapPhotoFrameRate: boolean {read-only, access after open}

Remarks	If true, can handle and change the capture frame rate. Otherwise, it is false. 
		This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	PhotoFrameRate Property

CapPhotoResolution Property

Syntax	CapPhotoResolution: boolean {read-only, access after open}

Remarks	If true, taking photo resolution is handled and can be changed. 			Otherwise, it is false. This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	PhotoResolution Property

Goto Table40

CapPhotoType Property

Syntax	CapPhotoType: boolean {read-only, access after open}

Remarks	If true, photo image format type can be changed. Otherwise, it is 				false. This property is initialized by the open method.

Errors		A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	PhotoType Property

CapSaturation Property

Syntax	CapSaturation: boolean {read-only, access after open}

Remarks	If true, can change the saturation of camera. Otherwise, it is false. 			This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	Saturation Property

CapStorage Property

Goto Table41

Syntax	CapStorage: int32 {read-only, access after open}

Remarks	This is an enumeration and announces where the device is able to write the recorded video or photo sound data file to.
It holds one of the following values.

Value　　　　　　　　　　　	Meaning

VCAP_CST_HARDTOTALS_ONLY　　				　Only an associate Hard Totals device is supported.

VCAP_CST_HOST_ONLY	Only the host’s file system is supported.

VCAP_CST_ALL	Both, the associated Hard Totals device and the host’s file system is supported.

This property is initialized by the open method.

If a Hard Totals device is supported the Storage, the property value should be VCAP_CST_HARDTOTALS_ONLY or VCAP_CST_ALL, and the property CapAssociatedHardTotalsDevice holds the open name of the associated Hard Totals device.

Errors		UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	Storage Property, CapAssociatedHardTotalsDevice Property

CapVerticalFlip Property

Syntax	      CapCameraVerticalFlip: boolean {read-only, access after open}

Remarks       If true, can change the vertical flip of camera. Otherwise, it is false. 		       This property is initialized by the open method.

Errors         A UposException may be thrown when this property is accessed. 		       For further information, see “Errors" on page Intro-20.

See also      VerticalFlip Property

CapVideo Property

Syntax	       CapVideo: boolean {read-only, access after open}

Remarks         If true, video function is supported. Otherwise, it is false. If this property is true, taking video and recording can be done by calling the startVideo method. And to activate the video mode, the VideoCaptureMode property value needs to set VCAP_VCMODE_VIDEO. If false, taking video and recording cannot be performed. This property is initialized by the open method.

Errors		    A UposException may be thrown when this property is accessed. 		   For further information, see “Errors" on page Intro-20.

See also	         StartVideo Method, VideoCaptureMode Property

CapVideoColorSpace Property

Syntax	CapVideoColorSpace: boolean {read-only, access after open}

Remarks	If true, can change the color space when taking the video. Otherwise, it is 		false. This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 		For further information, see “Errors" on page Intro-20

See also	VideoColorSpace Property

CapVideoFrameRate Property

Syntax		CapVideoFrameRate: boolean {read-only, access after open}

Remarks	If true, can change the video frame rate from 1 to up to VideoMaxFrameRate property value. Otherwise, it is false. 
This property is initialized by the open method.

Errors		A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See also		VideoMaxFrameRate Property, VideoFrameRate Property

CapVideoResolution Property

Syntax		CapVideoResolution: boolean {read-only, access after open}

Remarks	If true, taking video resolution can be changed and all of possible values are listed in the VideoResolutionList property values. If false, taking video resolution cannot be changed. This property is initialized by the open method.

Errors	    A UposException may be thrown when this property is accessed. 		For further information, see “Errors" on page Intro-20.

See also		VideoResolutionList Property, VideoResolution Property

CapVideoType Property

Syntax		CapVideoType: boolean {read-only, access after open}

Remarks 	If true, taking video type can be changed, and all of possible values are      listed in the VideoTypeList values. Otherwise, it is false.
This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed.
For further information, see “Errors" on page Intro-20.

See also		VideoTypeList Property, VideoType Property

Goto Table42

Contrast Property

Syntax		Contrast: int32 {read-write, access after open, claim, enable}

Remarks	Indicate the contrast of the camera. Valid values range from 0 to 100.			This property is initialized by the open method.

Errors		A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20. 					Some possible values of the exception’s ErrorCode property are:

Value			Meaning                                                         			E_ILLEGAL		An invalid value was specified. 								Or it does not support this function.

See Also	CapContrast Property

Goto Table43

Exposure Property

Syntax	Exposure: int32 {read-write, access after open, claim, enable}

Remarks	Indicate the exposure of camera. Valid values range from 0 to 100. 				This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20. 					Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified.                          					Or it does not support this function.

See also	CapExposure Property

Goto Table44

Gain Property

Syntax	Gain: int32 {read-write, access after open, claim, enable}

Remarks	Indicate the gain of camera. Valid values range from 0 to 100.                      If AutoGain property is true, it is possible to read the value of Gain property. However, it is not possible to write and change the value of Gain property.  If AutoGain property is false, then, it is possible to read, write and change the value of Gain property.                                                                      This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20. 				Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified. 							Or it does not support this function.

See also 	CapGain Property, AutoGain Property

HorizontalFlip Property

Syntax	HorizontalFlip: boolean {read-write, access after open, claim, enable}

Goto Table45

Remarks	If true, horizontal flip of camera is enabled and it is possible to reverse the camera captured image horizontally.  Otherwise, it is false. There is a similar property called VerticalFlip property. However, each VerticalFlip property and HorizontalFlip property value can be set independently.  This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20. 					Some possible values of the exception’s ErrorCode property are:				Value		Meaning

E_ILLEGAL	An invalid value was specified. 			     			Or it does not support this function.

See Also	CapHorizontalFlip property, VerticalFlip property,
			CapVerticalFlip property

Goto Table46

Hue Property

Syntax	Hue: int32 {read-write, access after open, claim, enable}

Remarks	Indicate the hue of camera. Valid values range from 0 to 100.				This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20. 					Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified.                                 				Or it does not support this function.

See also	CapHue Property

PhotoColorSpace Property

Syntax		PhotoColorSpace: string {read-write, access after open, claim, enable}

Goto Table47

Remarks	Indicates the photo color space ID of the frame data to be acquired by the Video Capture Device, if CapPhotoColorSpace property is true and it is used takePhoto method.  Valid values are one of the values listed in the CapPhotoColorSpaceList property.  This property is referred to when VideoCaptureMode property value is VCAP_VCMODE_PHOTO and CapPhoto is true. This property is initialized by the open method.

Errors		A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20. 					Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified.

See also            PhotoColorSpaceList Property, VideoCaptureMode property,

CapPhoto Property, CapPhotoColorSpace Property, takePhoto Method

PhotoColorSpaceList Property

Syntax	PhotoColorSpaceList: string {read-only, access after open}

Goto Table48

Remarks	Photo Color space information supported by the device is indicated in a		comma-separated list. Each color space information is composed 			of the following information and is shown in the following order 			separated by a colon (":").							This property is initialized by the open method.

Parameter		Description

Color space ID		ID for identifying the color space of RGB, YUV 	422, etc. Then if RGB Depth was 16 bits,                            YUV422 Depth was 32 bits, And they are indicating like 				“RGB:16, YUV422:32,…..”

Depth			Number of bits per 1 pixel

Errors		A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also    CapPhotoColorSpace Property, PhotoColorSpace Property, VideoCaptureMode Property

PhotoFrameRate Property

Syntax	PhotoFrameRate: int32 {read-write, access after open, claim, enable}

Goto Table49

Remarks 	Indicates the frame rate of frame data recorded to be acquried by the Video Capture Device and the photo image capturing and recorded with the takePhoto method.  This property is only applied when VCAP_VCMODE_PHOTO is set in VideoCaptureMode property. Valid values range from 1 to PhotoMaxFrameRate property and CapPhoto property is true. This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

Value			Meaning

E_ILLEGAL		An invalid value was specified.

See also	CapPhoto Property, CapPhotoFrameRate Property, PhotoMaxFrameRate Property,  VideoCaptureMode Property, takePhoto Method

PhotoMaxFrameRate Property

Syntax	PhotoMaxFrameRate: int32 {read-only, access after open}

Remarks	Indicates the maximum frame rate that can be set for the 				PhotoFrameRate property.							This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 			For further Information, see “Errors" on page Intro-20.

See also	PhotoFrameRate Property, VideoCaptureMode Property

PhotoResolution Property

Syntax		PhotoResolution: string {read-write, access after open, claim, enable}

Goto Table50

Remarks	It shows the resolution of the frame data acquired by the Video Capture Device and the photo taken and recorded with the takePhoto method. Valid values are one of those listed in PhotoResolutionList property.    This property is only applied when VCAP_VCMODE_PHOTO is set in VideoCaptureMode property and if CapPhoto is true. This property is initialized by the open method.

Errors		A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20. 					Some possible values of the exception’s ErrorCode property are:

Value			Meaning                                                     				E_ILLEGAL		An invalid value was specified.

See also	CapPhoto Property, PhotoResolutionList Property、VideoCaptureMode Property,   takePhoto Method

PhotoResolutionList Property

Syntax		PhotoResolutionList: string {read-only, access after open}

Goto Table51

Remarks	Indicating the comma-separated list of possible resolutions for the PhotoResolution property. Resolution is indicated in "horizontal x height" format. For example, when you support 320x240, 640x480, 640x360, it is the following: "320x240,640x480,640x360".
This property is initialized by the open method.

Errors		A UposException may be thrown when this property is accessed. 			              For further information, see “Errors" on page Intro-20.

See also              CapPhotoResolution Property, PhotoResolution Property, VideoCaptureMode property

PhotoType Property

Syntax		PhotoType:  string {read-write, access after open, claim, enable}

Goto Table52

Remarks	Indicates the data format of photo taken with the takePhoto method. Valid values are one of the values listed in the CapPhotoTypeList property. 
This property is initialized by the open method.

Errors		A UposException may be thrown when this property is accessed.
			For further information, see “Errors" on page Intro-20.
			Some possible values of the exception’s ErrorCode property are:

Value			Meaning						E_ILLEGAL		An invalid value was specified.

See also	CapPhoto Property, CapPhotoTypeList Property, takePhoto Method, VideoCaptureMode Property

PhotoTypeList Property

Syntax		PhotoTypeList: string {read-only, access after open}

Remarks	A comma-separated list of photo image format values that can be set for 			the PhotoType property.									For example, when supporting BMP and JPEG, it is the following. 				"BMP,JPEG".

Note: The notation contents may be different depending on the device.		This property is initialized by the open method.

Errors			A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	PhotoType Property, VideoCaptureMode property

RemainingRecordingTimeInSec Property

Syntax	RemainingRceordingTimeInSec: 				                                      int32 {read-only, access after open-claim-enable}

Remarks	This property holds the remaining recording time in seconds if a video recording is ongoing. If no video recording is ongoing its value is 0. When a call to method startVideo returns, this property initially holds the time passed as argument recordingTime to that call. If this argument value is FOREVER   (-1), this property also holds this value unchanged until stopVideo method has been called. This property is initialized during device set DeviceEnabled method to 0.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	startVideo Method, stopVideo Method

Saturation Property

Syntax	Saturation: int32 {read-write, access after open, claim, enable }

Goto Table53

Remarks	Indicate the saturation of camera. 	Valid values range from 0 to 100. 			This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20. 					Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified. 						Or it does not support this function.

See also	CapSaturation Property

Storage Property

Syntax	Storage: int32 {read-write, access after open-claim-enable}

Remarks	This is an enumeration and defines where the device writes the recorded video or photo data file to. Should be set before a call to startVideo or takePhoto method. It holds one of the following values.

Value                                 Meaning			                           		      VCAP_ST_HARDTOTALS		   			                                                                                                                      				         The video or photo data file is written to the 					         associated Hard Totals device. The property 					         CapAssociatedHardTotalsDevice holds the 					         open name of the associated Hard Totals device.

VCAP_ST_HOST              The vide or photo data file is written to the host’s  file system.

VCAP_ST_HOST_HARDTOTALS

The video or photo data file is written to the associated Hard Totals device and host’s file system. The property CapAssociatedHardTotalsDevice holds the open name of the associated Hard Totals device.

This property is initialized by the open method according to the value hold by CapStorage. If CapStorage has the value VCAP_CST_ALL, it is initialized to VCAP_ST_HOST_HARDTOTALS.

Errors	        UposException may be thrown when this property is accessed.
	        For further information, see “Errors" on page Intro-20. Some possible values                                                                                            	        of the exception’s ErrorCode property are:

Goto Table54

Value		Meaning

E_ILLEGAL	An invalid value was specified or recording is ongoing.

See Also	CapStorage Property

VerticalFlip Property

Syntax		VerticalFlip: boolean {read-write, access after open, claim, enable}

Goto Table55

Remarks　	If true, vertical flipping of the video is enabled and it is possible to reverse the video or photo image capturing vertically. Otherwise, it is false.  There is a similar property called HorizontalFlip property and each  VerticalFlip property and HorizontalFlip property value can be set independently. This property is initialized by the open method.

Errors	    	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20. 				Some possible values of the exception’s ErrorCode property are:

Value		Meaning

E_ILLEGAL	An invalid value was specified. 							Or it does not support this function.

See also	     	CapVerticalFlip Property, HorizontalFlip Property,
              	CapHorizontalFlip Property

VideoCaptureMode Property

Syntax		VideoCaptureMode: int32 {read-write, access after open}

Remarks	Indicate the operation mode of video capture device.
			Valid values are as follows

Parameter	Description

VCAP_VCMODE_PHOTO								This mode is for taking photo. and their data 				recording. Can be set when CapPhoto property is true.
			The values of the PhotoType property, 					PhotoColorSpace property, PhotoResolution property 			PhotoFrameRate property are applied to the taking 			photo image formats list in the PhotoTypeList property, 			the color space values list in the PhotoColorSpaceList 			property, the resolution values list in the 					PhotoResolutionList property, and the frame rate values 			within the values of PhotoMaxFrameRate property.  			And taking photo is executed by the takePhoto method.  VCAP_VCMODE_VIDEO

This mode is for taking the videos and their data 			recording. Can be set when CapVideo property is true. 		The value of the VideoType property, VideoColorSpace 		property, VideoResolution property and 				VideoFrameRate property are applied to the taking 		video image format list in the VideoTypeList property, 		the color space values list in the VideoColorSpaceList 		property, the resolution values list in the 				VideoResolutionList property and frame rate values 		within the values of VideoMaxFrameRate property.  		Taking the videos and their data recording will be 			executed by the startVideo method and ends taking the 		video by using the stopVideo method.

This property is initialized by the by the open method. The default value of 		this property is VCAP_VCMODE_PHOTO.

Goto Table56

Errors                A UposException may be thrown when this property is accessed.
		              For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value		Meaning

E_ILLEGAL	An invalid value was specified.

See also		PhotoColorSpace Property,  VideoColorSpace Property, 						PhotoResolution Property, VideoResolution Property, VideoFrameRate 				Property, PhotoFrameRate Property, CapPhotoColorSpace Property, 				CapVideoColorSpace Property, CapPhotoResolution Property, 					CapVidoeResolution Property, VideoMaxFrameRate Property,
				PhotoMaxFrameRate Property, VideoCaptureMode Property, 					CapPhoto Property, CapVideo Property, VideoType Property, 					VideoTypeList Property PhotoType Property, PhotoTypeList Property, 				takePhoto Method, startVideo Method, stopVideo Method.

VideoColorSpace Property

Syntax	VideoColorSpace: string {read-write, access after open, claim, enable}

Goto Table57

Remarks	Indicates the video color space ID of the frame data to be acquired by the   Video Capture Device, if CapVideoColorSpace property is true and it is used by startVideo method. Valid values are one of the values listed in the VideoColorSpaceList property.  This property is referred to when VideoCaptureMode property value is VCAP_VCMODE_VIDEO and CapVideo is true. This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20. 				Some possible values of the exception’s ErrorCode property are:

Value			Meaning

E_ILLEGAL		An invalid value was specified.

See also	CapVideoColorSpace Property, VideoColorSpaceList Property, VideoCaptureMode Property, startVideo Method

VideoColorSpaceList Property

Syntax	VideoColorSpaceList: string {read-only, access after open}

Goto Table58

Remarks	Video Color space information supported by the device is indicated in a 		comma-separated list. Each color space information is composed 			of the following information and is shown in the following order 			separated by a colon (":").							This property is initialized by the open method.

Parameter		Description

Color space ID	 ID for identifying the color space of RGB,         YUV422, etc.                                                                                                                                                                              Then if RGB Depth was 16 bits,                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                           YUV422 Depth was 32 bits, And they are indicating like“RGB:16, ;YUV422:32, ;…..”

Depth			Number of bits per 1 pixel

Errors		A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20.

See also	CapVideoColorSpace Property, VideoCaptureMode Property, VideoColorSpace Property

VideoFrameRate Property

Syntax	           VideoFrameRate; int32 {read-write, access after open､claim, enable}

Goto Table59

Remarks	Indicates the frame rate of the frame data recorded by the Video Capture Device and the video image capturing and recorded with the startVideo method. This property is only applied when VCAP_VCMODE_VIDEO is set in VideoCaptureMode property. Valid values range from 1 to VideoMaxFrameRate property and CapVideo property is true.      	　　This property is initialized by the open method.

Errors			A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20. 				Some possible values of the exception’s ErrorCode property are:

Value			Meaning 		     				E_ILLEGAL		An invalid value was specified.

See also            CapVideo Property, CapVideoFrameRate, Property, VideoCaptureMode Property, VideoMaxFrameRate Property, startVideo Method

VideoMaxFrameRate Property

Syntax		VideoMaxFrameRate: int32 {read-only, access after open}

Goto Table60

Remarks	Indicates the maximum video recording frame rate that can be set in 			VideoFrameRate property. 								This property is initialized by the open method.

Errors     	A UposException may be thrown when this property is accessed. 				For further information, see “Errors" on page Intro-20.

See also		VideoFrameRate Property, VideoCaptureMode Property

VideoResolution Property

Syntax		VideoResolution: string {read-write, access after open, claim, enable}

Goto Table61

Remarks	Indicates the resolution of video image data acquired by the Video Capture Device and recorded with the execution of startVideo method. Valid values are one of the values listed in the VideoResolutionList property. This property is only applied when VCAP_VCMODE_VIDEO is set in VideoCaptureMode property and if CapVideo property is true.      This property is initialized by the open method.

Errors			A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20. 				Some possible values of the exception’s ErrorCode property are:

Value			Meaning			           	E_ILLEGAL		An invalid value was specified.

See also	VideoResolutionList Property, CapVideo Property, 				VideoCaptureMode Property、startVideo Method

VideoResolutionList Property

Syntax		VideoResolutionList: string {read-only, access after open}

Remarks	A comma-separated list of possible resolutions for the VideoResolution property. Resolution is indicated by "Horizontal resolution number x Vertical resolution number" format. For example, when it supports 320x240, 640x480, 640x360, it is the following: "320x240,640x480,640x360" This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed.
For further information, see “Errors" on page Intro-20.

See also		CapVideoResolution Property, VideoResolution Property

VideoType Property

Syntax		VideoType; string {read-write, access after open, claim, enable}

Goto Table62

Remarks             Indicate the shape of the taking video and recorded with the	 startVideo method. Valid values are one of those listed in VideoTypeList property. This property is applied when VCAP_VCMODE_VIDEO is set in VideoCaptureMode property and if CapVideo property is true. 	     This property is initialized by the open method.

Errors			A UposException may be thrown when this property is accessed. 			For further information, see “Errors" on page Intro-20. 				Some possible values of the exception’s ErrorCode property are:

Value			Meaning             		    		E_ILLEGAL		An invalid value was specified.

See also		VideoCaptureMode Property, CapVideo Property, 	VideoTypeList Property, startVideo Method

Goto Table63

VideoTypeList Property

Syntax		VideoTypeList: string {read-only, access after open}

Remarks	A comma-separated list of video image format values that can be set for                the VideoType property. *1For example, when AVI_IYUV, AVI_MJPG is supported, it is the following "AVI_IYUV, AVI_MJPG".
Note: The notation contents may be different depending on the device.
This property is initialized by the open method.

Errors		A UposException may be thrown when this property is accessed.
			For further information, see “Errors" on page Intro-20.

See also	CapVideoType Property, VideoType Property, VideoCaptureMode Property

Note *1: The Video type related information are listed in here as the reference.													  AVI : Digital container format : 		                   https://en.wikipedia.org/wiki/Digital_container_format											             				              MJPG : Motion JPEG :                                                                                https://en.wikipedia.org/wiki/Motion_JPEG 															 	             IYUV : 4:2:0 Video Pixel Formats : 			              	 https://docs.microsoft.com/en-us/windows-hardware/drivers/display/4-2-0-video-pixel-formats											   4:2:2 Video Pixel Formats : 				   	             https://docs.microsoft.com/en-us/windows-hardware/drivers/display/4-2-2-video-pixel-formats  											  Video Formats and their Abbreviation : 			 http://technewzbd.blogspot.com/2013/05/video-formats-and-their-abbreviation.html

Note: Video Capture Device Property Value Relationship

Properties listed below are related within each Photo / Video Mode group, and if any value change occurs, other values may change accordingly.

Photo Mode Group Properties

PhotoType, PhotoColorSpace, PhotoColorSpaceList, PhotoFrameRate, 	PhotoMaxFrameRate, PhotoResolution, PhotoResolutionList

Video Mode Group Properties

VideoType, VideoColorSpace, VideoColorSpaceList, VideoFrameRate, 	VideoMaxFrameRate, VideoResolution, VideoResolutionList

Methods (UML operations)

startVideo Method

Syntax	startVideo (fileName : string, overwrite: boolean,                            	recordingTime: int32):        		            			     void{raises-exception, use after open-claim-enable}
Parameter	Description

Goto Table64

filename	Specify the name of the video file to be recorded. 	Overwrite	Specify the behavior when the same name file exists. 			If true, it is overwritten. 							If false, it will raise the UposException. 			recordingTime 	Specify the time for video recording in seconds.				If FOREVER (-1) is specified, recording will continue                        			until the stopVideo method is called.

Remarks	Before calling this method, it needs to set the VideoCaptureMode property to VCAP_VCMODE_VIDEO and CapVideo property needs to be true. Video capturing and recording starts with the setting contents of the VideoColorSpace property, VideoResolution property, VideoFrameRate property and VideoType property. This method is executed synchronously.	During the video image capturing, the recorded data storing is processed asynchronously and when the start and stop state are changed, StatusUpdateEvents are invoked for the application. StatusUpdateEvent will notify the application that there is a change in the power status or a state change during video capturing and recording.
When the time specified in recordingTime has elapsed, or by calling the stopVideo method, recording is completed and the video file specified by fileName is recorded and can deliver to the application.
Also, S_BUSY is set in the Status property during video capturing and recording. The place where video files are recorded is controlled through the Storage Property.

Errors		A UposException may be thrown when this method is invoked.
For further information, see “Errors" on page Intro-20.
Some possible values of the exception's ErrorCode property are:

Value	              Meaning     	       			          E_ILLEGAL	fileName is too long or contains characters that  				cannot be used, or 0 is specified for recordingTime. 			VideoCaptureMode property is not  					VCAP_VCMODE_VIDEO and CapVideo is not true.                   E_EXISTS	fileName already exists. (If overwrite is false)	     E_BUSY	Cannot execute because it is recording.

See also 	VideoColorSpace Property, VideoResolution Property,  VideoFrameRate Property, VideoType Property, stopVideo Method, StatusUpdateEvent Event, VideoCaptureMode Property

stopVideo Method

Syntax		stopVideo ( ):										    void {raises-exception, use after open-claim-enable}

Remarks	The video capturing and recording process started by the startVideo method has been ended and the taking video is completed. This method processed synchronously. The device invokes StatusUpdateEvent will to notify the application that there is a change in the power status or a state change during taking video capturing and recording were stopped.

Goto Table65

Errors		A UposException may be thrown when this method is invoked. 				For further information, see “Errors" on page Intro-20.					Some possible values of the exception's ErrorCode property are:

Value		Meaning

E_ILLEGAL	It is not recorded.

See also		startVideo Method, StatusUpdateEvent Event

takePhoto Method

Syntax		takePhoto (fileName: string, 								overwrite: boolean, timeout: int32 ):				           			void{raises-exception, use after open-claim-enable}

Goto Table66

Parameter	Description                  			   	   		fileName	Specify the image file name to be recorded.			overwrite	Specify the behavior when the same name file exists. 				If true it overwrites. If false, UposException is thrown. 		timeout		Allowed execution time in milliseconds, before the 				method fails and a timeout ErrorEvent is sent to the 				application. If FOREVER (-1) the service will wait until 				a photograph is taken or an application error occurs.

Remarks	Take photo and record with setting contents of PhotoColorSpace property, PhotoResolution property, PhotoFrameRate Property and PhotoType property. Before calling this method, it needs to set the VideoCaptureMode property to VCAP_VCMODE_PHOTO and this method can be executed if CapPhoto property is true. This method is executed synchronously. The process of recorded data storing is performed asynchronously. StatusUpdateEvents are invoked for the application when the start and the end state were changed.   The location where photo files are recorded is controlled through the Storage Property. The timeout specifies the number of milliseconds

Errors		A UposException may be thrown when this method is invoked.				For further information, see “Errors" on page Intro-20.					Some possible values of the exception's ErrorCode property are:

Value           	Meaning

E_ILLEGAL	One of the following occurred. 			  FileName is too long or contains unusable characters. VideoCaptureMode property is not                                                                                                                                                                                                                                                         VCAP_VCMODE_PHOTO and CapPhoto property is not true.

E_EXISTS	fileName already exist. (When overwrite=false)

See also	VideoCaptureMode Property, PhotoColorSpace Property,
PhotoResolution Property, CapPhoto Property, PhotoType Property, PhotoFrameRate Property, StatusUpdateEvent Event

Events (UML interfaces)

DirectIOEvent

<<event>>	upos::events::DirectIOEvent

EventNumber	: int32 {read-only}
Data			: int32 {read-write}
Obj			: object {read-write}

Description 	Provides Service information directly to the application. This event provides a means for a vendor-specific Video Capture Service to provide events to the application that are not otherwise supported by the device control.

Attributes	This event contains the following attributes:

Attribute	Type	Description

EventNumber	int32	Event number whose specific values are assigned by the Service.

Data	int32	Additional numeric data. Specific values vary by the EventNumber and the Service. This attribute is settable.

Obj	object	Additional data whose usage varies by the EventNumber and the Service. This attribute is settable.

Remarks	This event is to be used only for those types of vendor specific functions that are not otherwise described.

Use of this event may restrict the application program programform being used with other vendor’s devices which may not have any knowledge of the Service’s need for this event.

See Also	“Events" on page Intro-19, directIO method

ErrorEvent

<<event>>     upos::events::ErrorEvent

Goto Table67

ErrorCode		 : int32 {read-only}
		      ErrorCodeExtended	 : int32 {read-only}
		      ErrorLocus		 : int32 {read-only}
		      ErrorResponse	 : int32 {read-write}

Description	Notifies the application that a Video Capture Device error has been detected and suitable response by the application is necessary to process the error condition.

Attributes	This event contains the following attributes:

Attributes	Type	Description

ErrorCode	int32	Error code causing the error event. 			See a list of Error Codes on page 20.

ErrorCodeExtended	int32	Extended Error code causing the error event.

If ErrorCode is E_EXTENDED, then see values below. Otherwise, it may contain a Service-specific value.

ErrorLocus	int32	Location of the error. If EL_INOUTPUT is specified. An error occurred during asynchronous processaction.

ErrorResponse	int32	Error Response, whose default value may be overridden by the application. (i.e., this attribute is settable). See ErrorResponse below for values.

If ErrorCode is E_EXTENDED, then ErrorCodeExtended has one of the following values:

Value	 	       Meaning                                                                      EVCAP_NOROOM	       The image data storage area does not have enough 	 		       room to store.

The ErrorLocus attribute has one of the following values:

Value	Meaning

EL_INOUTPUT	Error occurred while processing asynchronous output.

The application’s error event handler can set the ErrorResponse attribute to one of the following values:

Value	Meaning

ER_RETRY	Retry recorded sending the data storing. The error state is exited.
Typically, valid for asynchronous recorded data storing output devices when the locus is EL_INOUTPUT, in which case the asynchronous recorded data storing output is retried, and the error state is exited. This is the default response when the locus is EL_OUTPUT.

ER_CLEAR	Valid for loci: EL_OUTPUT.  Clear all buffered captured input or stored output data (including all asynchronous output).  The error state is exited.

Remarks	This event is enqueued when an error is detected, and the Device’s State transitions into the error state.

See Also 	“Error Handling" on page Intro-23, “Device Output Models" on page 	Intro-25. Device information Reporting Model “ on Page Intro-30

StatusUpdateEvent

<< event >>	upos::events::StatusUpdateEvent

Status		: int32 {read-only}

Description	Notifies the application that there is a change in the power status or a state change of the Video Capture device.

Attributes		This event contains the following attribute:

Attributes	Type	 Description

Status                 int32	Indicates a change in the power status or a sate                                                     change of the unit.

Note that Release 1.3 added Power State Reporting with additional Power 	reporting StatusUpdateEvent values.

The Update Firmware capability added additional Status values for communicating the status/progress of an asynchronous update firmware process. See “StatusUpdateEvent” description on page 1-34.

Value            		Meaning

Goto Table147

VCAP_SUE_START_VIDEO_RECORDING
						It will be notified when video recording starts.

VCAP_SUE_STOP_VIDEO_RECORDING
						It will be notified when video recording stops.

VCAP_SUE_START_PHOTO 											It will be notified when photo capturing starts.

VCAP_SUE_END_PHOTO 											It will be notified when photo capturing ends.

Remarks	      Enqueued when the Video Capture Device detects a power state change or a status change.

See Also 	“Events" on page Intro-19.

Chapter 40

Individual Recognition

This Chapter defines the Individual Recognition device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.16 | Open |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.16 | Open |
| CapPowerReporting: | int32 | {read-only} | 1.16 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.16 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.16 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.16 | open |
| CheckHealthText: | string | {read-only} | 1.16 | open |
| Claimed: | boolean | {read-only} | 1.16 | open |
| DataCount: | int32 | {read-only} | 1.16 | open |
| DataEventEnabled: | boolean | {read-write} | 1.16 | open |
| DeviceEnabled: | boolean | {read-write} | 1.16 | open & claim |
| FreezeEvents: | boolean | {read-write} | 1.16 | open |
| OutputID: | int32 | {read-only} | 1.16 | Not supported |
| PowerNotify: | int32 | {read-write} | 1.16 | open |
| PowerState: | int32 | {read-only} | 1.16 | open |
| State: | int32 | {read-only} | 1.16 | -- |
| DeviceControlDescription: | string | {read-only} | 1.16 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.16 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.16 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.16 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.16 | open |
| PhysicalDeviceName: | string | {read-only} | 1.16 | open |

| Properties (Continued) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specific | Type | Mutability | Version | May Use After |
| CapIndividualList: | string | {read-only} | 1.16 | open |
| IndividualIDs: | string | {read-only} | 1.16 | open, claim & enable |
| IndividualRecognitionFilter: | string | {read-write} | 1.16 | open |
| IndividualRecognitionInformation | string | {read-only} | 1.16 | open |

Goto Table74

Goto Table73

Goto Table72

Goto Table68

Goto Table71

Goto Table70

Goto Table69

| Methods (UML operations) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common |  |  |  |  |
| Name |  |  | Version |  |
| open (logicalDeviceName: string): void {raises-exception} |  |  | 1.16 |  |
| close ( ): void {raises-exception, use after open} |  |  | 1.16 |  |
| claim (timeout: int32): void {raises-exception, use after open} |  |  | 1.16 |  |
| release ( ): void {raises-exception, use after open, claim} |  |  | 1.16 |  |
| checkHealth (level: int32): void {raises-exception, use after open, enable} |  |  | 1.16 |  |
| clearInput ( ): void {raises-exception, use after open, claim} |  |  | 1.16 |  |
| clearInputProperties ( ): void {raises-exception, use after open, claim} |  |  | 1.16 |  |
| clearOutput ( ): void { } |  |  | Not supported |  |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, claim, enable} |  |  | 1.16 |  |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} |  |  | 1.16 |  |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  | 1.16 |  |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  | 1.16 |  |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, claim, enable} |  |  | 1.16 |  |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  | 1.16 |  |
| Events (UML interfaces) |  |  |  |  |
| Name | Type | Mutability |  | Version |
| upos::events::DataEvent |  |  |  | 1.16 |
| Status: | int32 | {read-only} |  |  |
| upos::events::DirectIOEvent |  |  |  | 1.16 |
| EventNumber: | int32 | {read-only} |  |  |
| Data: | int32 | {read-write} |  |  |
| Obj: | object | {read-write} |  |  |
| upos::events::ErrorEvent |  |  |  | 1.16 |
| ErrorCode: | int32 | {read-only} |  |  |
| ErrorCodeExtended: | int32 | {read-only} |  |  |
| ErrorLocus: | int32 | {read-only} |  |  |
| ErrorResponse: | int32 | {read-write} |  |  |
| upos::events::OutputCompleteEvent |  | Not supported |  | 1.16 |
| upos::events::StatusUpdateEvent |  |  |  | 1.16 |
| Status: | int32 | {read-only} |  |  |
| upos::events::TransitionEvent |  | Not supported |  | 1.16 |

General Information

The Individual Recognition programmatic name is “Individual Recognition”.

Capabilities

The Individual Recognition has the following set of capabilities:

Analyzes the image of the camera and recognizes individuals such as people and listed goods.

Individual Recognition Class Diagram

The following diagram shows the relationships between the Individual Recognition classes.

Fig. Chap.40-1 Individual Recognition Class Diagram

Model

The Individual Recognition follows the general “Device Input Model” for event-driven input:

Input Model

- When an individual is recognized by this device, a DataEvent is delivered to the application after the IndividualIDs property was set to indicate the recognized individuals.

- Identifiable individuals are indicated by the CapIndividualList property.

- Check the functions supported by the device, set validity / invalidity, etc. with the IndividualRecognitionInformation property.

- Recognized data is stored in the IndividualRecognitionInformation property, IndividualIDs property.

- How to recognize the individuals depends on the IndividualRecognitionFilter function, therefore, please refer to the IndividualRecognitionFilter section.

- Other device behavior about this device supports the general device input model as listed below.

- If the AutoDisable property is true, then the device automatically disables itself when a DataEvent is enqueued.

- An enqueued DataEvent can be delivered to the application when the DataEventEnabled property is true and other event delivery requirements are met. Just before delivering this event, data is copied into corresponding properties, and further data events are disabled by setting DataEventEnabled to false. This causes subsequent input data to be enqueued while the application processes the current input and associated properties. When the application has finished processing the current input and is ready for more data, it reenables events by setting DataEventEnabled to true.

- An ErrorEvent (or events) is enqueued if an error occurs while gathering or processing input and is delivered to the application when DataEventEnabled is true and other event delivery requirements are met.

- The DataCount property may be read to obtain the total number of enqueued DataEvents.

- All enqueued input may be deleted by calling clearInput method. See the clearInput method description for more details.

- All data properties that are populated, as a result of firing a DataEvent or ErrorEvent can be set back to their default values by calling the clearInputProperties method.

- The application will be informed about any status change with a StatusUpdateEvent, also all corresponding status properties will be updated before event delivery.

Device Sharing

The Individual Recognition is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before the device begins reading input.

- See the “Summary” table for precise usage prerequisites.

IndividualRecognitionFilter

The IndividualRecognitionFilter property defines the following data as information for the individual recognition function of Individual Recognition Device.

･	Various support function existence or not.	
	(Supported functions are defined by the device)

･	Enable, disable status of various functions.

･	Types handled by various functions (examples: “male”, “female” of gender recognition)

･	Filter setting of various functions.

The following data is defined in the IndividualRecognitionInformation property

･	Individual Recognition input data

The device defines the individual recognition function information and the individual recognition input data.

The application refers to these contents to determine the support range and so on.

In addition, the application changes the enabled / disabled state of various functions, the filter setting, and controls each function.

The enabled / disabled state of the various functions set by the application, and the filter settings are applied by setting the DeviceEnabled property to true and enabling the individual recognition function.

When the application set various functions, it is possible to specify and set only the target ones.

The device fires a DataEvent based on the content set by the application and stores the input data in IndividualRecognitionInformation property.

IndividualRecognitionFilter Property Example Format

The IndividualRecognitionFilter property of the individual recognition device may define various information. Here is the example described by using the JSON format.

- Basic Items

| Key |  |  |  | Value | Value change capability | Explanation |
| --- | --- | --- | --- | --- | --- | --- |
| IndividualRecognitionFilter |  |  |  | object | N | Information for the various individual recognition. Target device define the supporting individual recognition object. |
|  | [IndividualRecognitionID] |  |  | object | N | Recognizable individual recognition information. Key name is the ID of recognized individual |
|  |  | Enabled |  | boolean | Y | Enable or disable state of target individual recognition. Application can control the target individual recognition by referring or changing. |
|  |  | Properties |  | object | N | Property information of the target individual recognition. Application control the target individual recognition by referring or changing the defined property value. |
|  |  |  | [Property01] | - | - |  |
|  |  |  | [Property02] | - | - |  |
|  |  | Filters |  | object | N | Input data filter setting information. Application filter the target individual recognition input data by changing the defined value. |
|  |  |  | [Filter01] | - | - |  |
|  |  |  | [Filter02] | - | - |  |

- Face Recognition device example

| Key |  |  |  |  |  | Value | Value change capability | Explanation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IndividualRecognitionFilter |  |  |  |  |  | object | N |  |
|  | Face |  |  |  |  | object | N |  |
|  |  | Enabled |  |  |  | boolean | Y |  |
|  |  | Properties |  |  |  | object | N |  |
|  |  |  | FaceImageNamePrefix |  |  | string | Y | Output image file prefix for face recognition |
|  |  |  | Gender |  |  | object | N | Information on gender recognition |
|  |  |  |  | Enabled |  | boolean | Y | Gender recognition enable, disable state |
|  |  |  |  | CapTypeList |  | array | N | Type list (“female”, “male”) |
|  |  |  | Age |  |  | object | N | Information on age recognition. |
|  |  |  |  | Enabled |  | boolean | Y | Age recognition enable, disable state |
|  |  |  | Facial Expression |  |  | object | N | Information on facial expression recognition |
|  |  |  |  | Enabled |  | boolean | Y | Facial expression recognition enable, disable state. |
|  |  |  |  | CapTypeList |  | array | N | Type list (“smile”, “angry”,…) |
|  |  |  | Gaze |  |  | object | N | Information on gaze recognition |
|  |  |  |  | Enabled |  | boolean | Y | Gaze recognition enable, disable state. |
|  |  |  |  | CapTypeList |  | array | N | Type list (“gaze”, “nogaze”) |
|  |  |  | Distance |  |  | object | N | Information on distance recognition |
|  |  |  |  | Enabled |  | boolean | Y | Distance recognition enable, disable state |
|  |  |  |  | CapTypeList |  | array | N | Type list (“near”, “far”, “very far”,…) |
|  |  |  |  | NearLength |  | number | Y | Distance to recognize as “near”. A recognition event is fired when a person is recognized in the range from 0 to Near Length. |
|  |  |  |  | FarLength |  | number | Y | Distance to recognize as “far”, “very far”. A recognition event is fired when a person is recognized in the range from Near Length to Far Length. A recognition event is fired when a person is recognized in the range more than Far Length. |
|  |  |  | Authentication |  |  | object | N | Information on face authentication |
|  |  |  |  | Enabled |  | boolean | Y | Face authentication enable, disable state. |
|  |  |  |  | ImageList |  | array | Y | Image file name list for comparison. Event is fired when it matches the image specified here. (Wild card can be specified) |
|  |  | Filters |  |  |  | object | N |  |
|  |  |  | Gender |  |  | object | N | Information on gender recognition filter. |
|  |  |  |  | TypeList |  | array | Y | Target Filter TypeList. Valid values are defined by CapTypeList. Recognition target is specified. To disable the filter, null should be assigned in its value. |
|  |  |  |  | Score |  | number | Y | Recognition score. Valid values are from 0 to 100. The range of the score specified here is the recognition target. To disable the filter, -1 should be assigned in its value. |
|  |  |  | Age |  |  | object | N | Information on age recognition. |
|  |  |  |  |  | Min | number | Y | Minimum age. The age below the specified is not a recognition target. To disable the filter -1 should be specified in its value. |
|  |  |  |  |  | Max | number | Y | Maximum age. The age above the specified is not a recognition target. To disable the filter -1 should be specified in its value. |
|  |  |  | Expression |  |  | object | N | Information on facial expression recognition filter. |
|  |  |  |  |  | TypeList | array | Y | Filter target type list. Valid values are defined in CapTypeList. Recognition target type is specified. To disable the filter null should be assigned in its value. |
|  |  |  |  |  | Score | number | Y | Recognition score. Valid values are from 0 to 100. The range of the score specified here is to be recognized. To disable the filter -1 should be assigned in its value. |
|  |  |  | Gaze |  |  | object | N | Information on gaze recognition filter |
|  |  |  |  |  | TypeList | array | Y | Filter target type list. Valid values are defined by CapTypeList. Recognition target is specified. To disable the filter, null should be assigned in its value. |
|  |  |  | Distance |  |  | object | N | Information on distance recognition filter |
|  |  |  |  |  | TypeList | array | Y | Filter target type list. Valid values are defined by CapTypeList. Recognition target is specified. To disable the filter, null should be assigned in its value. |

IndividualRecognition Information Property Example Format

IndividualRecognitionInformation property of individual recognition device may define various information and here is the example format described by JSON.

- Basic Items

| Key |  |  |  | Value | Value change capability | Explanation |
| --- | --- | --- | --- | --- | --- | --- |
| IndividualRecognitionInformation |  |  |  | object | N | Various Individual recognition input data. |
|  | [IndividualRecognitionID] |  |  | object | N | Store the input data of individual recognition. Key name is ID of individual recognition. |
|  |  | Properties |  | Array <object> | N | Input data list of target individual recognition. The content of the data is different for each device or function. |
|  |  |  | [Data01] | - | - |  |
|  |  |  | [Data02] | - | - |  |

- Face Recognition Device Example

| Key |  |  |  |  | Value | Value change capability | Explanation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IndividualRecognitionInformation |  |  |  |  | object | N |  |
|  | Face |  |  |  | object | N |  |
|  |  | DataLists |  |  | array <object> | N |  |
|  |  |  | FaceID |  | string | N | ID assigned to the recognized face |
|  |  |  | FaceImageName |  | string | N | Recognized face image file name |
|  |  |  | Gender |  | object | N | Recognized gender information |
|  |  |  |  | Type | string | N | Recognized type |
|  |  |  |  | Score | number | N | Confidence score of recognized type. |
|  |  |  | Age |  | object | N | Recognized age information |
|  |  |  |  | Age | number | N | Recognized age |
|  |  |  | Expression |  | object | N | Recognized facial expression information |
|  |  |  |  | Type | string | N | Recognized type. One of CapTypeList items is set. |
|  |  |  |  | Score | number | N | Confidence score of recognized type. |
|  |  |  | Gaze |  | object | N | A gaze list for each recognized face ID. |
|  |  |  |  | Type | string | N | Recognized type |
|  |  |  | Distance |  | object | N | Recognized distance information |
|  |  |  |  | Type | string | N | Recognized type. One of CapTypeList items is set. |
|  |  |  | Authentication |  | object | N | Authentication result information |
|  |  |  |  | ImageName | string | N | Matched image file name |

Properties (UML attributes)

CapIndividualList Property

Syntax	CapIndividualList: string {read-only, access after open}

Remarks	Recognizable individual information is indicated by the list separated by a separator ",".

Each Individual information consists of the following information and is shown in the following order, separated with a colon (":").

Parameter	Meaning

IndividualID	An ID indicated an identifiable Individual

IndividualName	A Name of an Individual.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	IndividualIDs Property

IndividualIDs Property

Syntax	IndividualIDs: string {read-only, access after open}

Remarks	Set the IndividualIDs recognizable Individual recognition device.

IndividualIDs values are indicated by separated with a colon (":").

Its value is set prior to a DataEvent being delivered to the application.

Errors			A UposException may be thrown when this property is accessed.
							For further information, see “Errors" on page Intro-20.

See Also	CapIndividualList Property

IndividualRecognitionFilter Property

Syntax	IndividualRecognitionFilter: string {read-write, access after open, claim, enable}

Remarks		Individual Recognition Function Information:

Goto Table75

Supporting the various functions (Refer to the Individual Recognition Filter Example Format written by JSON and supported function examples .).

- Various Valid / Invalid State functions.

- Various handled function types.  (e.g., "male" "female" in gender recognition, etc.).

- Various filter function settings. All Individual Recognition function data information is defined by the device. By referring to these contents, the application can determine the supporting scope. Thereby, the application can control each function by changing the valid / invalid state and / or the various filter function settings. This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20. Some possible values of the exception’s ErrorCode property are:

Value		Meaning

E_ILLEGAL		An invalid value was specified.

IndividualRecognitionInformation Property

Syntax	IndividualRecognitionInformation: string {read-only, access after open}

Remarks	Holds data indicating the following. Individual recognition input data. All Individual recognition input data is defined by the device.

Errors			A UposException may be thrown when this property is accessed.
							For further information, see “Errors" on page Intro-20.

Events (UML interfaces)

DataEvent

<<event>>	upos::events::DataEvent

Status			: int32{read-only}

Description 	Notifies the application when data from the Individual Recognition device is available to be read.

Attributes	This event contains the following attributes:

Attribute	Type		Description

Status	int32		Set to 0.

Goto Table148

Remarks	Before this event is delivered, the data is copied into corresponding properties individual recognition information is enqueued into the area that is indicated by the readValue method.  Since the stored individual recognition information might be managed by the associated “Hard Totals” device service, therefore, the application might also support the “Hard Totals” service.

See Also	VideoCaptureMode Property, “Events" on page Intro-19, directIO method.

DirectIOEvent

<<event>>	upos::events::DirectIOEvent

EventNumber	: int32 {read-only}
Data			: int32 {read-write}
Obj			: object {read-write}

Description 	Provides Service information directly to the application. This event provides a means for a vendor-specific Individual Recognition Service to provide events to the application that are not otherwise supported by the device control.

Attributes	This event contains the following attributes:

Attribute	Type	Description

EventNumber	int32	Event number whose specific values are assigned by the Service.

Data	int32	Additional numeric data. Specific values vary by the EventNumber and the Service. This attribute is settable.

Obj	object	Additional data whose usage varies by the EventNumber and the Service. This attribute is settable.

Remarks	This event is to be used only for those types of vendor specific functions that are not otherwise described.

Use of this event may restrict the application program programform being used with other vendor’s devices which may not have any knowledge of the Service’s need for this event.

See Also	"Events" on page Intro-19, directIO method.

ErrorEvent

<<event>>	upos::events:: ErrorEvent

ErrorCode			: int32 {read-only}
ErrorCodeExtended	: int32 {read-only}
ErrorLocus			: int32 {read-only}
ErrorResponse		: int32{read-write}

Description	Notifies the application that an Individual Recognition Device error has been detected and suitable response by the application is necessary to process the error condition.

Attributes	This event contains the following attributes:

Attributes	Type	Description

ErrorCode	int32	Error code causing the error event. 			See a list of Error Codes on page 20.

ErrorCodeExtended	int32	Extended Error code causing the error event.

If ErrorCode is E_EXTENDED, then see values below. Otherwise, it may contain a Service-specific value.

ErrorLocus	int32	Location of the error. See values below. If EL_OUTPUT is specified. An error occurred during asynchronous action.

Goto Table76

ErrorResponse	int32	Error Response, whose default value may be overridden by the application. (i.e., this attribute is settable). See ErrorResponse below for values.

The ErrorLocus attribute has one of the following values:

Value	Meaning

EL_OUTPUT	Error occurred while processing asynchronous output.

EL_INPUT	Error occurred while gathering or processing event-driven input. No previously buffered input data is available.

EL_INPUT_DATA	Error occurred while gathering or processing event-driven input, and some previously buffered data is available.

The application’s error event handler can set the ErrorResponse attribute to one of the following values:

Value	Meaning

ER_RETRY	Retry sending the data. The error state is exited.
May be valid for some input devices when the locus is EL_INPUT, in which case the input is re-tried, and the error state is exited. Typically, valid for asynchronous output devices when the locus is EL_OUTPUT, in which case the asynchronous output is re-tried, and the error state is exited. This is the default response when the locus is EL_OUTPUT.

ER_CLEAR	Valid for all locusloci: EL_INPUT, EL_INPUT_DATA, and EL_OUTPUT.  Clear all buffered input or output data (including all asynchronous output).  The error state is exited.  This is the default response when the locus is EL_INPUT.

ER_CONTINUEINPUT 
Only valid when the locus is EL_INPUT_DATA.
Acknowledges that a data error has occurred and directs the Device to continue input processing. The Device remains in the error state and will deliver additional DataEvents as directed by the DataEventEnabled property. When all input has been delivered and DataEventEnabled is again set to true, then another ErrorEvent is delivered with locus EL_INPUT.
This is the default response when the locus is EL_INPUT_DATA.

Remarks	This event is enqueued when an error is detected, and the Device’s State transitions into the error state. Input error events are not delivered until DataEventEnabled is true, so that proper application sequencing occurs.

Unlike a DataEvent, the Device does not disable further DataEvents or input ErrorEvents; it leaves the DataEventEnabled property value at true.    Note that the application may set DataEventEnabled to false within its event handler if subsequent input events need to be disabled for a period of time.

See Also 	“Device Input Model" on page Intro-22, “Error Handling" on page Intro-23, “Device Output Models" on page Intro-25.

StatusUpdateEvent

<<event>>	upos::events:: StatusUpdateEvent
Status			: int32 {read-only}

Description	Notifies the application that there is a change in the power status or a status

of the Individual Recognition device.

Attributes	This event contains the following attribute:

Attribute	Type	Description

Status	int32	Indicates a change in the power status of the unit.

Note that Release 1.3 added Power State Reporting with additional Power reporting StatusUpdateEvent values.

The Update Firmware capability added additional Status values

For communicating the status/progress of an asynchronous update firmware

process. See “StatusUpdateEvent” description on page 1-34.

Remarks	Enqueued when the Individual Recognition Device detects a power state

change or a status change.

See Also 	“Events" on page Intro-19

Chapter 41

Sound Recorder

This Chapter defines the Sound Recorder device category.

Summary

| Properties(UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.16 | open |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.16 | open |
| CapPowerReporting: | int32 | {read-only} | 1.16 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.16 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.16 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.16 | open |
| CheckHealthText: | string | {read-only} | 1.16 | open |
| Claimed: | boolean | {read-only} | 1.16 | open |
| DataCount: | int32 | {read-only} | 1.16 | open |
| DataEventEnabled: | boolean | {read-write} | 1.16 | open |
| DeviceEnabled: | boolean | {read-write} | 1.16 | open & claim |
| FreezeEvents: | boolean | {read-write} | 1.16 | open |
| OutputID: | int32 | {read-only} | 1.16 | Not supported |
| PowerNotify: | int32 | {read-write} | 1.16 | open |
| PowerState: | int32 | {read-only} | 1.16 | open |
| State: | int32 | {read-only} | 1.16 | open |
| DeviceControlDescription: | string | {read-only} | 1.16 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.16 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.16 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.16 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.16 | open |
| PhysicalDeviceName: | string | {read-only} | 1.16 | open |

| Properties (Continued) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specific | Type | Mutability | Version | May Use After |
| CapAssociatedHardTotalsDevice: | string | {read-only} | 1.16 | open |
| CapChannel: | boolean | {read-only} | 1.16 | open |
| CapRecordingLevel: | boolean | {read-only} | 1.16 | open |
| CapSamplingRate: | boolean | {read-only} | 1.16 | open |
| CapSoundType: | boolean | {read-only} | 1.16 | open |
| CapStorage | int32 | {read-only} | 1.16 | open |
| Channel: | string | {read-write} | 1.16 | open, claim & enable |
| ChannelList: | string | {read-only} | 1.16 | open |
| RecordingLevel: | int32 | {read-write} | 1.16 | open, claim & enable |
| RemainingRecordingTimeInSec: | int32 | {read-only} | 1.16 | open, claim & enable |
| SamplingRate: | string | {read-write} | 1.16 | open, claim & enable |
| SamplingRateList: | string | {read-only} | 1.16 | open |
| SoundData: | binary | {read-only} | 1.16 | open |
| SoundType: | string | {read-write} | 1.16 | open, claim & enable |
| SoundTypeList: | string | {read-only} | 1.16 | open |
| Storage | int32 | {read-write} | 1.16 | open, claim & enable |

Goto Table77

| Methods(UML operations) |  |
| --- | --- |
| Common |  |
| Name | Version |
| open (logicalDeviceName: string): void {raises-exception} | 1.16 |
| close ( ): void {raises-exception, use after open} | 1.16 |
| claim (timeout: int32): void {raises-exception, use after open} | 1.16 |
| release ( ): void {raises-exception, use after open, claim} | 1.16 |
| checkHealth (level: int32): void {raises-exception, use after open, enable} | 1.16 |
| clearInput ( ): void {raises-exception, use after open, claim} | 1.16 |
| Me Methods (UML operations)(continued) |  |
| Common |  |
| Name | Version |
| clearInputProperties ( ): void {raises-exception, use after open, claim} | 1.16 |
| clearOutput ( ): void { } | Not supported |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, claim, enable} | 1.16 |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} | 1.16 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | 1.16 |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | Goto Table81 1.16 |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, claim, enable} | 1.16 |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | 1.16 |
| Specific |  |
| Name | Version |
| startRecording (FileName: string, OverWrite: boolean, RecordingTime:int32): void {raises-exception, use after open, claim, enable} | 1.16 |
| stopRecording ( ): Void {raises-exception, use after open, claim, enable} | 1.16 |

Goto Table83

Goto Table82

Goto Table79

Goto Table78

Goto Table80

| Events (UML interfaces) |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Mutability | Version |
| upos::events::DataEvent |  |  | 1.16 |
| Status: | int32 | {read-only} |  |
| upos::events::DirectIOEvent |  |  | 1.16 |
| EventNumber: | int32 | {read-only} |  |
| Data: | int32 | {read-write} |  |
| Obj: | object | {read-write} |  |
| upos::events::ErrorEvent |  |  | 1.16 |
| ErrorCode: | int32 | {read-only} |  |
| ErrorCodeExtended: | int32 | {read-only} |  |
| ErrorLocus: | int32 | {read-only} |  |
| *pErrorResponse: | int32 | {read-write} |  |
| upos::events::OutputCompleteEvent |  | Not supported | 1.16 |
| upos::events::StatusUpdateEvent |  |  | 1.16 |
| Status: | int32 | {read-only} |  |
| upos::events::TransitionEvent |  | Not supported | 1.16 |

General Information

The Sound Recorder programmatic name is "Sound Recorder".

Capabilities

The Sound Recorder has the following capability:

Record the real-time audio to a file, deliver the recorded sound data to the property that application may read and / or retrieve, and save the recorded sound data file to device memory and / or other storage devices.

Sound Recorder Class Diagram

The following diagram shows the relationships between the Sound Recorder classes.

Fig. Chap. 41-1 Sound Recorder Class Diagram

Model

Sound Recorder Control follows a general “Device Input Model” in a broad sense. One point of difference is that the Sound Recorder device required the execution of methods to start and stop the sound recording process and creates a sound data file in real time, deliver the data to the property and save the file in device and / or associated storage device.

The Sound Recorder Model defines the following behavior: Sound Recorder device controls the Sound Recorder device to set the input (recording) conditions, specifies the start / end of input data acquisition by the method. And makes the sound data file in real time from the acquired audio and delivers the data to the appropriate property. At the same time, saves the recorded data file in device and /or associated storage devices.

“Sound Recorder” device control starts recording with the startRecording method.   Prior to execute the startRecording method each value setting of Channel property, SamplingRate property, and RecordingLevel property are required, if each of CapChannel property CapSamplingRate property is true. And also need to set the DataEventEnabled property to true. At the same time, the recording format setting starts with the SoundType property value, if CapSoundType property is true.

The recording ends after the specified time has elapsed or when stopRecording method is called or when clearInput method is called. The generated sound data file will be recorded for either the host file or the Hard Totals device or both, after the end of recording. And generated sound data will be delivered to the SoundData property. Just after the delivery of sound data to the property, when the DataEventEnabled property is true, the DataEvent is enqueued and delivered to the application.

If the AutoDisable property is true, the device will automatically disable itself after the DataEvent is enqueued.

The remaining recording time in seconds can be obtained from the property RemainingRecordingTimeInSec

StatusUpdateEvent with status SERC_SUE_START_SOUND_RECORDING is evoked when startRecording method is executed to notify the application that recording state with has started.

When the sound recording is finished, if the specified time of startRecording method has elapsed or stopRecording method has been called, a StatusUpdateEvent with status SERC_SUE_STOP_SOUND_RECORDING is evoked to notify the application that recording has been stopped.

An enqueued DataEvent can be delivered to the application when the DataEventEnabled property is true and other event delivery requirements are met. Just before enqueuing this event, the device provides the recorded data to the SoundData property and disables further data events by setting the DataEventEnabled property to false. This causes subsequent input data to be buffered by the device while the application processes the current input and associated properties. When the application has finished processing the current input and is ready for more data, it re-enables events by setting DataEventEnabled to true.

If ErrorEvent response is ER_CONTINUEINPUT, the process of input processing continues. However, as long as the cause of the error is not resolved, the ErrorEvent will occur again immediately.

If ErrorEvent is ER_CLEAR, the input processing process is terminated, and the record is discarded.

If the time specified by the startRecording method is FOREVER (-1), execution will       continue until the stopRecording method is called in the application. When stopRecording is called, the previous recording data is recorded to the host file, the Hard Totals device, or both, with the specified file name, and the sound data will be delivered to the SoundData property. When DataEventEnabled property is true, the DataEvent is enqueued and delivered to the application.

Only one call to startRecording method can be in progress at a time. An attempt to nest sound recorder operations will result in an UposException being thrown.

If Error occurs during the execution of the startRecording method application should call the stopRecording method to terminate the recording process or cancel the recording process by calling the clearInput method before ending the ErrorEvent processing. After this when the stopRecording method is called, the recording data until just before the ErrorEvent occurs is recorded to the host file, the Hard Totals device, or both. When DataEventEnabled property is true, the DataEvent is enqueued and delivered to the application.

If there is no Error during the execution of startRecording method can terminate the recording process and can stop the recording at any time. When the stopRecording method is called, the recording data until just before the method call is recorded to the host file, the Hard Totals or both. When DataEventEnabled property is true, the DataEvent is enqueued and delivered to the application.

All input data enqueued by the device may be deleted by calling the clearInput method．All data properties that are populated as a result of a DataEvent or Error Event can be set back to their default values by calling the clearInputProperties method.

The device may have the ability to write encoded sound data files to either the Hard Totals devices or the host file system, or both, and the CapStorage property will show the device’s data storage location capability.

If device supports either or both Hard Totals devices and the host file system, the application should set the Storage property accordingly to tell where to write the encoded sound data file.

If device needs to be able to write the encoded sound data to an associated Hard Totals device, the CapAssociatedHardTotalsDevice property holds the open name of the associated Hard Totals device.

Device Sharing

The Sound Recorder is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before accessing some properties or calling methods that update the device.

- See the “Summary” table for precise usage prerequisites.

- The image display mode of the graphics device control is as follows.

Properties(UML attributes)

CapAssociatedHardTotalsDevice Property

Syntax	CapAssociatedHardTotalsDevice: string {read-write, access after open}

Goto Table84

Remarks	Holds the open name of the associated Hard Totals device, if the device is able to write to such devices which is the case if CapStorage is either SREC_CST_ALL or SREC_CST_HARDTOTALS_ONLY. If CapStorage is SREC_CST_HOST_ONLY this property value must be the empty string.  This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	CapStorage Property

CapChannel Property

Syntax	CapChannel: boolean {read-only, access after open}

Remarks	If true, the application can change the channel.
	If false, the application cannot change the channel.
This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	Channel Property

CapSamplingRate Property

Syntax	CapSamplingRate: boolean {read-only, access after open}

Remarks	If true, the application can change the sampling rate.
If false, the application cannot change the sampling rate.
This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed.
For further information, see “Errors" on page Intro-20.

See Also	SamplingRate Property.

CapSoundType Property

Syntax	CapSoundType: boolean {read-only, access after open}

Remarks	If true, the application can change the sound file type.
If false, the application cannot change the sound file type.
This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	SoundType Property.

CapStorage Property

Syntax	CapStorage: int32 {read-only, access after open}

Remarks	This is an enumeration and announces where the device is able to write the recorded sound data file to.
It holds one of the following values.

Value	Meaning

SREC_CST_HARDTOTALS_ONLY					    Only an associate Hard Totals device is supported.

SREC_CST_HOST_ONLY	Only the host’s file system is supported.

SREC_CST_ALL	Both, the associated Hard Totals device and the host’s file system is supported.

This property is initialized by the open method.

If a Hard Totals device is supported the Storage the property value should be SREC_CST_HARDTOTALS_ONLY or SREC_CST_ALL, and the property CapAssociatedHardTotalsDevice holds the open name of the associated Hard Totals device.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	Storage Property, CapAssociatedHardTotalsDevice Property

CapRecordingLevel Property

Syntax	CapRecordingLevel: boolean {read-only, access after open}

Remarks	If true, the application can change the recording level.
If false, the application cannot change the recording level.
This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	CapRecordingLevel Property.

Channel Property

Syntax	Channel: string {read-write, access after open-claim-enable}

Remarks	Holds the channel during recording.
Valid values are one of the values listed in the ChannelList property.
This property is initialized by the open method.

Goto Table85

Errors	UposException may be thrown when this property is accessed.
For further information, see “Errors" on page Intro-20.　Some possible values of the exception’s ErrorCode property are:

Value		Meaning

E_ILLEGAL	An invalid value was specified.

See Also	CapChannel Property, ChannelList Property

ChannelList Property

Syntax	ChannelList: string {read only, access after open}

Remarks	Contains the comma-delimited list of channels that is supported by the device.

For example, if the device only supports channel1and channel2 and channel4, then this property should be set to "1,2,4".
This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	Channel Property.

RecordingLevel Property

Syntax	RecordingLevel: int32 {read-write, access after open- claim-enable}

Remarks	Holds the recording level during recording.
Legal values range from zero through 100.
This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	CapRecordingLevel Property

RemainingRecordingTimeInSec Property

Syntax	RemainingRceordingTimeInSec: 				                                      int32 {read-only, access after open-claim-enable}

Remarks	This property holds the remaining recording time in seconds if a recording is ongoing. If no recording is ongoing its value is 0. When a call to method startRecording returns, this property initially holds the time passed as argument recordingTime to that call. If this argument value is FOREVER, this property also holds this value unchanged until stopRecording has been called.

This property is initialized during device setDeviceEnbaled method to 0.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	startRecording Method, stopRecording Method

SamplingRate Property

Syntax	SamplingRate: string {read-write, access after open-claim-enable}

Remarks	Holds the sampling rate during recording.
Valid values are one of the values listed in the SamplingRateList property.
This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

Goto Table86

See Also	CapSamplingRate Property, SamplingRateList Property Some possible values of the exception’s ErrorCode property are:

Value		Meaning

E_ILLEGAL		An invalid value was specified.

SamplingRateList Property

Syntax	SamplingRateList: string {read only, access after open}

Remarks	Contains the comma-delimited list of sampling rate that are supported by the device.
For example, if the device only supports 44.1kHz and 48kHz and 96kHz, then this property should be set to "44100,48000,96000".
This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	SamplingRate Property.

SoundData Property

Syntax	　SoundData: binary { read-only, access after open }

Remarks　	This property is used to store the sound data after the recording time elapse of startRecording method or stopRecording method is called. If no recorded sound data was available, the SoundData property will be set to zero length (or empty). Its value is set prior to a DataEvent to be enqueued. This property is initialized to zero length by the open method.

Errors	A UposException may be thrown when this property is accessed. 	       For further information, see “Errors" on page Intro-21.

See Also     startRecording Method, stopRecording Method, DataEvent.

SoundType Property

Syntax	SoundType: string {read-write, access after open, claim, enable}

Remarks	Holds the audio file format to be recorded.
Valid values are one of the values listed in the CapSoundTypeList property.
This property is initialized by the open method.

Goto Table87

Errors	UposException may be thrown when this property is accessed.
For further information, see “Errors" on page Intro-20. Some possible values of the exception’s ErrorCode property are:

Value		Meaning

E_ILLEGAL	An invalid value was specified.

See Also	CapSoundType Property, CapSoundTypeList Property.

SoundTypeList Property

Syntax	SoundTypeList: string {read only, access after open}

Remarks		Contains the comma-delimited list of sound file type that is supported by the device.
For example, if the device only supports WAV and OGG, then this property should be set to "WAV,OGG".
This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed.
For further information, see “Errors" on page Intro-20.

See Also	SoundType Property.

Storage Property

Syntax	Storage: int32 {read-write, access after open-claim-enable}

Remarks	This is an enumeration and defines where the device writes the recorded sound data file to. Should be set before a call to startRecording.
It holds one of the following values.

Value                                 Meaning

SREC_ST_HARDTOTALS						     The encoded data file is written to the associated Hard Totals device. The property CapAssociatedHardTotalsDevice holds the open name of the associated Hard Totals device.

SREC_ST_HOST              	The encoded data file is written to the host’s file system.

SREC_ST_HOST_HARDTOTALS

The encoded data file is written to the associated Hard Totals device and host’s file system. The property CapAssociatedHardTotalsDevice holds the open name of the associated Hard Totals device.

This property is initialized by the open method according to the value hold by CapStorage. If CapStorage has the value SREC_CST_ALL, it is initialized to SREC_ST_HOST_HARDTOTALS.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

Value	Meaning

E_ILLEGAL	An invalid value was specified, or recording is ongoing.

See Also	CapStorage Property, CapAssociatedHardTotalsDevice Property

Methods(UML operations)

startRecording Method

Syntax	startRecording (fileName: string, overWrite: boolean, 	recordingTime: int32):
		       void {raises-exception, use after open-claim-enable}

Parameter	Description

fileName	Specify the file name of the sound to be recorded.

Goto Table 149

overWrite	Specify the behavior when the same name file exists. If it is true it will be overwritten and if false it will raise the UPOSException.

recordingTime	Specify the time for recording in seconds. If OPOS_FOREVER (-1) is specified, recording will continue until the stopRecording method is called.

Remarks	Sound recording starts with the settings of the Channel property, SamplingRate property, and RecordingLevel property and need to set DataEventEnabled property to true. At the same time, recording format setting starts with the SoundType property. When this method is called, if specified recording time is elapsed, recording process will be ended and recorded sound data is provided at the SoundData property that the application may read it and / or process the stored sound data file given as filename argument. When the DataEventEnabled property is true, the DataEvent is enqueued and delivered to the application. StatusUpdateEvent with state SREC_SUE_START_SOUND_RECORDING is evoked when startRecording method is executed to notify the application, the recording has started. When the sound recording is finished, if the specified time of startRecording method has elapsed or stopRecording method has been called, the value of StatusUpdateEvent with state SREC_SUE_STOP_SOUND_RECORDING is evoked to notify the application, the recording has stopped

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	FileName is too long or contains characters that cannot be used, or 0 is specified for RecordingTime.

E_EXISTS	FileName already exists. (When OverWrite is FALSE)

E_BUSY	It cannot be executed as it is recording.

See Also	Channel Property, SamplingRate Property, SoundData Property, SoundType Property, RecordingLevel Property, stopRecording Method, StatusUpdateEvent Event

stopRecording Method

Syntax	stopRecording ():
			void {raises-exception, use after open-claim-enable}

Remarks	When this method is called the sound recording process that started by startRecording method is ended and the recording is finished. This method is processed synchronously. After recording and decoding process has been finished, the recorded sound data will be provided at the SoundData property prior to the Data Event is enqueued, when DataEventEnabled property is true. When stopRecording method is called, a StatusUpdateEvent with status SREC_SUE_STOP_SOUND_RECORDING is evoked to notify the application, the recording has stopped.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20 Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	It is not recorded.

See Also	StartRecording Property, SoundData Property, StatusUpdateEvent. Event

Events(UML interfaces)

DataEvent

<<event>>	upos::events::DataEvent

Status			:int32{read-only}

Description 	Notifies the application when data from the Sound Recorder  device is available to be read.

Attributes	This event contains the following attributes:

Attribute	Type		Description

Status	int32		Set to 0.

Remarks	Before this event is delivered, the Sound Recorder information is enqueued into the area that is indicated by the startRecording method.  Since the stored sound recorder device information might be managed by the associated “Hard Totals” device service, therefore, the application might also support the “Hard Totals” service.

See Also        Channel Property, SamplingRate Property, SoundType property, RecordingLevel Property, stopRecording Method, startRecording Method

DirectIOEvent

<<event>>	upos::events::DirectIOEvent

EventNumber	: int32 {read-only}
Data			: int32 {read-write}
Obj			: object {read-write}

Description 	Provides Service information directly to the application. This event provides a means for a vendor-specific Individual Recognition Service to provide events to the application that are not otherwise supported by the device control.

Attributes	This event contains the following attributes:

Attribute	Type	Description

EventNumber	int32	Event number whose specific values are assigned by the Service.

Data	int32	Additional numeric data. Specific values vary by the EventNumber and the Service. This attribute is settable.

Obj	object	Additional data whose usage varies by the EventNumber and the Service. This attribute is settable.

Remarks	This event is to be used only for those types of vendor specific functions that are not otherwise described.

Use of this event may restrict the application program programform being used with other vendor’s devices which may not have any knowledge of the Service’s need for this event.

See Also	"Events" on page Intro-19, directIO method.

ErrorEvent

Goto Table88

<<event>> 	upos::events:: ErrorEvent
ErrorCode			: int32{read-write}
ErrorCodeExtended	: int32{read-write}
ErrorLocus 		: int32{read-write}
*pErrorResponse		: int32{read-write}

Description	Notifies the application that a Sound Recorder Device error has been detected and suitable response by the application is necessary to process the error condition.

Attributes  This event contains following attributes.

Attributes	Type	 Description

Error Code	int32	Error Code causing the error event. 			See the list of Error Code.

ErrortCodeExtended	int32	Error Code causing the error event. 			These values are device category    			specific.

ErrorLocus	int32	Location of the error. See values below.

pErrorResponse	int32	Pointer to the error event response.			See ErrorResponse values below.

The ErrorLocus attribute has one of the following values:

Value	Meaning

EL_INPUT 	Error occurred while gathering or Processing event-driven input. No previously buffered input data is available.

EL_INPUT_DATA	Error occurred while gathering or processing

event-driven input, and some previously buffered

data is available.

If ResultCode is E_EXTENDED, ResultCodeExtended is set to one of the following values.

Value	Meaning

ESREC_NOROOM	There is not enough space to store the data file.

The application’s error event handler can set the ErrorResponse attribute to one of the following values:

Value	 	Meaning

ER_CLEAR	I will try its asynchronous output again.

The error condition is exited.

ER_CONTINUEINPUT

Only valid when the locus is EL_INPUT_DATA.					Acknowledges that a data error has occurred and directs 				the Device to continue input processing. The Device 				remains in the error state and will deliver additional 				DataEvents as directed by the DataEventEnabled 				property. When all input has been delivered and 					DataEventEnabled is again set to true, then another 				ErrorEvent is delivered with locus EL_INPUT. 					This is the default response when the locus is 					EL_INPUT_DATA.

Remarks  	    It notifies you when an error is detected during recording. Input error events   	are 	not delivered until DataEventEnabled is true, so that proper application  	sequencing occurs.

See Also	Status, Error code, State model, “Device Input Model” on page Intro-22, “Error Handling” on page Intro-23

StatusUpdateEvent

<<event>>	upos::events:: StatusUpdateEvent
Status			: int32 {read-only}

Description	Notifies the application that there is a change in the power status or a status of the Sound Recorder device.

Attributes		This event contains the following attribute:

Attributes	Type	Description

Status                   int32	Indicates a change in the power status or a status of the unit.

Note that Release 1.3 added Power State Reporting with additional Power 	reporting StatusUpdateEvent values.

The Update Firmware capability added additional Status values for communicating the status/progress of an asynchronous update firmware process. See “StatusUpdateEvent” description on page 1-34.

Value            		Meaning

SREC_SUE_START_SOUND_RECORDING									It will be notified when sound recording starts.

SREC_SUE_STOP_SOUND_RECORDING									It will be notified when sound recording stops.

Remarks  	Enqueued when the Sound Recorder Device detects a power state change or a status change.

See Also	“Events" on page Intro-19.

Chapter 42

Voice Recognition

This Chapter defines the Voice Recognition device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.16 | open |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.16 | open |
| CapPowerReporting: | int32 | {read-only} | 1.16 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.16 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.16 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.16 | open |
| CheckHealthText: | string | {read-only} | 1.16 | open |
| Claimed: | boolean | {read-only} | 1.16 | open |
| DataCount: | int32 | {read-only} | 1.16 | open |
| DataEventEnabled: | boolean | {read-write} | 1.16 | open |
| DeviceEnabled: | boolean | {read-write} | 1.16 | open & claim |
| FreezeEvents: | boolean | {read-write} | 1.16 | open |
| OutputID: | int32 | {read-only} | 1.16 | Not supported |
| PowerNotify: | int32 | {read-write} | 1.16 | open |
| PowerState: | int32 | {read-only} | 1.16 | open |
| State: | int32 | {read-only} | 1.16 | -- |
| DeviceControlDescription: | string | {read-only} | 1.16 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.16 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.16 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.16 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.16 | open |
| PhysicalDeviceName: | string | {read-only} | 1.16 | open |

Goto Table91

Goto Table90

Goto Table89

| Properties (Continued) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Specific | Type | Mutability | Version | May Use After |  |
| CapLanguage: | boolean | {read-only} | 1.16 | open |  |
| HearingDataPattern: | string | {read-only} | 1.16 | open, claim & enable |  |
| HearingDataWord: | string | {read-only} | 1.16 | open, claim & enable |  |
| HearingDataWordList: | string | {read-only} | 1.16 | open, claim & enable |  |
| HearingResult: | int32 | {read-only} | 1.16 | open, claim & enable |  |
| HearingStatus: | int32 | {read-only} | 1.16 | open, claim & enable |  |
| LanguageList: | string | {read-only} | 1.16 | open |  |
| Me Methods (UML operations) |  |  |  |  |  |
| Common |  |  |  |  |  |
| Name |  |  |  |  | Version |
| open (logicalDeviceName: string): void {raises-exception} |  |  |  |  | 1.16 |
| close ( ): void {raises-exception, use after open} |  |  |  |  | 1.16 |
| claim ( timeout: int32 ): void {raises-exception, use after open} |  |  |  |  | 1.16 |
| release ( ): void {raises-exception, use after open, claim} |  |  |  |  | 1.16 |
| checkHealth ( level: int32 ): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| clearInput ( ): void {raises-exception, use after open, claim} |  |  |  |  | 1.16 |
| clearInputProperties ( ): void {raises-exception, use after open, claim} |  |  |  |  | 1.16 |
| clearOutput ( ): void { } |  |  |  |  | Not supported |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} |  |  |  |  | 1.16 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| retrieveStatistics ( inout statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| Me Methods (UML operations)(continued) |  |  |  |  |  |
| Common |  |  |  |  |  |
| Name |  |  |  |  | Version |
| updateFirmware ( firmwareFileName: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| updateStatistics ( statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| Specific |  |  |  |  |  |
| Name |  |  |  |  |  |
| startHearingFree (language: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| startHearingSentence (language: string, wordList: string, patternList: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| startHearingWord (language: string, wordList: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| startHearingYesNo (language: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| stopHearing ( ): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |

| Events (UML interfaces) |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Mutability | Version |
| upos::events::DataEvent |  |  | 1.16 |
| Status: | int32 | {read-only} |  |
| upos::events::DirectIOEvent |  |  | 1.16 |
| EventNumber: | int32 | {read-only} |  |
| Data: | int32 | {read-write} |  |
| Obj: | object | {read-write} |  |
| upos::events::ErrorEvent |  |  | 1.16 |
| ErrorCode: | int32 | {read-only} |  |
| ErrorCodeExtended: | int32 | {read-only} |  |
| ErrorLocus: | int32 | {read-only} |  |
| ErrorResponse: | int32 | {read-write} |  |
| upos::events::OutputCompleteEvent |  | Not supported |  |
| upos::events::StatusUpdateEvent |  |  | 1.16 |
| Status: | int32 | {read-only} |  |
| upos::events::TransitionEvent |  | Not supported | 1.16 |

General Information

The Voice Recognition programmatic name is "Voice Recognition".

Capabilities

The Voice Recognition has the following capability:

- Convert spoken words to strings.

Voice Recognition Class Diagram

The following diagram shows the relationships between the Voice Recognition classes.

Fig. Chap. 42-1 Voice Recognition Class Diagram

Model

The Voice Recognition follows the general “Device Input Model” for event-driven input:

Device control starts voice recognition with the startHearingYesNo method, startHearingSentence method, etc., and generates DataEvent when recognizing voice.

If the AutoDisable property is true, then the device automatically disables itself when a DataEvent is enqueued.

An enqueued DataEvent can be delivered to the application when the DataEventEnabled property is true and other event delivery requirements are met. Just before delivering this event, data is copied into corresponding properties, and further data events are disabled by setting DataEventEnabled to false. This causes subsequent input data to be enqueued while the application processes the current input and associated properties. When the application has finished processing the current input and is ready for more data, it reenables events by setting DataEventEnabled to true.

An ErrorEvent (or events) is enqueued if an error occurs while gathering or processing input, and is delivered to the application when DataEventEnabled is true and other event delivery requirements are met.

The DataCount property may be read to obtain the total number of enqueued DataEvents.

All enqueued input may be deleted by calling clearInput method. See the clearInput method description for more details.

All data properties that are populated as a result of firing a DataEvent or ErrorEvent can be set back to their default values by calling the clearInputProperties method.

The application will be informed about any status change with a StatusUpdateEvent, also all corresponding status properties will be updated before event delivery.

Types of voice recognition

Voice recognition is mainly a method of specifying word candidates to be recognized and waiting for those words.

There are the following four types of voice recognition.

Yes/No/Cancel recognition

It listens to the sound of words classified as Yes / No / Cancel defined by the device.

For example, the voice ""OK."" is classified as Yes.

The recognized content is set in the HearingDataWord property.

For details, refer to the startHearingYesNo method.

Word recognition

The application specifies a list of words and listens for the voice of that word.

The recognized content is set in the HearingDataWord property.

For details, refer to the startHearingWord method.

Sentence recognition

The application specifies a word and a list of patterns of the sentences using it and awaits the sound of the sentence.

The recognized content is set in the HearingDataWordList property, HearingDataPattern property.

For details, see the startHearingSentence method.

Free recognition

Voice recognition leave to the device is performed without specifying the word to wait.

It does not specify waiting words and performs voice recognition entrusted to the device.

The recognized content is set in the HearingDataWord property.

For details, see the startHearingFree method.

When recognizing voice, the kind of recognition was stored in the HearingResult property.

Device Sharing

The Voice Recognition is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before accessing some properties or calling methods that update the device.

- See the “Summary” table for precise usage prerequisites.

Properties (UML attributes)

CapLanguage Property

Syntax	CapLanguage: boolean {read-only, access after open}

Remarks	If true, the application can change the language. If false, the application cannot change the language.
This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

HearingDataPattern Property

Syntax	HearingDataPattern: string {read-only, access after open-claim-enable}

Remarks	The pattern ID recognized by the startHearingSentence method is set.
This property is set by the device control just before the DataEvent is  enqueued.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	startHearingSentence Method

HearingDataWord Property

Syntax	HearingDataWord: string {read-only, access after open-claim-enable}

Remarks	The content of voice recognition is set.
This property is set as input data of the following method. To know which method it is for, check the HearingResult property.

Methods	Meaning

startHearingYesNo Method

The recognized word is set.

startHearingWord Method

Recognized words are set among the word candidates

specified by the startHearingWord method.

startHearingFree Method

Recognized words and sentences are set.

The alphabet 's uppercase letters, Japanese kanji,

hiragana, katakana, etc., the contents to be set varies

depending on the device.

This property is set by the device control just before the DataEvent is enqueued.

Errors	A UposException may be thrown when this property is accessed.          	      For further information, see “Errors" on page Intro-20.

See Also	HearingResult Property, startHearingYesNo Method,

startHearingWord Method, startHearingFree Method

HearingDataWordList Property

Syntax	HearingDataWordList: string {read-only, access after open-claim-enable}

Remarks	Comma-separated list of word information recognized by the startHearingSentence method.
Each word information consists of the following information and is shown in the following order separated by a colon (":").

Parameter	Description

WordGoupID	Recognized word group ID

Word	Recognized words. The content defined in the word

group is set.

For example, in the startHearingSentence method, set candidates as follows, Word list:"item:coffee:tea, count:a:two:three"

Pattern list: "P1:[count] cup of [item], P2:[item]"
startHearingSentence ("en-US", "item:coffee:tea, count:a:two", "P1:[count] cup of [item],P2:[item]")

If you speak "Give me two cups of coffee", device recognize “Pattern” as "P1" and “WordList” as "item:coffee, count:two".

The properties are set as follows,  		                    HearingDataPattern="P1";  		                    HearingDataWordList="item:coffee, count:two";

This property is set by the device control just before the DataEvent is enqueued.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	startHearingSentence Method

HearingResult Property

Syntax	HearingResult: int32 {read-only, access after open-claim-enable}

Remarks	A value indicating the voice recognition result is set.
The parameters to be set are as follows.

Value	Meaning

VRCG_HRESULT_YESNO_YES

Voice recognition result of StartHearingYesNo 	methods. Also, Device got an answer that is classified

as YES. The recognized content is set in the          HearingDataWord 	property.

VRCG_HRESULT_YESNO_NO

Voice recognition result of startHearingYesNo method. Also, Device got an answer that is classified as NO. The recognition content is set in the   	HearingDataWord property.

VRCG_HRESULT_YESNO_CANCEL

Voice recognition result of startHearingYesNo

method. Also, Device got responses that are classified

as CANCEL. The recognition content is set in the

HearingDataWord property.

VRCG_HRESULT_WORD

Recognition result of startHearingWord method.  The recognition content is set in the HearingDataWord property.

VRCG_HRESULT_SENTENCE

Recognition result of startHearingSentence method.

The recognition content is set in the

HearingDataWordList property and

HearingDataPattern property.

VRCG_HRESULT_FREE

Recognition result of startHearingFree method. The

recognition content is set in the HearingDataWord

property.

This property is set by the device control just before the DataEvent is enqueued.

Errors	A UposException may be thrown when this property is accessed.
For further information, see “Errors" on page Intro-20.

See Also	HearingDataWord Property, HearingDataWordList Property, HearingDataPattern Property, startHearingYesNo Method, startHearingWord Method, startHearingSentence Method, startHearingFree Method.

HearingStatus Property

Syntax	HearingStatus: int32 {read-only, access after open-claim-enable}

Remarks	A value indicating the voice recognition status is set.

Value		Meaning

VRCG_HSTATUS_NONE

Voice recognition is not running.

VRCG _HSTATUS_YESNO

Voice recognition by the startHearingYesNo  	method is in progress.

VRCG _HSTATUS_WORD

Voice recognition by the 	startHearingWord 	method is in progress.

VRCG _HSTATUS_SENTENCE

Voice recognition by the 	startHearingSentence 	method is in progress.

VRCG _HSTATUS_FREE

Voice recognition by the startHearingFree

method is in progress.

This property is initialized by the open method. Also, it is set by the device control just before the voice recognition state changes.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	startHearingYesNo Method, startHearingWord Method, startHearingSentence Method, startHearingFree Method

LanguageList Property

Syntax	LanguageList: string {read-only, access after open}

Remarks	Contains the comma-delimited list of language that are supported by the device.
The value representing the language is a value consisting of the language and country code defined in RFC 4664.
For example, when the device supports US / English, Japan / Japanese, it will be as follows.
"en-US, ja-JP"
This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	startHearingYesNo Method, startHearingWord Method, startHearingSentence Method, startHearingFree Method.

Methods (UML operations)

startHearingFree Method

Syntax	startHearingFree (language: string):

void {raises-exception, use after open-claim-enable}

Parameter	Description

Language	Specify the language to recognize. Specify one of the

values listed in the LanguageList property.

Remarks	This method can make a voice recognition from the listed language in the LanguageList property. In addition, this method can be called without specifying the word candidate to be recognized from the application, however recognized word depends on the word recognizing device capability.  When this method is called, proper values are set in the HearingDataWord property, HearingResult property and HearingStatus property just before the DataEvent issuing.  This method is executed asynchronously. Voice recognition ends when stopHearing method is called.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value		Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported language was specified.

E_BUSY	Voice recognition in progress so it cannot be executed.

See Also	HearingDataWord Property, HearingResult Property, HearingStatus Property, LanguageList Property, stopHearing Method.

startHearingSentence Method

Syntax	startHearingSentence (language: string, wordList: string,           patternList: string):

void {raises-exception, use after open-claim-enable}
Parameter	Description

language	Specify the language to recognize. Specify one of the values listed in the LanguageList property.

wordList	Specify word candidates to be waited on in a comma-separated list.

patternList	Specify the sentence pattern information to be waited for in a comma-separated list.

Each word information specified in wordList consists of the following information and is shown in the following order, separated by a colon (":").

Parameter	Description

wordGroupID	ID to identify word list

wordList	A word candidate to be waited for being separated by a colon (":")

For example, to specify word candidates "one" and "two" for word candidate’s "coffee" "tea" and word group "number" in the single item group "product", specify as follows. "item:coffee:tea, number:one:two"

Each word information specified in patternList consists of the following information, and it is shown in the following order separated by a colon (":").

Parameter	Description

patternID	ID to identify the pattern

pattern	A sentence pattern to wait. To add the word list specified in wordList to the candidate, enclose the word group ID with "[" and "]". Example: "[word group ID1]" [word group ID2] "

Example: You can order coffee or tea. You can also specify how many cups you need. If you want to recognize it by voice, do as follows.

Set the startHearingSentence method parameter as follows:
WordList:"item:coffee:tea, count:a:two:three" 			              Coffee, Tea	         -> item:coffee:tea
How many cups   -> count:a:two:three

Invoke the method.
startHearingSentence ("en-US", "item:coffee:tea,count:a:two", "P1:[count] cup of [item],P2:[item]")
HearingStatus=VRCG_HSTATUS_SENTENCE;

People talk to "Give me two cups of coffee"

Speech recognition is performed, properties are set, and an event is notified.
HearingResult=VRCG_HRESULT_SENTENCE;
HearingDataPattern="P1";
HearingDataWordList="item:coffee,count:two";
raise DataEvent(0);

Remarks       This method can make a voice recognition from the listed language in the 	        	     LanguageList property. In addition, this method can recognize the words and 	                   sentences that are defined in wordList and patternList as parameter. When this 	       	      method is called, proper values are set in the HearingDataWord property. 	       	      HearingResult property and HearingStatus property, just before DataEvent 	       	      issuing. This method is executed asynchronously. Voice recognition ends when 	        	      stopHearing method is called.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported

language was specified.

E_BUSY	Voice recognition in progress so it cannot be 		executed.

See Also	HearingDataWord Property, HearingResult Property, HearingStatus Property, LanguageList Property, stopHearing Method

startHearingWord Method

Syntax	startHearingWord (language: string, wordList: string):

void {raises-exception, use after open-claim-enable}
Parameter		     Description

language	Specify the language to recognize. Specify one of the values listed in the LanguageList property.

wordList	Specify word candidates to be waited on in a comma-separated list.
Example: "word1, word2, word3"

Remarks        This method can make a voice recognition from the listed language in the

LanguageList property. In addition, this method can recognize the words that

are defined in wordList as parameter. When this method is called, proper

values are set in the HearingDataWord property, HearingResult property and

HearingStatus property just before DataEvent issuing.

This method is executed asynchronously.

Voice recognition ends when stopHearing method is called.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported

language was specified.

E_BUSY	Voice recognition in progress so it cannot be

executed.

See Also	HearingDataWord Property, HearingResult Property, HearingStatus Property, LanguageList Property, stopHearing Method.

startHearingYesNo Method

Syntax	startHearingYesNo (language: string):

void {raises-exception, use after open-claim-enable}
Parameter		Description

language                    Specify the language to recognize. Specify one of the

values listed in the LanguageList property.

Remarks	This method can make a voice recognition from the listed language in the LanguageList property. In addition, this method can recognize the words that are defined in the device as the recognition candidate corresponding to "Yes" "No" "Cancel".  When this method is called, proper values are set in the HearingDataWord property, HearingResult property and HearingStatus property, just before DataEvent issuing.   This method is executed asynchronously. Voice recognition ends when stopHearing method is called.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported

language was specified.

E_BUSY	  Voice recognition in progress so it cannot be	      	executed.

See Also	LanguageList Property, HearingDataWord Property, Hearing Result Property, LanguageList Property, stopHearing Method.

stopHearing Method

Syntax	stopHearing ( ):

void {raises-exception, use after open-claim-enable}

Remarks         Voice Recognition ends when this property called.

This method is executed synchronously.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported

language was specified.

Events (UML interfaces)

DataEvent

<<event>>	upos::events::DataEvent

Status			: int32{read-only}

Description 	Notifies the application when data from the Voice Recognition device is available to be read.

Attributes	This event contains the following attributes:

Attribute	Type		Description

Status	int32		Set to 0.

Remarks	Before this event is delivered, the voice recognition information is enqueued into the area that is indicated by the startHearingXXX kinds of methods.

See Also	HearingResult Property, “Events" on page Intro-19, StartHearingYesNo Method,  StartHearingWord Method, StartHearingSentence Method, StartHearingFree Method, directIO Method.

DirectIOEvent

<<event>>	upos::events::DirectIOEvent

EventNumber	: int32 {read-only}
Data			: int32 {read-write}
Obj			: object {read-write}

Description 	Provides Service information directly to the application. This event provides a means for a vendor-specific Voice Recognition Service to provide events to the application that are not otherwise supported by the device control.

Attributes	This event contains the following attributes:

Attribute	Type	Description

EventNumber	int32	Event number whose specific values are assigned by the Service.

Data	int32	Additional numeric data. Specific values vary by the EventNumber and the Service. This attribute is settable.

Obj	object	Additional data whose usage varies by the EventNumber and the Service. This attribute is settable.

Remarks	This event is to be used only for those types of vendor specific functions that are not otherwise described.

Use of this event may restrict the application program programform being used with other vendor’s devices which may not have any knowledge of the Service’s need for this event.

See Also	“Events" on page Intro-19, directIO Method.

ErrorEvent

<<event>> 	upos::events:: ErrorEvent
ErrorCode			: int32{read-write}
ErrorCodeExtended	: int32{read-write}
ErrorLocus 		: int32{read-write}
ErrorResponse		: int32{read-write}

Description	Notifies the application that a Voice Recognition Device error has been detected and suitable response by the application is necessary to process the error condition.

Attributes	This event contains the following attributes:

Attributes	Type	Description

ErrorCode	int32	Error code causing the error event. 				See a list of Error Codes on page 20.

ErrorCodeExtended	int32	Extended Error code causing the error event.

If ErrorCode is E_EXTENDED, then see values below. Otherwise, it may contain a Service-specific value.

ErrorLocus	int32	Location of the error. See values below. If EL_OUTPUT is specified. An error occurred during asynchronous action.

Goto Table92

ErrorResponse	int32	Error response, whose default value may              be overridden by the application  (i.e., this attribute is settable). 	See values below.

The ErrorLocus attribute has one of the following values:

Value	Meaning

EL_OUTPUT	Error occurred while processing asynchronous output.

EL_INPUT	Error occurred while gathering or processing event-driven input. No previously buffered input data is available.

EL_INPUT_DATA	Error occurred while gathering or processing event-driven input, and some previously buffered data is available.

The application’s error event handler can set the ErrorResponse attribute to one of the following values:

Value	Meaning

ER_RETRY	Retry sending the data. The error state is exited.
May be valid for some input devices when the locus is EL_INPUT, in which case the input is re-tried, and the error state is exited. Typically, valid for asynchronous output devices when the locus is EL_OUTPUT, in which case the asynchronous output is re-tried, and the error state is exited. This is the default response when the locus is EL_OUTPUT.

ER_CLEAR	Valid for all locusloci: EL_INPUT, EL_INPUT_DATA, and EL_OUTPUT.  Clear all buffered input or output data (including all asynchronous output).  The error state is exited.  This is the default response when the locus is EL_INPUT.

ER_CONTINUEINPUT 
Only valid when the locus is EL_INPUT_DATA.
Acknowledges that a data error has occurred and directs the Device to continue input processing. The Device remains in the error state and will deliver additional DataEvents as directed by the DataEventEnabled property. When all input has been delivered and DataEventEnabled is again set to true, then another ErrorEvent is delivered with locus EL_INPUT.
This is the default response when the locus is EL_INPUT_DATA.

Remarks	This event is enqueued when an error is detected and the Device’s State transitions into the error state. Input error events are not delivered until DataEventEnabled is true, so that proper application sequencing occurs.

Unlike a DataEvent, the Device does not disable further DataEvents or input ErrorEvents; it leaves the DataEventEnabled property value at true.    Note that the application may set DataEventEnabled to false within its event handler if subsequent input events need to be disabled for a period of time.

See Also 	“Device Input Model" on page Intro-22, “Error Handling" on page Intro-23, “Device Output Models" on page Intro-25.

StatusUpdateEvent

<<event>>	upos::events:: StatusUpdateEvent
Status			: int32 {read-only}

Description	Notifies the application that there is a change in the power status or a status of the Voice  Recognition device.

Attributes		This event contains the following attribute:

Attributes	Type	Description

Status	int32	Indicates a change in the power status of the unit.

Note that Release 1.3 added Power State Reporting with additional Power reporting StatusUpdateEvent values.

The Update Firmware capability added additional Status values for communicating the status/progress of an asynchronous update firmware process. See “StatusUpdateEvent” description on page 1-34.

Value			Meaning

VRCG_SUE_START_HEARING _FREE
					It will be notified when hearing free starts.

VRCG_SUE_START_HEARING _SENTENCE
					It will be notified when hearing sentence starts.

VRCG_SUE_START_HEARING _WORD
				 	It will be notified when hearing word starts.

VRCG_SUE_START_HEARING _YESNO
					It will be notified when hearing yesno starts.

VRCG_SUE_STOP_HEARING
					It will be notified when hearing stops.

Remarks	Enqueued when the Voice Recognition Device detects a power state change or a status change.

See Also 	“Events" on page Intro-19.

Chapter 43

Sound Player

This Chapter defines the Sound Player device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.16 | Not supported |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.16 | open |
| CapPowerReporting: | int32 | {read-only} | 1.16 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.16 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.16 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.16 | open |
| CheckHealthText: | string | {read-only} | 1.16 | open |
| Claimed: | boolean | {read-only} | 1.16 | open |
| DataCount: | int32 | {read-only} | 1.16 | Not supported |
| DataEventEnabled: | boolean | {read-write} | 1.16 | Not supported |
| DeviceEnabled: | boolean | {read-write} | 1.16 | open & claim |
| FreezeEvents: | boolean | {read-write} | 1.16 | open |
| OutputID: | int32 | {read-only} | 1.16 | open |
| PowerNotify: | int32 | {read-write} | 1.16 | open |
| PowerState: | int32 | {read-only} | 1.16 | open |
| State: | int32 | {read-only} | 1.16 | -- |
| DeviceControlDescription: | string | {read-only} | 1.16 | - |
| DeviceControlVersion: | int32 | {read-only} | 1.16 | - |
| DeviceServiceDescription: | string | {read-only} | 1.16 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.16 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.16 | open |
| PhysicalDeviceName: | string | {read-only} | 1.16 | open |

| Properties (Continued) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specific | Type | Mutability | Version | May Use After |
| CapAssociatedHardTotalsDevice | string | {read-write} | 1.16 | open |
| CapMultiPlay: | boolean | {read-only} | 1.16 | open |
| CapSoundTypeList: | string | {read-only} | 1.16 | open |
| CapStorage | int32 | {read-only} | 1.16 | open |
| CapVolume: | boolean | {read-only} | 1.16 | open |
| DeviceSoundList: | string | {read-only} | 1.16 | open |
| OutputIDList: | string | {read-only} | 1.16 | open, claim & enable |
| Storage | int32 | {read-write} | 1.16 | open, claim & enable |
| Volume: | int32 | {read-write} | 1.16 | open, claim & enable |

Goto Table94

Goto Table95

Goto Table93

| Me Methods (UML operations) |  |
| --- | --- |
| Common |  |
| Name | Version |
| open (logicalDeviceName: string): void {raises-exception} | 1.16 |
| close ( ): void {raises-exception, use after open} | 1.16 |
| claim (timeout: int32): void {raises-exception, use after open} | 1.16 |
| release ( ): void {raises-exception, use after open, claim} | 1.16 |
| checkHealth (level: int32): void {raises-exception, use after open, claim, enable} | 1.16 |
| clearInput ( ): void {raises-exception, use after open, claim } | 1.16 |
| clearInputProperties ( ): void {raises-exception, use after open, claim } | 1.16 |
| clearOutput ( ): void { } | Not supported |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} | 1.16 |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, claim, enable} | 1.16 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | 1.16 |
| retrieveStatistics ( inout statisticsBuffer: string ): void {raises-exception, use after open, claim, enable} | 1.16 |
| Me Methods (UML operations)(continued) |  |
| Common |  |
| Name | Version |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, claim, enable} | 1.16 |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | 1.16 |
| Specific |  |
| Name | Version |
| playSound (fileName: string, loop: boolean): void {raises-exception, use after open, claim, enable} | 1.16 |
| stopSound(outputID:int32): void {raises-exception, use after open, claim, enable} | 1.16 |

| Events (UML interfaces) |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Mutability | Version |
| upos::events::DataEvent |  | Not supported | 1.16 |
| upos::events::DirectIOEvent |  |  | 1.16 |
| EventNumber: | int32 | {read-only} |  |
| Data: | int32 | {read-write} |  |
| Obj: | object | {read-write} |  |
| upos::events::ErrorEvent |  |  | 1.16 |
| ErrorCode: | int32 | {read-only} |  |
| ErrorCodeExtended: | int32 | {read-only} |  |
| ErrorLocus: | int32 | {read-only} |  |
| ErrorResponse: | int32 | {read-write} |  |
| upos::events::OutputCompleteEvent |  |  | 1.16 |
| OutputID: | int32 | {read-only} |  |
| upos::events::StatusUpdateEvent |  |  | 1.16 |
| Status: | int32 | {read-only} |  |
| upos::events::TransitionEvent |  | Not supported | 1.16 |

General Information

The Sound Player programmatic name is "Sound Player".

Capabilities

The Sound Player has the following capability:

- Play audio file.

Sound Player Class Diagram

The following diagram shows the relationships between the Sound player classes.

Fig. Chap.43-1 Sound Player Class Diagram

Model

The Sound Player follows the general device behavior model for asynchronous output devices:

- The Device validates the method parameters and produces an error condition immediately if necessary. If the validation is successful, the Device does the following:

- Audio files will be played sequentially. When playSound method is called, device starts the playing sound that is specified by the method parameters and the requested sound file data placed in a queue and corresponding OutputID is stored at OutputID property and added to the OutputIDList property as a listed value. And sets the OutputID property to a unique integer identifier for this request.

- When the sound playing starts StatusUpdateEvent is evoked as the value of  SPLY_SUE_START_PLAY_SOUND. 					   When the sound playing is finished an OutputCompleteEvent is enqueued for the delivery to the application and corresponding OutputID is stored in OutputID property. At the same time, StatusUpdateEvent is evoked as the value of SPLY_SUE_STOP_PLAY_SOUND. The application should compare the returned OutputCompleteEvent property OutputID value with the OutputID value set by the asynchronous process method call used to send the data in order to track what data has been successfully sent to the device.

- When stopSound method is called, device stop the playing sound according to the OutputID property value and the current playing sound is terminated and enqueued sound file data is cleared. After this method is executed, corresponding OutputID property and OutputIDList values are not changed. No OutputCompleteEvent is fired and only StatusUpdateEvent will be evoked the value of SPLY_SUE_STOP_PLAY_SOUND.

- If an error occurs while processing a request, an ErrorEvent is enqueued which will be delivered to the application after the events already enqueued, including OutputCompleteEvent. No further asynchronous output will occur until the event has been delivered to the application. If the response is ER_CLEAR, then outstanding asynchronous output is cleared. If the response is ER_RETRY, then output is retried; note that if several outputs were simultaneously in progress at the time that the error was detected, then the Service may need to retry all of these outputs.

- Asynchronous output is always performed on a first-in first-out basis. If the device supports concurrent playback, the request will be executed simultaneously. To check if the device supports simultaneous playback, check the CapMultiPlay property.

- If the request is terminated before completion, due to reasons such as the application calling the clearOutput method, then no OutputCompleteEvent is delivered.

- Application can also delete the output individually by calling the stopSound method. Also, in this case OutputCompleteEvent will not be notified."

- The CapSoundTypeList property lists audio file types that the device can play.

- The application will be informed about any status change with a StatusUpdateEvent, also all corresponding status properties will be updated before event delivery.

- If device supports either or both of Hard Totals devices and the host file system, the                                                                                   application should set the Storage property accordingly to tell where to access the data file.

- If device needs to be able to access the audio files played with playSound method from a Hard Totals device, the CapAssociatedHardTotalsDevice property holds the open name of the associated Hard Totals device.

Device Sharing

The Sound Player is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before accessing some properties or calling methods that update the device.

- See the “Summary” table for precise usage prerequisites.

Properties(UML attributes)

CapAssociatedHardTotalsDevice Property

Goto Table96

Syntax	CapAssociatedHardTotalsDevice: string {read-onlywrite, access after open}

Remarks	Holds the open name of the associated Hard Totals device if the device is able to write to such devices which is the case if CapStorage is either SPLY_ CST_ALL or SPLY_CST_HARDTOTALS_ONLY. If CapStorage is SPLY_CST_HOST_ONLY this property value must be the empty string.  This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	CapStorage Property

CapMultiPlay Property

Syntax	CapMultiPlay: boolean {read-only, access after open}

Remarks	If true, the application can play sound simultaneously.

If false, the application cannot play sound simultaneously.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	playSound Method.

CapSoundTypeList Property

Syntax	CapSoundTypeList: string {read-only, access after open}

Remarks	Contains the comma-delimited list of file type that is supported by the device.

For example, if the device only supports WAV and OGG, then this property should be set to “WAV, OGG". This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	playSound Method

## CapStorage Property

Syntax	CapStorage: int32 {read-only, access after open}

Goto Table97

Remarks	This is an enumeration and announces where the device is able to write the recorded sound data file to.
It holds one of the following values.

Value	Meaning

SPLY_ CST_HARDTOTALS_ONLY

Only an associate Hard Totals device is supported.

SPLY_CST_HOST_ONLY	Only the host’s file system is supported.

SPLY_CST_ALL	Both, the associated Hard Totals device and the host’s file system is supported.

This property is initialized by the open method.

If a Hard Totals device is supported the Storage, the property value should be SPLY_CST_HARDTOTALS_ONLY or SPLY_CST_ALL and the property CapAssociatedHardTotalsDevice holds the open name of the associated Hard Totals device.

See Also        Storage Property, CapAssociatedHardTotalsDevice Property

CapVolume Property

Syntax	CapVolume: boolean {read-only, access after open}

Remarks	If true, the application can change the volume during playback.

If false, the application cannot change the volume during playback.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	Volume Property.

DeviceSoundList Property

Syntax	DeviceSoundList: string {read-only, access after open}

Remarks	Contains the comma-delimited list of device sound ID that is supported by the device. This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	playSound Method

OutputIDList Property

Syntax	OutputIDList: string {read-only, access after open, claim-enable}

Remarks	Contains the comma-delimited list of OutputID that is output by the playSound method. This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	playSound Method

Storage Property

Syntax	Storage: int32 {read-write, access after open-claim-enable}

Remarks	It holds one of the following values.

Value                                 Meaning

SPLY_ST_HARDTOTALS

The encoded data file is written to the associated  Hard Totals device. The property CapAssociatedHardTotalsDevice holds the open name of the associated Hard Totals device.

Goto Table98

SPLY_ST_HOST              	The encoded data file is written to the host’s file system.

SPLY_ST_HOST_HARDTOTALS

The encoded data file is written to the associated Hard Totals device and host’s file system. The property CapAssociatedHardTotalsDevice holds the open name of the associated Hard Totals device.

This property is initialized by the open method according to the value hold by CapStorage. If CapStorage has the value SPLY_CST_ALL, it is initialized to SPLY_ST_HOST_HARDTOTALS.

Errors	UposException may be thrown when this property is accessed.
For further information, see “Errors" on page Intro-20. Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified or recording is ongoing.

See Also	CapStorage Property

Volume Property

Syntax	Volume : int32 {read-write, access after open-claim-enable}

Remarks	Holds the volume at playing sound.

Legal values range from zero through 100.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	playSound Method

Methods (UML operations)

playSound Method

Syntax	playSound (fileName : string, loop : boolean):
		void{raises-exception, use after open-claim-enable}

Parameter	Description

fileName	Specifies the file name of audio file. Or, specifies one of the sound ID defined by DeviceSoundList.

loop	When true is specified, loop playback is performed, and if false is specified, loop playback will not be performed.

Remarks	Play audio file specified by fileName or device definition sound.

Audio files might be located in the area managed by "Hard Totals" service.

This method will be performed asynchronously. To stop playback, call the stopSound method.

Errors	A UposException may be thrown when this method is invoked. For further information , see “Errors” on page Intro-20.  Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported sound file was specified.

E_NOEXIST	File does not exist.

See Also	CapSoundType Property, DeviceSoundList Property, stopSound Method

stopSound Method

Syntax	stopSound(outputID: int32):
		void{raises-exception, use after open-claim-enable}

Parameter	Description

outputID	Specify the outputID of the sound to stop.

Remarks	Terminates specified audio playback according to the OutputID property value.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors” on page Intro-20.  Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	The specified sound is not being played.

See Also	OutputID Property, startSound Method

Events (UML interfaces)

DirectIOEvent

<<event>>	upos::events::DirectIOEvent

EventNumber	: int32 {read-only}
Data			: int32 {read-write}
Obj			: object {read-write}

Description 	Provides Service information directly to the application. This event provides a means for a vendor-specific Sound Player Service to provide events to the application that are not otherwise supported by the device control.

Attributes	This event contains the following attributes:

Attribute	Type	Description

EventNumber	int32	Event number whose specific values are assigned by the Service.

Data	int32	Additional numeric data. Specific values vary by the EventNumber and the Service. This attribute is settable.

Obj	object	Additional data whose usage varies by the EventNumber and the Service. This attribute is settable.

Remarks	This event is to be used only for those types of vendor specific functions that are not otherwise described.

Use of this event may restrict the application program programform being used with other vendor’s devices which may not have any knowledge of the Service’s need for this event.

See Also	“Events" on page Intro-19, directIO Method

ErrorEvent

Goto Table99

<<event>> 	upos::events:: ErrorEvent
ErrorCode			: int32{read-write}
ErrorCodeExtended	: int32{read-write}
ErrorLocus 		: int32{read-write}
ErrorResponse		: int32{read-write}

Description	Notifies the application that a Sound Player Device error has been detected and suitable response by the application is necessary to process the error condition.

Attributes	This event contains the following attributes:

Attributes	Type	Description

ErrorCode	int32	Error code causing the error event. 				See a list of Error Codes on page 20.

ErrorCodeExtended	int32	Extended Error code causing the error event.

If ErrorCode is E_EXTENDED, then see values below. Otherwise, it may contain a Service-specific value.

ErrorLocus	int32	Location of the error. If EL_OUTPUT is specified. It is indicating that the error occurred while processing asynchronous recorded data storing. An error occurred during asynchronous action.

ErrorResponse	int32	Error response, whose default value may         be overridden by the application (i.e., this attribute is settable). See values below.

If ErrorCode is E_EXTENDED, then ErrorCodeExtended has one of the following values:

Value	 	       Meaning                                                                      ESPLY_NOROOM	       The encoded data storage area does not have 			       	       enough room to store.

The ErrorLocus attribute has one of the following values:

Value	Meaning

EL_OUTPUT	Error occurred while processing asynchronous output.

The application’s error event handler can set the ErrorResponse attribute to one of the following values:

Value	Meaning

ER_RETRY	Retry sending the asynchronous output data. The error state is exited. May be valid for some input devices when the locus is EL_INPUT, in which case the input is re-tried, and the error state is exited. Typically, valid for asynchronous output devices when the locus is EL_OUTPUT, in which case the asynchronous output is re-tried, and the error state is exited. This is the default response when the locus is EL_OUTPUT.

ER_CLEAR	Valid for all loci: EL_INPUT, EL_INPUT_DATA, and EL_OUTPUT.  Clear all buffered input or output data (including all asynchronous output).  (The effect is the same as when clearOutput method is called.) The error state is exited.

Remarks	This event is enqueued when an error is detected, and the Device’s State transitions into the error state.

See Also 	 “Error Handling" on page Intro-23, “Device Output Models" on page Intro-25.

OutputCompleteEvent

<<event>>	upos::events::OutputCompleteEvent 
OutputID		: int32{read-only}

Description 	Notify the application that the queued output request associated with the outputID property has completed successfully.

Attributes	This event contains the following attributes:

Attribute	Type	Description

OutputID 	int32	The ID number of the asynchronous output request that is complete.

Remarks	This event is enqueued after the request’s data has been both sent, and the Service has confirmation that it was processed by the device successfully.

See Also	“Device Output Models" on page Intro-25

StatusUpdateEvent

<<event>>	upos::events:: StatusUpdateEvent
Status			: int32 {read-only}

Description    Notifies the application that there is an operation status change or a status of the sound player device.

Attributes	This event contains the following attribute:

Attributes	    Type	      Description	    			         		Status	    int32        Indicates a change of operation status of sound player 				　  device.

Note that Release 1.3 added Power State Reporting with additional Power reporting StatusUpdateEvent values.

The Update Firmware capability added additional Status values for communicating the status/progress of an asynchronous update firmware process.

See “StatusUpdateEvent” description on page 1-34.

Value            		Meaning					SPLY_SUE_START_PLAY_SOUND									It will be notified when sound playing start.

SPLY_SUE_STOP_PLAY_SOUND										It will be notified when sound playing stop.

Remarks	Enqueued when the Sound Player Device detects a power state change or a status change.

See Also 	“Events" on page Intro-19.

Chapter 44

Speech Synthesis

This Chapter defines the Speech Synthesis device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.16 | Not supported |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.16 | open |
| CapPowerReporting: | int32 | {read-only} | 1.16 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.16 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.16 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.16 | open |
| CheckHealthText: | string | {read-only} | 1.16 | open |
| Claimed: | boolean | {read-only} | 1.16 | open |
| DataCount: | int32 | {read-only} | 1.16 | Not supported |
| DataEventEnabled: | boolean | {read-write} | 1.16 | Not supported |
| DeviceEnabled: | boolean | {read-write} | 1.16 | open & claim |
| FreezeEvents: | boolean | {read-write} | 1.16 | open |
| OutputID: | int32 | {read-only} | 1.16 | open |
| PowerNotify: | int32 | {read-write} | 1.16 | open |
| PowerState: | int32 | {read-only} | 1.16 | open |
| State: | int32 | {read-only} | 1.16 | -- |
| DeviceControlDescription: | string | {read-only} | 1.16 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.16 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.16 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.16 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.16 | open |
| PhysicalDeviceName: | string | {read-only} | 1.16 | open |

Goto Table102

Goto Table101

| Properties (Continued) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Specific |  | Type | Mutability | Version | May Use After |  |
| CapLanguage: |  | boolean | {read-only} | 1.16 | open |  |
| CapPitch: |  | boolean | {read-only} | 1.16 | open |  |
| CapSpeed: |  | boolean | {read-only} | 1.16 | open |  |
| CapVoice: |  | boolean | {read-only} | 1.16 | open |  |
| CapVolume: |  | boolean | {read-only} | 1.16 | open |  |
| Language: |  | string | {read-write} | 1.16 | open, claim & enable |  |
| LanguageList: |  | string | {read-only} | 1.16 | open |  |
| OutputIDList: |  | string | {read-only} | 1.16 | open, claim & enable |  |
| Pitch: |  | int32 | {read-write} | 1.16 | open, claim & enable |  |
| Speed: |  | int32 | {read-write} | 1.16 | open, claim & enable |  |
| Voice: |  | string | {read-write} | 1.16 | open, claim & enable |  |
| VoiceList: |  | string | {read-only} | 1.16 | open |  |
| Volume: |  | int32 | {read-write} | 1.16 | open, claim & enable |  |
| Me Methods (UML operations) |  |  |  |  |  |  |
| Common |  |  |  |  |  |  |
| Name |  |  |  |  | Version |  |
| open (logicalDeviceName: string): void {raises-exception} |  |  |  |  | 1.16 |  |
| close ( ): void {raises-exception, use after open} |  |  |  |  | 1.16 |  |
| claim ( timeout: int32): void {raises-exception, use after open} |  |  |  |  | 1.16 |  |
| release ( ): void {raises-exception, use after open, claim} |  |  |  |  | 1.16 |  |
| checkHealth (level: int32): void {raises-exception, use after open, claim, enable} |  |  |  |  | Goto Table100 1.16 |  |
| clearInput ( ): void {raises-exception, use after open, claim} |  |  |  |  | 1.16 |  |
| clearInputProperties ( ): void {raises-exception, use after open, claim} |  |  |  |  | 1.16 |  |
| Methods (UML operations)(continued) |  |  |  |  |  |  |
| clearOutput ( ): void {raises-exception, use after open, claim} |  |  |  |  | 1.16 |  |
| Common |  |  |  |  |  |  |
| Name |  |  |  |  | Version |  |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} |  |  |  |  | 1.16 |  |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |
| Specific |  |  |  |  |  |  |
| Name |  |  |  |  |  |  |
| speak (text: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |
| speakImmediate (text: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |
| stopCurrentSpeaking ( ): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |
| stopSpeaking (outputID: int32 ): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |  |

Goto Table103

| Events (UML interfaces) |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Mutability | Version |
| upos::events::DataEvent |  | Not supported |  |
| upos::events::DirectIOEvent |  |  | 1.16 |
| EventNumber: | int32 | {read-only} |  |
| Data: | int32 | {read-write} |  |
| Obj: | object | {read-write} |  |
| upos::events::ErrorEvent |  |  | 1.16 |
| ErrorCode: | int32 | {read-only} |  |
| ErrorCodeExtended: | int32 | {read-only} |  |
| ErrorLocus: | int32 | {read-only} |  |
| *pErrorResponse: | int32 | {read-write} |  |
| upos::events::OutputCompleteEvent |  |  | 1.16 |
| OutputID: | int32 | {read-only} |  |
| upos::events::StatusUpdateEvent |  |  | 1.16 |
| Status: | int32 | {read-only} |  |
| upos::events::TransitionEvent |  | Not supported |  |

General Information

The Speech Synthesis programmatic name is "Speech Synthesis".

Capabilities

The Speech Synthesis has the following capability:

- Convert text to speech and read it aloud.

Speech Synthesis Class Diagram

The following diagram shows the relationships between the Speech Synthesis classes.

Fig. Chap. 44-1 Speech Synthesis Class Diagram

Model

The Speech Synthesis follows the general device behavior model for output devices with some enhancements.

The application calls a speak method or speakImmediate method to speech.

The speak method acts to start speaking from the words specified by text, while the speakImmediate method ends immediately previous speak method, and starts speaking the word specified by text asynchronously and immediately.

When speak or speakImmediate method is called device start the speaking based on the setting value of Language, Volume, Pitch and Speed properties. And requested utterance written by text data placed in a queue and corresponding OutputID is stored at OutputID property and added to the OutputIDList property as listed value. And sets the OutputID property to a unique integer identifier for this request.

When an utterance of speak method or speakImmediate method starts, StatusUpdateEvent is evoked as the value of SPSY_SUE_START_SPEAK. When the utterance is finished an OutputCompleteEvent is enqueued for the delivery to the application and corresponding OutputID is stored in OutputID property. At the same time StatusUpdateEvent is evoked as the value of SPSY_SUE_STOP_SPEAK. The application should compare the returned OutputCompleteEvent property OutputID value with OutputID value set by the asynchronous process method call used to send the data in order to track what data has been successfully sent to the device

When speakImmediate method is called during the utterance of speak method or speakImmediate method call, utterance will be stopped immediately. And StatusUpdateEvent is evoked as the value of SPSY_SUE_STOP_SPEAK.  However, OutputCompleteEvent is not fired. And current speak method or speakImmediate method corresponding OutputID property and OutputIDList property values are not changed.

When stopCurrentSpeaking method is called, current utterance generated by speak method or speakImmediate method will be stopped and StatusUpdateEvent is evoked as the value of  SPSY_SUE_STOP_SPEAK. And no OutputCompleteEvent is fired. And current speak method or speakImmediate method corresponding OutputID property and OutputIDList property values are not changed.

When stopSpeaking method is called, specified OutputID valued utterance is stopped and deleted. And OutputID property value in the OutputIDList property is eliminated.

When utterance is stopped StatusUpdateEvent is evoked as the value of SPSY_SUE_STOP_SPEAK. And no OutputCompleteEvent is fired.

If an error occurs while processing a request, an ErrorEvent is enqueued which will be delivered to the application after the events already enqueued, including OutputCompleteEvent. No further asynchronous output will occur until the event has been delivered to the application. If the response is ER_CLEAR, then outstanding asynchronous output is cleared. If the response is ER_RETRY, then output is retried; note that if several outputs were simultaneously in progress at the time that the error was detected, then the service may need to retry all of these outputs.

Asynchronous output is always performed on a first-in first-out basis.

If the request is terminated before completion, due to reasons such as the application calling the clearOutput method, then no OutputCompleteEvent is delivered.

The application will be informed about any status change with a StatusUpdateEvent, also all corresponding status properties will be updated before event delivery.

Device Sharing

The Speech Synthesis is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before accessing some properties or calling methods that update the device.

- See the “Summary” table for precise usage prerequisites.

Properties (UML attributes)

CapLanguage Property

Syntax	CapLanguage: boolean {read-only, access after open}

Remarks	If true, the application can change the language. If false, the application cannot change the language.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	Language Property

CapPitch Property

Syntax	CapPitch: boolean {read-only, access after open}

Remarks	If true, the application can change the pitch. If false, the application cannot change the pitch.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	Pitch Property

CapSpeed Property

Syntax	CapSpeed: boolean {read-only, access after open}

Remarks	If true, the application can change the speed. If false, the application cannot change the speed.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	Speed Property

CapVoice Property

Syntax	CapVoice: boolean {read-only, access after open}

Remarks	If true, the application can change the voice. If false, the application cannot change the voice.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	Voice Property

CapVolume Property

Syntax	CapVolume: boolean {read-only, access after open}

Remarks	If true, the application can change the volume. If false, the application cannot change the volume.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	Volume Property

Language Property

Syntax	Language: string {read-write, access after open-claim-enable}

Remarks	Indicates the language to speak. Valid values are one of the values listed in the LanguageList property.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported

language was specified.

See Also	speak Method, speakImmediate Method

LanguageList Property

Syntax	LanguageList: string {read-only, access after open}

Remarks	Contains the comma-delimited list of language that are supported by the device. The value representing the language is a value consisting of the language and country code defined in RFC 4664. For example, when the device supports US / English, Japan / Japanese, it will be as follows.
"en-US, ja-JP"

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	Language Property

OutputIDList Property

Syntax	OutputIDList: string {read-write, access after open-claim-enable}

Remarks	Comma-separated list of OutputID property values of audio being played by speak method or speakImmediate method. This list indicates the capability how many and what kinds of utterance can be done by the targeted Speech Synthesis device

This property is initialized by the open method. It will also be updated as the speech request increases or decreases.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	speak Method, speakImmediate Method

Pitch Property

Syntax	Pitch: int32 {read-write, access after open-claim-enable}

Remarks	Holds the pitch at speech. Legal values range from 50% through 200%.

This property is initialized to 100% by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	speak Method, speakImmediate Method

Speed Property

Syntax	Speed: int32 {read-write, access after open-claim-enable}

Remarks	Holds the speed at speech. Legal values range from 50% through 200%.

This property is initialized to 100% by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	speak Method, speakImmediate Method

Voice Property

Syntax	Voice : string {read-write, access after open-claim-enable }

Remarks	Indicates the voice tone to speak. Valid values are one of the values listed in the VoiceList property.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported

voice was specified.

See Also	speak Method, speakImmediate Method

VoiceList Property

Syntax	VoiceList: string {read-only, access after open}

Remarks	A list of speech able voices is shown in a comma-separated list. For example, when the device supports male and female voice tones, it looks like the following.
"MALE_VOICE, FEMALE_VOICE"
(The content of the value depends on the device)

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	Voice Property

Volume Property

Syntax	Volume: int32 {read-write, access after open-claim-enable}

Remarks	Holds the volume at speech. Legal values range from zero through 100.

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	speak Method, speakImmediate Method

Methods (UML operations)

speak Method

Syntax	speak (text: string):

void {raises-exception, use after open-claim-enable}

Parameter		Description

Text	Specify the text to speak.

Remarks	Device utters after converting the specified string into speech.

The utterance is executed according to the setting contents of Language property, Volume property, Pitch property, Speed property, but by inserting the following tag in the text, it is possible to change the utterance after the tag.

Content written in text is uttered with the following parameter settings.

| Tag | Description | Value (decimal integer) | Default Value (decimal integer) |
| --- | --- | --- | --- |
| volume | Specify the volume of the uttered voice. | 1 to 100 | 50 |
| pitch | Specify the high or low of the uttered voice. | 50 to 200 | 100 |
| speed | Specify the speed of the uttered voice. | 50 to 200 | 100 |
| pause | Specify the time to pause in milliseconds. | 1 to 50000 | 1 |
| reset | Rest the effect of volume, pitch, speed to the default value. | - | - |

If dialogue is “Hello. Today, it’s nice weather.”

Then if you would like to use the default setting of speed, volume, pitch for the “Hello”. And would like to put a pose between “Hello” and “Today” 1000 milliseconds and would like to change the speaking pith of “Today” to 150 and increase the volume to 80. Then for the “It’s nice weather” would like return to the default value by using the reset. It is described as follows

Hello.{pause=1000,pitch=150,volue=80}Today,{reset}It's nice weather.

Those utterance defined as follows.

| Name | Data | Remarks |  |
| --- | --- | --- | --- |
| Utterance written by text with the speak method parameter. Text will be spoken under the assigned parameter condition. | {#=f}XXXX{#=f}YYYY | #:Tag names It is volume, pitch, speed, pause and reset. | f:Tag values It is described in the Tag Value Table. |

When this method is called by the application, device validate the method parameters, and if validation is successful buffer the request in program memory and deliver it to the device and process it. And device sets the unique integer identifier into the OutputID property. When device successfully complete a request an OutputCompleteEvent is enqueued for delivery to the application.

If the device does not support volume change etc., that tag will be ignored.  This method is executed asynchronously. To end an utterance halfway, call the stopCurrentSpeaking method or the stopSpeaking method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.  					Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL		An invalid value was specified. The language set in	the Language property and the language specified	by Text do not match.

See Also	Language Property, Volume Property, Pitch Property, OutputID Property, Speed Property, stopCurrentSpeaking Method, stopSpeaking Method

speakImmediate Method

Syntax	speakImmediate (text: string):

void {raises-exception, use after open-claim-enable}
Parameter		Description

text	Specify the text to speak.

Remarks	The speak method acts to start speaking the words specified by text, while the speakImmediate method ends immediately previous speak method, and starts speaking the word specified by text asynchronously and immediately.

After executing the same processing as the clearOutput method, speak the wording specified by text.

Like this speak method, this method can also change a specific wording by inserting a tag. For details, refer to the description of speak method.

This method is executed asynchronously. To end an utterance halfway, call the stopCurrentSpeaking method or the stopSpeaking method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.  Some possible values of the exception’s ErrorCode property are:

Value          	Meaning

E_ILLEGAL 	An invalid value was specified. The language set in    the Language property and the language specified by Text do not match.

See Also	Language Property, Volume Property, Pitch Property, Speed Property,

speak Method, stopCurrentSpeaking Method, stopSpeaking Method

stopCurrentSpeaking Method

Syntax	stopCurrentSpeaking ( ):

void {raises-exception, use after open-claim-enable}

Remarks	The speak method and speakImmediate method start the speaking words specified by text and ends when stopCurrentSpeaking method is called.     This method handles asynchronously.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	Speech is not running.

See Also	speak Method, speakImmediate Method

stopSpeaking Method

Syntax	stopSpeaking (outputID: int32):

void {raises-exception, use after open, claim, enable}

Parameter	Description

outputID	Specify the value of the OutputID property you wish to terminate.

Remarks	Stop and delete the utterance specified in OutputID.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	OutputID Property, speak Method, speakImmediate Method

Events (UML interfaces)

DirectIOEvent

<<event>>	upos::events::DirectIOEvent

EventNumber	: int32 {read-only}
Data			: int32 {read-write}
Obj			: object {read-write}

Description 	Provides Service information directly to the application. This event provides a means for a vendor-specific Sound Player Service to provide events to the application that are not otherwise supported by the device control.

Attributes	This event contains the following attributes:

Attribute	Type	Description

EventNumber	int32	Event number whose specific values are assigned by the Service.

Data	int32	Additional numeric data. Specific values vary by the EventNumber and the Service. This attribute is settable.

Obj	object	Additional data whose usage varies by the EventNumber and the Service. This attribute is settable.

Remarks	This event is to be used only for those types of vendor specific functions that are not otherwise described.

Use of this event may restrict the application program programform being used with other vendor’s devices which may not have any knowledge of the Service’s need for this event.

See Also	“Events" on page Intro-19, directIO method

ErrorEvent

Goto Table104

<<event>> 	upos::events:: ErrorEvent
ErrorCode			: int32{read-write}
ErrorCodeExtended	: int32{read-write}
ErrorLocus 		: int32{read-write}
ErrorResponse		: int32{read-write}

Description	Notifies the application that a Speech Synthesis Device error has been detected and suitable response by the application is necessary to process the error condition.

Attributes	This event contains the following attributes:

Attributes	Type	Description

ErrorCode	int32	Error code causing the error event. 				See a list of Error Codes on page 20.

ErrorCodeExtended	int32	Extended Error code causing the error event.

If ErrorCode is E_EXTENDED, then see values below. Otherwise, it may contain a Service-specific value.

ErrorLocus	int32	Location of the error. If EL_OUTPUT is specified. It is indicating that the error occurred while processing asynchronous output. An error occurred during asynchronous action.

ErrorResponse	int32	Error response, whose default value may              be overwritten by the application     	     (i.e., this attribute is settable). 	      See values below.

The ErrorLocus attribute has one of the following values:

Value	Meaning

EL_OUTPUT	Error occurred while processing asynchronous output.

The application’s error event handler can set the ErrorResponse attribute to one of the following values:

Value	Meaning

ER_RETRY	Retry sending the asynchronous output data. The error state is exited. Typically, valid for asynchronous output devices when the locus is EL_OUTPUT, in which case the asynchronous output is re-tried, and the error state is exited. This is the default response when the locus is EL_OUTPUT.

ER_CLEAR	Valid for loci: EL_OUTPUT.  Clear all buffered input or output data (including all asynchronous output).  (The effect is the same as when clearOutput method is called.) The error state is exited.

Remarks	This event is enqueued when an error is detected, and the Device’s State transitions into the error state.

See Also 	 “Error Handling" on page Intro-23, “Device Output Models" on page Intro-25.

OutputCompleteEvent

<<event>>	upos::events::OutputCompleteEvent
OutputID		: int32{read-only}

Description 	Notify the application that the queued output request associated with the outputID property has completed successfully.

Attributes	This event contains the following attributes:

Attribute	Type	Description

OutputID 	int32	The ID number of the asynchronous output request that is complete.

Remarks	This event is enqueued after the request’s data has been both sent, and the Service has confirmation that it was processed by the device successfully.

See Also	“Device Output Models" on page Intro-25

StatusUpdateEvent

<<event>>	upos::events:: StatusUpdateEvent
Status			: int32 {read-only}

Description	Notifies the application that there is an operation status change or a status of the Speech Synthesis device.

Attributes		This event contains the following attribute:

Attribute	Type	Description

Status            int32	Indicates a change of operation status of sound player 			device

.

Note that Release 1.3 added Power State Reporting with additional Power 	reporting StatusUpdateEvent values.

The Update Firmware capability added additional Status values for communicating the status/progress of an asynchronous update firmware process. See “StatusUpdateEvent” description on page 1-34.

Value            		Meaning

SPCH_SUE_START_SPEAK									It will be notified when speech synthesis starts.

SPCH_SUE_STOP_SPEAK										It will be notified when speech synthesis stops.

Remarks	Enqueued when the Speech Synthesis Device detects a power state change or a status change.

See Also 	“Events" on page Intro-19.

Chapter 45

Gesture Control

This Chapter defines the Gesture Control device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | --- | Not supported |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.16 | open |
| CapPowerReporting: | int32 | {read-only} | 1.16 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.16 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.16 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.16 | open |
| CheckHealthText: | string | {read-only} | 1.16 | open |
| Claimed: | boolean | {read-only} | 1.16 | open |
| DataCount: | int32 | {read-only} | --- | Not supported |
| DataEventEnabled: | boolean | {read-write} | --- | Not supported |
| DeviceEnabled: | boolean | {read-write} | 1.16 | open & claim |
| FreezeEvents: | boolean | {read-write} | 1.16 | open |
| OutputID: | int32 | {read-only} | 1.16 | open |
| PowerNotify: | int32 | {read-write} | 1.16 | open |
| PowerState: | int32 | {read-only} | 1.16 | open |
| State: | int32 | {read-only} | 1.16 | -- |
| DeviceControlDescription: | string | {read-only} | 1.16 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.16 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.16 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.16 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.16 | open |
| PhysicalDeviceName: | string | {read-only} | 1.16 | open |

| Properties (Continued) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specific | Type | Mutability | Version | May Use After |
| CapAssociatedHardTotalsDevice: | string | {read-only} | 1.16 | open |
| CapMotion: | boolean | {read-only} | 1.16 | open |
| CapMotionCreation: | boolean | {read-only} | 1.16 | open |
| CapPose: | boolean | {read-only} | 1.16 | open |
| CapPoseCreation: | boolean | {read-only} | 1.16 | open |
| CapStorage: | int32 | {read-only} | 1.16 | open |
| AutoMode: | string | {read-write} | 1.16 | open, claim & enable |
| AutoModeList: | string | {read-only} | 1.16 | open |
| JointList: | string | {read-only} | 1.16 | open |
| MotionList: | string | {read-only} | 1.16 | open |
| PoseCreationMode: | boolean | {read-write} | 1.16 | open, claim & enable |
| PoseList: | string | {read-only} | 1.16 | open |
| Storage: | int32 | {read-write} | 1.16 | open, claim & enable |

| Methods (UML operations) |  |
| --- | --- |
| Common |  |
| Name | Version |
| open (logicalDeviceName: string): void {raises-exception} | 1.16 |
| close ( ): void {raises-exception, use after open} | 1.16 |
| claim (timeout: int32): void {raises-exception, use after open} | 1.16 |
| release ( ): void {raises-exception, use after open, claim} | 1.16 |
| checkHealth (level: int32): void {raises-exception, use after open, claim, enable} | 1.16 |
| clearInput ( ): void {raises-exception, use after open, claim} | 1.16 |
| clearInputProperties ( ): void {raises-exception, use after open, claim} | 1.16 |
| clearOutput ( ): void {raises-exception, use after open, claim} | 1.16 |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, claim, enable} | 1.16 |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} | 1.16 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | 1.16 |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | 1.16 |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, claim, enable} | 1.16 |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} | 1.16 |

Goto Table113

Goto Table112

Goto Table111

Goto Table110

Goto Table109

Goto Table108

Goto Table107

Goto Table106

Goto Table105

| Me Methods (UML operations)(continued) |  |
| --- | --- |
| Specific |  |
| Name | Version |
| createMotion (fileName: string, poseList: string): void { raises-exception, use after open, claim, enable } | 1.16 |
| createPose (fileName: string, time: int32): void { raises-exception, use after open, claim, enable } | 1.16 |
| getPosition (jointID: string, out position: int32): void { raises-exception, use after open, claim, enable } | 1.16 |
| setPosition (positionList: string, time: int32 , absolute: boolean): void { raises-exception, use after open, claim, enable } | 1.16 |
| setSpeed (speedList: string, time: int32): void { raises-exception, use after open, claim, enable } | 1.16 |
| startMotion (fileName: string): void { raises-exception, use after open, claim, enable } | 1.16 |
| startPose (fileName: string): void { raises-exception, use after open, claim, enable } | 1.16 |
| stopControl (outputID: int32): void { raises-exception, use after open, claim, enable } | 1.16 |

| Events (UML interfaces) |  |  |  |
| --- | --- | --- | --- |
| Name | Type | Mutability | Version |
| upos::events::DataEvent |  | Not supported |  |
| upos::events::DirectIOEvent |  |  | 1.16 |
| EventNumber: | int32 | {read-only} |  |
| Data: | int32 | {read-write} |  |
| Obj: | object | {read-write} |  |
| upos::events::ErrorEvent |  |  | 1.16 |
| ErrorCode: | int32 | {read-only} |  |
| ErrorCodeExtended: | int32 | {read-only} |  |
| ErrorLocus: | int32 | {read-only} |  |
| ErrorResponse: | int32 | {read-write} |  |
| upos::events::OutputCompleteEvent |  |  | 1.16 |
| OutputID: | int32 | {read-only} |  |
| upos::events::StatusUpdateEvent |  |  | 1.16 |
| Status: | int32 | {read-only} |  |
| upos::events::TransitionEvent |  | Not supported |  |

General Information

The Gesture Control device programmatic name is "Gesture Control".

Capabilities

The Gesture Control device has the following capability:

- It controls the behavior of various joint components and parts.

- The operation is automatically controlled by interlocking various joints and other devices.

- Register and play the defined pose and motion.

Gesture Control Class Diagram

The following diagram shows the relationships between the Gesture Control classes.

Fig. Chap. 45-1 Gesture Control Class Diagram

Model

The Gesture Control follows the general device behavior model for asynchronous output devices:

- The application calls a setPosition, setSpeed, startPose, startMotion method to start output. The Device validates the method parameters and produces an error condition immediately if necessary. If the validation is successful, the Device does the following:

･Buffers the request in program memory, for delivery to the Physical Device as soon as the Physical Device can receive and process it.

･Sets the OutputID property to a unique integer identifier for this request.

･Returns as soon as possible.

- When the Device successfully completes a request, an OutputCompleteEvent is enqueued for delivery to the application. A property of this event contains the outputID of the completed request. The application should compare the returned OutputCompleteEvent property OutputID value with the OutputID value set by the asynchronous process method call used to send the data, in order to track what data has been successfully sent to the device.

- If an error occurs while processing a request, an ErrorEvent is enqueued which will be delivered to the application after the events already enqueued, including OutputCompleteEvent. No further asynchronous output will occur until the event has been delivered to the application. If the response is ER_CLEAR, then outstanding asynchronous output is cleared. If the response is ER_RETRY, then output is retried; note that if several outputs were simultaneously in progress at the time that the error was detected, then the Service may need to retry all of these outputs.

- Asynchronous output is always performed on a first-in first-out basis.

- If the request is terminated before completion, due to reasons such as the application calling the clearOutput method, then no OutputCompleteEvent is delivered.

- Application can also delete the output individually by calling the stopControl method. Also, in this case OutputCompleteEvent will not be notified.

- The application will be informed about any status change with a StatusUpdateEvent, also all corresponding status properties will be updated before event delivery.

Automatic control

Automatic control of a joint means to automatically control a joint on the device side, such as tracking according to the movement of a person's face, in cooperation with a camera or the like connected to the device.

The automatic control function is device dependent. For possible automatic control, it is enabled by confirming with the AutoModeList property and setting a value in the AutoMode property.

Pose / Motion

Pose refers to setting the position of one or more defined joints.

For example, it is an action that lifts a hand.

To execute a pose, specify the pose file name by the startPose method or the pose name defined in the device.

Create the pose file with the createPose method described later. Pose defined in the device will be checked in the value of PoseList property.

To execute motion, specify the motion file name or the motion name defined in the device with the startMotion method.

Motion files are created by the createMotion method to be described later. Motion defined in the device can be checked with the value of MotionList property.

To create a pose file, first set the PoseCreationMode property to TRUE and enable the pose registration function. When pose registration function is enabled, each joint is set to the default position. At this time, if the automatic control mode is enabled, the automatic control mode is temporarily invalidated.

Then, application can create a pose file by setting the value defined as a pose with the setPosition method and calling the createPose method.

A motion file can be created and recorded by specifying the pose defined in the 	       created pose file or the pose defined in the device and creating it as a series of continuously changing actions and calling the createMotion method.

Since the created pose and motion files are recorded in the area may store in either the “Hard Totals” devices or the host file system, or both, and the CapStorage property will show the device’s data file storage location capability.

If device supports either of both Hard Totals devices and the host file system, the application should set the Storage property accordingly to tell where to write the data file.

If device needs to be able to write the pose and motion files to a Hard Totals device, the CapAssociatedHardTotalsDevice property holds the open name of the associated Hard Totals device.

Device Sharing

The Gesture Control device is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before accessing some properties or calling methods that update the device.

- See the “Summary” table for precise usage prerequisites.

Properties (UML attributes)

AutoMode Property

Syntax	AutoMode: string {read-write, access after open-claim-enable}

Remarks	Indicates automatic control mode ID. Valid values are the empty string "" or one of the AutoModeList properties listed.

If one of the properties described in the AutoModeList property is set, the automatic control mode will be enabled in the set mode.

Setting the empty character "" disables the automatic control mode.

This property is initialized to the empty string "" by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	AutoModeList Property

AutoModeList Property

Syntax	AutoModeList: string {read-only, access after open}

Remarks	Comma-separated list of joint automatic control IDs supported by the device.

For example, in conjunction with the camera, if the mode of tracking the face of a person by moving only the joint of Joint01, this is “FaceTrack_Joint01”.

Another example, in conjunction with the camera, if the mode of tracking the face of a person by moving all joints are supported, this is “FaceTrack_ALL".

(Content and order are dependent on the device.)

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	AutoMode Property.

CapAssociatedHardTotalsDevice Property

Syntax	CapAssociatedHardTotalsDevice: string {read-only, access after open}

Goto Table114

Remarks	Holds the open name of the associated Hard Totals device if the device is able to write to such devices which is the case if CapStorage is either GCTL_CST_ALL or GCTL_CST_HARDTOTALS_ONLY. If CapStorage is GCTL_CST_HOST_ONLY this property value must be the empty string.  This property is initialized by the open method.

Errors		UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	CapStorage Property

CapMotion Property

Syntax	CapMotion: boolean {read-only, access after open}

Remarks	If true, the device supports making the motion function. Otherwise, it is false.  When this property is false, startMotion method, createMotion method is not available. This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	startMotion Method, createMotion Method.

CapMotionCreation Property

Syntax	CapMotionCreation: boolean {read-only, access after open}

Remarks	If true, the device supports motion registration function.

If false, the device does not support motion registration function.

If this property is FALSE, the createMotion method is not available.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	createMotion Method.

CapPose Property

Syntax	CapPose: boolean {read-only, access after open}

Remarks	If true, the device supports pose function. Otherwise, it is false.

When this property is FALSE, PoseCreationMode property value cannot be changed, in addition, startPose method, and createPose method are not available.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	PoseCreationMode Property, startPose Method, createPose Method.

CapPoseCreation Property

Syntax	CapPoseCreation: boolean {read-only, access after open}

Remarks	If true, the device supports pose registration function.

If false, the device does not support pose registration function.

When this property is FALSE, the createPose method that can change the PoseCreationMode property is not available.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	PoseCreationMode Property, createPose Method.

CapStorage Property

Syntax	CapStorage: int32 {read-only, access after open}

Remarks	This is an enumeration and announces where the device is able to write the recorded motion and/or pose data file to.
It holds one of the following values.

Value	Meaning

GCTL_CST_HARDTOTALS_ONLY					     Only an associate Hard Totals device is supported.

GCTL_CST_HOST_ONLY	Only the host’s file system is supported.

GCTL_CST_ALL	Both, the associated Hard Totals device and the host’s file system is supported.

This property is initialized by the open method.

If a Hard Totals device is supported the Storage the property value should be GCTL_CST_HARDTOTALS_ONLY or GCTL_CST_ALL, and the property CapAssociatedHardTotalsDevice holds the open name of the associated Hard Totals device.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	Storage Property, CapAssociatedHardTotalsDevice Property

JointList Property

Syntax	JointList: string {read-only, access after open}

Remarks	Comma-separated list of joint information supported by the device.

Each piece of joint information consists of the following information and is 	shown in the following order, separated by a colon (":").

Parameter   	Description

JointID	Indicates a unique ID in the service that identifies the

joint.

Position range availability:

If position range is 0, the Joint does not have the

position range.

If position range is 1, the joint holds the position range.

For example, arm joint has a range of rotation width

but wheel for movement does not have the range of

movement amount.

If there is a device with joints that supports pitch, roll,

yaw and wheels that supports rotating and moving back

and forth.

In this case they are indicated as follows:

"Joint01_Pitch:1, Joint01_Roll:1, Joint01_Yaw:1,

Wheel_Turn:0, Wheel_Move:0"

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

MotionList Property

Syntax	MotionList: string {read-only, access after open}

Remarks	Comma-separated list of motion IDs defined on the device.

For example, “bowing, welcoming, clapping,…”

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

PoseCreationMode Property

Syntax	PoseCreationMode: boolean {read-write, access after open-claim-enable}

Remarks	If true, pose registration function is enabled.

If false, pose registration function is invalid.

When this property is set to true, pose registration function is enabled. When false is set, the pose registration function is disabled.

This property is initialized to false when you first enable the device after calling the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	CapPose Property, CapPoseCreation Property.

PoseList Property

Syntax	PoseList: string {read-only, access after open}

Remarks	A comma-separated list of pose IDs defined on the device.

For example, “surprise, bow, think,….”

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Storage Property

Syntax	Storage: int32 {read-write, access after open-claim-enable}

Remarks	This is an enumeration and defines where the device writes the recorded motion and/or pose data file to. Should be set before an appropriate method call.
It holds one of the following values.

Value                                 Meaning

GCTL_ST_HARDTOTALS

The motion and/or pose data file is written to the associated  Hard Totals device. The property CapAssociatedHardTotalsDevice holds the open name of the associated Hard Totals device.

GCTL_ST_HOST              	The motion and/or pose data file is written to the host’s file system.

GCTL_ST_HOST_HARDTOTALS

The motion and/or pose data file is written to the associated Hard Totals device and host’s file system. The property CapAssociatedHardTotalsDevice holds the open name of the associated Hard Totals device.

This property is initialized by the open method according to the value hold by CapStorage. If CapStorage has the value GCTL_CST_ALL, it is initialized to GCTL_ST_HOST_HARDTOTALS.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

Value	Meaning

E_ILLEGAL	An invalid value was specified, or recording is ongoing.

See Also	CapStorage Property, CapAssociatedHardTotalsDevice Property

Table of Gesture Control Device Listed Items in Property

| Property Name | Item ID, File Name, Name | Parameter |
| --- | --- | --- |
| AutoModeList | Face Track | Joint01 Joint_ALL |
|  | Chase | Joint01, Wheel01, Wheel02 Joint_ALL, Wheel_ALL, |
| MotionList | Bowing, Welcoming, Clapping, Farewelling01, Farewelling02, Greeting01, Greeting02 , |  |
| PoseList | Surprise, Bow01, Bow02, Think01, Think02 Doubt01, Doubt02 |  |
| JointList | Joint | Pitch Roll Yaw |
|  | Wheel | Turn Move Back Move Forth |

Methods (UML operations)

createMotion Method

Syntax	createMotion (fileName: string, poseList: string): 
		void {raises-exception, use after open-claim-enable}

Parameter	Description

fileName	Specify the motion file name recorded as motion.

poseList	Specify the comma-separated list of pose information to be registered.

Remarks      A motion file can be created and recorded by specifying the pose defined in the 	                    created pose file or the pose defined in the device and creating it as a series of 	                    continuously changing actions.

The place where the motion file is recorded is the area value of the Storage property.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	fileName is too long or contains unusable characters.

E_EXISTS	fileName already exists.

createPose Method

Syntax	     createPose (fileName: string, time: int32): 
          				void {raises-exception, use after open-claim-enable}

Parameter	Description

fileName	Specify the pose file name to record the pose.

time	Specify the time to reach the pose position.

Remarks	Record the position of each joint in the pose file.

Before calling this method, it needs to set the PoseCreationMode property to TRUE and to make enabling pose registration mode.

The place where the motion file is recorded is the area value of the Storage property.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	FileName is too long or contains unusable characters. Or PoseCreationMode is FALSE.

E_EXISTS	FileName already exists.

See Also	PoseCreationMode Property.

getPosition Method

Syntax	getPosition (jointID: string, out position: int32): 
void {raises-exception, use after open-claim-enable}

Parameter	Description

jointID		Specify the one of the joint ID values that are listed in the JointList property.

And specified JointList property should be the position range present one.

position		Store the specified value as the position associated with jointID.

Remarks	It acquires the position specified by jointID and stores it in position.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	JointList Property.

setPosition Method

Syntax	setPosition (positionList: string, time: int32, absolute: boolean): 
		void {raises-exception, use after open-claim-enable}

Parameter	Description

positionList	Specify the position information in a comma-separated list.

time	Specify the time of device control completion in seconds. If this value is too small, it will be changed to an appropriate value depending on the service.

absolute	If true, the specified position indicates the absolute value. If false, the specified position indicates relative value.

Each position information specified in the positionList consists of the following information and is shown in the following order separated by a colon (":").

Parameter	Description

jointID	Specify the joint ID. Specify one of the values listed in the JointList property. However, it must be an ID whose position range is present.

position	Specify the position to be set. Valid values range from -100 to 100.

100 represents the limit value in the positive direction of the target joint, and -100 represents the limit value in the negative direction.

If absolute is a relative value (false) and the value specified here exceeds the limit value, it will be changed to an appropriate value by the service

For example, to move Yow of Joint01 up to the limit of the positive direction and move Pitch of Joint02to the middle, specify as follows.

"Joint01_Yaw:100,Joint02:Pitch:0"

Remarks	The joint position is set with the contents specified in PositionList and device control is started so that device control is completed at the time specified by Time.

Joints that can be specified with this method are only those that have a position range.

Check the JointList property for the presence or absence of the position range.

This method is executed asynchronously. To terminate the operation prematurely, call the stopControl method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	JointList Property, stopControl Method.

setSpeed Method

Syntax	setSpeed (speedList: string, time: int32): 
		void {raises-exception, use after open-claim-enable}

Parameter	Description

speedList		Specify speed information in a comma-separated list.

time		Specify the time to device control in seconds. If the value of FOREVER(-1) is specified, it will continue to operate until you call the stopControl method.

Each speed information specified in the SpeedList consists of the following

information, and it is shown in the following order separated by a colon (":").

Parameter	Description

jointID	Specify the joint ID. Specify one of the values listed in the JointList property.

speed	Specify the speed to set. Valid values range from       -100 to 100. 100 represents the maximum speed in the positive direction of the target joint, and -100 represents the maximum speed in the negative direction.

For example, to move Wheel's X at the maximum speed in the positive direction and Y at the Wheel at half the speed in the negative direction, specify as follows.

"Wheel_X:100, Wheel_Y:-50"

Remarks	It sets the speed of the joint with the contents specified by speedList and performs device control for the time specified by time.

This method is executed asynchronously. To terminate the operation prematurely, call the stopControl method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	JointList Property, stopControl Method.

startMotion Method

Syntax	startMotion (fileName: string): 
		void {raises-exception, use after open-claim-enable}

Parameter	Description

fileName	Prior to start this method, need to specify the name of the motion file or the motion ID value that is listed in the MotionList property.

Remarks	Start the motion defined by fileName or motion defined by the device. 	     This method is executed asynchronously and when the device successfully completes a request, an OutputCompleteEvent is enqueued and a property of corresponding event’s OutputID is placed into the OutputID property.         The application should compare the returned OutputCompleteEvent property outputID value set by this method to track what data has been sent to device.

Motion files are placed in the area as the value of Storage property.

To terminate motion control prematurely, call the stopControl method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

E_NOEXIST	File does not exist.

See Also	MotionList Property.

startPose Method

Syntax	startPose (fileName: string): 
		void {raises-exception, use after open-claim-enable}

Parameter	Description

fileName	Specify the name of the pose file to start. Or one of the pose ID lists listed in the PoseList property.

Remarks        Start the pose defined by the pose file or device specified by 		        fileName. This method is executed asynchronously and when the device successfully completes a request, an OutputCompleteEvent is enqueued and a property of corresponding event’s OutputID is placed into the OutputID property. The application should compare the returned OutputCompleteEvent         property OutputID value set by this method to track what data has been sent         to device. Pose files are placed in the area as the values of Storage property.          To terminate pause control prematurely, call the stopControl method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

E_NOEXISTS	File does not exist.

See Also	PoseList Property, stopControl Method.

stopControl Method

Syntax	stopControl (outputID: int32): 
		void {raises-exception, use after open-claim-enable}

Parameter	Description

outputID	Specify the value of the OutputID property to be terminated.

Remarks	Stop the control specified for outputID. When device successfully complete the request, and OutputCompleteEvent is enqueued. A property of this event contains the outputID of the completed request. The application should compare the returned OutputCompleteEvent property OutputID value with OutputID value set by this method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	setPosition Method, setSpeed Method, startPose Method, startMotion Method.

Events (UML interfaces)

DirectIOEvent

<<event>>	upos::events::DirectIOEvent

EventNumber	: int32 {read-only}
Data			: int32 {read-write}
Obj			: object {read-write}

Description 	Provides Service information directly to the application. This event provides a means for a vendor-specific Sound Player Service to provide events to the application that are not otherwise supported by the device control.

Attributes	This event contains the following attributes:

Attribute	Type	Description

EventNumber	int32	Event number whose specific values are assigned by the Service.

Data	int32	Additional numeric data. Specific values vary by the EventNumber and the Service. This attribute is settable.

Obj	object	Additional data whose usage varies by the EventNumber and the Service. This attribute is settable.

Remarks	This event is to be used only for those types of vendor specific functions that are not otherwise described.

Use of this event may restrict the application program programform being used with other vendor’s devices which may not have any knowledge of the Service’s need for this event.

See Also	“Events" on page Intro-19, directIO method

ErrorEvent

Goto Table115

<<event>> 	upos::events:: ErrorEvent
ErrorCode			: int32{read-write}
ErrorCodeExtended	: int32{read-write}
ErrorLocus 		: int32{read-write}
ErrorResponse		: int32{read-write}

Description	Notifies the application that a Gesture Control Device error has been detected and suitable response by the application is necessary to process the error condition.

Attributes	This event contains the following attributes:

Attributes	Type	Description

ErrorCode	int32	Error code causing the error event. 				See a list of Error Codes on page 20.

ErrorCodeExtended	int32	Extended Error code causing the error event.

If ErrorCode is E_EXTENDED, then see values below. Otherwise, it may contain a Service-specific value.

ErrorLocus	int32	Location of the error. If EL_OUTPUT is specified it is indicating that error occurred while processing asynchronous output. An error occurred during asynchronous action.

ErrorResponse	int32	Error response, whose default value may               be overridden by the application     	     (i.e., this attribute is settable). 	       See values below.

If ErrorCode is E_EXTENDED, then ErrorCodeExtended has one of the following values:

Value	                    Meaning                                                                      EGCTL_NOROOM	      There is not enough room for the targeted data file 			      storage area.

The ErrorLocus attribute has one of the following values:

Value	Meaning                                                                      EL_OUTPUT	Error occurred while processing asynchronous output.

The application’s error event handler can set the ErrorResponse attribute to one of the following values:

Value	Meaning

ER_RETRY	Retry sending the asynchronous output data. The error state is exited. Typically, valid for asynchronous output devices when the locus is EL_OUTPUT, in which case the asynchronous output is re-tried, and the error state is exited. This is the default response when the locus is EL_OUTPUT.

ER_CLEAR	Valid for all loci: EL_OUTPUT.  Clear all buffered input or output data (including all asynchronous output).  (The effect is the same as when clearOutput method is called.) The error state is exited.

Remarks	This event is enqueued when an error is detected, and the Device’s State transitions into the error state.

See Also 	 “Error Handling" on page Intro-23, “Device Output Models" on page

OutputCompleteEvent

<<event>>	upos::events::OutputCompleteEvent
		OutputID: int32{read-only}

Description 	Notify the application that the queued output request associated with the outputID property has completed successfully.

Attributes	This event contains the following attributes:

Attribute	Type	Description

OutputID 	int32	The ID number of the asynchronous output request that is complete.

Remarks	This event is enqueued after the request’s data has been both sent, and the Service has confirmation that it was processed by the device successfully.

See Also	“Device Output Models" on page Intro-25

StatusUpdateEvent

<<event>>	upos::events:: StatusUpdateEvent
		Status : int32 {read-only}

Description	Notifies the application that there is an operation status change or a status of the Gesture Control device.

Attributes		This event contains the following attribute:

Attributes	Type	   Description

Status           int32        Indicates a change of operation status of sound player              			   device

.	Note that Release 1.3 added Power State Reporting with additional Power  reporting StatusUpdateEvent values.

The Update Firmware capability added additional Status values for communicating the status/progress of an asynchronous update firmware process. See “StatusUpdateEvent” description on page 1-34.

Value            		Meaning

GCTL_SUE_START_MOTION									It will be notified when Gesture Motion start.

GCTL_SUE_STOP_MOTION									It will be notified when Gesture Motion stop.

Remarks	Enqueued when the Gesture Control Device detects a power state change or a status change.

See Also 	“Events" on page Intro-19.

Chapter 46

Device Monitor

This Chapter defines the Device Monitor device category.

Summary

| Pro Properties (UML attributes) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Common |  | Type |  | Mutability |  | Version |  | May Use After |  |
| AutoDisable: |  | boolean |  | {read-write} |  | 1.16 |  | open |  |
| CapCompareFirmwareVersion: |  | boolean |  | {read-only} |  | 1.16 |  | open |  |
| CapPowerReporting: |  | int32 |  | {read-only} |  | 1.16 |  | open |  |
| CapStatisticsReporting: |  | boolean |  | {read-only} |  | 1.16 |  | open |  |
| CapUpdateFirmware: |  | boolean |  | {read-only} |  | 1.16 |  | open |  |
| CapUpdateStatistics: |  | boolean |  | {read-only} |  | 1.16 |  | open |  |
| CheckHealthText: |  | string |  | {read-only} |  | 1.16 |  | open |  |
| Claimed: |  | boolean |  | {read-only} |  | 1.16 |  | open |  |
| DataCount: |  | int32 |  | {read-only} |  | 1.16 |  | open |  |
| DataEventEnabled: |  | boolean |  | {read-write} |  | 1.16 |  | open |  |
| DeviceEnabled: |  | boolean |  | {read-write} |  | 1.16 |  | open & claim |  |
| FreezeEvents: |  | boolean |  | {read-write} |  | 1.16 |  | open |  |
| OutputID: |  | int32 |  | {read-only} |  | 1.16 |  | Not supported |  |
| PowerNotify: |  | int32 |  | {read-write} |  | 1.16 |  | open |  |
| PowerState: |  | int32 |  | {read-only} |  | 1.16 |  | open |  |
| State: |  | int32 |  | {read-only} |  | 1.16 |  | -- |  |
| DeviceControlDescription: |  | string |  | {read-only} |  | 1.16 |  | -- |  |
| DeviceControlVersion: |  | int32 |  | {read-only} |  | 1.16 |  | -- |  |
| DeviceServiceDescription: |  | string |  | {read-only} |  | 1.16 |  | open |  |
| DeviceServiceVersion: |  | int32 |  | {read-only} |  | 1.16 |  | open |  |
| PhysicalDeviceDescription: |  | string |  | {read-only} |  | 1.16 |  | open |  |
| PhysicalDeviceName: |  | string |  | {read-only} |  | 1.16 |  | open |  |

Goto Table117

Goto Table118

Goto Table123

Goto Table122

Goto Table121

Goto Table120

Goto Table119

Goto Table116

| Properties (Continued) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Specific | Type | Mutability |  | Version |  | May Use After |  |
| DeviceData: | string | {read-only} |  | 1.16 |  | open, claim & enable |  |
| DeviceList: | string | {read-only} |  | 1.16 |  | open |  |
| MonitoringDeviceList: | string | {read-only |  | 1.16 |  | open, claim & enable |  |
| Me Methods (UML operations) |  |  |  |  |  |  |  |
| Common |  |  |  |  |  |  |  |
| Name |  |  |  |  |  |  | Version |
| open (logicalDeviceName: string): void {raises-exception} |  |  |  |  |  |  | 1.16 |
| close ( ): void {raises-exception, use after open} |  |  |  |  |  |  | 1.16 |
| claim (timeout: int32): void {raises-exception, use after open} |  |  |  |  |  |  | 1.16 |
| release ( ): void {raises-exception, use after open, claim} |  |  |  |  |  |  | 1.16 |
| checkHealth (level: int32): void {raises-exception, use after open, claim, enable} |  |  |  |  |  |  | 1.16 |
| clearInput ( ): void {raises-exception, use after open, claim} |  |  |  |  |  |  | 1.16 |
| clearInputProperties ( ): void {raises-exception, use after open, claim} |  |  |  |  |  |  | 1.16 |
| clearOutput ( ): void { } |  |  |  |  |  |  | Not supported |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, claim, enable} |  |  |  |  |  |  | 1.16 |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} |  |  |  |  |  |  | 1.16 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  |  |  |  |  | 1.16 |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  |  |  |  |  | 1.16 |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, claim, enable} |  |  |  |  |  |  | 1.16 |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  |  |  |  |  | 1.16 |
| Specific |  |  |  |  |  |  |  |
| addMonitoringDevice (deviceID: string, monitoringMode: int32, boundary: int32, subBoundary: int32, intervalTime: int32 ): void {raises-exception, use after open, claim, enable} |  |  |  |  |  |  | 1.16 |
| clearMonitoringDevices ( ): void {raises-exception, use after open, claim, enable} |  |  |  |  |  |  | 1.16 |
| deleteMonitoringDevice (deviceID: string ): void {raises-exception, use after open, claim, enable} |  |  |  |  |  |  | 1.16 |
| getDeviceValue (deviceID: string, pValue: int32) void {raises-exception, use after open} |  |  |  |  |  |  | 1.16 |
| Events (UML interfaces) |  |  |  |  |  |  |  |
| Name | Type |  | Mutability |  | Version |  |  |
| upos::events::DataEvent |  |  |  |  | 1.16 |  |  |
| Status: | int32 |  | {read-only} |  |  |  |  |
| upos::events::DirectIOEvent |  |  |  |  | 1.16 |  |  |
| EventNumber: | int32 |  | {read-only} |  |  |  |  |
| Data: | int32 |  | {read-write} |  |  |  |  |
| Obj: | object |  | {read-write} |  |  |  |  |
| upos::events::ErrorEvent |  |  |  |  | 1.16 |  |  |
| ErrorCode: | int32 |  | {read-only} |  |  |  |  |
| ErrorCodeExtended: | int32 |  | {read-only} |  |  |  |  |
| ErrorLocus: | int32 |  | {read-only} |  |  |  |  |
| ErrorResponse: | int32 |  | {read-write} |  |  |  |  |
| upos::events::OutputCompleteEvent |  |  | Not supported |  |  |  |  |
| upos::events::StatusUpdateEvent |  |  |  |  | 1.16 |  |  |
| Status: | int32 |  | {read-only} |  |  |  |  |
| upos::events::TransitionEvent |  |  | Not supported |  |  |  |  |

General Information

The Device Monitor programmatic name is "Device Monitor".

Capabilities

The Device Monitor Device has the following capability:

- Get values measured by various devices.

- Notify the application of changes in values measured by various devices.

Device Monitor Class Diagram

The following diagram shows the relationships between the Device Monitor classes.

Fig. Chap. 46-1 Device Monitor Class Diagram

Model

The Device Monitor follows the general “Device Input Model” for event-driven input:

- The Device Monitor supports monitoring of values measured by multiple devices connected to the device. A device that can be monitored and its type / value unit is listed in the DeviceList property.

- Device Monitor receives a change in the value measured by the device set as the monitoring target and generates a DataEvent when it matches the specified condition.

- To add a device to be monitored, specify the monitoring mode with the addMonitoringDevice method and add it. For details on monitoring mode, see the description of addMonitoringDevice method.

- If the AutoDisable property is true, the device will automatically disable itself when a DataEvent is enqueued.

- An enqueued DataEvent can be delivered to the application when the DataEventEnabled property is true and other event delivery requirements are met. Just before delivering this event, data is copied into corresponding properties, and further data events are disabled by setting DataEventEnabled to false. This causes subsequent input data to be enqueued while the application processes the current input and associated properties. When the application has finished processing the current input and is ready for more data, it reenables events by setting DataEventEnabled to true.

- An ErrorEvent (or events) is enqueued if an error occurs while gathering or processing input and is delivered to the application when DataEventEnabled is true and other event delivery requirements are met.

- The DataCount property can be read to obtain the total number of enqueued DataEvents.

- All enqueued input may be deleted by calling ClearInput method. See the ClearInput method description for more details.

- All data properties that are populated as a result of firing a DataEvent or ErrorEvent can be set back to their default values by calling the clearInputProperties method.

- The notified data is stored in the DeviceData property.

- In the Device Monitor device control, the measured values of the devices are managed most of cases with the int32 type integers, but some are decimals.

- In that case, the decimals are implicit, and the actual value can be calculated by dividing the measured value by the coefficient of each device that can be  obtained in the DeviceList property.

The application will be informed about any status change with a    StatusUpdateEvent, also, all corresponding status properties will be updated    before event delivery.

Device Sharing

The Device Monitor is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before the device begins reading input, or before calling methods that manipulate the device.

See the “Summary” table for precise usage prerequisites.

Properties (UML attributes)

DeviceData Property

Syntax	DeviceData: string {read-only, access after open-claim-enable}

Remarks	Measurement information of the device that matches the condition registered by addMonitoringDevice method is set.

Each measurement information consists of the following information and is shown in the following order, separated by a colon (":").

Parameter	Description

DeviceID	The target device ID.

Measured value		Measurement value of the device. The measured value is represented by an integer type. To convert it to an actual value, divide the measured value by the coefficient acquired by the DeviceList property.

For example,"Device01:365"

Its value is set prior to a DataEvent being delivered   to the application.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

DeviceList Property

Syntax	DeviceList: string {read-only, access after open}

Remarks	Contains the comma-delimited list of device information that are supported by the device.

Each object information consists of the following information and is shown in the following order, separated by a colon (":").

Parameter	Description

DeviceID	Indicates a unique ID in the service that identifies the device.

Type	Indicates the device type. For example, if it is a touch sensor it is expressed as "Touch Sensor" and so on. However, this value depends on the service.

Unit	Indicates the unit of value held by various devices. For example, it is expressed as "on / off" for a touch sensor, "rad / s" for a gyroscope. However, this value depends on the service.

Coefficient	Indicates the coefficient for calculating the actual measured value held by various devices. The DeviceData property and the measured value of the device that can be obtained with the GetDeviceValue method are expressed as integers, but by dividing this value by the coefficient it is the actual value.  Example: Device value = 365, coefficient = 10, actual value = 36.5		         For example, if one device supports one touch sensor and one gyroscope, it will be as follows. "Touch 01: Touch Sensor: ON/OFF: 1, GyroX: Gyroscope: rad/s: 100000, GyroY: Gyroscope: rad/s: 100000, GyroZ: Gyroscope: rad/s: 100000"

This property is initialized by the open method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	DeviceData Property, addMonitoringDevice Method, getDeviceValue Method.

MonitoringDeviceList Property

Syntax	MonitoringDeviceList: string {read-only, access after open-claim-enable}

Remarks	Contains the comma-delimited list of monitoring information on registered devices that are supported by the device.

Each monitoring information consists of the following information and is shown in the following order, separated by a colon (":").

Parameter	Description

DeviceID	Registered devices ID.

Monitoring mode	Registered monitoring mode

Boundary	Registered boundary value. This value is set to 0                                              when the monitoring mode does not require a boundary value.

Sub boundary		Registered sub boundary value. This value is set to 0 	when the monitoring mode does not require a sub 	boundary value.

Interval		Registered interval. (millisecond)

For example, if you set monitoring targets as follows,

[Monitor target 1]

Device ID = Device 01, monitoring mode = DMON_MM_UPDATE,

boundary line = 0, sub boundary line = 0, interval time = 0

[Monitor target 2]

Device ID = Device 02, monitoring mode = DMON_MM_STRADDLED,

boundary line = 365, sub boundary line = 0, interval time = 500

The values shown are as follows.

"Device01:0:0:0:0, Device02:1:365:0:500"

This property is initialized by the open method. It is also updated by calling

addMonitoringDevice method, deleteMonitoringDevice method,

clearMonitoringDevice method.

Errors	A UposException may be thrown when this property is accessed. For further information, see “Errors" on page Intro-20.

See Also	addMonitoringDevice Method, deleteMonitoringDevice Method, clearMonitoringDevice Method.

Methods (UML operations)

addMonitoringDevice Method

Syntax	addMonitoringDevice (deviceID:string, monitoringMode:int32, boundary:int32, subBoundary:int32, intervalTime:int32):
		void{raises-exception, use after open-claim-enable}

Parameter	Description

deviceID	The deviceID of the monitored device. Valid values 		are one of the device ID lists listed in the DeviceList 		property.

monitoringMode	Specify the monitoring mode for monitoring.

boundary	Specify the boundary value to be monitored.

subBoundary	Specify the sub boundary value to be monitored. 		This value must be less than Boundary.

intervalTime	Specify the interval in milliseconds between the 		occurrence of the event and the start of the next 		monitoring.

The monitoring modes specified for MonitoringMode are as follows.

Value	Description

DMON_MMODE_UPDATE
		Every time the measured value of the target device is 		updated, an event is notified. When set to this mode, 		the values of the argument boundary and 			subBoundary are ignored.

DMON_MMODE_STRADDLED
		When the measured value of the target device 			crosses the value of the argument boundary, 			it notifies the event. In addition, when the measured		value matches the value of boundary, it notifies the		event even when it changes from the matched state. 		When set to this mode, the value of the argument		subBoundary is ignored.

DMON_MMODE_HIGH
		When the measured value of the target device			becomes equal to or larger than the value of the		argument Boundary, it notifies the event. 			Even if the measured value is updated and it was 		again equal to or greater than the value of 			boundary, the event will be notified in 			each time. When it is set to this mode, the value of 		the argument subBoundary is ignored.

DMON_MMODE_LOW
		          	          Notifies the event when the measured value of the 		                        target device becomes less than or equal to the   	                                      value of the argument boundary. Even when the 		                        measured value is updated and it was again less 		                        than the value of boundary, the event will be                                   	                        	          notified in each time.

When it is set to this mode, the value of the argument subBoundary is ignored.

DMON_MMODE_WITHIN
			           It notifies the event while the measured value of 			           the target device is within the range specified by 			           the argument boundary and subBoundary.
			           Even if the measured value is updated and its value 			           is	 within the range again, the event is notified in 			           each time.

DMON_MMODE_OUTSIDE
			           It notifies the event while the measured value of 			           the target device is outside the range specified 			           by the argument boundary and subBoundary.
			           Even if the measured value is updated and its value 			           was out of range again, the event                   			           will be notified in each time.

DMON_MMODE_POLLING
			           It notifies the measured value of the target device 			           at the interval specified by intervalTime.
			           When it is set to this mode, the values of the 			           argument boundary and subBoundary are ignored.

Remarks	Add the device specified by deviceID to the monitoring target.
The monitoring mode is specified for monitoringMode, but there are monitoring modes not supported by some devices. In that case, E_ILLEGAL is raised as the UPOS exception.
Devices added by this method will be added to the list of MonitoringDeviceList properties. If a device to be monitored is specified, it will be changed to a new condition. To exclude the added device from the monitoring target, call deleteMonitoringDevice method or clearMonitoringDevice method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.  Some possible values of the exception’s ErrorCode property are:

Value                	Description

E_ILLEGAL	An invalid value was specified, or unsupported 					operation with the Device

See Also	DeviceList Property, MonitoringDeviceList Property, deleteMonitoringDevice Method, clearMonitoringDevice Method, DataEvent.

clearMonitoringDevices Method

Syntax	clearMonitoringDevices ():
		void {raises-exception, use after open-claim-enable}

Remarks	Exclude all devices to be monitored.

Errors	A UposException may be thrown when this method is invoked.
For further information, see “Errors" on page Intro-20.

See Also	addMonitoringDevice Method.

deleteMonitoringDevice Method

Syntax	deleteMonitoringDevice (deviceID: string):
		void {raises-exception, use after open-claim-enable}

Parameter		Description

deviceID		Specify the device ID of the device to be excluded from 			monitoring targets.

Remarks	Exclude the device specified by deviceID from monitoring targets.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.
Some possible values of the exception’s ErrorCode property are:

Value		    Description

E_ILLEGAL	An invalid value was specified, or unsupported operation with the Device.  An invalid value was specified, or  unsupported operation with the Device.

See Also	AddMonitoringDevice Method.

getDeviceValue method

Syntax		getDeviceValue (deviceID: string, pValue: *int32):
		void {raises-exception, use after open}

Parameter	Description

deviceID		Specify the device ID of the device from which the 			measurement value is to be acquired. Specify one of the 			device ID lists listed in the DeviceList property.

pValue 		Pointer that stores measurement values obtained from the 			device.

Remarks		Get the measured value of the device specified by deviceID. The retrieved value is stored in pValue.

Errors		A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.
Some possible values of the exception’s ErrorCode property are:

Value		Description

E_ILLEGAL	An invalid value was specified, or unsupported operation with the Device.

See Also		DeviceList Property.

Events (UML interfaces)

DataEvent

<<event>>	upos::events::DataEvent

Status			: int32{read-only}

Description 	Notifies the application when data from the Device Monitor device is available to be read.

Attributes	This event contains the following attributes:

Attribute	Type		Description

Status	int32		Set to 0.

Remarks	Before this event is delivered, the individual recognition information is enqueued into the area that is indicated by the addMonitoringDevice method.

See Also	addMonitoringDevice method.

DirectIOEvent

<<event>>	upos::events::DirectIOEvent

EventNumber	: int32 {read-only}				           Data			: int32 {read-write}
Obj			: object {read-write}

Description 	Provides Service information directly to the application. This event provides a means for a vendor-specific Device Monitor Device Service to provide events to the application that are not otherwise supported by the device control.

Attributes	This event contains the following attributes:

Attribute	Type	Description

EventNumber	int32	Event number whose specific values are assigned by the Service.

Data	int32	Additional numeric data. Specific values vary by the EventNumber and the Service. This attribute is settable.

Obj	object	Additional data whose usage varies by the EventNumber and the Service. This attribute is settable.

Remarks	This event is to be used only for those types of vendor specific functions that are not otherwise described.

Use of this event may restrict the application program programform being used with other vendor’s devices which may not have any knowledge of the Service’s need for this event.

See Also	“Events" on page Intro-19, directIO method

ErrorEvent

<<event>> 	upos::events:: ErrorEvent
ErrorCode			: int32{read-write}
ErrorCodeExtended	: int32{read-write}
ErrorLocus 		: int32{read-write}
ErrorResponse		: int32{read-write}

Description	Notifies the application that a Device Monitor Device error has been detected and suitable response by the application is necessary to process the error condition.

Attributes	This event contains the following attributes:

Attributes	Type	Description

ErrorCode	int32	Error code causing the error event. 				See a list of Error Codes on page 20.

ErrorCodeExtended	int32	Extended Error code causing the error event.

If ErrorCode is E_EXTENDED, then see values below. Otherwise, it may contain a Service-specific value.

ErrorLocus	int32	Location of the error.

ErrorResponse	int32	Error response, whose default value may               be overridden by the application     	     (i.e., this attribute is settable). 	       See values below.

The ErrorLocus attribute has one of the following values:

Value	Meaning

EL_INPUT	Error occurred while gathering or processing event-driven input. No previously buffered input data is available.

EL_INPUT_DATA	Error occurred while gathering or processing event-driven input, and some previously buffered data is available.

The application’s error event handler can set the ErrorResponse attribute to one of the following values:

Value	Meaning

Goto Table 124

ER_RETRY	Retry sending the data. The error state is exited.
May be valid for some input devices when the locus is EL_INPUT, in which case the input is re-tried, and the error state is exited.

ER_CLEAR	Valid for all locusi: EL_INPUT and EL_INPUT_DATA.  Clear all buffered input or output data (including all asynchronous output).  The error state is exited.  This is the default response when the locus is EL_INPUT.

ER_CONTINUEINPUT 
Only valid when the locus is EL_INPUT_DATA.
Acknowledges that a data error has occurred and directs the Device to continue input processing. The Device remains in the error state and will deliver additional DataEvents as directed by the DataEventEnabled property. When all input has been delivered and DataEventEnabled is again set to true, then another ErrorEvent is delivered with locus EL_INPUT.
This is the default response when the locus is EL_INPUT_DATA.

Remarks	This event is enqueued when an error is detected, and the Device’s State transitions into the error state. Input error events are not delivered until DataEventEnabled is true, so that proper application sequencing occurs.

Unlike a DataEvent, the Device does not disable further DataEvents or input ErrorEvents; it leaves the DataEventEnabled property value at true.    Note that the application may set DataEventEnabled to false within its event handler if subsequent input events need to be disabled for a period of time.

See Also 	“Device Input Model" on page Intro-22, “Error Handling" on page Intro-23,

StatusUpdateEvent

<<event>>	       upos::events:: StatusUpdateEvent
                                    Status			: int32 {read-only}

Description	Notifies the application that there is an operation status change or a status of the Device Monitor device.

Attributes		This event contains the following attribute:

Attributes        Type	Description

Status               int32	Indicates a change in the Device Monitor status 	of the unit.

Note that Release 1.3 added Power State Reporting with additional Power 	reporting StatusUpdateEvent values.

The Update Firmware capability added additional Status values for communicating the status/progress of an asynchronous update firmware process. See “StatusUpdateEvent” description on page 1-34.

Value            		Meaning

DMON_SUE_START_MONITERING
						It will be notified when Device Monitoring start.

DMON_SUE_STOP_MONITORING
						It will be notified when Device Monitoring stop.

Remarks	      Enqueued when the Device Monitor Device detects a power state change or a status change.

See Also 	“Events" on page Intro-19.

Chapter 47

Graphic Display

This Chapter defines the Graphic Display device category.

Summary

| Properties (UML attributes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common | Type | Mutability | Version | May Use After |
| AutoDisable: | boolean | {read-write} | 1.16 | Not supported |
| CapCompareFirmwareVersion: | boolean | {read-only} | 1.16 | open |
| CapPowerReporting: | int32 | {read-only} | 1.16 | open |
| CapStatisticsReporting: | boolean | {read-only} | 1.16 | open |
| CapUpdateFirmware: | boolean | {read-only} | 1.16 | open |
| CapUpdateStatistics: | boolean | {read-only} | 1.16 | open |
| CheckHealthText: | string | {read-only} | 1.16 | open |
| Claimed: | boolean | {read-only} | 1.16 | open |
| DataCount: | int32 | {read-only} | 1.16 | Not supported |
| DataEventEnabled: | boolean | {read-write} | 1.16 | Not supported |
| DeviceEnabled: | boolean | {read-write} | 1.16 | open, & claim |
| FreezeEvents: | boolean | {read-write} | 1.16 | open |
| OutputID: | int32 | {read-only} | 1.16 | open |
| PowerNotify: | int32 | {read-write} | 1.16 | open |
| PowerState: | int32 | {read-only} | 1.16 | open |
| State: | int32 | {read-only} | 1.16 | -- |
| DeviceControlDescription: | string | {read-only} | 1.16 | -- |
| DeviceControlVersion: | int32 | {read-only} | 1.16 | -- |
| DeviceServiceDescription: | string | {read-only} | 1.16 | open |
| DeviceServiceVersion: | int32 | {read-only} | 1.16 | open |
| PhysicalDeviceDescription: | string | {read-only} | 1.16 | open |
| PhysicalDeviceName: | string | {read-only} | 1.16 | open |

Goto Table127

Goto Table126

| Properties (Continued) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Specific | Type | Mutability | Version | May Use After |  |
| CapAssociatedHardTotalsDevice: | string | {read-only} | 1.16 | open |  |
| CapBrightness: | boolean | {read-only} | 1.16 | open |  |
| CapImageType: | boolean | {read-only} | 1.16 | open |  |
| CapStorage: | int32 | {read-only} | 1.16 | open |  |
| CapURLBack: | boolean | {read-only} | 1.16 | open |  |
| CapURLForward: | boolean | {read-only} | 1.16 | open |  |
| CapVideoType: | boolean | {read-only} | 1.16 | open |  |
| CapVolume: | boolean | {read-only} | 1.16 | open |  |
| Brightness: | int32 | {read-write} | 1.16 | open, claim & enable |  |
| DisplayMode: | int32 | {read-write} | 1.16 | open, claim & enable |  |
| ImageType: | string | {read-write} | 1.16 | open, claim & enable |  |
| ImageTypeList: | string | {read-only} | 1.16 | open |  |
| LoadStatus: | int32 | {read-only} | 1.16 | open |  |
| Storage: | int32 | {read-write} | 1.16 | open, claim & enable |  |
| URL: | string | {read-only} | 1.16 | open |  |
| VideoType: | string | {read-write} | 1.16 | open, claim & enable |  |
| VideoTypeList: | string | {read-only} | 1.16 | open |  |
| Volume: | int32 | {read-write} | 1.16 | open, claim & enable |  |
| Methods (UML operations) |  |  |  |  |  |
| Common |  |  |  |  |  |
| Name |  |  |  |  | Version |
| open (logicalDeviceName: string): void {raises-exception} |  |  |  |  | 1.16 |
| close ( ): void {raises-exception, use after open} |  |  |  |  | 1.16 |
| claim (timeout: int32): void {raises-exception, use after open} |  |  |  |  | 1.16 |
| release ( ): void {raises-exception, use after open, claim} |  |  |  |  | Goto Table125 1.16 |
| checkHealth (level: int32): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| clearInput ( ): void {raises-exception, use after open, claim} |  |  |  |  | 1.16 |
| clearInputProperties ( ): void {raises-exception, use after open, claim} |  |  |  |  | 1.16 |
| Methods (UML operations)(Continued) |  |  |  |  |  |
| clearOutput ( ): void {raises-exception, use after open, claim} |  |  |  |  | 1.16 |
| compareFirmwareVersion (firmwareFileName: string, out result: int32): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| directIO (command: int32, inout data: int32, inout obj: object): void {raises-exception, use after open} |  |  |  |  | 1.16 |
| resetStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| retrieveStatistics (inout statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| updateFirmware (firmwareFileName: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| updateStatistics (statisticsBuffer: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| Specific |  |  |  |  |  |
| Name |  |  |  |  | Version |
| cancelURLLoading ( ): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| goURLBack ( ): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| goURLForward ( ): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| loadImage (fileName: string): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| loadURL (uRL: string ): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| playVideo (fileName: string, loop: boolean): void { raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| stopVideo ( ): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |
| updateURLPage ( ): void {raises-exception, use after open, claim, enable} |  |  |  |  | 1.16 |

Goto Table130

Goto Table133

Goto Table132

Goto Table131

Goto Table129

Goto Table128

| Events (UML interfaces) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name |  | Type | Mutability | Version |
| upos::events::DataEvent |  |  |  |  |
| Status: |  |  | Not supported |  |
| upos::events::DirectIOEvent |  |  |  | 1.16 |
| EventNumber: |  | int32 | {read-only} |  |
| Data: |  | int32 | {read-write} |  |
| Obj: |  | object | {read-write} |  |
| upos::events::ErrorEvent |  |  |  | 1.16 |
| ErrorCode: |  | int32 | {read-only} |  |
| ErrorCodeExtended: |  | int32 | {read-only} |  |
| ErrorLocus: |  | int32 | {read-only} |  |
| ErrorResponse |  | int32 | {read-write} |  |
| upos::events::OutputCompleteEvent |  |  |  | 1.16 |
| OutputID: | int32 | {read-only} |  |  |
| upos::events::StatusUpdateEvent |  |  |  | 1.16 |
| Status: |  | int32 | {read-only} |  |
| upos::events::TransitionEvent |  |  | Not supported |  |

General Information

The Graphic Display programmatic name is “Graphic Display”.

Capabilities

The Graphic Display has the following capability:

Displays the specified image files.

Play the specified video.

Display the specified web page.

Notify the application of changes in the load status of the web page.

Graphics Display Class Diagram

The following diagram shows the relationships between the Graphic Display classes.

Fig. Chap. 47-1 Graphic Display Class Diagram

Model

The following display modes exist in the graphics control, and the model differs depending on the display mode:

- Image display mode

- Video display mode.

- Web display mode.

The application can change the display mode by changing the value of the DisplayMode property.

Image Display Mode

The image display mode of the graphics control is as follows.

Goto Table134

The application calls the loadImage method to display the image.                                       The ImageTypeList property lists image files that the device can display.     	            Applications need to support “hard total” services as image files displaying with 	            loadImage method must be placed in the area managed by the “hard total” service.			        							               Prior to start this mode, need to set the appropriate image type file value in the 	             ImageType property from the listed values in the ImageTypeList property, if 	       CapImageType property is true.  Then the application can call the loadImage method         to display the image.								       Raises StatusUpdateEvent at the status change timing of image load start with status         GDSP_SUE_START_IMAGE_LOAD and image load end with status 	   	       GDSP_SUE_END_IMAGE_LOAD. The ImageTypeList property lists image files that        the device can display.

Applications may need to support “Hard Totals” services as image files displaying                     with loadImage method might be placed in the area managed by the associated “Hard Totals" service device. If the CapStorage is either GDSP_CST_ALL or GDSP_CST_HARDTOTALS_ONLY, it is possible to store it in the Associated Hard Totals device and storage device’s open name is held in the CapAssociatedHardTotalsDevice property.

If device supports both Hard Totals device and the host file system, the application should set the Storage property accordingly to tell where to write the image data file.

Video Display Mode

The video display mode of Graphic Display follows the general device behavior model for asynchronous output devices. 					      The graphics control of video display modes are as follows.

Prior to start this mode, need to set the appropriate video type file value in the VideoType property from the listed values in the VideoTypeList property, if CapVideoType property is true. 						          Then the application can call the playVideo method to display the video. Also, the video being displayed is stopped by calling the stopVideo method.

Raises StatusUpdateEvent at the status change timing of start play video with status GDSP_SUE_START  PLAY_VIDEO, and stop play video with status GDSP_SUE_STOP_PLAY_VIDEO.

The Device validates the method parameters an error condition immediately if necessary. If the validation is successful, the Device does the following:

•Buffers the request in program memory, for delivery to the Physical Device as soon as the Physical Device can receive and process it.

• Sets the OutputID property to a unique integer identifier for this request.

• Returns as soon as possible.

When the Device successfully completes a request, an OutputCompleteEvent is enqueued for delivery to the application.

A property of this event contains the output ID of the completed request.

The application should compare the returned OutputCompleteEvent property OutputID value with the OutputID value set by the asynchronous process method call used to send the data in order to track what data has been successfully sent to the device.

If an error occurs while processing a request, an ErrorEvent is enqueued which will be delivered to the application after the events already enqueued, including OutputCompleteEvents. No further asynchronous output will occur until the event has been delivered to the application. If the response is ER_CLEAR, then outstanding asynchronous output is cleared.

Goto Table135

If the response is ER_RETRY, then output is retried; note that if several outputs were simultaneously in progress at the time that the error was detected, then	the Service may need to retry all of these outputs.

Asynchronous output is always performed on a first-in first-out basis. If the device supports concurrent playback, the request will be executed simultaneously. To check if the device supports simultaneous playback, check the CapMultiPlay property.

If the request is terminated before completion, due to reasons such as the application calling the clearOutput method, then no OutputCompleteEvent is delivered. It can also delete the output individually by calling the stopVideo method. Also, in this case OutputCompleteEvent will not be notified.

The video files that the device can display are listed in the VideoTypeList property. Since video files to be displayed using the playVideo method must be placed in an area managed by the associated “Hard Totals" service device. If the CapStorage is either GDSP_CST_ALL or GDSP_CST_HARDTOTALS_ONLY, it is possible to store it in the Associated Hard Totals device and storage device’s open name is held in the CapAssociatedHardTotalsDevice property.

If device supports either or both Hard Totals device and the host file system, the application should set the Storage property accordingly to tell where to write the image data file.

The video display mode of graphics control follows an asynchronous output model. Raises StatusUpdateEvent if Graphic Display device power status or a device status changes are occurred during the video displaying.

Web Display Mode

The web display mode of graphics control is as follows.

The application calls the loadURL method to display the web page.

Raises StatusUpdateEvent at the timing of Web page load start with status GDSP_SUE_START_LOAD_WEBPAGE, load finish with status GDSP_SUE_FINISH_LOAD_WEBPAGE, and load cancel with status GDSP_SUE_CANCEL_LOAD_WEBPAGE. And application can detect the web page loading status.

The latest loading status of the web page is stored in the LoadStatus property when loadURL method is called, and its URL information is stored in the URL property.

Goto Table136

In case when cancelURLLoading method is called during the loading process, current accessed URL information will be stored in the URL property.

The graphics control web display mode follows an asynchronous output model.

Device Sharing

The Graphic Display Device is an exclusive-use device, as follows:

- The application must claim the device before enabling it.

- The application must claim and enable the device before accessing some properties or calling methods that update the device.

See the “Summary” table for precise usage prerequisites.

Properties (UML attributes)

Brightness Property

Syntax	Brightness: int32 {read-write, access after open-claim-enable}

Remarks	Holds the brightness of screen. Legal values range from zero through 100.
This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	CapBrightness Property.

CapAssociatedHardTotalsDevice Property

Syntax	CapAssociatedHardTotalsDevice: string {read-onlywrite, access after open}

Goto Table137

Remarks	Holds the open name of the associated Hard Totals device if the device is able to write to such devices which is the case if CapStorage is either GDSP_CST_ALL or GDSP_CST_HARDTOTALS_ONLY. If CapStorage is GDSP_CST_HOST_ONLY this property value must be the empty string. This property is initialized by the open method.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	CapStorage Property

CapBrightness Property

Syntax	CapBrightness: boolean {read-only, access after open}

Remarks	If true, the application can change the screen brightness.
If false, the application cannot change the screen brightness.
This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	Brightness Property.

CapImageType Property

Syntax	CapImageType: boolean {read-only, access after open}

Remarks	If true, indicate the image type file to be used in this target device as the value of the ImageType property. Otherwise, it is false.  This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	ImageType Property, ImageTypeList Property

CapStorage Property

Syntax	CapStorage: int32 {read-only, access after open}

Remarks	This is an enumeration and announces where the device is able to write the image data file to.
It holds one of the following values.

Value	Meaning

GDSP_CST_HARDTOTALS_ONLY					    Only an associate Hard Totals device is supported.

GDSP_CST_HOST_ONLY	Only the host’s file system is supported.

GDSP_CST_ALL	Both, the associated Hard Totals device and the host’s file system is supported.

This property is initialized by the open method.

If a Hard Totals device is supported the Storage the property value should be GDSP_CST_HARDTOTALS_ONLY or GDSP_CST_ALL, and the property CapAssociatedHardTotalsDevice holds the open name of the associated Hard Totals device.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

See Also	Storage Property, CapAssociatedHardTotalsDevice Property

CapURLBack Property

Syntax	CapURLBack: boolean {read-only, access after open}

Remarks	If true, the previous page exists in the browsing history. Application can return to the previous page with goURLBack method.

If false, there is no previous page in the browsing history.

This property is initialized to false by the open method. Also, as the web page loading state changes, it is set by the device control.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	goURLBack Method.

CapURLForward Property

Syntax	CapURLForward: boolean {read-only, access after open}

Remarks	If true, the next page exists in the browsing history. Application can go to the next page with the goURLForward method.
If false, there is no next page in the browsing history.
This property is initialized to false by the open method. Also, as the web page loading state changes, it is set by the device control.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	goURLForward Method.

CapVideoType Property

Syntax	CapVideoType: boolean {read-only, access after open}

Remarks	If true, indicate the vide type value that can be used in this targeted graphics display device as the value of VideoType Property. Otherwise, it is false.            This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	VideoType Property, VideoTypeList Property

CapVolume Property

Syntax	CapVolume: boolean {read-only, access after open}

Remarks	If true, the application can change the volume of video.
If false, the application cannot change the volume of video.
This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	Volume Property.

DisplayMode Property

Syntax	DisplayMode: int32 {read-write, access after open-claim-enable}

Remarks	Holds the image and/or video displaying mode.

Value	Meaning

GDSP_DMODE_HIDDEN

It is a mode to hide images and/or video

GDSP_DMODE_IMAGE_FIT

It is a mode to display images. The displayed image is enlarged / reduced to the size that maintains the aspect and fits on the screen.

GDSP_DMODE_IMAGE_FILL

It is a mode to display images.

The displayed image is scaled to the size that

maintains the aspect and covers the entire screen.

GDSP_DMODE_IMAGE_CENTER

It is a mode to display images.

The displayed image is displayed in the center of the screen without changing the size.

GDSP_DMODE_VIDEO_NORMAL

It is a mode to display video. The displayed video will be displayed in the center of the screen without resizing.

GDSP_DMODE_VIDEO_FULL

It is a mode to display video.

The displayed video will be displayed in full screen.

GDSP_DMODE_WEB

Display the web screen.

If application hide other modes and screens while displaying images, videos, or web, all displayed contents will be cleared. The video will be stopped while the video is playing.

This property is initialized by the open method.

Goto Table138

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value		       Meaning

E_ILLEGAL	An invalid value was specified.

See Also	loadImage Method, playVideo Method CapCaptureColorSpaceList Property, VideoCaptureMode Property  (They are Video Capture Device Properties)

ImageType Property

Syntax	ImageType: string {read-write, access after open-claim-enable}

Goto Table139

Remarks	Contains the image file type that are support by the device, if CapImageType property is true. For example, if the device supports BMP, then this property should be set to “BMP”. This property value should be set prior to execute the loadImage method.  All of the capable image file types are listed in the ImageTypeList property.  *Notation contents may be different depending on the device. This property is initialized by the open method.  Some possible values of the exception’s ErrorCode property are:

Value		       Meaning

E_ILLEGAL	An invalid value was specified.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	CapImageType Property, ImageTypeList Property, loadImage Method.

ImageTypeList Property

Syntax	ImageTypeList: string {read-only, access after open}

Remarks	Contains the comma-delimited list of image file type that are support by the device. For example, if the device only supports BMP and JPEG, then this property should be set to “BMP,JPEG”.  One of value in the property should be set in the ImageType property, if CapImageType property is true, prior to execute the loadImage method.

*Notation contents may be different depending on the device.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	CapImageType Property, ImageType Property, loadImage Method.

LoadStatus Property

Syntax	LoadStatus: int32 {read-only, access after open-claim-enable}

Goto Table140

Remarks	Holds loading state of web page.

The parameters to be set are as follows.

Value			Meaning

GDSP_LSTATUS_START		Start loading the web page.

GDSP_LSTATUS_FINISH		It has finished loading the web page.

GDSP_LSTATUS_CANCEL 	It has canceled loading the web page

Its value is set prior to a StatusUpdateEvent being delivered to the application.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Storage Property

Syntax	Storage: int32 {read-write, access after open-claim-enable}

Remarks	This is an enumeration and defines where the device writes the recorded image data file to. Should be set before an appropriate method call.
It holds one of the following values.

Value                                 Meaning

GDSP_ST_HARDTOTALS

The image data file is written to the associated Hard Totals device. The property CapAssociatedHardTotalsDevice holds the open name of the associated Hard Totals device.

GDSP_ST_HOST              	The image data file is written to the host’s file system.

GDSP_ST_HOST_HARDTOTALS

The encoded data file is written to the associated Hard Totals device and host’s file system. The property CapAssociatedHardTotalsDevice holds the open name of the associated Hard Totals device.

This property is initialized by the open method according to the value hold by CapStorage. If CapStorage has the value GDSP_CST_ALL, it is initialized to GDSP_ST_HOST_HARDTOTALS.

Errors	UposException may be thrown when this property is accessed.
	For further information, see “Errors" on page Intro-20.

Value	Meaning

E_ILLEGAL	An invalid value was specified, or recording is ongoing.

See Also	CapStorage Property, CapAssociatedHardTotalsDevice Property

URL Property

Syntax	URL: string {read-only, access after open-claim-enable}

Remarks	When the LoadStatus property is GDSP_LSTATUS_START, the URL of the Web page that starts loading is set.

When the LoadStatus property is GDSP _LSTATUS_FINISH, the URL of the loaded Web page is set.

When the LoadStatus property is GDSP _LSTATUS_CANCEL, the URL of the canceled Web page is set.

Its value is set prior to a StatusUpdateEvent being delivered to the application.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	loadStatus Property.

VideoType Property

Syntax	VideoType: string {read-write, access after open-claim-enable}

Goto Table141

Remarks	Contains the video file type that are support by the device, if CapVideoType property is true. For example, if the device supports AVI MJPG, then this property should be set to “AVI MJPG”. This property value should be set prior to execute the playVideo method.  All of the capable video file types are listed in the VideoTypeList property.         					　*Notation contents may be different depending on the device.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20. Some possible values of the exception’s ErrorCode property are:

Value		       Meaning

E_ILLEGAL	An invalid value was specified.

See Also	CapVideoType Property, VideoTypeList Property, playVideo Method.

VideoTypeList Property

Syntax	VideoTypeList: string {read-only, access after open}

Remarks	Contains the comma-delimited list of video file type that are support by the device. if the device only supports AVI_IYUV and AVI_MJPG, then this property should be set to “AVI_IYUV,AVI_MJPG”.　One of value in the property should be set in the VideoType property, if CapImageType property is true, prior to execute the playVideo method.

*Notation contents may be different depending on the device.

This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

See Also	CapVideoType Property, VideoType Property, playVideo Method.

Volume Property

Syntax	Volume: int32 {read-write, access after open-claim-enable}

Remarks	Holds the volume at playing video. Legal values range from zero through 100.
This property is initialized by the open method.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

See Also	CapVolume Property, playVideo Method.

Methods (UML operations)

cancelURLLoading Method

Syntax	cancelURLLoading ( ): 
		void {raises-exception, use after open-claim-enable}

Remarks	Cancel loading web page.

This method is executed asynchronously. The load status is reported by StatusUpdateEvent and OutputCompleteEvent or ErrorEvent.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	It is not loading.

goURLBack Method

Syntax	goURLBack ( ): 
		void {raises-exception, use after open-claim-enable}

Remarks	It returns to the previous page of browsing history.

This method is executed asynchronously. The load status is reported by StatusUpdateEvent and OutputCompleteEvent or ErrorEvent.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	There is no previous page in the browsing history.

See Also	CapURLBack Property.

goURLForward Method

Syntax	goURLForward ( ):
		void {raises-exception, use after open-claim-enable}

Remarks	Go to the next page of browsing history.

This method is executed asynchronously. The load status is reported by StatusUpdateEvent and OutputCompleteEvent or ErrorEvent.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	There is no next page in the browsing history.

See Also	CapURLForward Property.

loadImage Method

Syntax	loadImage (fileName: string): 
		void {raises-exception, use after open-claim-enable}

Parameter	Description

fileName	Specify the file name of the image to be loaded.

Remarks	Load the specified image.

This method fails if the value of the DisplayMode Property is not set to GDSP_DMODE_IMAGE_FIT, GDSP_DMODE_IMAGE_FILL, or   	  GDSP_DMODE_IMAGE_CENTER.

Image files are located in the area as the stored values of the Storage property.

This method is executed asynchronously. Image file loading status is reported by StatusUpdateEvent and OutputCompleteEvent or ErrorEvent.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	 Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported image file was specified.

E_NOEXIST	File does not exist.

See Also	DisplayMode Property.

loadURL Method

Syntax	loadURL (uRL: string): 
		void {raises-exception, use after open-claim-enable}

Parameter	Description

uRL	Specify the uRL of the web page to load.

Remarks	Load the web page with the specified URL.

This method is executed asynchronously. The load status is reported by StatusUpdateEvent and OutputCompleteEvent or ErrorEvent.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified.

playVideo Method

Syntax	playVideo (fileName: string, loop: boolean): 
		void {raises-exception, use after open-claim-enable}

Parameter	Description

fileName		Specify the file name of the video to be played.

loop		If true, loop playback is performed, and if false, loop playback is not performed.

Goto Table142

Remarks	Play the specified video type file that are set in the VideoType property loaded in the storage area by the loadImage method. All of the video loaded file values images are listed in the ImageVideoTypeList property, if CapVideoType property is true.

If the value of the DisplayMode property is not set to GDISP_DMODE_VIDEO_NORMAL, GDISP_DMODE_VIDEO_FULL, this method will fail.

This method is executed asynchronously. To stop video displaying in the middle, call the stopVideo method.

Video files are located in the area as the stored values of the Storage property.

The video file playing status will be informed by the StatusUpdateEvent.

This method is executed asynchronously. Image file loading status and video file playing status are reported by StatusUpdateEvent and OutputCompleteEvent or ErrorEvent.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	An invalid value was specified. Or an unsupported video file was specified.

E_NOEXIST	File does not exist.

See Also	DisplayMode Property.

stopVideo Method

Goto Table143

Syntax	stopVideo ( ): 
void {raises-exception, use after open-claim-enable}

Remarks	Stop the video being displayed.

This method is executed asynchronously.VideoInage file loading status is reported by StatusUpdateEvent and OutputCompleteEvent or ErrorEvent.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	The Video is not playing.

See Also	startplayVideo Method.

updateURLPage Method

Syntax	updateURLPage ( ): 
		void {raises-exception, use after open-claim-enable}

Remarks	Reload the current web page.

This method is executed asynchronously. The load status is reported by StatusUpdateEvent and OutputCompleteEvent or ErrorEvent.

Errors	A UposException may be thrown when this method is invoked. For further information, see “Errors" on page Intro-20.

Some possible values of the exception’s ErrorCode property are:

Value	Meaning

E_ILLEGAL	Web page loading.

Events (UML interfaces)

DirectIOEvent

<<event>>	upos::events::DirectIOEvent

EventNumber	: int32 {read-only}
Data			: int32 {read-write}
Obj			: object {read-write}

Description 	Provides Service information directly to the application. This event provides a means for a vendor-specific Sound Player Service to provide events to the application that are not otherwise supported by the device control.

Attributes	This event contains the following attributes:

Attribute	Type	Description

EventNumber	int32	Event number whose specific values are assigned by the Service.

Data	int32	Additional numeric data. Specific values vary by the EventNumber and the Service. This attribute is settable.

Obj	object	Additional data whose usage varies by the EventNumber and the Service. This attribute is settable.

Remarks	This event is to be used only for those types of vendor specific functions that are not otherwise described.

Use of this event may restrict the application program programform being used with other vendor’s devices which may not have any knowledge of the Service’s need for this event.

See Also	“Events" on page Intro-19, directIO method

ErrorEvent

<<event>> 	upos::events:: ErrorEvent
ErrorCode			: int32{read-write}
ErrorCodeExtended	: int32{read-write}		      ErrorLocus 		: int32{read-write}
ErrorResponse		: int32{read-write}

Goto Table144

Description	Notifies the application that a Graphic Display Device error has been detected and suitable response by the application is necessary to process the error condition.

Attributes	This event contains the following attributes:

Attributes	Type	Description

ErrorCode	int32	Error code causing the error event. 			See a list of Error Codes on page 20.

ErrorCodeExtended	int32	Extended Error code causing the error event.

If ErrorCode is E_EXTENDED, then see values below. Otherwise, it may contain a Service-specific value.

ErrorLocus	int32	Location of the error. If EL_OUTPUT is specified it is indicating that the error occurred while processing asynchronous output. An error occurred during asynchronous action.

ErrorResponse	int32	Error response, whose default value may               be overridden by the application     	     (i.e., this attribute is settable). 	       See values below.

If ErrorCode is E_EXTENDED, then ErrorCodeExtended has one of the following values:

Value	　　　　　Meaning                                                                      EGDSP_NOROOM　　There is not enough room to store the targeted device 　           	                                  for the  image data file.

The ErrorLocus attribute has one of the following values:

Value	Meaning

EL_OUTPUT	Error occurred while processing asynchronous output.

The application’s error event handler can set the ErrorResponse attribute to one of the following values:

Value	Meaning

ER_RETRY	Retry sending the asynchronous output data. The error state is exited. Typically, valid for asynchronous output devices when the locus is EL_OUTPUT, in which case the asynchronous output is retried and the error state is exited. This is the default response when the locus is EL_OUTPUT.

ER_CLEAR	Valid for loci: EL_OUTPUT.  Clear all buffered input or output data (including all asynchronous output). (The effect is the same as when clearOutput method is called.)  The error state is exited.

Remarks	This event is enqueued when an error is detected, and the Device’s State transitions into the error state.

See Also 	 “Error Handling" on page Intro-23, “Device Output Models" on page Intro-25.

OutputCompleteEvent

<<event>>	upos::events::OutputCompleteEvent 
		OutputID : int32{read-only}

Description 	Notify the application that the queued output request associated with the outputID property has completed successfully.

Attributes	This event contains the following attributes:

Attribute	Type	Description

OutputID 	int32	The ID number of the asynchronous output request that is complete.

Remarks	This event is enqueued after the request’s data has been both sent and the Service has confirmation that it was processed by the device successfully.

See Also	“Device Output Models" on page Intro-25

StatusUpdateEvent

<<event>>		upos::events:: StatusUpdateEvent
	Status			 : int32 {read-only}

Description	Notifies the application that there is an operation status change or a status of the Graphic Display device.

Attributes		This event contains the following attribute:

Attributes	Type	Description

Status           int32     Indicates a change of operation status of graphic display 			device

Note that Release 1.3 added Power State Reporting with additional Power 	reporting StatusUpdateEvent values.

The Update Firmware capability added additional Status values for communicating the status/progress of an asynchronous update firmware process. See “StatusUpdateEvent” description on page 1-34.

Value            		Meaning

GDSP_SUE_START_IMAGE_LOAD
				It will be notified when image loading start.

GDSP_SUE_END_IMAGE_LOAD
										It will be notified when image loading end.

GDSP_SUE_START_LOAD_WEBPAGE
		Start loading the web page.

GDSP_SUE_FINISH_LOAD_WEBPAGE
		It has finished loading the web page.

GDSP_SUE_CANCEL_LOAD_WEBPAGE
		It has canceled loading the web page.

GDSP_SUE_START_PLAY_VIDEO

Start playing video.

GDSP_SUE_STOP_PLAY_VIDEO

Stop playing video.

Remarks	Enqueued when the Graphic Display Device detects a power state change or a status change.

See Also 	“Events" on page Intro-19.

Appendix K

Relationship to other OMG  specification and activities

Robotics Domain Task Force

Activities in Robotics Domain Task Force

The OMG Robotics Domain Task Force (Robotics DTF) fosters the integration of robotics systems from modular components through the adoption of OMG standards. It recommends the adoption and extends OMG technologies that apply to the specific domain of robotics systems where no current baseline specifications exist, such as MDA for Robotics. The object technology is not solely limited to software but is extended to real objects. It also collaborates with other organizations for standardization, such as the one for home information appliances, and makes an open effort to increase interoperability in the field of robotics.

(https://www.omg.org/robotics/)

RoIS Specification

Robotic Interaction Service Framework [RoIS] defines several functional components for robotic interaction services.

Definitions related to locations of entities in robotic services will be described with Robotic Localization Service[RLS]. Definitions of status of components in services will be described in conjunction with Robotic Technology Component [RTC], Finite State Machine Component for RTC [FSM4RTC] and Unified Component Model for Distributed Real-Time and Embedded Systems [UCM].

RoIS specification seeks that specify a RoIS framework, on top of which   various service robot applications are developed.

Scope of RoIS specification

They are summarized in the following items.

- Interface between service application and Human Robot Interaction (HRI) engine

- Interface to obtain information from HRI Engine according to the timing of the service application’s needs (Query)

- Interface to receive information from HRI Engine triggered by real time events (Event notification / subscription / cancellation)

- Interface for instructions to device control HRI Engine functions (Command)

- Definition of common messages for all HRI Engines

Robot Service Ontology [RoSO] RFP

A new RFP of Robot Service Ontology[RoSO] currently being discussed in Robotics DTF are based on the concept of RoIS.

RoSO is aiming to define the specification (ontology) that clarifies the concept of a common vocabulary and / or a robot service in order to describe a service provided by a robot or exchange a description of a service provided by a service robot

Below is an example of HRI main component examples from this point of view.

Table K-1 – (From RoIS 1.2) Basic HRI Components

| HRI Component Name | Description |
| --- | --- |
| system information | Provides the information of the system such as status of the system and position of the physical unit. |
| person detection | Detects number of people |
| person localization | Detects position of people |
| person identification | Identifies ID (name) of people |
| face detection | Detects number of human faces |
| face localization | Detects position of human faces |
| sound detection | Detects number of sound sources |
| sound localization | Detects position of sound sources |
| speech recognition | Recognizes person’s speech |
| gesture recognition | Recognizes person’s gesture |
| speech synthesis | Generates robot speech |
| reaction | Performs specified reaction |
| navigation | Moves to specified target location |
| follow | Follows a specified target object |
| move | Moves to specified distance or curve |

Interoperability between UPOS RCSD and RoIs

Rleationsihp between UPOS RCSD and RoIS

OMG's Robotics standard provides a lower level control layer to manage Robot Device with finer granularity and higher accuracy to accommodate a wide range of industry applications.

On the other hand, the UPOS RCSD specification focuses on the functioning of robotic equipment within the retail store environment. In the UPOS RCSD specification robots are treated as peripheral equipment of the latest POS system. Therefore, the UPOS RCSD specification focuses on the definition of the interface between the POS and the robotic device.

RoIS is already existing as OMG standard and it defined a component frame service that was intended for robotic communication services with people.

Therefore, ROIS developed a general robot service framework, which is different from UPOS RCSD, but it is possible to describe the function of UPOS RCSD.

To confirm the compatibility and interoperability of the RCSD functions of RoIS and UPOS, both DTFs created and confirmed the function mapping table.

For this purpose, we use the general RoIS HRI component defined in the RoIS 1.2 specification.

The two teams continue to collaborate between the part of their separate RFP’s and standards that will be established.

For that purpose, it is very necessary to understand the common vocabulary of the robot service and the needs of the ontology.

If each team’s specification satisfies the above mapping table, it is confirmed that the standard can be maintained independently.

In addition, the figure below shows a typical scenario where RCSD and RoIS work independently or in conjunction.

Document History

Version History

| Ver | Date | Sections | Description of Change |
| --- | --- | --- | --- |
| 1.0 | 2019-2-18 |  | Initial Version – additions and updates to UPOS v1.15 |
| 1.1 | 2019-7-09 |  | Revised for the issues and additions from the Review |
| 1.2 | 2020-2-21 |  | Issues, Updates are added version from the Review |
| 1.3 | 2020-7-16 |  | Issues, Updates are added version from the Review |
| 1.4 | 2021-08-10 |  | Issues, Updates are added version from the Review |

Glossary

| Term | Definition |
| --- | --- |
| EVRW | Electronic Value Reader Writer |
| CAT | Credit Authorization Terminal |