Date: March 2024

# Risk Analysis and Assessment Modeling Language (RAAML) 
Libraries and Profiles

Version 1.1 Beta 2

__________________________________________________

OMG Document Number:   ptc/24-03-02

Standard document URL:  http://www.omg.org/spec/RAAML/

__________________________________________________

Copyright © 2021, Dassault Systemes

Copyright © 2021, Ford Motor Company

Copyright © 2021, Gesellschaft für Systems Engineering

Copyright © 2022, Object Management Group, Inc.

USE OF SPECIFICATION - TERMS, CONDITIONS & NOTICES

The material in this document details an Object Management Group specification in accordance with the terms, conditions and notices set forth below. This document does not represent a commitment to implement any portion of this specification in any company's products. The information contained in this document is subject to change without notice.

LICENSES

The companies listed above have granted to the Object Management Group, Inc. (OMG) a nonexclusive, royalty-free, paid up, worldwide license to copy and distribute this document and to modify this document and distribute copies of the modified version. Each of the copyright holders listed above has agreed that no person shall be deemed to have infringed the copyright in the included material of any such copyright holder by reason of having used the specification set forth herein or having conformed any computer software to the specification.

Subject to all of the terms and conditions below, the owners of the copyright in this specification hereby grant you a fully-paid up, non-exclusive, nontransferable, perpetual, worldwide license (without the right to sublicense), to use this specification to create and distribute software and special purpose specifications that are based upon this specification, and to use, copy, and distribute this specification as provided under the Copyright Act; provided that: (1) both the copyright notice identified above and this permission notice appear on any copies of this specification; (2) the use of the specifications is for informational purposes and will not be copied or posted on any network computer or broadcast in any media and will not be otherwise resold or transferred for commercial purposes; and (3) no modifications are made to this specification. This limited permission automatically terminates without notice if you breach any of these terms or conditions. Upon termination, you will destroy immediately any copies of the specifications in your possession or control.

PATENTS

The attention of adopters is directed to the possibility that compliance with or adoption of OMG specifications may require use of an invention covered by patent rights. OMG shall not be responsible for identifying patents for which a license may be required by any OMG specification, or for conducting legal inquiries into the legal validity or scope of those patents that are brought to its attention. OMG specifications are prospective and advisory only. Prospective users are responsible for protecting themselves against liability for infringement of patents.

GENERAL USE RESTRICTIONS

Any unauthorized use of this specification may violate copyright laws, trademark laws, and communications regulations and statutes. This document contains information which is protected by copyright. All Rights Reserved. No part of this work covered by copyright herein may be reproduced or used in any form or by any means--graphic, electronic, or mechanical, including photocopying, recording, taping, or information storage and retrieval systems--without permission of the copyright owner.

DISCLAIMER OF WARRANTY

WHILE THIS PUBLICATION IS BELIEVED TO BE ACCURATE, IT IS PROVIDED "AS IS" AND MAY CONTAIN ERRORS OR MISPRINTS. THE OBJECT MANAGEMENT GROUP AND THE COMPANIES LISTED ABOVE MAKE NO WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, WITH REGARD TO THIS PUBLICATION, INCLUDING BUT NOT LIMITED TO ANY WARRANTY OF TITLE OR OWNERSHIP, IMPLIED WARRANTY OF MERCHANTABILITY OR WARRANTY OF FITNESS FOR A PARTICULAR PURPOSE OR USE.  IN NO EVENT SHALL THE OBJECT MANAGEMENT GROUP OR ANY OF THE COMPANIES LISTED ABOVE BE LIABLE FOR ERRORS CONTAINED HEREIN OR FOR DIRECT, INDIRECT, INCIDENTAL, SPECIAL, CONSEQUENTIAL, RELIANCE OR COVER DAMAGES, INCLUDING LOSS OF PROFITS, REVENUE, DATA OR USE, INCURRED BY ANY USER OR ANY THIRD PARTY IN CONNECTION WITH THE FURNISHING, PERFORMANCE, OR USE OF THIS MATERIAL, EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

The entire risk as to the quality and performance of software developed using this specification is borne by you. This disclaimer of warranty constitutes an essential part of the license granted to you to use this specification.

RESTRICTED RIGHTS LEGEND

Use, duplication or disclosure by the U.S. Government  is subject to the restrictions set forth in subparagraph (c) (1) (ii) of The Rights in Technical Data and Computer Software Clause at DFARS 252.227-7013 or in subparagraph (c)(1) and (2) of the Commercial Computer Software - Restricted Rights clauses at 48 C.F.R. 52.227-19 or as specified in 48 C.F.R. 227-7202-2 of the DoD F.A.R. Supplement and its successors, or as specified in 48 C.F.R. 12.212 of the Federal Acquisition Regulations and its successors, as applicable. The specification copyright owners are as indicated above and may be contacted through the Object Management Group, 9C Medway Road, PMB 274, Milford, MA 01757, U.S.A.

TRADEMARKS

CORBA®, CORBA logos®, FIBO®, Financial Industry Business Ontology®, FINANCIAL INSTRUMENT GLOBAL IDENTIFIER®, IIOP®, IMM®, Model Driven Architecture®, MDA®, Object Management Group®, OMG®, OMG Logo®, SoaML®, SOAML®, SysML®, UAF®, Unified Modeling Language®, UML®, UML Cube Logo®, VSIPL®, and XMI® are registered trademarks of the Object Management Group, Inc.

For a complete list of trademarks, see: http://www.omg.org/legal/tm_list.htm. All other products or company names mentioned are used for identification purposes only, and may be trademarks of their respective owners.

COMPLIANCE

The copyright holders listed above acknowledge that the Object Management Group (acting itself or through its designees) is and shall at all times be the sole entity that may authorize developers, suppliers and sellers of computer software to use certification marks, trademarks or other special designations to indicate compliance with these materials.

Software developed under the terms of this license may claim compliance or conformance with this specification if and only if the software compliance is of a nature fully matching the applicable compliance points as stated in the specification. Software developed only partially matching the applicable compliance points may claim only that the software was based on this specification, but may not claim compliance or conformance with this specification. In the event that testing suites are implemented or approved by Object Management Group, Inc., software developed using this specification may claim compliance or conformance with the specification only if the software satisfactorily completes the testing suites.

# Table of Contents

Preface	1

1.	Scope	4

1.1	Introduction	4

1.2	RAAML Background	4

1.3	Intended Usage	5

1.4	Related Documents	5

2.	Conformance	7

3.	References	8

3.1	Normative References	8

3.2	OMG Documents (Normative References)	8

3.3	Other Normative References	8

3.4	Informative References	8

4.	Acknowledgements	9

5.	Terms and Definitions	10

6.	Acronyms and Abbreviations	11

7.	Additional Information (non-normative)	12

7.1	Language Architecture	12

7.2	Philosophy	12

7.3	Principles of Creating, Editing, and Displaying of Composite Situations in Diagrammatic and Tabular Views	12

7.3.1	Diagrammatic Situation Specification	13

7.3.2	Tabular Situation Specification	15

8.	Diagram Legend (non-normative)	18

9.	Risk Analysis and Assessment Modeling Language (RAAML) Library and Profile	20

9.1	Core	20

9.1.1	Core::Core Library	21

9.1.2	Core::Core Profile	23

9.2	General	25

9.2.1	General::General Concepts Library	26

9.2.2	General::General Concepts Profile	37

9.3	General Security	43

9.3.1	General Security::General Security Concepts Library	43

9.3.2	General Security::General Security Concepts Profile	45

9.4	Methods::FMEA	47

9.4.1	Methods::FMEA::FMEALibrary	48

9.4.2	Methods::FMEA::FMEAProfile	52

9.5	Methods::FTA	53

9.5.1	Methods::FTA::FTALibrary	53

9.5.2	Methods::FTA::FTAProfile	65

9.6	Methods::STPA	78

9.6.1	Methods::STPA::STPA Library	78

9.6.2	Methods::STPA::STPA Profile	88

9.7	GSN	91

9.7.1	GSN::GSN Profile	92

9.8	Methods::ISO 26262	99

9.8.1	Methods::ISO 26262::ISO 26262 Library	99

9.8.2	Methods::ISO 26262::ISO 26262 Profile	113

9.9	Methods::FHA	122

9.9.1	Methods::FHA::FHA Library	122

9.9.2	Methods::FHA::FHA Profile	133

9.10	Methods::RBD	134

9.10.1	Methods::RBD::RBD Library	135

9.10.2	Methods::RBD::RBD Library::ConstraintBlocks	146

9.10.3	Methods::RBD::RBD Profile	169

10.	Views	174

10.1	Core	174

10.1.1	Core::Core Library	174

10.1.2	Core::Core Profile	174

10.2	General	175

10.2.1	General::General Concepts Library	175

10.2.2	General::General Concepts Profile	175

10.3	General Security	176

10.3.1	General Security::General Security Concepts Library	176

10.3.2	General Security::General Security Concepts Profile	177

10.4	Methods::FMEA	178

10.4.1	Methods::FMEA::FMEA Library	178

10.4.2	Methods::FMEA::FMEA Profile	179

10.5	Methods::FTA	179

10.5.1	Methods::FTA::FTALibrary	179

10.5.2	Methods::FTA::FTAProfile	181

10.6	Methods::STPA	183

10.6.1	Methods::STPA::STPA Library	183

10.6.2	Methods::STPA::STPA Profile	185

10.7	GSN	186

10.7.1	GSN::GSN Profile	186

10.8	Methods::ISO 26262	188

10.8.1	Methods::ISO 26262::ISO 26262 Library	188

10.8.2	Methods::ISO 26262::ISO 26262 Profile	189

10.9	Methods::FHA	191

10.9.1	Methods::FHA::FHA Library	191

10.9.2	Methods::FHA::FHA Profile	192

10.10	Methods::RBD	193

10.10.1	Methods::RBD::RBD Library	193

10.10.2	Methods::RBD::RBD Library::ConstraintBlocks	197

10.10.3	Methods::RBD::RBD Profile	199

# Table of Figures

Figure 7.1 – Fundamental situation modeling principles	13

Figure 7.2 – Typical Automotive Situation definition in the ISO26262 Library	14

Figure 7.3 – User Model Defining Operational Situation “Highway Driving Straight at Speed”	15

Figure 7.4 – HazardousEvent Definition in the ISO26262 Library	16

Figure 8.1 – Legend of color codes	18

Figure 8.2 – An example of using a legend	18

Figure 9.1 – Core concepts domain model	20

Figure 9.2 – AnySituation	22

Figure 9.3 – Causality	22

Figure 9.4 – Situation	23

Figure 9.5 – RelevantTo	23

Figure 9.6 – ControllingMeasure	24

Figure 9.7 – Violates	25

Figure 9.8 – IDCarrier	25

Figure 9.9 – AbstractEvent	26

Figure 9.10 – AbstractCause	27

Figure 9.11 – Cause	27

Figure 9.12 – DysfunctionalEvent	28

Figure 9.13 – AbstractFailureMode	28

Figure 9.14 – FailureMode	29

Figure 9.15 – AbstractEffect	30

Figure 9.16 – Effect	30

Figure 9.17 – Activation	31

Figure 9.18 – ErrorPropagation	32

Figure 9.19 – ErrorRealization	32

Figure 9.20 – HarmPotential	33

Figure 9.21 – Hazard	33

Figure 9.22 – Scenario	34

Figure 9.23 – AbstractRisk	34

Figure 9.24 – UndesiredState	35

Figure 9.24 – Limitation	35

Figure 9.24 – Loss	36

Figure 9.24 – Factor	36

Figure 9.25 – FailureMode	37

Figure 9.26 – Error	37

Figure 9.27 – Fault	38

Figure 9.28 – Detection	38

Figure 9.29 – Prevention	39

Figure 9.30 – Mitigation	39

Figure 9.31 – Recommendation	40

Figure 9.32 – FailureState	40

Figure 9.32 – Hazard	41

Figure 9.32 – Item	41

Figure 9.32 – PresentIn	42

Figure 9.32 – SingleElementItem	42

Figure 9.32 – ElementGroupBasedItem	43

Figure 9.41 - Threat	44

Figure 9.42 - Weakness	44

Figure 9.43 - Vulnerability	44

Figure 9.44 - Threat	45

Figure 9.45 - Impacts	45

Figure 9.46 - Asset	46

Figure 9.47 - Valuates	46

Figure 9.48 - SecurityActor	47

Figure 9.49 - PresentedBy	47

Figure 9.33 – AbstractFMEAItem	48

Figure 9.34 – FMEAItem	49

Figure 9.35 – LossOfFunction	50

Figure 9.36 – DegradationOfFunction	51

Figure 9.37 – IntermittentFunction	51

Figure 9.38 – PartialFunction	51

Figure 9.39 – UnintendedFunction	52

Figure 9.40 – ExceedingFunction	52

Figure 9.41 – DelayedFunction	52

Figure 9.42 – FMEAItem	53

Figure 9.43 – FTAElement	54

Figure 9.44 – FTATree	54

Figure 9.45 – Event	55

Figure 9.46 – BasicEvent	55

Figure 9.47 – IntermediateEvent	56

Figure 9.48 – TopEvent	56

Figure 9.49 – ConditionalEvent	57

Figure 9.50 – DormantEvent	57

Figure 9.51 – UndevelopedEvent	57

Figure 9.52 – HouseEvent	58

Figure 9.53 – ZeroEvent	58

Figure 9.54 – Gate	59

Figure 9.55 – AND	59

Figure 9.56 – OR	60

Figure 9.57 – NOT	60

Figure 9.58 – XOR	61

Figure 9.59 – SEQ	61

Figure 9.60 – INHIBIT	62

Figure 9.61 – MAJORITY_VOTE	62

Figure 9.62 – Tree	66

Figure 9.63 – Gate	66

Figure 9.64 – Event	66

Figure 9.65 – DormantEvent	67

Figure 9.66 – BasicEvent	68

Figure 9.67 – ConditionalEvent	69

Figure 9.68 – ZeroEvent	69

Figure 9.69 – HouseEvent	70

Figure 9.70 – AND	71

Figure 9.71 – OR	72

Figure 9.72 – SEQ	72

Figure 9.73 – XOR	73

Figure 9.74 – INHIBIT	74

Figure 9.75 – MAJORITY_VOTE	75

Figure 9.76 – NOT	75

Figure 9.77 – IntermediateEvent	76

Figure 9.78 – TopEvent	77

Figure 9.79 – TransferIn	77

Figure 9.80 – TransferOut	78

Figure 9.81 – OutOfSequence	79

Figure 9.82 – Late	79

Figure 9.83 – Early	79

Figure 9.84 – TooLong	80

Figure 9.85 – TooShort	80

Figure 9.86 – Provided	80

Figure 9.87 – NotProvided	81

Figure 9.88 – LossScenario	81

Figure 9.89 – ProcessModelFlaw	82

Figure 9.89 – InadequateControllerDecisions	83

Figure 9.90 – InadequateControlExecution	83

Figure 9.91 – InadequateProcessBehavior	83

Figure 9.92 – InadequateFeedbackAndInputs	84

Figure 9.93 – UndesiredControlAction	85

Figure 9.95 – RiskRealization	86

Figure 9.95 – ProcessModelFlawFactor	86

Figure 9.95 – ProcessModelFlawConsequence	87

Figure 9.95 – UndesiredControlActionHarmPotential	87

Figure 9.96 – ControlAction	88

Figure 9.97 – Feedback	88

Figure 9.98 – UndesiredControlAction	89

Figure 9.98 – UnsafeControlAction	89

Figure 9.99 – ControlledProcess	89

Figure 9.100 – Actuator	90

Figure 9.101 – Sensor	90

Figure 9.102 – Controller	90

Figure 9.103 – ControlStructure	91

Figure 9.104 – LossScenario	91

Figure 9.105 – Standard UML notation for stereotyped elements (from UML 2.5.1, Figure 12.25)	92

Figure 9.106 - Strategy notation	92

Figure 9.107 - Combined notation	92

Figure 9.108 – GSNNode	93

Figure 9.109 – GSNArgumentNode	93

Figure 9.110 – Solution	94

Figure 9.111 – Goal	94

Figure 9.112 – Strategy	95

Figure 9.113 – ContextualInformation	95

Figure 9.114 – ContextStatement	95

Figure 9.115 – Assumption	96

Figure 9.116 – Justification	96

Figure 9.117 – InContextOf	97

Figure 9.118 – SupportedBy	98

Figure 9.119 – TrafficAndPeople	99

Figure 9.120 – VehicleUsage	99

Figure 9.121 – RoadCondition	100

Figure 9.122 – Location	100

Figure 9.123 – EnvironmentalCondition	101

Figure 9.124 – OperationalCondition	101

Figure 9.125 – AbstractOperationalSituation	102

Figure 9.126 – TypicalAutomotiveSituation	102

Figure 9.127 – Exposure	103

Figure 9.128 – Severity	103

Figure 9.129 – ASIL	104

Figure 9.130 – Controllability	104

Figure 9.131 – Less	105

Figure 9.132 – More	105

Figure 9.133 – No	105

Figure 9.134 – Intermittent	106

Figure 9.135 – Unintended	106

Figure 9.136 – Early	107

Figure 9.137 – Late	107

Figure 9.138 – Inverted	107

Figure 9.139 – HazardousEvent	108

Figure 9.140 – AnyMalfunction	109

Figure 9.141 – AutomotiveEffect	109

Figure 9.142 – ISO26262SafetyRequirementTemplate	110

Figure 9.143 – AccidentScenario	110

Figure 9.144 – AnyTrafficAndPeople	111

Figure 9.145 – AnyVehicleUse	111

Figure 9.146 – AnyRoadCondition	112

Figure 9.147 – AnyLocation	112

Figure 9.148 – AnyEnvironmentalCondition	112

Figure 9.149 – SystemLevelEffect	113

Figure 9.150 – VehicleLevelEffect	113

Figure 9.151 – OperationalSituation	114

Figure 9.152 – MalfunctioningBehavior	114

Figure 9.153 – IndependenceRequirement	115

Figure 9.154 – ASILDecompose	115

Figure 9.155 – SafeState	116

Figure 9.156 – UserInfoRequirement	116

Figure 9.157 – RecoveryRequirement	116

Figure 9.158 – OperatingMode	117

Figure 9.159 – FunctionalSafetyRequirement	117

Figure 9.160 – SoftwareSafetyRequirement	118

Figure 9.161 – HardwareSafetyRequirement	118

Figure 9.162 – TechnicalSafetyRequirement	119

Figure 9.163 – SafetyGoal	119

Figure 9.164 – DependabilityRequirement	119

Figure 9.165 – Verified	120

Figure 9.166 – Confirmed	120

Figure 9.167 – HazardAndRiskAssessment	121

Figure 9.168 – LessonLearned	121

Figure 9.169 – ASILAssignment	121

Figure 9.170 – ASILOverrideRationale	122

Figure 9.192 - ExternalEvent	123

Figure 1.1 - DevelopmentAssuranceLevel	124

Figure 1.1 - OperationalCondition	125

Figure 1.1 - FunctionalFailure	126

Figure 1.1 - Assumption	127

Figure 1.1 - EnvironmentalCondition	128

Figure 1.1 - FDAL	128

Figure 1.1 - FailureEffect	129

Figure 1.1 - FlightPhase	130

Figure 1.1 - FailureCondition	130

Figure 1.1 - IDAL	132

Figure 1.1 - OperationalEvent	132

Figure 1.1 - FunctionalFailure	134

Figure 9.192 - AbstractReliabilitySituation	135

Figure 9.193 - Restorable	136

Figure 9.194 - ComponentReliabilitySituation	137

Figure 9.195 - RestorableComponentReliabilitySituation	138

Figure 9.196 - NonRestorableComponentReliabilitySituation	139

Figure 9.197 - SystemReliabilitySituation	140

Figure 9.198 - RestorableSystemReliabilitySituation	141

Figure 9.199 - InSeries	142

Figure 9.200 - RestorableInSeries	142

Figure 9.201 - InParallel	143

Figure 9.202 - RestorableInParallel	143

Figure 9.203 - HomogeneousKofN	144

Figure 9.204 - RestorableHomogeneousKofN	145

Figure 9.205 - HeterogenerousKofN	145

Figure 9.206 - RestorableHeterogeneousKofN	146

Figure 10.1 – Core Library	174

Figure 10.2 – CoreProfile	174

Figure 10.3 - General Concepts Library	175

Figure 10.4 – General Concepts Profile	176

Figure 10.5 - General Security Concepts Library	176

Figure 10.6 - General Security Concepts Profile	177

Figure 10.7 – FMEA Library	178

Figure 10.8 – FMEA Profile	179

Figure 10.9 – Events	180

Figure 10.10 – FTA Library	181

Figure 10.11 – FTA Profile	182

Figure 10.12 – STPA Library	185

Figure 10.13 – STPA Profile	186

Figure 10.14 – GSN Profile	187

Figure 10.15 – ISO 26262 Library	188

Figure 10.16 – All-Encompassing Operational Situations	189

Figure 10.17 – RequirementManagement	190

Figure 10.18 – ISO 26262 Profile	191

Figure 0.1 - FHA Library	192

Figure A.1 - FHA Profile	193

Figure 10.19 - RBD Library	193

Figure 10.20 - AbstractReliabilitySituation	194

Figure 10.21 - ComponentReliabilitySituation	194

Figure 10.22 - InSeries	194

Figure 10.23 - RestorableInSeries	195

Figure 10.24 - InParallel	195

Figure 10.25 - RestorableInParallel	195

Figure10.26 - HomogeneousKofN	195

Figure 10.27 - RestorableHomogeneousKofN	196

Figure 10.28 - HeterogeneousKofN	196

Figure 10.29 - RestorableHeterogeneousKofN	197

Figure 10.30 - Distributions	198

Figure 10.31 - ReliabilityParameterDistribution	199

Figure 10.32 - RBD Profile	199

Figure 10.33 - NonRestorableComponentReliabilitySituation	200

Figure 10.34 - RestorableComponentReliabilitySituation	200

Table of Tables

Table 1.1 – Table of Related Documents	5

Table 5.1 – Description of terms and definitions used in this specification	10

Table 6.1 – Description of acronyms used in this specification	11

Table 7.1 – Table for Specifying Operational Situations with Situation “Highway Driving Straight at Speed” Defined	15

Table 7.2 – Hazardous Event Table with Grouped Columns	17

Table 9.1 – Mapping of core concepts to the SysML/UML language	21

# Preface

OMG

Founded in 1989, the Object Management Group, Inc. (OMG) is an open membership, not-for-profit computer industry standards consortium that produces and maintains computer industry specifications for interoperable, portable and reusable enterprise applications in distributed, heterogeneous environments. Membership includes Information Technology vendors, end users, government agencies and academia. OMG member companies write, adopt, and maintain its specifications following a mature, open process. OMG's specifications implement the Model Driven Architecture® (MDA®), maximizing ROI through a full-lifecycle approach to enterprise integration that covers multiple operating systems, programming languages, middleware and networking infrastructures, and software development environments. OMG's specifications include: UML® (Unified Modeling Language™); CORBA® (Common Object Request Broker Architecture); CWM™ (Common Warehouse Metamodel); and industry-specific standards for dozens of vertical markets. More information on the OMG is available at https://www.omg.org/.

OMG Specifications

As noted, OMG specifications address middleware, modeling, and vertical domain frameworks. All OMG Specifications are available from this URL: https://www.omg.org/spec.

Specifications are organized by the following categories:

Domain Categories

Platform Categories

Other Categories

All of OMG’s formal specifications may be downloaded without charge from our website. (Products implementing OMG specifications are available from individual suppliers.) Copies of specifications, available in PDF format, may be obtained from the Specifications Catalog cited above or by contacting the Object Management Group, Inc. at:

OMG Headquarters 
92 Medway Road, PMB 274
Milford, MA 01757 USA 
Tel: +1- 781-444-0404 
Fax: +1-781-444-0320 
Email: pubs@omg.org

Certain OMG specifications are also available as ISO standards. Please consult https://www.iso.org

Typographical Conventions

The type styles shown below are used in this document to distinguish programming statements from ordinary English. However, these conventions are not used in tables or section headings where no distinction is necessary.

Times/Times New Roman - 10 pt.: Standard body text

Helvetica/Arial - 10 pt. Bold: OMG Interface Definition Language (OMG IDL) and syntax elements.

Courier - 10 pt. Bold: Programming language elements.

Helvetica/Arial - 10 pt: Exceptions

Note – Terms that appear in italics are defined in the glossary. Italic text also represents the name of a document, specification, or other publication.

Issues

All OMG specifications are subject to continuous review and improvement. As part of this process, we encourage readers to report any ambiguities, inconsistencies, or inaccuracies they may find by completing the Issue Reporting Form listed on the main web page https://www.omg.org, under Documents, Report a Bug/Issue (https://issues.omg.org/issues/create-new-issue).

Scope

Introduction

There are two parts to this specification, one being normative and another informative. The normative part is:

- The Risk Analysis and Assessment Modeling Language (RAAML) Library and Profile (this document) defines concepts and relationships for capturing safety and reliability aspects of a system in the library and profile form.

The informative part is:

- The RAAML Example Model, Annex A (see document ad/2020-11-01), which illustrates practical usages of RAAML.

RAAML Background

Model-Based Systems Engineering (MBSE) is gaining popularity in organizations creating complex systems where it is crucial to collaborate in a multi-disciplinary environment. SysML, being one of the key MBSE components, has a good foundation for capturing requirements, architecture, constraints, views, and viewpoints. However, SysML does not provide the constructs to capture safety, reliability and security information in the system model. A group of industry experts at the OMG has been working since 2016 to define a new specification providing the necessary capabilities.

The need for a standardized UML profile/library for addressing safety, reliability and security aspects emerged long ago. Working group members have seen multiple commercial-grade model-based safety, reliability and security solution implementations being developed during the recent years and successfully used in practice. While the various safety, reliability and security implementations may fit the needs for a specific purpose, there are many instances where information needs to be traced and shared across multiple organizations. These inconsistent model-based solutions prohibit direct model sharing between organizations and across the various tools.  One of the key goals for the working group is to reconcile these different approaches to alleviate the industry from repeatedly formulating safety, reliability and security constructs in their tools. The specification provides the modeling capabilities for tool vendors to build safety, reliability and security modeling tools that provide traditional representations (e.g., trees, tables, etc.) while using a modern model-based approach.

This RAAML 1.1 specification defines extensions to SysML needed to support safety, reliability and security analysis. It describes:

- the core concepts and shows how the simple concepts are powerful enough to unite all safety, reliability and security information across a variety of analysis methods

- the approach to automating several safety and reliability analyses, which is built on leveraging existing SysML functionalities to ensure that the profile and library is usable with existing tooling

- specific safety and reliability analysis methods and application domains that are supported

  - Failure Mode and Effect Analysis (FMEA)

  - Fault Tree Analysis (FTA)

  - Systems Theoretic Process Analysis (STPA)

  - Goal Structuring Notation (GSN)

  - ISO 26262 Road Vehicles - Functional Safety

  - Reliability Block Diagrams (RBD)

  - Functional Hazard Assessment (FHA)

- extension mechanisms that are typically needed by the industry to apply the specification in practice

Intended Usage

The RAAML specification provides the foundation for conducting various safety and quality engineering activities including safety and reliability analysis methods. Besides the method support, linkages to the SysML model-of-interest are provided, enabling integration with and traceability to the analyses. The specification can be used for modeling safety, reliability and security aspects directly in the model or as a standard language to import and export from external safety and reliability tools.

The organization of RAAML facilitates tailoring the methodologies to specific engineering domains and industries to support the various assessment and certification agencies.

Related Documents

The specification is delivered as a set of related documents. The primary normative document is this document, while a set of additional machine-readable documents is provided to specify the UML profiles and model libraries, specified by this standard.

For each safety/reliability domain, supported by this standard (FMEA, FTA, ISO-26262, STPA,  RBD and FHA) there is a pair of profile and library.

In addition to that there is a pair of profile and library for the concepts used in multiple domains – General and General Security; and a pair of profile and library for the very core concepts that might be useful for the implementers of other standards in the safety/reliability/security domain.

GSN stands separately, as it is an add-on, which can be used with any of the aforementioned domains for additional substantiation of the safety models. It consists of just the profile; no library is necessary. The GSN profile only covers the GSN version 2 standard core notation.

Non-normative examples document is also provided, illustrating how to apply RAAML for capturing safety and reliability data.

Table 1.1 – Table of Related Documents

| Document Number | Description | File Name | Nor-mative | Machine Readable |
| --- | --- | --- | --- | --- |
| ptc/24-03-04 | Core portion of the RAAML. | CoreRAAML.xmi | Y | Y |
| ptc/24-03-05 | Library portion of the RAAML. | CoreRAAMLLib.xmi | Y | Y |
| ptc/24-03-06 | General portion, shared across domains of the RAAML. | GeneralRAAML.xmi | Y | Y |
| ptc/24-03-07 | General Library portion, shared across domains of the RAAML. | GeneralRAAMLLib.xmi | Y | Y |
| ptc/24-03-08 | Goal Structuring Notation profile. | GSN.xmi | Y | Y |
| ptc/24-03-09 | FMEA portion of the RAAML. | FMEA.xmi | Y | Y |
| ptc/24-03-10 | FMEA Library portion of the RAAML. | FMEALib.xmi | Y | Y |
| ptc/24-03-11 | FTA (Fault Tree Analysis) portion of the RAAML. | FTA.xmi | Y | Y |
| ptc/24-03-12 | FTA (Fault Tree Analysis) Library portion of the RAAML. | FTALib.xmi | Y | Y |
| ptc/24-03-13 | ISO26262 Functional Safety Standard portion of the RAAML | ISO26262.xmi | Y | Y |
| ptc/24-03-14 | ISO26262 Functional Safety Standard Library portion of the RAAML | ISO26262Lib.xmi | Y | Y |
| ptc/24-03-15 | STPA (Systems Theoretic Process Analysis) portion of the RAAML | STPA.xmi | Y | Y |
| ptc/24-03-16 | STPA (Systems Theoretic Process Analysis) Library portion of the RAAML | STPALib.xmi | Y | Y |
| ptc/24-03-17 | Security profile portion of the RAAML | GeneralRAAMLSecurity.xmi | Y | Y |
| ptc/24-03-18 | Security library portion of the RAAML | GeneralRAAMLSecurityLib.xmi | Y | Y |
| ptc/24-03-19 | RBD (Reliability Block Diagram) profile portion of the RAAML | RBD.xmi | Y | Y |
| ptc/24-03-20 | RBD (Reliability Block Diagram) library portion of the RAAML | RBDLib.xmi | Y | Y |
| ptc/25-TBD-TBD | FHA (Functional Hazard Assessment) profile portion of RAAML | FHA.xmi | Y | Y |
| ptc/25-TBD-TBD | FHA (Functional Hazard Assessment) library portion of RAAML | FHALib.xmi | Y | Y |
| ptc/24-03-21 | MagicDraw model from which all XMIs and images were produced | Safety and Reliability Library and Profile.583.mdzip | N | Y |
| ptc/21-11-22 | Risk Analysis and Assessment Modeling Language 1.1 Examples | OMG RAAML Examples 1.1.docx | N | N |

Conformance

RAAML specifies two types of conformance.

- Type 1 Conformance: RAAML model interchange conformance. A tool demonstrating model interchange conformance can import and export conformant XMI for all valid RAAML models.

- Type 2 Conformance: RAAML View specification conformance. A tool demonstrating view specification conformance shall implement the views specified in RAAML specification.

A tool vendor may choose to implement one method supported by the specification (FMEA, FTA, STPA, GSN, ISO 26262, RBD or FHA) and claim conformance to it.

References

Normative References

The following normative documents contain provisions which, through reference in this text, constitute provisions of this specification. For dated references, subsequent amendments to, or revisions of, any of these publications do not apply.

OMG Documents (Normative References)

- Unified Modeling Language (UML), 2.5.1, December 2017, https://www.omg.org/spec/UML

- Object Constraint Language (OCL), 2.4, February 2014, https://www.omg.org/spec/OCL

- System Modeling Language (SysML) ,1.6, December 2019, https://www.omg.org/spec/SysML

- XMI Metadata Interchange (XMI), 2.5.1, June 2015, https://www.omg.org/spec/XMI

Other Normative References

- IEC 60812 for FMEA, https://webstore.iec.ch/publication/26359 [accessed on October 28, 2020]

- IEC 61025 for FTA, https://webstore.iec.ch/publication/4311 [accessed on October 28, 2020]

- IEC 61508:2010 for Functional safety of electrical/electronic/programmable electronic safety-related systems, https://webstore.iec.ch/publication/22273 [accessed on October 28, 2020]

- International Standardization Organization. ISO PAS 21448:2019(en) Road vehicles – Safety of the intended functionality, https://www.iso.org/standard/70939.html [accessed on October 28, 2020]

- International Standardization Organization. ISO 26262-1:2018 Road vehicles Functional safety - Part 1, Part 3. https://www.iso.org/standard/68383.html [accessed on June 19, 2023]

- N. Leveson and J. Thomas, STPA Handbook, Boston, MA: MIT, March 2018, https://psas.scripts.mit.edu/home/get_file.php?name=STPA_handbook.pdf [accessed on October 28, 2020]

- GSN specification 2, document number SCSC-141B https://scsc.uk/gsn?page=gsn%202standard [accessed on September 29, 2021]

- GSN metamodel mapping to SACM, https://scsc.uk/file/gc/GSN_metamodelV2-2-1210.pdf [accessed on October 7, 2021]

- IEC 61078:2016, Reliability Block Diagrams, https://webstore.iec.ch/publication/25647 [accessed on October 21, 2023]

- ARP4761A, Guidelines for Conducting the Safety Assessment Process on Civil Aircraft, Systems, and Equipment, https://www.sae.org/standards/content/arp4761a/ [accessed on October 28, 2024]

Informative References

- ISO/IEC 15288:2015, Systems Engineering - Systems Life Cycle Processes,
https://www.iso.org/iso/home/store/catalogue_tc/catalogue_detail.htm?csnumber=63711 [accessed on October 28, 2020]

- International Council On Systems Engineering (INCOSE), Systems Engineering Handbook V4, 2015, https://www.incose.org/products-and-publications/se-handbook [accessed on October 28, 2020]

- National Institute of Standards and Technology, “NIST/Sematech Engineering Statistics Handbook”, https://www.itl.nist.gov/div898/handbook/ [accessed on October 21, 2023]

Acknowledgements

The following companies and organizations submitted or supported parts of the original version of this standard:

Industry

- Dassault Systemes (submitter)

- Ford Motor Company (submitter)

- The Aerospace Corporation

Government

- NASA/Jet Propulsion Laboratory

- Commissariat à l’énergie atomique

- German Aerospace Center

- National Institute of Advanced Industrial Science and Technology (AIST)

Vendors

- No Magic owned by Dassault Systemes

Academia

- Massachusetts Institute of Technology

Liaisons

- Gesellschaft für Systems Engineering (submitter)

- MACE

- Assystem

The following persons were members of the team that designed and wrote this International Standard: Achim Weiss, Andreas Knapp, Andrius Armonas, Annelisa Sturgeon, Axel Berres, Christian Lalitsch-Schneider, Christoph Barchanski, Christopher Davey, Damun Mollahassani, Dave Banham, Edith Holland, Geoffrey Biggs, George Walley, Ilse Adamek, Jean-Francois Castet, Jianlin Shi, John Thomas, Kyle Post, Laura Hart, Manfred Koethe, Mark Sampson, Matthias Nagorni, Myron Hecht, Nataliya Yakymets, Rajiv Murali, Regis Casteran, Sarra Yako, Stephan Boutenko, Thomas Krynicki, Tim Weilkiens, Tomas Juknevicius, Vanessa Sehon, Victor Arcos Barraquero, Yan Liu.

For the final edition of the standard, the following people contributed: Andrius Armonas, Axel Berres, Dave Banham, Kyle Post, George Walley, Tomas Juknevicius.

Terms and Definitions

New terms and definitions have been required to create this specification. They are listed in the table below.

Table 5.1 – Description of terms and definitions used in this specification

| Situation | A situation describes a set of situation occurrences of some type. The system, place, time, and state parameters are described by classifiers rather than individual descriptions. A situation occurrence is a system being in a given place at given time and in a given state. For example, “Boeing 747 with S/N 12305 is being refueled at Gate 7 of Amsterdam Schiphol at 11:45 on Monday, 30th of July 2018.” |
| --- | --- |
| Causality | Identifies cause-effect relationship between two situations. Causality could be direct (non-conditional), conditional, probabilistic or any other inter-situation relationship, defined by the user. Multiple situations can cause one situation and vice versa - one situation can cause multiple other situations. For example, a car in frequent contact with salt, causing safety-critical parts to corrode, which causes leaks in the brake line, causing the brakes to fail, causing a car accident, causing a passenger injury. |
| Relevant To | The Relevant To relationship is used to link situations to system model elements to provide context and relevance for the Situation. For example, in an insulin pump, a Situation where the insulin pump cannot be charged would be related to the main battery element in the system model. |
| Controlling Measure | A measure taken to address (mitigate severity, reduce probability of occurrence, increase probability of detection) a potential or real adverse situation. |

Acronyms and Abbreviations

For the purposes of this specification, the following List of acronyms and abbreviations apply.

Table 6.1 – Description of acronyms used in this specification

| AC | Advisory Circular |
| --- | --- |
| AMC | Acceptable Means of Compliance |
| ARP | Aerospace Recommended Practice |
| ASIL | Automotive Safety Integrity Level |
| CDF | Cumulative Distribution Function |
| DAL | Development Assurance Level |
| DET | Detectability |
| FDAL | Function Development Assurance Level |
| FHA | Functional Hazard Assessment |
| FMEA | Failure Mode and Effect Analysis |
| FTA | Fault Tree Analysis |
| GSN | Goal Structuring Notation |
| HARA | Hazard Analysis and Risk Assessment |
| HAZOP | A hazard and operability study |
| HIRF | High-Intensity Radiated Fields |
| IDAL | Item Development Assurance Level |
| MBSE | Model-Based Systems Engineering |
| MTBF | Mean Time Between Failures |
| MTTF | Mean Time to Failure |
| MTTR | Mean Time to Restore |
| ISO | International Standardization Organization |
| OCC | Occurrence |
| OMG | Object Management Group |
| PDF | Probability Density Function |
| RAAML | Risk Analysis and Assessment Modeling Language |
| RBD | Reliability Block Diagram |
| RPN | Risk priority number |
| SEV | Severity |
| STPA | Systems Theoretic Process Analysis |
| SysML | Systems Modeling Language |
| UAF | Universal Architecture Framework |
| UML | Unified Modeling Language |

Additional Information (non-normative)

Language Architecture

The RAAML specification reuses a subset of UML 2.5.1 and SysML 1.6 and provides additional extensions needed to address the Safety and Reliability for UML RFP (ad/2017-03-05) requirements. Those requirements form the basis for this specification. This document specifies the language architecture in terms of UML 2.5.1 and SysML 1.6. It explains the design principles and how they are applied to implement RAAML.

Philosophy

The RAAML working group uses a library approach heavily with a light UML profile support. Using model libraries has several significant benefits compared with implementing everything in a profile:

- It makes use of the full UML structural modeling capabilities instead of just using metamodeling, which are further limited by the UML prescriptions for stereotyping. The tools with good support for UML/SysML class and composite structure diagrams can make use of their existing generic functionality for modeling safety and reliability aspects of a system.

- It enables end users to extend the libraries and profiles provided by the specification because safety and reliability practices vary across domains (automotive, aerospace, nuclear, etc.) and organizations.

- Finally, it is typically easier to make modifications and extensions to model libraries than to profiles, as extensions occur at lower metalevels.

The RAAML development uses a model-driven approach. A simple description of the work process is:

- The specification is generated from the UML model used to describe RAAML. This approach allows the working group members to concentrate on architecture issues rather than documentation production. The UML tool automatically maintains consistency.

Principles of Creating, Editing, and Displaying of Composite Situations in Diagrammatic and Tabular Views

This standard uses UML/SysML structural modeling capabilities to capture safety and reliability data. The safety and reliability data are captured by a collection of scenarios and situations as shown in

Figure 7.1.

Figure 7.1 – Fundamental situation modeling principles

Complex scenarios can be built by inheriting from other scenarios and composing other situations as parts. Scenarios defined in libraries of this standard provide template scenarios from which to be inherited from.  This way multilevel composite situations can be built.

- Situations are UML Classes, SysML Blocks.

- Scenario steps are captured using SysML parts - UML Properties with aggregation set to composite, and type set to sub-situation (which is UML Class, SysML Block); usually an association is also created for this property.

- Situation attribute values are captured using value properties - UML properties with type describing possible values (which is UML DataType, SysML ValueType) with the value specified in the defaultValue field.

When inheriting from library situations the properties of the user defined situations redefine or subset the properties of the library situation.

Note that user’s model can have additional properties (including sub situations, and attributes and other kinds of properties), beyond those defined in the library. However, from the viewpoint of this standard, they carry user-specific extensions and are not relevant.

Situation in the user model can be inherited from the situation in the standard library indirectly through intermediate situations. This can be used to capture generality/specificity between the real-world situations being described and introduce user-specific library extensions.

Creation and Displaying of situation and scenario models can be done in diagrams, usual for UML/SysML tools, e.g., Class or Block Definition and Composite Structure or Internal Block diagrams. This suits rather well for the safety and reliability domains, which are used to graphical information input such as Fault Tree Analysis and Reliability Block Diagrams.However, users of many safety and reliability domains such as FMEA, STPA or ISO26262 are accustomed to tabular information input. Therefore, the principles of how these models can be described in a tabular format are explained in section §7.3.2.

Diagrammatic Situation Specification

Taking the operational situation TypicalAutomotiveSituation from ISO26262 library as an example, here is how the situation “Highway Driving Straight as Speed” would be defined in a diagram.

The ISO26262 library shown in (Figure 7.2) stipulates, that TypicalAutomotiveSituation is described by specifying trafficAndPeople, vehicleUsage, roadCondition, location, and environmentalCondition sub-situations and an Exposure attribute.

Figure 7.2 – Typical Automotive Situation definition in the ISO26262 Library

The “Highway Driving Straight at Speed” situation, in the user model (Figure 7.3) specifies, that Exposure level is E4 (chosen from the level enumeration defined in the library), trafficAndPeople is “Traffic Free Flow” (another situation defined by the user or coming from a library of operational conditions), the vehicleUsage is “Driving at Speed”, location is “City Roads” and “Highway” (two values), while roadCondition and environmentalCondition are left unspecified.

Note that:

- The scenario and sub-situations are inherited from the situations defined in the library.

- Exposure, which is a value attribute (i.e., an attribute, whose type is not a situation, but some data type instead a numeric or enumerated value) is specified by redefining a library attribute and specifying a default value.

- The trafficAndPeople and vehicleUsage attributes, which specify sub-situations, are redefining corresponding library attributes, and specifying a different type. The normal rules for UML attribute redefinition apply, i.e., redefined attribute type must be narrower that the parent attribute type.

- The roadCondition and environmentalCondition are not redefined, therefore they are left unspecified. The attributes type remains the maximally wide, library type (“RoadCondition” and “EnvironmentalCondition” library types).

- Two values are being specified for location attribute. Therefore, two attributes location1 and location2 are defined in the situation. These attributes are sub-setting the parent location attribute instead of redefining, as in case 3 above. Note that, according to UML rules, names of the sub-setting attributes are not regulated and therefore they can be anything. However, it is strongly recommended that the tool vendor adopt some intuitive, user-friendly naming scheme like parent_attribute_name+number.

Figure 7.3 – User Model Defining Operational Situation “Highway Driving Straight at Speed”

Tabular Situation Specification

The same TypicalAutomotiveSituation, defined by the ISO26262 library and again shown in Figure 7.2, can also define a table format for entering automotive situation user model data in a tabular format.

The table for specifying typical automotive situations comprises the main Name column for defining the situation itself, plus one column per each attribute. A table for typical automotive situations, as defined by TypicalAutomotiveSituation library situation class would then have columns for Exposure, vehicleUsage, trafficAndPeople, location, roadCondition, and environmentalCondition. The column’s name does not need to follow library attributes strictly. They can be beautified, for the sake of user-friendliness. It is important that when the user adds or edits rows in this table, the underlying model data must be created in accordance with the chapters above.

The table below (Table 7.1) shows the same “Highway Driving Straight as Speed” situation defined in tabular format as in the previous chapter. Therefore, the underlying UML model structures must be the same as those shown in diagrammatic format (

Figure 7.1).

Table 7.1 – Table for Specifying Operational Situations with Situation “Highway Driving Straight at Speed” Defined

| # | Name | Exposure | Vehicle Usage | Traffic and People | Location | Road Condition | Environmental Condition |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Highway Driving Straight at Speed | E4 | Driving at Speed | Traffic Free Flow | Highway, City Roads |  |  |
| 1.1 | Highway Driving Straight at Speed, Dangerous Conditions | E3 | Driving at Speed | Traffic Free Flow | Highway, City Roads | Wet, Ice | Reduced Visibility |

A typical safety and reliability domain such as ISO26262 will then use multiple tables, one for each of the structures defined in the library for that domain.

The tables can have additional columns, at the vendor’s discretion, for specifying additional data about the situation, being described in a row. An example of such data could be a description (realized by e.g., UML Comment) of the situation.

Sub-classing by using a generalization relationship between situations can be expressed in tabular format, using hierarchical indented text in table row. In the above table, the “Highway Driving Straight at Speed, Dangerous Conditions” situation is a subclass of the “Highway Driving Straight at Speed” situation. Therefore, a generalization relationship is created between the two in the model. Note that the more specific situation can narrow down the field types of the parent. In this example, the sub-classing situation provides additional data for road and environmental conditions by using attributes and redefining attributes from the library.  Using UML redefinition overrides the parent exposure to E3.  The vehicle use, traffic and people, and location settings are inherited from the parent and do not require additional model elements.

In case of multiple composition levels between the situations defined the in the library, it is possible to show multi-level composite situation data in a single table instead of the multiple interrelated tables by using hierarchical grouped column approach.

An example of using this hierarchical approach is shown for the main situation - HazardousEvent - in the library for ISO26262 standard (Figure 7.4):

Figure 7.4 – HazardousEvent Definition in the ISO26262 Library

The HazardousEvent comprises sub-situations hazard, systemLevelEffect, vehicleLevelEffect which are elementary and an accidentScenario which is a composite sub-situation. AccidentScenario is composed of the elementary malfunctioningBehavior and operationalSituation.  OperationalSituation is composed of a multitude of operational condition sub-situations vehicleUsage, trafficAndPeople, location, roadCondition, and environmentalCondition.

If tabular format is used for entering this information, there could be 3 simple tables:

- Table for operational situations, having columns for vehicleUsage, trafficAndPeople, location, roadCondition, and environmentalCondition.

- Table for accident scenarios, having columns for malfunctioningBehavior and operationalSituation.

- Table for hazardous events, having columns for hazard, systemLevelEffect, vehicleLevelEffect, and accidentScenario.

Alternatively, all this data can be entered in a single table, as shown in Table 7.2:

- Table for hazardous events, having columns for hazard, systemLevelEffect, vehicleLevelEffect, and an accidentScenario.

  - Accident scenario is a column group, comprising of columns malfunctioningBehavior and operationalSituation.

    - Operational situation is a column group comprising of columns vehicleUsage, trafficAndPeople, location, roadCondition, and environmentalCondition.

Table 7.2 – Hazardous Event Table with Grouped Columns

| Name | Hazard | Accident Scenario |  |  |  |  |  |  |  | System Level Effect | Vehicle Level Effect |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Malfunctioning Behavior | Operational Situation |  |  |  |  |  | Controllability |  |  |
|  |  |  | Vehicle Usage | Traffic and People | Location | Road Condition | Environmental Condition | Exposure |  |  |  |

Note – some columns (like ASIL level, or names of accident scenario, operational situation) have been skipped in the table for compactness reasons; in the actual tool that is not limited by page width they would be present.

Diagram Legend (non-normative)

The section 9 is comprised of diagrams that represent elements from the RAAML 1.0 specification. The diagrams are color-coded to help the reader to understand the model easier. Please refer to the legend in Figure 8.1 to understand the diagrams.

|  | Diagram shapes color-coded using gray color represent elements belonging to other packages than the one being specified in the current diagram. |
| --- | --- |
|  | Diagram shapes color-coded using white color represent elements belonging to packages that are being specified in the current diagram. |

Figure 8.1 – Legend of color codes

An example in Figure 8.2 demonstrates how legends are used. Elements that belong to FTA (Fault Tree Analysis) library will be represented in white color in diagrams which belong to FTA method specification. Other elements like DysfunctionalEvent will be represented in gray since they belong to the General part of the specification.

Figure 8.2 – An example of using a legend

This page intentionally left blank.

Risk Analysis and Assessment Modeling Language (RAAML) Library and Profile

The RAAML library and profile imports the entire SysML profile. The use of this import is intended to provide more seamless integration with system modeling using SysML and to be able to fully leverage the capabilities of SysML.

Core

The core concepts domain model is depicted in Figure 9.1. The submission team uses this domain model to derive the CoreLibrary and CoreProfile packages (specified in sections 9.1.1 and 9.1.2 respectively). The other libraries and profiles of the specification are based on the CoreLibrary and CoreProfile packages and contain elements and relationships representing concepts common across safety and reliability analysis methods.

Figure 9.1 – Core concepts domain model

The central element in the core concepts domain model is the “Situation” concept. A situation occurrence is defined as a system being in a given place at given time and in a given state. For example, “Boeing 747 with S/N 12305 is being refueled at Gate 7 of Amsterdam Schiphol at 11:45 on Monday, 30th of July 2018.” An elementary situation is a classifier. It describes a set of situation occurrences of some type. The system, place, time, and state parameters are described by classifiers rather than individual descriptions.

When describing a situation, some of its parameters may be omitted if the situation does not need to be specific with respect to that parameter. For example:

- Fire in the engine compartment of the ship.

- Finger injury of the circular saw operator.

Different Situations can have generalization/specialization relationships between them. Generalization between two situations expresses the subset/superset relationship between the sets of occurrences that these situations represent. For example, “bone fracture” may be defined as a subtype of “Injury”.

Situations can have quantitative attributes, such as probability of occurrence. These are defined using the DependabilityAttribute class. Quantitative attributes can be related to each other and to attributes of the system by formulae using the AttributeRelation class. Formulae can be expressed in any language that the modeling tool can compute, including OCL and other executable languages. For example:

FMEAItem.RiskPriorityNumber = Cause.Occurrence × FailureMode.Detectability ×

Effect.Severity

Different Situations can be associated with each other using the Causality class, expressing semantic relationships between situations such as simple causality, conditional causality, and probabilistic connections. These relations may also have quantitative attributes, such as the probability of occurrence of the “to” situation if the “from” situation occurs. For example, a car in frequent contact with salt, causing safety-critical parts to corrode, which causes leaks in the brake line, causing the brakes to fail, causing a car accident, causing a passenger injury.

A non-elementary situation (the “Composition” relationship in Figure 9.1) is a concept encompassing multiple elementary situations: a single system or combination of several systems in a mutable layout, flowing in time through a sequence of states. The choice of whether to use a composite situation with parts described by subsituations, or to use a single situation, is at the discretion of the modeler. It depends on the modeler's needs, such as the depth of analysis required.

Situations can violate requirements, constraints defined/prescribed for the system, or other specifications describing how the system should operate. For example, a Situation where the system can-not detect glucose level violates the requirement that “the insulin pump must work for 1 week without the need to replace batteries”.

The RelevantTo relationship is used to link situations to system model elements to provide context and relevance for the Sitution. For example, in the aforementioned insulin pump, a Situation where the insulin pump cannot be charged would be related to the main battery element in the system model.

Situations can be mitigated, detected, and prevented via the ControllingMeasure. The use of this relationship introduces new safety requirements.

It was decided early on to reuse as many concepts from the SysML language as possible and only add concepts that are missing in SysML to address safety and reliability aspects of systems. This avoids duplication between two languages that will typically be used together. It also enables tool vendors to implement the new profile and library without requiring new tool capabilities, assuming SysML is supported. This leads to a very small library and profile on top of SysML/UML being sufficient to cover all core concepts. The core domain model is covered by SysML/UML concepts as shown in Table 1. The CoreLibrary package is specified in section 9.1.1. The CoreProfile package is shown in 9.1.2. The Core profile and library are used by all domain-specific methods in the specification.

Table 9.1 – Mapping of core concepts to the SysML/UML language

| Core concept | SysML/UML concept |
| --- | --- |
| Situation | A specialization of a Block in SysML and a new stereotype «Situation » |
| DependabilityAttribute | SysML Value Property |
| AttributeRelation | SysML Constraint Block |
| Generalization | UML Generalization relationship |
| Composition | UML Composition relationship |
| Violates | A stereotyped UML dependency |
| RelevantTo | A stereotyped UML dependency |
| Causality | An association/connector combination |
| ControllingMeasure | A stereotyped UML dependency |

Core::Core Library

AnySituation

Package: Core Library

isAbstract: Yes

Applied Stereotype: «Situation»

Description

AnySituation is the universal root of all situations. All situations inherit from AnySituation. A situation describes a set of situation occurrences of some type. The system, place, time, and state parameters are described by classifiers rather than individual descriptions. A situation occurrence is a system being in a given place at given time and in a given state.

For example, “Boeing 747 with S/N 12305 is being refueled at Gate 7 of Amsterdam Schiphol at 11:45 on Monday, 30th of July 2018.”

Figure 9.2 – AnySituation

Attributes

| from : AnySituation[0..*] (member end of Causality association) | A situation which precedes the one at the other end of the Causality relationship. |
| --- | --- |
| to : AnySituation[0..*] (member end of Causality association) | A situation which follows the one at the other end of the Causality relationship. |

Causality

Package: Core Library

Description

Universal root relationship between situations. All situation relationships inherit from this relationship. Identifies cause and effect relationship between two situations. Causality could be direct (non-conditional), conditional or probabilistic or any other inter-situation relationship, defined by the user. Multiple situations can cause one situation and vice versa - one situation can cause multiple other situations.

For example, a car in frequent contact with salt, causing safety-critical parts to corrode, which causes leaks in the brake line, causing the brakes to fail, causing a car accident, causing a passenger injury.

Figure 9.3 – Causality

Association ends

| to : AnySituation[0..*] (member end of Causality association) | A situation which follows the one at the other end of the Causality relationship. |
| --- | --- |
| from : AnySituation[0..*] (member end of Causality association) | A situation which precedes the one at the other end of the Causality relationship. |

Core::Core Profile

Situation

Package: Core Profile

isAbstract: No

Generalization: Block

Extension: Class

Description

A situation is a SysML v1.6 Block. The situation reuses the following functionality from the Block concept: generalizations, parts, value properties, and Parametrics. The situation stereotype is only needed to distinguish situations from other types of blocks. See AnySituation for the definition of a situation concept.

Figure 9.4 – Situation

RelevantTo

Package: Core Profile

isAbstract: No

Generalization: DirectedRelationshipPropertyPath

Extension: Dependency

Description

The RelevantTo relationship is used to link situations to system model elements to provide context and relevance for the Situation. For example, in an insulin pump, a Situation where the insulin pump cannot be charged would be related to the main battery element in the system model. The RelevantTo relationship reuses the following functionality from the DirectedRelationshipPropertyPath concept: targetContext and targetPropertyPath.

Figure 9.5 – RelevantTo

Constraints

| [1] ClientIsSituation | -- client of the RelevantTo must be a Situation Situation.allInstances().base_Class->includesAll(self.base_Dependency.client) |
| --- | --- |

ControllingMeasure

Package: Core Profile

isAbstract: Yes

Generalization: DirectedRelationshipPropertyPath

Extension: Dependency

Description

A measure taken to address (mitigate severity, reduce probability of occurrence, increase probability of detection) a potential or real adverse situation.

Figure 9.6 – ControllingMeasure

Attributes

| affects : Property[0..*] | Indicates that this controlling measure influences (typically improves) a particular quantitative attribute of the situation. |
| --- | --- |

Constraints

| [1] SupplierIsSituation | -- supplier of the ControllingMeasure must be a Situation Situation.allInstances().base_Class->includesAll(self.base_Dependency.supplier) |
| --- | --- |

Violates

Package: Core Profile

isAbstract: No

Extension: Dependency

Description

The violates relationship indicates a situation where a system is violating a prescription (requirement, constraint, etc.). It is used to connect situations to requirements, design constraints and any other elements of system models which prescribe a characteristic of the system.

For example, a Situation where the insulin pump drains the battery in 3 days violates the requirement that “The system must work for 1 week without the need to replace batteries”.

Figure 9.7 – Violates

Constraints

| [1] ClientIsSituation | -- client of the Violates must be a Situation Situation.allInstances().base_Class->includesAll(self.base_Dependency.client) |
| --- | --- |

IDCarrier

Package: ISO 26262 Profile

isAbstract: No

Extension: Element

Description

Additional stereotype for carrying human-readable identification data.

Figure 9.8 – IDCarrier

Attributes

| Id : String[1] | Human readable identifier. |
| --- | --- |

General

The specification includes a general safety and reliability package that extends the core package. It defines common concepts that are used or extended in the method- and domain-specific reliability and safety packages. The package provides a model library, specified in section 9.2.1, and a profile, specified in section 9.2.2.

The general concepts contained in this package can be used as-is to model the safety and reliability related aspects of a system. However, the intended purposes of the package are as follows:

- Provide a common base for the method- and domain-specific reliability and safety modeling packages. The same concepts are used in a number of safety and reliability techniques (such as FMEA and FTA), so the role of this package is to prevent duplication of common concepts in other packages. This also enables movement of information between domains for cross-domain issues. This is particularly important as different domains may use the same concepts with different vocabulary. A common foundation provides a way to translate between these.

- Provide traceability links between safety and reliability artefacts across the system life cycle. For example, the failure modes defined during Hazard Analysis and Risk Assessment (HARA, defined in the ISO 26262 package) and in an FMEA could be traced and considered during an FTA.

- Provide a foundation on which additional methods, techniques and domains with safety and reliability concerns not currently included in the profile can be built by users. For example, a tool vendor could build an additional package for the railway domain by building on the general safety and reliability foundation. This both reduces effort to introduce an additional domain and allows additional domain packages to be compatible with the existing specification content.

General::General Concepts Library

AbstractEvent

Package: General Concepts Library

isAbstract: Yes

Generalization: AnySituation

Applied Stereotype: «Situation»

Description

Anything that causes a change in a system under analysis or environment. Event has an identifiable starting point in time.

Figure 9.9 – AbstractEvent

Attributes

| likelihood : [1] | A placeholder attribute for indicating likelihood of occurrence of an event. It is intentionally left without a type. Method developers can derive more specialized ways to characterize likelihood. |
| --- | --- |

AbstractCause

Package: General Concepts Library

isAbstract: Yes

Generalization: AbstractEvent, Factor

Applied Stereotype: «Situation»

Description

An AbstractCause is a precursor event that activates other events. The AbstractCause is a root class for all kinds of causes; method developers should derive from it more specific kinds of causes with specific types for occurrence property. One case is demonstrated in the Cause element that redefines the occurrence property of the AbstractCause with the type Real.

See the diagram GeneralConceptsLibrary.

See also: fault association end of the Activation association.

Figure 9.10 – AbstractCause

Attributes

| occurrence : [1], redefines likelihood | A placeholder attribute without a type declared, for indicating how often this situation occurs. It is a redefinition of likelihood. |
| --- | --- |
| premitigationOccurrences : [0..*] | A placeholder attribute for indicating how often this situation occurred prior to mitigation. This property can have more than one value. |

Cause

Package: General Concepts Library

isAbstract: Yes

Generalization: AbstractCause

Applied Stereotype: «Situation»

Description

A Cause is a specific implementation of AbstractCause that defines occurrence property with the type Real.

Figure 9.11 – Cause

Attributes

| occurrence : Real[1], redefines occurrence | An attribute with the type Real, for indicating how often this situation occurs. |
| --- | --- |
| premitigationOccurrences : Real[0..*], redefines premitigationOccurrences | An attribute for indicating how often this situation occurred prior to mitigation. This property can have more than one value. |

DysfunctionalEvent

Package: General Concepts Library

isAbstract: Yes

Generalization: AbstractEvent

Applied Stereotype: «Situation»

Description

An event whose occurrence can cause a dysfunctional behavior of a system or a part of the system.

The DysfunctionalEvent concept is a generalization of such concepts as failure, feared event, etc. that are considered in the domain-specific safety standards. It might be extended for introducing new safety and reliability methods and techniques.

Figure 9.12 – DysfunctionalEvent

AbstractFailureMode

Package: General Concepts Library

isAbstract: Yes

Generalization: UndesiredState

Applied Stereotype: «FailureMode»

Description

The manner in which a system or part of a system (e.g., functions, components, hardware, software, hardware parts, software units), can fail (ISO 26262-1:2018, definition 3.51, modified).

The AbstractFailureMode is a root class for all failure modes; method developers should derive more specific kinds of failure modes with specific types for the detectability property. One case is demonstrated in the FailureMode element that redefines the detectability property of the AbstractFailureMode with the type Real.

Figure 9.13 – AbstractFailureMode

Attributes

| detectability : [1] | A placeholder attribute without a type declared, for indicating how easy it is to detect this failure mode. |
| --- | --- |
| premitigationDetectabilities : [0..*] | A placeholder attribute for indicating how easy it would have been to detect the situation with the previous design iteration. This property can have more than one value. |

FailureMode

Package: General Concepts Library

isAbstract: Yes

Generalization: AbstractFailureMode

Applied Stereotype: «FailureMode»

Description

FailureMode is a specific implementation of AbstractFailureMode that defines the detectability property with the type Real.

A failure is an instance of a FailureMode.

Figure 9.14 – FailureMode

Attributes

| detectability : Real[1], redefines detectability | An attribute with the type Real, for indicating how easy it is to detect the situation. |
| --- | --- |
| premitigationDetectabilities : Real[0..*], redefines premitigationDetectabilities | An attribute for indicating how easy it would have been to detect the situation with the previous design iteration. This property can have more than one value. |

AbstractEffect

Package: General Concepts Library

isAbstract: Yes

Generalization: DysfunctionalEvent

Applied Stereotype: «Situation»

Description

An AbstractEffect is a DysfunctionalEvent that is a result or a consequence of another Situation. The AbstractEffect is a root class for all effects; method developers should derive more specific kinds of effects with specific types for the severity property.

One case is demonstrated in the Effect element that redefines the severity property of the AbstractEffect with the type Real.

See the diagram GeneralConceptsLibrary.

See also: ErrorPropagation, ErrorRealization associations.

Figure 9.15 – AbstractEffect

Attributes

| severity : [1] | A placeholder attribute without a type declared, for indicating the estimate of the extent of harm. |
| --- | --- |
| premitigationSeverities : [0..*] | A placeholder attribute for indicating the estimate of the extent of harm that would have resulted from the previous design iterations. This property can have more than one value. |

Effect

Package: General Concepts Library

isAbstract: Yes

Generalization: AbstractEffect

Applied Stereotype: «Situation»

Description

An Effect is a specific implementation of AbstractEffect that defines the severity property with the type Real.

Figure 9.16 – Effect

Attributes

| severity : Real[1], redefines severity | An attribute with the type Real, for indicating the estimate of the extent of harm. |
| --- | --- |
| premitigationSeverities : Real[0..*], redefines premitigationSeverities | An attribute for indicating the estimate of the extent of harm that would have resulted from the previous design iterations. This property stores more than one value. |

Activation

Package: General Concepts Library

Generalization: Causality

Description

A causal relationship describing the propagation of the initial AbstractCause situation to the DysfunctionalEvent situation in the system.

Figure 9.17 – Activation

Association ends

| error : DysfunctionalEvent[0..*] (member end of Activation association, redefines to) | The dysfunctional situation (error) of the system. |
| --- | --- |
| fault : AbstractCause[0..*] (member end of Activation association, redefines from) | The causal fault. |

ErrorPropagation

Package: General Concepts Library

Generalization: Causality

Description

A causal relationship describing the propagation of errors (one error leading to another) throughout the system.

Figure 9.18 – ErrorPropagation

Association ends

| toError : DysfunctionalEvent[0..*] (member end of ErrorPropagation association, redefines to) | The successor error. |
| --- | --- |
| fromError : DysfunctionalEvent[0..*] (member end of ErrorPropagation association, redefines from) | The predecessor error. |

ErrorRealization

Package: General Concepts Library

Generalization: Causality

Description

A causal relationship describing the propagation of an error to a failure.

Figure 9.19 – ErrorRealization

Association ends

| failure : DysfunctionalEvent[0..*] (member end of ErrorRealization association, redefines to) | The resulting failure. |
| --- | --- |
| error : DysfunctionalEvent[0..*] (member end of ErrorRealization association, redefines from) | The predecessor error. |

HarmPotential

Package: General Concepts Library

isAbstract: Yes

Generalization: AnySituation

Applied Stereotype: «Situation»

Description

A state where there is the potential of harm. This includes all types of harm arising from malicious or non-malicious causes.

Figure 9.20 – HarmPotential

Hazard

Package: General Concepts Library

isAbstract: Yes

Generalization: HarmPotential

Applied Stereotype: «Situation»

Description

A potential source of harm (IEC 61508-4, 3.1.2). Source of harm is non-malicious.

The term includes danger to persons arising within a short time scale (for example, fire and explosion) and also those that have a long-term effect on a person’s health (for example, release of a toxic substance).

Figure 9.21 – Hazard

Scenario

Package: General Concepts Library

isAbstract: Yes

Generalization: AnySituation

Applied Stereotype: «Situation»

Description

A composite situation, consisting of multiple steps (that are themselves situations). Steps should have causal ordering, indicated by Causality relationships or sub-types thereof.

Figure 9.22 – Scenario

Attributes

| scenarioStep : AnySituation[0..*] (member end of association) | A situation which is a part of a bigger situation - scenario. |
| --- | --- |

AbstractRisk

Package: General Concepts Library

isAbstract: Yes

Generalization: Scenario

Applied Stereotype: «Situation»

Description

An AbstractRisk is a Scenario - combination of harm potential (Hazard or Vulnerability), triggering event (AbstractEvent), and resulting harm (AbstractEffect).

The AbstractRisk is a placeholder to enable modelers to specify methodology-specific kinds of risks.

Figure 9.23 – AbstractRisk

Attributes

| score : | Combination of the probability of occurrence of abstract event resulting from abstract harm and the severity of that harm (IEC 61508-4, 3.1.5, modified). An example could be risk priority number (RPN) in FMEA analysis. |
| --- | --- |
| trigger : AbstractEvent[0..*] (member end of association, subsets scenarioStep) | Triggering event (AbstractEvent) which causes harm to materialize. |
| harm : AbstractEffect[0..*] (member end of association, subsets scenarioStep) | Resulting harm (AbstractEffect). |
| harmPotential : HarmPotential[0..*] (member end of association, subsets scenarioStep) | Pre-existing risk (HarmPotential). |

UndesiredState

Package: General Concepts Library

isAbstract: Yes

Generalization: DysfunctionalEvent

Applied Stereotype: «Situation»

Description

An element's condition as a specific time which represents an unintended situation.

Figure 9.24 – UndesiredState

Limitation

Package: General Concepts Library

isAbstract: Yes

Generalization: Factor

Applied Stereotype: «Situation»

Description

A limiting condition; restrictive weakness; lack of capacity; inability or handicap.

Limitation is a restriction of Capability.

Figure 9.25 – Limitation

Loss

Package: General Concepts Library

isAbstract: Yes

Generalization: AbstractEffect

Applied Stereotype: «Situation»

Description

In STPA, is any effect that is unacceptable and should be prevented. Some factors such as environmental conditions may contribute to a loss but are outside our control.  Examples for losses are:

- Loss of human life or injury

- Vehicle/property damage

- Mission loss (inadequate transportation)

- Loss of customer satisfaction

- Financial loss

- Loss of public image

- Environmental pollution

Figure 9.26 – Loss

Factor

Package: General Concepts Library

isAbstract: Yes

Generalization: AnySituation

Applied Stereotype: «Situation»

Description

A situation that contributes to a result or outcome

Figure 9.27 – Factor

General::General Concepts Profile

FailureMode

Package: General Concepts Profile

isAbstract: No

Generalization: Situation

Extension: Class

Description

See FailureMode library class for the definition of a situation concept.

The FailureMode stereotype is only needed to distinguish FailureModes from other types of situations.

Figure 9.28 – FailureMode

Error

Package: General Concepts Profile

isAbstract: No

Generalization: Situation

Extension: Class

Description

The discrepancy between a computed, observed, or measured value or condition and the true, specified or theoretically correct value or condition. [IEC 61508-4, 3.6.11].

The Error stereotype is needed to distinguish this type of situations.

Figure 9.29 – Error

Fault

Package: General Concepts Profile

isAbstract: No

Generalization: Situation

Extension: Class

Description

Abnormal condition that may cause a reduction in, or loss of, the capability of a functional unit to perform a required function. [IEC 61508-4, 3.6.1].

Abnormal or undesired condition that can cause an element or a system to fail. [ISO 26262-1:2018, 3.54, modified]

The Fault stereotype is needed to distinguish this type of situations.

Figure 9.30 – Fault

Detection

Package: General Concepts Profile

isAbstract: No

Generalization: ControllingMeasure

Extension: Dependency

Description

A kind of ControllingMeasure taken to increase probability of detecting the situation under analysis. In hardware these measures may include built-in diagnostic tests, or physical inspection and manual tests.

Figure 9.31 – Detection

Prevention

Package: General Concepts Profile

isAbstract: No

Generalization: ControllingMeasure

Extension: Dependency

Description

A kind of ControllingMeasure taken to reduce probability of occurrence of the situation under analysis.

Figure 9.32 – Prevention

Mitigation

Package: General Concepts Profile

isAbstract: No

Generalization: ControllingMeasure

Extension: Dependency

Description

A kind of ControllingMeasure taken to reduce severity of the situation under analysis.

Figure 9.33 – Mitigation

Recommendation

Package: General Concepts Profile

isAbstract: No

Generalization: ControllingMeasure

Extension: Dependency

Description

Recommendation is used to connect the situation to an action item.

An action item is normally a Requirement, but it can be a less "strong" type of advice - comment, rationale, etc.

The requirement is further managed by the requirements management system - it can have responsible persons, due date, verification properties etc.

Figure 9.34 – Recommendation

FailureState

Package: General Concepts Profile

isAbstract: No

Extension: State

Description

State, which the system or a part of the system enters after occurrence of FailureMode (failure).

The Failure state concept might be used in various formal safety and reliability analysis methods based on the state machine notation. Failure states could be tied to FailureModes via the RelevantTo dependency.

Figure 9.35 – FailureState

Undeveloped

Package: General Concepts Profile

isAbstract: No

Extension: Element

Description

Undeveloped stereotype is meant to identify incomplete concepts.

This stereotype can be applied in combination with Goal or Strategy stereotype to express the fact that the goal or strategy is not fully developed, and therefore may lack crucial details.

This stereotype can also be applied to basic event in fault trees to express the fact that it is not fully developed.

##### Hazard

Package: General Concepts Profile

isAbstract: No

Generalization: Situation

Extension: Class

Description

A marker stereotype for hazards (see the Hazard library element for definition).

Figure 9.36 – Hazard

##### Item

Package: General Concepts Profile

isAbstract: Yes

Extension: Element

Description

An item defines a notional boundary for a risk analysis.

Figure 9.37 – Item

Attributes

| member : Element[0..*] | Elements that are inside the boundary of an item. |
| --- | --- |
| boundaryMember : Element[0..*], subsets member | Elements that are on the boundary of an item. They act as interaction points between inside and outside of the item. |

##### PresentIn

Package: General Concepts Profile

isAbstract: No

Generalization: RelevantTo

Extension: Dependency

Description

Declares that a specific situation (e.g. weakness, vulnerability) is persistent when the part/block is included in the system.

Figure 9.38 – PresentIn

##### SingleElementItem

Package: General Concepts Profile

isAbstract: No

Generalization: Item

Extension: Class

Description

An item that is defined by an individual block.

Figure 9.39 – SingleElementItem

Attributes

| member : Element[0..*], redefines member | This is a redefinition of Item::member. When the item is defined by an individual block, this property collects features of this block. |
| --- | --- |
| referencedElement : Element[1] | A reference to a block defining the item. |

##### ElementGroupBasedItem

Package: General Concepts Profile

isAbstract: No

Generalization: ElementGroup, Item

Extension: Comment

Description

An item that is defined by a set of elements using the SysML element goup mechanism.

Figure 9.40 – ElementGroupBasedItem

Attributes

| member : Element[0..*], redefines member, member | The members of the group are specified by SysML ElementGroup::member. |
| --- | --- |

##### Assumption

Package: General Concepts Profile

isAbstract: No

Extension: Constraint

Description

Statements, principles and/or premises offered without proof. An evaluation process which may include one or more types of analysis and experience. [Source: ARP 4761A]

Figure 9.196 - Assumption

General Security

General Security::General Security Concepts Library

Threat

Package: General Security Concepts Library

isAbstract: Yes

Generalization: HarmPotential

Applied Stereotype: «Situation»

Description

A Threat is any circumstance or event (Situation in RAAML) with the potential to adversely impact Assets.

A Threat is the potential cause of unacceptable asset loss and the undesirable consequences or impact of such a loss.

Figure 9.41 - Threat

Weakness

Package: General Security Concepts Library

isAbstract: No

Generalization: Limitation

Applied Stereotype: «Situation»

Description

A “weakness” is a condition under certain circumstances, could contribute to the introduction of Vulnerabilities.

Figure 9.42 - Weakness

Vulnerability

Package: General Security Concepts Library

isAbstract: No

Generalization: Limitation

Applied Stereotype: «Situation»

Description

A Weakness that can be exploited or triggered by a SecurityActor to produce an undesirable behavior (DysfunctionalEvent).

Vulnerability can then be used as a (exploit)scenario step.

Figure 9.43 - Vulnerability

General Security::General Security Concepts Profile

Threat

Package: General Security Concepts Profile

isAbstract: No

Generalization: Situation

Extension: Class

Description

A marker stereotype for Threat.

Figure 9.44 - Threat

Impacts

Package: General Security Concepts Profile

isAbstract: No

Generalization: RelevantTo

Extension: Dependency

Description

How a particular Situation affects the properties of an Asset.

Figure 9.45 - Impacts

Attributes

| affects : Property[0..1] | Indicates which aspect of the asset (e.g. financial, or operational etc.) is being impacted. |
| --- | --- |
| rating : [0..1] | Rating of the Impact on the Asset |

Asset

Package: General Security Concepts Profile

isAbstract: No

Extension: Class

Description

An Asset represents anything that has value to a person or organization.

Asset can be contextualized by the Item.

For that, the recommended way is to create Asset-typed property in the appropriate Item.

Note: this only works for the Item flavors that are based on UML Class. If there are methodologies that have custom kinds of items, not based on UML Class, those methodologies need to specify their own mechanisms to contextualize the Asset

Figure 9.46 - Asset

Valuates

Package: General Security Concepts Profile

isAbstract: No

Generalization: DirectedRelationshipPropertyPath

Extension: Dependency

Description

Relationship connecting Asset description with the underlying system model element.

Figure 9.47 - Valuates

SecurityActor

Package: General Security Concepts Profile

isAbstract: No

Extension: Classifier

Description

A SecurityActor represents an entity that has the potential to affect a Risk - typically a threat actor.

Figure 9.48 - SecurityActor

PresentedBy

Package: General Security Concepts Profile

isAbstract: No

Generalization: RelevantTo

Extension: Dependency

Description

Presentation/Initiation of a Situation by a SecurityActor.

Figure 9.49 - PresentedBy

Methods::FMEA

The Failure Mode and Effects Analysis (FMEA) is a method of inspecting a system to analyze potential failures. Therefore, as many components, assemblies, and subsystems as possible are examined in order to identify these failure modes in a system and their causes and effects.

Methods::FMEA::FMEALibrary

AbstractFMEAItem

Package: FMEALibrary

isAbstract: Yes

Generalization: AbstractRisk

Applied Stereotype: «FMEAItem»

Description

An AbstractFMEAItem is a scenario (more specifically - AbstractRisk scenario) composed of a failure mode, (potentially multiple) cause(s) and effect(s). It stores assessed and mitigated risk priority numbers.

Figure 9.50 – AbstractFMEAItem

Attributes

| RPN : [1], redefines score | The risk priority number ranks the risk of the FMEA item. It is a specialization of AbstractRisk::score. |
| --- | --- |
| failureMode : AbstractFailureMode[1] (member end of association, subsets scenarioStep) | Represents the failure mode which is reached if a system element fails. |
| cause : AbstractCause[1..*] (member end of association, subsets scenarioStep) | Represents the cause of the failure of a system element. |
| finalEffect : AbstractEffect[1..*] (member end of association, redefines harm) | Represents the effect which occurs on the system border. |
| previousRPNValues : [0..*] | Represents the assessed risk priority number before mitigating the risk of a failure. |
| harmPotential : HarmPotential[0] (member end of association, redefines harmPotential, subsets scenarioStep) | Pre-existing risk. Not used in FMEA method, therefore redefined in this library with multiplicity [0] |

FMEAItem

Package: FMEALibrary

isAbstract: Yes

Generalization: AbstractFMEAItem

Applied Stereotype: «FMEAItem»

Description

A FMEAItem is a specialization of AbstractFMEAItem with the Real implementation of quantitative attributes.

Figure 9.51 – FMEAItem

Attributes

| finalEffect : Effect[1..*] (member end of association, redefines finalEffect) | The specialization of AbstractFMEAItem :: finalEffect with the implementation of Effect with Real severity. |
| --- | --- |
| cause : Cause[1..*] (member end of association, redefines cause) | The specialization of AbstractFMEAItem :: cause with the implementation of Cause with Real occurrence. |
| RPN : Real[1], redefines RPN | The specialization of AbstractFMEAItem :: RPN with the type Real. |
| failureMode : FailureMode[1] (member end of association, redefines failureMode) | The specialization of AbstractFMEAItem :: failureMode with the implementation of FailureMode with Real detectability. |
| calculation : RPNCalculation | Link to a formula for RPN calculation. |
| previousRPNValues : Real[0..*], redefines previousRPNValues | The specialization of AbstractFMEAItem :: previousRPNValues with the type Real. |

RPNCalculation

Package: FMEALibrary

isAbstract: No

Applied Stereotype: «ConstraintBlock»

Description

A formula for RPN calculation. This implementation uses multiplication of Occurrence x Detectability x Severity to calculate RPN.

Attributes

| RPN : | Risk priority number |
| --- | --- |
| SEV : | Severity |
| OCC : Real | Occurrence |
| DET : | Detectability |

Constraints

| [1] | Reduced priority number is calculated by simple multiplication of Severity, Detectability and Occurrence. |
| --- | --- |

LossOfFunction

Package: FMEALibrary

isAbstract: Yes

Generalization: FailureMode

Applied Stereotype: «FailureMode»

Description

A failure mode representing loss of function e.g., the function is inoperable, or suddenly fails.

Figure 9.52 – LossOfFunction

DegradationOfFunction

Package: FMEALibrary

isAbstract: Yes

Generalization: FailureMode

Applied Stereotype: «FailureMode»

Description

A failure mode representing a degradation of function or loss of function over time.

Figure 9.53 – DegradationOfFunction

IntermittentFunction

Package: FMEALibrary

isAbstract: Yes

Generalization: FailureMode

Applied Stereotype: «FailureMode»

Description

A failure mode representing an intermittent function or the random stops and starts of a function.

Figure 9.54 – IntermittentFunction

PartialFunction

Package: FMEALibrary

isAbstract: Yes

Generalization: FailureMode

Applied Stereotype: «FailureMode»

Description

A failure mode representing a partial function or loss of performance.

Figure 9.55 – PartialFunction

UnintendedFunction

Package: FMEALibrary

isAbstract: Yes

Generalization: FailureMode

Applied Stereotype: «FailureMode»

Description

A failure mode representing an unintended function, function operating at the wrong time, with unintended direction, or unequal performance.

Figure 9.56 – UnintendedFunction

ExceedingFunction

Package: FMEALibrary

isAbstract: Yes

Generalization: FailureMode

Applied Stereotype: «FailureMode»

Description

A failure mode representing a function exceeding the acceptable operational performance.

Figure 9.57 – ExceedingFunction

DelayedFunction

Package: FMEALibrary

isAbstract: Yes

Generalization: FailureMode

Applied Stereotype: «FailureMode»

Description

A failure mode representing a delayed function or function operating after an unintended time interval.

Figure 9.58 – DelayedFunction

Methods::FMEA::FMEAProfile

FMEAItem

Package: FMEAProfile

isAbstract: No

Generalization: Block

Extension: Class

Description

See AbstractFMEAItem library class for the definition of a FMEA Item concept.

Figure 9.59 – FMEAItem

Constraints

| [1] FMEAItemIsAbstractFMEAItem | -- FMEAItem stereotype can only be applied on any class, specialized from AbstractFMEAItem from FMEA Library self.base_Class->asSet()->closure(general).name->includes('AbstractFMEAItem') |
| --- | --- |

Methods::FTA

Fault Tree Analysis (FTA) is a top-down failure analysis in which an undesired state of a system is analyzed using Boolean logic to combine a series of lower-level (basic) events. This analysis method is used to understand how systems can fail, to identify the best ways to reduce risk and to determine event rates of a safety accident or a functional failure.

The FTA package contains all required elements to implement this analysis. Support for Fault Tree Analysis (FTA) modeling is based on the IEC 61025:2006 standard. Using this standard ensures that the specification offers a form of FTA that is based on best practices and accepted by practitioners. It is also possible for a user to extend the capabilities of the FTA package to enable, for example, dynamic fault tree analysis and component fault tree modeling while still remaining compatible with other information modeled using the specification.

In order to combine FMEA and FTA analysis, a connection between a failure mode and a fault tree event needs to be made. Therefore, the Cause of an FMEAItem can be interpreted as the event which leads to a failure of a system item. By combining FMEAs and FTAs, both analyses can be used to verify the analysis results. This may lead to a better understanding of the behavior of a system during erroneous behavior.

Methods::FTA::FTALibrary

FTAElement

Package: FTALibrary

isAbstract: Yes

Generalization: DysfunctionalEvent

Applied Stereotype: «Situation»

Description

Any of the Events and Gates needed for the evaluation of the TopEvent probability.

Figure 9.60 – FTAElement

Attributes

| probability : Real, redefines likelihood | The probability that the event represented by the owning FTA element occurs. Probability is a Real value between 0 and 1. |
| --- | --- |
| source Gate : Gate (member end of input association) |  |

FTATree

Package: FTALibrary

isAbstract: No

Generalization: FTAElement, Scenario

Applied Stereotype: «Tree»

Description

A collection of FTAElements and their interrelationships for the evaluation of the top event probability.

Figure 9.61 – FTATree

Attributes

| topEvent : Event[1] (member end of association) | Undesired event which lead to the failure of the system. |
| --- | --- |

Methods::FTA::FTALibrary::Events

Package of events for building fault trees.

Event

Package: Events

isAbstract: Yes

Generalization: FTAElement

Applied Stereotype: «Situation»

Description

The Event is a base class for all types of fault tree events. It is a kind of DysfunctionalEvent.

Figure 9.62 – Event

Attributes

| priority : Integer[0..1] | The priority field is only used to indicate the order of this event when multiple events are inputs of Priority AND (SEQ) gate. |
| --- | --- |
| target Gate : Gate (member end of output association) |  |

BasicEvent

Package: Events

isAbstract: No

Generalization: Event

Applied Stereotype: «BasicEvent»

Description

A basic initiating failure requiring no further development.

Figure 9.63 – BasicEvent

IntermediateEvent

Package: Events

isAbstract: No

Generalization: Event

Applied Stereotype: «IntermediateEvent»

Description

An intermediate event is a failure which occurs because of one or more antecedent events acting through logic gates.

Figure 9.64 – IntermediateEvent

Attributes

| probability : Real, redefines probability | Probability of the intermediate event is derived. It is calculated by the gate from the probabilities of the more basic events. |
| --- | --- |

TopEvent

Package: Events

isAbstract: No

Generalization: Event

Applied Stereotype: «TopEvent»

Description

Undesired event - failure or effect - at the top of the fault tree.

Figure 9.65 – TopEvent

Attributes

| probability : Real, redefines probability | The (derived) probability of the top event is the result of the fault tree calculation. |
| --- | --- |

ConditionalEvent

Package: Events

isAbstract: No

Generalization: Event

Applied Stereotype: «ConditionalEvent»

Description

Specific conditions or restrictions that apply to any logic gate (used primarily with PRIORITY AND and INHIBIT gates).

Figure 9.66 – ConditionalEvent

DormantEvent

Package: Events

isAbstract: No

Generalization: Event

Applied Stereotype: «DormantEvent»

Description

The dormant event is similar to BasicEvent but indicates the latent failure which is discovered by periodical tests.

Figure 9.67 – DormantEvent

UndevelopedEvent

Package: Events

isAbstract: No

Generalization: Event

Applied Stereotype: «BasicEvent», «Undeveloped»

Description

An event which is not further developed either because it is of insufficient consequence or because information is unavailable.

Figure 9.68 – UndevelopedEvent

HouseEvent

Package: Events

isAbstract: No

Generalization: Event

Applied Stereotype: «HouseEvent»

Description

An event which can be set to occur or not occur.

Figure 9.69 – HouseEvent

Attributes

| probability : HouseEventProbability, redefines probability | Probability of the house event is 0 or 1. It is set before doing a fault tree evaluation. |
| --- | --- |

ZeroEvent

Package: Events

isAbstract: No

Generalization: Event

Applied Stereotype: «ZeroEvent»

Description

An event which represents a condition or an event that will never occur.

Figure 9.70 – ZeroEvent

Attributes

| probability : Real, redefines probability | The probability of zero event is always 0. |
| --- | --- |

Methods::FTA::FTALibrary::Gates

Package of logical conditions for building fault trees.

Gate

Package: Gates

isAbstract: Yes

Applied Stereotype: «Situation»

Description

An FTAElement that combines input Event probabilities in a prescribed manner to determine output Event probability. The output event occurs if the combination of input events is satisfied. The gate subtypes specify the necessary combination.

Figure 9.71 – Gate

Attributes

| source Event : Event[0..*] (member end of input association) |  |
| --- | --- |
| target Event : Event[1] (member end of output association) |  |

AND

Package: Gates

isAbstract: No

Generalization: Gate

Applied Stereotype: «Block», «AND»

Description

The output event occurs only if all input events occur.

Figure 9.72 – AND

OR

Package: Gates

isAbstract: No

Generalization: Gate

Applied Stereotype: «Block», «OR»

Description

The output event occurs if at least one of input event occurs.

Figure 9.73 – OR

NOT

Package: Gates

isAbstract: No

Generalization: Gate

Applied Stereotype: «Block», «NOT»

Description

The output event occurs if the input event does not occur.

Figure 9.74 – NOT

XOR

Package: Gates

isAbstract: No

Generalization: Gate

Applied Stereotype: «Block», «XOR»

Description

The output event occurs if exactly one of the input events occurs.

Figure 9.75 – XOR

SEQ

Package: Gates

isAbstract: No

Generalization: Gate

Applied Stereotype: «Block», «SEQ»

Description

The output event occurs if all the input events occur in a specific sequence.

Figure 9.76 – SEQ

INHIBIT

Package: Gates

isAbstract: No

Generalization: Gate

Applied Stereotype: «INHIBIT», «Block»

Description

The output event occurs if the (single) input event occurs in the presence of an enabling condition.

Figure 9.77 – INHIBIT

Attributes

| condition : Event[0..*] (member end of condition association) |  |
| --- | --- |

MAJORITY_VOTE

Package: Gates

isAbstract: No

Generalization: Gate

Applied Stereotype: «Block», «MAJORITY_VOTE»

Description

The output event occurs if the majority of the input events occurs. It has a threshold parameter m.

Figure 9.78 – MAJORITY_VOTE

Attributes

| m : Integer | The m parameter defines the number of input events that form a majority. It is not necessarily ceil(number_of_inputs / 2). It is possible to stipulate that e.g., 5 (or 2) input events have to occur out of total of 7 events for majority gate to fire. |
| --- | --- |

Methods::FTA::FTALibrary::Gates::ConstraintBlocks

Reference implementation for the FTA gates.

ANDConstraintBlock

Package: ConstraintBlocks

isAbstract: No

Applied Stereotype: «ConstraintBlock»

Description

Reference implementation for the AND gate.

Attributes

| output : |  |
| --- | --- |
| input : [0..*] |  |

Constraints

| [1] | Probability of AND node is simply a multiplication of probabilities of incoming nodes. Note - this simplistic calculation assumes that incoming node events are mutually independent. |
| --- | --- |

ORConstraintBlock

Package: ConstraintBlocks

isAbstract: No

Applied Stereotype: «ConstraintBlock»

Description

Reference implementation for the OR gate.

Attributes

| output : |  |
| --- | --- |
| input : [0..*] |  |

Constraints

| [1] | Probability of OR node is calculated as opposite probability of the event where neither of the input events happen. This follows De Morgan's theorem - OR(input1, input2, input3...) is equal to NOT AND (NOT input1, NOT input2, NOT input3...). Note - this simplistic calculation assumes that incoming node events are mutually independent. |
| --- | --- |

SEQConstraintBlock

Package: ConstraintBlocks

isAbstract: No

Applied Stereotype: «ConstraintBlock»

Description

Reference implementation for the SEQ gate.

Attributes

| output : |  |
| --- | --- |
| input : Real[0..*] |  |

Constraints

| [1] | Probability of SEQ node is calculated the same way as AND node - it is simply a multiplication of probabilities of incoming nodes. This simplistinc calculation cannot capture time-dependency of the events; only more complex simulations can estimate this probability. |
| --- | --- |

XORConstraintBlock

Package: ConstraintBlocks

isAbstract: No

Applied Stereotype: «ConstraintBlock»

Description

Reference implementation for the XOR gate.

Attributes

| output : |  |
| --- | --- |
| input : [0..*] |  |

Constraints

| [1] | In case of two inputs, XOR probability is calculated by ORing of two event combintation probabilities - probability that first event happened and second did not ORed with probability that second event happened while first did not. Input1 XOR Input2 = Input1 AND NOT Input2 OR Input2 AND NOT Input1 Since combinations are mutually exclusive, simple (+) operation can be used for ORing them. Therefore Input1 XOR Input2 = Input1 AND NOT Input2 + Input2 AND NOT Input1 Further expanding ANDs and NOTs using their corresponding formulas, we get Input1 XOR Input2 = Input1*(1 - Input2) + Input2*(1 - Input1) = Input1 + Input2 - 2 * Input1 * Input2 This formula can be iteratively applied for the case with number of inputs greater than two. Note - this simplistic calculation assumes that incoming node events are mutually independent. |
| --- | --- |

INHIBITConstraintBlock

Package: ConstraintBlocks

isAbstract: No

Applied Stereotype: «ConstraintBlock»

Description

Reference implementation for the INHIBIT gate.

Attributes

| output : |  |
| --- | --- |
| input : [0..*] |  |
| condition : Real |  |

Constraints

| [1] | Probability of INHIBIT node is calculated the same way as AND node - it is simply a multiplication of probabilities of input nodes and condition nodes. Note - this simplistic calculation assumes that incoming node events and conditions are mutually independent. |
| --- | --- |

MAJORITY_VOTEConstraintBlock

Package: ConstraintBlocks

isAbstract: No

Applied Stereotype: «ConstraintBlock»

Description

Reference implementation for the MAJORITY_VOTE gate.

Attributes

| output : |  |
| --- | --- |
| input : [0..*] |  |
| m : |  |

Constraints

| [1] | Majority Vote probability can be calculated by iteratively examining all the combinations of input events, taking those combinations that satisfy the condition that at least m input events happen, then calculating probability of each combination using AND formula (multiplying all individual event probabilities in that combination) and then calculating cumulative probability of all combinations by ORing them. Note - this simplistic calculation assumes that incoming node events are mutually independent. |
| --- | --- |

NOTConstraintBlock

Package: ConstraintBlocks

isAbstract: No

Applied Stereotype: «ConstraintBlock»

Description

Reference implementation for the NOT gate.

Attributes

| output : |  |
| --- | --- |
| input : [1] |  |

Constraints

| [1] | Probability of NOT node is calculated as probability of the event opposite to the input event. Thereby it is unity minus probability of input event. |
| --- | --- |

Methods::FTA::FTAProfile

Tree

Package: FTAProfile

isAbstract: No

Generalization: Situation

Extension: Class

Description

A marker stereotype for fault trees. See FTATree library class for definition.

Figure 9.79 – Tree

Constraints

| [1] TreeIsFTATree | -- Tree stereotype can only be applied on any class specialized from FTATree from FTA Library self.base_Class->asSet()->closure(general).name->includes('FTATree') |
| --- | --- |

Gate

Package: FTAProfile

isAbstract: Yes

Extension: Class, Property

Description

A marker stereotype for fault tree gates. See Gate library class for definition.

Figure 9.80 – Gate

Event

Package: FTAProfile

isAbstract: Yes

Extension: Class, Property

Description

A marker stereotype for fault tree events. See Event library class for definition.
If the Event stereotype is applied to a class, then that class also must have the Situation stereotype (or its descendants) applied.

Figure 9.81 – Event

DormantEvent

Package: FTAProfile

isAbstract: No

Generalization: Event

Extension: Class, Property

Description

A marker stereotype, carrying icon for dormant events. See DormantEvent library class for definition.

Figure 9.82 – DormantEvent

Constraints

| [1] DormantEventIsDormantEvent | if not self.base_Class->isEmpty() then --DormantEvent stereotype can only be applied on any class specialized from DormantEvent from FTA Library self.base_Class->asSet()->closure(general).name->includes('DormantEvent') else --DormantEvent stereotype can only be applied on any property whose type is specialized from DormantEvent from FTA Library self.base_Property.type->asSet()->closure(general).name->includes('DormantEvent') endif |
| --- | --- |

BasicEvent

Package: FTAProfile

isAbstract: No

Generalization: Event

Extension: Class, Property

Description

A marker stereotype, carrying icon for basic events. See BasicEvent library class for definition.

Figure 9.83 – BasicEvent

Constraints

| [1] BasicEventIsBasicEvent | if not self.base_Class->isEmpty() then --BasicEvent stereotype can only be applied on any class specialized from BasicEvent from FTA Library self.base_Class->asSet()->closure(general).name->includes('BasicEvent') else --BasicEvent stereotype can only be applied on any property whose type is specialized from BasicEvent from FTA Library self.base_Property.type->asSet()->closure(general).name->includes('BasicEvent') endif |
| --- | --- |
| [2] UndevelopedEventIsUndevelopedEvent | --BasicEvent + Undeveloped stereotype combination can be applied on any class specialized from UndevelopedEvent from FTA Library Undeveloped.allInstances().base_Element->includesAll(self.base_Class) implies self.base_Class->asSet()->closure(general).name->includes('UndevelopedEvent') |

ConditionalEvent

Package: FTAProfile

isAbstract: No

Generalization: Event

Extension: Class, Property

Description

A marker stereotype, carrying icon for conditional events. See ConditionalEvent library class for definition.

Figure 9.84 – ConditionalEvent

Constraints

| [1] ConditionalEventIsConditionalEvent | if not self.base_Class->isEmpty() then --ConditionalEvent stereotype can only be applied on any class specialized from ConditionalEvent from FTA Library self.base_Class->asSet()->closure(general).name->includes('ConditionalEvent') else --ConditionalEvent stereotype can only be applied on any property whose type is specialized from ConditionalEvent from FTA Library self.base_Property.type->asSet()->closure(general).name->includes('ConditionalEvent') endif |
| --- | --- |

ZeroEvent

Package: FTAProfile

isAbstract: No

Generalization: Event

Extension: Class, Property

Description

A marker stereotype, carrying icon for zero events. See ZeroEvent library class for definition.

Figure 9.85 – ZeroEvent

Constraints

| [1] ZeroEventIsZeroEvent | if not self.base_Class->isEmpty() then --ZeroEvent stereotype can only be applied on any class specialized from ZeroEvent from FTA Library self.base_Class->asSet()->closure(general).name->includes('ZeroEvent') else --ZeroEvent stereotype can only be applied on any property whose type is specialized from ZeroEvent from FTA Library self.base_Property.type->asSet()->closure(general).name->includes('ZeroEvent') endif |
| --- | --- |

HouseEvent

Package: FTAProfile

isAbstract: No

Generalization: Event

Extension: Class, Property

Description

A marker stereotype, carrying icon for house events. See HouseEvent library class for definition.

Figure 9.86 – HouseEvent

Constraints

| [1] HouseEventIsHouseEvent | if not self.base_Class->isEmpty() then --HouseEvent stereotype can only be applied on any class specialized from HouseEvent from FTA Library self.base_Class->asSet()->closure(general).name->includes('HouseEvent') else --HouseEvent stereotype can only be applied on any property whose type is specialized from HouseEvent from FTA Library self.base_Property.type->asSet()->closure(general).name->includes('HouseEvent') endif |
| --- | --- |

AND

Package: FTAProfile

isAbstract: No

Generalization: Gate

Extension: Class, Property

Description

A marker stereotype, carrying icon for AND gates. See AND library class for definition.

Figure 9.87 – AND

Constraints

| [1] ANDIsAND | if not self.base_Class->isEmpty() then --AND stereotype can only be applied on any class specialized from AND gate from FTA Library self.base_Class->asSet()->closure(general).name->includes('AND') else --AND stereotype can only be applied on any property whose type is specialized from AND from FTA Library self.base_Property.type->asSet()->closure(general).name->includes('AND') endif |
| --- | --- |

OR

Package: FTAProfile

isAbstract: No

Generalization: Gate

Extension: Class, Property

Description

A marker stereotype, carrying icon for OR gates. See OR library class for definition.

Figure 9.88 – OR

Constraints

| [1] ORIsOR | if not self.base_Class->isEmpty() then --OR stereotype can only be applied on any class specialized from OR gate from FTA Library self.base_Class->asSet()->closure(general).name->includes('OR') else --OR stereotype can only be applied on any property whose type is specialized from OR from FTA Library self.base_Property.type->asSet()->closure(general).name->includes('OR') endif |
| --- | --- |

SEQ

Package: FTAProfile

isAbstract: No

Generalization: Gate

Extension: Class, Property

Description

A marker stereotype, carrying icon for SEQ gates. See SEQ library class for definition.

Figure 9.89 – SEQ

Constraints

| [1] SEQIsSEQ | if not self.base_Class->isEmpty() then --SEQ stereotype can only be applied on any class specialized from SEQ gate from FTA Library self.base_Class->asSet()->closure(general).name->includes('SEQ') else --SEQ stereotype can only be applied on any property whose type is specialized from SEQ from FTA Library self.base_Property.type->asSet()->closure(general).name->includes('SEQ') endif |
| --- | --- |

XOR

Package: FTAProfile

isAbstract: No

Generalization: Gate

Extension: Class, Property

Description

A marker stereotype, carrying icon for XOR gates. See XOR library class for definition.

Figure 9.90 – XOR

Constraints

| [1] XORIsXOR | if not self.base_Class->isEmpty() then --XOR stereotype can only be applied on any class specialized from XOR gate from FTA Library self.base_Class->asSet()->closure(general).name->includes('XOR') else --XOR stereotype can only be applied on any property whose type is specialized from XOR from FTA Library self.base_Property.type->asSet()->closure(general).name->includes('XOR') endif |
| --- | --- |

INHIBIT

Package: FTAProfile

isAbstract: No

Generalization: Gate

Extension: Class, Property

Description

A marker stereotype, carrying icon for INHIBIT gates. See INHIBIT library class for definition.

Figure 9.91 – INHIBIT

Constraints

| [1] INHIBITIsINHIBIT | if not self.base_Class->isEmpty() then --INHIBIT stereotype can only be applied on any class specialized from INHIBIT gate from FTA Library self.base_Class->asSet()->closure(general).name->includes('INHIBIT') else --INHIBIT stereotype can only be applied on any property whose type is specialized from INHIBIT from FTA Library self.base_Property.type->asSet()->closure(general).name->includes('INHIBIT') endif |
| --- | --- |

MAJORITY_VOTE

Package: FTAProfile

isAbstract: No

Generalization: Gate

Extension: Class, Property

Description

A marker stereotype, carrying icon for MAJORITY_VOTE gates. See MAJORITY_VOTE library class for definition.

Figure 9.92 – MAJORITY_VOTE

Constraints

| [1] MAJORITY_VOTEIsMAJORITY_VOTE | if not self.base_Class->isEmpty() then --MAJORITY_VOTE stereotype can only be applied on any class specialized from MAJORITY_VOTE gate from FTA Library self.base_Class->asSet()->closure(general).name->includes('MAJORITY_VOTE') else --MAJORITY_VOTE stereotype can only be applied on any property whose type is specialized from MAJORITY_VOTE from FTA Library self.base_Property.type->asSet()->closure(general).name->includes('MAJORITY_VOTE') endif |
| --- | --- |

NOT

Package: FTAProfile

isAbstract: No

Generalization: Gate

Extension: Class, Property

Description

A marker stereotype, carrying icon for NOT gates. See NOT library class for definition.

Figure 9.93 – NOT

Constraints

| [1] NOTIsNOT | if not self.base_Class->isEmpty() then --NOT stereotype can only be applied on any class specialized from NOT gate from FTA Library self.base_Class->asSet()->closure(general).name->includes('NOT') else --NOT stereotype can only be applied on any property whose type is specialized from NOT from FTA Library self.base_Property.type->asSet()->closure(general).name->includes('NOT') endif |
| --- | --- |

IntermediateEvent

Package: FTAProfile

isAbstract: No

Generalization: Event

Extension: Class, Property

Description

A marker stereotype, carrying icon for intermediate events. See IntermediateEvent library class for definition.

Figure 9.94 – IntermediateEvent

Constraints

| [1] IntermediateEventIsIntermediateEvent | if not self.base_Class->isEmpty() then --IntermediateEvent stereotype can only be applied on any class specialized from IntermediateEvent from FTA Library self.base_Class->asSet()->closure(general).name->includes('IntermediateEvent') else --IntermediateEvent stereotype can only be applied on any property whose type is specialized from IntermediateEvent from FTA Library self.base_Property.type->asSet()->closure(general).name->includes('IntermediateEvent') endif |
| --- | --- |

TopEvent

Package: FTAProfile

isAbstract: No

Generalization: Event

Extension: Class, Property

Description

A marker stereotype, carrying icon for top events. See TopEvent library class for definition.

Figure 9.95 – TopEvent

Constraints

| [1] TopEventIsTopEvent | if not self.base_Class->isEmpty() then --TopEvent stereotype can only be applied on any class specialized from TopEvent from FTA Library self.base_Class->asSet()->closure(general).name->includes('TopEvent') else --TopEvent stereotype can only be applied on any property whose type is specialized from TopEvent from FTA Library self.base_Property.type->asSet()->closure(general).name->includes('TopEvent') endif |
| --- | --- |

TransferIn

Package: FTAProfile

isAbstract: No

Extension: Property

Description

The node of the current fault tree that indicates that the tree is developed further as a separate fault tree - TransferOut.

Figure 9.96 – TransferIn

Constraints

| [1] TypeIsTransferOut | -- type of TransferIn property must be TransferOut FTA Tree TransferOut.allInstances().base_Class->includesAll(self.base_Property.type) |
| --- | --- |

TransferOut

Package: FTAProfile

isAbstract: No

Generalization: Tree

Extension: Class

Description

A marker stereotype for partial fault trees. It indicates that this tree is used as a part of another fault tree through TransferIn. The computed probability of the top event of the TransferOut tree is used as a probability of the TransferIn node.

Figure 9.97 – TransferOut

Methods::STPA

The System Theoretical Process Analysis (STPA) is a hazard analysis technique based on control and system theory. In comparison, most existing hazard analysis techniques are based on reliability theory. In STPA, however, the easy goals are pursued as in any hazard analysis, i.e., collecting information on how hazards may occur. For further information on this approach the handbook describes the method and show the application.

Methods::STPA::STPA Library

OutOfSequence

Package: STPA Library

isAbstract: Yes

Generalization: UndesiredControlAction

Applied Stereotype: «UndesiredControlAction»

Description

STPA Guideword, describing kind of control.

Figure 9.98 – OutOfSequence

Late

Package: STPA Library

isAbstract: Yes

Generalization: UndesiredControlAction

Applied Stereotype: «UndesiredControlAction»

Description

STPA Guideword, describing kind of control.

Figure 9.99 – Late

Early

Package: STPA Library

isAbstract: Yes

Generalization: UndesiredControlAction

Applied Stereotype: «UndesiredControlAction»

Description

STPA Guideword, describing kind of control.

Figure 9.100 – Early

TooLong

Package: STPA Library

isAbstract: Yes

Generalization: UndesiredControlAction

Applied Stereotype: «UndesiredControlAction»

Description

STPA Guideword, describing kind of control.

Figure 9.101 – TooLong

TooShort

Package: STPA Library

isAbstract: Yes

Generalization: UndesiredControlAction

Applied Stereotype: «UndesiredControlAction»

Description

STPA Guideword, describing kind of control.

Figure 9.102 – TooShort

Provided

Package: STPA Library

isAbstract: Yes

Generalization: UndesiredControlAction

Applied Stereotype: «UndesiredControlAction»

Description

STPA Guideword, describing a kind of control.

Figure 9.103 – Provided

NotProvided

Package: STPA Library

isAbstract: Yes

Generalization: UndesiredControlAction

Applied Stereotype: «UndesiredControlAction»

Description

STPA Guideword, describing kind of control.

Figure 9.104 – NotProvided

LossScenario

Package: STPA Library

isAbstract: Yes

Generalization: Scenario

Applied Stereotype: «LossScenario

Description

A sequence of situations starting from causalFactors, that (through process deficiencies) leads to an UndesiredControlAction (which further leads to risks and possibly losses).

Figure 9.105 – LossScenario

Attributes

| causalFactor : Factor[0..*] (member end of association, subsets scenarioStep) | A causalFactor can be used to further refine process inadequacies - specifying causes of deficiencies in the process and/or other contributing factors. |
| --- | --- |
| undesiredControlAction : UndesiredControlAction[0..*] (member end of association, subsets scenarioStep) | Undesired control action related to the loss scenario. |
| controlFlaw : ControlFlaw[0..*] (member end of association, subsets scenarioStep) | Control flaw related to the loss scenario. |

ControlFlaw

Package: STPA Library

isAbstract: Yes

Applied Stereotype: «Situation»

Description

A ControlFlaw describes a process / control loop model that may lead to an Undesired Control Action. The four high level kinds of process model deficiencies can be used to specify the section of the control loop.

Process model deficiencies are often called (high level) Scenario in STPA theory.

Figure 9.106 – ControlFlaw

Attributes

| undesiredControlAction : UndesiredControlAction[0..*] (member end of ControlFlawConsequence association, redefines to) | Undesired control action related to control flaw. |
| --- | --- |

InadequateControllerDecisions

Package: STPA Library

isAbstract: Yes

Generalization: ControlFlaw

Applied Stereotype: «Situation»

Description

A kind of ControlFlaw.

Figure 9.107 – InadequateControllerDecisions

InadequateControlExecution

Package: STPA Library

isAbstract: Yes

Generalization: ControlFlaw

Applied Stereotype: «Situation»

Description

A kind of ControlFlaw.

Figure 9.108 – InadequateControlExecution

InadequateProcessBehavior

Package: STPA Library

isAbstract: Yes

Generalization: ControlFlaw

Applied Stereotype: «Situation»

Description

A kind of ControlFlaw.

Figure 9.109 – InadequateProcessBehavior

InadequateFeedbackAndInputs

Package: STPA Library

isAbstract: Yes

Generalization: ControlFlaw

Applied Stereotype: «Situation»

Description

A kind of ControlFlaw.

Figure 9.110 – InadequateFeedbackAndInputs

UndesiredControlAction

Package: STPA Library

isAbstract: Yes

Generalization: UndesiredState

Applied Stereotype: «UndesiredControlAction»

Description

An Undesired Control Action (UCA), used in STPA, describes in what context providing / not providing a Control Action might lead to an undesired result.

A UCA generally consist of four parts:

- Controller (Subject) that issues the Control Action - inferred from Control Action and model of the system (block/part producing the control action).

- Guideword (provides, does not provide, etc.) - indicated using Generalization relationship

- Control Action - connected with RelevantTo relationship.

- Context in which Control Action leads to undesired outcome - sub situation of (part of) UCA situation.

Figure 9.111 – UndesiredControlAction

Attributes

| Context : AbstractOperationalSituation[1] (member end of association) | The context of the undesired control action. |
| --- | --- |
| controlFlaw : ControlFlaw[0..*] (member end of ControlFlawConsequence association, redefines from) | Control flaw related to the undesired control action. |
| harmPotential : HarmPotential[0..*] (member end of UndesiredControlActionHarmPotential association, redefines to) | Harm potential related to the undesired control action. |

HarmRealization

Package: STPA Library

isAbstract: No

Generalization: AbstractRisk, Causality

Applied Stereotype: «Block»

Description

Association between the Loss and Hazard (potential harm).

Figure 9.112 – HarmRealization

ControlFlawFactor

Package: STPA Library

Generalization: Causality

Description

Causal relationship between causal Factor and ControlFlaw

Figure 9.113 – ControlFlawFactor

Association ends

| controlFlaw : ControlFlaw[0..*] (member end of ControlFlawFactor association, redefines to) | Control flaw related to the control flaw factor. |
| --- | --- |
| causalFactor : Factor[0..*] (member end of ControlFlawFactor association, redefines from) | A causalFactor can be used to further refine process inadequacies - specifying causes of deficiencies in the process and/or other contributing factors. |

ControlFlawConsequence

Package: STPA Library

Generalization: Causality

Description

Causal relationship between ControlFlaw and UndesiredControlAction

Figure 9.114 – ControlFlawConsequence

Association ends

| undesiredControlAction : UndesiredControlAction[0..*] (member end of ControlFlawConsequence association, redefines to) | Undesired control action related to control flaw. |
| --- | --- |
| controlFlaw : ControlFlaw[0..*] (member end of ControlFlawConsequence association, redefines from) | Control flaw related to the undesired control action. |

UndesiredControlActionHarmPotential

Package: STPA Library

Generalization: Causality

Description

Causal relationship between UndesiredControlAction and HarmPortential.

Figure 9.115 – UndesiredControlActionHarmPotential

Association ends

| harmPotential : HarmPotential[0..*] (member end of UndesiredControlActionHarmPotential association, redefines to) | Harm potential related to the undesired control action. |
| --- | --- |
| undesiredControlAction : UndesiredControlAction[0..*] (member end of UndesiredControlActionHarmPotential association, redefines from) | Harm potential (or hazard, or threat) related to the undesired control action. |

Methods::STPA::STPA Profile

ControlAction

Package: STPA Profile

isAbstract: No

Extension: Signal, Class, DataType

Description

A Control Action (CA) is an output signal from a functional / logical Controller to a ControlledProcess (via the Actuator), that determines the receiving process behaviour.

Figure 9.116 – ControlAction

Feedback

Package: STPA Profile

isAbstract: No

Extension: Signal, Class, DataType

Description

A Feedback is an input signal to a functional / logical Controller from a ControlledProcess (via the Sensor), that characterizes the current processes behavior (or the environment).

Figure 9.117 – Feedback

UndesiredControlAction

Package: STPA Profile

isAbstract: No

Generalization: Situation

Extension: Class

Description

Stereotype used to demarcate all the UndesiredControlActions.

Figure 9.118 – UndesiredControlAction

UnsafeControlAction

Package: STPA Profile

isAbstract: No

Generalization: UndesiredControlAction

Extension: Class

Description

Stereotype used to demarcate all the UnsafeControlActions.

Figure 9.119 – UnsafeControlAction

ControlledProcess

Package: STPA Profile

isAbstract: No

Extension: Property, Class

Description

An abstract representation of the system and its behaviours that need to be supervised and governed.

Controller is controlling this process through the ControlAction via the Actuator.

Figure 9.120 – ControlledProcess

Actuator

Package: STPA Profile

isAbstract: No

Extension: Property, Class

Description

Actuator receives ControlActions from Controller and influences the ControlledProcess in some way.

Figure 9.121 – Actuator

Sensor

Package: STPA Profile

isAbstract: No

Extension: Property, Class

Description

Sensor assesses the ControlledProcess (also environment or other controllers) and gives Feedback to the Controller.

Figure 9.122 – Sensor

Controller

Package: STPA Profile

isAbstract: No

Extension: Property, Class

Description

Controller sends the ControlActions and receives Feedback.

Figure 9.123 – Controller

ControlStructure

Package: STPA Profile

isAbstract: No

Generalization: Block

Extension: Class

Description

ControlStructure is a system-of-systems composed of ControlledProcess, Controller and their functional relationships - ControllActions, Feedbacks, describing feedback control loops.

Figure 9.124 – ControlStructure

LossScenario

Package: STPA Profile

isAbstract: No

Generalization: Situation

Extension: Class

Description

Stereotype used to demarcate all the LossScenarios.

Figure 9.125 – LossScenario

GSN

The GSN profile is an implementation of the core notation described in the GSN version 2 standard. The GSN standard is made available under creative commons licence version 4:

“To view a copy of this license, visit http://creativecommons.org/licenses/by/4.0/ or send a letter to Creative Commons, PO Box 1866, Mountain View, CA 94042, USA.”.

The OMG acknowledges the work of the SCSC ACWG in the production of the GSN standard.

Whilst GSN is an extension of the OMG SACM standard, which has a defined meta-model based on the OMG MOF standard, the objectives of RAAML to integrate with SysML 1.6 necessitate the use of a UML profile interpretation of the GSN standard.

GSN::GSN Profile

Notation

Most of the stereotypes in GSN profile have stereotype images specified. Displaying the stereotyped GSN elements in UML Class diagram may follow the UML standard prescription (UML 2.5.1, Chapter 12.3.4.1 Icon presentation) for displaying elements having stereotypes with icons, namely:

- Showing model element as an image with element name below

- Showing model element as a box with the iconic form image inside the box at the top left

Figure 9.126 – Standard UML notation for stereotyped elements (from UML 2.5.1, Figure 12.25)

However, in addition to the notation described in UML standard, this standard allows additional notation. Namely – using stereotype image as a (resizable) outline/shape of the box, with the same compartments that are prescribed by the UML standard (including name/stereotype/tag values compartment) inside. This notation is recommended i.e., preferred over the standard UML notation.

An example of the SCSC/GSN standard representation of the GSN extension is shown in Figure 9.127. See the SCSC/GSN standard for the shapes and text placement to be used for various model element types.

S1

Strategy

Figure 9.127 - Strategy notation

Combined Stereotype Notation

The UML standard allows a combination of several stereotypes applied on the model element. Namely – the combination of Goal+Undeveloped stereotypes and Strategy+Undeveloped stereotypes is being used. An example of this notation is depicted in Figure 9.128. See the SCSC/GSN standard for the shapes and text placement to be used for various model element types.

Undeveloped

Goal

Figure 9.128 - Combined notation

GSNNode

Package: GSN Profile

isAbstract: Yes

Extension: Element

Description

Root type for all the different kinds of nodes in GSN.

Note: name versus human-readable ID

GSN domain elements frequently have both a short phrase, describing the element and human-readable identifier. For example:

G1 Control System is acceptably safe to operate

In this example “Control System is acceptably safe to operate” is a short phrase, describing the goal, while G1 is a human-readable identifier of the goal.

In this standard, the short phrase shall be captured as UML model element name – NamedElement::name field. Human-readable identifier shall be stored in a separate tag, defined in the Core profile – IDCarrier::id..

Figure 9.129 – GSNNode

Attributes

| id : String[0..1] |  |
| --- | --- |

GSNArgumentNode

Package: GSN Profile

isAbstract: Yes

Generalization: GSNNode

Extension: Element

Description

A Goal or a Strategy.

Figure 9.130 – GSNArgumentNode

Solution

Package: GSN Profile

isAbstract: No

Generalization: GSNNode

Extension: Class

Description

A solution presents a reference to an evidence item or items.

Figure 9.131 – Solution

Goal

Package: GSN Profile

isAbstract: No

Generalization: GSNArgumentNode

Extension: Class

Description

A goal presents a claim forming part of the argument.

Figure 9.132 – Goal

Strategy

Package: GSN Profile

isAbstract: No

Generalization: GSNArgumentNode

Extension: Class

Description

A strategy describes the nature of the inference that exists between a goal and its supporting goal(s).

Figure 9.133 – Strategy

ContextualInformation

Package: GSN Profile

isAbstract: Yes

Extension: Element

Description

A Context or an Assumption or a Justification.

Figure 9.134 – ContextualInformation

Attributes

| id : String[0..1] |  |
| --- | --- |

Context

Package: GSN Profile

isAbstract: No

Generalization: ContextualInformation

Extension: Class

Description

A context presents a contextual artefact. This can be a reference to contextual information, or a statement.

Figure 9.135 – ContextStatement

Assumption

Package: GSN Profile

isAbstract: No

Generalization: SupportingInformation

Extension: Class

Description

An assumption presents an intentionally unsubstantiated statement.

Figure 9.136 – Assumption

Justification

Package: GSN Profile

isAbstract: No

Generalization: ContextualInformation

Extension: Class

Description

A justification presents a statement of rationale.

Figure 9.137 – Justification

InContextOf

Package: GSN Profile

isAbstract: No

Extension: Dependency

Description

InContextOf declares a contextual relationship.

Permitted connections are: goal-to-context, goal-to-assumption, goal-to-justification, strategy-to-context,

strategy-to-assumption and strategy-to-justification.

Figure 9.138 – InContextOf

Constraints

| [1] ClientIsArgumentNode | -- client of InContextOf must be GSNArgumentNode GSNArgumentNode.allInstances().base_Element->includesAll(self.base_Dependency.client) |
| --- | --- |
| [2] SupplierIsNotGSNNode | -- supplier of InContextOf can be ContextualInformation or non-GSN concept, but it can not be GSNNode ContextualInformation.allInstances().base_Element->includesAll(self.base_Dependency.supplier) or not GSNNode.allInstances().base_Element->includesAll(self.base_Dependency.supplier) |

SupportedBy

Package: GSN Profile

isAbstract: No

Extension: Dependency

Description

SupportedBy allows inferential or evidential relationships to be documented. Inferential relationships declare that there is an inferencebetween goals in the argument. Evidential relationships declare the link between a goal and the evidence used to substantiate it. Permitted supported by connections are: goal-to-goal, goal-to-strategy, goal-to-solution, strategy to goal.

Figure 9.139 – SupportedBy

Constraints

| [1] ClientIsGSNArgumentNode | -- client of SupportedBy must be GSNArgumentNode GSNArgumentNode.allInstances().base_Element->includesAll(self.base_Dependency.client) |
| --- | --- |
| [2] StrategyToGoal | -- if client is Strategy then supplier must be Goal Strategy.allInstances().base_Class->includesAll(self.base_Dependency.client) implies Goal.allInstances().base_Class->includesAll(self.base_Dependency.supplier) |
| [3] SupplierIsNotContextualInformation | -- supplier of SupportedBy can be GSNNode or non-GSN concept, but it can not be ContextualInformation GSNNode.allInstances().base_Element->includesAll(self.base_Dependency.supplier) or not ContextualInformation.allInstances().base_Element->includesAll(self.base_Dependency.supplier) |
| [4] ClientIsNotUndeveloped | -- client can not be Undeveloped Strategy nor Goal -- if strategy or goal is client of SupportedBy - it is developed not Undeveloped.allInstances().base_Element->includesAll(self.base_Dependency.client) |

Methods::ISO 26262

The ISO 26262 package contains elements supporting the analysis and requirement specification aspects of Functional Safety, as specified by ISO 26262 standard for automotive applications. ISO 26262 is a risk-based standard derived from IEC 61508. The ISO 26262 package redefines or extends concepts from the Core concepts package and the General Concepts package.

The ISO 26262 package enables modeling a HAZOP, which is typically used to identify malfunctioning behaviors. The failure modes concept is used from the General Concepts and specialized as a malfunctioning behavior. This allows the malfunctioning behavior to be related to the system behaviors through the HAZOP guidewords for construction of the HAZOP table. The risk analysis is performed by identifying Hazards that could result from the MalfunctioningBehavior, which in combination with a particular OperationalSituation could result in an AccidentScenario. This information is contained in the HazardousEvent which provides the risk level assessment for the event. Each of these concepts are modeled using elements defined in the ISO 26262 package as extensions of the Core and General concepts. This means that the same elements can be used in other analyses in the model, such as in an FMEA.

Methods::ISO 26262::ISO 26262 Library

TrafficAndPeople

Package: ISO 26262 Library

isAbstract: Yes

Generalization: OperationalCondition

Applied Stereotype: «OperationalSituation»

Description

TrafficAndPeople extends the <<situation>> class and is used to describe the presence and behaviour of any motorists or non-motorists considered in a hazardous event.

Figure 9.140 – TrafficAndPeople

VehicleUsage

Package: ISO 26262 Library

isAbstract: Yes

Generalization: OperationalCondition

Applied Stereotype: «OperationalSituation»

Description

VehicleUsage extends the <<situation>> class and is used to describe the usage of a vehicle during a hazardous event.

Figure 9.141 – VehicleUsage

RoadCondition

Package: ISO 26262 Library

isAbstract: Yes

Generalization: OperationalCondition

Applied Stereotype: «OperationalSituation»

Description

RoadConditions extends the <<situation>> class, and is used to describe the conditions or state of the surface a vehicle is driving on (Low-traction, Grade(Slope), etc.) during a hazardous event.

Figure 9.142 – RoadCondition

Location

Package: ISO 26262 Library

isAbstract: Yes

Generalization: OperationalCondition

Applied Stereotype: «OperationalSituation»

Description

VehicleLocation extends the <<situation>> class and is used to describe the physical location (high speed road, intersection, parking lot, etc.) of a vehicle during a hazardous event.

Figure 9.143 – Location

EnvironmentalCondition

Package: ISO 26262 Library

isAbstract: Yes

Generalization: OperationalCondition

Applied Stereotype: «OperationalSituation»

Description

EnvironmentalConditions extends the <<situation>> class and is used to describe the environmental conditions at the time of vehicle operation in a hazardous event.

Figure 9.144 – EnvironmentalCondition

OperationalCondition

Package: ISO 26262 Library

isAbstract: Yes

Generalization: AbstractEvent

Applied Stereotype: «OperationalSituation»

Description

Component/part of operational situation.

Figure 9.145 – OperationalCondition

AbstractOperationalSituation

Package: ISO 26262 Library

isAbstract: Yes

Generalization: OperationalCondition

Applied Stereotype: «OperationalSituation»

Description

Operational situation is a scenario that can occur in vehicle's life.

Figure 9.146 – AbstractOperationalSituation

Attributes

| conditions : OperationalCondition[*] (member end of association) |  |
| --- | --- |
| Exposure : Exposure[1] , redefines likelihood | Likelihood of being in a particular operational situation. Must have a Rationale attached. |

TypicalAutomotiveSituation

Package: ISO 26262 Library

isAbstract: Yes

Generalization: AbstractOperationalSituation

Applied Stereotype: «OperationalSituation»

Description

A grouping of operational conditions, including traffic and people, vehicle usage, road conditions, location, and environmental conditions.

Figure 9.147 – TypicalAutomotiveSituation

Attributes

| trafficAndPeople : TrafficAndPeople[0..*] (member end of association, subsets conditions) |  |
| --- | --- |
| vehicleUsage : VehicleUsage[0..*] (member end of association, subsets conditions) |  |
| roadCondition : RoadCondition[0..*] (member end of association, subsets conditions) |  |
| location : Location[0..*] (member end of association, subsets conditions) |  |
| environmentalCondition : EnvironmentalCondition[0..*] (member end of association, subsets conditions) |  |

Exposure

Package: ISO 26262 Library

isAbstract: No

Applied Stereotype: «ValueType»

Description

Possible values of exposure.

Figure 9.148 – Exposure

Severity

Package: ISO 26262 Library

isAbstract: No

Applied Stereotype: «ValueType»

Description

Possible values for severity.

Figure 9.149 – Severity

ASIL

Package: ISO 26262 Library

isAbstract: No

Applied Stereotype: «ValueType»

Description

Possible ASIL values.

Figure 9.150 – ASIL

Controllability

Package: ISO 26262 Library

isAbstract: No

Applied Stereotype: «ValueType»

Description

Possible values of controllability.

Figure 9.151 – Controllability

Less

Package: ISO 26262 Library

isAbstract: Yes

Generalization: AnyMalfunction

Applied Stereotype: «MalfunctioningBehavior»

Description

A subclass of malfunctioning behaviour used for classification purposes. Must be connected to a behavioural element (Use Case or Function). This kind of malfunctioning behaviour represents a failure resulting from providing less output/behaviour than required.

Figure 9.152 – Less

More

Package: ISO 26262 Library

isAbstract: Yes

Generalization: AnyMalfunction

Applied Stereotype: «MalfunctioningBehavior»

Description

A subclass of malfunctioning behaviour used for classification purposes. Must be connected to a behavioural element (Use Case or Function). This kind of malfunctioning behaviour represents a failure resulting from providing more output/behaviour than required.

Figure 9.153 – More

No

Package: ISO 26262 Library

isAbstract: Yes

Generalization: AnyMalfunction

Applied Stereotype: «MalfunctioningBehavior»

Description

A subclass of malfunctioning behaviour used for classification purposes. Must be connected to a behavioural element (Use Case or Function). This kind of malfunctioning behaviour represents a failure resulting from the behaviour not being performed when required.

Figure 9.154 – No

Intermittent

Package: ISO 26262 Library

isAbstract: Yes

Generalization: AnyMalfunction

Applied Stereotype: «MalfunctioningBehavior»

Description

A subclass of malfunctioning behaviour used for classification purposes. Must be connected to a behavioural element (Use Case or Function). This kind of malfunctioning behaviour represents a failure from the behaviour being performed intermittently.

Figure 9.155 – Intermittent

Unintended

Package: ISO 26262 Library

isAbstract: Yes

Generalization: AnyMalfunction

Applied Stereotype: «MalfunctioningBehavior»

Description

A subclass of malfunctioning behaviour used for classification purposes. Must be connected to a behavioural element (Use Case or Function). This kind of malfunctioning behaviour represents a failure resulting from the behaviour being provided when not required.

Figure 9.156 – Unintended

Early

Package: ISO 26262 Library

isAbstract: Yes

Generalization: AnyMalfunction

Applied Stereotype: «MalfunctioningBehavior»

Description

A subclass of malfunctioning behaviour used for classification purposes. Must be connected to a behavioural element (Use Case or Function). This kind of malfunctioning behaviour represents a failure resulting from the behaviour being performed earlier than required.

Figure 9.157 – Early

Late

Package: ISO 26262 Library

isAbstract: Yes

Generalization: AnyMalfunction

Applied Stereotype: «MalfunctioningBehavior»

Description

A subclass of malfunctioning behaviour used for classification purposes. Must be connected to a behavioural element (Use Case or Function). This kind of malfunctioning behaviour represents a failure resulting from the behaviour being performed later than required.

Figure 9.158 – Late

Inverted

Package: ISO 26262 Library

isAbstract: Yes

Generalization: AnyMalfunction

Applied Stereotype: «MalfunctioningBehavior»

Description

A subclass of malfunctioning behaviour used for classification purposes. Must be connected to a behavioural element (Use Case or Function). This kind of malfunctioning behaviour represents a failure resulting from the behaviour providing an inverted output.

Figure 9.159 – Inverted

HazardousEvent

Package: ISO 26262 Library

isAbstract: Yes

Generalization: AbstractRisk

Applied Stereotype: «Situation»

Description

Combination of hazard and operational situation to identify automotive safety integrity level.

Figure 9.160 – HazardousEvent

Attributes

| scenario : AccidentScenario[1] (member end of association, redefines trigger) |  |
| --- | --- |
| hazard : Hazard[0..*] (member end of association, redefines harmPotential) |  |
| systemLevelEffect : SystemLevelEffect[0..*] (member end of association) |  |
| vehicleLevelEffect : VehicleLevelEffect[0..*] (member end of association, redefines harm) |  |
| ASIL : ASIL[0..*], redefines score | Automotive Safety Integrity Level value - one of four levels to specify necessary requirements for ISO-26262 and safety measures for avoiding unreasonable risks. |

AnyMalfunction

Package: ISO 26262 Library

isAbstract: Yes

Generalization: UndesiredState

Applied Stereotype: «MalfunctioningBehavior»

Description

Root of all malfunctioning behaviours.

Figure 9.161 – AnyMalfunction

AutomotiveEffect

Package: ISO 26262 Library

isAbstract: Yes

Generalization: AbstractEffect

Applied Stereotype: «Situation»

Description

System- or vehicle-level effect which is or could result in harm.

Figure 9.162 – AutomotiveEffect

Attributes

| Severity : Severity[1], redefines severity | Estimate of the extent of harm. Must have a Rationale attached. |
| --- | --- |

ISO26262SafetyRequirementTemplate

Package: ISO 26262 Library

isAbstract: No

Applied Stereotype: «DependabilityRequirement»

Description

A template for dependability requirements.

Figure 9.163 – ISO26262SafetyRequirementTemplate

Attributes

| ASIL : ASIL[1] | ASIL value of the requirement. |
| --- | --- |
| FTTI : time[1] | Fault Tolerant Time Interval. |

AccidentScenario

Package: ISO 26262 Library

isAbstract: Yes

Generalization: DysfunctionalEvent, Scenario

Applied Stereotype: «Situation»

Description

A combination of operational situation and malfunctioning behaviour.

Figure 9.164 – AccidentScenario

Attributes

| situation : AbstractOperationalSituation[1..*] (member end of association, subsets scenarioStep) |  |
| --- | --- |
| Controllability : Controllability[1] | Ability to avoid a specified harm or damage through timely reactions of individuals involved in the scenario. Must have a Rationale attached. |
| malfunctioningBehavior : AnyMalfunction[1] (member end of association, subsets scenarioStep) |  |

AnyTrafficAndPeople

Package: ISO 26262 Library

isAbstract: No

Generalization: OperationalCondition, TrafficAndPeople

Applied Stereotype: «OperationalSituation»

Description

TrafficAndPeople extends the <<situation>> class and is used to describe the presence and behaviour of any motorists or non-motorists considered in a hazardous event.

Figure 9.165 – AnyTrafficAndPeople

AnyVehicleUse

Package: ISO 26262 Library

isAbstract: No

Generalization: OperationalCondition, VehicleUsage

Applied Stereotype: «OperationalSituation»

Description

TrafficAndPeople extends the <<situation>> class and is used to describe the presence and behaviour of any motorists or non-motorists considered in a hazardous event.

Figure 9.166 – AnyVehicleUse

AnyRoadCondition

Package: ISO 26262 Library

isAbstract: No

Generalization: OperationalCondition, RoadCondition

Applied Stereotype: «OperationalSituation»

Description

TrafficAndPeople extends the <<situation>> class and is used to describe the presence and behaviour of any motorists or non-motorists considered in a hazardous event.

Figure 9.167 – AnyRoadCondition

AnyLocation

Package: ISO 26262 Library

isAbstract: No

Generalization: Location, OperationalCondition

Applied Stereotype: «OperationalSituation»

Description

TrafficAndPeople extends the <<situation>> class and is used to describe the presence and behavior of any motorists or non-motorists considered in a hazardous event.

Figure 9.168 – AnyLocation

AnyEnvironmentalCondition

Package: ISO 26262 Library

isAbstract: No

Generalization: EnvironmentalCondition, OperationalCondition

Applied Stereotype: «OperationalSituation»

Description

TrafficAndPeople extends the <<situation>> class and is used to describe the presence and behaviour of any motorists or non-motorists considered in a hazardous event.

Figure 9.169 – AnyEnvironmentalCondition

SystemLevelEffect

Package: ISO 26262 Library

isAbstract: Yes

Generalization: AutomotiveEffect

Applied Stereotype: «Situation»

Description

System- or vehicle-level effect which is or could result in harm.

Figure 9.170 – SystemLevelEffect

VehicleLevelEffect

Package: ISO 26262 Library

isAbstract: Yes

Generalization: AutomotiveEffect

Applied Stereotype: «Situation»

Description

System- or vehicle-level effect which is or could result in harm.

Figure 9.171 – VehicleLevelEffect

Methods::ISO 26262::ISO 26262 Library::Diagrams by elements

Methods::ISO 26262::ISO 26262 Profile

OperationalSituation

Package: ISO 26262 Profile

isAbstract: No

Generalization: Situation

Extension: Class

Description

A situation describes the operational scenario or driving scenario which is considered in a hazardous event, as part of the Hazard Analysis and Risk Assessment process.

Figure 9.172 – OperationalSituation

MalfunctioningBehavior

Package: ISO 26262 Profile

isAbstract: No

Generalization: FailureMode

Extension: Class

Description

A malfunctioning behaviour describes a failure or unintended behaviour of an item with respect to its design intent. It is a subtype of failure mode.

Figure 9.173 – MalfunctioningBehavior

Methods::ISO 26262::ISO 26262 Profile::RequirementManagement

IndependenceRequirement

Package: RequirementManagement

isAbstract: No

Generalization: DeriveReqt

Extension: Abstraction

Description

A relationship between requirement elements indicating that the child requirement specifies an independence criterion that needs to be satisfied in order for an ASIL decomposition to be valid.  The decomposition between the parent requirement and 2 other children requirements.

Figure 9.174 – IndependenceRequirement

ASILDecompose

Package: RequirementManagement

isAbstract: No

Generalization: DeriveReqt

Extension: Abstraction

Description

An ASIL decompose relation is used to connect two safety requirements for the purposes of performing ASIL decomposition.  The target requirement (supplier) should be of a higher abstraction than the source (client).  ASIL decompose relations shall be applied in pairs (e.g., a requirement cannot be the supplier of a single ASIL decompose relation).

Figure 9.175 – ASILDecompose

SafeState

Package: RequirementManagement

isAbstract: No

Extension: Dependency

Description

A state of function realized by one or more architectural components.  May be composed of serval subfunctions or called by other functions.  Associated with safety specific behaviours, typically (but not necessarily) triggered by a failure mode.

Figure 9.176 – SafeState

UserInfoRequirement

Package: RequirementManagement

isAbstract: No

Generalization: Satisfy

Extension: Abstraction

Description

A UserInfoRequirement relationship is a dependency which links a State to a requirement.  The arrow direction points from a state (client) to a FSR or TSR (supplier).  Linked requirements specify information that must be presented to vehicle occupants when the vehicle enters a safe state.

Figure 9.177 – UserInfoRequirement

RecoveryRequirement

Package: RequirementManagement

isAbstract: No

Generalization: Satisfy

Extension: Abstraction

Description

A RecoveryRequirement relationship is a dependency between a safe state and requirement where the requirement indicates the criteria to recover from the safe state to another operational mode.

Figure 9.178 – RecoveryRequirement

OperatingMode

Package: RequirementManagement

isAbstract: No

Extension: Dependency

Description

A state of function realized by one or more architectural components.  May be composed of serval subfunctions or called by other functions.  Associated with specific behaviours.

Figure 9.179 – OperatingMode

FunctionalSafetyRequirement

Package: RequirementManagement

isAbstract: No

Generalization: DependabilityRequirement, Requirement

Extension: Class

Description

A functional safety requirement specifies an implementation independent safety behaviour, or an implementation independent safety measure, required for achievement of a safety goal from which it is derived.

Figure 9.180 – FunctionalSafetyRequirement

SoftwareSafetyRequirement

Package: RequirementManagement

isAbstract: No

Generalization: DependabilityRequirement, Requirement

Extension: Class

Description

A software safety requirement provides implementation details for software.  They can express behaviours or specific software mechanisms which realize the technical safety requirements from which they are derived.

Figure 9.181 – SoftwareSafetyRequirement

HardwareSafetyRequirement

Package: RequirementManagement

isAbstract: No

Generalization: DependabilityRequirement, Requirement

Extension: Class

Description

A hardware safety requirement specifies hardware behaviours or hardware specific details necessary for implementing the safety concept.  Hardware safety requirements are implementation specific and assigned to components or subcomponents.

Figure 9.182 – HardwareSafetyRequirement

TechnicalSafetyRequirement

Package: RequirementManagement

isAbstract: No

Generalization: DependabilityRequirement, Requirement

Extension: Class

Description

A technical safety requirement specifies the implementation of the functional safety requirement(s) from which it is derived.  Technical safety requirements express the behaviours and details necessary to realize the safety aspects of the item at the system level.  Additional details that do not act at the system level can be specified in the hardware safety requirements or software safety requirements.

Figure 9.183 – TechnicalSafetyRequirement

SafetyGoal

Package: RequirementManagement

isAbstract: No

Generalization: DependabilityRequirement, Requirement

Extension: Class

Description

A safety goal extends the SysML <<Requirement>> stereotype.  It represents a top-level safety requirement, defined as a result of the Hazard Analysis and Risk Assessment process.

Figure 9.184 – SafetyGoal

DependabilityRequirement

Package: RequirementManagement

isAbstract: Yes

Generalization: AbstractRequirement, Block

Extension: Class

Description

Parent type of all subtypes of safety requirements

Figure 9.185 – DependabilityRequirement

Verified

Package: ISO 26262 Profile

isAbstract: No

Extension: Class

Description

Marker, indicating that hazardous event has been verified.

Figure 9.186 – Verified

Attributes

| result : String[1] | Verification result |
| --- | --- |

Confirmed

Package: ISO 26262 Profile

isAbstract: No

Extension: Class

Description

Marker, indicating that hazardous event has been confirmed.

Figure 9.187 – Confirmed

Attributes

| result : String[1] | Confirmation result |
| --- | --- |

HazardAndRiskAssessment

Package: ISO 26262 Profile

isAbstract: No

Extension: Package

Description

Grouping package for storing hazardous events.

Figure 9.188 – HazardAndRiskAssessment

LessonLearned

Package: ISO 26262 Profile

isAbstract: No

Extension: Comment

Description

Comments about lessons learned from hazard and risk assessment.

Figure 9.189 – LessonLearned

ASILAssignment

Package: ISO 26262 Profile

isAbstract: No

Extension: Element

Description

Stereotype for assigning ASIL values on system design elements.

Figure 9.190 – ASILAssignment

Attributes

| ASIL : ASIL[1] | The associated ASIL value of the system design element. |
| --- | --- |
| ASILOverride : ASIL[0..1] | An ASIL value which does not follow from the normal ASIL derivation rules but is exceptional. This exceptional value needs to have an associated rationale. |

ASILOverrideRationale

Package: ISO 26262 Profile

isAbstract: No

Generalization: Rationale

Extension: Comment

Description

A rationale specifically justifying ASIL Override value.

Figure 9.191 – ASILOverrideRationale

  - Methods::FHA

The Functional Hazard Assessment (FHA), as described in the ARP 4761A, is a systematic, comprehensive evaluation of the functional decomposition of a system or platform, performed at early stages of the V development process. This assessment identifies and classifies the failure conditions associated with each function. The goal in conducting an FHA is to clearly identify each failure condition along with the rationale for its severity classification.  The classification of these failure conditions establishes the safety objectives to be met.

An FHA should also identify assumptions associated with the system/platform of interest and crew mitigations to limit the severity classification, e.g., operating limits, crew information presentation, operating procedures. The FHAs are then the starting point for the subsequent safety assessments and analyses.

The RAAML FHA package contains all required elements to implement this assessment at different levels (e.g., platform, system). Support for Functional Hazard Assessment (FHA) modeling is based on the ARP 4761A standard. The FHA package redefines or extends concepts from the Core and General packages. It is also tightly related to ISO 26262 and FMEA library methods within the RAAML standard.

Methods::FHA::FHA Library

#### ExternalEvent

Package: FHA Library

isAbstract: Yes

Generalization: AbstractEvent

Applied Stereotype: «Situation»

Description

An occurrence which has its origin distinct from the aircraft or the system being examined, such as atmospheric conditions (e.g., wind gusts/shear, temperature variations, icing, lightning strikes), operating environment (e.g., runway conditions, conditions of communication, navigation, and surveillance services), cabin and baggage fires, and bird-strike. The term is not intended to cover sabotage. [Source: ARP 4761A]

Figure 9.192 - ExternalEvent

Constraints

| [1] FHA |  |
| --- | --- |

#### DevelopmentAssuranceLevel

Package: FHA Library

isAbstract: No

Applied Stereotype: «ValueType»

Description

All those planned and systematic actions used to substantiate, at an adequate level of confidence, that development errors have been identified and corrected such that the system satisfies the applicable certification basis (derived from AC 25.1309 Draft ARSENAL revised and AMC 25.1309). [Source ARP 4761A]

Figure 9.193 - DevelopmentAssuranceLevel

Constraints

| [1] FHA |  |
| --- | --- |

#### OperationalCondition

Package: FHA Library

isAbstract: Yes

Generalization: AbstractEvent

Applied Stereotype: «Situation»

Description

Specific state or circumstances under which the aircraft is operated within the approved aircraft operating envelope, relevant when evaluating the effects of a failure condition. Example of operational conditions include aircraft weight, center of gravity, speed...

Figure 9.194 - OperationalCondition

Constraints

| [1] FHA |  |
| --- | --- |

#### DAL

Package: FHA Library

isAbstract: No

Description

All those planned and systematic actions used to substantiate, at an adequate level of confidence, that development errors have been identified and corrected such that the system satisfies the applicable certification basis (derived from AC 25.1309 Draft ARSENAL revised and AMC 25.1309). [Source: ARP 4761A]

Attributes

| dal : DevelopmentAssuranceLevel | The level of rigor of development assurance tasks. |
| --- | --- |

Constraints

| [1] FHA |  |
| --- | --- |

#### FunctionalFailure

Package: FHA Library

isAbstract: Yes

Generalization: UndesiredState

Applied Stereotype: «FunctionalFailure»

Description

An occurrence which affects the operation of an aircraft, system, equipment, item, or piece-part such that it can no longer function as intended (this includes both loss of function and malfunction). Failure conditions can be broadly categorized as the loss of a function or as a malfunction. Note: Errors may cause Failures but are not considered to be Failures. [Source: ARP 4761A]

Function: Intended behavior of an aircraft, system, equipment, or item regardless of implementation. [Source: ARP 4761A]

Figure 9.195 - FunctionalFailure

Attributes

| failureConditions : FailureCondition[1..*] (member end of association, redefines harmPotential) | A condition having an effect on the aircraft and/or its occupants, either direct or consequential, which is caused or contributed to by one or more failures or errors, considering flight phase and relevant adverse operational or environmental conditions, or external events (AMC 25.1309). [Source: ARP 4761A] |
| --- | --- |

Constraints

| [1] FHA |  |
| --- | --- |

#### EnvironmentalCondition

Package: FHA Library

isAbstract: Yes

Generalization: AbstractEvent

Applied Stereotype: «Situation»

Description

Atmospheric/physical factors within the approved aircraft operating envelope, relevant when evaluating the effects of a failure condition. Example of environmental conditions include weather, HIRF, volcanic ash...

Figure 9.197 - EnvironmentalCondition

Constraints

| [1] FHA |  |
| --- | --- |

#### FDAL

Package: FHA Library

isAbstract: No

Generalization: DAL

Description

The level of rigor of development assurance tasks performed to Functions. Note: The FDAL is used to identify the ARP4754B/ED-79B objectives that need to be satisfied for the aircraft/system functions. [Source: ARP 4761A]

Figure 9.198 - FDAL

Constraints

| [1] FHA |  |
| --- | --- |

#### FailureEffectAssessment

Package: FHA Library

isAbstract: No

Generalization: AbstractEffect

Description

The effects of a failure condition for any particular flight phase are all the expected effects during the flight when the failure condition occurs in that flight phase. This includes immediate effects and effects that would occur during subsequent flight phases due to the same failure condition. The effects are described by a narrative and characterized with brief statements regarding the effect in aircraft, crew and occupant categories. [Source: ARP 4761A]

Figure 9.199 - FailureEffectAssessment

Attributes

| effectOnAircraft : AbstractEffect[0..*] (member end of association) | Ability of the aircraft to perform its functions and to the aircraft’s structural integrity. Aircraft effects are characterized by a statement evaluating the reduction in aircraft capability and safety margin caused by the failure condition. [Source: ARP 4761A] |
| --- | --- |
| effectOnFlightCrew : AbstractEffect[0..*] (member end of association) | Any direct physiological effect on flight deck crew members or any increase in physical or mental activity required beyond the normal flight crew activity levels to counteract the effects of the failure and complete the flight safely. Typically, this includes the crew’s means of detecting the failure condition (if any) and their expected actions. Effects on crew are characterized by a statement evaluating the adverse physiological effects on the crew (if any) and the increase in crew workload caused by the failure condition (if any). [Source: ARP461A] |
| effectOnOccupants : AbstractEffect[0..*] (member end of association) | Cabin crew or passenger discomfort, injury or fatalities. These effects may be the direct result of the failure condition, such as high G maneuvering or abnormal environmental conditions, or an indirect effect, such as loss of life due to loss of aircraft control resulting in an uncontrolled crash. Occupant effects are characterized by a statement evaluating the adverse physiological effects on the cabin crew or passengers caused by the failure condition. [Source: ARP461A] |
| aggregatedSeverity : ARP 4761A FailureConditionClassification, redefines severity | A discrete scale allowing categorization of the severity of the effects of a failure condition. Classification levels are defined in the applicable regulation and advisory material. For example, AC 25.1309 Draft ARSENAL revised and AMC 25.1309 define the following classifications; Catastrophic, Hazardous, Major, Minor, and No Safety Effect. [Source: ARP 4761A] |
| flightPhases : FlightPhase[0..*] (member end of association) | Distinct time period within an average flight duration [Source: ARP4761A] |
| assumptions : Assumption[0..*] (member end of association) | Statements, principles and/or premises offered without proof. An evaluation process which may include one or more types of analysis and experience. [Source: ARP 4761A] |
| severity : ARP 4761A FailureConditionClassification[0..1] | A discrete scale allowing categorization of the severity of the effects of a failure condition. Classification levels are defined in the applicable regulation and advisory material. For example, AC 25.1309 Draft ARSENAL revised and AMC 25.1309 define the following classifications; Catastrophic, Hazardous, Major, Minor, and No Safety Effect. [Source: ARP 4761A] |

Constraints

| [1] FHA |  |
| --- | --- |

#### FlightPhase

Package: FHA Library

isAbstract: Yes

Generalization: AbstractEvent

Applied Stereotype: «Situation»

Description

Distinct time period within an average flight duration [Source: ARP4761A]

Figure 9.200 - FlightPhase

Constraints

| [1] FHA |  |
| --- | --- |

#### FailureCondition

Package: FHA Library

isAbstract: Yes

Generalization: AbstractRisk, Hazard

Description

A condition having an effect on the aircraft and/or its occupants, either direct or consequential, which is caused or contributed to by one or more failures or errors, considering flight phase and relevant adverse operational or environmental conditions, or external events (AMC 25.1309). [Source: ARP 4761A]

Hazard: A condition resulting from failures, external events, errors, or a combination thereof where safety is potentially affected. [Source: ARP 4761A]

Figure 9.201 - FailureCondition

Attributes

| environmentalConditions : EnvironmentalCondition[0..*] (member end of association) | Atmospheric/physical factors within the approved aircraft operating envelope, relevant when evaluating the effects of a failure condition. Example of environmental conditions include weather, HIRF, volcanic ash... |
| --- | --- |
| operationalConditions : OperationalCondition[0..*] (member end of association) | Specific state or circumstances under which the aircraft is operated within the approved aircraft operating envelope, relevant when evaluating the effects of a failure condition. Example of operational conditions include aircraft weight, center of gravity, speed... |
| externalEvents : ExternalEvent[0..*] (member end of association) | An occurrence which has its origin distinct from the aircraft or the system being examined, such as atmospheric conditions (e.g., wind gusts/shear, temperature variations, icing, lightning strikes), operating environment (e.g., runway conditions, conditions of communication, navigation, and surveillance services), cabin and baggage fires, and bird-strike. The term is not intended to cover sabotage. [Source: ARP 4761A] |
| operationalEvents : OperationalEvent[0..*] (member end of association) | Distinct occurrences and flight operations performed in response to specific occurrences or failures. These events may be considered operational events if they meet certain criteria, such as occurring at a distinct time, having a known statistical probability or rate of occurrence, and being infrequent or unlikely for a particular aircraft during its service life. Examples of operational events are: rejected take-off, or in-flight diversion. [Source: ARP4761A]. |
| crewAwareness : Boolean | Understanding and perception that flight crew members have regarding a failure condition. |
| functionalFailure : FunctionalFailure[1] (member end of association, redefines undesiredState) | An occurrence which affects the operation of an aircraft, system, equipment, item, or piece-part such that it can no longer function as intended (this includes both loss of function and malfunction). Failure conditions can be broadly categorized as the loss of a function or as a malfunction. Note: Errors may cause Failures but are not considered to be Failures. [Source: ARP 4761A] Function: Intended behavior of an aircraft, system, equipment, or item regardless of implementation. [Source: ARP 4761A |

Constraints

| [1] FHA |  |
| --- | --- |

#### IDAL

Package: FHA Library

isAbstract: No

Generalization: DAL

Description

The level of rigor of development assurance tasks performed on Item(s); e.g., IDAL is the appropriate software level in DO-178C/ED-12C, and design assurance level in DO-254/ED-80 objectives that need to be satisfied for an item. [Source: ARP 4761A]

Figure 9.202 - IDAL

Constraints

| [1] FHA |  |
| --- | --- |

#### OperationalEvent

Package: FHA Library

isAbstract: Yes

Generalization: AbstractEvent

Applied Stereotype: «Situation»

Description

Distinct occurrences and flight operations performed in response to specific occurrences or failures. These events may be considered operational events if they meet certain criteria, such as occurring at a distinct time, having a known statistical probability or rate of occurrence, and being infrequent or unlikely for a particular aircraft during its service life. Examples of operational events are: rejected take-off, or in-flight diversion. [Source: ARP4761A].

Figure 9.203 - OperationalEvent

Constraints

| [1] FHA |  |
| --- | --- |

#### PartialLossOfFunction

Package: FHA Library

isAbstract: No

Generalization: FunctionalFailure

Applied Stereotype: «FunctionalFailure»

Description

Partial losses of function are conditions where the function can still be performed but only at reduced capability (e.g., reduced effectiveness or with increased difficulty). [Source: ARP 4761A]

#### Malfunction

Package: FHA Library

isAbstract: No

Generalization: FunctionalFailure

Applied Stereotype: «FunctionalFailure»

Description

A condition where the operation of a function is different than intended, excluding the loss of function [Source: ARP 4761A]

#### LossOfFunction

Package: FHA Library

isAbstract: No

Generalization: FunctionalFailure

Applied Stereotype: «FunctionalFailure»

Description

Loss of function may be total or partial. Total loss of function is a condition where the function cannot be performed by any means. [Source: ARP 4761A]

#### ARP 4761A FailureConditionClassification

Package: FHA Library

isAbstract: No

Applied Stereotype: «ValueType»

Description

All those planned and systematic actions used to substantiate, at an adequate level of confidence, that development errors have been identified and corrected such that the system satisfies the applicable certification basis (derived from AC 25.1309 Draft ARSENAL revised and AMC 25.1309). [Source ARP 4761A]

Constraints

| [1] FHA |  |
| --- | --- |

Methods::FHA::FHA Profile

#### FunctionalFailure

Package: FHA Profile

isAbstract: No

Generalization: «Situation»

Extension: Class

Description

An occurrence which affects the operation of an aircraft, system, equipment, item, or piece-part such that it can no longer function as intended (this includes both loss of function and malfunction). Failure conditions can be broadly categorized as the loss of a function or as a malfunction. Note: Errors may cause Failures but are not considered to be Failures. [Source: ARP 4761A]

Function: Intended behavior of an aircraft, system, equipment, or item regardless of implementation. [Source: ARP 4761A]

Figure 9.204 - FunctionalFailure

#### FunctionalFailure Customization

Package: FHA Profile

isAbstract: No

Applied Stereotype: «Customization»

Description

Attributes

| Traceability : |  |
| --- | --- |
| Behavior : Element[0..*] |  |

  - Methods::RBD

A Reliability Block Diagram (RBD) is a graphical representation used in reliability engineering to analyze the reliability and availability of complex systems. It is a method for assessing the performance of systems composed of multiple components, sub-systems, or processes.  In an RBD, system elements (components or groups of components defined as  an assemblies, subsystems, or other system architectural elements) are depicted as blocks, and these blocks are connected by lines to represent how they are interconnected or dependent on each other. The goal is to evaluate the overall reliability and availability of the entire system by considering the reliability characteristics of each component and their interconnections.

The method is based on IEC 61078:2016 (“Reliability block diagrams”) includes the following concepts:

- Probability distributions, cumulative distribution functions (CDFs), probability density functions (PDFs), and hazard functions (the exponential, Weibull, and lognormal distributions are included in the library but the framework allows for additional distributions)

- Component reliability, failure probability, restoration completion probability, failure rate, restoration rate, mean time to failure, and mean time to restore

- Restorable and non-restorable systems

- System reliability and availability calculations for series, parallel, homogeneous k-out-n, and heterogeneous k-out-of-n systems

Mathematical methods for calculation of cumulative distributions functions (CDFs), probability density functions (PDFs), hazard functions, and mean values of distributions are based on U.S. National Institute of Standards Handbook of Engineering Statistics (see references).

    - Methods::RBD::RBD Library

#### AbstractReliabilitySituation

Package: RBD Library

isAbstract: Yes

Generalization: AnySituation

Applied Stereotype: «Situation»

Description

The parent situation of the RBD library.  Specialization of AnySituation from Core Library.

Figure 9.205 - AbstractReliabilitySituation

Attributes

| reliability : Real | Value property for reliability in AbstractReliabilityituation (used to hold result of system or component reliability in non-restorable and restorable systems) |
| --- | --- |
| failureProbability : Real | Value property that is the complement of reliability |
| pcf : ProbabilityComplementFunction | Constraint property for calculating the complement of a value between 0 and 1 |

#### Restorable

Package: RBD Library

isAbstract: Yes

Generalization: AbstractReliabilitySituation

Applied Stereotype: «Situation»

Description

Specialization of AbstractReliabilitySituation for restorable systems.  Parent of ResteorabailitySystemReliabiltiySituation and RestorableComponentReliabilitySituationSituation.  Also has composition relationship (owns) RestorableSystemReliabilitySituation to enable recursion

Figure 9.206 - Restorable

Attributes

| availability : Real | Availability value property of Restorable |
| --- | --- |

#### ComponentReliabilitySituation

Package: RBD Library

isAbstract: Yes

Generalization: AbstractReliabilitySituation

Applied Stereotype: «Situation»

Description

Situation for Components in RBDs.  Specialization of AbstractReliabiltiySituation and parent of RestorableComponentReliabilitiySituation, NonrestorableComponentReliabilitiySituations.  Owns Reliability ParameterGroup constraint and failurerate, shape, scale, and location distribution parameters that it provides its children

Figure 9.207 - ComponentReliabilitySituation

Attributes

| failureRPD : ReliabilityParameterDistribution | Constraint property for expression of the reliability probability distirbution |
| --- | --- |
| failureRate : Real | Value property for the failure rate |
| shape : Real | Value property for the shape parameter of the reliability propbability distribution |
| scale : Real | Value property for the scale parameter of the reliability propbability distribution |
| location : Real | Value property for the location parameter of the reliability propbability distribution |

#### RestorableComponentReliabilitySituation

Package: RBD Library

isAbstract: Yes

Generalization: ComponentReliabilitySituation, Restorable

Applied Stereotype: «Situation»

Description

Situation for restorable (repairable) component RBDs.  Specialization of ComponentReliabilitySituation. Owns restorationRPG (Reliability Parameter Group) and RatioOfPartTotal (for determining availability).  Value properties include MTBF, MTTR, restoration distribution parameters (shape, scale, and location),  restorationCompletionProbability (calculated from restoration CDF)  and restorationrate (calculated from restoration hazard function, i.e., ratio of PDF to CDF)

Figure 9.208 - RestorableComponentReliabilitySituation

Attributes

| MTBF : time | Value property for Mean Time Between Failures (MTBF is used for restorable systems) |
| --- | --- |
| MTTR : time | Value property for Mean Time to Restoration |
| restorationRPD : ReliabilityParameterDistribution | Constraint property for restoration probabiltiy distribution |
| restoreCompletionProbability : Real | Value property for completion probability of restoration action (the value of the CDF for resteoration at a specified time) |
| restoreNonCompletionProbability : Real | Value property for the complement of the restoration completion probability |
| restorationRate : Real | Value property for the ratio of the restoration pdf to the restoration CDF at a specified time |
| restorationShape : Real | Value property shapre parameter of the restoration probability distribution |
| restorationScale : Real | Value property for the scale parameter of the restoration distribution |
| restorationLocation : Real | Value property for the location parameter of the restoration distribution (value of the location parameter must be less than the specified time) |
| availabilityCalc : RatioOfPartToTotal | Constraint property for the calculation of availability |

#### NonRestorableComponentReliabilitySituation

Package: RBD Library

isAbstract: Yes

Generalization: ComponentReliabilitySituation

Applied Stereotype: «Situation»

Description

Situation for norestorable component RBDs.  Specialization of ComponentReliabilitySituation. Owns  MTTF value property

Figure 9.209 - NonRestorableComponentReliabilitySituation

Attributes

| MTTF : time | Value property for the Mean Time to Failure (MTTF is used for non-restorable components or systems) |
| --- | --- |

#### SystemReliabilitySituation

Package: RBD Library

isAbstract: Yes

Generalization: AbstractReliabilitySituation, Scenario

Applied Stereotype: «Situation»

Description

The parent situation for calculating a system reliability or availablity.  Speciaalization of AbstractReliabilitySituation.

Figure 9.210 - SystemReliabilitySituation

Attributes

| rb : AbstractReliabilitySituation[0..*] (member end of association, redefines scenarioStep) | Part property of SystemReliabiltiySituation of type Restorable (see Restorable situation) |
| --- | --- |

#### RestorableSystemReliabilitySituation

Package: RBD Library

isAbstract: Yes

Generalization: Restorable, SystemReliabilitySituation

Applied Stereotype: «Situation»

Description

Situation for Resetorable Systems.  Specialization of SystemReliabiltiySituation and part of Restorable Situation

Figure 9.211 - RestorableSystemReliabilitySituation

Attributes

| rb : Restorable[0..*] (member end of association, redefines rb) | Part property of RestorableSystemReliabiltiySituation of type Restorable (see Restorable situation) |
| --- | --- |

#### InSeries

Package: RBD Library

isAbstract: Yes

Generalization: SystemReliabilitySituation

Applied Stereotype: «Situation»

Description

Situation for calculation of reliability oir avaliability for series systems.  Specialization of SystemReliabilitySituation.  Owns InSeries constraint block.

Figure 9.212 - InSeries

Attributes

| rc : InSeriesCalculation | Constraint property for series reliability calculation |
| --- | --- |

#### RestorableInSeries

Package: RBD Library

isAbstract: Yes

Generalization: InSeries, RestorableSystemReliabilitySituation

Applied Stereotype: «Situation»

Description

Situation for calculation of avaliability for restorable series systems.  Specialization of SystemReliabilitySituation and the InSeries situation.  Owns InSeries constraint block.

Figure 9.213 - RestorableInSeries

Attributes

| ac : InSeriesCalculation | Constraint property for availability |
| --- | --- |

#### InParallel

Package: RBD Library

isAbstract: Yes

Generalization: SystemReliabilitySituation

Applied Stereotype: «Situation»

Description

Situation for calculation of reliability oir avaliability  for parallel systems.  Specialization of SystemReliabilitySituation.  Owns InParallel constraint block.

Figure 9.214 - InParallel

Attributes

| rc : InParallelCalculation | Constraint property for availability |
| --- | --- |

#### RestorableInParallel

Package: RBD Library

isAbstract: Yes

Generalization: InParallel, RestorableSystemReliabilitySituation

Applied Stereotype: «Situation»

Description

Situation for calculation of avaliability  for restorable parallel systems.  Specialization of RestorableSystemReliabilitySituation and InParallel situation.  Owns InParallel constraint block.

Figure 9.215 - RestorableInParallel

Attributes

| ac : InParallelCalculation | Constraint property for availability |
| --- | --- |

#### HomogeneousKofN

Package: RBD Library

isAbstract: Yes

Generalization: SystemReliabilitySituation

Applied Stereotype: «Situation»

Description

Situation for calculation of reliability or availablity for Homogenerous K out of N systems.  Specialization of SystemReliabilitySituation.  Owns HomogenousKofN constraint block.

Figure 9.216 - HomogeneousKofN

Attributes

| rc : HomogeneousKofNCalculation | Constraint propertyfor series reliability calculation |
| --- | --- |
| k : Integer | Value property for number of items needed for operation |
| n : Integer | Value property for Number of items installed or ready at start of operation |

#### RestorableHomogeneousKofN

Package: RBD Library

isAbstract: Yes

Generalization: HomogeneousKofN, RestorableSystemReliabilitySituation

Applied Stereotype: «Situation»

Description

Situation for calculation of avaliability for Homogeneous K out of N systems.  Specialization of RestorablSystemReliabiltiySituation  and the InSeries situation.  Owns KofN constraint block.

Figure 9.217 - RestorableHomogeneousKofN

Attributes

| ac : HomogeneousKofNCalculation | Constraint property for availability |
| --- | --- |

#### HeterogenerousKofN

Package: RBD Library

isAbstract: Yes

Generalization: SystemReliabilitySituation

Applied Stereotype: «Situation»

Description

Situation for calculation of reliability or availablity for Heterogenerous K out of N systems.  Specialization of SystemReliabilitySituation.  Owns HeterogenousKofN constraint block.

Figure 9.218 - HeterogenerousKofN

Attributes

| rc : HeterogeneousKofNCalculation | Constraint property for series reliability calculation |
| --- | --- |
| k : Integer | Number of items needed for operation |

#### RestorableHeterogeneousKofN

Package: RBD Library

isAbstract: Yes

Generalization: HeterogenerousKofN, RestorableSystemReliabilitySituation

Applied Stereotype: «Situation»

Description

Situation for calculation of avaliability for Heterogeneous K out of N systems.  Specialization of RestorablSystemReliabiltiySituation  and the InSeries situation.  Owns HeterogenousKofN constraint block.

Figure 9.219 - RestorableHeterogeneousKofN

Attributes

| ac : HeterogeneousKofNCalculation | Constraint property for availability |
| --- | --- |

    - Methods::RBD::RBD Library::ConstraintBlocks

#### Methods::RBD::RBD Library::ConstraintBlocks::Probability

###### OneVariableFunction

Package: Probability

isAbstract: Yes

Applied Stereotype: «ConstraintBlock»

Description

This is an abstract constraint block which defines an input and an output for a constraint expression which has a single input and output

Figure 9.220 – OneVariableFunction

Attributes

| x : Real | The input parameter |
| --- | --- |
| y : Real | The output parameter |

###### ProbabilityComplementFunction

Package: Probability

isAbstract: No

Generalization: OneVariableFunction

Applied Stereotype: «ConstraintBlock»

Description

This constraint block calculates the complemenet (i.e., 1 - ) the input and is used for converting behind reliability and failure probability or between restoration completion success probability and restoration completion failure probabiltiy

Figure 9.221 – ProbabilityComplementFunction

Constraints

| [1] | y=1.0-x |
| --- | --- |

###### ReciprocalFunction

Package: Probability

isAbstract: No

Generalization: OneVariableFunction

Applied Stereotype: «ConstraintBlock»

Description

Provides the reciprocal of the input

Figure 9.222 – ReciprocalFunction

Constraints

| [1] | y=1.0/x |
| --- | --- |

###### RatioOfPartToTotal

Package: Probability

isAbstract: No

Applied Stereotype: «ConstraintBlock»

Description

A constraint expression to calculate the proportion of one value to its sum (e.g., availability = MTBF/(MTBF+MTTR))

Figure 9.223 – RationOfPartToTotal

Attributes

| x : Real | The numerator |
| --- | --- |
| y : Real | The other value |
| ratio : Real | =x/(x+y) |

Constraints

| [1] | ratio = x/(x+y) |
| --- | --- |

###### ProbabilityDensityFunction

Package: Probability

isAbstract: Yes

Applied Stereotype: «ConstraintBlock»

Description

This is an abstract constraint block which defines 3 inputs (shape, scale, and locatrion) and one output for a probability density function

Figure 9.224 – ProbabilityDensityFunction

Attributes

| time : time | The specified operating (exposure) time |
| --- | --- |
| shape : Real | the shape parameter of a probability distribution (see NIST/Sematech Engineering Statistics Handbook, section 8.1.6) https://www.itl.nist.gov/div898/handbook/apr/section1/apr16.htm |
| scale : Real | the scale parameter of a probability distribution (see NIST/Sematech Engineering Statistics Handbook, section 8.1.6) https://www.itl.nist.gov/div898/handbook/apr/section1/apr16.htm |
| location : time | the location parameter of a probability distribution (see NIST/Sematech Engineering Statistics Handbook, section 8.1.6) https://www.itl.nist.gov/div898/handbook/apr/section1/apr16.htm |
| probabilityDensity : Real | The output parameter providing the value of the probability density function at the specified operating time |

###### ExponentialProbabilityDensityFunction

Package: Probability

isAbstract: No

Generalization: ProbabilityDensityFunction

Applied Stereotype: «ConstraintBlock»

Description

This constraint block calculates the cumulative distribution (CDF) for function for the exponential distribution.

Figure 9.225 – ExponentialProbabilityDensityFunction

Constraints

| [1] | probabilityDensity = scale*Math.pow(Math.E, -1.0*scale*(time-location)) |
| --- | --- |

###### WeibullProbabilityDensityFunction

Package: Probability

isAbstract: No

Generalization: ProbabilityDensityFunction

Applied Stereotype: «ConstraintBlock»

Description

This constraint block calculates the probablity desnity function (PDF) for function for the Weibull distribution.

Figure 9.226 – WeibullProbabilityDensityFunction

Constraints

| [1] | probabilityDencity = Math.pow(Math.E, -1.0 * Math.pow((time-location)/scale, shape)) |
| --- | --- |

###### LognormalProbabilityDensityFunction

Package: Probability

isAbstract: No

Generalization: ProbabilityDensityFunction

Applied Stereotype: «ConstraintBlock»

Description

This constraint block calculates the probablity desnity function (PDF) for function for the lognormal distribution.

Figure 9.227 – LognormalProbabilityDensityFunction

Constraints

| [1] | probabilityDensity = Math.pow(Math.E, -1.0 * Math.pow((time-location)/scale, shape)) |
| --- | --- |

###### CumulativeDistributionFunction

Package: Probability

isAbstract: Yes

Applied Stereotype: «ConstraintBlock»

Description

This abstract constraint block defines the parameters and is the parent of specific cumulative distribution constraint blocks (exponential, Weibull, lognormal, etc.).  The parameters include the shape, scale, and location paraemters of the distribtuion, the time at which the value is to be evaluated, and the resultant probability (an output result).  This probability is defined for the reliabiltiy distribution.  Note that the cumulative is for the reliabiltiy distribution, which is often the complement of the distribution used by other software tools (e.g., Excel or R)

Figure 9.228 – CumulativeDistributionFunction

Attributes

| time : time | The time at which the CDF is evaluated. If a location parameter is used, then the value of time must be greater than the value of the location paraemter |
| --- | --- |
| shape : Real | The shape parameter for the CDF |
| scale : Real | The scale parameter for the CDF |
| location : Real | The location parameter (in units of time) for the CDF |
| probability : Real | The output probability calculated by the CDF |

###### ExponentialCumulativeDistributionFunction

Package: Probability

isAbstract: No

Generalization: CumulativeDistributionFunction

Applied Stereotype: «ConstraintBlock»

Description

This constraint block calculates the cumulative distribution (CDF) for function for the exponential distribution.  See Cumulative Distribution Function constraint block note  explaining how the CDF is defined.

Figure 9.229 – ExponentialCumulativeDistributionFunction

Constraints

| [1] | probability = Math.pow(Math.E, -1.0*scale*(time-location)) |
| --- | --- |

###### WeibullCumulativeDistributionFunction

Package: Probability

isAbstract: No

Generalization: CumulativeDistributionFunction

Applied Stereotype: «ConstraintBlock»

Description

This constraint block calculates the cumulative distribution (CDF) for function for the Weibull  distribution.  See Cumulative Distribution Function constraint block note  explaining how the CDF is defined.

Figure 9.230 – WeibullCumulativeDistributionFunction

Constraints

| [1] | probability = Math.pow(Math.E, -1.0 * Math.pow((time-location)/scale, shape)) |
| --- | --- |

###### LognormalCumulativeDistributionFunction

Package: Probability

isAbstract: No

Generalization: CumulativeDistributionFunction

Applied Stereotype: «ConstraintBlock»

Description

This constraint block calculates the cumulative distribution (CDF) for function for the lognormal distribution.  See Cumulative Distribution Function constraint block note  explaining how the CDF is defined.

Figure 9.231 – LognormalCumulativeDistributionFunction

Constraints

| [1] | probability = Math.pow(Math.E, -1.0 * Math.pow((time-location)/scale, shape)) |
| --- | --- |

###### MeanFunction

Package: Probability

isAbstract: Yes

Applied Stereotype: «ConstraintBlock»

Description

This abstract constraint blocks defines the input and output paraemters for calculating the mean and is the parent of specific cumulative distribution constraint blocks (exponential, Weibull, lognormal, etc.).  The parameters include the shape, scale, and location paraemters of the distribtuion, the time at which the value is to be evaluated, and the resultant probability (an output result).  The mean is defined for the reliabiltiy distribution.

Figure 9.232 – MeanFunction

Attributes

| time : time | The time at which the mean function is evaluated. If a location parameter is used, then the value of time must be greater than the value of the location paraemter |
| --- | --- |
| shape : Real | The shape parameter for the probabiltiy distribution for which the mean is being calculated |
| scale : Real | The scale parameter of the probabiltiy distribution function for which the mean is being calculated |
| location : Real | The location parameer of the probabiltiy distribution function for which the mean is being calculated |
| meanTimeToEvent : time | The output of the mean function |

###### ExponentialMeanFunction

Package: Probability

isAbstract: No

Generalization: MeanFunction

Applied Stereotype: «ConstraintBlock»

Description

This constraint block calculates the mean of the exponential function (i.e., MTBF or MTTR).  Note that for the exponential function, the mean is equiavlent to the reciprocal of the hazard function.

Figure 9.233 – ExponentialMeanFunction

Constraints

| [1] | meanTimeToEvent = 1.0/scale |
| --- | --- |

###### WeibullMeanFunction

Package: Probability

isAbstract: No

Generalization: MeanFunction

Applied Stereotype: «ConstraintBlock»

Description

This constraint block calculates the mean of the Webiull distribution (i.e., MTBF or MTTR).  Note that for the exponential function, the mean is equiavlent to the reciprocal of the hazard function.

Figure 9.234 – WeibullMeanFunction

Constraints

| [1] | meanTimeToEvent = Math.pow(Math.E, -1.0 * Math.pow((time-location)/scale, shape)) |
| --- | --- |

###### LognormalMeanFunction

Package: Probability

isAbstract: No

Generalization: MeanFunction

Applied Stereotype: «ConstraintBlock»

Description

This constraint block calculates the mean of the lognormal function (i.e., MTBF or MTTR).  Note that for the exponential function, the mean is equiavlent to the reciprocal of the hazard function.

Figure 9.235 – LognormalMeanFunction

Constraints

| [1] | meanTimeToEvent = Math.pow(Math.E, -1.0 * Math.pow((time-location)/scale, shape)) |
| --- | --- |

###### HazardFunction

Package: Probability

isAbstract: Yes

Applied Stereotype: «ConstraintBlock»

Description

This abstract constraint block defines the parameters used in the hazard function. The hazard function is the ratio of the probability density function and the cumulative distribtuion function

Figure 9.236 – HazardFunction

Attributes

| time : time | The time at which the hazard function is being evaluated |
| --- | --- |
| shape : Real | The shape parameter for the hazard for which the hazard is being calculated |
| scale : Real | The scale parameter for the hazard function for which the sis being calculated |
| location : Real | The location parameter for the hazard function for which the sis being calculated |
| eventRate : Real | The output of the hazard distribution function |

###### ExponentialHazardFunction

Package: Probability

isAbstract: No

Generalization: HazardFunction, MeanFunction

Applied Stereotype: «ConstraintBlock»

Description

This is the constraint block is a specialization of the hazard function constraint block and calculates the hazard function (failure rate) for the exponential distribution.  Note that for the exponential distribution, the hazard function is a constant over time.

Figure 9.237 – ExponentialHazardFunction

Constraints

| [1] | eventRate = scale |
| --- | --- |

###### WeibullHazardFunction

Package: Probability

isAbstract: No

Generalization: HazardFunction, MeanFunction

Applied Stereotype: «ConstraintBlock»

Description

This is the constraint block is a specialization of the hazard function constraint block and calculates the hazard function (failure rate) for the Weibull distribution.  .

Figure 9.238 – WeibullHazardFunction

Constraints

| [1] | eventRate = Math.pow(Math.E, -1.0 * Math.pow((time-location)/scale, shape)) |
| --- | --- |

###### LognormalHazardFunction

Package: Probability

isAbstract: No

Generalization: HazardFunction, MeanFunction

Applied Stereotype: «ConstraintBlock»

Description

This is the constraint block is a specialization of the hazard function constraint block and calculates the hazard function (failure rate) for the lognormal distribution.  .

Figure 9.239 – LognormalHazardFunction

Constraints

| [1] | eventRate = Math.pow(Math.E, -1.0 * Math.pow((time-location)/scale, shape)) |
| --- | --- |

###### ReliabilityParameterDistribution

Package: Probability

isAbstract: Yes

Applied Stereotype: «ConstraintBlock»

Description

This constraint block is the parent of probability distributions and parameters.  It owns the pdf, cdf, mean, hazard function, and compelement constraint blocks.  The parameters inherited by the children disributions include shape, scale, location, the instantaneous probability density value, probability value, complement value  (e.g., failure probability from reliability), mean time to event (e.g., mean time between failures or mean time to failure), and event rate (e.g., failure rate).  It also includes a time parameter which is the argument for the other paraemters

Figure 9.240 – ReliabilityParameterDistribution

Attributes

| time : time | The time at which the probability function is to be evaluated. Time must be greater than the location value |
| --- | --- |
| shape : Real | The shape parameter of the distribution |
| scale : Real | The scale parameter of the distribution |
| location : Real | The location parameter of the distribution |
| probabilityDensity : Real | The value of the probability density function at a specified time |
| probability : Real | The probability (i.e., value of the CDF) at a specified time |
| pdf : ProbabilityDensityFunction | The constraint expression of the probability density function |
| cdf : CumulativeDistributionFunction | The constraint expression for the cumulative density function |
| mean : MeanFunction | The constraint expression for the mean of the CDF |
| hazardFunction : HazardFunction | The constraint expression for the hazard function (ratio of PDF to CDF) |
| complement : ProbabilityComplementFunction | A general expression to take the complement (i.e., 1-value). |
| probabilityComplement : Real | A general expression to take the complement of the probability (i.e., 1-value). The value for which the complement is calculated must be less than 1 |
| meanTimeToEvent : time | The constraint expression to calculate the mean of the probability distribution |
| eventRate : Real | The constraint expression for determining the event rate (e.g., failure rate or repair rate) of the probability distribution |

###### ExponentialDistribution

Package: Probability

isAbstract: No

Generalization: ReliabilityParameterDistribution

Applied Stereotype: «ConstraintBlock»

Description

This constraint block calculates owns 4 lower level constraint blocks for calculating the values of the cumulative, density, and hazard functions, as well as the mean for the exponential distribution.  Note that for the exponential distribuhtion function, only scale and location parameters are defined

Figure 9.241 – ExponentialDistribution

Attributes

| pdf : ExponentialProbabilityDensityFunction, redefines pdf | The PDF for the exponential distribution defined in the NIST/Sematech Engineering Statistics Handbook, section 8.1.6, https://www.itl.nist.gov/div898/handbook/apr/section1/apr16.htm |
| --- | --- |
| cdf : ExponentialCumulativeDistributionFunction, redefines cdf | The CDF for the exponential distribution defined in the NIST/Sematech Engineering Statistics Handbook, section 8.1.6, https://www.itl.nist.gov/div898/handbook/apr/section1/apr16.htm |
| mean : ExponentialMeanFunction, redefines mean | The mean for the exponential distribution defined in the NIST/Sematech Engineering Statistics Handbook, section 8.1.6, https://www.itl.nist.gov/div898/handbook/apr/section1/apr16.htm |
| hazardFunction : ExponentialHazardFunction, redefines hazardFunction | The hazard function for the exponential distribution defined in the NIST/Sematech Engineering Statistics Handbook, section 8.1.6, https://www.itl.nist.gov/div898/handbook/apr/section1/apr16.htm |

###### WeibullDistribution

Package: Probability

isAbstract: No

Generalization: ReliabilityParameterDistribution

Applied Stereotype: «ConstraintBlock»

Description

This constraint block calculates owns 4 lower level constraint blocks for calculating the values of the cumulative, density, and hazard functions, as well as the mean for the Weibull distribution.

Figure 9.242 – WeibullDistribution

Attributes

| pdf : WeibullProbabilityDensityFunction, redefines pdf | The PDF for the Weibull distribution defined in the NIST/Sematech Engineering Statistics Handbook, section 8.1.6, https://www.itl.nist.gov/div898/handbook/apr/section1/apr16.htm |
| --- | --- |
| cdf : WeibullCumulativeDistributionFunction, redefines cdf | The PDF for the Weibull distribution defined in the NIST/Sematech Engineering Statistics Handbook, section 8.1.6, https://www.itl.nist.gov/div898/handbook/apr/section1/apr16.htm |
| mean : WeibullMeanFunction, redefines mean | The mean for the Weibull distribution defined in the NIST/Sematech Engineering Statistics Handbook, section 8.1.6, https://www.itl.nist.gov/div898/handbook/apr/section1/apr16.htm |
| hazardFunction : WeibullHazardFunction, redefines hazardFunction | The hazard function for the Weibull distribution defined in the NIST/Sematech Engineering Statistics Handbook, section 8.1.6, https://www.itl.nist.gov/div898/handbook/apr/section1/apr16.htm |

###### LognormalDistribution

Package: Probability

isAbstract: No

Generalization: ReliabilityParameterDistribution

Applied Stereotype: «ConstraintBlock»

Description

This constraint block calculates owns 4 lower level constraint blocks for calculating the values of the cumulative, density, and hazard functions, as well as the mean for the lognormal distribution.

Figure 9.243 – LognormalDistribution

Attributes

| pdf : LognormalProbabilityDensityFunction, redefines pdf | The PDF for the lognroml distribution defined in the NIST/Sematech Engineering Statistics Handbook, section 8.1.6, https://www.itl.nist.gov/div898/handbook/apr/section1/apr16.htm |
| --- | --- |
| cdf : LognormalCumulativeDistributionFunction, redefines cdf | The CDF for the lognormal distribution defined in the NIST/Sematech Engineering Statistics Handbook, section 8.1.6, https://www.itl.nist.gov/div898/handbook/apr/section1/apr16.htm |
| mean : LognormalMeanFunction, redefines mean | The mean for the lognormal distribution defined in the NIST/Sematech Engineering Statistics Handbook, section 8.1.6, https://www.itl.nist.gov/div898/handbook/apr/section1/apr16.htm |
| hazardFunction : LognormalHazardFunction, redefines hazardFunction | The hazard function for the lognormal distribution defined in the NIST/Sematech Engineering Statistics Handbook, section 8.1.6, https://www.itl.nist.gov/div898/handbook/apr/section1/apr16.htm |

#### Methods::RBD::RBD Library::ConstraintBlocks::SystemBlocks

###### InSeriesCalculation

Package: SystemBlocks

isAbstract: No

Applied Stereotype: «ConstraintBlock»

Description

Constraint block for calculation of series system reliability or availability

Figure 9.244 – InseriesCalculation

Attributes

| input : Real[0..*] | Probabilities (reliablity or availability) of input components |
| --- | --- |
| output : Real | Series system reliability or availability |

Constraints

| [1] | if (input && !input.isEmpty()) { output = 1.0; } for (var i = input.iterator(); i.hasNext();) { output = i.next() * output; } |
| --- | --- |

###### InParallelCalculation

Package: SystemBlocks

isAbstract: No

Applied Stereotype: «ConstraintBlock»

Description

Constraint block for calculation of parallel system reliability or availability

Figure 9.245 – InparalllelCalculation

Attributes

| input : Real[0..*] | Reliability or availability of components |
| --- | --- |
| output : Real[1] | Reliability or availability of system |

Constraints

| [1] | if (input && !input.isEmpty()) { output = 1.0; } for (var i = input.iterator(); i.hasNext();) { output = (1.0 - i.next()) * output; } output = 1.0 - output; |
| --- | --- |

###### HomogeneousKofNCalculation

Package: SystemBlocks

isAbstract: No

Applied Stereotype: «ConstraintBlock»

Description

Constraint block for calculation of k out of n system reliability or availability

Figure 9.246 – HomogeneousKofNCalculation

Attributes

| input : Real[1] | Reliability or availability of components (all components must have equal reliability or availability) |
| --- | --- |
| output : Real[1] | k ou tof n system reliability or availability |
| k : Integer[1] | Number of components or channels needed for system ro operate |
| n : Integer[1] | Number of components installed in system |

Constraints

| [1] | function factorial (n) { if (n <2) return 1; return factorial(n-1)*n; } function C_k_of_n (k, n) { return (factorial(n)/factorial(k))/factorial(n-k); } var R = input; var result = 0.0; for (var i = k; i <=n; ++i) { result += C_k_of_n(i, n) * Math.pow(R, i) * Math.pow(1.0-R, n-i); } output = result; |
| --- | --- |

###### HeterogeneousKofNCalculation

Package: SystemBlocks

isAbstract: No

Applied Stereotype: «ConstraintBlock»

Description

Constraint block for calculation of k out of n system reliability or availability where components have different reliabilities or availabilities

Figure 9.247 – HeterogeneousKofNCalculation

Attributes

| input : Real[0..*] | Reliability or availability of components (components do not necessarily have equal reliability or availability) |
| --- | --- |
| output : Real[1] | Reliability or availability of system |
| k : Integer[1] | Number of components or channels needed for system ro operate |

Constraints

| [1] | function getCombinations(array, min) { var res = []; function iter(i, temp, count) { if (i === array.length) { if (count >= min) { res.push(temp); } return; } iter(i + 1, temp.concat(array[i]), count + 1); iter(i + 1, temp.concat(1.0 - array[i]), count); } iter(0, [], 0); return res; } function and(arrayOfProbabilities) { var output = 1.0; for (i = 0; i < arrayOfProbabilities.length; i++) { output = arrayOfProbabilities[i] * output; } return output; } function sum(arrayOfProbabilities) { var output = 0.0; for (i = 0; i < arrayOfProbabilities.length; i++) { output = output + arrayOfProbabilities[i]; } return output; } var allCombinations = getCombinations(input.toArray(), k); var andsArray = []; for (j = 0; j < allCombinations.length; j++) { andsArray.push(and(allCombinations[j])); } output = sum(andsArray); |
| --- | --- |

    - Methods::RBD::RBD Profile

#### ReliabilitySituation

Package: RBD Profile

isAbstract: Yes

Generalization: Situation

Extension: Class

Description

A marker stereotype for all reliability situations. See AbstractReliabilitySituation library class for definition.

Figure 9.248 – ReliabilitySituation

#### Restorable

Package: RBD Profile

isAbstract: No

Extension: Class

Description

A mixin stereotype for all restorable reliability situations - both component and system. See Restorable library class for definition.

Figure 9.249 – Restorable

Constraints

| [1] RestorableIsRestorable | self.base_Class->asSet()->closure(general).name->includes('Restorable') |
| --- | --- |

#### ComponentReliability

Package: RBD Profile

isAbstract: No

Generalization: ReliabilitySituation

Extension: Class

Description

A marker stereotype for non-composite reliability situations. See ComponentReliabilitySituation library class for definition.

Figure 9.250 –ReliabilitySituation

Constraints

| [1] ComponentReliabilityIsComponentReliabilitySituation | self.base_Class->asSet()->closure(general).name->includes('ComponentReliabilitySituation') |
| --- | --- |

#### SystemReliability

Package: RBD Profile

isAbstract: Yes

Generalization: ReliabilitySituation

Extension: Class

Description

A marker stereotype for composite reliability situations. See SystemReliabilitySituation library class for definition.

Figure 9.251 –SystemReliability

#### InSeries

Package: RBD Profile

isAbstract: No

Generalization: SystemReliability

Extension: Class

Description

A marker stereotype for in-series composite reliability situations. See InSeries library class for definition.

Figure 9.252 – InSeries

Constraints

| [1] InSeriesIsInSeries | self.base_Class->asSet()->closure(general).name->includes('InSeries') |
| --- | --- |

#### InParallel

Package: RBD Profile

isAbstract: No

Generalization: SystemReliability

Extension: Class

Description

A marker stereotype for in-parallel composite reliability situations. See InParallel library class for definition.

Figure 9.253 – InParallel

Constraints

| [1] InParallelIsInParallel | self.base_Class->asSet()->closure(general).name->includes('InParallel') |
| --- | --- |

#### HomogeneousKofN

Package: RBD Profile

isAbstract: No

Generalization: SystemReliability

Extension: Class

Description

A marker stereotype for homogeneous k-of-n composite reliability situations. See HomogeneousKofN library class for definition.

Figure 9.254 – HomogeneousKofN

Constraints

| [1] HomogeneousKofNIsHomogeneousKofN | self.base_Class->asSet()->closure(general).name->includes('HomogeneousKofN') |
| --- | --- |

#### HeterogeneousKofN

Package: RBD Profile

isAbstract: No

Generalization: SystemReliability

Extension: Class

Description

A marker stereotype for heterogeneous k-of-n composite reliability situations. See HeterogenerousKofN library class for definition.

Figure 9.255 – HeterogeneousKofN

Constraints

| [1] HeterogeneousKofNIsHeterogeneousKofN | self.base_Class->asSet()->closure(general).name->includes('HeterogeneousKofN') |
| --- | --- |

Views

Core

Core::Core Library

View  Core::Core Library::Core Library

Figure 10.1 – Core Library

Elements

- AnySituation

- Causality

Core::Core Profile

View  Core::Core Profile::CoreProfile

Figure 10.2 – CoreProfile

Elements

- ControllingMeasure

- RelevantTo

- Situation

- Violates

General

General::General Concepts Library

View  General::General Concepts Library::General Concepts Library

Figure 10.3 - General Concepts Library

Elements

- AbstractCause

- AbstractEffect

- AbstractEvent

- AbstractFailureMode

- AbstractRisk

- Activation

- AnySituation

- Causality

- Cause

- DysfunctionalEvent

- Effect

- ErrorPropagation

- ErrorRealization

- FailureMode

- HarmPotential

- Hazard

- Scenario

- UndesiredState

General::General Concepts Profile

View  General::General Concepts Profile::General Concepts Profile

Figure 10.4 – General Concepts Profile

Elements

- ControllingMeasure

- Detection

- Error

- FailureMode

- FailureState

- Fault

- Mitigation

- Prevention

- Recommendation

- Situation

- Undeveloped

- Hazard

- Item

- PresentIn

- SingleElementItem

- ElementGroupBasedItem

- Assumption

General Security

General Security::General Security Concepts Library

#### View  General Security::General Security Concepts Library::General Security Concepts Library

Figure 10.5 - General Security Concepts Library

Elements

- AbstractRisk

- AnySituation

- Causality

- Factor

- HarmPotential

- Hazard

- Limitation

- Scenario

- Threat

- Vulnerability

- Weakness

General Security::General Security Concepts Profile

#### View  General Security::General Security Concepts Profile::General Security Concepts Profile

Figure 10.6 - General Security Concepts Profile

Elements

- Asset

- Hazard

- Impacts

- PresentedBy

- RelevantTo

- SecurityActor

- Situation

- Threat

- Valuates

Methods::FMEA

Methods::FMEA::FMEA Library

View  Methods::FMEA::FMEA Library::FMEA Library

Figure 10.7 – FMEA Library

Elements

- AbstractCause

- AbstractEffect

- AbstractFailureMode

- AbstractFMEAItem

- AbstractRisk

- Cause

- DegradationOfFunction

- DelayedFunction

- Effect

- ExceedingFunction

- FailureMode

- FMEAItem

- IntermittentFunction

- LossOfFunction

- PartialFunction

- UnintendedFunction

Methods::FMEA::FMEA Profile

View  Methods::FMEA::FMEA Profile::FMEA Profile

Figure 10.8 – FMEA Profile

Elements

- FMEAItem

Methods::FTA

Methods::FTA::FTALibrary

Methods::FTA::FTALibrary::Events

View  Methods::FTA::FTALibrary::Events::Events

Figure 10.9 – Events

Elements

- BasicEvent

- ConditionalEvent

- DormantEvent

- Event

- HouseEvent

- IntermediateEvent

- TopEvent

- UndevelopedEvent

- ZeroEvent

View  Methods::FTA::FTALibrary::FTA Library

Figure 10.10 – FTA Library

Elements

- AbstractEvent

- AND

- AnySituation

- Causality

- DysfunctionalEvent

- Event

- FTAElement

- FTATree

- Gate

- INHIBIT

- MAJORITY_VOTE

- NOT

- OR

- Scenario

- SEQ

- XOR

Methods::FTA::FTAProfile

Methods::FTA::FTAProfile::Diagrams by elements

View  Methods::FTA::FTAProfile::FTA Profile

Figure 10.11 – FTA Profile

Elements

- AND

- BasicEvent

- ConditionalEvent

- DormantEvent

- Event

- Gate

- HouseEvent

- INHIBIT

- IntermediateEvent

- MAJORITY_VOTE

- NOT

- OR

- SEQ

- Situation

- TopEvent

- TransferIn

- TransferOut

- Tree

- Undeveloped

- XOR

- ZeroEvent

Methods::STPA

Methods::STPA::STPA Library

View  Methods::STPA::STPA Library::STPA Library

Figure 10.12 – STPA Library

Elements

- AbstractCause

- AbstractEffect

- AbstractEvent

- AbstractOperationalSituation

- AbstractRisk

- AnySituation

- Causality

- Early

- Factor

- HarmPotential

- Hazard

- Inadequate Control Execution

- Inadequate Controller Decisions

- Inadequate Feedback and Inputs

- Inadequate Process Behavior

- Late

- Limitation

- Loss

- LossScenario

- NotProvided

- OperationalCondition

- OutOfSequence

- ProcessModelFlaw

- ProcessModelFlawConsequence

- ProcessModelFlawFactor

- Provided

- RiskRealization

- Scenario

- Threat

- TooLong

- TooShort

- UndesiredState

- UndesiredControlAction

- UndesiredControlActionHarmPotential

- Vulnerability

- Weakness

Methods::STPA::STPA Profile

View  Methods::STPA::STPA Profile::STPA Profile

Figure 10.13 – STPA Profile

Elements

- Actuator

- ControlAction

- ControlledProcess

- Controller

- ControlStructure

- FailureMode

- Feedback

- Sensor

- UndesiredControlAction

- UnsafeControlAction

GSN

GSN::GSN Profile

View  GSN::GSN Profile::GSN Profile

Figure 10.14 – GSN Profile

Elements

- Assumption

- Context

- Goal

- GSNArgumentNode

- GSNNode

- InContextOf

- Justification

- Solution

- Strategy

- SupportedBy

- ContextualInformation

- Undeveloped

Methods::ISO 26262

Methods::ISO 26262::ISO 26262 Library

View  Methods::ISO 26262::ISO 26262 Library::ISO26262 Library

Figure 10.15 – ISO 26262 Library

Elements

- AbstractEffect

- AbstractEvent

- AbstractOperationalSituation

- AbstractRisk

- AccidentScenario

- AnyMalfunction

- AnySituation

- ASIL

- AutomotiveEffect

- Causality

- Controllability

- DysfunctionalEvent

- Early

- EnvironmentalCondition

- Exposure

- HarmPotential

- Hazard

- HazardousEvent

- Intermittent

- Inverted

- ISO26262SafetyRequirementTemplate

- Late

- Less

- Location

- More

- No

- OperationalCondition

- RoadCondition

- Scenario

- Severity

- SystemLevelEffect

- TrafficAndPeople

- TypicalAutomotiveSituation

- UndesiredState

- Unintended

- VehicleLevelEffect

- VehicleUsage

View  Methods::ISO 26262::ISO 26262 Library::All-Encompassing Operational Situations

Figure 10.16 – All-Encompassing Operational Situations

Elements

- AnyEnvironmentalCondition

- AnyLocation

- AnyRoadCondition

- AnyTrafficAndPeople

- AnyVehicleUse

- EnvironmentalCondition

- Location

- RoadCondition

- TrafficAndPeople

- VehicleUsage

Methods::ISO 26262::ISO 26262 Profile

Methods::ISO 26262::ISO 26262 Profile::RequirementManagement

View  Methods::ISO 26262::ISO 26262 Profile::RequirementManagement::RequirementManagement

Figure 10.17 – RequirementManagement

Elements

- ASILDecompose

- DependabilityRequirement

- FunctionalSafetyRequirement

- HardwareSafetyRequirement

- IndependenceRequirement

- OperatingMode

- RecoveryRequirement

- SafeState

- SafetyGoal

- SoftwareSafetyRequirement

- TechnicalSafetyRequirement

- UserInfoRequirement

View  Methods::ISO 26262::ISO 26262 Profile::ISO26262 Profile

Figure 10.18 – ISO 26262 Profile

Elements

- ASILAssignment

- ASILOverrideRationale

- Confirmed

- ControllingMeasure

- FailureMode

- HazardAndRiskAssessment

- IDCarrier

- LessonLearned

- MalfunctioningBehavior

- Mitigation

- OperationalSituation

- Situation

- Verified

  - Methods::FHA

Methods::FHA::FHA Library

##### View  Methods::FHA::FHA Library::FHA Library

Figure 10.19 - FHA Library

Elements

- ARP 4761A FailureConditionClassification

- DAL

- DevelopmentAssuranceLevel

- EnvironmentalCondition

- ExternalEvent

- FailureCondition

- FailureEffect

- FDAL

- FlightPhase

- FunctionalFailure

- IDAL

- LossOfFunction

- Malfunction

- OperationalCondition

- OperationalEvent

- PartialLossOfFunction

      - Methods::FHA::FHA Profile

##### View  Methods::FHA::FHA Profile::FHA Profile

Figure 10.20 - FHA Profile

Elements

- FunctionalFailure

  - Methods::RBD

    - Methods::RBD::RBD Library

#### View  Methods::RBD::RBD Library::RBD Library

Figure 10.21 - RBD Library

Elements

- AbstractReliabilitySituation

- AnySituation

- ComponentReliabilitySituation

- HeterogenerousKofN

- HomogeneousKofN

- InParallel

- InSeries

- NonRestorableComponentReliabilitySituation

- Restorable

- RestorableComponentReliabilitySituation

- RestorableHeterogeneousKofN

- RestorableHomogeneousKofN

- RestorableInParallel

- RestorableInSeries

- RestorableSystemReliabilitySituation

- Scenario

- SystemReliabilitySituation

#### View  Methods::RBD::RBD Library::AbstractReliabilitySituation::AbstractReliabilitySituation

Figure 10.22 - AbstractReliabilitySituation

#### View  Methods::RBD::RBD Library::ComponentReliabilitySituation::ComponentReliabilitySituation

Figure 10.23 - ComponentReliabilitySituation

View  Methods::RBD::RBD Library::NonRestorableComponentReliabilitySituation::NonRestorableComponentReliabilitySituation

Figure 10.24 - InSeries

#### View  Methods::RBD::RBD Library::RestorableInSeries::RestorableInSeries

Figure 10.25 - RestorableInSeries

#### View  Methods::RBD::RBD Library::InParallel::InParallel

Figure 10.26 - InParallel

#### View  Methods::RBD::RBD Library::RestorableInParallel::RestorableInParallel

Figure 10.27 - RestorableInParallel

#### View  Methods::RBD::RBD Library::HomogeneousKofN::HomogeneousKofN

Figure10.28 - HomogeneousKofN

#### View  Methods::RBD::RBD Library::RestorableHomogeneousKofN::RestorableHomogeneousKofN

Figure 10.29 - RestorableHomogeneousKofN

#### View  Methods::RBD::RBD Library::HeterogenerousKofN::HeterogeneousKofN

Figure 10.30 - HeterogeneousKofN

#### View  Methods::RBD::RBD Library::RestorableHeterogeneousKofN::RestorableHeterogeneousKofN

Figure 10.31 - RestorableHeterogeneousKofN

    - Methods::RBD::RBD Library::ConstraintBlocks

#### Methods::RBD::RBD Library::ConstraintBlocks::Probability

###### View  Methods::RBD::RBD Library::ConstraintBlocks::Probability::Distributions

Figure 10.32 - Distributions

Elements

- ExponentialDistribution

- LognormalDistribution

- ReliabilityParameterDistribution

- WeibullDistribution

###### View  Methods::RBD::RBD Library::ConstraintBlocks::Probability::ReliabilityParameterDistribution::ReliabilityParameterDistribution

Figure 10.33 - ReliabilityParameterDistribution

    - Methods::RBD::RBD Profile

#### View  Methods::RBD::RBD Profile::RBD Profile

Figure 10.34 - RBD Profile

Elements

- ComponentReliability

- HeterogeneousKofN

- HomogeneousKofN

- InParallel

- InSeries

- ReliabilitySituation

- Restorable

- Situation

- SystemReliability

Figure 10.35 - NonRestorableComponentReliabilitySituation

#### View  Methods::RBD::RBD Library::RestorableComponentReliabilitySituation::RestorableComponentReliabilitySituation

Figure 10.36 - RestorableComponentReliabilitySituation

https://psas.scripts.mit.edu/home/get_file.php?name=STPA_handbook.pdf