    - Approach

Figure 8.30: Approach

Table 8.58: Approach

| Description: This component is to be used to move a specified part close to the target pose. This component performs autonomous movement to a specified pose. Approach function is essential for typical robot services to move a hand or arm to a target pose. This component allows Service Application to command robots to perform approach without considering the actual arm configuration or degrees of freedom. The target pose shall be specified as a spatial pose based on a coordinate system appropriate to the specified part to be moved. The actual motion trajectory to the target pose, and strategies such as trajectory generation and obstacle avoidance are left to the the component implementation. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Command Method |  |  |  |  |
| set_parameter |  | Specifies parameters for approach. |  |  |
| argument | action_target | UnlimitedNatural | M | Part of the robot structure which the action is performed. Specify the entire robot, hand position, arm posture, etc. |
| argument | target_pose | Data [RLS] | M | Target pose of the action. It is defined in a coordinate system according to the part of the action. |
| Query Method |  |  |  |  |
| get_parameter |  | Obtains parameters for approach. |  |  |
| result | action_target | UnlimitedNatural | M | Part of the robot structure which the action is performed. The entire robot, hand position, arm posture, etc. are specified. |
| result | target_pose | Data [RLS] | M | Target pose of the action. It is defined in a coordinate system according to the part of the action. |

    - Leave

Figure 8.31: Leave

Table 8.59: Leave

| Description: This component is used to move a specified part away to a pose from the target. This component is used in conjunction with Approach. This component performs autonomous return of a specified part to a specified pose, such as to its initial pose. Similar to the approach function, the leave function is essential for typical robot services to move a hand or arm away to a target pose. This component allows Service Application to command robots to perform leave without considering the actual arm configuration or degrees of freedom. The target pose shall be specified as a spatial pose based on a coordinate system appropriate to the specified part being moved. The actual motion trajectory to the target pose, and strategies such as trajectory generation and obstacle avoidance are left to the the component implementation. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Command Method |  |  |  |  |
| set_parameter |  | Specifies parameters for leave. |  |  |
| argument | action_target | UnlimitedNatural | M | The part on which the action is performed. Specify the entire robot, hand position, arm posture, etc. |
| argument | target_pose | Data [RLS] | M | Target pose of the action. It is defined in a coordinate system according to the part of the action. |
| Query Method |  |  |  |  |
| get_parameter |  | Obtains parameters for leave. |  |  |
| result | action_target | UnlimitedNatural | M | The part on which the action is performed. The entire robot, hand position, arm posture, etc. are specified. |
| result | target_pose | Data [RLS] | M | Target pose of the action. It is defined in a coordinate system according to the part of the action. |

    - Touch

Figure 8.32: Touch

Table 8.60: Touch

| Description: This component performs make contact with a human using the specified pose or force. Touch function is essential for robots to perform services that involve human contact. This component is solely responsible for contacting with humans. To move the robot close to the contact point, robot services need to use the approach component in advance. Ensuring safety in the event of contact depends on the component's implementation. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Command Method |  |  |  |  |  |
| set_parameter |  | Specifies parameters for touch. |  |  |  |
| argument | distance_threshold |  | Real | C | Distance threshold to contact location. Used when contact force threshold is not specified. |
| argument | force_threshold |  | Real | C | Contact Force Threshold． Used when distance threshold is not specified. |
| get_parameter |  | Obtains parameters for touch. |  |  |  |
| result | distance_threshold |  | Real | C | Distance threshold to contact location. Setted when force threshold is not specified. |
| result | force_threshold |  | Real | C | Contact Force Threshold． Setted when distance threshold is not specified. |

    - Touch Detection

Figure 8.33: Touch Detection

Table 8.61: Touch Detection

| Description: This component is used for detecting number of touch points. This component notifies a list of identifiers of detected contact parts when the number of contact parts changes. The body part is defined according to the service to be performed, such as the right upper arm, the left lower arm, or the hand. Touch Detection function is essential for typical robotic services that involve contact with some objects. Additionally, for services that do not intend for contact to occur, Touch Detection can also be used to stop the service when contact is detected in order to prevent harm to those around. This component also detects changes in the number of contact points, and can also notify when contact is lost. The method of contact detection, the detectable parts, etc. are left to the component implementation. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Command Method |  |  |  |  |
| set_parameter |  | Specifies parameters for touch detection. |  |  |
| argument | force_threshold | Real | M | The force threshold for determining contact. |
| Query Method |  |  |  |  |
| get_parameter |  | Obtains parameters for touch detection. |  |  |
| result | force_threshold | Real | M | The force threshold for determining contact. |
| Event Method |  |  |  |  |
| touch_detected |  | Notifies touch detected. |  |  |
| argument | timestamp | DateTime [ISO8601] | O | Measurement time. |
| argument | parts_data | List<UnlimitedNatural> | M | A list of identifiers of the parts where contact was detected. |

    - Object Localization

Figure 8.34: Object Localization

Table 8.62: Object Localization

| Description: This component detects the position of a specific type of object. Specifically, it detects the position of a specified type of object. If the RoIS engine (typically a robot) can handle objects in the real world, then this component can be included. Person and sound localization work passively because they transmit information about an object as an event when a sensor detects it. However, this component actively detects the location of a specified type of object. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Command Method |  |  |  |  |
| set_parameter |  | Specifies parameters for object localization. |  |  |
| argument | target_kind | String | M | Specific type of object whose position is to be detected |
| argument | accuracy | Real | O | Accuracy of position localization of the object. Unit:% |
| argument | recognition_time | UnlimitedNatural | O | Minimum time needed to recognize the object. Unit:sec |
| Query Method |  |  |  |  |
| get_parameter |  | Obtains parameters for object localization. |  |  |
| result | target_kind | String | M | Specific type of object whose position is to be detected |
| result | accuracy | Real | O | Accuracy of position localization of the object. Unit:% |
| result | recognition_time | UnlimitedNatural | O | Minimum time needed to recognize the object. Unit:sec |
| result | timestamp | DateTime [ISO8601] | O | Time detection is executed |
| result | object_ref | List<RoIS_Identifier> | M | List of detected object IDs. Reference information related to the ID shall be provided with the each ID. By referring to the reference for the IDs, the Service Application can understand the relationship between the obtained IDs and the other IDs that are obtained from another component. |
| result | position_data | List<Data> [RLS] | M | List of detected object data. Each data entry at least contains position of the detected object. This may also be accompanied with additional information such as pose of the detected object. It may also contain certainty of the detection act. |

    - Object Grasping

Figure 8.35: Object Grasping

Table 8.63: Object Grasping

| Description: This component is used to grasp a specified object. It simply performs the grasping motion and does not move the robot to a position where the object can be grasped. It may be included when the RoIS engine (typically a robot) is handling an object in the real world. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Command Method |  |  |  |  |  |
| set_parameter |  |  | Specifies parameters for object grasping. |  |  |
| argument |  | object_id | RoIS_Identifier | M | Identifier of the grasping object |
| argument |  | grasp_force | Real | M | Required force to hold the object. Unit: N |
| Query Method |  |  |  |  |  |
| get_parameter |  |  | Obtains parameters for object grasping. |  |  |
| result | object_id |  | RoIS_Identifier | M | Identifier of the grasping object |
| result | grasp_force |  | Real | M | Required force to hold the object. Unit: N |

    - Object Release

Figure 8.36: Object Release

Table 8.64: Object Release

| Description: This component is used to grasp a specified object. It simply performs the grasping motion and does not move the robot to a position where the object can be grasped. It may be included when the RoIS engine (typically a robot) is handling an object in the real world. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Command Method |  |  |  |  |
| set_parameter |  | Specifies parameters for object grasping. |  |  |
| argument | object_id | RoIS_Identifier | M | Identifier of the grasping object |
| argument | grasp_force | Real | M | Required force to hold the object. Unit: N |
| Query Method |  |  |  |  |
| get_parameter |  | Obtains parameters for object grasping. |  |  |
| result | object_id | RoIS_Identifier | M | Identifier of the grasping object |
| result | grasp_force | Real | M | Required force to hold the object. Unit: N |

    - Move Arm

Figure 8.37: Move Arm

Table 8.65: Move Arm

| Description: This component moves the tip of the arm to the specified target pose. It specifies the destination position and posture data, then moves the tip of the arm to the target posture. This component may be included when the ERI Engine (usually a robot) handles objects in the physical world. The Move Arm function is essential for moving the end-effector of the arm to the target posture in general robot services. Using this component allows service applications to instruct the arm to move without considering the actual hardware device. Note that the trajectory to the target pose, maximum speed, and other details are left to the component's implementation. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Command Method |  |  |  |  |
| set_parameter |  | Specifies parameters for arm move. |  |  |
| argument | target_pose | Data [RLS] | M | Target pose of the arm tip: Defined as the position and posture within the specified reference coordinate system. |
| Query Method |  |  |  |  |
| get_parameter |  | Obtains parameters for arm move. |  |  |
| result | target_pose | Data [RLS] | M | Target pose of the arm tip: Defined as the position and posture within the specified reference coordinate system. |